LG PBX System

Software Technical Information

STI-0033

Jul. 28, 2000

| Title | #### CSTA Phase II PICS Proforma |
| --- | --- |
| System | CS1000 |
| Abstract | This document is identified for available services in cs1000. The mark “X” in supported field is shown for the current state. |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 07.28.2000 | B.S Kim | First issue |

Table of Contents

1	Implementation identification	6

2	Switching function services	7

2.1	Alternate call	7

2.2	Answer call	7

2.3	Associate data	8

2.4	Call completion	8

2.5	Clear call	9

2.6	Clear connection	9

2.7	Conference call	10

2.8	Consultation call	11

2.9	Divert call	11

2.10	Hold call	12

2.11	Make call	12

2.12	Make predictive call	13

2.13	Park call	14

2.14	Query device	15

2.15	Reconnect call	16

2.16	Retrieve call	16

2.17	Send DTMF tones	17

2.18	Set feature	18

2.19	Single step conference	19

2.20	Single step transfer	20

2.21	Transfer call	21

3	Switching function events	22

3.1	Event report service	22

3.2	Call events	22

3.2.1	Call cleared	22

3.2.2	Conferenced	23

3.2.3	Connection cleared	23

3.2.4	Delivered	24

3.2.5	Diverted	24

3.2.6	Established	25

3.2.7	Failed	25

3.2.8	Held	26

3.2.9	Network reached	26

3.2.10	Originated	26

3.2.11	Queued	27

3.2.12	Retrieved	27

3.2.13	Service initiated	27

3.2.14	Transferred	28

3.3	Agent events	28

3.3.1	Agent logged on	28

3.3.2	Agent logged off	29

3.3.3	Agent ready	29

3.3.4	Agent not ready	29

3.3.5	Agent working after call	30

3.3.6	Agent busy	30

3.4	Other feature events	30

3.4.1	Call information	30

3.4.2	Do not disturb	31

3.4.3	Forwarding	31

3.4.4	Message waiting	31

3.4.5	Auto answer	31

3.4.6	Microphone mute	32

3.4.7	Speaker mute	32

3.4.8	Speaker volume	32

3.5	Maintenance events	32

3.5.1	Back in service	32

3.5.2	Out of service	33

3.6	Private events	33

3.6.1	Private	33

3.7	Voice unit events	33

3.7.1	Play	33

3.7.2	Record	34

3.7.3	Review	34

3.7.4	Stop	34

3.7.5	Suspend play	35

3.7.6	Suspend record	35

3.7.7	Voice attribute change	35

4	Computing function services	36

4.1	Route request	36

4.2	Reroute request	36

4.3	Route select request	37

4.4	Route used request	37

4.5	Route end request	38

5	Bidirectional services	39

5.1	Escape	39

5.2	System status	39

6	Status reporting services	40

6.1	Change monitor filter	40

6.2	Monitor start	41

6.3	Monitor stop	41

6.4	Snapshot call	42

6.5	Snapshot device	42

7	Input/output services	43

7.1	Data path resumed	43

7.2	Data path suspended	43

7.3	Fast data	44

7.4	Resume data path	44

7.5	Send broadcast data	45

7.6	Send data	45

7.7	Send multicast data	46

7.8	Start data path	47

7.9	Stop data path	47

7.10	Suspend data path	48

8	Voice unit services	49

8.1	Concatenate message	49

8.2	Delete message	49

8.3	Play message	50

8.4	Query voice attribute	51

8.5	Record message	52

8.6	Reposition	53

8.7	Resume	53

8.8	Review	54

8.9	Set voice attribute	54

8.10	Stop	55

8.11	Suspend	55

8.12	Synthesize message	56

Implementation Considerations	57

CSTA Phase II parameter size constraints	58

Supported Monitor Types	58

CS1000 CSTA Phase II PICS Proforma

This document content and layout is based on ECMA-218.

# 1	Implementation identification

| Supplier | LGE |
| --- | --- |
| Protocol Version | 1.0.0 |
| Date of Statement | 2000-07-28 |
| Contact point for queries about the PICS | Byeong-Soo Kim |
| Implementation Name(s) and Version(s) |  |
| Other information necessary for full identification - e.g. Name(s) and Version(s) for machines and/or operating systems; system name(s) |  |

# 2	Switching function services

## 2.1	Alternate call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A1 | Alternate call service | S10.1 P9.1 | o |  | Yes [X] No [ ] |
| A1a | Active call | P18.6 | A1:o1 |  | Yes [X] No [ ] |
| A1b | Held call | P18.6 | A1:o1 |  | Yes [X] No [ ] |
| A1c | Private data in request | P18.8 | A1:o |  | Yes [ ] No [X] |
| A1d | Security data in request | P18.7 | A1:o |  | Yes [ ] No [X] |
| A1e | Service result positive | P9.1 | A1:m | [ ] | Yes [X] |
| A1f | Service result negative | P16 | A1:m | [ ] | Yes [X] |
| A1g | Private data in result | P18.8 | A1e:o |  | Yes [ ] No [X] |
| A1h | Security data in result | P18.7 | A1e:o |  | Yes [ ] No [X] |

## 2.2	Answer call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A2 | Answer call service | S10.2 P9.2 | o |  | Yes [X] No [ ] |
| A2a | Call to be answered | P9.2 | A2:m | [ ] | Yes [X] |
| A2b | Private data in request | P18.8 | A2:o |  | Yes [ ] No [X] |
| A2c | Security data in request | P18.7 | A2:o |  | Yes [ ] No [X] |
| A2d | Service result positive | P9.2 | A2:m | [ ] | Yes [X] |
| A2e | Service result negative | P16 | A2:m | [ ] | Yes [X] |
| A2f | Private data in result | P18.8 | A2d:o |  | Yes [ ] No [X] |
| A2g | Security data in result | P18.7 | A2d:o |  | Yes [ ] No [X] |

## 2.3	Associate data

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A3 | Associate data service | S10.3 P9.3 | o |  | Yes [ ] No [X] |
| A3a | Existing call | P9.3 | A3:m | [ ] | Yes [ ] |
| A3b | Account code | P9.3 | A3:o1 |  | Yes [ ] No [ ] |
| A3c | Authorization code | P9.3 | A3:o1 |  | Yes [ ] No [ ] |
| A3d | Correlator data | P9.3 | A3:o1 |  | Yes [ ] No [ ] |
| A3e | Private data in request | P18.8 | A3:o |  | Yes [ ] No [ ] |
| A3f | Security data in request | P18.7 | A3:o |  | Yes [ ] No [ ] |
| A3g | Service result positive | P9.3 | A3:m | [ ] | Yes [ ] |
| A3h | Service result negative | P16 | A3:m | [ ] | Yes [ ] |
| A3i | Private data in result | P18.8 | A3g:o |  | Yes [ ] No [ ] |
| A3j | Security data in result | P18.7 | A3g:o |  | Yes [ ] No [ ] |

## 2.4	Call completion

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A4 | Call completion service | S10.4 P9.4 | o |  | Yes [ ] No [X] |
| A4a | Camp on caller | P18.6 | A4:o1 |  | Yes [ ] No [ ] |
| A4b | Call back caller | P18.6 | A4:o1 |  | Yes [ ] No [ ] |
| A4c | Intrude caller | P18.6 | A4:o1 |  | Yes [ ] No [ ] |
| A4d | Call back message caller | P18.6 | A4:o1 |  | Yes [ ] No [ ] |
| A4e | Private data in request | P18.8 | A4:o |  | Yes [ ] No [ ] |
| A4f | Security data in request | P18.7 | A4:o |  | Yes [ ] No [ ] |
| A4g | Service result positive | P9.4 | A4:m | [ ] | Yes [ ] |
| A4h | Service result negative | P16 | A4:m | [ ] | Yes [ ] |
| A4i | Private data in result | P18.8 | A4g:o |  | Yes [ ] No [ ] |
| A4j | Security data in result | P18.7 | A4g:o |  | Yes [ ] No [ ] |

## 2.5	Clear call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A5 | Clear call service | S10.5 P9.5 | o |  | Yes [ ] No [X] |
| A5a | Call to be cleared | P9.5 | A5:m | [ ] | Yes [ ] |
| A5b | Private data in request | P18.8 | A5:o |  | Yes [ ] No [ ] |
| A5c | Security data in request | P18.7 | A5:o |  | Yes [ ] No [ ] |
| A5d | Service result positive | P9.5 | A5:m | [ ] | Yes [ ] |
| A5e | Service result negative | P16 | A5:m | [ ] | Yes [ ] |
| A5f | Private data in result | P18.8 | A5d:o |  | Yes [ ] No [ ] |
| A5g | Security data in result | P18.7 | A5d:o |  | Yes [ ] No [ ] |

## 2.6	Clear connection

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A6 | Clear connection service | S10.6 P9.6 | o |  | Yes [X] No [ ] |
| A6a | Connection to be cleared | P9.6 | A6:m | [ ] | Yes [X] |
| A6b | Private data in request | P18.8 | A6:o |  | Yes [ ] No [X] |
| A6c | Security data in request | P18.7 | A6:o |  | Yes [ ] No [X] |
| A6d | Service result positive | P9.6 | A6:m | [ ] | Yes [X] |
| A6e | Service result negative | P16 | A6:m | [ ] | Yes [X] |
| A6f | Private data in result | P18.8 | A6d:o |  | Yes [ ] No [X] |
| A6g | Security data in result | P18.7 | A6d:o |  | Yes [ ] No [X] |

## 2.7	Conference call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A7 | Conference call service | S10.7 P9.7 | o |  | Yes [X] No [ ] |
| A7a | Held call | P18.6 | A7:o1 |  | Yes [X] No [ ] |
| A7b | Active call | P18.6 | A7:o1 |  | Yes [X] No [ ] |
| A7c | Private data in request | P18.8 | A7:o |  | Yes [ ] No [X] |
| A7d | Security data in request | P18.7 | A7:o |  | Yes [ ] No [X] |
| A7e | Service result positive | P9.7 | A7:m | [ ] | Yes [X] |
| A7f | Service result negative | P16 | A7:m | [ ] | Yes [X] |
| A7g | Conference call | P9.7 | A7e:m | [ ] | Yes [X] |
| A7h | Connections | P18.4 | A7e:o |  | Yes [X] No [ ] |
| A7i | New connection | P18.4 | A7h:o2 |  | Yes [X] No [ ] |
| A7j | Device ID | P18.4 | A7h:o2 |  | Yes [X] No [ ] |
| A7k | Old connection | P18.4 | A7h:o2 |  | Yes [ ] No [X] |
| A7l | Private data in result | P18.8 | A7e:o |  | Yes [ ] No [X] |
| A7m | Security data in result | P18.7 | A7e:o |  | Yes [ ] No [X] |

## 2.8	Consultation call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A8 | Consultation call service | S10.8 P9.8 | o |  | Yes [X] No [ ] |
| A8a | Existing call | P9.8 | A8:m | [ ] | Yes [X] |
| A8b | Consulted device | P9.8 | A8:m | [ ] | Yes [X] |
| A8c | Consulted device profile | P9.8 | A8:o |  | Yes [ ] No [X] |
| A8d | Device class | P18.6 | A8c:m | [X] | Yes [ ] |
| A8e | ISDN set up | P18.6 | A8c:o |  | Yes [ ] No [X] |
| A8f | Account code | P9.8 | A8:o |  | Yes [ ] No [X] |
| A8g | Authorization code | P9.8 | A8:o |  | Yes [ ] No [X] |
| A8h | Correlator data | P9.8 | A8:o |  | Yes [X] No [ ] |
| A8i | Private data in request | P18.8 | A8:o |  | Yes [ ] No [X] |
| A8j | Security data in request | P18.7 | A8:o |  | Yes [ ] No [X] |
| A8k | Service result positive | P9.8 | A8:m | [ ] | Yes [X] |
| A8l | Service result negative | P16 | A8:m | [ ] | Yes [X] |
| A8m | Initiated call | P9.8 | A8k:m | [ ] | Yes [X] |
| A8n | Private data in result | P18.8 | A8k:o |  | Yes [ ] No [X] |
| A8o | Security data in result | P18.7 | A8k:o |  | Yes [ ] No [X] |

## 2.9	Divert call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A9 | Divert call service | S10.9 P9.9 | o |  | Yes [X] No [ ] |
| A9a | Deflect | P18.6 | A9:o1 |  | Yes [X] No [ ] |
| A9b | Pickup | P18.6 | A9:o1 |  | Yes [X] No [ ] |
| A9c | Group pickup | P18.6 | A9:o1 |  | Yes [ ] No [X] |
| A9d | Correlator data | P9.9 | A9:o |  | Yes [X] No [ ] |
| A9e | Private data in request | P18.8 | A9:o |  | Yes [ ] No [X] |
| A9f | Security data in request | P18.7 | A9:o |  | Yes [ ] No [X] |
| A9g | Service result positive | P9.9 | A9:m | [ ] | Yes [X] |
| A9h | Service result negative | P16 | A9:m | [ ] | Yes [X] |
| A9i | Private data in result | P18.8 | A9g:o |  | Yes [ ] No [X] |
| A9j | Security data in result | P18.7 | A9g:o |  | Yes [ ] No [X] |

## 2.10	Hold call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A10 | Hold call service | S10.10 P9.10 | o |  | Yes [X] No [ ] |
| A10a | Call to be held | P9.10 | A10:m | [ ] | Yes [X] |
| A10b | Connection reservation | P9.10 | A10:o |  | Yes [ ] No [X] |
| A10c | Private data in request | P18.8 | A10:o |  | Yes [ ] No [X] |
| A10d | Security data in request | P18.7 | A10:o |  | Yes [ ] No [X] |
| A10e | Service result positive | P9.10 | A10:m | [ ] | Yes [X] |
| A10f | Service result negative | P16 | A10:m | [ ] | Yes [X] |
| A10g | Private data in result | P18.8 | A10e:o |  | Yes [ ] No [X] |
| A10h | Security data in result | P18.7 | A10e:o |  | Yes [ ] No [X] |

## 2.11	Make call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A11 | Make call service | S10.11 P9.11 | o |  | Yes [X] No [ ] |
| A11a | Calling device | P9.11 | A11:m | [ ] | Yes [X] |
| A11b | Called device | P9.11 | A11:m | [ ] | Yes [X] |
| A11c | Device profile | P9.11 | A11:o |  | Yes [ ] No [X] |
| A11d | Device class | P18.6 | A11c:m | [X] | Yes [ ] |
| A11e | ISDN set up | P18.6 | A11c:o |  | Yes [ ] No [X] |
| A11f | Account code | P9.11 | A11:o |  | Yes [ ] No [X] |
| A11g | Authorization code | P9.11 | A11:o |  | Yes [ ] No [X] |
| A11h | Correlator data | P9.11 | A11:o |  | Yes [X] No [ ] |
| A11i | Private data in request | P18.8 | A11:o |  | Yes [ ] No [X] |
| A11j | Security data in request | P18.7 | A11:o |  | Yes [ ] No [X] |
| A11k | Service result positive | P9.11 | A11:m | [ ] | Yes [X] |
| A11l | Service result negative | P16 | A11:m | [ ] | Yes [X] |
| A11m | Initiated call | P9.11 | A11k:m | [ ] | Yes [X] |
| A11n | Private data in result | P18.8 | A11k:o |  | Yes [ ] No [X] |
| A11o | Security data in result | P18.7 | A11k:o |  | Yes [ ] No [X] |

## 2.12	Make predictive call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A12 | Make predictive call service | S10.12 P9.12 | o |  | Yes [ ] No [X] |
| A12a | Calling device | P9.12 | A12:m | [ ] | Yes [ ] |
| A12b | Called device | P9.12 | A12:m | [ ] | Yes [ ] |
| A12c | Allocation | P9.12 | A12:o |  | Yes [ ] No [ ] |
| A12d | Device profile | P9.12 | A12:o |  | Yes [ ] No [ ] |
| A12e | Device class | P18.6 | A12d:m | [ ] | Yes [ ] |
| A12f | ISDN set up | P18.6 | A12d:o |  | Yes [ ] No [ ] |
| A12g | Account code | P9.12 | A12:o |  | Yes [ ] No [ ] |
| A12h | Authorization code | P9.12 | A12:o |  | Yes [ ] No [ ] |
| A12i | Correlator data | P9.12 | A12:o |  | Yes [ ] No [ ] |
| A12j | Private data in request | P18.8 | A12:o |  | Yes [ ] No [ ] |
| A12k | Security data in request | P18.7 | A12:o |  | Yes [ ] No [ ] |
| A12l | Service result positive | P9.12 | A12:m | [ ] | Yes [ ] |
| A12m | Service result negative | P16 | A12:m | [ ] | Yes [ ] |
| A12n | Initiated call | P9.12 | A12l:m | [ ] | Yes [ ] |
| A12o | Private data in result | P18.8 | A12l:o |  | Yes [ ] No [ ] |
| A12p | Security data in result | P18.7 | A12l:o |  | Yes [ ] No [ ] |

## 2.13	Park call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A13 | Park call service | S10.13 P9.13 | o |  | Yes [ ] No [X] |
| A13a | Call to park | P9.13 | A13:m | [ ] | Yes [ ] |
| A13b | Park to | P9.13 | A13:m | [ ] | Yes [ ] |
| A13c | Park device profile | P9.13 | A13:o |  | Yes [ ] No [ ] |
| A13d | Device class | P18.6 | A13c:m | [ ] | Yes [ ] |
| A13e | ISDN set up | P18.6 | A13c:o |  | Yes [ ] No [ ] |
| A13f | Correlator data | P9.13 | A13:o |  | Yes [ ] No [ ] |
| A13g | Private data in request | P18.8 | A13:o |  | Yes [ ] No [ ] |
| A13h | Security data in request | P18.7 | A13:o |  | Yes [ ] No [ ] |
| A13i | Service result positive | P9.13 | A13:m | [ ] | Yes [ ] |
| A13j | Service result negative | P16 | A13:m | [ ] | Yes [ ] |
| A13k | Private data in result | P18.8 | A13i:o |  | Yes [ ] No [ ] |
| A13l | Security data in result | P18.7 | A13i:o |  | Yes [ ] No [ ] |

## 2.14	Query device

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A14 | Query device service | S10.14 P9.14 | o |  | Yes [X] No [ ] |
| A14a | Device | P9.14 | A14:m | [ ] | Yes [X] |
| A14b | Feature | P9.14 | A14:m | [ ] | Yes [X] |
| A14c | Private data in request | P18.8 | A14:o |  | Yes [ ] No [X] |
| A14d | Security data in request | P18.7 | A14:o |  | Yes [ ] No [X] |
| A14e | Service result positive | P9.14 | A14:m | [ ] | Yes [X] |
| A14f | Service result negative | P16 | A14:m | [ ] | Yes [X] |
| A14g | Device information in service result | P9.14 | A14e:m | [ ] | Yes [X] |
| A14h | Message waiting on | P18.6 | A14g:o1 |  | Yes [X] No [ ] |
| A14i | Do not disturb on | P18.6 | A14g:o1 |  | Yes [X] No [ ] |
| A14j | Forward | P18.6 | A14g:o1 |  | Yes [X] No [ ] |
| A14k | Forwarding type | P18.6 | A14j:m | [ ] | Yes [X] |
| A14l | Forward to device | P18.6 | A14j:m | [ ] | Yes [X] |
| A14m | Device information | P18.6 | A14g:o1 |  | Yes [X] No [ ] |
| A14n | Device ID | P18.6 | A14m:o |  | Yes [X] No [ ] |
| A14o | Device type | P18.6 | A14m:m | [ ] | Yes [X] |
| A14p | Device class | P18.6 | A14m:m | [ ] | Yes [X] |
| A14q | Agent state | P18.6 | A14g:o1 |  | Yes [X] No [ ] |
| A14r | Routing enabled | P18.6 | A14g:o1 |  | Yes [ ] No [X] |
| A14s | Auto answer on | P18.6 | A14g:o1 |  | Yes [X] No [ ] |
| A14t | Microphone mute on | P18.6 | A14g:o1 |  | Yes [X] No [ ] |
| A14u | Speaker mute on | P18.6 | A14g:o1 |  | Yes [X] No [ ] |
| A14v | Speaker volume | P18.6 | A14g:o1 |  | Yes [X] No [ ] |
| A14w | Private data in result | P18.8 | A14e:o |  | Yes [ ] No [X] |
| A14x | Security data in result | P18.7 | A14e:o |  | Yes [ ] No [X] |

## 2.15	Reconnect call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A15 | Reconnect call service | S10.15 P9.15 | o |  | Yes [X] No [ ] |
| A15a | Held call | P18.6 | A15:o1 |  | Yes [X] No [ ] |
| A15b | Active call | P18.6 | A15:o1 |  | Yes [X] No [ ] |
| A15c | Private data in request | P18.8 | A15:o |  | Yes [ ] No [X] |
| A15d | Security data in request | P18.7 | A15:o |  | Yes [ ] No [X] |
| A15e | Service result positive | P9.15 | A15:m | [ ] | Yes [X] |
| A15f | Service result negative | P16 | A15:m | [ ] | Yes [X] |
| A15g | Private data in result | P18.8 | A15e:o |  | Yes [ ] No [X] |
| A15h | Security data in result | P18.7 | A15e:o |  | Yes [ ] No [X] |

## 2.16	Retrieve call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A16 | Retrieve call service | S10.16 P9.16 | o |  | Yes [X] No [ ] |
| A16a | Call to be retrieved | P9.16 | A16:m | [ ] | Yes [X] |
| A16b | Private data in request | P18.8 | A16:o |  | Yes [ ] No [X] |
| A16c | Security data in request | P18.7 | A16:o |  | Yes [ ] No [X] |
| A16d | Service result positive | P9.16 | A16:m | [ ] | Yes [X] |
| A16e | Service result negative | P16 | A16:m | [ ] | Yes [X] |
| A16f | Private data in result | P18.8 | A16d:o |  | Yes [ ] No [X] |
| A16g | Security data in result | P18.7 | A16d:o |  | Yes [ ] No [X] |

## 2.17	Send DTMF tones

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A17 | Send DTMF tones service | S10.17 P9.17 | o |  | Yes [X] No [ ] |
| A17a | Connection to send | P9.17 | A17:m | [ ] | Yes [X] |
| A17b | Characters to send | P9.17 | A17:m | [ ] | Yes [X] |
| A17c | Tone duration | P9.17 | A17:o |  | Yes [ ] No [X] |
| A17d | Pause duration | P9.17 | A17:o |  | Yes [ ] No [X] |
| A17e | Private data in request | P18.8 | A17:o |  | Yes [ ] No [X] |
| A17f | Security data in request | P18.7 | A17:o |  | Yes [ ] No [X] |
| A17g | Service result positive | P9.17 | A17:m | [ ] | Yes [X] |
| A17h | Service result negative | P16 | A17:m | [ ] | Yes [X] |
| A17i | Private data in result | P18.8 | A17g:o |  | Yes [ ] No [X] |
| A17j | Security data in result | P18.7 | A17g:o |  | Yes [ ] No [X] |

## 2.18	Set feature

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A18 | Set feature service | S10.18 P9.18 | o |  | Yes [X] No [ ] |
| A18a | Device | P9.18 | A18:m | [ ] | Yes [X] |
| A18b | Feature | P9.18 | A18:m | [ ] | Yes [X] |
| A18c | Message waiting on | P18.6 | A18b:o1 |  | Yes [ ] No [X] |
| A18d | Do not disturb on | P18.6 | A18b:o1 |  | Yes [X] No [ ] |
| A18e | Forward | P18.6 | A18b:o1 |  | Yes [X] No [ ] |
| A18f | Forward immediate | P18.6 | A18e:o2 | [ ] | Yes [X] No [ ] |
| A18g | Forward busy | P18.6 | A18e:o2 | [ ] | Yes [X] No [ ] |
| A18h | Forward no answer | P18.6 | A18e:o2 | [ ] | Yes [X] No [ ] |
| A18i | Forward busy internal | P18.6 | A18e:o2 | [ ] | Yes [ ] No [X] |
| A18j | Forward busy external | P18.6 | A18e:o2 | [ ] | Yes [ ] No [X] |
| A18k | Forward no answer internal | P18.6 | A18e:o2 | [ ] | Yes [ ] No [X] |
| A18l | Forward no answer external | P18.6 | A18e:o2 | [ ] | Yes [ ] No [X] |
| A18m | Forward immediate internal | P18.6 | A18e:o2 | [ ] | Yes [ ] No [X] |
| A18n | Forward immediate external | P18.6 | A18e:o2 | [ ] | Yes [ ] No [X] |
| A18o | Forward to device | P18.6 | A18e:m | [ ] | Yes [X] |
| A18p | Agent state | P18.6 | A18b:o1 |  | Yes [X] No [ ] |
| A18q | Agent working after call | P18.6 | A18p:o3 | [ ] | Yes [X] No [ ] |
| A18r | Agent ready | P18.6 | A18p:o3 | [ ] | Yes [X] No [ ] |
| A18s | Agent logged on info | P18.6 | A18p:o3 | [ ] | Yes [X] No [ ] |
| A18t | Agent logged off info | P18.6 | A18p:o3 | [ ] | Yes [X] No [ ] |
| A18u | Agent not ready | P18.6 | A18p:o3 | [ ] | Yes [X] No [ ] |
| A18v | Enable routing | P18.6 | A18b:o1 |  | Yes [ ] No [X] |
| A18w | Auto answer on | P18.6 | A18b:o1 |  | Yes [X] No [ ] |
| A18x | Microphone mute on | P18.6 | A18b:o1 |  | Yes [X] No [ ] |
| A18y | Speaker mute on | P18.6 | A18b:o1 |  | Yes [ ] No [X] |
| A18z | Speaker volume | P18.6 | A18b:o1 |  | Yes [X] No [ ] |
| A18aa | Device profile | P9.18 | A18:o |  | Yes [ ] No [X] |
| A18ab | Device class | P18.6 | A18aa:m | [X] | Yes [ ] |
| A18ac | ISDN set up | P18.6 | A18aa:o |  | Yes [ ] No [X] |
| A18ad | Private data in request | P18.8 | A18:o |  | Yes [ ] No [X] |
| A18ae | Security data in request | P18.7 | A18:o |  | Yes [ ] No [X] |
| A18af | Service result positive | P9.18 | A18:m | [ ] | Yes [X] |
| A18ag | Service result negative | P16 | A18:m | [ ] | Yes [X] |
| A18ah | Private data in result | P18.8 | A18af:o |  | Yes [ ] No [X] |
| A18ai | Security data in result | P18.7 | A18af:o |  | Yes [ ] No [X] |

A18w, A18x, A18z will be only support LGP keyset.

## 2.19	Single step conference

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A19 | Single step conference service | S10.19 P9.19 | o |  | Yes [ ] No [X] |
| A19a | Active call | P9.19 | A19:m | [ ] | Yes [ ] |
| A19b | Device to join | P9.19 | A19:m | [ ] | Yes [ ] |
| A19c | Participation type | P9.19 | A19:o |  | Yes [ ] No [ ] |
| A19d | Device profile | P9.19 | A19:o |  | Yes [ ] No [ ] |
| A19e | Device class | P18.6 | A19d:m | [ ] | Yes [ ] |
| A19f | ISDN set up | P18.6 | A19d:o |  | Yes [ ] No [ ] |
| A19g | Account information | P9.19 | A19:o |  | Yes [ ] No [ ] |
| A19h | Authorization code | P9.19 | A19:o |  | Yes [ ] No [ ] |
| A19i | Correlator data | P9.19 | A19:o |  | Yes [ ] No [ ] |
| A19j | Private data in request | P18.8 | A19:o |  | Yes [ ] No [ ] |
| A19k | Security data in request | P18.7 | A19:o |  | Yes [ ] No [ ] |
| A19l | Service result positive | P9.19 | A19:m | [ ] | Yes [ ] |
| A19m | Service result negative | P16 | A19:m | [ ] | Yes [ ] |
| A19n | Conferenced call | P9.19 | A19l:m | [ ] | Yes [ ] |
| A19o | Private data in result | P18.8 | A19l:o |  | Yes [ ] No [ ] |
| A19p | Security data in result | P18.7 | A19l:o |  | Yes [ ] No [ ] |

## 2.20	Single step transfer

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A20 | Single step transfer service | S10.20 P9.20 | o |  | Yes [ ] No [X] |
| A20a | Active call | P9.20 | A20:m | [ ] | Yes [ ] |
| A20b | Device to transfer | P9.20 | A20:m | [ ] | Yes [ ] |
| A20c | Device profile | P9.20 | A20:o |  | Yes [ ] No [ ] |
| A20d | Device class | P18.6 | A20c:m | [ ] | Yes [ ] |
| A20e | ISDN set up | P18.6 | A20c:o |  | Yes [ ] No [ ] |
| A20f | Account information | P9.20 | A20:o |  | Yes [ ] No [ ] |
| A20g | Authorization code | P9.20 | A20:o |  | Yes [ ] No [ ] |
| A20h | Correlator data | P9.20 | A20:o |  | Yes [ ] No [ ] |
| A20i | Private data in request | P18.8 | A20:o |  | Yes [ ] No [ ] |
| A20j | Security data in request | P18.7 | A20:o |  | Yes [ ] No [ ] |
| A20k | Service result positive | P9.20 | A20:m | [ ] | Yes [ ] |
| A20l | Service result negative | P16 | A20:m | [ ] | Yes [ ] |
| A20m | Transferred call | P9.20 | A20k:o |  | Yes [ ] No [ ] |
| A20n | Connections | P18.4 | A20k:o |  | Yes [ ] No [ ] |
| A20o | New connection | P18.4 | A20n:o1 |  | Yes [ ] No [ ] |
| A20p | Device ID | P18.4 | A20n:o1 |  | Yes [ ] No [ ] |
| A20q | Old connection | P18.4 | A20n:o1 |  | Yes [ ] No [ ] |
| A20r | Private data in result | P18.8 | A20k:o |  | Yes [ ] No [ ] |
| A20s | Security data in result | P18.7 | A20k:o |  | Yes [ ] No [ ] |

## 2.21	Transfer call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A21 | Transfer call service | S10.21 P9.21 | o |  | Yes [X] No [ ] |
| A21a | Transfer information | P9.21 | A21:m | [ ] | Yes [X] |
| A21b | Held call | P18.6 | A21a:o1 |  | Yes [X] No [ ] |
| A21c | Active call | P18.6 | A21a:o1 |  | Yes [X] No [ ] |
| A21d | Private data in request | P18.8 | A21:o |  | Yes [ ] No [X] |
| A21e | Security data in request | P18.7 | A21:o |  | Yes [ ] No [X] |
| A21f | Service result positive | P9.21 | A21:m | [ ] | Yes [X] |
| A21g | Service result negative | P16 | A21:m | [ ] | Yes [X] |
| A21h | Transferred call | P9.21 | A21f:o |  | Yes [X] No [ ] |
| A21i | Connections | P18.4 | A21f:o |  | Yes [X] No [ ] |
| A21j | New connection | P18.4 | A21i:o2 |  | Yes [X] No [ ] |
| A21k | Device ID | P18.4 | A21i:o2 |  | Yes [ ] No [X] |
| A21l | Old connection | P18.4 | A21i:o2 |  | Yes [ ] No [X] |
| A21m | Private data in result | P18.8 | A21f:o |  | Yes [ ] No [X] |
| A21n | Security data in result | P18.7 | A21f:o |  | Yes [ ] No [X] |

# 3	Switching function events

## 3.1	Event report service

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B1 | Event report service | S11.2 P10 | o |  | Yes [X] No [ ] |
| B1a | Monitor cross reference | P5.4 P10 | B1:m | [ ] | Yes [X] |
| B1b | Event specific information | P10 | B1:m | [ ] | Yes [X] |
| B1c | Private data | P18.8 | B1:o |  | Yes [ ] No [X] |
| B1d | Security data | P18.7 | B1:o |  | Yes [ ] No [X] |

## 3.2	Call events

### 3.2.1	Call cleared

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B2 | Call cleared event | S11.2.3.1 P10.1.1 | B1b:o |  | Yes [ ] No [X] |
| B2a | Cleared call | P10.1.1 | B2:m | [ ] | Yes [ ] |
| B2b | Local connection state | P10.1.1 | B2:o |  | Yes [ ] No [ ] |
| B2c | Correlator data | P10.1.1 | B2:o |  | Yes [ ] No [ ] |
| B2d | Cause | P10.1.1 | B2:o |  | Yes [ ] No [ ] |

### 3.2.2	Conferenced

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B3 | Conferenced event | S11.2.3.2 P10.1.2 | B1b:o |  | Yes [X] No [ ] |
| B3a | Primary old call | P10.1.2 | B3:m | [ ] | Yes [X] |
| B3b | Conference controller | P10.1.2 | B3:m | [ ] | Yes [X] |
| B3c | Added party | P10.1.2 | B3:m | [ ] | Yes [X] |
| B3d | Secondary old call | P10.1.2 | B3:C | [ ] | Yes [X] No [ ] |
| B3e | Connections | P10.1.2 | B3:o |  | Yes [X] No [ ] |
| B3f | New connection | P18.4 | B3e:o1 |  | Yes [X] No [ ] |
| B3g | Device ID | P18.4 | B3e:o1 |  | Yes [ ] No [X] |
| B3h | Old connection | P18.4 | B3e:o1 |  | Yes [ ] No [X] |
| B3i | Local connection information | P10.1.2 | B3:o |  | Yes [ ] No [X] |
| B3j | Correlator data | P10.1.2 | B3:o |  | Yes [ ] No [X] |
| B3k | Cause | P10.1.2 | B3:o |  | Yes [X] No [ ] |

- C: If provided in previous events then mandatory

else optional

### 3.2.3	Connection cleared

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B4 | Connection cleared event | S11.2.3.3 P10.1.3 | B1b:o |  | Yes [X] No [ ] |
| B4a | Dropped connection | P10.1.3 | B4:m | [ ] | Yes [X] |
| B4b | Releasing device | P10.1.3 | B4:m | [ ] | Yes [X] |
| B4c | Local connection information | P10.1.3 | B4:o |  | Yes [X] No [ ] |
| B4d | Correlator data | P10.1.3 | B4:o |  | Yes [ ] No [X] |
| B4e | Cause | P10.1.3 | B4:o |  | Yes [ ] No [X] |

### 3.2.4	Delivered

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B5 | Delivered event | S11.2.3.4 P10.1.4 | B1b:o |  | Yes [X] No [ ] |
| B5a | Connection | P10.1.4 | B5:m | [ ] | Yes [X] |
| B5b | Alerting device | P10.1.4 | B5:m | [ ] | Yes [X] |
| B5c | Calling device | P10.1.4 | B5:m | [ ] | Yes [X] |
| B5d | Called device | P10.1.4 | B5:m | [ ] | Yes [X] |
| B5e | Last redirection device | P10.1.4 | B5:m | [ ] | Yes [X] |
| B5f | Originating connection | P10.1.4 | B5:o |  | Yes [ ] No [X] |
| B5g | Local connection information | P10.1.4 | B5:o |  | Yes [X] No [ ] |
| B5h | Correlator data | P10.1.4 | B5:o |  | Yes [X] No [ ] |
| B5i | Cause | P10.1.4 | B5:o |  | Yes [X] No [ ] |

Correlator data depends on application service flow.

### 3.2.5	Diverted

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B6 | Diverted event | S11.2.3.5 P10.1.5 | B1b:o |  | Yes [X] No [ ] |
| B6a | Diverted connection | P10.1.5 | B6:C | [ ] | Yes [X] No [ ] |
| B6b | Diverting device | P10.1.5 | B6:m | [ ] | Yes [X] |
| B6c | New destination | P10.1.5 | B6:m | [ ] | Yes [X] |
| B6d | Local connection information | P10.1.5 | B6:o |  | Yes [X] No [ ] |
| B6e | Correlator data | P10.1.5 | B6:o |  | Yes [ ] No [X] |
| B6f | Cause | P10.1.5 | B6:o |  | Yes [X] No [ ] |

- C: If the call alerted the device then mandatory

else optional

### 3.2.6	Established

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B7 | Established event | S11.2.3.6 P10.1.6 | B1b:o |  | Yes [X] No [ ] |
| B7a | Established connection | P10.1.6 | B7:m | [ ] | Yes [X] |
| B7b | Answering device | P10.1.6 | B7:m | [ ] | Yes [X] |
| B7c | Calling device | P10.1.6 | B7:m | [ ] | Yes [X] |
| B7d | Called device | P10.1.6 | B7:m | [ ] | Yes [X] |
| B7e | Last redirection device | P10.1.6 | B7:m | [ ] | Yes [X] |
| B7f | Originating connection | P10.1.6 | B7:o |  | Yes [ ] No [X] |
| B7g | Local connection information | P10.1.6 | B7:o |  | Yes [X] No [ ] |
| B7h | Correlator data | P10.1.6 | B7:o |  | Yes [X] No [ ] |
| B7i | Cause | P10.1.6 | B7:o |  | Yes [X] No [ ] |

Correlator data depends on application service flow.

### 3.2.7	Failed

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B8 | Failed event | S11.2.3.7 P10.1.7 | B1b:o |  | Yes [X] No [ ] |
| B8a | Failed connection | P10.1.7 | B8:m | [ ] | Yes [X] |
| B8b | Failed device | P10.1.7 | B8:m | [ ] | Yes [X] |
| B8c | Called device | P10.1.7 | B8:m | [ ] | Yes [X] |
| B8d | Local connection information | P10.1.7 | B8:o |  | Yes [X] No [ ] |
| B8e | Correlator data | P10.1.7 | B8:o |  | Yes [ ] No [X] |
| B8f | Cause | P10.1.7 | B8:o |  | Yes [X] No [ ] |

### 3.2.8	Held

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B9 | Held event | S11.2.3.8 P10.1.8 | B1b:o |  | Yes [X] No [ ] |
| B9a | Held connection | P10.1.8 | B9:m | [ ] | Yes [X] |
| B9b | Holding device | P10.1.8 | B9:m | [ ] | Yes [X] |
| B9c | Local connection information | P10.1.8 | B9:o |  | Yes [X] No [ ] |
| B9d | Correlator data | P10.1.8 | B9:o |  | Yes [ ] No [X] |
| B9e | Cause | P10.1.8 | B9:o |  | Yes [ ] No [X] |

### 3.2.9	Network reached

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B10 | Network reached event | S11.2.3.9 P10.1.9 | B1b:o |  | Yes [X] No [ ] |
| B10a | Connection | P10.1.9 | B10:m | [ ] | Yes [X] |
| B10b | Trunk used | P10.1.9 | B10:m | [ ] | Yes [X] |
| B10c | Called device | P10.1.9 | B10:m | [ ] | Yes [X] |
| B10d | Local connection information | P10.1.9 | B10:o |  | Yes [X] No [ ] |
| B10e | Correlator data | P10.1.9 | B10:o |  | Yes [ ] No [X] |
| B10f | Cause | P10.1.9 | B10:o |  | Yes [ ] No [X] |

### 3.2.10	Originated

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B11 | Originated event | S11.2.3.10 P10.1.10 | B1b:o |  | Yes [X] No [ ] |
| B11a | Originated connection | P10.1.10 | B11:m | [ ] | Yes [X] |
| B11b | Calling device | P10.1.10 | B11:m | [ ] | Yes [X] |
| B11c | Called device | P10.1.10 | B11:m | [ ] | Yes [X] |
| B11d | Originating device | P10.1.10 | B11:o |  | Yes [ ] No [X] |
| B11e | Local connection information | P10.1.10 | B11:o |  | Yes [X] No [ ] |
| B11f | Correlator data | P10.1.10 | B11:o |  | Yes [ ] No [X] |
| B11g | Cause | P10.1.10 | B11:o |  | Yes [ ] No [X] |

### 3.2.11	Queued

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B12 | Queued event | S11.2.3.11 P10.1.11 | B1b:o |  | Yes [X] No [ ] |
| B12a | Queued connection | P10.1.11 | B12:m | [ ] | Yes [X] |
| B12b | Queue | P10.1.11 | B12:m | [ ] | Yes [X] |
| B12c | Calling device | P10.1.11 | B12:m | [ ] | Yes [X] |
| B12d | Called device | P10.1.11 | B12:m | [ ] | Yes [X] |
| B12e | Last redirection device | P10.1.11 | B12:m | [ ] | Yes [X] |
| B12f | Number of calls in queue | P10.1.11 | B12:o |  | Yes [X] No [ ] |
| B12g | Calls in front | P10.1.11 | B12:o |  | Yes [ ] No [X] |
| B12h | Local connection information | P10.1.11 | B12:o |  | Yes [X] No [ ] |
| B12i | Correlator data | P10.1.11 | B12:o |  | Yes [ ] No [X] |
| B12j | Cause | P10.1.11 | B12:o |  | Yes [X] No [ ] |

Number of calls in queue is current queue’s depth.

### 3.2.12	Retrieved

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B13 | Retrieved event | S11.2.3.12 P10.1.12 | B1b:o |  | Yes [X] No [ ] |
| B13a | Retrieved connection | P10.1.12 | B13:m | [ ] | Yes [X] |
| B13b | Retrieving device | P10.1.12 | B13:m | [ ] | Yes [X] |
| B13c | Local connection information | P10.1.12 | B13:o |  | Yes [X] No [ ] |
| B13d | Correlator data | P10.1.12 | B13:o |  | Yes [ ] No [X] |
| B13e | Cause | P10.1.12 | B13:o |  | Yes [ ] No [X] |

### 3.2.13	Service initiated

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B14 | Service initiated event | S11.2.3.13 P10.1.13 | B1b:o |  | Yes [X] No [ ] |
| B14a | Initiated connection | P10.1.13 | B14:m | [ ] | Yes [X] |
| B14b | Local connection information | P10.1.13 | B14:o |  | Yes [X] No [ ] |
| B14c | Cause | P10.1.13 | B14:o |  | Yes [ ] No [X] |

### 3.2.14	Transferred

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B15 | Transferred event | S11.2.3.14 P10.1.14 | B1b:o |  | Yes [X] No [ ] |
| B15a | Primary old call | P10.1.14 | B15:m | [ ] | Yes [X] |
| B15b | Secondary old call | P10.1.14 | B15:C | [ ] | Yes [X] No [ ] |
| B15c | Transferring device | P10.1.14 | B15:m | [ ] | Yes [X] |
| B15d | Transferred-to device | P10.1.14 | B15:m | [ ] | Yes [X] |
| B15e | Connections | P10.1.14 | B15:o |  | Yes [X] No [ ] |
| B15f | New connection | P18.4 | B15e:o1 |  | Yes [X] No [ ] |
| B15g | Device ID | P18.4 | B15e:o1 |  | Yes [ ] No [X] |
| B15h | Old connection | P18.4 | B15e:o1 |  | Yes [ ] No [X] |
| B15i | Local connection information | P10.1.14 | B15:o |  | Yes [X] No [ ] |
| B15j | Correlator data | P10.1.14 | B15:o |  | Yes [ ] No [X] |
| B15k | Cause | P10.1.14 | B15:o |  | Yes [X] No [ ] |

- C: If parameter in previous events then mandatory

else optional

## 3.3	Agent events

### 3.3.1	Agent logged on

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B16 | Agent logged on event | S11.2.2.2 P10.3.2 | B1b:o |  | Yes [X] No [ ] |
| B16a | Agent device | P10.3.2 | B16:m | [ ] | Yes [X] |
| B16b | Agent ID | P10.3.2 | B16:o |  | Yes [X] No [ ] |
| B16c | Agent group | P10.3.2 | B16:o |  | Yes [X] No [ ] |
| B16d | Password | P10.3.2 | B16:o |  | Yes [ ] No [X] |
| B16e | Cause | P10.3.2 | B16:o |  | Yes [ ] No [X] |

Agent group is the same CS1000’s ACD split representation number.

### 3.3.2	Agent logged off

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B17 | Agent logged off event | S11.2.2.1 P10.3.3 | B1b:o |  | Yes [X] No [ ] |
| B17a | Agent device | P10.3.3 | B17:m | [ ] | Yes [X] |
| B17b | Agent ID | P10.3.3 | B17:o |  | Yes [X] No [ ] |
| B17c | Password | P10.3.3 | B17:o |  | Yes [ ] No [X] |
| B17d | Agent group | P10.3.3 | B17:o |  | Yes [X] No [ ] |
| B17e | Cause | P10.3.3 | B17:o |  | Yes [ ] No [X] |

### 3.3.3	Agent ready

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B18 | Agent ready event | S11.2.2.4 P10.3.5 | B1b:o |  | Yes [X] No [ ] |
| B18a | Agent device | P10.3.5 | B18:m | [ ] | Yes [X] |
| B18b | Agent ID | P10.3.5 | B18:o |  | Yes [X] No [ ] |
| B18c | Cause | P10.3.5 | B18:o |  | Yes [ ] No [X] |

### 3.3.4	Agent not ready

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B19 | Agent not ready event | S11.2.2.3 P10.3.4 | B1b:o |  | Yes [X] No [ ] |
| B19a | Agent device | P10.3.4 | B19:m | [ ] | Yes [X] |
| B19b | Agent ID | P10.3.4 | B19:o |  | Yes [X] No [ ] |
| B19c | Cause | P10.3.4 | B19:o |  | Yes [ ] No [X] |

### 3.3.5	Agent working after call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B20 | Agent working after call event | S11.2.2.5 P10.3.6 | B1b:o |  | Yes [X] No [ ] |
| B20a | Agent device | P10.3.6 | B20:m | [ ] | Yes [X] |
| B20b | Agent ID | P10.3.6 | B20:o |  | Yes [X] No [ ] |
| B20c | Agent group | P10.3.6 | B20:o |  | Yes [X] No [ ] |
| B20d | Cause | P10.3.6 | B20:o |  | Yes [ ] No [X] |

### 3.3.6	Agent busy

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B21 | Agent busy event | S11.2.2.6 P10.3.1 | B1b:o |  | Yes [X] No [ ] |
| B21a | Agent device | P10.3.1 | B21:m | [ ] | Yes [X] |
| B21b | Agent ID | P10.3.1 | B21:o |  | Yes [X] No [ ] |
| B21c | Agent group | P10.3.1 | B21:o |  | Yes [X] No [ ] |
| B21d | Cause | P10.3.1 | B21:o |  | Yes [ ] No [X] |

## 3.4	Other feature events

### 3.4.1	Call information

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B22 | Call information event | S11.2.4.1 P10.2.2 | B1b:o |  | Yes [ ] No [X] |
| B22a | Connection ID | P10.2.2 | B22:m | [ ] | Yes [ ] |
| B22b | Device | P10.2.2 | B22:m | [ ] | Yes [ ] |
| B22c | Account information | P10.2.2 | B22:o |  | Yes [ ] No [ ] |
| B22d | Authorization code | P10.2.2 | B22:o |  | Yes [ ] No [ ] |
| B22e | Correlator data | P10.2.2 | B22:o |  | Yes [ ] No [ ] |

### 3.4.2	Do not disturb

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B23 | Do not disturb event | S11.2.4.2 P10.2.3 | B1b:o |  | Yes [X] No [ ] |
| B23a | Device | P10.2.3 | B23:m | [ ] | Yes [X] |
| B23b | Do not disturb on | P10.2.3 | B23:m | [ ] | Yes [X] |

### 3.4.3	Forwarding

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B24 | Forwarding event | S11.2.4.3 P10.2.4 | B1b:o |  | Yes [X] No [ ] |
| B24a | Device | P10.2.4 | B24:m | [ ] | Yes [X] |
| B24b | Forwarding type | P18.6 | B24:m | [ ] | Yes [X] |
| B24c | Forward DN | P18.6 | B24:o |  | Yes [X] No [ ] |

ForwardDN is internal telephone number.

### 3.4.4	Message waiting

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B25 | Message waiting event | S11.2.4.4 P10.2.5 | B1b:o |  | Yes [ ] No [X] |
| B25a | Device for message | P10.2.5 | B25:m | [ ] | Yes [X] |
| B25b | Invoking device | P10.2.5 | B25:m | [ ] | Yes [X] |
| B25c | Message waiting on | P10.2.5 | B25:m | [ ] | Yes [X] |

### 3.4.5	Auto answer

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B26 | Auto answer event | S11.2.4.5 P10.2.1 | B1b:o |  | Yes [X] No [ ] |
| B26a | Device | P10.2.1 | B26:m | [ ] | Yes [X] |
| B26b | Auto answer on | P10.2.1 | B26:m | [ ] | Yes [X] |

### 3.4.6	Microphone mute

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B27 | Microphone mute event | S11.2.4.6 P10.2.6 | B1b:o |  | Yes [X] No [ ] |
| B27a | Device | P10.2.6 | B27:m | [ ] | Yes [X] |
| B27b | Microphone mute on | P10.2.6 | B27:m | [ ] | Yes [X] |

### 3.4.7	Speaker mute

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B28 | Speaker mute event | S11.2.4.7 P10.2.7 | B1b:o |  | Yes [X] No [ ] |
| B28a | Device | P10.2.7 | B28:m | [ ] | Yes [X] |
| B28b | Speaker mute on | P10.2.7 | B28:m | [ ] | Yes [X] |

### 3.4.8	Speaker volume

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B29 | Speaker volume event | S11.2.4.8 P10.2.8 | B1b:o |  | Yes [X] No [ ] |
| B29a | Device | P10.2.8 | B29:m | [ ] | Yes [X] |
| B29b | Speaker volume | P10.2.8 | B29:m | [ ] | Yes [X] |

## 3.5	Maintenance events

### 3.5.1	Back in service

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B30 | Back in service event | S11.2.5.1 P10.4.1 | B1b:o |  | Yes [ ] No [X] |
| B30a | Device | P10.4.1 | B30:m | [ ] | Yes [ ] |
| B30b | Cause | P10.4.1 | B30:o |  | Yes [ ] No [ ] |

### 3.5.2	Out of service

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B31 | Out of service event | S10.2.5.2 P10.4.2 | B1b:o |  | Yes [ ] No [X] |
| B31a | Device | P10.4.2 | B31:m | [ ] | Yes [ ] |
| B31b | Cause | P10.4.2 | B31:o |  | Yes [ ] No [ ] |

## 3.6	Private events

### 3.6.1	Private

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B32 | Is/are private event(s) | S11.2.6 P10.5 | B1b:o |  | Yes [ ] No [X] |

## 3.7	Voice unit events

### 3.7.1	Play

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B33 | Play event | S11.2.7.1 P10.6.1 | B1b:o |  | Yes [ ] No [X] |
| B33a | Connection | P10.6.1 | B33:m | [ ] | Yes [ ] |
| B33b | Message | P10.6.1 | B33:m | [ ] | Yes [ ] |
| B33c | Message length | P10.6.1 | B33:o |  | Yes [ ] No [ ] |
| B33d | Current position in message | P10.6.1 | B33:o |  | Yes [ ] No [ ] |
| B33e | Speed | P10.6.1 | B33:o |  | Yes [ ] No [ ] |
| B33f | Cause | P10.6.1 | B33:o |  | Yes [ ] No [ ] |

### 3.7.2	Record

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B34 | Record event | S11.2.7.2 P10.6.2 | B1b:o |  | Yes [ ] No [X] |
| B34a | Connection | P10.6.2 | B34:m | [ ] | Yes [ ] |
| B34b | Message | P10.6.2 | B34:m | [ ] | Yes [ ] |
| B34c | Message length | P10.6.2 | B34:o |  | Yes [ ] No [ ] |
| B34d | Current position in message | P10.6.2 | B34:o |  | Yes [ ] No [ ] |
| B34e | Cause | P10.6.2 | B34:o |  | Yes [ ] No [ ] |

### 3.7.3	Review

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B35 | Review event | S11.2.7.3 P10.6.3 | B1b:o |  | Yes [ ] No [X] |
| B35a | Connection | P10.6.3 | B35:m | [ ] | Yes [ ] |
| B35b | Message | P10.6.3 | B35:m | [ ] | Yes [ ] |
| B35c | Message length | P10.6.3 | B35:o |  | Yes [ ] No [ ] |
| B35d | Current position in message | P10.6.3 | B35:o |  | Yes [ ] No [ ] |
| B35e | Cause | P10.6.3 | B35:o |  | Yes [ ] No [ ] |

### 3.7.4	Stop

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B36 | Stop event | S11.2.7.4 P10.6.4 | B1b:o |  | Yes [ ] No [X] |
| B36a | Connection | P10.6.4 | B36:m | [ ] | Yes [ ] |
| B36b | Message | P10.6.4 | B36:m | [ ] | Yes [ ] |
| B36c | Message length | P10.6.4 | B36:o |  | Yes [ ] No [ ] |
| B36d | Current position in message | P10.6.4 | B36:o |  | Yes [ ] No [ ] |
| B36e | Cause | P10.6.4 | B36:o |  | Yes [ ] No [ ] |

### 3.7.5	Suspend play

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B37 | Suspend play event | S11.2.7.5 P10.6.5 | B1b:o |  | Yes [ ] No [X] |
| B37a | Connection | P10.6.5 | B37:m | [ ] | Yes [ ] |
| B37b | Message | P10.6.5 | B37:m | [ ] | Yes [ ] |
| B37c | Message length | P10.6.5 | B37:o |  | Yes [ ] No [ ] |
| B37d | Current position in message | P10.6.5 | B37:o |  | Yes [ ] No [ ] |
| B37e | Cause | P10.6.5 | B37:o |  | Yes [ ] No [ ] |

### 3.7.6	Suspend record

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B38 | Suspend record event | S11.2.7.6 P10.6.6 | B1b:o |  | Yes [ ] No [X] |
| B38a | Connection | P10.6.6 | B38:m | [ ] | Yes [ ] |
| B38b | Message | P10.6.6 | B38:m | [ ] | Yes [ ] |
| B38c | Message length | P10.6.6 | B38:o |  | Yes [ ] No [ ] |
| B38d | Current position in message | P10.6.6 | B38:o |  | Yes [ ] No [ ] |
| B38e | Cause | P10.6.6 | B38:o |  | Yes [ ] No [ ] |

### 3.7.7	Voice attribute change

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B39 | Voice attribute change event | S11.2.7.7 P10.6.7 | B1b:o |  | Yes [ ] No [X] |
| B39a | Connection | P10.6.7 | B39:m | [ ] | Yes [ ] |
| B39b | Message | P10.6.7 | B39:m | [ ] | Yes [ ] |
| B39c | Speaker volume | P10.6.7 | B39:o |  | Yes [ ] No [ ] |
| B39d | Recording level | P10.6.7 | B39:o |  | Yes [ ] No [ ] |
| B39e | Speed | P10.6.7 | B39:o |  | Yes [ ] No [ ] |
| B39f | Current position in message | P10.6.7 | B39:o |  | Yes [ ] No [ ] |
| B39g | Cause | P10.6.7 | B39:o |  | Yes [ ] No [ ] |

# 4	Computing function services

## 4.1	Route request

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| C1 | Route request service | S12.3 P11.1 | o |  | Yes [ ] No [X] |
| C1a | Cross reference | P11.1 | C1:m | [ ] | Yes [ ] |
| C1b | Current route | P11.1 | C1:m | [ ] | Yes [ ] |
| C1c | Calling device | P11.1 | C1:o |  | Yes [ ] No [ ] |
| C1d | Routing device | P11.1 | C1:o |  | Yes [ ] No [ ] |
| C1e | Routed call | P11.1 | C1:o |  | Yes [ ] No [ ] |
| C1f | Route select algorithm | P11.1 | C1:o |  | Yes [ ] No [ ] |
| C1g | Priority | P11.1 | C1:o |  | Yes [ ] No [ ] |
| C1h | Device profile | P11.1 | C1:o |  | Yes [ ] No [ ] |
| C1i | Device class | P18.6 | C1h:m | [ ] | Yes [ ] |
| C1j | ISDN set up | P18.6 | C1h:o |  | Yes [ ] No [ ] |
| C1k | Correlator data | P11.1 | C1:o |  | Yes [ ] No [ ] |
| C1l | Private data in request | P18.8 | C1:o |  | Yes [ ] No [ ] |
| C1m | Security data in request | P18.7 | C1:o |  | Yes [ ] No [ ] |
| C1n | Service result negative | P16 | C1:m | [ ] | Yes [ ] |

## 4.2	Reroute request

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| C2 | Reroute request service | S12.1 P11.2 | o |  | Yes [ ] No [X] |
| C2a | Cross reference | P11.2 | C2:m | [ ] | Yes [ ] |
| C2b | Private data in request | P18.8 | C2:o |  | Yes [ ] No [ ] |
| C2c | Security data in request | P18.7 | C2:o |  | Yes [ ] No [ ] |
| C2d | Service result negative | P16 | C2:m | [ ] | Yes [ ] |

## 4.3	Route select request

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| C3 | Route select request service | S12.4 P11.3 | o |  | Yes [ ] No [X] |
| C3a | Cross reference | P11.3 | C3:m | [ ] | Yes [ ] |
| C3b | Route selected | P11.3 | C3:m | [ ] | Yes [ ] |
| C3c | Remaining retries | P11.3 | C3:o |  | Yes [ ] No [ ] |
| C3d | Device profile | P11.3 | C3:o |  | Yes [ ] No [ ] |
| C3e | Device class | P18.6 | C3d:m | [ ] | Yes [ ] |
| C3f | ISDN set up | P18.6 | C3d:o |  | Yes [ ] No [ ] |
| C3g | Route used request positive | P11.3 | C3:o |  | Yes [ ] No [ ] |
| C3h | Correlator data | P11.3 | C3:o |  | Yes [ ] No [ ] |
| C3i | Private data in request | P18.8 | C3:o |  | Yes [ ] No [ ] |
| C3j | Security data in request | P18.7 | C3:o |  | Yes [ ] No [ ] |
| C3k | Service result negative | P16 | C3:m | [ ] | Yes [ ] |

## 4.4	Route used request

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| C4 | Route used request service | S12.5 P11.4 | o |  | Yes [ ] No [X] |
| C4a | Cross reference | P11.4 | C4:m | [ ] | Yes [ ] |
| C4b | Route used | P11.4 | C4:m | [ ] | Yes [ ] |
| C4c | Calling device | P11.4 | C4:o |  | Yes [ ] No [ ] |
| C4d | Domain | P11.4 | C4:o |  | Yes [ ] No [ ] |
| C4e | Correlator data | P11.4 | C4:o |  | Yes [ ] No [ ] |
| C4f | Private data in request | P18.8 | C4:o |  | Yes [ ] No [ ] |
| C4g | Security data in request | P18.7 | C4:o |  | Yes [ ] No [ ] |
| C4h | Service result negative | P16 | C4:m | [ ] | Yes [ ] |

## 4.5	Route end request

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| C5 | Route end request service | S12.2 P11.5 | o |  | Yes [ ] No [X] |
| C5a | Cross reference | P11.5 | C5:m | [ ] | Yes [ ] |
| C5b | Error value | P11.5 | C5:o |  | Yes [ ] No [ ] |
| C5c | Private data in request | P18.8 | C5:o |  | Yes [ ] No [ ] |
| C5d | Security data in request | P18.7 | C5:o |  | Yes [ ] No [ ] |
| C5e | Service result negative | P16 | C5:o |  | Yes [ ] No [ ] |

# 5	Bidirectional services

## 5.1	Escape

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| D1 | Escape service | S13.1 P12.1 | o |  | Yes [ ] No [X] |
| D1a | Security data in request | P18.7 | D1:o |  | Yes [ ] No [ ] |
| D1b | Private data in request | P18.8 | D1:m | [ ] | Yes [ ] |
| D1c | Service result positive | P12.1 | D1:m | [ ] | Yes [ ] |
| D1d | Service result negative | P16 | D1:m | [ ] | Yes [ ] |
| D1e | Security data in result | P18.7 | D1c:o |  | Yes [ ] No [ ] |
| D1f | Private data in result | P18.8 | D1c:o |  | Yes [ ] No [ ] |

## 5.2	System status

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| D2 | System status service | S13.2 P12.2 | o |  | Yes [X] No [ ] |
| D2a | System status cause | P12.2 | D2:m | [X] | Yes [ ] |
| D2b | Initializing | P18.6 | D2a:o |  | Yes [ ] No [X] |
| D2c | Enabled | P18.6 | D2a:o |  | Yes [ ] No [X] |
| D2d | Normal | P18.6 | D2a:o |  | Yes [X] No [ ] |
| D2e | Messages lost | P18.6 | D2a:o |  | Yes [ ] No [X] |
| D2f | Disabled | P18.6 | D2a:o |  | Yes [ ] No [X] |
| D2g | Overload imminent | P18.6 | D2a:o |  | Yes [ ] No [X] |
| D2h | Overload reached | P18.6 | D2a:o |  | Yes [X] No [ ] |
| D2i | Overload relieved | P18.6 | D2a:o |  | Yes [ ] No [X] |
| D2j | Security data in request | P18.7 | D2:o |  | Yes [ ] No [X] |
| D2k | Private data in request | P18.8 | D2:o |  | Yes [ ] No [X] |
| D2l | Service result positive | P12.2 | D2:m | [ ] | Yes [X] |
| D2m | Service result negative | P16 | D2:m | [ ] | Yes [X] |
| D2n | Security data in result | P18.7 | D2l:o |  | Yes [ ] No [X] |
| D2o | Private data in result | P18.8 | D2l:o |  | Yes [ ] No [X] |

# 6	Status reporting services

## 6.1	Change monitor filter

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| E1 | Change monitor filter service | S11.1 P13.2 | o |  | Yes [ ] No [X] |
| E1a | Cross reference ID in request | P13.2 | E1:m | [ ] | Yes [ ] |
| E1b | Filter list in request | P13.2 | E1:m | [ ] | Yes [ ] |
| E1c | Security data in request | P18.7 | E1:o |  | Yes [ ] No [ ] |
| E1d | Private data in request | P18.8 | E1:o |  | Yes [ ] No [ ] |
| E1e | Service result positive | P13.2 | E1:m | [ ] | Yes [ ] |
| E1f | Service result negative | P16 | E1:m | [ ] | Yes [ ] |
| E1g | Filter list in result | P13.2 | E1e:o |  | Yes [ ] No [ ] |
| E1h | Security data in result | P18.7 | E1e:o |  | Yes [ ] No [ ] |
| E1I | Private data in result | P18.8 | E1e:o |  | Yes [ ] No [ ] |

## 6.2	Monitor start

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| E2 | Monitor start service | S11.3 P13.1 | o |  | Yes [X] No [ ] |
| E2a | Monitor object | P13.1 | E2:m | [ ] | Yes [X] |
| E2b | Monitor object device | P18.5 | E2a:o1 |  | Yes [X] No [ ] |
| E2c | Monitor object call | P18.5 | E2a:o1 |  | Yes [ ] No [X] |
| E2d | Monitor type device | P18.5 | E2a:o |  | Yes [X] No [ ] |
| E2e | Monitor type call | P18.5 | E2a:o |  | Yes [ ] No [X] |
| E2f | Monitor filter for call processing events | P18.5 | E2:o |  | Yes [X] No [ ] |
| E2g | Monitor filter for feature events | P18.5 | E2:o |  | Yes [X] No [ ] |
| E2h | Monitor filter for agent events | P18.5 | E2:o |  | Yes [X] No [ ] |
| E2i | Monitor filter for maintenance events | P18.5 | E2:o |  | Yes [ ] No [X] |
| E2j | Monitor filter for voice unit events | P18.5 | E2:o |  | Yes [ ] No [X] |
| E2k | Monitor filter for private filter event | P18.5 | E2:o |  | Yes [ ] No [X] |
| E2l | Service result positive | P13.1 | E2:m | [ ] | Yes [X] |
| E2m | Service result negative | P16 | E2:m | [ ] | Yes [X] |
| E2n | Cross reference ID in result | P13.1 | E2l:m | [ ] | Yes [X] |
| E2o | Monitor filter in result | P13.1 | E2l:o |  | Yes [X] No [ ] |
| E2p | Security data in request | P18.7 | E2:o |  | Yes [ ] No [X] |
| E2q | Private data in request | P18.8 | E2:o |  | Yes [ ] No [X] |
| E2r | Security data in result | P18.7 | E2l:o |  | Yes [ ] No [X] |
| E2s | Private data in result | P18.8 | E2l:o |  | Yes [ ] No [X] |

## 6.3	Monitor stop

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| E3 | Monitor stop service | S11.4 P13.3 | o |  | Yes [X] No [ ] |
| E3a | Cross reference ID in request | P13.3 | E3:m | [ ] | Yes [X] |
| E3b | Security data in request | P18.7 | E3:o |  | Yes [ ] No [X] |
| E3c | Private data in request | P18.8 | E3:o |  | Yes [ ] No [X] |
| E3d | Service result positive | P13.3 | E3:m | [ ] | Yes [X] |
| E3e | Service result negative | P16 | E3:m | [ ] | Yes [X] |
| E3f | Security data in result | P18.7 | E3d:o |  | Yes [ ] No [X] |
| E3g | Private data in result | P18.8 | E3d:o |  | Yes [ ] No [X] |

## 6.4	Snapshot call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| E4 | Snapshot call service | S11.5 P13.5 | o |  | Yes [ ] No [X] |
| E4a | Snapshot object in request | P13.5 | E4:m | [ ] | Yes [ ] |
| E4b | Security data in request | P18.7 | E4:o |  | Yes [ ] No [ ] |
| E4c | Private data in request | P18.8 | E4:o |  | Yes [ ] No [ ] |
| E4d | Service result positive | P13.5 | E4:m | [ ] | Yes [ ] |
| E4e | Service result negative | P16 | E4:m | [ ] | Yes [ ] |
| E4f | Device ID in result | P13.5 | E4d:m | [ ] | Yes [ ] |
| E4g | Connection ID in result | P13.5 | E4d:m | [ ] | Yes [ ] |
| E4h | Device profile | P13.5 | E4d:o |  | Yes [ ] No [ ] |
| E4i | Device class | P18.6 | E4h:m | [ ] | Yes [ ] |
| E4j | ISDN set up | P18.6 | E4h:o |  | Yes [ ] No [ ] |
| E4k | Local connection state in result | P13.5 | E4d:o |  | Yes [ ] No [ ] |
| E4l | Security data in result | P18.7 | E4d:o |  | Yes [ ] No [ ] |
| E4m | Private data in result | P18.8 | E4d:o |  | Yes [ ] No [ ] |

## 6.5	Snapshot device

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| E5 | Snapshot device service | S11.6 P13.4 | o |  | Yes [ ] No [X] |
| E5a | Device ID in request | P13.4 | E5:m | [ ] | Yes [ ] |
| E5b | Security data in request | P18.7 | E5:o |  | Yes [ ] No [ ] |
| E5c | Private data in request | P18.8 | E5:o |  | Yes [ ] No [ ] |
| E5d | Service result positive | P13.4 | E5:m | [ ] | Yes [ ] |
| E5e | Service result negative | P16 | E5:m | [ ] | Yes [ ] |
| E5f | Connection ID in result | P13.4 | E5d:m | [ ] | Yes [ ] |
| E5g | Call state in result | P13.4 | E5d:m | [ ] | Yes [ ] |
| E5h | Device profile | P13.4 | E5d:o |  | Yes [ ] No [ ] |
| E5i | Device class | P18.6 | E5h:m | [ ] | Yes [ ] |
| E5j | ISDN set up | P18.6 | E5h:o |  | Yes [ ] No [ ] |
| E5k | Security data in result | P18.7 | E5d:o |  | Yes [ ] No [ ] |
| E5l | Private data in result | P18.8 | E5d:o |  | Yes [ ] No [ ] |

# 7	Input/output services

## 7.1	Data path resumed

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| F1 | Data path resumed service | S14.1 P14.9 | o |  | Yes [ ] No [X] |
| F1a | Cross reference ID | P14.9 | F1:m | [ ] | Yes [ ] |
| F1b | Security data in request | P18.7 | F1:o |  | Yes [ ] No [ ] |
| F1c | Private data in request | P18.8 | F1:o |  | Yes [ ] No [ ] |
| F1d | Service result positive | P14.9 | F1:m | [ ] | Yes [ ] |
| F1e | Service result negative | P16 | F1:m | [ ] | Yes [ ] |
| F1f | Security data in result | P18.7 | F1d:o |  | Yes [ ] No [ ] |
| F1g | Private data in result | P18.8 | F1d:o |  | Yes [ ] No [ ] |

## 7.2	Data path suspended

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| F2 | Data path suspended service | S14.2 P14.7 | o |  | Yes [ ] No [X] |
| F2a | Cross reference ID | P14.7 | F2:m | [ ] | Yes [ ] |
| F2b | Security data in request | P18.7 | F2:o |  | Yes [ ] No [ ] |
| F2c | Private data in request | P18.8 | F2:o |  | Yes [ ] No [ ] |
| F2d | Service result positive | P14.7 | F2:m | [ ] | Yes [ ] |
| F2e | Service result negative | P16 | F2:m | [ ] | Yes [ ] |
| F2f | Security data in result | P18.7 | F2d:o |  | Yes [ ] No [ ] |
| F2g | Private data in result | P18.8 | F2d:o |  | Yes [ ] No [ ] |

## 7.3	Fast data

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| F3 | Fast data service | S14.3 P14.10 | o |  | Yes [ ] No [X] |
| F3a | CSTA object | P14.10 | F3:m | [ ] | Yes [ ] |
| F3b | Object device | P14.10 | F3a:o1 |  | Yes [ ] No [ ] |
| F3c | Object call | P14.10 | F3a:o1 |  | Yes [ ] No [ ] |
| F3d | I/O data | P14.10 | F3:m | [ ] | Yes [ ] |
| F3e | Data path type text | P14.10 | F3:o |  | Yes [ ] No [ ] |
| F3f | Data path type voice | P14.10 | F3:o |  | Yes [ ] No [ ] |
| F3g | Security data in request | P18.7 | F3:o |  | Yes [ ] No [ ] |
| F3h | Private data in request | P18.8 | F3:o |  | Yes [ ] No [ ] |
| F3i | Service result positive | P14.10 | F3:m | [ ] | Yes [ ] |
| F3j | Service result negative | P16 | F3:m | [ ] | Yes [ ] |
| F3k | Security data in result | P18.7 | F3i:o |  | Yes [ ] No [ ] |
| F3l | Private data in result | P18.8 | F3i:o |  | Yes [ ] No [ ] |

## 7.4	Resume data path

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| F4 | Resume data path service | S14.4 P14.8 | o |  | Yes [ ] No [X] |
| F4a | Cross reference ID | P14.8 | F4:m | [ ] | Yes [ ] |
| F4b | Security data in request | P18.7 | F4:o |  | Yes [ ] No [ ] |
| F4c | Private data in request | P18.8 | F4:o |  | Yes [ ] No [ ] |
| F4d | Service result positive | P14.8 | F4:m | [ ] | Yes [ ] |
| F4e | Service result negative | P16 | F4:m | [ ] | Yes [ ] |
| F4f | Security data in result | P18.7 | F4d:o |  | Yes [ ] No [ ] |
| F4g | Private data in result | P18.8 | F4d:o |  | Yes [ ] No [ ] |

## 7.5	Send broadcast data

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| F5 | Send broadcast data service | S14.5 P14.5 | o |  | Yes [ ] No [X] |
| F5a | I/O data | P14.5 | F5:m | [ ] | Yes [ ] |
| F5b | Data path type text | P14.5 | F5:o |  | Yes [ ] No [ ] |
| F5c | Data path type voice | P14.5 | F5:o |  | Yes [ ] No [ ] |
| F5d | Security data in request | P18.7 | F5:o |  | Yes [ ] No [ ] |
| F5e | Private data in request | P18.8 | F5:o |  | Yes [ ] No [ ] |
| F5f | Service result positive | P14.5 | F5:m | [ ] | Yes [ ] |
| F5g | Service result negative | P16 | F5:m | [ ] | Yes [ ] |
| F5h | Security data in result | P18.7 | F5f:o |  | Yes [ ] No [ ] |
| F5i | Private data in result | P18.8 | F5f:o |  | Yes [ ] No [ ] |

## 7.6	Send data

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| F6 | Send data service | S14.6 P14.3 | o |  | Yes [ ] No [X] |
| F6a | Cross reference ID | P14.3 | F6:m | [ ] | Yes [ ] |
| F6b | I/O data | P14.3 | F6:m | [ ] | Yes [ ] |
| F6c | Cause code | S10.4 P16 | F6:o |  | Yes [ ] No [ ] |
| F6d | Security data in request | P18.7 | F6:o |  | Yes [ ] No [ ] |
| F6e | Private data in request | P18.8 | F6:o |  | Yes [ ] No [ ] |
| F6f | Service result positive | P14.3 | F6:m | [ ] | Yes [ ] |
| F6g | Service result negative | P16 | F6:m | [ ] | Yes [ ] |
| F6h | Security data in result | P18.7 | F6f:o |  | Yes [ ] No [ ] |
| F6i | Private data in result | P18.8 | F6f:o |  | Yes [ ] No [ ] |

## 7.7	Send multicast data

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| F7 | Send multicast data service | S14.7 P14.4 | o |  | Yes [ ] No [X] |
| F7a | I/O cross reference ID list | P14.4 | F7:m | [ ] | Yes [ ] |
| F7b | I/O data | P14.4 | F7:m | [ ] | Yes [ ] |
| F7c | Security data in request | P18.7 | F7:o |  | Yes [ ] No [ ] |
| F7d | Private data in request | P18.8 | F7:o |  | Yes [ ] No [ ] |
| F7e | Service result positive | P14.4 | F7:m | [ ] | Yes [ ] |
| F7f | Service result negative | P16 | F7:m | [ ] | Yes [ ] |
| F7g | Security data in result | P18.7 | F7e:o |  | Yes [ ] No [ ] |
| F7h | Private data in result | P18.8 | F7e:o |  | Yes [ ] No [ ] |

## 7.8	Start data path

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| F8 | Start data path service | S14.8 P14.1 | o |  | Yes [ ] No [X] |
| F8a | CSTA object | P14.1 | F8:m | [ ] | Yes [ ] |
| F8b | Object device | P14.1 | F8:o1 |  | Yes [ ] No [ ] |
| F8c | Object call | P14.1 | F8:o1 |  | Yes [ ] No [ ] |
| F8d | Data path from the computing function | P18.6 | F8:o |  | Yes [ ] No [ ] |
| F8e | Data path to the computing function | P18.6 | F8:o |  | Yes [ ] No [ ] |
| F8f | Data path bidirectional | P18.6 | F8:o |  | Yes [ ] No [ ] |
| F8g | Data path type text | P14.1 | F8:o |  | Yes [ ] No [ ] |
| F8h | Data path type voice | P14.1 | F8:o |  | Yes [ ] No [ ] |
| F8i | Number of characters to collect | P14.1 | F8:o |  | Yes [ ] No [ ] |
| F8j | Termination character | P14.1 | F8:o |  | Yes [ ] No [ ] |
| F8k | Timeout | P14.1 | F8:o |  | Yes [ ] No [ ] |
| F8l | Cross reference ID in result | P14.1 | F8:m | [ ] | Yes [ ] |
| F8m | Number of characters to collect in result | P14.1 | F8:o |  | Yes [ ] No [ ] |
| F8n | Termination character in result | P14.1 | F8:o |  | Yes [ ] No [ ] |
| F8o | Timeout in result | P14.1 | F8:o |  | Yes [ ] No [ ] |
| F8p | Security data in request | P18.7 | F8:o |  | Yes [ ] No [ ] |
| F8q | Private data in request | P18.8 | F8:o |  | Yes [ ] No [ ] |
| F8r | Service result positive | P14.1 | F8:m | [ ] | Yes [ ] |
| F8s | Service result negative | P16 | F8:m | [ ] | Yes [ ] |
| F8t | Security data in result | P18.7 | F8r:o |  | Yes [ ] No [ ] |
| F8u | Private data in result | P18.8 | F8r:o |  | Yes [ ] No [ ] |

## 7.9	Stop data path

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| F9 | Stop data path service | S14.9 P14.2 | o |  | Yes [ ] No [X] |
| F9a | CSTA cross reference ID | P14.2 | F9:m | [ ] | Yes [ ] |
| F9b | Security data in request | P18.7 | F9:o |  | Yes [ ] No [ ] |
| F9c | Private data in request | P18.8 | F9:o |  | Yes [ ] No [ ] |
| F9d | Service result positive | P14.2 | F9:m | [ ] | Yes [ ] |
| F9e | Service result negative | P16 | F9:m | [ ] | Yes [ ] |
| F9f | Security data in result | P18.7 | F9d:o |  | Yes [ ] No [ ] |
| F9g | Private data in result | P18.8 | F9d:o |  | Yes [ ] No [ ] |

## 7.10	Suspend data path

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| F10 | Suspend data path service | S14.10 P14.6 | o |  | Yes [ ] No [X] |
| F10a | CSTA cross reference ID | P14.6 | F10:m | [ ] | Yes [ ] |
| F10b | Security data in request | P18.7 | F10:o |  | Yes [ ] No [ ] |
| F10c | Private data in request | P18.8 | F10:o |  | Yes [ ] No [ ] |
| F10d | Service result positive | P14.6 | F10:m | [ ] | Yes [ ] |
| F10e | Service result negative | P16 | F10:m | [ ] | Yes [ ] |
| F10f | Security data in result | P18.7 | F10d:o |  | Yes [ ] No [ ] |
| F10g | Private data in result | P18.8 | F10d:o |  | Yes [ ] No [ ] |

# 8	Voice unit services

## 8.1	Concatenate message

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G1 | Concatenate message service | S15.1 P15.1 | o |  | Yes [ ] No [X] |
| G1a | Message ID list | P15.1 | G1:m | [ ] | Yes [ ] |
| G1b | Security data in request | P18.7 | G1:o |  | Yes [ ] No [ ] |
| G1c | Private data in request | P18.8 | G1:o |  | Yes [ ] No [ ] |
| G1d | Service result positive | P15.1 | G1:m | [ ] | Yes [ ] |
| G1e | Service result negative | P16 | G1:m | [ ] | Yes [ ] |
| G1f | Security data in result | P18.7 | G1d:o |  | Yes [ ] No [ ] |
| G1g | Private data in result | P18.8 | G1d:o |  | Yes [ ] No [ ] |

## 8.2	Delete message

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G2 | Delete message service | S15.2 P15.2 | o |  | Yes [ ] No [X] |
| G2a | Message ID | P15.2 | G2:m | [ ] | Yes [ ] |
| G2b | Security data in request | P18.7 | G2:o |  | Yes [ ] No [ ] |
| G2c | Private data in request | P18.8 | G2:o |  | Yes [ ] No [ ] |
| G2d | Service result positive | P15.2 | G2:m | [ ] | Yes [ ] |
| G2e | Service result negative | P16 | G2:m | [ ] | Yes [ ] |
| G2f | Security data in result | P18.7 | G2d:o |  | Yes [ ] No [ ] |
| G2g | Private data in result | P18.8 | G2d:o |  | Yes [ ] No [ ] |

## 8.3	Play message

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G3 | Play message service | S15.3 P15.3 | o |  | Yes [ ] No [X] |
| G3a | Message ID | P15.3 | G3:m | [ ] | Yes [ ] |
| G3b | Call associated with message | P15.3 | G3:m | [ ] | Yes [ ] |
| G3c | Duration | P15.3 | G3:o |  | Yes [ ] No [ ] |
| G3d | Duration exceeded | P15.3 | G3:o |  | Yes [ ] No [ ] |
| G3e | DTMF detected | P15.3 | G3:o |  | Yes [ ] No [ ] |
| G3f | End of data detected | P15.3 | G3:o |  | Yes [ ] No [ ] |
| G3g | Speech detected | P15.3 | G3:o |  | Yes [ ] No [ ] |
| G3h | Security data in request | P18.7 | G3:o |  | Yes [ ] No [ ] |
| G3i | Private data in request | P18.8 | G3:o |  | Yes [ ] No [ ] |
| G3j | Service result positive | P15.3 | G3:m | [ ] | Yes [ ] |
| G3k | Service result negative | P16 | G3:m | [ ] | Yes [ ] |
| G3l | Security data in result | P18.7 | G3j:o |  | Yes [ ] No [ ] |
| G3m | Private data in result | P18.8 | G3j:o |  | Yes [ ] No [ ] |

## 8.4	Query voice attribute

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G4 | Query voice attribute service | S15.4 P15.4 | o |  | Yes [ ] No [X] |
| G4a | Message ID | P15.4 | G4:m | [ ] | Yes [ ] |
| G4b | Connection associated with message | P15.4 | G4:o |  | Yes [ ] No [ ] |
| G4c | Query attribute | P15.4 | G4:m | [ ] | Yes [ ] |
| G4d | Attribute encode algorithm | P15.4 | G4:o1 |  | Yes [ ] No [ ] |
| G4e | Attribute sampling rate | P15.4 | G4:o1 |  | Yes [ ] No [ ] |
| G4f | Attribute duration | P15.4 | G4:o1 |  | Yes [ ] No [ ] |
| G4g | Attribute file name | P15.4 | G4:o1 |  | Yes [ ] No [ ] |
| G4h | Attribute current position | P15.4 | G4:o1 |  | Yes [ ] No [ ] |
| G4i | Attribute current speed | P15.4 | G4:o1 |  | Yes [ ] No [ ] |
| G4j | Attribute current volume | P15.4 | G4:o1 |  | Yes [ ] No [ ] |
| G4k | Attribute current level | P15.4 | G4:o1 |  | Yes [ ] No [ ] |
| G4l | Attribute current state | P15.4 | G4:o1 |  | Yes [ ] No [ ] |
| G4m | Attribute in result | P15.4 | G4ag:m | [ ] | Yes [ ] |
| G4n | ADPCM-6k | P15.4 | G4d:o2 |  | Yes [ ] No [ ] |
| G4o | ADPCM-8k | P15.4 | G4d:o2 |  | Yes [ ] No [ ] |
| G4p | Mu-Law PCM 6k | P15.4 | G4d:o2 |  | Yes [ ] No [ ] |
| G4q | A-Law PCM 6k | P15.4 | G4d:o2 |  | Yes [ ] No [ ] |
| G4r | Sampling rate in result | P15.4 | G4e:m | [ ] | Yes [ ] |
| G4s | Duration in result | P15.4 | G4f:m | [ ] | Yes [ ] |
| G4t | File name in result | P15.4 | G4g:m | [ ] | Yes [ ] |
| G4u | Current position in result | P15.4 | G4h:m | [ ] | Yes [ ] |
| G4v | Current speed in result | P15.4 | G4i:m | [ ] | Yes [ ] |
| G4w | Current volume in result | P15.4 | G4j:m | [ ] | Yes [ ] |
| G4x | Current level in result | P15.4 | G4k:m | [ ] | Yes [ ] |
| G4y | Stop | P15.4 | G4l:o3 |  | Yes [ ] No [ ] |
| G4z | Play | P15.4 | G4l:o3 |  | Yes [ ] No [ ] |
| G4aa | Record | P15.4 | G4l:o3 |  | Yes [ ] No [ ] |
| G4ab | Suspend play | P15.4 | G4l:o3 |  | Yes [ ] No [ ] |
| G4ac | Suspend record | P15.4 | G4l:o3 |  | Yes [ ] No [ ] |
| G4ad | Review | P15.4 | G4l:o3 |  | Yes [ ] No [ ] |
| G4ae | Security data in request | P18.7 | G4:o |  | Yes [ ] No [ ] |
| G4af | Private data in request | P18.8 | G4:o |  | Yes [ ] No [ ] |
| G4ag | Service result positive | P15.4 | G4:m | [ ] | Yes [ ] |
| G4ah | Service result negative | P16 | G4:m | [ ] | Yes [ ] |
| G4ai | Security data in result | P18.7 | G4ag:o |  | Yes [ ] No [ ] |
| G4aj | Private data in result | P18.8 | G4ag:o |  | Yes [ ] No [ ] |

## 8.5	Record message

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G5 | Record message service | S15.5 P15.5 | o |  | Yes [ ] No [X] |
| G5a | Connection associated with record | P15.5 | G5:m | [ ] | Yes [ ] |
| G5b | Sampling rate | P15.5 | G5:o |  | Yes [ ] No [ ] |
| G5c | ADPCM-6k | P15.5 | G5:o1 |  | Yes [ ] No [ ] |
| G5d | ADPCM-8k | P15.5 | G5:o1 |  | Yes [ ] No [ ] |
| G5e | Mu-Law PCM 6k | P15.5 | G5:o1 |  | Yes [ ] No [ ] |
| G5f | A-Law PCM 6k | P15.5 | G5:o1 |  | Yes [ ] No [ ] |
| G5g | Maximum duration | P15.5 | G5:o |  | Yes [ ] No [ ] |
| G5h | DTMF detected | P15.5 | G5:o |  | Yes [ ] No [ ] |
| G5i | No speech detected | P15.5 | G5:o |  | Yes [ ] No [ ] |
| G5j | Security data in request | P18.7 | G5:o |  | Yes [ ] No [ ] |
| G5k | Private data in request | P18.8 | G5:o |  | Yes [ ] No [ ] |
| G5l | Service result positive | P15.5 | G5:m | [ ] | Yes [ ] |
| G5m | Service result negative | P16 | G5:m | [ ] | Yes [ ] |
| G5n | Security data in result | P18.7 | G5l:o |  | Yes [ ] No [ ] |
| G5o | Private data in result | P18.8 | G5l:o |  | Yes [ ] No [ ] |

## 8.6	Reposition

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G6 | Reposition service | S15.6 P15.6 | o |  | Yes [ ] No [X] |
| G6a | Connection associated with message | P15.6 | G6:m | [ ] | Yes [ ] |
| G6b | Period | P15.6 | G6:m | [ ] | Yes [ ] |
| G6c | Message ID | P15.6 | G6:o |  | Yes [ ] No [ ] |
| G6d | Security data in request | P18.7 | G6:o |  | Yes [ ] No [ ] |
| G6e | Private data in request | P18.8 | G6:o |  | Yes [ ] No [ ] |
| G6f | Service result positive | P15.6 | G6:m | [ ] | Yes [ ] |
| G6g | Service result negative | P16 | G6:m | [ ] | Yes [ ] |
| G6h | Security data in result | P18.7 | G6f:o |  | Yes [ ] No [ ] |
| G6i | Private data in result | P18.8 | G6f:o |  | Yes [ ] No [ ] |

## 8.7	Resume

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G7 | Resume service | S15.7 P15.7 | o |  | Yes [ ] No [X] |
| G7a | Connection associated with message | P15.7 | G7:m | [ ] | Yes [ ] |
| G7b | Message ID | P15.7 | G7:o |  | Yes [ ] No [ ] |
| G7c | Duration | P15.7 | G7:o |  | Yes [ ] No [ ] |
| G7d | Security data in request | P18.7 | G7:o |  | Yes [ ] No [ ] |
| G7e | Private data in request | P18.8 | G7:o |  | Yes [ ] No [ ] |
| G7f | Service result positive | P15.7 | G7:m | [ ] | Yes [ ] |
| G7g | Service result negative | P16 | G7:m | [ ] | Yes [ ] |
| G7h | Security data in result | P18.7 | G7f:o |  | Yes [ ] No [ ] |
| G7i | Private data in result | P18.8 | G7f:o |  | Yes [ ] No [ ] |

## 8.8	Review

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G8 | Review service | S15.8 P15.8 | o |  | Yes [ ] No [X] |
| G8a | Connection associated with message | P15.8 | G8:m | [ ] | Yes [ ] |
| G8b | Period | P15.8 | G8:m | [ ] | Yes [ ] |
| G8c | Message ID | P15.8 | G8:o |  | Yes [ ] No [ ] |
| G8d | Security data in request | P18.7 | G8:o |  | Yes [ ] No [ ] |
| G8e | Private data in request | P18.8 | G8:o |  | Yes [ ] No [ ] |
| G8f | Service result positive | P15.8 | G8:m | [ ] | Yes [ ] |
| G8g | Service result negative | P16 | G8:m | [ ] | Yes [ ] |
| G8h | Security data in result | P18.7 | G8f:o |  | Yes [ ] No [ ] |
| G8i | Private data in result | P18.8 | G8f:o |  | Yes [ ] No [ ] |

## 8.9	Set voice attribute

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G9 | Set voice attribute service | S15.9 P15.9 | o |  | Yes [ ] No [X] |
| G9a | Connection associated with message | P15.9 | G9:m | [ ] | Yes [ ] |
| G9b | Speed | P15.9 | G9:o1 |  | Yes [ ] No [ ] |
| G9c | Volume | P15.9 | G9:o1 |  | Yes [ ] No [ ] |
| G9d | Level | P15.9 | G9:o1 |  | Yes [ ] No [ ] |
| G9e | Message ID | P15.9 | G9:o |  | Yes [ ] No [ ] |
| G9f | Security data in request | P18.7 | G9:o |  | Yes [ ] No [ ] |
| G9g | Private data in request | P18.8 | G9:o |  | Yes [ ] No [ ] |
| G9h | Service result positive | P15.9 | G9:m | [ ] | Yes [ ] |
| G9i | Service result negative | P16 | G9:m | [ ] | Yes [ ] |
| G9j | Security data in result | P18.7 | G9h:o |  | Yes [ ] No [ ] |
| G9k | Private data in result | P18.8 | G9h:o |  | Yes [ ] No [ ] |

## 8.10	Stop

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G10 | Stop service | S15.10 P15.10 | o |  | Yes [ ] No [X] |
| G10a | Connection associated with message | P15.10 | G10:m | [ ] | Yes [ ] |
| G10b | Message ID | P15.10 | G10:o |  | Yes [ ] No [ ] |
| G10c | Security data in request | P18.7 | G10:o |  | Yes [ ] No [ ] |
| G10d | Private data in request | P18.8 | G10:o |  | Yes [ ] No [ ] |
| G10e | Service result positive | P15.10 | G10:m | [ ] | Yes [ ] |
| G10f | Service result negative | P16 | G10:m | [ ] | Yes [ ] |
| G10g | Security data in result | P18.7 | G10e:o |  | Yes [ ] No [ ] |
| G10h | Private data in result | P18.8 | G10e:o |  | Yes [ ] No [ ] |

## 8.11	Suspend

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G11 | Suspend service | S15.11 P15.11 | o |  | Yes [ ] No [X] |
| G11a | Connection associated with message | P15.11 | G11:m | [ ] | Yes [ ] |
| G11b | Message ID | P15.11 | G11:o |  | Yes [ ] No [ ] |
| G11c | Security data in request | P18.7 | G11:o |  | Yes [ ] No [ ] |
| G11d | Private data in request | P18.8 | G11:o |  | Yes [ ] No [ ] |
| G11e | Service result positive | P15.11 | G11:m | [ ] | Yes [ ] |
| G11f | Service result negative | P16 | G11:m | [ ] | Yes [ ] |
| G11g | Security data in result | P18.7 | G11e:o |  | Yes [ ] No [ ] |
| G11h | Private data in result | P18.8 | G11e:o |  | Yes [ ] No [ ] |

## 8.12	Synthesize message

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G12 | Synthesize message service | S15.12 P15.12 | o |  | Yes [ ] No [X] |
| G12a | Text | P15.12 | G12:m | [ ] | Yes [ ] |
| G12b | Male | P15.12 | G12:o1 |  | Yes [ ] No [ ] |
| G12c | Female | P15.12 | G12:o1 |  | Yes [ ] No [ ] |
| G12d | Language | P15.12 | G12:o |  | Yes [ ] No [ ] |
| G12e | Message ID in result | P15.12 | G12:m | [ ] | Yes [ ] |
| G12f | Security data in request | P18.7 | G12:o |  | Yes [ ] No [ ] |
| G12g | Private data in request | P18.8 | G12:o |  | Yes [ ] No [ ] |
| G12h | Service result positive | P15.12 | G12:m | [ ] | Yes [ ] |
| G12i | Service result negative | P16 | G12:m | [ ] | Yes [ ] |
| G12j | Security data in result | P18.7 | G12h:o |  | Yes [ ] No [ ] |
| G12k | Private data in result | P18.8 | G12h:o |  | Yes [ ] No [ ] |

# Implementation Considerations

Physical link		TCP/IP via Ethernet (10BaseT)

connecting TCP/IP port 2555 on the switch

Session association	ISO remote operations Association Control Service Element

version 1 (ref : ISO 8649 & 8650)

ROSE Invoke IDs		Maximum length of 4 bytes

Monitored CrossRef IDs	2 bytes

Directory Numbers		Fields representing directory numbers have a maximum length

of 16

Call IDs			2 bytes

Encoding of a device type

| Device | Encoding |
| --- | --- |
| Pilot Group | NumberDigits |
| Internal Agent | NumberDigits |
| “other” device | NumberDigits |
| Dialled digits (external calling Party and external called party) | NumberDigits Explicit Public |
| Trunk ID | DeviceNumber |

TCP  Buffer			Maximum length of  4096 bytes  per  one packet

# CSTA Phase II parameter size constraints

| Parameter | Maxinum length (in bytes) | Value |
| --- | --- | --- |
| Call (part of ConnectionID) | 4 | “0,1,2,3,4,5,6,7,8,9,A,B,C,D,E,F” |
| MonitorCrossRefID | 2 | “0,1,2,3,4,5,6,7,8,9,A,B,C,D,E,F” |
| ApplicationCorrelator | 16 | “0,1,2,3,4,5,6,7,8,9,*,#” |
| PublicTON | 16 | “0,1,2,3,4,5,6,7,8,9,*,#” |
| NumberDigits | 16 | “0,1,2,3,4,5,6,7,8,9,*,#” |
| DeviceNumber | 4 | In case without CLI information, Trunk’s DeviceID will be selected by DeviceNumber. |
| AgentID | 6 | “0,1,2,3,4,5,6,7,8,9” |
| ROSE InvokeID | 4 | 1 ~ 65000 |
| Number In Queue | 2 |  |

# Supported Monitor Types

- Single Line Telephone Number

- LKD Telephone Number

- LGP Telephone Number

- ACD Split Queue (ACD Split Representation Number)