LG PBX System

Software Technical Information

STI-0061

July. 30, 2001

| Title | #### Fault Report/Recover Message description |
| --- | --- |
| System | CS1000 |
| Abstract | Fault Message Description |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 07.30.2001 | S.H Jin | First issue |

Table of Contents

Alarm Definition	2

RSU Injection	2

RSU Power On/Off	2

RGU Injection	3

RGU Power On/Off	3

Local Board Injection	4

Dual Board Injection	4

H/W Back Connector On/Off	5

EXPU Sub Board Removal / Injection	5

CLKU Sub Board Removal / Injection	6

IOTU Sub Board Removal / Injection	6

I/O Port Injection On/Off	7

Synch Fail / Recover	7

S/W_Watch Dog	8

Control Restart	8

Local Board Restart	9

CLOCK Check	9

Synch Master/Slave Assign	10

Line Lock Out / Release	10

Title : Fault Report/Recover Message description

# Alarm Definition

## RSU Injection

- Reason happened

- PSU(Power Supply Unit) Removal/Injection

- PSU(Power Supply Unit) own fault

- Fault Report / Recover Message

[ FAULT REPORT ]

2001-07-04 06:44:38

CATEGORY : POWER

MESSAGE   : POWER INJECTION

LOCATION : BASIC SHELF, DC POWER, DUAL 0

[ FAULT RECOVER ]

2001-07-04 06:44:38

CATEGORY : POWER

MESSAGE   : POWER INJECTION

LOCATION : BASIC SHELF, DC POWER, DUAL 0

## RSU Power On/Off

- Reason happened

- PSU(Power Supply Unit) Removal/Injection

- PSU(Power Supply Unit) own Fault

- PSU(Power Supply Unit) Power supply Switch On/Off

- Fault Report / Recover Message

[ FAULT REPORT ]

2001-07-04 06:44:38

CATEGORY : POWER

MESSAGE   : POWER ALARM

LOCATION : BASIC SHELF, DC POWER, DUAL 0

[ FAULT RECOVER ]

2001-07-04 06:44:38

CATEGORY : POWER

MESSAGE   : POWER ALARM

LOCATION : BASIC SHELF, DC POWER, DUAL 0

## RGU Injection

- Reason happened

- RGU(Ring Generate Unit) Removal/Injection

- RGU(Ring Generate Unit) own Fault

- Fault Report / Recover Message

[ FAULT REPORT ]

2001-07-04 06:44:38

CATEGORY : POWER

MESSAGE   : RING INJECTION

LOCATION : BASIC SHELF, RING, DUAL 0

[ FAULT RECOVER ]

2001-07-04 06:44:38

CATEGORY : POWER

MESSAGE   : RING INJECTION

LOCATION : BASIC SHELF, RING, DUAL 0

## RGU Power On/Off

- Reason happened

- RGU(Ring Generate Unit) Removal/Injection

- RGU(Ring Generate Unit) own Fault

- RGU(Ring Generate Unit) Power supply Switch On/Off

- Fault Report / Recover Message

[ FAULT REPORT ]

2001-07-04 06:44:38

CATEGORY : POWER

MESSAGE   : RING ALARM

LOCATION : BASIC SHELF, RING, DUAL 0

[ FAULT RECOVER ]

2001-07-04 06:44:38

CATEGORY : POWER

MESSAGE   : RING ALARM

LOCATION : BASIC SHELF, RING, DUAL 0

## Local Board Injection

- Reason happened

- LPB(Local Processor Board) Removal/Injection

- Fault Report / Recover Message

[ FAULT REPORT ]

2001-07-04 06:44:38

CATEGORY : LINE or TRUNK or RESOURCE

MESSAGE   : LOCAL PROCESSOR INJECTION

LOCATION : SHELF 1 LB 4, ASLC

[ FAULT RECOVER ]

2001-07-04 06:44:38

CATEGORY : LINE or TRUNK or RESOURCE

MESSAGE   : LOCAL PROCESSOR INJECTION

LOCATION : SHELF 1 LB 4, ASLC

## Dual Board Injection

- Reason happened

- Dual Processor Board Removal/Injection

- Dual Processor Board own Fault

- Fault Report / Recover Message

[ FAULT REPORT ]

2001-07-04 06:44:38

CATEGORY : CONTROL

MESSAGE   : DUAL INJECTION

LOCATION : STAND - BY CPM, EMPTY

[ FAULT RECOVER ]

2001-07-04 06:44:38

CATEGORY : CONTROL

MESSAGE   : DUAL INJECTION

LOCATION : STAND - BY CPM, EMPTY

## H/W Back Connector On/Off

- Reason happened

- HWBC(Highway Back Connector) Removal/Injection

- HWBC Inject Flag Fault

- Fault Report / Recover Message

[ FAULT REPORT ]

2001-07-04 06:44:38

CATEGORY : CONTROL

MESSAGE   : EXU INJECTION

LOCATION : EX UNIT, FCC, FCC1/ FCC2/ FCC3/ FCC4/ FCC5

[ FAULT RECOVER ]

2001-07-04 06:44:38

CATEGORY : CONTROL

MESSAGE   : EXU INJECTION

LOCATION : EX UNIT, FCC, FCC1/ FCC2/ FCC3/ FCC4/ FCC5

## EXPU Sub Board Removal / Injection

- Reason happened

- EXPU Sub Board Removal / Injection

- EXPU Inject Flag Fault

- Fault Report / Recover Message

[ FAULT REPORT ]

2001-07-04 06:44:38

CATEGORY : CONTROL

MESSAGE   : EXU INJECTION

LOCATION : EX UNIT, EXBP, EXPU

[ FAULT RECOVER ]

2001-07-04 06:44:38

CATEGORY : CONTROL

MESSAGE   : EXU INJECTION

LOCATION  : EX UNIT, EXBP, EXPU

## CLKU Sub Board Removal / Injection

- Reason happened

- CLK Unit Removal / Injection

- EXPU Inject Flag Fault

- Fault Report / Recover Message

[ FAULT REPORT ]

2001-07-04 06:44:38

CATEGORY : CONTROL

MESSAGE   : EXU INJECTION

LOCATION : EX UNIT, EXBP, CLKU

[ FAULT RECOVER ]

2001-07-04 06:44:38

CATEGORY : CONTROL

MESSAGE   : EXU INJECTION

LOCATION : EX UNIT, EXBP, CLKU

## IOTU Sub Board Removal / Injection

- Reason happened

- IOBS(Input Output Back Sub Board) Removal / Injection

- IOBS Inject Flag Fault

- Fault Report / Recover Message

[ FAULT REPORT ]

2001-07-04 06:44:38

CATEGORY : CONTROL

MESSAGE   : EXU INJECTION

LOCATION : EX UNIT, EXBP, IOTU

[ FAULT RECOVER ]

2001-07-04 06:44:38

CATEGORY : CONTROL

MESSAGE   : EXU INJECTION

LOCATION : EX UNIT, EXBP, IOTU

## I/O Port Injection On/Off

### =

- Reason happened

- IO Port Removal / Injection

- IO Port Inject Flag Fault

- Fault Report / Recover Message

[ FAULT REPORT ]

2001-07-04 06:44:38

CATEGORY : CONTROL

MESSAGE   : EXU INJECTION

LOCATION : EX UNIT, SIO/LAN, SIO1/ SIO2 /SIO3/LAN

[ FAULT RECOVER ]

2001-07-04 06:44:38

CATEGORY : CONTROL

MESSAGE   : EXU INJECTION

LOCATION : EX UNIT, SIO/LAN, SIO1/ SIO2 /SIO3/LAN

## Synch Fail / Recover

- Reason happened

- Digital Trunk Synch. Fault

- Fault Report / Recover Message

[ FAULT REPORT ]

2001-07-04 06:44:38

CATEGORY : CLOCK

MESSAGE   : SYNCH FAIL/RECOVER

LOCATION : SHELF 1 LB 6, DNIC DTC

[ FAULT RECOVER ]

2001-07-04 06:44:38

CATEGORY : CLOCK

MESSAGE   : SYNCH FAIL/RECOVER

LOCATION : SHELF 1 LB 6, DNIC DTC

## S/W_Watch Dog

- Reason happened

- When system abnormally works for a while.

- Fault Report / Recover Message

[ FAULT REPORT ]

2001-07-04 06:44:38

CATEGORY : CONTROL

MESSAGE   : Software Watch-dog

LOCATION : ACTIVE CPM, EMPTY

[ FAULT RECOVER ]

2001-07-04 06:44:38

CATEGORY : CONTROL

MESSAGE   : Software Watch-dog

LOCATION : ACTIVE CPM, EMPTY

## Control Restart

- Reason happened

- Power connecting first, CPM Board Restarting, heavy traffic Calls

- Fault Report / Recover Message

[ FAULT REPORT ]

2001-07-04 06:44:38

CATEGORY : CONTROL

MESSAGE   : CONTROL RESTART

LOCATION : ACTIVE CPM, EMPTY

FAULT CLASS : MAJOR

[ FAULT RECOVER ]

2001-07-04 06:44:38

CATEGORY : CONTROL

MESSAGE   : CONTROL RESTART

LOCATION : ACTIVE CPM, EMPTY

FAULT CLASS : MAJOR

## Local Board Restart

- Reason happened

- Local Board Restart.

- Fault Report / Recover Message

[ FAULT REPORT ]

2001-07-04 06:44:38

CATEGORY : CLOCK

MESSAGE   : LOCAL PROCESSOR RESTART

LOCATION : SHELF 1 LB 4, ASLC

[ FAULT RECOVER ]

2001-07-04 06:44:38

CATEGORY : CLOCK

MESSAGE   : LOCAL PROCESSOR RESTART

LOCATION : SHELF 1 LB 4, ASLC

FAULT CLASS : MAJOR

## CLOCK Check

- Reason happened

- Time range has an error when system time is checked.

- Fault Report / Recover Message

[ FAULT REPORT ]

2001-07-04 06:44:38

CATEGORY : CLOCK

MESSAGE   : ClOCK

LOCATION : ACTIVE CPM, EMPTY

FAULT CLASS : MAJOR

[ FAULT RECOVER ]

2001-07-04 06:44:38

CATEGORY : CLOCK

MESSAGE   : ClOCK

LOCATION : ACTIVE CPM, EMPTY

FAULT CLASS : MAJOR

## Synch Master/Slave Assign

- Reason happened

- Digital Trunk Synch. Fault

- Fault Report / Recover Message

[ FAULT REPORT ]

2001-07-04 06:44:38

CATEGORY : CLOCK

MESSAGE   : SET SYNCH

LOCATION : SHELF 1 LB 6, DNIC DTC

[ FAULT RECOVER ]

2001-07-04 06:44:38

CATEGORY : CLOCK

MESSAGE   : SET SYNCH

LOCATION : SHELF 1 LB 6, DNIC DTC

## Line Lock Out / Release

- Reason happened

- When subscriber’s hand set is off hook without dialing

- Fault Report / Recover Message

[ FAULT REPORT ]

2001-07-04 06:44:38

CATEGORY : LINE

MESSAGE   : LINE Lock Out/Release

LOCATION : SHELF 1 LB 6, ASLC

[ FAULT RECOVER ]

2001-07-04 06:44:38

CATEGORY : LINE

MESSAGE   : LINE Lock Out/Release

LOCATION : SHELF 1 LB 6, ASLC