LG PBX System

Software Technical Information

STI-0063

Aug. 22, 2001

| Title | #### Indication Tone for autohold call. |
| --- | --- |
| System | CS1000 |
| Abstract | The sending indication tone to notify that the second call is waiting to autohold extension. |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 22.08.2001 | H.G Jung |  |

Table of Contents

Title : Autohold Indication Tone	2

Functional description	2

Operating procedure	2

Service conditions	2

Data generation	2

Interactions	2

Title : Autohold Indication Tone

Functional description

The function is to notify the autohold extension which is conversation of the second call is waiting by indication tone.

Operating procedure

- D-Tel telephone (Autohold extension)

1	Autohold extension D-Tel (A) is conversation with another D-Tel (B).

2	D-Tel (c) or External subscriber calls Autohold extension D-Tel (A).

- Autohold extension D-Tel (A) hears the indication tone to notify the second call is waiting.

Service conditions

- Pool/Loop button should be already assigned in LKD flexible button.

- Indication tone is transmitted at regular intervals to the extension. The OMS can adjust the time interval.

Data generation

- Assign Autohold ExtensionLKD

- [3.1 Extension Information  Extension Characteristic  32 Auto Hold]

- Assign LKD Auto Hold Ring Use Option

- [2.6 System Feature Option  LKD Auto Hold No Ring Use]

- Change the duration time of indication tone.

[2.4 System Tone Parameter ⇒ Camp On Alert Tone]

Interactions

- N/A