LG PBX System

### Software Technical Information

STI-0065

Aug 23, 2001

| Title | #### MSN/Sub Address Day/Night Service |
| --- | --- |
| System | CS1000 |
| Abstract | Function to separate MSN/Sub Address numbers of incoming calls to BRI Trunk into Day and Night by Day/Night service mode. |

Revision History

| ###### Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 2001.08.23 | SY Jeong | Initial Development |

Table of Contents

Software Technical Information	1

Title : MSN/SUB ADDRESS DAY/NIGHT SERVICE	2

Functional Description	2

Operation Procedure	2

Conditions	2

Data Generation	2

Interaction	2

Hardware	2

OMS Menu 7.3	MSN/Sub Address Destination	3

	OMS Functional description	3

	How to use	3

Title : MSN/SUB ADDRESS DAY/NIGHT SERVICE

Functional Description

This function is to separate MSN/Sub Address numbers of incoming calls to BRI Trunk into Day and Night by Day/Night service mode.

Operation Procedure

N/A

Conditions

- Assign Day/Night destination numbers to MSN or Sub Address number.

- Destination number can be assigned for extension No., representative No. of calling group, outside No., QSIG No., ACD No..

Data Generation

- Assign BRI port to be used for MSN or Sub address for ISDN BRI Trunk

- [4.1 Trunk Information  Trunk Link Option  ISDN BRI MSN Use] or [4.1 Trunk Information  Trunk Link Option  ISDN BRI Sub Address]

- Assign Destination to be terminated for MSN/Sub Address number terminated from PSTN.

- [7.3 MSN/Sub Address Destination]

- Assign representative No. of calling group in case destination number is the representative No. of calling group

- [6.4 Calling Group]

- Assign related information in case destination number is QSIG

- [8. QSIG Data Management]

- Assign related information in case destination number is ACD Pilot Number.

- ACDManagement

Interaction

- If you upgrade version 2.00 or lower, in which one destination number is assigned for incoming call, using CS1000.DT, the assigned number will be duplicated both to Day and Night mode.

Hardware

N/A

OMS Menu 7.3	MSN/Sub Address Destination

	OMS Functional description

Extension subscriber can check, assign, change, and delete trunk access code on his own.

	How to use

1	Query

- Input ISDN BRI Trunk Port NO.

- Click the Query button.

2	Add

- Click a Trunk Port No. you want to assign.

- Select a number to enter in Trunk Port.

- Select Day Type. The type includes extension No., outside No., representative No. of calling group, QSIG No.,  ACD No.

- Input Day Destination.

- Select Night Type. The type is in the same as the above Day Type.

- Input Night Destination number.

- Click the Add button.

- Click the Send button.

- Click the Query button to check the added items

3	Delete

- Position the cursor on an item you want to delete.

- Click the Delete button.

- Click the Send button.

- Click the Query button to check the deleted items

	Contents

| Item | Contents |
| --- | --- |
| Extension | Extension No. |
| ACD No. | ACD Pilot No. |