LG PBX System

### Software Technical Information

STI-0066

Aug 22, 2001

| Title | #### ITGC Automatic Call Routing by Link Status |
| --- | --- |
| System | CS1000 |
| Abstract | Function to send an outgoing call via a route in case it is impossible to make a call using ITGC, by automatically checking the Physical / Logical Link Status of ITGC |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 2001.08.22 | HK Jung | Initial Development |

Table of Contents

Software Technical Information	1

Title : ITGC Automatic Call Routing by Link Status	2

Functional Description	2

Requirements for Use	2

Data Generation	2

Interaction	3

Hardware	3

Title : ITGC Automatic Call Routing by Link Status

Functional Description

This function is to route an outgoing call using the function of Trunk Alternative Routing set in the system in case it is impossible to make a call using ITGC, by automatically detecting the Physical / Logical Link Status of ITGC to check whether to make a call using ITGC.

- Physical Link Detection

Check the physical LAN Connection of ITGC and the initial Device.

If Physical LAN Connection failed, this case gets to block all ports of ITGC so that outgoing calls cannot be made.

- Logical Link Detection

Perform PING at intervals of 30 seconds using Destination IP registered in Call Routing Table of each VOIP Group in order to check whether to make a call.

Whether it is possible to make a call depends on Call Prefix assigned, although a call is made using the same ITGC according to the result of PING test.

* Under the conditions below is available to use Trunk Alternative Routing by Logical Link Detection.

- If LCR is not used :

If AND Digit used for Access Code is equal to or higher than Prefix of Call Routing Table

- If LCR is used :

If Digit converted by DMT per LTD is equal to or higher than Prefix of Call Routing Table

- Functions for Trunk Alternative Routing

  - ARS of Trunk Access Code

  - LCR

Requirements for Use

- To use functions for Trunk Alternative Routing by ITGC Link Status, related data must be chosen.

Data Generation

- Assigning AND Digit per Trunk Access Code

- [4.2 Trunk Access Code  AND Digit Item]

- Assigning ARS Code per Trunk Access Code

- [4.2 Trunk Access Code  ARS Code Item]

- Assigning Trunk Access Code for LCR Service

- [4.2 Trunk Access Code  Info 2 Item]

- Assigning LCR Information Table

- [12.1 LCR Information]

- Assigning LCR Leading Digit Table

- [12.2 LDT(Leading Digit Table)]

- Assigning LCR Digit Modification Table

- [12.3 DMT(Digit Modification Table)]

Interaction

- N/A

Hardware

N/A