LG PBX System

- Software Technical Information

STI-0069

July 09, 2001

| Title | #### VPM Installation Guide |
| --- | --- |
| System | CS1000 |
| Abstract | Guide to install VPM which serves as Voice Processing Module built in CS1000 and functions as Local Board to provide the voice services of IVR (Interactive Voice Response : Auto Attendant), VMS(Voice Mail Service), Voice Tone, Bulletin, etc. |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 11.30.2000 | H.G Jung | First issue. |
| 1.1 | 05.10.2001 | H.G Jung | Third issue. |
| 1.2 | 07.09.2001 | H.G Jung | IVR service added issue. |
| 1.3 | 09.09.2001 | soo | User Defined Token issue |

Table of Contents

Overview	4

IVR(Interactive Voice Routing : Auto Attendant) Service.	4

Voice Mail Service	4

Voice Tone	4

Bulletin	5

Token	6

Type 1 (Fixed Token)	6

Type 2 (User Defined Token)	6

Type 3 (Bulletin Token)	6

VPM Installation	7

Fixed Token List	7

Fixed Token List	7

Error messages	9

Date (month)	10

Date (day)	10

Time (hour)	11

Time (minute)	12

Number	13

VPM Board Setting In OMS.	15

VPM Board Generation	15

Assigning Representative No. and VPM Port per Service	15

VPM S/W Installation	16

Files Layout	16

How to Add User Defined Token	16

How To Load File	17

Service Type	25

VMS	25

Description	25

Service Method	25

Service Scenario	29

Data Management	34

Correlation	35

IVR(Interactive Voice Response : Auto Attendant)	36

Description	36

Service Method.	36

Service Scenario	37

Data Management	39

Correlation	39

Voice Tone	40

Description	40

Installation	40

Service Method	40

Data Management	41

Correlation	41

Voice Bulletin	42

Description	42

Setup Procedure	42

Service Method	43

Service Scenario	44

Data Management	46

Correlation	46

Bulletin Message Checkup	46

Trouble Shooting Guide	47

Title : VPM Installation Guide

# Overview

VPM serves as Module built in CS1000 and provides the voice services of IVR (Interactive Voice Response : Auto Attendant), VMS(Voice Mail Service), Voice Tone, Bulletin, etc through voice announcement.

## IVR(Interactive Voice Routing : Auto Attendant) Service.

VPM first respond to an incoming trunk call or an extension call through Auto ATD Service Representative Number assigned to a system and then provides voice announcement to the caller. If the caller enters DTMF following the VPM voice announcement, the call will be switched to the destination extension. This function provides the service to supersede existing operator’s simple switching work so that the service can be cost-effective in manpower and timesaving. The features of VPM are as below.

- This function is enabled to automatically identify digits entered by a caller using Inter Digit Lock as extension number and CCR(Call Control Routing) Menu number.

- Various types of CCR destinations are assignable for use as below.

        - Different CCR Table Index

        - Extension number

        - Calling Group Representative number

        - ACD Pilot number

        - System Speed Code

        - Individual Speed Code of defined extension number

        - QSIG number

        - Internal Zone Paging

        - ATD Access Code

        - UNA

- Every CCR Table is allowed to have three individual announcements max.

- Every CCR Table has individual Night Bar needed to recognize Day/Night Mode.

- Every CCR Table has Wait Time needed to enter DTMF after announcement transmission.

- An operator may use CCR Table within the limit of the max resources available in the system in a flexible way.

## Voice Mail Service

VMS are the initials of Voice Mail Service, which is used to store voice message over a telephone using voice digitization and transmission technology or receive the stored voice message. This service is based on functions to record/listen/delete voice message and to arrange the process. When there is an incoming call in the absence or DND state, this service is to give greeting announcement to a caller and to lead the caller to leaving a message so that it may raise successful call rate.

## Voice Tone

This service is to replace various tones (Hold, Split Tone, ACD Service Waiting Tone, etc) provided in system Tone PROM with voice announcement. This service is helpful in using multi-VPM port for the same tone. The subscriber can hear the tone from the start of the announcement

## Bulletin

This service is to give extension subscribers in-house bulletin through voice announcement. The extension subscribers can record the bulletin through a telephone.

# Token

Token corresponds to voice announcements used in VPM and is classified into three by service type as below.

## Type 1 (Fixed Token)

Type 1 is basic announcement (constantly fixed regardless of users) for IVR, VMS, and Bulletin services. Type 1 is made in a wave file, recorded by system operator, compiled in system file (vpidx.hex & vpdat.hex) by LGE, and loaded in VPM for use.

The recoding time of one token is not limited, but that of the whole Fixed Tokens is limited to 30 minutes.

## Type 2 (User Defined Token)

Type 2 is additional voice announcement used for voice tone, and for IVR Service per CCR Table. (It depends on user customization. Type 2 is made in wave file, recorded by system operator, compiled by system operator, added to system file (usridx.hex & usrdat.hex) provided by LGE, and loaded in VPM for use.

The recoding time of one token is not limited, but that of the whole User Defined Tokens is limited to 25 minutes. Type 2 is allowed for using up to 50 tokens.

## Type 3 (Bulletin Token)

Type 3 is voice announcement for in-house bulletin. The extension subscribers can record/check/delete the bulletin through a telephone.

The recording time of one token is limited to max 1 minute.

# VPM Installation

## Fixed Token List

LGE has composed Token Type 1 in English so that non-English-speaking countries are recommended to see the following when entering into a contract with LGE for CS 1000 supply: Record each token in their own language only once according to the List Table and make it in wave file. Then feed them to LG.

LGE must convert wave file into system file (vpidex.hex & vpdat.hex) to feed them back to the customer. Or if the customer uploads the file using VPM Card over the computer, he can receive VPM service in his own language. (Refer to 2.2.1.3 How to upload VPM File)

Example of file name) RINGBACK.wav

### Fixed Token List

| File Name | Voice Announcement |
| --- | --- |
| RINGBACK | 3 seconds ring back tone (Not announcement) |
| BEEP | 1 second beep (Not announcement) |
| END | 30 Seconds congestion tone (Not announcement) |
| VP100000 | Press “1” to leave a message, press “3” for subscriber’s service, press “4” for bulletin service. |
| VP100001 | Press “1” to leave a message or press “3” for subscriber’s service. |
| VP100002 | Press “1” to leave a message |
| VP100003 | Press “3” for subscriber’s service or press “4” for voice bulletin |
| VP100004 | Press “3” for subscriber’s service. |
| VP110000 | Sorry |
| VP110001 | Press an extension number |
| VP110002 | You will be connected shortly |
| VP110003 | Please leave a message after the beep tone and press the “*” button when you are finished. |
| VP110004 | Your message has been properly recorded |
| VP110005 | Thanks for your call |
| VP110006 | Your message has been cancelled |
| VP110007 | Please wait for a while |
| VP110008 | Please try again later |
| VP110010 | Press * key and extension number for direct connection. |
| VP130000 | Please press your mail box number. |
| VP130001 | Please press 4 digits password |
| VP130002 | Press “1” to listen your message or press “3” for voice mail service |
| VP110003 | Press “0” to listen this menu again or press “*” to quit |
| VP131000 | You have |
| VP131001 | messages. |
| VP131002 | You have 1 message. |
| VP131003 | This message has been recorded at |
| VP131004 | Press “1” to listen to the next message, press “2” to listen again, press “8” to delete the message. |
| VP131005 | The message has been deleted |
| VP131006 | There are no more messages in your mail box. |
| VP131007 | There is no message in your mail box. |
| VP131008 | We’re sorry, the mailbox is full. No more messages can be recorded. |
| VP131009 | message |
| VP131010 | messages |
| VP131011 | Press “0” to return to top menu or press “*” to quit. |
| VP132201 | Please record greetings after beep and press “*” when you are done. |
| VP132202 | The greeting has not been registered. |
| VP133000 | Please press “1” to change the password and press “2” to check your greeting message. |
| VP133001 | Please press “1” to change the password and press “2” to check your greeting and press “3” to register for “Absence”. |
| VP133002 | Press “0” to return to previous menu or press “*” to quit |
| VP133101 | Press a new password of 4 digits. |
| VP133102 | The change has been confirmed. |
| VP133200 | Press “1” to register, press “2” to confirm, press “8” to delete |
| VP133300 | Press “1” to register, press “2” to cancel |
| VP133301 | The registration has been completed. |
| VP133302 | The cancellation has been completed. |
| VP140000 | This is the bulletin service |
| VP140001 | There is no message for bulletin |
| VP140002 | Press “1” to record or press “2” to confirm or press “8” to delete. |
| VP140003 | Press the voice bulletin number to record or delete. |
| VP140004 | Press “*” to quit |
| ABS001 | The person doesn’t answer your call. |
| ABS002 | The line is busy. |
| ABS003 | The person cannot answer your call. |
| ABS004 | The person is not in right now. |
| ABS005 | The person is out to lunch. |
| ABS006 | The person is on vacation. |
| ABS007 | The person is out of the office on business. |
| ABS008 | The person is in office, but doesn’t be connected. |
| ABS009 | The person is in a meeting. |
| ABS010 | The person has left for the day. |
| ABS011 | The person is away on business. |
| ABS012 | You have pressed the unavailable number. |
| ABS013 | The number cannot be called. |

### Error messages

| File Name | Voice Announcement |
| --- | --- |
| E00001 | You have pressed a wrong number |
| E00002 | Please find out the instructions and try to call again later |
| E00003 | Your request has been accepted, please wait for a while |
| E00004 | Your request has been delayed. Please try again later. |
| E00005 | The system requires a maintenance service please try again later |
| E00006 | Sorry, you cannot be connected due to the overload of call traffic. |
| E00007 | The system is down, please try again later |
| E00008 | Please try to call again after rechecking your password |
| E00009 | You have entered the wrong data |
| E00010 | Someone is listening to the message; you cannot delete it now. |

### Date (month)

| File Name | Voice Announcement | File Name | Voice Announcement |
| --- | --- | --- | --- |
| SOLAR | Solar calendar | LUNAR | Lunar calendar |
| MON00 | Month |  |  |
| MON01 | January | MON07 | July |
| MON02 | February | MON08 | August |
| MON03 | March | MON09 | September |
| MON04 | April | MON10 | October |
| MON05 | May | MON11 | November |
| MON06 | June | MON12 | December |

### Date (day)

| File Name | Voice Announcement | File Name | Voice Announcement |
| --- | --- | --- | --- |
| DAY00 | Day | DAY16 | Sixteenth |
| DAY01 | First | DAY17 | Seventeenth |
| DAY02 | Second | DAY18 | Eighteenth |
| DAY03 | Third | DAY19 | Nineteenth |
| DAY04 | Fourth | DAY20 | Twentieth |
| DAY05 | Fifth | DAY21 | Twenty first |
| DAY06 | Sixth | DAY22 | Twenty second |
| DAY07 | Seventh | DAY23 | Twenty third |
| DAY08 | Eighth | DAY24 | Twenty fourth |
| DAY09 | Ninth | DAY25 | Twenty fifth |
| DAY10 | Tenth | DAY26 | Twenty sixth |
| DAY11 | Eleventh | DAY27 | Twenty seventh |
| DAY12 | Twelfth | DAY28 | Twenty eighth |
| DAY13 | Thirteenth | DAY29 | Twenty ninth |
| DAY14 | Fourteenth | DAY30 | Thirtieth |
| DAY15 | Fifteenth | DAY31 | Thirty First |

### Time (hour)

| File Name | Voice Announcement | File Name | Voice Announcement |  |
| --- | --- | --- | --- | --- |
| AM | a.m | PM |  | p.m. |
| HH00 | Zero |  |  |  |
| HH01 | One | HH13 |  | One |
| HH02 | Two | HH14 |  | Two |
| HH03 | Three | HH15 |  | Three |
| HH04 | Four | HH16 |  | Four |
| HH05 | Five | HH17 |  | Five |
| HH06 | Six | HH18 |  | Six |
| HH07 | Seven | HH19 |  | Seven |
| HH08 | Eight | HH20 |  | Eight |
| HH09 | Nine | HH21 |  | Nine |
| HH10 | Ten | HH22 |  | Ten |
| HH11 | Eleven | HH23 |  | Eleven |
| HH12 | Twelve | HH24 |  | Twelve |

### Time (minute)

| File Name | Voice Announcement | File Name | Voice Announcement |
| --- | --- | --- | --- |
| MM00 | 0 |  |  |
| MM01 | 1 | MM31 | 31 |
| MM02 | 2 | MM32 | 32 |
| MM03 | 3 | MM33 | 33 |
| MM04 | 4 | MM34 | 34 |
| MM05 | 5 | MM35 | 35 |
| MM06 | 6 | MM36 | 36 |
| MM07 | 7 | MM37 | 37 |
| MM08 | 8 | MM38 | 38 |
| MM09 | 9 | MM39 | 39 |
| MM10 | 10 | MM40 | 40 |
| MM11 | 11 | MM41 | 41 |
| MM12 | 12 | MM42 | 42 |
| MM13 | 13 | MM43 | 43 |
| MM14 | 14 | MM44 | 44 |
| MM15 | 15 | MM45 | 45 |
| MM16 | 16 | MM46 | 46 |
| MM17 | 17 | MM47 | 47 |
| MM18 | 18 | MM48 | 48 |
| MM19 | 19 | MM49 | 49 |
| MM20 | 20 | MM50 | 50 |
| MM21 | 21 | MM51 | 51 |
| MM22 | 22 | MM52 | 52 |
| MM23 | 23 | MM53 | 53 |
| MM24 | 24 | MM54 | 54 |
| MM25 | 25 | MM55 | 55 |
| MM26 | 26 | MM56 | 56 |
| MM27 | 27 | MM57 | 57 |
| MM28 | 28 | MM58 | 58 |
| MM29 | 29 | MM59 | 59 |
| MM30 | 30 | MM60 | 60 |

### Number

| File Name | Voice Announcement | File Name | Voice Announcement |  |
| --- | --- | --- | --- | --- |
| CN000 | Zero | ON000 |  | Zero |
| CN001 | One | ON001 |  | First |
| CN002 | Two | ON002 |  | Second |
| CN003 | Three | ON003 |  | Third |
| CN004 | Four | ON004 |  | Fourth |
| CN005 | Five | ON005 |  | Fifth |
| CN006 | Six | ON006 |  | Sixth |
| CN007 | Seven | ON007 |  | Seventh |
| CN008 | Eight | ON008 |  | Eighth |
| CN009 | Nine | ON009 |  | Ninth |
| CN010 | Ten | ON010 |  | Tenth |
| CN011 | Eleven | ON011 |  | Eleventh |
| CN012 | Twelve | ON012 |  | Twelfth |
| CN013 | Thirteen | ON013 |  | Thirteenth |
| CN014 | Fourteen | ON014 |  | Fourteenth |
| CN015 | Fifteen | ON015 |  | Fifteenth |
| CN016 | Sixteen | ON016 |  | Sixteenth |
| CN017 | Seventeen | ON017 |  | Seventeenth |
| CN018 | Eighteen | ON018 |  | Eighteenth |
| CN019 | Nineteen | ON019 |  | Nineteenth |
| CN020 | Twenty | ON020 |  | Twentieth |
| ONC00 | Zero |  |  |  |
| ONC01 | One |  |  |  |
| ONC02 | Two |  |  |  |
| ONC03 | Three |  |  |  |
| ONC04 | Four |  |  |  |
| ONC05 | Five |  |  |  |
| ONC06 | Six |  |  |  |
| ONC07 | Seven |  |  |  |
| ONC08 | Eight |  |  |  |
| ONC09 | Nine |  |  |  |
| ONC10 | Ten |  |  |  |
| ONC11 | Eleven |  |  |  |
| ONC12 | Twelve |  |  |  |
| ONC13 | Thirteen |  |  |  |
| ONC14 | Fourteen |  |  |  |
| ONC15 | Fifteen |  |  |  |
| ONC16 | Sixteen |  |  |  |
| ONC17 | Seventeen |  |  |  |
| ONC18 | Eighteen |  |  |  |
| ONC19 | Nineteen |  |  |  |
| ONC20 | Twenty |  |  |  |

## VPM Board Setting In OMS.

### VPM Board Generation

OMS [2.2 Local Board Information]에서 [ⓡ Board Information view  Click VPM Board]  [Board Data Generation] [Extension Data Handling Plan Dialog Window][Extension Information]

Note) ⓡ ; Right-click

### Assigning Representative No. and VPM Port per Service

Use numbers, which are currently not used as extension numbers, within the range of numbering plan existing in OMS [6.4 Calling Group], [Representative Number], [2.1 System Numbering PlanExtension Numbering Plan].

## VPM S/W Installation

### Files Layout

| File Name | Description |
| --- | --- |
| ram.hex | Scenario Control of All VPM Service |
| codec.hex codecknl.hex | Voice processing DSP file |
| dtmf.hex dtmfknl.hex | DTMF DSP file |
| vpidex.hex & vpdat.hex | File including Fixed Token and User Define Token |
| usridex.hex & usrdat.hex | Scenario Control of All VPM Service |

### How to Add User Defined Token

- Record user Defined Token needed for IVR Service & Voice Token Service and save it to a specified (ZUSER01.wav ~ ZUSER50.wav).

- Install LGE-provided VPC Program on PC. (VPC.EXE and USRDEF.DAT files)

- Copy the recorded User Defined Token file to a directory, for example, “E:\WORK\VPM\VPC\TOKENS”.

- Execute LGE-provided VPC Program.

- Choose ‘Token Dir’ for the collection of User Defined Token files.

- Choose ‘Hexa Dir’ for indicating USRDAT.HEX and USRIDX.HEX, for example, “E:\WORK\VPM\VPC\HEX”.

- Select radio button “User Defined Token” in the “Mode” box.

- Click the  button in the tool bar or go to the ‘File’ menu to select “Check Token List” so that you can check if all wave files you want exist.

- Click the  button in the tool bar or go to the ‘File’ menu to select “Run” so that you can convert wave files into UsrDat.hex and UsrIdx.hex files to be used in VPM.

- If the conversion of all the files is completed, “OK” button will be active in the progress box. Click the “OK button. Then vpdat.hex and vpidx.hex will be created in the directory as selected in the above step “6”.

### How To Load File

1) Set VPM Board.

1.1) Use RS232C Cable to connect both 9Pin SIO Connectors attached to PC and VPM Faceplate.

1.2) Execute a program for SIO communication like Hyper Terminal..

Setting  Baud Rate 19200. No Parity, Data bit 8, Stop Bit 1.

2) Assign IP address to VPM for Network use.

2.1) Disable Auto Boot status by setting it to “0”.

Open Hyper terminal in the state of PC and VPM being connected by RS-232C, and execute the commands indicated as below.

| Command | AB 0 : Autoboot Disable AB 1 : Autoboot Enable AB : Verify Autoboot state. |
| --- | --- |
| VPMBUG $ ab [Enter] AutoBoot mode[DISABLE] Usage: ab [1 ¦ 0] VPMBUG $ VPMBUG $ ab 0 [Enter] |  |

2.2) Reset the board. If a dialog box is displayed, press any key to go into [M]odify mode. Of if you do not want to go into [M]odify mode, press [Enter] key to go to the next item

2.3)  Set the START-UP mode to “4. Run the VPM Boot ROM”.

How should the board boot? 1. pROBE+ stand-alone mode 2. pROBE+ waiting for host debugger via serial connection 3. pROBE+ waiting for host debugger via a network connection 4. Run the VPM Boot ROM Which one do you want? [4] 4 [Enter]

2.4) Enter VPM IP address to set NETWORK INTERFACE PARAMETERS.

Do you want a LAN interface? [Y] y [Enter] This board's LAN IP address(0.0.0.0 = RARP)? [165.243.134.57] 165.243.134.11 [Enter] Subnet mask for LAN (0 for none)? [255.255.255.0] 255.255.255.0 [Enter] Should there be a default gateway for packet routing? [N] n [Enter]

2.5) Setting HARDWARE PARAMETERS

Set VPM ethernet address. The entry of each value must be shown in hexa decimal.

Do you want to change the board's Ethernet address? [N] y [Enter] Bytes 0 through 2 are always 08:00:3E What should byte 3 be? [96] F3[Enter] What should byte 4 be? [3D] 86[Enter] What should byte 5 be? [45] 0B[Enter]

2.6) Setting CPU delay time

Set the delay time in second until ROM code is performed after the display of dialog box.

How long (in seconds) should CPU delay before starting up? [1] 1 [Enter]

When the dialog box will be re-displayed after setting the above items, check the changed values. If correct, enter C. If you want to re-change the value, enter M and repeat the above steps.

2.7)  Verifying inputted values.

Check NVRAM when the values displayed in the dialog box are not changed.

VPMBUG $ ab [Enter] AutoBoot mode[DISABLE] Usage: ab [1 ¦ 0] VPMBUG $ VPMBUG $ ab 1 [Enter] VPMBUG $ ab [Enter] AutoBoot mode[ENABLE] Usage: ab [1 ¦ 0] VPMBUG $

3) Create a directory named ‘VPM’ in a random-selected drive on PC.

4) Copy LGE-provided VPM Software File to the VPM Directory.

5) Set up LGE-provided TFTP Program.

6) Run TFTP Server on PC.

7) Select Configure from the Menu to configure TFTP.

8) Choose a directory in which the File to upload is located.

9) Start TFTP.

10) Setup for TFTP Uploading is completed

For reference, TFTP Client is imbedded in VPM Board

11) Connect PC and VPM by SIO

Setting  Baud Rate 19200. No Parity, Data bit 8, Stop Bit 1.

12) Use LAN cable to connect VPM to Network

13) Check the IP Address.

| Command | IP |
| --- | --- |
| VPMBUG $ ip [Enter] |  |

14) File Loading.

Auto Boot Disable.

Case 1) Upload simultaneously all files to be used in VPM, like ram.hex, codec.hex, codecknl.hex, dtmf.hex, dtmfknl.hex, vpidex.hex, vpdat.hex, etc. (Uploading simultaneously all files are possible only when Autoboot is in the Disable state.)

| Command | UPA PC_IP_Address ; PC opened Tftp Application. |
| --- | --- |
| VPMBUG $ upa 165.243.134.10 [Enter] Uploading "ram.hex" from HOST 165.243.134.10 / TFTP download completed Uploading "codec.hex" from HOST 165.243.134.10 / TFTP download completed Uploading "dtmf.hex" from HOST 165.243.134.10 / TFTP download completed Uploading "vpidx.hex" from HOST 165.243.134.10 / TFTP download completed Uploading "vpdat.hex" from HOST 165.243.134.10 / TFTP download completed Uploading "codecknl.hex" from HOST 165.243.134.10 / TFTP download completed Uploading "dtmfknl.hex" from HOST 165.243.134.10 / TFTP download completed VPMBUG $ |  |

Case2) Upload Files individually (possible when Autoboot is in both states of Enable and Disable).

| Command | UPLD File_Name PC_IP_Address.; PC opened Tftp Application. |
| --- | --- |
| VPMBUG $ upld ram.hex 165.243.134.10 [Enter] Uploading "ram.hex" from HOST 165.243.134.10 \ TFTP download completed VPMBUG $ |  |

In the state of Autoboot set to Disabled, you cannot upload usrdat.hex and usridx.hex. Although you use the ‘upldt’ command, you cannot verify the state of files that has been already uploaded. Only when Autoboot is enabled, you can upload Usrdat.hex and usridx.hexus files and use the ‘upldt’ command to verify the uploaded state.

Verification) Use upldt command to verify in the table to whether you have normally uploaded files. In this case, the files that have been uploaded must be included in ACTive area.(when Autoboot is in the Disable state)

| Command | UPLDT |
| --- | --- |
| VPMBUG $ upldt [Enter] >>>>>>>>>>>>>>>>> Upload Table Information <<<<<<<<<<<<<<<<<<< ------+-----------+----------+----------+-------------------------- ACT ¦ ram.hex ¦ 01000000 ¦ 1089864 ¦ 2000/10/01 09:10:23 ¦ codec.hex ¦ 013C0000 ¦ 13DFF1E ¦ 2000/10/01 09:11:07 ¦ dtmf.hex ¦ 013E0000 ¦ 13E7FC0 ¦ 2000/10/01 09:11:24 ------+-----------+----------+----------+-------------------------- STBY ¦ ram.hex ¦ 011E0000 ¦ 0 ¦ 0000/00/00 00:00:00 ¦codec.hex ¦ 017C0000 ¦ 0 ¦ 0000/00/00 00:00:00 ¦ dtmf.hex ¦ 017E0000 ¦ 0 ¦ 0000/00/00 00:00:00 ------+-----------+----------+----------+-------------------------- VOICE ¦vpdat.hex ¦ 01420000 ¦ 14914A8 ¦ 2000/10/01 09:12:56 PROMPT ¦vpidx.hex ¦ 01400000 ¦ 1401B18 ¦ 2000/10/01 09:11:45 ------+-----------+----------+----------+--------------------- VPMBUG $ |  |

Operating) If file uploading is completed, enable autoboot and reset the board. Setting and checking autoboot is as below.

| Command | AB 0 : Autoboot Disable AB 1 : Autoboot Enable AB : Verify Autoboot state. |
| --- | --- |
| VPMBUG $ ab 1 [Enter] VPMBUG $ ab [Enter] AutoBoot mode[ENABLE] Usage: ab [1 ¦ 0] VPMBUG $ |  |

Auto Boot Enable.

If you reset the VPM when ACTive area includes a normal file and Autoboot is enabled, the VPM_Debug> prompt will appear.

In this case, you cannot use the “upa” command because applications are running. Therefore, you can upload a file individually when the AutoBoot mode is in Enable mode. Uploading ram.hex, codec.hex, or dtmf.hex files is processed in STBY(stand-by) area so that it will not affect the currently active file.

You may also upload usrdat.hex and usridx.hex files when the AutoBoot mode is in Enable state.

Assuming that you have not uploaded usrdat.hex and usridx.hex files since the initial setup of VPM board:- Execute the ‘upldt’ command to view values in the table. Values corresponding to those files show wrong addresses. If you select user-defined token, VPM may go down. For those reasons, you must upload usrdat.hex and usridx.hex files.

| Command | UPLDT ; To verify current file status. |
| --- | --- |
| VPM_Debug> upldt [Enter] >>>>>>>>>>>>>>>>> Upload Table Information <<<<<<<<<<<<<<<<< ------+------------+----------+----------+--------------------- ACT ¦ ram.hex ¦ 20000000 ¦ 20094F48 ¦ 2001/09/09 12:43:25 ¦ codec.hex ¦ 203C0000 ¦ 203C77E0 ¦ 2001/09/07 13:30:18 ¦ dtmf.hex ¦ 203E0000 ¦ 203E3FC0 ¦ 2001/09/07 13:30:23 ------+------------+----------+----------+--------------------- STBY ¦ ram.hex ¦ 201E0000 ¦ 0 ¦ 0000/00/00 00:00:00 ¦ codec.hex ¦ 207C0000 ¦ 0 ¦ 0000/00/00 00:00:00 ¦ dtmf.hex ¦ 207E0000 ¦ 0 ¦ 0000/00/00 00:00:00 ------+------------+----------+----------+--------------------- VOICE¦ vpdat.hex ¦ 20420000 ¦ 20480D74 ¦ 2001/09/07 13:31:10 PROMPT¦ vpidx.hex ¦ 20400000 ¦ 20401968 ¦ 2001/09/07 13:30:27 ------+------------+----------+----------+--------------------- DSP ¦codecknl.hex¦ 203A0000 ¦ 203A01C0 ¦ 2001/09/07 13:31:13 ¦ dtmfknl.hex¦ 203B0000 ¦ 203B01C0 ¦ 2001/09/07 13:31:17 ------+------------+----------+----------+--------------------- USER ¦ usrdat.hex ¦ 20620000 ¦ 20626CDE ¦ 2001/09/09 13:08:40 PROMPT¦ usridx.hex ¦ 20600000 ¦ 206004B0 ¦ 2001/09/09 13:08:55 ------+------------+----------+----------+--------------------- |  |
| Command | UPLD File_Name PC_IP_Adress ; PC opened Tftp Application |
| VPM_Debug> upld ram.hex 165.243.134.10 [Enter] Uploading "ram.hex" from HOST 165.243.134.10 \ TFTP download completed VPM_Debug> upldt [Enter] >>>>>>>>>>>>>>>>> Upload Table Information <<<<<<<<<<<<<<<< ------+------------+----------+----------+--------------------- ACT ¦ ram.hex ¦ 20000000 ¦ 20094F48 ¦ 2001/09/09 12:43:25 ¦ codec.hex ¦ 203C0000 ¦ 203C77E0 ¦ 2001/09/07 13:30:18 ¦ dtmf.hex ¦ 203E0000 ¦ 203E3FC0 ¦ 2001/09/07 13:30:23 ------+------------+----------+----------+--------------------- STBY ¦ ram.hex ¦ 201E0000 ¦ 20274F48 ¦ 2001/09/09 13:49:12 ¦ codec.hex ¦ 207C0000 ¦ 0 ¦ 0000/00/00 00:00:00 ¦ dtmf.hex ¦ 207E0000 ¦ 0 ¦ 0000/00/00 00:00:00 ------+------------+----------+----------+--------------------- VOICE¦ vpdat.hex ¦ 20420000 ¦ 20480D74 ¦ 2001/09/07 13:31:10 PROMPT¦ vpidx.hex ¦ 20400000 ¦ 20401968 ¦ 2001/09/07 13:30:27 ------+------------+----------+----------+--------------------- DSP ¦codecknl.hex¦ 203A0000 ¦ 203A01C0 ¦ 2001/09/07 13:31:13 ¦ dtmfknl.hex¦ 203B0000 ¦ 203B01C0 ¦ 2001/09/07 13:31:17 ------+------------+----------+----------+--------------------- USER ¦ usrdat.hex ¦ 20620000 ¦ 20626CDE ¦ 2001/09/09 13:08:40 PROMPT¦ usridx.hex ¦ 20600000 ¦ 206004B0 ¦ 2001/09/09 13:08:55 ------+------------+----------+----------+--------------------- |  |

| Meaning of Auto Boot |  |
| --- | --- |
| Disable; Non-Operation Status Use VPM data setup before service. Files uploaded from PC will be saved to Active Table of VPM. If the data setup is completed, turn the Disable state into Enable for service and reset VPM. Enable; Operation Status VPM is in a state to provide the service. If you upload a file, it will be saved to the Standby Table of VPM that does not affect the current operation status. If you want to internally re-upload the file to Active Table of VPM, perform the following steps and reset VPM. |  |
| Command | CHGM |
| VPMBUG $ chgm file Name [Enter] ; file Name = ram.hex /codec.hex/dtmf.hex |  |

14) Initializing VPM Board completes the installation procedures of VPM Board.

# Service Type

## VMS

### Description

VMS is used to perform Mail Box Service, Mail Access Service, Call Forwarding Service.

### Service Method

      - Mail Box Service

Service Type

Three types of Mail Box Service are provided as below.

1.1) Type 1

An extension registered in Mail Box dials VMS representative number [6.4 calling GroupService type; MAILACC] to check and listen a message saved in the Mail Box assigned by his own ID.

1.2) Type 2

An extension registered in Mail Box dials VMS representative number [6.4 calling GroupService type; MAILACC] and connects to VMS for recording his voice information.

When the extension is not in the state of answering like “Busy, Absent, DND, or No-answer” in case of an outside subscriber call, he can use VMS call forwarding function by selecting a code (See 1.3.1) suitable for his present state through the phone. And the outside subscriber can hear the voice information saved by extension.

1.3.1) VMS Call forwarding registration by extension

SLT / D-Tel

OMS [2.1 System Numbering Plan  Feature Plan]

| Description | Code Use |
| --- | --- |
| All (unconditional) Call Forwarding registration. | (*527) |
| Busy Call Forwarding registration | (*528) |
| Busy or No Answer Call Forwarding registration | (*52*) |
| Busy Call Forwarding registration | (*529) |
| Note) How to select a code for VMS Call forwarding in the above SLT Ex) Off-hook  *52X  On-hook ; X = 7,8,9,* |  |

D-Tel

| Description | Code Use |
| --- | --- |
| All (unconditional) Call Forwarding registration. | MON + DND/FWD + 7 |
| Busy Call Forwarding registration | MON + DND/FWD + 8 |
| Busy or No Answer Call Forwarding registration | MON + DND/FWD + * |
| Busy Call Forwarding registration | MON + DND/FWD + 9 |
| Note) How to select a code for VMS Call forwarding in the above SLT Ex) Off-hook or MON  DND/FWD  X : X = 7,8,9,* |  |

1.4) Type 3

This type is a service for an extension registered in Mail Box to dial VMS representative number so that they may change his password by dialing on the phone.

      - Mail Access Service

This service is provided for an outside subscriber to leave his voice message when the destination (extension) is not in the state of answering the outside subscriber’s call.

1.1) Whole Capacity:

The Mail Access Service is capable of 254 Tokens of 60-second voice message in Token Category1. Assuming that Bulletin service is, the number of usable token will be reduced to 244.

1.2) Capacity of Mail Box ID for an extension

An extension registered in Mail Box can have ten voice messages.

- 2) To indicate message for D-Tel, SLT.

- 2.1) How to indicate Message arrival

- D-Tel

- Indicated on LCD

MSG EXT(0) TRK(0) VM(1)

LINE 11018       06:45 PM

- VM(1): Currently a voice mail is saved in Mail Box.

- SLT

- Message Wait Ring gives to SLT at the defined interval.

- OMS

- To adjust the interval

- [31. Extension Information  Extension Characteristics SLT MW No Ring]

- 2.2) How to check a message in Extension

- 2.2.1)  Check the message over D-Tel.

- A. Check the VM counter on LCD.

- B. Press MOM Button.

- C. Press CallBK Button.

- D. Follow VMS voice message for input.

- Check at SLT.

- Case1) When using Message Ring :

- OMS [Extension Data Management3.1 Extension

- Information=>Extension Characteristics->7.SLT M/W No

- Ring]

- Listen to Message Ring

- Go into Off-Hook mode and press *7 or VMS representative number.

- Follow VMS voice Message for input

- Note) In case of Message Ring SLT in the Check mode, access Mail Box to delete the message, or else there will be rings at a certain interval.

- Case2) when not using Message Ring

- Dial VMS (MAILACC) representative at the extension.

- Follow VMS voice Message for input.

- Check Recorded Message.

- Note) Make sure whether or not Single Line Telephone Message Ring

- [3.1 Extension InformationExtension Characteristics7.SLT M/W No Ring]

      - Call Forward Service

This service is used for an outsider subscriber to call an extension registered in Mail Box, who is not in the state of answering.

Case1) When an outside subscriber calls an extension through ATD

Case2) When an outside subscriber calls an extension in a direct way

The above both cases are given the following result:-

The outside subscriber hears the voice information, which the extension has assigned for his present state, provided by VMS. And then the subscriber may leave his message,

Note1) Exception to call forwarding) When an extension has set his phone to VMS call forwarding using feature code or function button (D-Tel) and an outside subscriber is directed to the extension. (See 4.1.2.1 Mail Box Service .12) Type 2)

Note2) Incase I-Phone calls to registered extension, this service don’t allow.

### Service Scenario

##### Mail Access Service

Mail Access

Message Wait

Answer Code ?

Mail box number exist ?

Password OK ?

C1

Retry

“Please press your mail box number.”  (VP130000)

Y

N

Input DTMF

“Please press 4 digits password.”  (VP130001)

Input DTMF

Y

“You have pressed the unavailable number.” (ABS012)

N

Y

N

“You have pressed the unavailable number..” (ABS012)

“You have   (VP131000 )

XX        (CN001, … )

messages.”  (VP131001 )

Input DTMF

Are there a mail ?

“There is no message in your

mail box.” (VP131007)

“Press ‘1’ to listen your message or press ‘3’ for voice mail service.”

(VP130002)

“Press ‘0’ to listen this menu again or press ‘*’ to quit.”   (VP130003)

1

3

Message number > 0

Y

Y

N

N

C1

C2

C3

End

*

0

C4

“ ( ex. 1st )”  (ON001, …)

Play recorded message.

“This message has been recorded at ( time ).” (VP131003)

Input DTMF

“The message has been deleted.” (VP131005)

Next Mail ?

“There are no more messages in your mail box.”     (VP131006)

“Press ‘0’ to return top menu or press ‘*’ to quit.”   (VP131011)

Y

N

“Press “1” to listen to the next message,

press “2” to listen again,

press “8” to delete the message. ” (VP131004)

Listen Next Mail

Mail Access again

Mail cancel

1

8

MONTH XX

DAY XX

HH XX

MM XX

C3

2

End

C4

*

0

Mail Box Service

“Please press “1” to change a password and press “2” to check greetings message. ” (VP133000)

“Press”0” to return to previous menu or press”*” to quit”

(VP133002)

“Press a new password

of 4 digits.”

(VP133101)

1

2

Input Password

“The change has been

confirmed”(VP133102)

DTMF Input

C2

“Press “1” to register, press “2” to confirm, press “8” to delete. ” (VP133200)

“Press”0” to return to previous menu or press”*” to quit”(VP133002)

“Please record

greeting after beep

and press ‘*’ when

you are done.”

(VP132201)

Listen Greeting

Greeting cancel

“The Message has been

deleted.” (VP131005)

1

2

8

Greeting exist?

Y

N

“The greeting has not

been registered.”

(VP132202)

C2

C2

End

*

0

End

*

0

Forward Call Service

Forward Call

Individual Greeting ?

End

Busy : “Sorry, The line is busy” (ABS002)

No answer : “The person doesn’t answer your call.” (ABS001)

DND : “The person cannot answer your call.” (ABS003)

Absence - out to lunch  : “The person is out to lunch” (ABS005)

- on vacation   : “The person is on vacation” (ABS006)

- out of office  : “The person is out of the office on business” (ABS007)

- in office : “The person is in office, but doesn’t be connected.” (ABS008)

- in a meeting  : “The person is in a meeting.” (ABS009)

- at home         : “The person has left for the day”  ( ABS010)

- in a business trip : “The person is away on business.”  (ABS011)

Listen

Individual Greeting

Y

N

“Please leave a message after beep and

press “*” when you are done.”(VP110003)

Recording + ‘*’

“Your message has been properly recorded (VP110004)

Thanks for your call.” (VP110005)

VM Message Wait Registration

“beep~“ (BEEP10)

HOTKEY : ‘0’

In this step, if user

press ‘0’, we enter

Mai l Access call.

VM : Voice Mail

### Data Management

How to generate Code to connect to VMS

1) To access Voice Mail Box

OMS [6.4 Calling Group] (Refer to the OMS Part to system Manual)

- Input Representative Number

- Select Service Type  MAILACC

- Select Ring Ser Method  First or UCD

- Select Busy & DND Option  Send Busy Tone.

- No Answer Option  Send Busy Tone.

- Calling Group Service Extension  VPM Board의 Port Telephone Number.

2) How to assign extension Mail Box ID

OMS [15.4 Mail Box Management]  (Refer to OMS part in System Manual)

2.1) Built-in VPM

Define OMS [15.4 Mail Box ManagementVPM Check & Slot √ / MB ID/ Unit/ Recorded Msg Counter]

MB ID must be assigned with the same number as the extension.

- 2.2) Separate VMS device (See OMS Part of System manual for further)

Define OMS [15.4 Mail Box ManagementExternal VMS √ / MB ID(Mail Box ID)/ Password / Unit / External TelNo / Recorded Msg Counter]. (See OMS VMS Part of System manual for further)

MB ID must be assigned with the same number as the extension.

3) Voice Mail Option Definition

Define OMS [15.5 Voice Mail Option  Retry Count (In case Vacant Number)] Value at 2 or upper.

- OMS [15.5 Voice Mail Option CFWD to VM (Busy)] is assigned to ‘YES’.

- [15.5 Voice Mail Option CFWD to VM (No Answer)] is assigned to ‘YES’

4) Message Ring for Single Line Telephone Definition.

4.1) Interval Message Ring Parameter Handling

OMS [2.5 MSG Ring] to transfer to SLT subscriber

4.2) Whether to use Interval Message Ring to transfer to SLT subscriber or not.

OMS [3.1 Extension InformationExtension Characteristics7.SLT M/W No Ring] √ = No Ring.

### Correlation

Call processing order is as follows:-

1st  check point

OMS [15.5 Voice Mail Option CFWD to VM (Busy)/(No Answer)]

2nd  check point

OMS [4.9 Trunk route base day/night incoming option]

3rd check point

Call Forwarding code(*52X).

So if [15.5 Voice Mail Option CFWD to VM (Busy)/(No Answer)] is not assigned, the processing order is as follows:-

1st check point

OMS [4.9 Trunk route base day/night incoming option]

2nd check point

Call Forwarding code(*52X).

Or if OMS [4.9 Trunk route base day/night incoming option] is not assigned , the processing order is as follows:-

1st check point

OMS[15.5 Voice Mail Option CFWD to VM (Busy)/(No Answer)]

2nd

Call Forwarding code(*52X).

Note) If ATD was not installed in system, the clerk (ATD) call on scenario will access to System PNA extension. [16.4 System PNA].

## IVR(Interactive Voice Response : Auto Attendant)

### Description

This function is applied to the case that VPM first respond to an incoming trunk call or an extension call through Auto ATD Service Representative Number assigned to a system and then provides voice announcement to the caller. If the caller enters DTMF following the VPM voice announcement, the call will be switched to the destination extension.

### Service Method.

If there is a trunk or extension call to VPM, the caller can dial the extension (destination) number following the VPM voice announcement or enter one-digit menu index of operator-set CCR (Call Control Routing). If a scenario is composed for using CCR, an outside subscriber may be requested to enter CCR menu index in one step or more according to system supplier’s features.

[Features]

          - This function is enabled to automatically identify digits entered by a caller using Inter Digit Lock as extension number and CCR(Call Control Routing) Menu number

          - Various types of CCR destinations are assignable for use as below.

        - Different CCR Table Index

        - Extension number

        - Calling Group Representative number

        - ACD Pilot number

        - System Speed Code

        - Individual Speed Code of defined extension number

        - QSIG number

        - Internal Zone Paging

        - ATD Access Code

        - UNA

          - Every CCR Table is allowed to have three individual announcements max.

          - Every CCR Table has individual Night Bar needed to recognize Day/Night Mode.

          - Every CCR Table has Wait Time needed to enter DTMF after announcement transmission.

          - An operator may use CCR Table within the limit of the max resources available in the system in a flexible way.

### Service Scenario

Refer to the next pages.

IVR Service by CCR Start Table

IVR Service by Selected CCR Table

Can Transfer ?

One Digit Input ?

End of Auto ATD Service

CCR Table Day=?

More Digit Input ?

Retry Count > 0

A

Avail Numbering Plan ?

A

Avail CCR ?

DTMF Wait Time = 0 ?

A

Next CCR Table ?

B

B

CCR Table Announcement Play

Y

Connect to the desired number

“Sorry, the line is busy” (Note 1)

“Thank you for your call” (VP110005)

“You will be connected shortly” (VP110002)

“You have pressed the unavailable number” (VP110002)

N

“Thank you for your call” (VP110005)

Y

Y

Y

N (Time Out : No Dialing)

Increase Retry Count

N

N (Check Night Event)

Y

N

CCR Table Announcement Play

N

Y

Note 1: The recording of the failure is as follows:

- Cause-Busy: “The line is busy” (ABS002)

- No Answer: “The person doesn’t answer your call.” (ABS001)

- Absent: “The person is not in right now.” (ABS004)

- DND: “The person cannot answer your call.” (ABS003)

- Vacant: “You have pressed the unavailable number.” (ABS012)

- IC Deny Class: “The number cannot be called.” (ABS013)

- Transfer EXT No Answer : “Sorry the person doesn’t answer your call.”(ABS001)

N

N

Y

Y

N

Index n

      

### Data Management

- 1) Choosing IVR Service option

- [15.5 Voice Mail Option  Phone Book Service Use ]

- 2) Composing IVR Scenario

- [15.7 IVR]

- 3) Selecting Announcement for each CCR Table

- [15.7 IVR  User Define Ment 1,2,3]

- 4) Choosing Night Bar for each CCR Table

- [15.7 IVR  Day Start Time and Night Start Time]

- 5) Setting DTMF Wait Time for each CCR Table

- [15.7 IVR  DTMF Wait Time]

- 6) Assigning Start CCR Table per System Mode

- [15.7 IVR  Auto ATD Calling Group in Pop Up Menu by Mouse Right Button]

- 7) Setting Inter Digit Lock Time to Recognize Extension No. and CCR Index

[15.5 Voice Mail Option  Inter Digit Lock]

### Correlation

Note) If ATD was not installed in system, the clerk (ATD) call on scenario will access to System PNA extension. [16.4 System PNA].

## Voice Tone

### Description

This function is to use User Defined Token( ZUSER01.wav ~ ZUSER50.wav) recorded and compiled by an operator and included in vplst.dat & vplst.dat, instead of system-provided tone.

### Installation

The installation is in such a way that adds User Defined Token in the VPM S/W Installation item

### Service Method

Outside/extension subscribers who are provided with voice tone constantly may hear the voice tone from the start. Each system may choose two modes below that are user-customized

[Real Mode]

This mode is to access an assigned VPM port to give a tone to any subscriber, whenever called.

The merit is that the mode is activated on a real time basis. But it needs a lot of VPM ports.

[Interval Mode]

Voice tone is repeatedly played through an assigned port. Whenever there is a call made by a subscriber, the subscriber will be waiting in the Queue while hearing an assigned system tone. Then the subscriber will hear the voice tone from the start of voice tone play. The number of subscribers waiting in the Queue for the voice tone is not limited. The limitless number can be a merit but the subscribers must wait in the Queue for seconds (Voice Tone Recording Time / Number of VPM Ports) until the voice tone starts to play.

Ex. of Use) The case that a caller hears RBT waiting in the Queue while using VPM Port 3 for 30-second voice tone instead of ACD Queuing Split Tone.

2nd Port Play Start

1st Port Play Start

3rd Port Play Start

2nd Port Play Start

1st Port Play Start

3rd Port Play Start

2nd Port Play Start

1st Port Play Start

3rd Port Play Start

10sec

10sec

10sec

Outside subscriber A, B, and C make calls, get to wait in Queue, and hear RBT.

The subscribers A, B, C waiting in Queue start to hear voice tone simultaneously.

### Data Management

Selection of token ID of voice tone to be substituted for system tone

OMS [15.1 Voice Token InformationTone Type  Voice Tone Token ID]

Select the item of tone type currently provided by system. Then type the ID of the token recorded by user definition to Voice Tone Token item.

Select Message channel recorded in Token ID of voice tone to be substituted for system tone.

OMS [15.2 VPM Token Information VPM Token ID  Recoding Time  Start PortEnd Port]

Verify the recording time (Message Playing Time) needed to be played in the VPM according to its VPM Token ID, and assign port range for its voice channel.

### Correlation

4 tokens are used for ATD. Therefore 16 Tokens can be used for voice tone and announcement.

## Voice Bulletin

### Description

Voice Bulletin is a function for in-house members to check company bulletin through the telephone.

The number of Bulletin token (Token category3) is 10 in all. The recording time of each token is limited to one minute

### Setup Procedure

Step1) Assign a representative number to each service type for an extension to use Bulletin in OMS. [6.4 calling GroupService type; BULLETIN / TOKENCHG

Step2) Set Option to ‘Yes’ in [15.5 Voice Mail Option VPM Bulletin Service Use] using Bulletin Service.

Step3) Dial the TOKENCHG representative number assigned in OMS. Press “0” for the service of Bulletin Menu in the section of Input DTMF in TOKEN Change Scenario.

Step 4) According to Scenario Flow, extension assigned in OMS can record, change, and delete menu and information about Bulletin through the telephone.

(Recording Type; Announcement on pressing DTMF Digit 1 + Announcement on pressing DTMF Digit 2+ Announcement on pressing DTMF Digit 3 …………+ Announcement on pressing DTMF Digit N {N=1~9};. assign the services per Digit up to Max 9 )

Step 5) Call the BULLETIN representative number and then confirm the voice information of each item, after all the above 4).

Note) If the announcement of “There is no message for bulletin.” is displayed on calling Bulletin representative number, ensure that the voice information for the Menu of the number “0” is recorded.

### Service Method

Change Bulletin Token.

Only extensions eligible for Bulletin Token service can record/save/delete/change menu and information in the token for Bulletin service.

An extension or an outside subscriber may hear an announcement of the information recorded on item desired according to Scenario Flow using DTMF Digit.

### Service Scenario

(For example, )

“Press ‘1’ for new information announcement …….” , “press * to quit.” (VP140004)

Input DTMF

Bulletin Service

Play message.

DTMF OK ?

Y

N

“You have pressed a

wrong number.”(E00001)

“This is the bulletin service.”(VP140000)

**

Bulletin service can support max 10 token.

0    : bulletin menu

1 ~ 9 : bulletin message

If administrator record or delete bulletin message, he/she can use token change service.

Bulletin Menu.

Bulletin registration ?

Y

N

“There is no message for bulletin.”  (VP140001)

Token Change

“Please leave a message after beep and press “*” when you are finished.” (VP110003)

Recording + ‘*’

“Your message has been

properly recorded.”(VP110004)

1

Token Play

“Please press ‘1’ to record, press ‘2’ to confirm or press ‘8’ to delete.”(VP140002)

“Press * to quit”          (VP140004 )

“The message has

been deleted.”

(VP131005)

2

8

“ Press the voice bulletin number

to record or delete. “ (VP140003)

Input DTMF

End

*

### Data Management

Ensure that Bulletin Service is available.

[15.5 Voice Mail Option VPM Bulletin Service Use]

Assign a representative number used to access Bulletin Service.

- Input Representative Number

- Select Service Type  Bulletin&TOKENCHG

- Select Ring Ser Method  First or UCD

- Select Busy & DND Option  Send Busy Tone

- No Answer Option  Send Busy Tone.

Assignment of Bulletin Token Change available to extensions

OMS [3.1 Extinction Information Extension Characteristics13.Token Change],

### Correlation

If a system serves for Bulletin, the max number of leaving voice message in VMS is reduced from 254 to 244.

### Bulletin Message Checkup

Record/delete the message using token change service and confirm it in VPM as follows:-

| Command | MBOX ; Change to Token Category1 Prompt |
| --- | --- |
| Command | CHK B ; To Check Recoding Status |
| VPM_Debug> mbox [Enter] MBOX> chk b [Enter] Flash adr[0x02020000] Size[0x2594] Attr[00] 2000/10/11 10:42:44 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 |  |

The above shows that the message of only No. “0” of Token 10 ID is recorded. If you delete it, Attr(attribute) field indicates FF

# Trouble Shooting Guide

ST1) When you mount VPM, the system goes to the Reset mode, check the PB switch of the board. In this case, all the switches must be off.

ST2) First installation in site

| Command | MBOX RSAT |
| --- | --- |
| VPM_Debug> mbox [Enter] MBOX> rsat [Enter] Used for system : 2 SCTR Used for user mail : 0 SCTR Free : 254 SCTR MBOX> |  |

In the first installation in the site, the SAT information must be displayed as above. Otherwise, go to the VPMBUG $ mode to execute the command as below.

| ###### Command | QUIT AB O CSAT AB 1 |
| --- | --- |
| MBOX> quit [Enter] VPM_Debug> ab 0 [Enter] reset VPM VPMBUG $ csat [Enter] Fixed pattern was written at 0x5000D000 Fixed pattern was written at 0x5001D000 VPMBUG $ ab 1 [Enter] |  |
| Reset VPM |  |

The “csat” command performs a function of initializing the Mail box information table. Therefore, be sure to execute the command when initially installing the board in the site. But never execute it in operation.

ST3) When a user forgot his own password.

Viewing the List of Mail Box Number and Password Created in the VPM

| Command | MBOX ML |
| --- | --- |
| VPM_Debug> mbox [Enter] MBOX> ml [Enter] [0x5000D000: 0] ID[99999999] PSWD[9999] [0x5000D100: 1] ID[ 1000] PSWD[AAAA] MBOX> |  |

The above figure shows that the password of ID 1000 has been created to be “0000.”  (the Mail box number 99999999 is the service number of Bulletin board)

Monitoring the password and listening of each Mail Box ID created in the VPM

| Command | MBOX CHK # ; # Mail Box ID |
| --- | --- |
| MBOX> chk 1000 [Enter] MBOX_ID[1000] Passwd[AAAA] MaxMAIL[3] VoxMAIL[2] CurRECD[0] Flash adr[0x02030000] Size[0x21C0] Attr[01] 2000/10/11 10:59:22 Flash adr[0x02040000] Size[0x0C4E] Attr[00] 2000/10/11 11:07:43 |  |

The above figure shows that the Mail box of ID 1000 can save max three voice messages (Max MAIL); and now it has two voice messages (VoxMAIL) recorded. In the Attr field, 00 means that the voice message is not heard yet and FF means that the voice message has been deleted already.

ST4) When Prompt indicates “pROBE+>” in the connection of VPM and PC by SIO

Cause:When Autoboot is in the Enable mode and upld table is different from Data in Flash, “pROBE+>” mode appears.

Solution :Change Autoboot into the Disable mode.

pROBE+>dm 600201d0..600201d1 600201d0 01 pROBE+>fm.c 600201d0..600201d1 00 pROBE+>dm 600201d0..600201d1 600201d0 00