LG PBX System

Software Technical Information

STI-0079

SEP 27, 2001

| Title | #### VOIP Overlap Sending |
| --- | --- |
| System | CS1000, NCS200 |
| Abstract | Function to process in the Overlap mode the address information (caller phone number) on a call that has been set up using a ITGC card |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 09.27.2001 | Y.H Lee, H.D Kim | First issue. |

Table of Contents

Title : VoIP Overlap sending	2

Functional Description	2

Operation Procedure	2

Conditions	2

Data Generation	3

Interaction	3

Hardware	3

Title : VoIP Overlap sending

Functional Description

Among modes of transmitting a destination number for a call setup using an ITGC card, Overlap is used in the way of sending the whole digits one by one on a press-and-send basis instead of sending a batch of the whole digits that have been pressed. This mode is to process a call in the way that the called party acknowledges the last digit.

[Service Type]

- Mode in which an extension subscriber calls an outside subscriber through an ITGC card after accessing the Gateway connected to a different VOIP network.

- Mode in which an incoming call from PSTN calls an outside subscriber through an ITGC card after accessing the Gateway connected to a different VOIP network.

- Mode in which an incoming call from IP Network is connected to an extension.

- Mode in which an incoming call from IP Network is transit-connected to Local PSTN linked with CS 1000.

Operation Procedure

- [The sequence that an Extension Subscriber calls an Extension Subscriber connected to other PBX through

- the VoIP network]

- An IP phone subscriber presses a VoIP Gateway code to select the PBX installed in the area to which

- the destination belongs.

- And then adds an extension number in the PBX.

- The destination party answers the call.

- The call is successfully connected.

- [The sequence that an extension subscriber calls a PSTN Subscriber after pressing the trunk access code through

- the VoIP network]

- An extension subscriber presses a VoIP Gateway code to select the PBX installed in the area to which

- the destination belongs.

- Presses the access code to access Local PSTN generated in the PBX.

- And then presses the destination number.

- The destination party answers the call.

- The call is successfully connected.

- [The sequence that an extension subscriber calls other PBX extension subscriber connected with Tie Line between

- the PBXs’ through the VoIP network.]

- An extension subscriber presses a VoIP Gateway code to select an accessible PBX.

- Presses the access code for the tie line generated in the PBX.

- And the presses the destination extension number belonging to the PBX selected on the tie line.

- The destination party answers the call.

- The call is successfully connected.

Conditions

- If VOIP Data is in Gatekeeper mode, Overlap sending is not supported.

- The basic data of transmitting VoIP digit is in Overlap mode of Direct type after the system initialization..

- From System Version 2.01a (based on EU version), ITGC version 3.01 forward, the function of overlap sending is provided.

- One ITGC Card provides 10 channels so that it can provide 10 calls at a time. The more ITGC cards, the more called parties at a time. The max number of ITGC cards to be mounted is 8.

- Indirect call is currently not provided in VOIP OG Method.

Data Generation

- ITGC card

  - System Local Board Information -> Selecting Slot -> Clicking Right Mouse Button -> Board Exchange ->[Selecting ITGC]  -> Selecting ITGC Board -> Clicking Right Mouse Button -> [Board Data Generation]

  - VOIP Data -> [VOIP Own Identification]

- Recommendations

  - If Code type is of G.711u or G.711 a, Frame/Packet Rate is 4.

  - In other Code types, Frame/Packet Rate is 1.

  - VOIP Data -> [VOIP O/G Prefix & Call Route ]

  - Trunk Data -> [Trunk Access Code]

  - Trunk Data -> Trunk Access Code -> [Trunk O/G Group Information]

- Trunk Transit

  - Trunk Data -> [Inter Route Connection Allow/Deny]

- Digit Sending Mode in VOIP

  - Overlap mode in Direct Call

  - Trunk Data -> Trunk Route Base All Option -> VOIP OG Method -> [Direct Call]

  - Trunk Data -> Trunk Route Base All Option -> ISDN Enbloc OG Mode -> [NO]

  - Enblock mode in Direct Call

  - Trunk Data -> Trunk Route Base All Option -> VOIP OG Method -> [Direct Call]

  - Trunk Data -> Trunk Route Base All Option -> ISDN Enbloc OG Mode -> [YES]

Interaction

- ITGC card is a must.

- Recommendations:

  - ITU-T, H.323 Ver3  “Packet-based multimedia communications systems”

  - ITU-T, H.245 Ver5  “Control protocol for multimedia communication”

  - ITU-T, H.225.0 Ver3  “Call signaling protocols and media stream packetization for packet-based multimedia communication systems”

Hardware

- LAN must be configured between ITGC and IP part.