LG PBX System

Software Technical Information

STI-XXXX

SEP 15, 2001

| Title | #### Caller ID - ASLC |
| --- | --- |
| System | CS1000 |
| Abstract | Function to provide in FSK mode a caller ID corresponding to an incoming call to analog SLT |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 09.15.2001 | Y.H Lee, S.H Jin | First issue. |

Table of Contents

Title : Caller ID	2

Functional Description	2

Operation Procedure	2

Conditions	2

Data Generation	2

Interaction	2

Hardware	2

Title : Caller ID

Functional Description

This function is to provide a caller ID corresponding to an incoming call to analog SLT, in FSK mode that is used between the first ring and the second.

[Service Type]

- Incoming Call From Extension : The calling extension number is presented.

- Incoming Call From Outside To SLT User :  The outside caller’s ID, which is received on PSTN, is presented.

Operation Procedure

- [Sequence of Providing Caller ID to Called SLT]

- Set up a trial incoming call from an extension or a DIL or DISA trunk to a SLT user.

- Select an idle port using a FSKC card to transmit a ring to the SLT.

- Transmit a caller’s number in FSK mode on the SLT line through the FSKC card between the first ring and the second.

- See that the number is displayed on the Caller ID terminal connected to the SLT line.

- Observe that the called side response will lead to a successful call connection.

Conditions

- This service is to support “Data transmission during ringing” of ETSI Specification ETS300  659-1 (Part 1 : On hook data transmission) or BellCore Specification TR-NWT-000031(Calling Number Delivery) Type 1.

- An analog SLT must be chosen to use Caller ID.

- One FSKC card is available to provide FSK signal to 16 subscribers at a time. One more card to be added increases the number of subscribers to be called simultaneously.

Data Generation

- Generating FSKC Card

- System Local Board Information  Select Corresponding Slot  Click Right Mouse  Board Exchange  Select FSKC  Select FSKC Board  Click Right Mouse  Board Data Generation

- Choosing Option of ‘SLT Caller ID Display’ for SLT User

- Extension Information -> [CLI/CNI Display]

Interaction

- FSKC card is a must.

- A caller ID is presented only when a ring is transmitted with SLT in the state of On Hook. (ETSI ETS300  659-1 “Data transmission during ringing”, BellCore Specification TR-NWT-000031(Calling Number Delivery) Type 1 )

- Provided are Date, Time, and Caller Number. (Caller Name excluded)

- Data in FSK mode is transmitted to CPE (Customer Premises Equipment :caller ID display) between the first ring and the second.

Hardware

- CPE(Customer Premises Equipment :caller ID display) is a must at SLT side for use of this service.

- FSKC Card is a must in the system.