LG PBX System

Software Technical Information

STI-0086

SEP 20, 2001

| Title | #### Group Call |
| --- | --- |
| System | CS1000 |
| Abstract | Feature used to enable a director to give directions to every member belonging to a specified group by way of calling the group |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 09.20.2001 | C.Y Kim, C.H Kim | First issue. |

Table of Contents

Title : Group Call	2

Feature Description	2

Operation Procedure	2

Conditions / Restriction	2

Data Generation	2

Interaction	2

Hardware	2

OMS Menu Description ( Based on OMS 1)	3

Title : Group Call

Feature Description

This feature enables a director to give directions to every member belonging to a specified group by way of calling the group.

Operation Procedure

[Group Call  One Way]

- Go off-hook (Lift the handset).

- Enter the feature code “*681”.

- Dial the group number.

- Press the “CONF” to start giving directions.

[Group Call – Conference ]

- Go off-hook (Lift the handset).

- Enter the feature code “*682”.

- Dial the group number.

- Press the “CONF” to start giving directions.

Conditions / Restriction

- A director must be specified per group in OMS.

- Directed parties cannot be more than 30 members per group in OMS.

- SLT users cannot be more than 20 members in a board per group in OMS.

- Use OMS to choose whether to re-call an extension when he is released on the way to receive directions(Note by VVV: on-hook use).

- 5. Note by VVV: DECT subscrieber cannot participate

Data Generation

- Specify a director in OMS

- Note by VVV: 3.1. Extension information- group call master

- Specify directed parties more than 30 members per group in OMS.

- Split SLT users so that they cannot be more than 20 members in a board per group in OMS.

- Choose whether to re-call an extension when he is released on the way to receive directions.

- 5. Note by VVV: 2.5. System ring parameters - group call ring

Interaction

- In case a director’s terminal is D-Tel in whose flexible button a party directed is registered, the director can judge the current status of the party directed on the button lamp indication as below.

  - When the bell rings at the party directed : The button lamp flashes on and off.

  - When the party directed answers the ring(goes off-hook) : The button lamp is turned on.

Hardware

- N/A

OMS Menu Description ( Based on OMS 1)

Below are the menus of OMS Group Call feature.

- Query Group Call Group

- Change Group Call Group Data

- Query Group Call Grouping

- Delete Group Call Grouping

- Add Group Call Grouping

[Sample Screen Description]  Menu 6.10 Group Call Group

Use Status per Group

Status of On-Hook Option

[Figure 1-1]

1. Query Group Call Group

Click the “Query” button in the above window to query information on the current group.

2.Change Group Call Data

Change On-Hook Use Option and Group Name per Group.

[Figure 1-2]

3.Query Group Call Grouping

Query information on subscribers per Group.

4.Add Group Call Grouping

Add subscribers per Group.

5.Delete Group Call Grouping

Delete subscribers per Group

[Operation Sequence]

- Query Group Call Group

  - Select 6.10 Group Call Group from the Menu to open its window as shown in the Figure 1-1

  - Click the “Query” button.

  - Check the data.

- Change Group Call Data

- 2.1 In the Group Call Group window, select an item from data you have queried and change it.

  - Click the “Change” button.

  - Click the “Send” button.

  - Check the data that is changed.

- Query Group Call Grouping

- 3.1 In the Group Call Group window, select a group.

- 3.2 Right-click on the selected group to bring up a popup menu.

- 3.3 Select ‘Group Call Grouping’ from the menu to bring up the Group Call Grouping window.

- Add Group Call Grouping

- 4.1 In the Group Call Group window, go down to the table and click a row to add a subscriber to the subscriber list. (The triangle indicator will move into the box in the leftmost of the row you selected.)

- 4.2 Enter the telephone number type and number in the fields of Tel Type of Subscriber, respectively.

- 4.3 Click the “Add” button.

- 4.4 Click the “Send” button.

- Delete Group Call Grouping

- 4.1 In the table of the Group Call Group window, select a subscriber you want to delete from the subscriber list.

- 4.2 Click the “Delete” button.

- 4.3 Click the “Send” button.

[Service Conditions]

- 1. Group is numbered from 1 ~ 30.

- 2. Up to 30 subscribers can be assigned to a Group.

[Contents]

[Item Description for Group Call Group ]

| Items | Contents |
| --- | --- |
| Group No | Group Number |
| Use Status | To indicate whether each Group includes subscribers |
| Group Name | To indicate the name per Group |

[Item Description for Group Call Grouping]

| Items | Contents |
| --- | --- |
| Group No | Group Number |
| Telno Type | Group Call subscriber’s telephone number type ( Extension / Trunk ) |
| Subscriber | Group Call subscriber’s telephone number |