LG PBX System

Software Technical Information

STI-0006

Jan. 20, 2001

| Title | #### Инструкция по установке платы VPM |
| --- | --- |
| System | CS1000 |
| Abstract | VP110009. |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 11.30.2000 | H.K Jung | First issue |
| 1.1 | 01.19.2001 | H.K Jung | First Revision |
| 1.1 | 05.10.2001 | H.K Jung | Second Revision |

Содержание

название : Установка платы VPM	3

Обзор	3

Основные определения VPM	3

Услуги речевой почты	3

Услуги автооператора.	3

Системные тоны	3

Услуги информационного меню	3

Token	4

Token Category1 (Fixed Token)	4

Token Category2 (Voice Tone and Company Greeting Token)	4

Token Category3 (Bulletin Token)	4

VPM Installation	5

Voice Prompt & Token List	5

VPM Board Setting In OMS.	14

S/W Installation	14

Service Type	22

VMS	22

Auto Attendant	32

Voice Tone	38

Voice Bulletin	42

Trouble Shooting Guide	47

Title: VPM Installation Guide

# Overview

VPM (Voice Processing Module) is a CS1000 in-built Module to provide such services as VMS (Voice Mail Service), Auto ATD (Auto Attendant), Voice Tone, Bulletin, etc using voice-announcing method.

## Terms Definition in VPM

Token: It is the saving place for recoding a voice.

Voice Prompt: It is consisted of several Tokens to play a recorded voice message according to the VPM scenario flow.

MOH: It is just played a recorded message instead of system tone including Music On Hold or a user’s announcing information in Token to caller simply, that is, MOH is used one token for above service without a scenario flow.

## Voice Mail Service

VMS is the initials of Voice Mail Service. This service is used for a caller to leave a voice message through the phone in system while the called extension is in the absence or the state of impossible termination. And then the extension can check his message left. Also it may raise telephone traffic by leading an incoming call to a message.

## Auto-Attendant Service.

This service provides the function of an automatic answer to an external call so that the call may be directed to a desired extension or the representative number of desired department. This function can supersede operator who does simple work and thus brings in cost-effectiveness in manpower and working time.

 Phone Book Service

When a call is terminated to Auto ATD, the caller presses DTMF Digit, which is the representative number of the desired department, following the voice menu.

The call is connected to the extension directly when the outside caller dials the extension number.

But, the call is connected to the following destination number corresponding to phone book index 0 to 9 if the outside caller does not dial any digit after pressing the first digit.

The destination can be assigned as follows:

	Auto attendant of the different department

	System speed dial number

	Calling group number

	QSIG number

	ACD group number

	Individual speed dial number of DKTU

The “Auto Attendant Service” is optional.

## Voice Tone

Voice Tone is used as voice announcement or music to supersede various tones (Hold, Split Tone, ACD Service Waiting Tone, etc) provided by tone PROM in the system. It can use multiple VPM port for the same announcement.

## Bulletin

The service is used for an extension to listen to in-house announcement through the phone. To do so, the extension must dial the representative number of the Bulletin service. However, the announcement can be recorded by only the designated extension.

# Token

The Token is a factor for constituted voice prompt that is used to VPM.

There are the categories according to service categories.

## Token Category1 (Fixed Token)

Category is used for voice announcements for VMS and Auto ATD (except Company Greeting), made up into Wav files. The files recorded with voice announcements by system operator are compiled and merged by LGE.

And the files after compiled and merged have to be loaded to the VPM module by system operator in other to do correctly work.

The recording time of one Token is limitless.

## Token Category2 (Voice Tone and Company Greeting Token)

Category 2 is used for the announcement of voice tone, Auto ATD-provided Company Greeting, and department information menu of Phone Book Service. These voice announcements are made up into a Wav files.

And the files have to play on the PC that is connected to VPM model with SIO and stereo cable in order to record the message at Token in VPM module. The recorded voice announcements can be confirmed / deleted under control of VPM module by SIO commend. (See. 4.3.2)

The recording time of one Token is limited to one minute.

## Token Category3 (Bulletin Token)

Category3 is for announcement of Bulletin Service and only pre-assigned extension can record/confirm/delete the voice announcement on the phone on the flow of the control scenario. (See.4.4.4)

The recording time of one Token is limited to one minute.

# VPM Installation

## Voice Prompt & Token List

The recording of Token Category1 and 2 is to load into Wav Files as per the table below, thus sending the file to LGE. LGE compiles and merges the received file into system files (vpidex.hex & vpdat.hex) and feeds it back to the system operator. The system operator uploads the system file from PC to VPM Card. (See 2.2.1.3 VPM File Loading)

Example of file name) RINGBACK.wav

### Voice Prompt - Menu List (This is a changeable.)

| File Name | Voice Announcement |
| --- | --- |
| RINGBACK | 3 seconds ring back tone(Not announcement) |
| BEEP10 | 1 second beep (Not announcement) |
| END30 | 30 seconds CGT (Not announcement) |
| VP100000 | Press “1” to leave a message, press “3” for subscriber’s service, press “4” for bulletin service. |
| VP100001 | Press “1” to leave a message or press “3” for subscriber’s service. |
| VP100002 | Press “1” to leave a message |
| VP100003 | Press “3” for subscriber’s service or press “4” for voice bulletin |
| VP100004 | Press “3” for subscriber’s service. |
| VP110000 | Sorry |
| VP110001 | Press an extension number |
| VP110002 | You will be connected shortly |
| VP110003 | Please leave a message after the beep tone and press the “*” button when you are finished. |
| VP110004 | Your message has been properly recorded |
| VP110005 | Thanks for your call |
| VP110006 | Your message has been cancelled |
| VP110007 | Please wait for a while |
| VP110008 | Please try again later |
| VP110009 | Press ‘0’ to connect reception clerk. |
| VP110010 | Press * key and extension number for direct connection. |
| VP130000 | Please press your mail box number. |
| VP130001 | Please press 4 digits password |
| VP130002 | Press “1” to listen your message or press “3” for voice mail service |
| VP130003 | Press “0” to listen this menu again or press “*” to quit |
| VP131000 | You have |
| VP131001 | messages. |
| VP131002 | You have 1 message. |
| VP131003 | This message has been recorded at |
| VP131004 | Press “1” to listen to the next message, press “2” to listen again, press “8” to delete the message. |
| VP131005 | The message has been deleted |
| VP131006 | There are no more messages in your mail box. |
| VP131007 | There is no message in your mail box. |
| VP131008 | We’re sorry, the mailbox is full. No more messages can be recorded. |
| VP131009 | message |
| VP131010 | messages |
| VP131011 | Press “0” to return to top menu or press “*” to quit. |
| VP132201 | Please record greetings after beep and press “*” when you are done. |
| VP132202 | The greeting has not been registered. |
| VP133000 | Please press “1” to change the password and press “2” to check your greeting message. |
| VP133001 | Please press “1” to change the password and press “2” to check your greeting and press “3” to register for “Absence”. |
| VP133002 | Press “0” to return to previous menu or press “*” to quit |
| VP133101 | Press a new password of 4 digits. |
| VP133102 | The change has been confirmed. |
| VP133200 | Press “1” to register, press “2” to confirm, press “8” to delete |
| VP133300 | Press “1” to register, press “2” to cancel |
| VP133301 | The registration has been completed. |
| VP133302 | The cancellation has been completed. |
| VP140000 | This is the bulletin service |
| VP140001 | There is no message for bulletin |
| VP140002 | Press “1” to record or press “2” to confirm or press “8” to delete. |
| VP14004 | Press “*” to quit |
| ABS001 | The person doesn’t answer your call. |
| ABS002 | The line is busy. |
| ABS003 | The person cannot answer your call. |
| ABS004 | The person is not in right now. |
| ABS005 | The person is out to lunch. |
| ABS006 | The person is on vacation. |
| ABS007 | The person is out of the office on business. |
| ABS008 | The person is in office, but doesn’t be connected. |
| ABS009 | The person is in a meeting. |
| ABS010 | The person has left for the day. |
| ABS011 | The person is away on business. |
| ABS012 | You have pressed the unavailable number. |
| ABS013 | The number cannot be called. |
| Company Greeting 01*Note) | This is LG Information & Communication Company. |
| Holiday Announcement 02*Note) | Today is the holiday of my company. |
| Working hours Announcement 03*Note) | LG Information & Communication, LGIC working hours are from 9AM to 6PM |
| Phone Book Menu Announcement 04*Note) | Press one for Sails team, two for Customer Service team, three for Engineering team, or zero for information |

*Note) The Tokens is for system operator to record in VPM.

How to record: See VPM Installation Manual “4.3.2”

### Token - Error messages

| File Name | Voice Announcement |
| --- | --- |
| E00001 | You have pressed a wrong number |
| E00002 | Please find out the instructions and try to call again later |
| E00003 | Your request has been accepted, please wait for a while |
| E00004 | Your request has been delayed. Please try again later. |
| E00005 | The system requires a maintenance service please try again later |
| E00006 | Sorry, you cannot be connected due to the overload of call traffic. |
| E00007 | The system is down, please try again later |
| E00008 | Please try to call again after rechecking your password |
| E00009 | You have entered the wrong data |
| E00010 | Someone is listening to the message; you cannot delete it now. |

### Token - Date (month)

| File Name | Voice Announcement | File Name | Voice Announcement |
| --- | --- | --- | --- |
| SOLAR | Solar calendar | LUNAR | Lunar calendar |
| MON00 | Month |  |  |
| MON01 | January | MON07 | July |
| MON02 | February | MON08 | August |
| MON03 | March | MON09 | September |
| MON04 | April | MON10 | October |
| MON05 | May | MON11 | November |
| MON06 | June | MON12 | December |

### Token - Date (day)

| File Name | Voice Announcement | File Name | Voice Announcement |
| --- | --- | --- | --- |
| DAY00 | Day | DAY16 | Sixteenth |
| DAY01 | First | DAY17 | Seventeenth |
| DAY02 | Second | DAY18 | Eighteenth |
| DAY03 | Third | DAY19 | Nineteenth |
| DAY04 | Fourth | DAY20 | Twentieth |
| DAY05 | Fifth | DAY21 | Twenty first |
| DAY06 | Sixth | DAY22 | Twenty second |
| DAY07 | Seventh | DAY23 | Twenty third |
| DAY08 | Eighth | DAY24 | Twenty fourth |
| DAY09 | Ninth | DAY25 | Twenty fifth |
| DAY10 | Tenth | DAY26 | Twenty sixth |
| DAY11 | Eleventh | DAY27 | Twenty seventh |
| DAY12 | Twelfth | DAY28 | Twenty eighth |
| DAY13 | Thirteenth | DAY29 | Twenty ninth |
| DAY14 | Fourteenth | DAY30 | Thirtieth |
| DAY15 | Fifteenth | DAY31 | Thirty First |

### Token - Time (hour)

| File Name | Voice Announcement | File Name | Voice Announcement |  |
| --- | --- | --- | --- | --- |
| AM | a.m | PM |  | p.m. |
| HH00 | Zero |  |  |  |
| HH01 | One | HH13 |  | One |
| HH02 | Two | HH14 |  | Two |
| HH03 | Three | HH15 |  | Three |
| HH04 | Four | HH16 |  | Four |
| HH05 | Five | HH17 |  | Five |
| HH06 | Six | HH18 |  | Six |
| HH07 | Seven | HH19 |  | Seven |
| HH08 | Eight | HH20 |  | Eight |
| HH09 | Nine | HH21 |  | Nine |
| HH10 | Ten | HH22 |  | Ten |
| HH11 | Eleven | HH23 |  | Eleven |
| HH12 | Twelve | HH24 |  | Twelve |

### Token - Time (minute)

| File Name | Voice Announcement | File Name | Voice Announcement |
| --- | --- | --- | --- |
| MM00 | 0 |  |  |
| MM01 | 1 | MM31 | 31 |
| MM02 | 2 | MM32 | 32 |
| MM03 | 3 | MM33 | 33 |
| MM04 | 4 | MM34 | 34 |
| MM05 | 5 | MM35 | 35 |
| MM06 | 6 | MM36 | 36 |
| MM07 | 7 | MM37 | 37 |
| MM08 | 8 | MM38 | 38 |
| MM09 | 9 | MM39 | 39 |
| MM10 | 10 | MM40 | 40 |
| MM11 | 11 | MM41 | 41 |
| MM12 | 12 | MM42 | 42 |
| MM13 | 13 | MM43 | 43 |
| MM14 | 14 | MM44 | 44 |
| MM15 | 15 | MM45 | 45 |
| MM16 | 16 | MM46 | 46 |
| MM17 | 17 | MM47 | 47 |
| MM18 | 18 | MM48 | 48 |
| MM19 | 19 | MM49 | 49 |
| MM20 | 20 | MM50 | 50 |
| MM21 | 21 | MM51 | 51 |
| MM22 | 22 | MM52 | 52 |
| MM23 | 23 | MM53 | 53 |
| MM24 | 24 | MM54 | 54 |
| MM25 | 25 | MM55 | 55 |
| MM26 | 26 | MM56 | 56 |
| MM27 | 27 | MM57 | 57 |
| MM28 | 28 | MM58 | 58 |
| MM29 | 29 | MM59 | 59 |
| MM30 | 30 | MM60 | 60 |

### Token - Number

| File Name | Voice Announcement | File Name | Voice Announcement |  |
| --- | --- | --- | --- | --- |
| CN000 | Zero | ON000 |  | Zero |
| CN001 | One | ON001 |  | First |
| CN002 | Two | ON002 |  | Second |
| CN003 | Three | ON003 |  | Third |
| CN004 | Four | ON004 |  | Fourth |
| CN005 | Five | ON005 |  | Fifth |
| CN006 | Six | ON006 |  | Sixth |
| CN007 | Seven | ON007 |  | Seventh |
| CN008 | Eight | ON008 |  | Eighth |
| CN009 | Nine | ON009 |  | Ninth |
| CN010 | Ten | ON010 |  | Tenth |
| CN011 | Eleven | ON011 |  | Eleventh |
| CN012 | Twelve | ON012 |  | Twelfth |
| CN013 | Thirteen | ON013 |  | Thirteenth |
| CN014 | Fourteen | ON014 |  | Fourteenth |
| CN015 | Fifteen | ON015 |  | Fifteenth |
| CN016 | Sixteen | ON016 |  | Sixteenth |
| CN017 | Seventeen | ON017 |  | Seventeenth |
| CN018 | Eighteen | ON018 |  | Eighteenth |
| CN019 | Nineteen | ON019 |  | Nineteenth |
| CN020 | Twenty | ON020 |  | Twentieth |
| ONC00 | Zero |  |  |  |
| ONC01 | One |  |  |  |
| ONC02 | Two |  |  |  |
| ONC03 | Three |  |  |  |
| ONC04 | Four |  |  |  |
| ONC05 | Five |  |  |  |
| ONC06 | Six |  |  |  |
| ONC07 | Seven |  |  |  |
| ONC08 | Eight |  |  |  |
| ONC09 | Nine |  |  |  |
| ONC10 | Ten |  |  |  |
| ONC11 | Eleven |  |  |  |
| ONC12 | Twelve |  |  |  |
| ONC13 | Thirteen |  |  |  |
| ONC14 | Fourteen |  |  |  |
| ONC15 | Fifteen |  |  |  |
| ONC16 | Sixteen |  |  |  |
| ONC17 | Seventeen |  |  |  |
| ONC18 | Eighteen |  |  |  |
| ONC19 | Nineteen |  |  |  |
| ONC20 | Twenty |  |  |  |

## VPM Board Setting In OMS.

### VPM Board Generation

OMS [2.2 Local Board Information], follow the sequence:- [ⓡ Board Information view  Click VPM Board]  [Board Data Generation] [Extension Data Handling Plan Dialog Window][Extension Information]

Note) ⓡ ; Mouse-click using the right Button.

### Assignment of Service Representative Number and VPM Port

In OMS [6.4 Calling Group], [Representative Number] must be the one that is not used for any extension, existing in the Numbering Plan of [2.1 System Numbering PlanExtension Numbering Plan].

## S/W Installation

### Files Layout

| File Name | Description |
| --- | --- |
| ram.hex | Scenario Control of All VPM Service. |
| Codec.hex Codecknl.hex | Voice processing DSP file |
| Dtmf.hex Dtmfknl.hex | DTMF DSP file |
| Vpidex.hex & vpdat.hex | Token file to be used for voice tone |

### File Loading

1) Set VPM Board.

1.1) Connect between the Com port of PC and the SIO of VPM Faceplate with RS232C Cable.

1.2) Execute the Program like Hyper Terminal for SIO communication.

Setting  Baud Rate 19200. No Parity, Data bit 8, Stop Bit 1

2) Assign IP Address of VPM to use Network.

2.1) Disable Auto Boot mode by defining “0” setting value.

In the connection of PC and VPM by RS-232C, open HyperTerminal. And then, type the command below.

| Command | AB 0 : Autoboot Disable AB 1 : Autoboot Enable AB : Verify Autoboot mode. |
| --- | --- |
| VPMBUG $ ab [Enter] AutoBoot mode[DISABLE] Usage: ab [1 ¦ 0] VPMBUG $ VPMBUG $ ab 0 [Enter] |  |

2.2) Reset the board. If a dialog box is displayed, press any key to go into [M]odify mode. If you do not want modification, press [Enter] key to go over to the next item

2.3) Set START-UP MODE to “4. Run the VPM Boot ROM”.

How should the board boot? 1. pROBE+ stand-alone mode 2. pROBE+ waiting for host debugger via serial connection 3. pROBE+ waiting for host debugger via a network connection 4. Run the VPM Boot ROM Which one do you want? [4] 4 [Enter]

2.4) NETWORK INTERFACE PARAMETERS assign IP Address of VPM.

Do you want a LAN interface? [Y] y [Enter] This board's LAN IP address(0.0.0.0 = RARP)? [165.243.134.57] 165.243.134.11 [Enter] Subnet mask for LAN (0 for none)? [255.255.255.0] 255.255.255.0 [Enter] Should there be a default gateway for packet routing? [N] n [Enter]

2.5) HARDWARE PARAMETERS setting.

Define VPM ethernet address setting value. If IP is 165.243.134.57, information on Class A only must be excluded and the decimal value 243.134.57 must be converted into hexa decimal

Do you want to change the board's Ethernet address? [N] y [Enter] Bytes 0 through 2 are always 08:00:3E What should byte 3 be? [96] F3[Enter] What should byte 4 be? [3D] 86[Enter] What should byte 5 be? [45] 0B[Enter]

2.6) CPU delay time setting

Define delay time value in second from dialog box display to ROM code execution

How long (in seconds) should CPU delay before starting up? [1] 1 [Enter]

After each setting of the above items, the dialog box is displayed. Verify the changed values. If they are correct, enter C. If you want to modify the settings, enter M to repeat the procedure.

2.7) To verify inputted values.

If the values in the dialog box are not changed after the modification, you must check and see whether or not something wrong is with NVRAM.

VPMBUG $ ab [Enter] AutoBoot mode[DISABLE] Usage: ab [1 ¦ 0] VPMBUG $ VPMBUG $ ab 1 [Enter] VPMBUG $ ab [Enter] AutoBoot mode[ENABLE] Usage: ab [1 ¦ 0] VPMBUG $

3) Create a random directory named VPM on the PC.

4) Copy out LG-provided VPM Software file and paste it in the VPM Directory.

5) Install LG-provided TFTP Program.

6) Open TFTP Server window on the PC.

7) Select Configure  of TFTP in the menu window.

8) Assign a directory to load with a file.

99) Select START in the TFTP server window.

10) TFTP Uploading is ready.

For reference, TFTP Client is embedded in VPM Board.

11) Connect PC and VPM by SIO

Setting  Baud Rate 19200. No Parity, Data bit 8, Stop Bit 1.

12) Connect VPM and Network by LAN Cable.

13) Check IP Address.

| Command | IP |
| --- | --- |
| VPMBUG $ ip [Enter] |  |

14) File Loading.

Auto Boot Disable.

Case 1) Upload at a time ram.hex, codec.hex, codecknl.hex, dtmf.hex, dtmfknl.hex, vpidex.hex, vpdat.hex, etc. to use in VPM  (The loading of all files at a time is possible in the Disable mode of Autoboot.)

| Command | UPA PC_IP_Address ; PC opened Tftp Application. If you try to type “upa” command to load all VPM files, Auto Boot mode must set to [Disable]. |
| --- | --- |
| VPMBUG $ upa 165.243.134.10 [Enter] Uploading "ram.hex" from HOST 165.243.134.10 / TFTP download completed Uploading "codec.hex" from HOST 165.243.134.10 / TFTP download completed Uploading "dtmf.hex" from HOST 165.243.134.10 / TFTP download completed Uploading "vpidx.hex" from HOST 165.243.134.10 / TFTP download completed Uploading "vpdat.hex" from HOST 165.243.134.10 / TFTP download completed Uploading "codecknl.hex" from HOST 165.243.134.10 / TFTP download completed Uploading "dtmfknl.hex" from HOST 165.243.134.10 / TFTP download completed VPMBUG $ |  |

Case2) Individual File Loading (possible in both Enable and Disable mode of Autoboot)

| Command | UPLD File_Name PC_IP_Address.; PC opened Tftp Application. |
| --- | --- |
| VPMBUG $ upld ram.hex 165.243.134.10 [Enter] Uploading "ram.hex" from HOST 165.243.134.10 \ TFTP download completed VPMBUG $ |  |

Verification) Verify the files uploaded are normally completed using the upldt command. The uploaded files must be included in ACTive area.(in the Disable mode of Autoboot )

| Command | UPLDT |
| --- | --- |
| VPMBUG $ upldt [Enter] >>>>>>>>>>>>>>>>>> Upload Table Information <<<<<<<<<<<<<<<<<<< ------+------------+----------+----------+--------------------- ACT ¦ ram.hex ¦ 20000000 ¦ 2008F438 ¦ 2000/10/12 11:24:12 ¦ codec.hex ¦ 203C0000 ¦ 203C77E0 ¦ 2000/10/12 11:24:22 ¦ dtmf.hex ¦ 203E0000 ¦ 203E3FC0 ¦ 2000/10/12 11:24:28 ------+------------+----------+----------+--------------------- STBY ¦ ram.hex ¦ 201E0000 ¦ 0 ¦ 2000/10/12 10:02:58 ¦ codec.hex ¦ 207C0000 ¦ 0 ¦ 0000/00/00 00:00:00 ¦ dtmf.hex ¦ 207E0000 ¦ 0 ¦ 2000/10/12 16:04:52 ------+------------+----------+----------+--------------------- VOICE¦ vpdat.hex ¦ 20420000 ¦ 2047B9B4 ¦ 2000/10/12 11:25:49 PROMPT¦ vpidx.hex ¦ 20400000 ¦ 204018F0 ¦ 2000/10/12 11:24:33 ------+------------+----------+----------+--------------------- DSP ¦codecknl.hex¦ 203A0000 ¦ 203A01C0 ¦ 2000/10/12 11:25:53 ¦ dtmfknl.hex¦ 203B0000 ¦ 203B01C0 ¦ 2000/10/12 11:25:57 ------+------------+----------+----------+--------------------- |  |

Operating) After completing the file upload, enable Autoboot and reset the board. Aautoboot setting and its verification is as follow:-

| Command | AB 0 : Autoboot Disable AB 1 : Autoboot Enable AB : Verify Autoboot mode. Note) Please make a reset VPM after changing Auto-boot always to adapt actual operation. |
| --- | --- |
| VPMBUG $ ab 1 [Enter] VPMBUG $ ab [Enter] AutoBoot mode[ENABLE] Usage: ab [1 ¦ 0] VPMBUG $ |  |

Auto Boot Enable.

When ACTive area includes a running files and Autoboot is in the enable mode, the prompt indicates "VPM_Debug>" after resetting the VPM. In this mode, "upa"command cannot be used because VPM application is already running.

Accordingly in Autoboot Enable mode, only each file can be uploaded one by one. Each of ram.hex, codec.hex, or dtmf.hex file goes into STBY(stand-by) mode, thus not affecting the current running files

| Command | UPLDT ; To verify current file status. |
| --- | --- |
| VPM_Debug> upldt [Enter] >>>>>>>>>>>>>>>>>> Upload Table Information <<<<<<<<<<<<<<<<<<< ------+------------+----------+----------+--------------------- ACT ¦ ram.hex ¦ 20000000 ¦ 2008F438 ¦ 2000/10/12 11:24:12 ¦ codec.hex ¦ 203C0000 ¦ 203C77E0 ¦ 2000/10/12 11:24:22 ¦ dtmf.hex ¦ 203E0000 ¦ 203E3FC0 ¦ 2000/10/12 11:24:28 ------+------------+----------+----------+--------------------- STBY ¦ ram.hex ¦ 201E0000 ¦ 0 ¦ 2000/10/12 10:02:58 ¦ codec.hex ¦ 207C0000 ¦ 0 ¦ 0000/00/00 00:00:00 ¦ dtmf.hex ¦ 207E0000 ¦ 0 ¦ 2000/10/12 16:04:52 ------+------------+----------+----------+--------------------- VOICE¦ vpdat.hex ¦ 20420000 ¦ 2047B9B4 ¦ 2000/10/12 11:25:49 PROMPT¦ vpidx.hex ¦ 20400000 ¦ 204018F0 ¦ 2000/10/12 11:24:33 ------+------------+----------+----------+--------------------- DSP ¦codecknl.hex¦ 203A0000 ¦ 207001C0 ¦ 2000/10/12 11:25:53 ¦ dtmfknl.hex¦ 203B0000 ¦ 207101C0 ¦ 2000/10/12 11:25:57 |  |
| Command | UPLD File_Name PC_IP_Adress ; PC opened Tftp application |
| VPM_Debug> upld ram.hex 165.243.134.10 [Enter] Uploading "ram.hex" from HOST 165.243.134.10 \ TFTP download completed VPM_Debug> upldt [Enter] >>>>>>>>>>>>>>>>> Upload Table Information <<<<<<<<<<<<<<<< ------+-----------+----------+----------+--------------------- ACT ¦ ram.hex ¦ 01000000 ¦ 1089864 ¦ 2000/10/01 09:10:23 ¦codec.hex ¦ 013C0000 ¦ 13DFF1E ¦ 2000/10/01 09:11:07 ¦ dtmf.hex ¦ 013E0000 ¦ 13E7FC0 ¦ 2000/10/01 09:11:24 ------+-----------+----------+----------+---------------------- STBY ¦ ram.hex ¦ 011E0000 ¦ 1269864 ¦ 2000/10/05 12:44:10 ¦codec.hex ¦ 017C0000 ¦ 0 ¦ 0000/00/00 00:00:00 ¦ dtmf.hex ¦ 017E0000 ¦ 0 ¦ 0000/00/00 00:00:00 ------+-----------+----------+----------+---------------------- VOICE ¦ vpdat.hex ¦ 01420000 ¦ 14914A8 ¦ 2000/10/01 09:12:56 PROMPT¦ vpidx.hex ¦ 01400000 ¦ 1401B18 ¦ 2000/10/01 09:11:45 ------+-----------+----------+----------+----------------------- DSP ¦codecknl.hex¦ 20700000 ¦ 207001C0 ¦ 2000/10/17 11:25:53 ¦ dtmfknl.hex¦ 20710000 ¦ 207101C0 ¦ 2000/10/17 11:25:57 ------+-----------+----------+----------+----------------------- |  |

| What Auto Boot means |  |
| --- | --- |
| Disable; Non-Operating The Disable mode is used for various setup-data for VPM installation before VMS service. The file uploaded from PC is stored in the Active table in VPM. If the Data setup is completed, you must enable Autoboot for VPM service and reset it. Enable; Operating VMS services mode. If you upload a file, it will be stored in the Standby table in VPM that does not affect the current running VMS. If you want to re-upload the uploaded file to Active Table in VPM, you must type the command below and then reset VPM. |  |
| Command | CHGM |
| VPMBUG $ chgm file Name [Enter] ; file Name = ram.hex /codec.hex/dtmf.hex |  |

15) Stop TFTP.

16) Initialize VPM Board. The installation of VPM Board is completed

# Service Type

## VMS

### Description

VMS is used to perform Mail Box Service, Mail Access Service, Call Forwarding Service.

### Service Method

      - Mail Box Service

Service Type

Three types of Mail Box Service are provided as below.

1.1) Type 1

An extension registered in Mail Box dials VMS representative number [6.4 calling GroupService type; MAILACC] to check and listen a message saved in the Mail Box assigned by his own ID.

1.2) Type 2

An extension registered in Mail Box dials VMS representative number [6.4 calling GroupService type; MAILACC] and connects to VMS for recording his voice information.

When the extension is in an answer-impossible state like “Busy, Absent, DND, or No-answer” in case of an outside subscriber call, he can use VMS call forwarding function by selecting a code(see 1.3.1) suitable for his present state through the phone. And the outside subscriber can hear the voice information saved by extension.

1.3.1) VMS Call forwarding registration by extension

SLT / D-Tel

OMS [2.1 System Numbering Plan  Feature Plan]

| Description | Code Use |
| --- | --- |
| All (unconditional) Call Forwarding registration. | (*527) |
| Busy Call Forwarding registration | (*528) |
| Busy or No Answer Call Forwarding registration | (*52*) |
| Busy Call Forwarding registration | (*529) |
| Note) How to select a code for VMS Call forwarding in the above SLT Ex) Off-hook  *52X  On-hook ; X = 7,8,9,* |  |

D-Tel

| Description | Code Use |
| --- | --- |
| All (unconditional) Call Forwarding registration. | MON + DND/FWD + 7 |
| Busy Call Forwarding registration | MON + DND/FWD + 8 |
| Busy or No Answer Call Forwarding registration | MON + DND/FWD + * |
| Busy Call Forwarding registration | MON + DND/FWD + 9 |
| Note) How to select a code for VMS Call forwarding in the above SLT Ex) Off-hook or MON  DND/FWD  X : X = 7,8,9,* |  |

1.4) Type 3

This type is a service for an extension registered in Mail Box to dial VMS representative number so that they may change his password by dialing on the phone.

      - Mail Access Service

Service for an outside subscriber to leave his voice message when the extension is in the state of being impossible to answer the outside subscriber’s call.

1.1) Whole Capacity:

The Mail Access Service is capable of 254 Tokens of 60-second voice message in Token Category1. Assuming that Bulletin service is, the number of usable token will be reduced to 244.

1.2) Capacity of Mail Box ID for an extension

An extension registered in Mail Box can have ten voice messages.

- 2) To indicate message for D-Tel, SLT.

- 2.1) How to indicate Message arrival.

- D-Tel

- Indicated on LCD

MSG EXT(0) TRK(0) VM(1)

LINE 11018       06:45 PM

- VM(1): Currently a voice mail is saved in Mail Box.

- SLT

- Message Wait Ring gives to SLT at the defined interval.

- OMS

- To adjusting the interval.

- [31. Extension Information  Extension Characteristics SLT MW No Ring]

- 2.2) How to check message in Extension

- 2.2.1) Check the message at D-Tel.

- A. Check the VM counter on LCD.

- B. Press MOM Button.

- C. Press CallBK Button.

- D. Follow VMS voice message for input.

- Check at SLT.

- Case1) When using Message Ring :

- OMS [Extension Data Management3.1 Extension

- Information=>Extension Characteristics->7.SLT M/W No

- Ring]

- Listen to Message Ring

- Go into Off-Hook mode and press *7 or VMS representative number.

- Follow VMS voice Message for input

- Note) In case of Message Ring SLT in the Check mode, access Mail Box to delete the message, or else there will be rings at a certain interval.

- Case2) when not using Message Ring

- Dial VMS(MAILACC) representative at the extension.

- Follow VMS voice Message for input.

- Check Recorded Message.

- Note) Make sure whether or not Single Line Telephone Message Ring

- [3.1 Extension InformationExtension Characteristics7.SLT M/W No Ring]

      - Call Forward Service

This service is used for an outsider subscriber to call a desired extension registered in Mail Box, who is in the answer-impossible state.

Case1) When an outside subscriber calls an extension through Auto Attendant

Case2) When an outside subscriber calls in a direct way an extension.

The above both cases are given the following result:-

The outside subscriber hears the voice information, which the extension has assigned for his present state, provided by VMS. And then the subscriber may leave his message,

Note1) Exception to call forwarding) When an extension has set his phone to VMS call forwarding using feature code or function button (D-Tel) and an outside subscriber is directed to the extension. (See 4.1.2.1 Mail Box Service .12) Type 2)

Note2) Incase I-Phone calls to registered extension, this service don’t allow.

### Service Scenario

##### Mail Access Service

Mail Access

Message Wait

Answer Code ?

Mail box number exist ?

Password OK ?

C1

Retry

“Please press your mail box number.”  (VP130000)

Y

N

Input DTMF

“Please press 4 digits password.”  (VP130001)

Input DTMF

Y

“You have pressed the unavailable number.” (ABS012)

N

Y

N

“You have pressed the unavailable number..” (ABS012)

“You have   (VP131000 )

XX        (CN001, … )

messages.”  (VP131001 )

Input DTMF

Are there a mail ?

“There is no message in your

mail box.” (VP131007)

“Press ‘1’ to listen your message or press ‘3’ for voice mail service.”

(VP130002)

“Press ‘0’ to listen this menu again or press ‘*’ to quit.”   (VP130003)

1

3

Message number > 0

Y

Y

N

N

C1

C2

C3

End

*

0

C4

“ ( ex. 1st )”  (ON001, …)

Play recorded message.

“This message has been recorded at ( time ).” (VP131003)

Input DTMF

“The message has been deleted.” (VP131005)

Next Mail ?

“There are no more messages in your mail box.”     (VP131006)

“Press ‘0’ to return top menu or press ‘*’ to quit.”   (VP131011)

Y

N

“Press “1” to listen to the next message,

press “2” to listen again,

press “8” to delete the message. ” (VP131004)

Listen Next Mail

Mail Access again

Mail cancel

1

8

MONTH XX

DAY XX

HH XX

MM XX

C3

2

End

C4

*

0

Mail Box Service

“Please press “1” to change a password and press “2” to check greetings message. ” (VP133000)

“Press”0” to return to previous menu or press”*” to quit”

(VP133002)

“Press a new password

of 4 digits.”

(VP133101)

1

2

Input Password

“The change has been

confirmed”(VP133102)

DTMF Input

C2

“Press “1” to register, press “2” to confirm, press “8” to delete. ” (VP133200)

“Press”0” to return to previous menu or press”*” to quit”(VP133002)

“Please record

greeting after beep

and press ‘*’ when

you are done.”

(VP132201)

Listen Greeting

Greeting cancel

“The Message has been

deleted.” (VP131005)

1

2

8

Greeting exist?

Y

N

“The greeting has not

been registered.”

(VP132202)

C2

C2

End

*

0

End

*

0

Forward Call Service

Forward Call

Individual Greeting ?

End

Busy : “Sorry, The line is busy” (ABS002)

No answer : “The person doesn’t answer your call.” (ABS001)

DND : “The person cannot answer your call.” (ABS003)

Absence - out to lunch  : “The person is out to lunch” (ABS005)

- on vacation   : “The person is on vacation” (ABS006)

- out of office  : “The person is out of the office on business” (ABS007)

- in office : “The person is in office, but doesn’t be connected.” (ABS008)

- in a meeting  : “The person is in a meeting.” (ABS009)

- at home         : “The person has left for the day”  ( ABS010)

- in a business trip : “The person is away on business.”  (ABS011)

Listen

Individual Greeting

Y

N

“Please leave a message after beep and

press “*” when you are done.”(VP110003)

Recording + ‘*’

“Your message has been properly recorded (VP110004)

Thanks for your call.” (VP110005)

VM Message Wait Registration

“beep~“ (BEEP10)

HOTKEY : ‘0’

In this step, if user

press ‘0’, we enter

Mai l Access call.

VM : Voice Mail

### Data Management

How to generate Code to connect to VMS

1) To access Voice Mail Box

OMS [6.4 Calling Group] (Refer to the OMS Part to system Manual)

- Input Representative Number

- Select Service Type  MAILACC

- Select Ring Ser Method  First or UCD

- Select Busy & DND Option  Send Busy Tone.

- No Answer Option  Send Busy Tone.

- Calling Group Service Extension  VPM Board의 Port Telephone Number.

2) How to assign extension Mail Box ID

OMS [15.4 Mail Box Management]  (Refer to OMS part in System Manual)

2.1) Built-in VPM

Define OMS [15.4 Mail Box ManagementVPM Check & Slot √ / MB ID/ Unit/ Recorded Msg Counter]

MB ID must be assigned with the same number as the extension.

- 2.2) Separate VMS device (See OMS Part of System manual for further)

Define OMS [15.4 Mail Box ManagementExternal VMS √ / MB ID(Mail Box ID)/ Password / Unit / External TelNo / Recorded Msg Counter]. (See OMS VMS Part of System manual for further)

MB ID must be assigned with the same number as the extension.

3) Voice Mail Option Definition

Define OMS [15.5 Voice Mail Option  Retry Count (In case Vacant Number)] Value at 2 or upper.

- OMS [15.5 Voice Mail Option CFWD to VM (Busy)] is assigned to ‘YES’.

- [15.5 Voice Mail Option CFWD to VM (No Answer)] is assigned to ‘YES’

4) Message Ring for Single Line Telephone Definition.

4.1) Interval Message Ring Parameter Handling

OMS [2.5 MSG Ring] to transfer to SLT subscriber

4.2) Whether or not to use Interval Message Ring to transfer to SLT subscriber.

OMS [3.1 Extension InformationExtension Characteristics7.SLT M/W No Ring] √ = No Ring.

### Correlation

Call processing order is as follows:-

1st  check point

OMS [15.5 Voice Mail Option CFWD to VM (Busy)/(No Answer)]

2nd  check point

OMS [4.9 Trunk route base day/night incoming option]

3rd check point

Call Forwarding code(*52X).

So if [15.5 Voice Mail Option CFWD to VM (Busy)/(No Answer)] is not assigned, the processing order is as follows:-

1st check point

OMS [4.9 Trunk route base day/night incoming option]

2nd check point

Call Forwarding code(*52X).

Or if OMS [4.9 Trunk route base day/night incoming option] is not assigned , the processing order is as follows:-

1st check point

OMS[15.5 Voice Mail Option CFWD to VM (Busy)/(No Answer)]

2nd

Call Forwarding code(*52X).

Note) If ATD was not installed in system, the clerk (ATD) call on scenario will access to System PNA extension. [16.4 System PNA].

## Auto Attendant

### Description

Auto Attendant in VPM is used for the service to automatically respond to an outside subscriber call by giving the defined voice information so that the outside subscriber may follow the information to be directed to the dialed extension or ATD.

And if Phone Book service is provided, the subscriber can be directed to the representative number of the selected department.

Token ID 01~03 of Token Category2 are used for the service of information on company, holiday, and working hours. And Token ID 04 is for Phone Book Service.

### Service Method.

If the calling group number is called by outside line, the announcement is sent out in accordance with day and night mode.

The related number is called by dialing the station number.

The related service is activated in accordance with day and night index if not dialing in the certain time period after dialing the first digit.

The service in menu is as follows:

 Another CCR Table

 ACD number

 Calling group number

 System speed dial code

 Individual speed dial

 QSIG number

 Internal paging

 Attendant number

CCR1

Day

Night

ANN1

ANN2

1

CCR2

2

CCR3

3

4

1

2

3

4

CCR2

Day

Night

ANN1

ANN2

1

CCR2

2

CCR3

3

4

1

2

3

4

### Service Scenario

Refer to the next pages.

Auto ATD Service

Idle?

Retry Counter=0?

DTMF=? (Note 1)

A1

End of Auto ATD Service

Night=?

Time Out=?

Extension No.=?

Note 1: The DTMF is including the station number, speed dial code, calling group number and attendant.

Note 2: The recording of the failure is as follows:

- Cause-Busy: “The line is busy” (ABS002)

- No Answer: “The person doesn’t answer your call.” (ABS001)

- Absent: “The person is not in right now.” (ABS004)

- DND: “The person cannot answer your call.” (ABS003)

- Vacant: “You have pressed the unavailable number.” (ABS012)

- IC Deny Class: “The number cannot be called.” (ABS013)

- Transfer EXT No Answer : “Sorry the person doesn’t answer your call.”(ABS001)

Night Announcement

Y

Dialing

N

Wrong Number

No Dialing

Connect to the desired number

“Sorry, the line is busy” (Note 2)

“Thank you for your call” (VP110005)

“You will be connected shortly” (VP110002)

“You have pressed the unavailable number” (ABS012)

“Your request has been delayed. Please try again later” (E00004)

N

“Thank you for your call” (VP110005)

Y

Available Phone Book Index

Y

N

Day Announcement

Routing according to CCR Index of day and night mode

Y

Y

Y

N

N

A1

Phone Book Service

A1

Idle?

Retry Counter=0?

End of Service

Phone Book Announcement

“Press 1 to connect Sales Section 1.”

“Press 2 to connect Sales Section 2.”

(Phone Book Index)

“Pres 3 to connect the desired person”

“Press 0 to connect reception clerk.”

Index 1

Index 2

Index n

      

1

2

3

0

“You will be connected shortly” (VP110002)

Wrong Number

No Dialing

“You have pressed the unavailable number” (ABS012)

“Your request has been delayed. Please try again later” (E00004)

Connect to the desired number

“Sorry, the line is busy” (Note 3)

“Thank you for your call” (VP110005)

N

“Thank you for your call” (VP110005)

Y

Y

N

### Data Management

1) To access Auto ATD

OMS [6.4 Calling Group]

- Input Representative Number

- Select Service Type  Auto ATD

- Select Ring Ser Method  First or UCD

- Select Busy & DND Option  Send Busy Tone.

- No Answer Option  Send Busy Tone.

- Calling Group Service Extension  VPM Board의 Port Telephone Number

2) Phone Book Service Yes or No

OMS [15.5 Voice Mail OptionVPM Phone book Service Use] -> YES

3) Phone Book Service Menu Digit number

OMS [15.5 Voice Mail OptionVPM Phone book Menu]

### Correlation

Note) If ATD was not installed in system, the clerk (ATD) call on scenario will access to System PNA extension. [16.4 System PNA].

## Voice Tone

### Description

Voice Tone can be used as voice announcement to supersede tones in system and as Hold Split Tone, ACD Service Waiting Tone, etc.

Add/change/Delete is run through VPM SIO port.

The ID 5~20 of Token category2 are capable of saving and playing changeable announcement, whenever necessary.

Caution1) The recording time of each Token is limited to one minute.

Caution2) Each VPM Card can be assigned max 8 ports for voice token. But if all ports are set for voice tone, VMS, Auto ATD, Bulletin, etc are unserviceable.

Caution3) Token ID 1~4 are used only for “Company Greeting”, “Holiday Announcement”, “Working Hours Announcement”, and “Phone Book Menu Announcement” in Auto ATD / VMS Scenario. Therefore they must play the announcement with the same intention as the descriptions in the above article 3.1 Token List

### Installation

- Token Recording and Confirmation

Preparation: PC (WinAmp S/W + Hyper Thermal + Recorded wav File + Sound card), RS232C Cable, Stereo cable and Jack, Headset

- Create user-defined Wav File for Voice Tone.

- Record voice tone in its Token

2.1) Recording environment.

LAN

MIC

SIO

SPK

VPM

Stereo Cable

SPK Out

COM Port

- Environment Setup Procedure

3.1) MIC : According to the figure of recording environment as above, connect audio output of PC sound card and input of VPM faceplate by a stereo cable.

3.2) SPK : Connect output of VPM faceplate to Headset.

3.3) SIO : Connect VPM SIO and PC by SIO and run Hyper Terminal.

Setting  Baud Rate 19200. No Parity, Data bit 8, Stop Bit 1.

2. Recoding Processor for Voice Tone or user’s Announcement.

Create user-defined Wave file for Voice Message. (See Application Program Manual)

Run the wave file-playing program (ex: WinAmp) on the PC connected with VPM by Stereo Cable and excute the command below using Hyper Terminal in the connection of VPM and PC by SIO.

| Comment | VPIL ; Change to Token Category2 Prompt |  |
| --- | --- | --- |
| VPM_Debug> Vpil VPIL> |  |  |
| ###### Comment | MOHR [Token index(1~20)]<"start" [time(1~60 sec)]¦"stop"> Input Token ID to record and recording time. |  |
| VPIL>mohr 5 start 10 [Enter] ; Token ID 5 / Recoding Time 10sec. VPIL> S_SCTER[206D0000] Done with[0x0] ; The message is displayed after the defined Recoding Time 10seconds and since then the message is not recorded. Note1) Recording Time might be the time displayed on wave file player plus the margin of 2 seconds. Note2) User may not be permitted to randomly use Voice Tone 1 ~ 4 for announcement other than the below-assigned ones. Voice Tone 1 : Company Information Announcement Voice Tone 2 : Holiday Announcement Voice Tone 3 : Working Hours Announcement Voice Tone 4 : Phone Book Service Announcement. Voice Tone 5 ~ 20 : User Recording Token. |  |  |
| Comment | MOHP [index(1~20)] <"start" / "stop"> Confirm the recorded voices in Headset connected to VPM, according Token ID. |  |
| mohp 5 start [Enter] ; Confirm voice information recorded in Token ID “5” |  |  |

### Service Method

Voice Tone and ACD Service.

Voice Tone : Use announcement recorded in a token, when necessary, instead of system tone. It runs as per OMS definition.

ACD : See ACD Manual.

### Data Management

Selection of token ID of voice tone to be substituted for system tone

OMS [15.1 Voice Token InformationTone Type  Voice Tone Token ID]

Select the item of tone type currently provided by system. Then type the ID of the token recorded by user definition to Voice Tone Token item.

Select Message channel recorded in Token ID of voice tone to be substituted for system tone.

OMS [15.2 VPM Token Information VPM Token ID  Recoding Time  Start PortEnd Port]

Verify the recording time (Message Playing Time) needed to be played in the VPM according to its VPM Token ID, and assign port range for its voice channel.

### Correlation

4 tokens are used for Auto ATD. Therefore 16 Tokens can be used for voice tone and announcement.

## Voice Bulletin

### Description

Voice Bulletin is a function for in-house members to check company announcement through the telephone.

The number of Bulletin token (Token category3) is 10 in all. The recording time of each token is limited to one minute

### Setup Procedure

Step1) Assign a representative number to each service type for an extension to use Bulletin in OMS. [6.4 calling GroupService type; BULLETIN / TOKENCHG

Step2) Set Option to ‘Yes’ in [15.5 Voice Mail Option VPM Bulletin Service Use] using Bulletin Service.

Step3) Dial the TOKENCHG representative number assigned in OMS. Press “0” for the service of Bulletin Menu in the section of Input DTMF in TOKEN Change Scenario.

Step 4) According to Scenario Flow, extension assigned in OMS can record, change, and delete menu and information about Bulletin through the telephone.

(Recording Type; Announcement on pressing DTMF Digit 1 + Announcement on pressing DTMF Digit 2+ Announcement on pressing DTMF Digit 3 …………+ Announcement on pressing DTMF Digit N {N=1~9};. assign the services per Digit up to Max 9 )

Step 5) Call the BULLETIN representative number and then confirm the voice information of each item, after all the above 4).

Note) If the announcement of “There is no message for bulletin.” is displayed on calling Bulletin representative number, ensure that the voice information for the Menu of the number “0” is recorded.

### Service Method

Change Bulletin Token.

Only extensions eligible for Bulletin Token service can record/save/delete/change menu and information in the token for Bulletin service.

An extension or an outside subscriber may hear an announcement of the information recorded on item desired according to Scenario Flow using DTMF Digit.

### Service Scenario

(For example, )

“Press ‘1’ for new information announcement …….” , “press * to quit.” (VP140004)

Input DTMF

Bulletin Service

Play message.

DTMF OK ?

Y

N

“You have pressed a

wrong number.”(E00001)

“This is the bulletin service.”(VP140000)

**

Bulletin service can support max 10 token.

0    : bulletin menu

1 ~ 9 : bulletin message

If administrator record or delete bulletin message, he/she can use token change service.

Bulletin Menu.

Bulletin registration ?

Y

N

“There is no message for bulletin.”  (VP140001)

Token Change

“Please leave a message after beep and press “*” when you are finished.” (VP110003)

Recording + ‘*’

“Your message has been

properly recorded.”(VP110004)

1

Token Play

“Please press ‘1’ to record, press ‘2’ to confirm or press ‘8’ to delete.”(VP140002)

“Press * to quit”          (VP140004 )

“The message has

been deleted.”

(VP131005)

2

8

“ Press the voice bulletin number

to record or delete. “ (VP140003)

Input DTMF

End

*

### Data Management

Ensure that Bulletin Service is available.

[15.5 Voice Mail Option VPM Bulletin Service Use]

Assign a representative number used to access Bulletin Service.

- Input Representative Number

- Select Service Type  Bulletin&TOKENCHG

- Select Ring Ser Method  First or UCD

- Select Busy & DND Option  Send Busy Tone

- No Answer Option  Send Busy Tone.

Assignment of Bulletin Token Change available to extensions

OMS [3.1 Extinction Information Extension Characteristics13.Token Change],

### Correlation

If system serves for Bulletin, the max number of leaving voice message in VMS is reduced from 254 to 244.

### Bulletin Message Checkup

Record/delete the message using token change service and confirm it in VPM as follows:-

| Command | MBOX ; Change to Token Category1 Prompt |
| --- | --- |
| Command | CHK B ; To Check Recoding Status |
| VPM_Debug> mbox [Enter] MBOX> chk b [Enter] Flash adr[0x02020000] Size[0x2594] Attr[00] 2000/10/11 10:42:44 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 |  |

The above shows that the message of only No. “0” of Token 10 ID is recorded. If you delete it, Attr(attribute) field indicates FF

# Trouble Shooting Guide

ST1) If the VPM goes to hold repeatedly at booting time, check the PB switch of the board. All the switches should be off

ST2) First installation in site

| - Command | MBOX RSAT |
| --- | --- |
| VPM_Debug> mbox [Enter] MBOX> rsat [Enter] Used for system : 2 SCTR Used for user mail : 0 SCTR Free : 254 SCTR MBOX> |  |

In the first installation in the site, the SAT information should be displayed as on the screen above. Otherwise, take the VPMBUG $ mode to execute the command as follows:-

| ###### Command | QUIT AB O CSAT AB 1 |
| --- | --- |
| MBOX> quit [Enter] VPM_Debug> ab 0 [Enter] reset VPM VPMBUG $ csat [Enter] Fixed pattern was written at 0x5000D000 Fixed pattern was written at 0x5001D000 VPMBUG $ ab 1 [Enter] |  |
| Reset VPM |  |

The “csat” command performs a function of initializing the Mail box information table. Therefore, be sure to execute the command when installing the board in the site. But never execute it in operation.

ST3) When a user forgots Password.

Viewing the List of Mail Box Number and Password Created in the VPM

| Command | MBOX ML |
| --- | --- |
| VPM_Debug> mbox [Enter] MBOX> ml [Enter] [0x5000D000: 0] ID[99999999] PSWD[9999] [0x5000D100: 1] ID[ 1000] PSWD[AAAA] MBOX> |  |

The above figure shows that the password of ID 1000 has been created “0000.”  (the Mail box number 99999999 is the service number of Bulletin board)

Monitoring the password and listening of each Mail Box ID created in the VPM

| Command | MBOX CHK # ; # Mail Box ID |
| --- | --- |
| MBOX> chk 1000 [Enter] MBOX_ID[1000] Passwd[AAAA] MaxMAIL[3] VoxMAIL[2] CurRECD[0] Flash adr[0x02030000] Size[0x21C0] Attr[01] 2000/10/11 10:59:22 Flash adr[0x02040000] Size[0x0C4E] Attr[00] 2000/10/11 11:07:43 |  |

The above figure shows that the Mail box of ID 1000 can save max. three pieces of voice message (Max MAIL); and now it has two pieces of voice message (VoxMAIL) recorded. In the Attr field, 00 means that the voice message is not listened yet and FF means that the voice message has been deleted already.

ST4) When Prompt indicates “pROBE+>” in the connection of VPM and PC by SIO

Cause:When Autoboot is in the Enable mode and upld table is different from Data in Flash, “pROBE+>” mode appears.

Solution :Change Autoboot into the Disable mode.

pROBE+>dm 600201d0..600201d1 600201d0 01 pROBE+>fm.c 600201d0..600201d1 00 pROBE+>dm 600201d0..600201d1 600201d0 00