TABLE OF CONTENTS

Abnormal Trunk I/C Call Process	5

Advice Of Charge (AOC)	8

Alternative Route Selection (ARS)	10

Announcement	12

Assign D-Tel Flexible Button	13

ATD 10EXT BUSY	15

Attendant Call Pick Up	17

Attendant Calling	18

Attendant Hold	20

Attendant Hold	23

Attendant Night Transfer	24

Attendant Overflow	26

Attendant to Attendant Calling	27

Auto Group Hunting	28

Auto Hold	29

Automatic Network Dialing (AND)	31

Automatic Recall	33

Busy Lamp Field	35

Busy Override	36

Call Forward – All Calls	40

Call Forward – All External Call	43

Call Forward – Busy/No Answer	46

Call Forward – Busy	49

Call Forward – Cancel	52

Call Forward – Follow Me	54

Call Forward – No Answer	57

Call Transfer	60

Call Waiting	62

Calling Group Name Display	65

Calling Party Name Search	67

Calling/Connected Line Identification (CLIP, COLP)	68

Camp On	71

Changeable System Ring Parameter	74

Changeable System Tone Parameter	75

Class Of Service (COS)	76

Collect Call	79

Conference	81

Consultation Transfer to Conference	84

Consultation Transfer	86

Consultation Transfer	88

DDI Divert	89

DDI Incoming Digit Remove & Convert	90

Dial Memo	92

Digital Direct Station : DDS	94

Direct Dialing In : DDI	96

Direct In Line (DIL): Direct In Line of the Trunk Call	98

Direct Inward System Access (DISA)	100

Direct Outward Dialing (DOD)	103

Do Not Disturb (DND)	107

D-Tel Ring Selection	110

Emergency Call	111

Exclusive Hold	112

External Paging	114

Flexible Numbering Plan	115

Forced Release	116

Group Hunting	117

Hot Line	120

House Phone	122

Incoming Mode Change	123

Individual Call Park	124

Individual Speed Dial	126

Individual Trunk Access:	129

Inter Position Transfer	131

Internal Zone Paging	133

Last Number Redial	135

Loud Bell Service	137

LCR (Least Cost Routing)	139

Line Lock-Out	142

Linked Station Pair	143

Manager/Secretary Calling	144

Manager/Secretary Calling	147

Message Waiting	150

MSN (Multiple Subscriber Number)	155

Mute	156

Name Display	157

Night Bar	159

Off-Hook Alarm	160

Other Group Call Pick-Up	161

OG CDR (Outgoing Call Duration Restriction)	163

Outgoing Restriction	165

Outgoing Trunk Queuing	167

Password Change	169

Privacy	170

Recall Button in D-Tel	172

Revertible Ring	174

Same Group Call Pick-Up	175

Save and Repeat	177

Station Branch	178

Station Call Back	180

SDMA (Station Message Detail Accounting)	183

Step Call	186

Sub Address	188

System Hold	189

System Call Park	191

System Predetermined Night Answer (System PNA)	193

System Speed Dial	195

Tenant Service	197

Through Dialing	199

Timed Reminder (Wake-Up)	201

Transfer to Attendant	204

Transit Service	205

Trunk Outgoing Pause Digit	207

Trunk Release Guard Time	209

Trunk Route Separation	210

Trunk Selection	212

Verify Busy Line Status	213

Virtual Dial Tone	215

Walking COS (Remote DOD)	216

Warm Line	218

DECT-Call Transfer	219

DECT-DND (Do Not Disturb)	221

DECT-Extension Call Back	223

DECT-Extension Calling	225

DECT-Internal Zone Paging	227

DECT-Last Number Redial	229

DECT-Linked Station Pair	230

DECT-Other Group Call Pick-Up	231

DECT-Outgoing Trunk Queuing	233

DECT-Receiving an Incoming Trunk Call	235

DECT-Receiving an Station Call	236

DECT-Same Group Call Pick-Up	237

DECT-Save and Repeat	239

DECT-Trunk Calling	241

SYSTEM FEATURES

Notation  ⓡ : " Press mouse's right button"

## Abnormal Trunk I/C Call Process

Functional description

This is a function to provide the service predefined by each route option in case that trunk incoming calls can not reach to the called party because of followed cases.

[Abnormal trunk call Cases]

- The called subscriber is busy.

- The called subscriber does not answer.

- The called subscriber registers the absence feature.

- The called subscriber registers the Do Not Disturb (DND) feature.

- The called number does not conform to the system numbering plan or is invalid.

- The called subscriber is denied receiving the incoming call through the trunk by system data.

- The calling external subscriber does not dial any digits.

- The called subscriber who is engaged in a previously received call has the incoming call forwarded to another subscriber, but the subscriber receiving the call does not answer.

[Supported Services]

- BUSY tone send-out

- CGT send-out

- Connection to system PNA subscriber

- Auto camp on registration

- Auto group hunting

- Recall to the transferring subscriber

- Call attempts until the connection is made

- Forced release

- Connection to voice announcement line designated by routes

- Connection to attendant

- Voice mail.

- No service provided holding the current status: This processing method applies to Direct In Line (DIL) service of analogue C/O trunks only.

Operating procedure

Service conditions

- It is possible to operate the service according to routes.

- It is possible to operate the service separating system mode into day and night.

- For abnormal processing method setup, relevant calls are forcibly released.

e.g.) The service mode of the route is set up as auto group hunting,

but the called subscriber does not belong to any hunting group.

Data generation

- Trunk route separation

[4.1 Trunk Information ⇒ Trunk Route]

[4.7 Trunk Route Assign]

- Service Assign per Day/Night Mode for Abnormal IC trunk Call

[4.10 Trunk Route Base IC Day/Night Option]

- Deny receiving the incoming call

[4.1 Trunk Information ⇒ I/C Allow Class]

- Assign of PNA subscribers

[16.4 System PNA Service]

- Assign of hunting group

[6.7 Hunting Group]

- Assign of announcement line

[16.9 Announcement Line]

Interactions

- Refer to the function ‘trunk route separation’.

- Refer to the function ‘system predetermined night subscriber’.

- Refer to the function ‘automatic recall’.

- Refer to the function ‘announcement line’.

- Refer to the function ‘group hunting’.

- Refer to the function ‘camp on’.

## Advice Of Charge (AOC)

Functional description

The function, which displays amount charged on the calling subscribers’ LCD (D_Tel, ATD, DECT) in real time analyzing charge information received from connected PSTN or PBX for ISDN trunk calls originated and sends the amount data to the charging device connected to the system, supports the following two schemes.

- ETSI standard AOC

AOC-S (Advice Of Charge at Call Setup Time) : Charge information is sent at the same time that a call is established.

AOC-D (Advice Of Charge during the Call) : Accumulative charge information is periodically sent during a call and the total charge information is sent again when the call is released.

AOC-E (Advice Of Charge at the End of the Call) : The total charge information is sent just once when a call is released.

- National type AOC

When it is designated in system that the Code sets and information elements is predefined by nations for AOC service and, if there are information elements matched through retrieval while an outgoing call is processed, the system processes the information elements deeming them as charge information.

Operating procedure

Service conditions

- This function applies to ISDN trunks and QSIG trunks only.

- The function supports only AOC-D and AOC-E of the ETSI standard scheme.

- To access the national type AOC, there shall be codes sets and information elements predefined by nations.

Data generation

- Assign of AOC scheme to be used for ISDN trunks

[7.2 ISDN Option ⇒ AOC Type]

- Assign of national type AOC code sets and information elements [7.4 ISDN National Type IE]

Interactions

- Station Detail Message Accounting (SMDA)

If the AOC function is used, the time based charge automatically becomes unavailable.

## Alternative Route Selection (ARS)

Functional description

This function is introduced in case that there exist multiple trunk routes connected to a system. If a subscriber attempts a trunk outgoing call but there is no trunk available, the function automatically searches for the trunks of alternative routes designated at the trunk access code dialing by the subscriber, selects a routes including trunks available and connects the call to a trunk of the alternative route.

Operating procedure

Service conditions

- If a specific trunk access code is an alternative trunk access code, the alternative access code shall be produced in advance.

- Up to 5 ARS routes may be assigned to a trunk access code.

- There may be no trunk designated to a trunk calling group connected to the trunk access code which has ARS routes designated.

- If a trunk of an alternative route is selected by ARS, repeat digits designated to the selected alternative route are sent out rather than AND digits designated to the trunk call group connected to the original trunk access code.

- In order to access the ARS function, subscribers must attempt their call including the trunk access code.

[Cases excluded by ARS service]

1	A subscriber attempts a call selecting a trunk line designated to one of the D-Tel flexible buttons.

2	A subscriber attempts a call selecting a specific trunk from an attendant.

Data generation

- Assign of trunk access code

[4.2 Trunk Access Code]

- Assign Trunk Outgoing Line for each Trunk Access Code

[4.2 Trunk Access Code – ⓡTrunk OG Group View menu at each Code]

- Assign of ARS code

[4.2 Trunk Access Code ⇒ The first entry of ARS Code & Repeat Digit  1, 2, 3, 4 and 5]

- Assign of repeat digits to be used for ARS code

[4.2 Trunk Access Code ⇒ The second entry of ARS Code & Repeat Digit 1, 2, 3, 4 and 5

Interactions

- Automatic Network Dialing (AND)

If a subscriber dials a trunk access code to attempt a trunk call, the designated AND digits are automatically dialed by the function.

- Least Cost Routing (LCR)

The function, a sort of combination of ARS and AND, not just only simply provides digit addition through selection of a specific trunk but also changes and send out digits dialed through digit addition/deletion/replacement. In general, the function is used if there are a number of communication service providers that apply different charge rates according to days, periods of time and call termination zones.

## Announcement

Functional description

The function provides subscribers who access system functions with relevant announcements rather than basic tones provided by the system.

Operating procedure

Service conditions

- External announcement devices may be connected to ASLC only and there shall be announcement lines designated.

- Up to 16 announcement lines may be designated to a system.

- All tones defined by the system may be replaced by announcement lines.

e.g.) Dial tone, music on hold, busy tone, out-of-service tone, weather service, etc.

- The number of announcement lines for abnormal trunk call to be designated is fixed to 8 per route. Those lines may be used for other purposes (or replaced by tones) if there is no specific route designated in the system or the system is so set up as not to forward abnormal trunk calls to announcement lines.

Data generation

- Assign of announcement line

[16.9 Announcement Line]

- Replacement of system tone by announcement line

[2.4 System Tone Parameter]

- Service Assign per Day/Night Mode for Abnormal IC trunk Call

[4.10 Trunk Route Base IC Day/Night Option]

Interactions

- MOH (Music On Hold)

An announcement may replace MOH (melody) supported by the system. This is a practical example of announcement line.

## Assign D-Tel Flexible Button

Functional description

The function allows subscribers to utilize flexible buttons for special purposes changing features of the flexible buttons with the D-Tel or DSS in various ways.

Operating procedure

- Registration

1	Assign of station line button :

Program Button + Flexible Button + 1 + Shelf No (1~3) + LB No (01~12) + Line No (01~32) + Check Tone + [Flexible Button + Shelf No (1~3) + LB No (01~12) + Line No (01~32) + Check Tone] + 14.. + [..14]

2	Assign of internal line button :

Program Button + Flexible Button + 2 + Internal Number + Check Tone + [Flexible Button + Internal Number + Check Tone] + 14.. + [..14]

3	Assign of function button :

Program Button + Flexible Button + 3 + Function No (00~14) + Check Tone + [Flexible Button + Function Code (00~14) + Check Tone] + 14.. + [14..]

* Functions are matched with function numbers as described below.

00 : Delete / 01 : Group Hunting / 02 : Call Pick Up

03 : Manager/Secretary Calling & Transfer / 04 : Station Answer

05 : Busy Override / 06 : Night Class of SVC / 07 : Day Class of SVC

08 : Password Change / 09 : Absence Status Registration & Cancellation

10 : Call Back / 11 : Timed Reminder / 12 : Call Receive Mode Change

13 : Call Waiting / 14 : Call Waiting : Other Group

4	Assign of station call button :

Program Button + Flexible Button + 4 + Station Call Code or Outgoing Number including Station Call Code + [Flexible Button + Station Call Code or Outgoing Number including Station Call Code] + 14.. + [14..]

5	Assign of pool/loop button

Program Button + Flexible Button + 5 + Check Tone

6	Assign of digit save button

Program Button + Flexible Button + 6 + Digits + [Flexible Button + Digits] + 15.. + [15..]

- Verification

1	Press the [Program] button.

2	Press a flexible button to verify.

3	Then, the property of the flexible button is displayed.

- Delete

1	Press the [Program] button.

2	Press a flexible button to verify.

3	Then, the property of the flexible button is displayed.

4	Dial the number “0”.

5	The check tone is played.

Service conditions

- This function is available to the D-Tel only.

- It is possible to determine whether to assign registration control over the flexible buttons of D-Tel.

- It is possible to determine whether to assign registration control over the flexible buttons of DSS terminal.

Data generation

- Determine whether to assign registration control over the flexible buttons of D-Tel.

[3.1 Extension Information – ⓡEach Extension – DTEL Flexible Button View ⇒ O/G Allow

- Determine whether to assign registration control over the flexible buttons of DSS terminal.

[3.1 Extension Information – ⓡEach Extension – DSS Flexible Button View ⇒ O/G Allow]

## ATD 10EXT BUSY

Functional description

The function is used to register the specific extension subscriber in 10EXT Busy Field of attendant to verify using status of the registered subscriber in real time.

Operating procedure

- Registration of the extension subscriber in 10EXT Busy lamp field

1	The operator presses the “FUNC”(Function) key.

2	The “FUNC”(Function) LED turns on.

3	Dial Index Number(0~9).

4	The registered extension number is displayed when the extension is registered in the 10EXT Busy Lamp Field. If there is no registered extension, “NO:######” is displayed on the LCD of attendant.

5	Enter the extension number to be registered.

6	The operator enters the “ENT”(Register) key.

7	Attendant is in idle state if the input operation is completed normally. If an error occurs, Error Message is displayed on the LCD of attendant.

- Deletion of the extension subscriber registered in 10EXT Busy lamp field

1	The operator presses the “FUNC”(Function) key.

2	The “FUNC”(Function) LED turns on.

3	Dial Index Number(0~9).

4	It the extension is registered in the 10EXT Busy Lamp Field the registered extension number is displayed, and if not, “NO:######” is displayed on the LCD of attendant.

5	When pressing “FORC RLS”(Forced Release) key after the registered extension is displayed, the displayed extension will be deleted in 10EXT Busy Field

6	If the deletion is completed normally, the attendant is in idle state.

- Deletion of all the extension subscriber registered in 10EXT Busy lamp field

1	The operator presses the “FUNC”(Function) key.

2	The “FUNC”(Function) LED turns on.

- Dial the ‘delete all’ (“*”) code.

- The following message is displayed on the LCD.

" Are you sure ?"

- Press '1' to delete, press '0' to continue.

Service conditions

- If this feature is not registered, ATD 10 EXT BUSY lamp is operated normal service.

## Attendant Call Pick Up

Functional description

If an operator connects the subscriber on the other end with the wrong subscriber, the function allows the operator to cancel the call transfer and reestablish a conversation with the transferred subscriber.

Operating procedure

- Attendant call pick up

1	An operator dials the number of the subscriber whom the operator connected to the wrong subscriber or the number of the subscriber whose phone is ringing.

2	Then, the internal subscriber’s status is displayed on the LCD of the attendant.

3	The operator presses the “ATD PICK-UP”(Attendant Pick-up) key while the internal subscriber is receiving the ring.

4	Then, the ring of the internal subscriber who tries to pick up the handset is canceled the status of the subscriber who requested the call transfer is displayed on the LCD of the attendant and the conversation between the subscriber and the operator is resumed.

Service conditions

- If “ATD Pick Up” key is pressed when the subscriber to whom the operator transferred the call does not exist or is answering a previously received call, the function is not performed and the “Error” message is displayed on the LCD of the attendant.

Interactions

- Operators can transfer a call by dialing a subscriber’s number and pressing the “CONNECT”(Connection) key and cancel the call transfer by pressing the “ATD PICK-UP”(Attendant Pick-up) key.

## Attendant Calling

Functional description

The function allows internal subscribers to call an operator by dialing the attendant calling code or the operator’s number as well as answer calls from operators.

Operating procedure

- Attendant calling from general telephone/D-Tel

1	An internal subscriber hooks off the handset.

2	The subscriber dials one of the attendant calling code, attendant number and representative attendant number.

3	Then, the internal subscribers receives the ring, and the “STA ANS”(Station Answer) LED flashes and the bell rings if the bell is on.

4	The called operator presses the “STA ANS”(Station Answer) or “ANSWER” key.

5	Then, a call between the subscriber and the operator is established.

Service conditions

- In general, if the attendant calling code or representative number is used to call, all incoming calls are assigned by auto call assignment.

- If designated numbers are assigned to individual attendants and an incoming call is forwarded to the individual number of a attendant, only the attendant is called and auto call assignment is not performed. Such the process of assigning numbers to individual attendants is determined in the stage of system production and may be modified by OMS.

- If a subscriber hooks off the handset while receiving the ring, the call is canceled.

- If the called attendant is in the night service, the calling signal is forwarded to the subscriber who is designated as the night transfer subscriber.

- If the called attendant is serving as the assistant operator and retains a number of subscribers waiting to be served over a given number, incoming calls to the attendant are forwarded to the internal subscriber designated as the assistant operator.

Data generation

- Connection information between the attendants (Representative attendant’s designation of link attendant by tenants)

[5.1 ATD Link]

- Assign of representative attendant by tenants

[5.2 Tenant Representative ATD]

- Individual ATD Assign for Extension

[5.3 Extension Individual ATD]

- Individual ATD Assign for Trunk

[5.4 Trunk Individual ATD]

- Assign of attendant representative number

[5.5 ATD Representative No]

- Assign of attendant service option

[5.6 ATD Service]

- Assign of attendant night subscriber

[5.7 ATD Night Subscriber]

- Assign of attendant overflow subscriber

[5.8 ATD Overflow Subscriber]

## Attendant Hold

Functional description

The function allows operators to hold a subscriber who is talking with the attendant itself or who is ringing to the attendant, answer a held subscriber or connect the current subscriber to a held subscriber retrieving held subscribers during a call.

Operating procedure

- Hold by attendant in state of Conversation.

1	An operator presses the “HOLD” key while talking to the subscriber on the other end.

2	The LCD of the attendant is cleared off, one of the “Hold Indicator” LEDs turns on and the subscriber hears MOH. In this case, the attendant enters into the idle status.

- Hold by attendant in state of Ringing.

1	Call terminated in the attendant.

2	An operator presses the “CALL HOLD” key.

3	The LCD of the attendant is cleared off, one of the “Hold Indicator” LEDs turns on and the subscriber hears MOH. In this case, the attendant enters into the idle status.

- Attendant answer to held subscriber

1	An operator presses the “HOLD” key while the attendant is idle and dials the one digit number corresponding to one of the “Hold Indicator” LEDs on.

2	The subscriber’s hold status is released as the LED turns off and displayed on the LCD of the attendant, and a call to the held subscriber is set up.

- Hold verification from attendant

1	An operator presses the “HOLD VERIFY”(Hold Verification) key while the attendant is idle and dials the one digit number corresponding to one of the “Hold Indicator” LEDs on.

2	A held subscriber’s status is indicated by the “Hold Indicator” LED and the other “Hold Indicator” LEDs hold their status.

3	The operator presses the “CLR” (Clear) key.

4	The held subscriber remains held and the held subscriber verification terminates as the LCD of the attendant is cleared off.

5	The operator keeps pressing the “HOLD VERIFY”(Hold Verification) key while a held subscriber’s status is displayed on the LCD of the attendant.

6	The currently held subscribers are displayed on the LCD of the attendant one after another and they remain held.

7	The operator presses the “Connect”(Connection) key while a held subscriber’s status is displayed on the LCD of the attendant.

8	The currently displayed subscriber’s hold status is released and the call between the subscriber and the operator is resumed as the “Hold Verification” LED turns off.

- Attendant connection to held subscriber

1	An operator presses the “HOLD VERIFY”(Hold Verification) key during a call and dials the one digit number corresponding to one of the “Hold Indicator” LEDs on.

2	The held subscriber’s status is indicated by the “Hold Indicator” LED and the other “Hold Indicator” LEDs hold their status.

3	The operator presses the “CONNECT”(Connection) key.

4	The currently displayed subscriber’ hold status is released and a call between the subscriber and the current subscriber talking to the operator over the phone is established as the “Hold Verification” LED turns off. In this case, the LCD of the attendant is cleared off and connection to held subscriber terminates.

Service conditions

- The maximum number of subscribers to be held by a attendant is 5.

- If an operator presses the “HOLD” key to hold another subscriber in addition to the 5th held subscriber, the additional subscriber is no longer held and the call between the subscriber and the attendant continues. Also, there is the “Error” message displayed on the LCD of the attendant.

- If an operator presses the “HOLD VERIFY”(Hold Verification) key followed by the number corresponding to one of the LEDs off, the “Error” message is displayed on the LCD of the attendant.

- If both the subscriber talking to the operator and the held subscriber to be connected are trunk subscribers, there shall be relevant data produced so that it is possible to connect calls by routers.

Data generation

- Connection Allow/Deny Option between Routes

[4.12 Inter Route Connection Allow/Deny]

- EXT Station Holding Tone

[2.4 System Tone Parameter]

## Attendant Hold

Functional description

“Same as COMMON”

Operating procedure

- Hold by attendant in state of Conversation.

“Same as COMMON”

- Hold by attendant in state of Ringing.

1	Extension subscriber or trunk subscriber calls the attendant.

2	An operator presses the “CALL HOLD” key.

3	The LCD of the attendant is cleared off, “Hold1 Indicator” LED turns on and the subscriber hears MOH. In this case, the attendant enters into the idle status.

- Attendant answer to held subscriber

1	An operator presses the “HOLD VERIFY”(Hold Verification) key

2	The subscriber’s hold status is released as the LED turns off and displayed on the LCD of the attendant, and a call to the held subscriber is set up.

- Attendant connection to held subscriber

1	The attendant is connected to the subscriber held first according to the held order, when pressing “HOLD VERIFY”(Hold Verification) key in idle state.

Service conditions

- The “CALL HOLD” key is not used when forwarding to attendant or holding the subscriber in busy.

- The maximum number of subscribers to be held by a attendant is 10.

- If an operator presses the “HOLD” key to hold another subscriber in addition to the 10th held subscriber, the additional subscriber is no longer held and the call between the subscriber and the attendant continues. Also, there is the “Error” message displayed on the LCD of the attendant.

- If at least one subscriber held exists, “Hold1 Indicator” LED is on.

Data generation

“Same as COMMON”

## Attendant Night Transfer

Functional description

The function allows operators to transfer incoming calls, which are received in the night transfer mode to their attendant to attendant night transfer subscribers.

Operating procedure

- Attendant night transfer

1	An operator presses the “NIGHT”(Night Mode) key attached to the side of the attendant in the day mode.

2	Then, the “Night”(Night Mode) LED of the attendant turns on.

3	An incoming call to the attendant is originated.

4	All incoming calls to the individual number of each attendant in the night transfer mode are automatically transferred to attendant night transfer subscribers.

5	The operator enters the attendant representative number or attendant calling code and, if there is any attendant operating in the day mode in the tenant, calls a attendant in the day mode. If all attendants in the tenant operate in the night transfer mode, incoming calls are automatically forwarded to attendant night transfer subscribers although the attendant representative number or attendant calling code is entered.

6	An operator presses the “NIGHT”(Night Mode) key attached to the side of the attendant in the night transfer mode.

7	Then, the “Night” LED of the attendant turns off.

Service conditions

- Operators should personally process calls which remain unprocessed in spite of the fact that they press the “NIGHT”(Night Mode) key while there are incoming calls remaining at attendants at their attendant and all incoming calls originated after then are automatically transferred to attendant night transfer subscribers.

- If all attendant night transfer subscribers are engaged, the operator hears the busy tone

- Just one attendant night transfer subscriber may be designated to a attendant or tenant. In this case, the subscriber’s number includes the calling group representative number.

Data generation

- Attendant night transfer subscriber

[5.7 ATD Night Subscriber]

- Assign of calling group representative number

[6.4 Calling Group]

Interactions

- If there is no attendant night transfer subscribers in the night transfer mode, incoming calls are forwarded to system PNA subscribers. If there is no system PNA subscriber either, it processed as busy.

## Attendant Overflow

Functional description

If the number of incoming calls to attendants abruptly increases and consequently there are too many calls waiting over a certain number, the calls are forwarded to specific internal subscribers designated by the function.

Operating procedure

- Attendant overflow

1	Judging that there are too many incoming calls waiting at a attendant, the operator presses the “OVF”(Attendant Overflow) key.

2	Then, the “OVF”(Attendant Overflow) LED of the attendant turns on.

3	Incoming calls over a certain number are forwarded to the attendant.

4	All incoming calls received by the attendant are transferred to the attendant overflow internal subscribers designated.

5	The operator presses the “OVF”(Attendant Overflow) key.

6	Then, attendant overflow is canceled and all incoming calls received after then are forwarded to the attendant.

Service conditions

- If all attendant overflow subscribers are engaged, the calling subscriber hears the busy tone.

- It may determined as requested by customers how many calls waiting there should be to transfer excessive calls to attendant overflow subscribers while the function is active.

Data generation

- Attendant overflow subscribers

[5.8 ATD Overflow Subscriber]

## Attendant to Attendant Calling

Functional description

The function allows operators to call other attendant operators by dialing their individual attendant number.

Operating procedure

- Attendant to attendant calling

1	An operator dials another operator’s individual attendant number to call.

2	The calling operator hears the calling tone and the “STA ANS”(Station Answer) lamp of the called attendant flashes.

3	The called operator answers.

4	An attendant-to-attendant call is set up.

Service conditions

- Attendant subscribers shall be assigned internal numbers.

Data generation

- Attendant production

[3.1 Extension Information]

Interactions

- If the calling attendant releases a call before the called attendant answers the call, the “STA ANS”(Station Answer) lamp of the called attendant turns off.

- If a attendant number dialed is an invalid number, the “Error” message is displayed on the LCD of the attendant.

- If the power of a called attendant is off, the calling subscriber hears the CGT tone.

- Though a called subscriber is in the night transfer mode, it is possible to set up a attendant-to-attendant call.

- Attendants are called by their own attendant number and such attendant numbers are defined when the attendants are produced.

## Auto Group Hunting

Functional description

If a called subscriber’s line is busy, the call is forwarded to one of the subscribers in the same hunting group that the called subscriber belongs to.

Operating procedure

- Auto Group Hunting

1	Internal subscriber A subscriber calls an internal subscriber.

2	If the called subscriber’s line is busy, the ring is sent to an idle subscriber of the subscribers in the same hunting group that the called subscriber belongs to.

3	A call between the two subscriber is set up once the subscriber answers the ring.

Service conditions

- This function is available to both general telephone and D-Tel.

- This function is unavailable while step call is active.

- The called original subscriber should set up the function so that auto group hunting may be performed in the subscriber’s hunting group.

- In order for the function to be performed, there should exist at fewest an idle subscriber in the same hunting group.

Data generation

- Assign of Auto Group Hunting

[6.7 Hunting Group ⇒ Auto Group Hunting]

- Assign whether step call is used or not.

[2.6 System Feature Option ⇒ Step call use]

## Auto Hold

Functional description

If extension incoming call or trunk incoming call is delivered to a D-Tel subscriber whose line is busy, the call is automatically held while MOH is sent to the calling subscriber and a call between the two subscribers is set up once the D-Tel subscriber answers the held subscriber.

Operating procedure

- Auto hold by D-Tel

1	An incoming call from subscribers of other extension or external CO line is delivered to a D-Tel subscriber whose line is busy.

2	The signal indicating there is an incoming call is sent to the D-Tel subscriber and the LED of the hold button of flexible buttons flashes. In this case, the calling subscriber hears the calling tone.

3	The D-Tel subscriber presses the flashing flexible button as soon as the subscriber finishes with the previously received call.

4	A call between the automatically held subscriber and the D-Tel subscriber is set up.

Service conditions

- Subscribers of general telephones cannot access auto hold.

- It should be determined whether auto hold is used or not.

- The flexible buttons of the automatically held subscriber should be designated to the subscriber to be held or as the Pool/Loop button.

- If a subscriber to be held is designated by the D-Tel subscriber’s flexible buttons, the subscriber is held by the flexible buttons.

- If a subscriber to be held is not designated by the D-Tel subscriber’s flexible buttons, the subscriber is held by the Pool/Loop button.

- If there is neither flexible button nor Pool/Loop button available, the flexible button 23 for LGP terminal and the flexible button 2 for LKD terminal are used for auto hold as long as no other function is designated to the flexible buttons.

- If none of a D-Tel subscriber’s flexible buttons is designated for auto hold, subscribers calling the D-Tel subscriber hear the lock out tone.

Data generation

- Determination of whether to activate auto hold

[3.1 Extension Information – Extension Characteristic ⇒ 32. Auto Hold]

- Assign of flexible button

[3.1 Extension Information – ⓡEach Extension – DTEL Flexible Button View]

Interactions

- If a subscriber is held by auto hold while the holding D-Tel subscriber’s line is not busy, the held subscriber has to wait until the D-Tel subscriber answers.

- If a holding D-Tel subscriber hooks off the handset before the subscriber held by auto hold answers, the auto hold is automatically canceled and the flexible button of the held subscriber LED turns off.

- If a called subscriber is engaged, auto group hunting is performed first followed by auto hold, auto call waiting and auto camp on according to the priority order.

## Automatic Network Dialing (AND)

Functional description

AND allows a subscriber to designate preferred digits in trunk access code and automatically prefixes the designated digits to the external number that the subscriber dialed first. If a number of connected switches are operated on a network and a specific switch subscriber tries to call extension subscriber of the called switch through a couple of transit switches, the function makes it possible for the call to be automatically transmitted through the transit switch only by dialing the prefix of the called switch (trunk access code designated by the calling switch office to call the switch) or its extension number.

Operating procedure

Service conditions

- A trunk access code can produce a set of AND digits.

Data generation

- Assign of trunk access code

[4.2 Trunk Access Code]

- Assign Trunk Outgoing Line for each Trunk Access Code

[4.2 Trunk access code – Trunk OG Group View menu at each Code]

- Assign of AND digit

[ 4.2 Trunk Access Code ⇒ AND Digit]

Interactions

- Alternative Route Selection (ARS)

If ARS selects a trunk of an alternative route, AND digits designated by the trunk calling group connected to the original trunk access code remain unused and the designated repeat digits of the selected route are sent out.

- Least Cost Routing (LCR)

The function, a sort of combination of ARS and AND, not just only simply provides digit addition through selection of a specific trunk but also changes and send out digits dialed through digit addition/deletion/replacement. In general, the function is used if there are a number of communication service providers that apply different charge rates according to days, periods of time and call termination zones.

## Automatic Recall

Functional description

If there is no answer to a hold call or a call transferred to another subscriber for predefined time, the call is automatically sent to the subscriber again by this function.

Operating procedure

- Hold call recall

1	An extension subscriber A holds another internal or trunk subscriber B on the other end of the line during a call. In this case, the held subscriber B hears MOH and the extension subscriber A hears the first dialing tone.

2	The extension subscriber A attempts a call to set up. (This step may be left out.)

3	On hooking on the handset, the extension subscriber A becomes idle.

4	The extension subscriber A does not answer the hold call in the designated period of time although the subscriber is idle.

5	The extension subscriber A and the held subscriber B hear the recall ring and the ring back tone, respectively.

6	The Extension subscriber A hooks off the handset.

7	The hold call between the two subscribers is resumed.

- Transferred call recall

1	An extension subscriber A who is talking to another extension or trunk subscriber B on the other end of the line attempts a call transfer to another extension or trunk subscriber C.

2	The extension subscriber C does not answer in the designated period of time.

When wrong extension number is dialed.

3	If the extension subscriber A is idle, the subscriber A and B hear the ring and the ring back tone, respectively.

Service conditions

- For hold recall, if the extension subscriber A does not answer in the designated period of time because the subscriber’s line is busy, the subscriber B is held again for the designated period of time.

- For transfer recall, if the subscriber C is a trunk subscriber, the subscriber should be an ISDN subscriber capable of sending the “No Answer” message. Trunks to which the “No Answer” message is unavailable are not able to access the recall service and automatically released. If the subscriber A does not answer the call received by the recall service in the designated period of time, the call is transferred to the attendant.

- For transfer recall, if the subscriber B is a trunk subscriber, it depends on the abnormal trunk IC call process options of the subscriber B’s route whether or not to recall.

- The recall ring period and duration comply with the item ‘Recall Ring’.

Data generation

- Setup of hold tone lock

[2.4 System Tone Parameter ⇒ Trunk Hold Tone]

[2.4 System Tone Parameter ⇒ Ext Hold Tone]

- Setup of transfer ring lock

[2.5 System Ring Parameter ⇒ Call Transfer Ring]

- Setup of recall ring lock

[2.5 System Ring Parameter ⇒ Recall Ring]

- Service Assign per Day/Night Mode for Abnormal IC trunk Call

[4.10 Trunk Route Base IC Day/Night Option

⇒ IC Call Transfer Ext No Answer]

Interactions

- No answer call forwarding and auto group hunting

For transfer recall, if the subscriber C is an extension subscriber and does not answer in the designated period of time, call forwarding is performed first followed by group hunting and recall according to the priority order.

## Busy Lamp Field

Functional description

The function designates the trunk or extension group as busy group to verify the status of the subscriber in the group.

Operating procedure

- Busy lamp field of attendant

1	The operator presses the “BUSY GRP”(Busy Group) key.

2	The “BUSY GRP”(Busy Group) LED turns on.

3	The operator dials the number of trunk or extension group to be verified.

4	The dialing group number is displayed on “BUSY:” of the LCD of the attendant.

5	The operator enters the “ENT”(Register) key.

6	The availability of the registered trunk or subscribers of the extension group is successively displayed on the LEDs.

7	The operator presses the “BUSY GRP”(Busy Group) key while the “BUSY GRP”(Busy Group) LED is on.

8	Then, the registered availability status is canceled as the number of the group displayed on the attendant LCD is cleared off.

Service conditions

- If an operator presses the “CLR”(Clear), “RLS”(Release) or another key after dialing the numbers of the trunk or extension group to be verified, the dialing of the group is canceled.

- If the group number for extension subscriber is designated: The remaining figures excepting the last double digits of each number are designated as the group number of the concerned number. For instance, the group number of the number 200 to 299 is 2 and the group number of the number 2000 to 2099 is 20. The group number is used for the availability status registration.

- If the group number for trunk subscriber is designated: The remaining figures excepting the last double digits of each trunk serial number are designated as the group number of the concerned number.

## Busy Override

Functional description

The function allows an extension subscriber or operator to override another subscriber call. First, the system sends the break-in tone to the subscriber whose line is busy to inform the subscriber of the override. And then, it is forwarded to 3-PTY conference call with the subscribers of one-to-one call as soon as the break-in tone ends.

Operating procedure

- Override on general telephone/D-Tel.

1	An extension subscriber hooks off the handset.

2	The subscriber dials another extension subscriber’s number.

3	The extension subscriber hears the busy tone if the subscriber’s line is busy.

4	The calling extension subscriber enters the override code (Default:3).

5	The break-in tone is sent to inform the called subscriber of the override.

6	The called extension subscriber hears the break-in tone and a 3-PTY conference call is set up.

- Override in attendant

1	An operator dials an extension subscriber’s number.

2	The extension subscriber’s status is displayed on the LCD of the attendant. If the subscriber’s line is busy, the operator hears the busy tone.

3	The operator presses the “BRK IN”(Break-In or Override) Key while hearing the busy tone.

4	A 3-PTY conference call among two subscribers and one operator is set up as soon as the operator hears the override warning tone for a second.

5	The operator presses the “RLS”(Release) key.

6	Then, only the operator is released from the conference call and the call between the two subscribers is maintained.

Service conditions

- The function is available for both general telephone and D-Tel.

- The overridden subscriber should be engaged in a one-to-one call.

- This function is unavailable while step call is available.

- It is possible to activate/deactivate the override function.

- It is not possible to override the privacy call or auto privacy call.

- It is possible to determine whether or not to send the break-in tone to the overridden subscriber.

- Although one of the three extension subscribers in a conference call hooks on the handset after an override is successfully performed, the one-to-one call between the remaining two subscribers can continue.

- It is not possible to override if any of the overriding and overridden subscribers meets the following conditions.

- ATD / Privacy call / Auto privacy call subscribers

- Subscribers participating in the two party call in case of consultation transfer

- If the conference call continues after the operator’s override, the break-in tone may be periodically sent to the three parties in the conference call to verify whether the operator taps the call.

- It is not possible to override extension subscribers engaged in 3-PTY conference call. Once the operator presses the “BRK  IN”(Override or Break-In) key, the “Error” message is displayed on the LCD of the attendant and the operator can hear the lock out tone.

Data generation

- Whether or not the break-in tone is sent in case of override attempt to extension

[2.6 System Feature Option ⇒ Break In Tone in EXT]

- Whether or not the break-in tone is sent in case of override attempt to trunk

[2.6 System Feature Option ⇒ Break In Tone in ATD]

- Whether or not to use a conference confirmation tone in conference mode

[2.6 System Feature Option ⇒ Conference Tone Send Method]

- Whether or not to use step call

[2.6 System Feature Option ⇒ Step Call Use]

- Whether or not to use auto privacy call for each subscriber

[3.1 Extension Information - Extension Characteristic ⇒ 14. Auto Privacy]

- Whether or not to allow override for each subscriber

[3.1 Extension Information - Feature Restriction ⇒ Break In]

- Tone value used for override by extension subscriber

[2.4 System Tone Parameter ⇒ Break In Tone]

- Tone value used for conference call by override

[2.4 System Tone Parameter ⇒ Conference Break-in High Tone]

- Tone value used for conference call by override

[2.4 System Tone Parameter ⇒ Conference Break-in Low Tone]

- Busy service code designation

[16.5 One Digit Busy Service]

Interactions

- If the called subscriber is busy, the calling subscriber may access a specific function by pressing the digit corresponding to the function while hearing the busy tone. Functions are matched with digits (Default Value) as shown below.

- “2“ : Group hunting

- “3” : Busy override

- “4” : Call back

- “5” : Call waiting

- “6” : Camp on

## Call Forward – All Calls

Functional description

The function allows extension subscribers to have their incoming calls forwarded to another extension subscriber or external subscriber who are designated by them.

Operating procedure

- Call forward - all calls from general telephone to extension subscriber

1	An extension subscriber hooks off the handset.

2	The subscriber dials the call forward - all calls code (Default:*521).

3	The subscriber enters the extension subscriber’s number to be forwarded.

4	If the dialed number is valid and the registration is successfully completed, the subscriber hears the special dial tone.

- Call forward - all calls from general telephone to external trunk subscriber

1	An extension subscriber hooks off the handset.

2	The subscriber dials the call forward - all calls code (Default:*521).

3	The subscriber dials the trunk access code.

4	The subscriber enters an external number.

5	The subscriber hooks on the handset after entering the number.

- Call forward - all calls from D-Tel to extension subscriber

1	A D-Tel subscriber presses the “SPEAKER” button.

2	The subscriber presses the “DND/CFWD” button. In this case, the following message is displayed on the LCD.

ENTER CALL FORWARD TYPE

3	The subscriber enters “1”. In this case, the following message is displayed on the LCD.

ENTER A DIAL DIGIT

4	The subscriber dials an extension number.

5	If the extension number is valid and the registration is successfully completed, the subscriber hears the service set tone, the “DND/CFWD” LED turns on and flashes, and the following message is displayed on the LCD.

FORWARD – ALL

XXXX 09:00 am

- Call forward - all calls from D-Tel to external trunk subscriber

1	A D-Tel subscriber hooks off the handset or presses the “SPEAKER” button.

2	The subscriber presses the “DND/CFWD” button. In this case, the following message is displayed on the LCD.

ENTER CALL FORWARD TYPE

3	The subscriber enters “1”. In this case, the following message is displayed on the LCD.

ENTER A DIAL DIGIT

4	The subscriber dials the trunk access code. If the station access code is incorrect, the warning tone is sent. In this case, the D-Tel subscriber should do it over again hooking on the handset.

5	The subscriber dials an external subscriber’s number.

6	After entering the external subscriber’s number to be forwarded and pressing the “HOLD/SAVE” button, the subscriber hears the service set tone, and the ”DND/CFWD” LED turns on and flashes.

7	Once the subscriber hooks on the handset or presses the “SPEAKER” button, the following registration information is displayed on the LCD.

FORWARD – ALL

XXXXXXXXX 09:00 am

Service conditions

- It is possible to determine whether to use call forward.

- An extension subscriber should not have two or more call forwards registered at once.

- If a subscriber requests another call forward while the call forward is already registered, the contents of the previously registered call forward are automatically modified.

- If a call forward subscriber hooks off the handset, the special dial tone(Call Forward alert tone) hears following the dial tone sounds.

- For call forward to external number, station access code is not displayed and the maximum of 12 digits is only displayed.

- It is not allowed to forward incoming calls to the representative attendant.

- It is not allowed to forward incoming calls to hot line subscribers.

- It is not allowed to forward incoming calls to the self-telephone.

- “Class E” or lower subscribers should not access call forward - all calls to external number.

- It is not possible to access call forward while DND is active.

Data generation

- Whether or not to use call forward

[3.1 Extension Information – Feature Restriction ⇒ Call Forwarding]

- Call Forward Alert Tone

[2.4 System Tone Parameter – Call Forward Alert Tone]

Interactions

- It is not possible to have call forward registered using buttons while absence message is active. In this case, call forward can be registered by codes and the previously recorded absence message is deleted.

- The operator can cancel the registered call forward forcibly if necessary.

- If a subscriber registers absence message on a D-Tel while call forward is active, call forward is canceled.

- If DND is active, subscribers should cancel the function before registering call forward.

- The call forward from external number is accompanied by toll check.

## Call Forward – All External Call

Functional description

The function allows extension subscribers to have all incoming calls from external subscribers forwarded to other designated extension subscriber or external subscriber.

Operating procedure

- Registration of Call Forward – All External Call to extension subscriber on general telephone.

1	An extension subscriber picks up the handset.

2	Dial the registration code for Call Forward – All External Call (Default:*526).

3	Enter the extension telephone line number to forward calls.

4	In case the telephone number is normal, and if registration is completed normally, service set tone is heard.

- Registration of Call Forward – All External Call to external subscriber on general telephone.

1	An extension subscriber picks up the handset.

2	Dial the registration code for Call Forward – All External Call (Default:*526).

3	Dial trunk access code.

4	Enter the external telephone number.

5	Hang up the handset after entering the telephone number.

- Registration of Call Forward – All External Call to extension subscriber on D-Tel telephone using buttons.

1	Press “SPEAKER” button.

2	Press “DND/CFWD” button. Then, following message is displayed on LCD.

ENTER CALL FORWARD TYPE

3	Dial “6”, then following message is displayed on LCD.

ENTER A DIAL DIGIT

4	Dial extension number.

5	In case the extension number is normal, and if registration is completed normally, service set tone is heard and “DND/CFWD” button LED is turned on and blinks.

6	Hang up the handset or press “SPEAKER” button, then following message is displayed on LCD.

FORWARD -- ALL (E)

XXXX 09:00 am

- Registration of Call Forward – All External Call to external trunk subscriber on D-Tel telephone using buttons

1	Press “Speaker button”.

2	Press “DND/CFWD” button. Following message is displayed on LCD.

ENTER CALL FORWARD TYPE

3	Dial “6”, then following message is displayed on LCD.

ENTER A DIAL DIGIT

4	Dial trunk access code. In case the trunk access code is incorrect, alarm tone is heard. Then, release D-Tel telephone and restart registration.

5	Dial external subscriber number.

6	Enter the external subscriber telephone number to forward the call, and press “SPEAKER” button, and then following registration information is displayed on LCD.

FORWARD -- ALL (E)

XXXXXXXXX 09:00 am

7	Enter the external subscriber telephone number to forward the call, and hang up the handset or press “HOLD/SAVE” button, then service set tone is heard and “DND/CFWD” button LED is turned on and blinks. Following registration information is displayed on LCD.

FORWARD -- ALL (E)

XXXXXXXXX 09:00 am

Service conditions

- It is possible to designate whether or not call forward function is allowed.

- Extension subscriber cannot register two or more call forwards at the same time.

- In case call forward registration is requested when call forward has already been registered, previous one is automatically changed.

- If a subscriber who has registered to the function picks up the handset, special dial tone is heard for a while and then dial tone is heard.

- When registering call forward to external subscriber number, trunk access code is not displayed, and maximum 12 digits are displayed.

- It is not possible to forward call to general attendant.

- It is not possible to forward call to hot line subscriber.

- It is not possible to forward call to its own telephone.

- Subscriber of "E class or below" cannot register Call Forward – All External Call to external number.

- In case DND(Do Not Disturb) function is registered, this should be cancelled first before registering the Call Forward – All External Call.

Data generation

- Whether Call Forward is used

[3.1 Extension Information – Feature Restriction ⇒ Call Forwarding]

- Call Forward special dial tone

[2.4 System Tone Parameter ⇒ Call Forward Alert Tone]

Interactions

- It is not possible to register Call Forward using buttons while absence announcement message is registered, but is possible to register Call Forward using codes. Existing absence announcement message is deleted.

- Registered Call Forward may be forcibly cancelled by operator if necessary.

- If Absence Message is registered on D-Tel telephone where Call Forward is registered, the Call Forward is cancelled.

- Call Forward should be registered after the DND(Do Not Disturb) is cancelled.

- Toll Check is required to register Call Forward to external subscriber.

## Call Forward – Busy/No Answer

Functional description

The function allows extension subscribers to have calls forwarded to other designated extension subscriber or external subscriber when an extension subscriber line is busy or there is no answer from the subscriber.

Operating procedure

- Registration of Call Forward – Busy/No Answer to extension subscriber on general telephone

1	An extension subscriber picks up the handset.

2	Dial the registration code for Call Forward – Busy/No Answer (Default:*524).

3	Enter the extension telephone line number to forward calls.

4	In case the telephone number is normal, and if registration is completed normally, service set tone is heard.

- Registration of Call Forward – Busy/No Answer to external trunk subscriber on general telephone

1	An extension subscriber picks up the handset.

2	Dial the registration code for Call Forward – Busy/No Answer (Default:*524).

3	Dial trunk access code.

4	Enter the external telephone number.

5	Hang up the handset after entering the telephone number.

- Registration of Call Forward – Busy/No Answer to extension subscriber on D-Tel telephone using buttons

1	Press “SPEAKER” button.

2	Press “DND/CFWD” button. Following message is displayed on LCD.

ENTER CALL FORWARD TYPE

3	Dial “4”, then following message is displayed on LCD.

ENTER A DIAL DIGIT

4	Dial extension number.

5	In case the extension number is normal, and if registration is completed normally, service set tone is heard and “DND/CFWD” button LED is turned on and blinks. The following message is displayed on LCD.

FORWARD – BUSY/NO ANS

XXXX 09:00 am

- Registration of Call Forward – Busy/No Answer to external trunk subscriber on D-Tel telephone using buttons

1	Press “SPEAKER” button.

2	Press “DND/CFWD” button. Following message is displayed on LCD.

ENTER CALL FORWARD TYPE

3	Dial “4”, then following message is displayed on LCD.

ENTER A DIAL DIGIT

4	Dial trunk access code. In case the trunk access code is incorrect, alarm tone is heard. Then, release D-Tel telephone and restart registration.

5	Dial external subscriber number.

6	Enter the external subscriber telephone number to forward the call, and press “HOLD/SAVE” button”, then service set tone is heard, and then “DND/CFWD” button LED is turned on and blinks.

7	Hang up the handset or press “SPEAKER” button, and then registration information is displayed on LCD as shown below.

FORWARD – BUSY/NO ANS

XXXXXXXXX 09:00 am

Service conditions

- It is possible to designate whether or not call forward function is allowed.

- Extension subscriber cannot register two or more call forwards at the same time.

- In case call forward registration is requested when call forward has already been registered, previous one is automatically changed.

- If a subscriber who has registered to the function picks up the handset, special dial tone is heard for a while and then dial tone is heard.

- When registering call forward to external subscriber number, trunk access code is not displayed, and maximum 12 digits are displayed.

- It is not possible to forward call to general attendant.

- It is not possible to forward call to hot line subscriber.

- It is not possible to forward call to its own telephone.

- Subscriber "E class or below" cannot register Call Forward to external number.

- In case DND(Do Not Disturb) function is registered, this should be cancelled first before registering the Call Forward.

Data generation

- Whether Call Forward is used

[3.1 Extension Information - Feature Restriction ⇒ Call Forwarding]

- Call Forward special dial tone

[2.4 System Tone Parameter ⇒ Call Forward Alert Tone]

Interactions

- It is not possible register Call Forward using buttons while absence announcement message is registered, but is possible to register Call Forward using codes. Then, existing absence announcement message is deleted.

- Call Forward cannot be registered when DND(Do Not Disturb) is registered by D-Tel phone.

- Registered Call Forward may be forcibly cancelled by operator if necessary.

- If Absence Message is registered on D-Tel telephone where Call Forward is registered, the Call Forward is cancelled.

- Call Forward should be registered after the DND(Do Not Disturb) is cancelled.

- Toll Check is required to register Call Forward to external subscriber.

## Call Forward – Busy

Functional description

The function allows extension subscribers to have calls forwarded to other designated extension subscriber or external subscriber when an extension subscriber line is busy.

Operating procedure

- Registration of Call Forward – Busy to extension subscriber on general telephone

1	An extension subscriber picks up the handset.

2	Dial the registration code for Call Forward – Busy (Default:*522).

3	Enter the extension telephone line number to forward calls.

4	In case the telephone number is normal, and if registration is completed normally, service set tone is heard.

- Registration of Call Forward – Busy to external trunk subscriber on general telephone

1	An extension subscriber picks up the handset.

2	Dial the registration code for Call Forward – Busy (Default:*522).

3	Dial Trunk access code.

4	Enter the external telephone number.

5	Hang up the handset after entering the telephone number.

- Registration of Call Forward – Busy to extension subscriber on D-Tel telephone using buttons

1	Press “SPEAKER” button.

2	Press “DND/CFWD” button. Following message is displayed on LCD.

ENTER CALL FORWARD TYPE

3	Dial “2”, then following message is displayed on LCD.

ENTER A DIAL DIGIT

4	Dial extension number.

5	In case the extension number is normal, and if registration is completed normally, service set tone is heard and “DND/CFWD” button LED is turned on and blinks. The following message is displayed on LCD.

FORWARD – BUSY

XXXX 09:00 am

- Registration of Call Forward – Busy to external trunk subscriber on D-Tel telephone using buttons

1	Press “Speaker button”.

2	Press “DND/Call Forward button”. Following message is displayed on LCD.

ENTER CALL FORWARD TYPE

3	Dial “2”, then following message is displayed on LCD.

ENTER A DIAL DIGIT

4	Dial trunk access code. In case the trunk access code is incorrect, alarm tone is heard. In this case, release D-Tel telephone and restart registration.

5	Dial external subscriber number.

6	Enter the external subscriber telephone number to forward the call, and press “HOLD/SAVE” button, then service set tone is heard, and then “DND/CFWD” button LED is turned on and blinks.

7	Hang up the handset or press “SPEAKER” button, and then registration information is displayed on LCD as shown below.

FORWARD – BUSY

XXXXXXXXX 09:00 am

Service conditions

- It is possible to designate whether or not call forward function is allowed.

- Extension subscriber cannot register two or more call forwards at the same time.

- In case call forward registration is requested when call forward has already been registered, previous one is automatically changed.

- If a subscriber who has registered to the function picks up the handset, special dial tone is heard for a while and then dial tone is heard.

- When registering call forward to external subscriber number, trunk access code is not displayed, and maximum 12 digits are displayed.

- It is not possible to forward call to general attendant.

- It is not possible to forward call to hot line subscriber.

- It is not possible to forward call to its own telephone.

- Subscriber "E class or below" cannot register Call Forward to external number.

- In case DND(Do Not Disturb) function is registered, this should be cancelled first before registering the Call Forward.

Data generation

- Whether Call Forward is used

[3.1 Extension Information - Feature Restriction ⇒ Call Forwarding]

- Call Forward special dial tone

[2.4 System Tone Parameter ⇒ Call Forward Alert Tone]

Interactions

- It is not possible register Call Forward using buttons while absence announcement message is registered, but is possible to register Call Forward using codes. Existing absence announcement message is deleted.

- Call Forward cannot be registered when DND(Do Not Disturb) is registered by D-Tel phone.

- Registered Call Forward may be forcibly cancelled by operator if necessary.

- If Absence Message is registered on D-Tel telephone where Call Forward is registered, the Call Forward is cancelled.

- Call Forward should be registered after the DND(Do Not Disturb) is cancelled.

- Toll Check is required to register Call Forward to external subscriber.

## Call Forward – Cancel

Functional description

This function allows the extension subscriber to cancel the registered Call Forward.

Operating procedure

- Canceling Call Forward on general telephone

1	An extension subscriber picks up the handset.

2	Dial the code for Call Forward – Cancel (Default:*520).

3	If deletion is completed normally, service set tone is heard.

- Canceling Call Forward on D-Tel telephone using buttons

1	Pick up the handset or press “SPEAKER” button.

2	Press “DND/CFWD” button.

3	If Call Forward is registered in the terminal, registration information is deleted. And the deletion is normally completed, service set tone is heard and the blinking “DND/CFWD” button LED is turned off.

4	Hang up the handset or press “SPEAKER” button, and then Call Forward registration information is not displayed anymore.

Service conditions

- It is possible to designate whether or not call forward function is allowed.

- Extension subscriber cannot register two or more call forwards at the same time.

- In case call forward registration is requested when call forward has already been registered, previous one is automatically changed.

- If a subscriber who has registered to the function picks up the handset, special dial tone is heard for a while and then dial tone is heard.

- When registering call forward to external subscriber number, trunk access code is not displayed, and maximum 12 digits are displayed.

- It is not possible to forward call to general attendant.

- It is not possible to forward call to hot line subscriber.

- It is not possible to forward call to its own telephone.

- Subscriber "E class or below" cannot register Call Forward to external number.

Data generation

- Whether Call Forward is used

[3.1 Extension Information - Feature Restriction ⇒ Call Forwarding]

- Call Forward special dial tone

[2.4 System Tone Parameter ⇒ Call Forward Alert Tone]

Interactions

- Block tone is heard when Call Forward – Cancel code is pressed while Call Forward is not registered.

- Registered Call Forward can be forcibly cancelled by operator if required.

## Call Forward – Follow Me

Functional description

This function allows extension subscribers to register Call Forward by using an extension terminal in remote place.

Operating procedure

- Registration of Call Forward – Follow Me on remote general telephone using registration code.

1	An extension subscriber picks up the handset.

2	Dial the remote registration code for Call Forward – Follow Me (Default:*52#).

3	Enter the extension number to register Call Forward.

4	Enter the password of the extension telephone to register Call Forward.

5	Enter the extension telephone number to forward calls.

6	In case the telephone number is normal, and if registration is completed normally, service set tone is heard.

- Remote registration of Call Forward – Follow Me on remote D-Tel telephone using buttons

1	Press “SPEAKER” button.

2	Press “DND/CFWD” button. Following message is displayed on LCD.

ENTER CALL FORWARD TYPE

3	Dial “#”, then following message is displayed on LCD.

ENTER A DIAL DIGIT

4	Dial the extension number to register Call Forward – Follow Me to. Following message is displayed in LCD.

ENTER CURRENT PASSWORD

5	Dial the password of the extension number to register Call Forward – Follow Me. Following message is displayed in LCD.

ENTER A STATION NUMBER

6	Dial the extension number of the terminal to which Call Forward – Follow Me is to be registered.

7	If registration is completed normally, service set tone is heard.

“DND/CFWD” button LED is turned on and blinks in the terminal.

8	Follow Me is registered. Following message is displayed on LCD.

FORWARD – ALL

XXXX 09:00 am

Service conditions

- It is possible to designate whether or not call forward function is allowed.

- Call Forward – Follow Me is available only between extension numbers.

- Extension subscriber cannot register two or more call forwards at the same time.

- In case call forward registration is requested when call forward has already been registered, previous one is automatically changed.

- If a subscriber who has registered to the function picks up the handset, special dial tone is heard for a while and then dial tone is heard.

- In case DND (Do Not Disturb) function is registered, this should be cancelled first before the Call Forward registration.

Data generation

- Whether Call Forward is used

[3.1 Extension Information - Feature Restriction ⇒ Call Forwarding]

- Call Forward special dial tone

[2.4 System Tone Parameter – Call Forward Alert Tone]

Interactions

- It is not possible register Call Forward using buttons while absence announcement message is registered, but is possible to register Call Forward using codes. Existing absence announcement message is deleted.

- Call Forward cannot be registered when DND(Do Not Disturb) is registered by D-Tel phone.

- Registered Call Forward may be forcibly cancelled by operator if necessary.

- If Absence Message is registered on D-Tel telephone where Call Forward is registered, the Call Forward is cancelled.

- Call Forward should be registered after the DND(Do Not Disturb) is cancelled.

## Call Forward – No Answer

Functional description

The function allows extension subscribers to have calls forwarded to other designated extension subscriber or external subscriber when an extension subscriber gives no answer.

Operating procedure

- Registration of Call Forward – No Answer to extension subscriber on general telephone

1	An extension subscriber picks up the handset.

2	Dial the registration code for Call Forward – No Answer (Default:*523).

3	Enter the extension telephone number to forward calls.

4	In case the telephone number is normal, and if registration is completed normally, service set tone is heard.

- Registration of Call Forward – No Answer to external trunk subscriber on general telephone

1	An extension subscriber picks up the handset.

2	Dial the registration code for Call Forward – Busy (Default:*523).

3	Dial trunk access code.

4	Enter the external telephone number.

5	Hang up the handset after entering the telephone number.

- Registration of Call Forward – No Answer to extension subscriber on D-Tel telephone using buttons

1	Press “SPEAKER” button.

2	Press “DND/CFWD” button. Following message is displayed on LCD.

ENTER CALL FORWARD TYPE

3	Dial “3”, then following message is displayed on LCD.

ENTER A DIAL DIGIT

4	Dial extension number.

5	In case the extension number is normal, and if registration is completed normally, service set tone is heard and “DND/CFWD” button LED is turned on and blinks. The following message is displayed on LCD.

FORWARD – NO ANSWER

XXXX 09:00 am

- Registration of Call Forward – Busy to external trunk subscriber on D-Tel telephone using buttons

1	Press “SPEAKER” button.

2	Press “DND/CFWD” button. Following message is displayed on LCD.

ENTER CALL FORWARD TYPE

3	Dial “3”, then following message is displayed on LCD.

ENTER A DIAL DIGIT

4	Dial trunk access code. In case the trunk access code is incorrect, alarm tone is heard. In this case, release D-Tel telephone and restart registration.

5	Dial external subscriber number.

6	Enter the external subscriber telephone number to forward the call, and press “HOLD/SAVE” button, then service set tone is heard, and then “DND/CFWD” button LED is turned on and blinks.

7	Hang up the handset or press “SPEAKER” button, and then registration information is displayed on LCD as shown below.

FORWARD – NO ANSWER

XXXXXXXXX 09:00 am

Service conditions

- It is possible to designate whether or not call forward function is allowed.

- Extension subscriber cannot register two or more call forwards at the same time.

- In case call forward registration is requested when call forward has already been registered, previous one is automatically changed.

- If a subscriber who has registered to the function picks up the handset, special dial tone is heard for a while and then dial tone is heard.

- When registering call forward to external subscriber number, trunk access code is not displayed, and maximum 12 digits are displayed.

- It is not possible to forward call to general attendant.

- It is not possible to forward call to hot line subscriber.

- It is not possible to forward call to its own telephone.

- Subscriber "E class or below" cannot register Call Forward to external number.

- In case DND(Do Not Disturb) function is registered, this should be cancelled first before the Call Forward registration.

Data generation

- Whether Call Forward is used

[3.1 Extension Information - Feature Restriction ⇒ Call Forwarding]

- Call Forward special dial tone

[2.4 System Tone Parameter ⇒ Call Forward Alert Tone]

Interactions

- It is not possible register Call Forward using buttons while absence announcement message is registered, but is possible to register Call Forward using codes. Existing absence announcement message is deleted.

- Call Forward cannot be registered when DND(Do Not Disturb) is registered by D-Tel phone.

- Registered Call Forward may be forcibly cancelled by operator if necessary.

- If Absence Message is registered on D-Tel telephone where Call Forward is registered, the Call Forward is cancelled.

- Call Forward should be registered after the DND(Do Not Disturb) is cancelled.

- Toll Check is required to register Call Forward to external subscriber.

## Call Transfer

Functional description

This function allows public subscribers or attendant subscribers to transfer calls with another extension or trunk to other extension subscriber.

Operating procedure

- Unscreened Transfer on extension.

1	Subscriber (A) is on line with other subscriber (B).

2	Subscriber (A) presses “Hook Flash” or “TRANS”(Transfer) button.

3	Subscriber (B) is held and hears holding tone.

4	Dial the telephone number of extension subscriber (C).

5	Ringing tone is transmitted to subscriber (A), and ringing signal is transmitted to subscriber (C).

6	Subscriber (A) hangs up the handset.

7	Subscriber (B) hears ringing tone, and ring signal is transmitted to subscriber (C).

8	Subscriber (C) replies.

9	Call is made between subscribers (B) and (C).

- Screened Transfer on extension line.

1	Subscriber (A) is on line with other subscriber (B).

2	Subscriber (A) presses “Hook Flash” or “TRANS”(Transfer) button.

3	Subscriber (B) is held and hears holding tone.

4	Dial the telephone number of extension subscriber (C).

5	Ringing tone is transmitted to subscriber (A), and ringing signal is transmitted to subscriber (C).

6	Subscriber (C) replies.

7	Call is made between subscribers (B) and (C).

8	Subscriber (A) hangs up the handset.

9	Call is made between subscribers (B) and (C).

- Unscreened Transfer on attendant.

1	Operator dials the telephone number of an extension subscriber while on call with trunk subscriber.

2	Status of extension subscriber is displayed on attendant LCD.

3	In case the extension subscriber is IDLE, operator presses

“CONNECT” (Connection) key.

4	Attendant is released with its LCD cleared, the trunk subscriber hears ringing tone, and ringing signal is transmitted to the extension subscriber.

5	Extension subscriber replies.

6	Call is made between the trunk subscriber and the extension subscriber.

Service conditions

- When attendant subscriber presses “CONNECT”(Connection) key when an extension subscriber is on line, the status of call with trunk subscriber is maintained and Error is displayed on LCD.

Interactions

- If subscriber (A) dials incorrect telephone number or calls a subscriber on line, lockout tone is heard. Pressing “Hoof Flash” or “TRANS”(Transfer) button in this stage resumes the call with the subscriber (A).

- If subscriber (A) dials incorrect telephone number or calls a subscriber on line, lockout tone is heard. If the handset is hung up at this stage, ringing is made on subscriber (B) to subscriber (A), and call is made if (A) replies.

- If (C) made no reply when (A) transfers (B) to (C) in Unscreened Transfer, (B) is terminated to (A) again.

- Subscriber can transfer the other subscriber to operator.

## Call Waiting

Functional description

If it is necessary to make an urgent call to an extension subscriber engaged in the other call, the function allows the calling subscriber to dial Call Waiting code to inform the called subscriber that there is an urgent call waiting. The called subscriber engaged in the other call hears the alarm tone, can have the other subscriber who have been calling to standby, and can make a call with the calling subscriber.

Operating procedure

- Call Waiting on general telephone

1	Extension subscriber (A) picks up the handset.

2	Dial the telephone number of extension subscriber (B) who is engaged in the other call.

3	Busy tone is heard.

4	Dial the code of Call Waiting (Default:5).

5	Extension subscriber (A) becomes held and hears holding tone, and the subscriber (B) hears alarm tone while engaging in the other call.

6	Subscriber (B) presses Hook Flash.

7	Subscriber (C) who has been talking with subscriber (B) over the phone becomes waiting and hears holding tone. Call is made between subscriber (A) and subscriber (B).

8	Subscriber (B) presses Hook Flash again.

9	Call resumes between subscriber (B) and subscriber (C) who has been held, and subscriber (A) becomes held again.

- Call Waiting on D-Tel telephone

1	D-Tel subscriber (A) picks up the handset.

2	Dial the telephone number of extension subscriber (B) who is engaged in the other call.

3	Busy tone is heard.

4	Dial the code of Call Waiting (Default:5).

5	D-Tel subscriber (A) becomes held and hears holding tone, and the subscriber (B) hears alarm tone while engaged in the other call.

6	Extension subscriber (B) presses “TRANS”(Transfer) button or “HOLD/SAVE” button.

7	Subscriber (C) who has been engaged in the call with subscriber (B) becomes waiting and hears holding tone. Call is made between subscriber (A) and subscriber (B).

8	Subscriber (B) presses “TRANS”(Transfer) button or “HOLD/SAVE button.

9	Call resumes between subscriber (B) and subscriber (C) who has been held, and subscriber (A) becomes held again.

Service conditions

- This function is not performed when Step Call is used in the system.

- Call Waiting is not made to the subscriber engaged in Privacy call.

- If the called subscriber is not engaged in 1 to 1 call, lockout tone is heard upon the Call Waiting request. In other words, this function is not available when the called subscriber is already in Call Waiting, conference call, or when the called subscriber is receiving ring signal.

- This function is not available if the called subscriber is an operator.

- When a subscriber is held, and the other two subscribers are engaged in a call, if a subscriber engaged in the call hangs up the handset, call is automatically made between the subscriber on held and the other subscriber.

- If the called subscriber hangs up the handset while holding tone is heard for Call Waiting, Ringing tone is transmitted to the subscriber on held, and ring signal is transmitted to the called subscriber.

Data generation

- Designate whether Step Call is used in the system

[2.6 System Feature Option ⇒ Step Call Use]

- Designate whether to use Auto Privacy

[3.1 Extension Information - Extension Characteristic ⇒ Auto. Privacy]

- Designate the digit that performs the same procedure as H/F

[2.6 System Feature Option ⇒ DP H/F Digit Type]

Interactions

- In case the subscriber who received Call Waiting while engaged in the call presses a digit, if the digit is the same as the value in [2.6 System Feature Option-DP H/F Digit Type], the same procedure as H/F is performed.

- If the called subscriber is engaged in a call, the services are provided in the order of Auto Group Hunting / Auto Hold / Auto Call Waiting / Auto Camp On.

## Calling Group Name Display

Functional description

In case the called number of all kinds of trunk calls (DIL, DISA, DDI, MSN, Sub Address) is decided as the representative number of the calling group, and when the called extension is either D-Tel or attendant, the name of the group is displayed on LCD with ringing instead of the trunk number.

Operating procedure

Service conditions

- Name should be designated for the calling group called.

- To display the name, it should be decided whether to use the function to each D-Tel.

- Attendant will display name.

Data generation

- Whether to display D-Tel name

[3.1 Extension information] – Extension characteristics information, 11.CLI/CNI Display

- Whether to display DECT name

[9.3 DECT Information] – 11.CLI/CNI Display of DECT Characteristics

- Calling group name designation

[6.4 Calling Group] – Rep. Name

- Assign of individual called party

[6.2 DIL Called Group]

- Assign of Line DISA

[4.1 Trunk Information] – 2. DISA trunk of Trunk Line Option.

- DDI DISA designation

[6.4 Calling group] – Designate service type to DISA type

- Assign of MSN, Sub Address destination

[7.3 MSN/Sub-Address Destination Assign]

Interactions

- Calling Party Name Search

This function is provided if the name is found in case of ISDN trunk call.

- Calling Name Identification Presentation (CNIP)

- CNIP function is provided in case of QSIG.

## Calling Party Name Search

Functional description

This function compares the calling number of trunk call and the external subscriber number in the system speed dial code, and if the numbers are the same, it displays the name in the speed dial code on LCD instead of the trunk number.

Operating procedure

Service conditions

- This function is limited to ISDN trunk so far.

- To display the name, it should be decided whether to use the function to each D-Tel.

- Attendant will display name.

Data generation

- Whether to display D-Tel name

[3.1 Extension Information] – Extension characteristics information, 11.CLI/CNI Display

- Whether to display DECT name

[9.3 DECT Information] – DECT Characteristic, 11.CLI/CNI Display

- Assign of system speed dial code plan

[2.1 System Numbering Plan] – System Speed Dial Code 1 of function code plan, System Speed Dial Code 2

- Assign of system speed dial code name

[16.7 System Speed Dial] – Destination Name

Interactions

- Calling Group Name Display

In case of ISDN trunk call, Calling Group Name Display function is provided.

- Calling Name Identification Presentation (CNIP)

In case of QSIG, CNIP function is provided.

## Calling/Connected Line Identification (CLIP, COLP)

Functional description

This function displays calling party number and called party number on the LCD of D-Tel, ATD, DECT subscribers.

Operating procedure

- Extension call calling party/called party number display

1	Extension subscriber 2000 calls extension number 3000.

- “CALL TO 3000” is displayed on the LCD of 2000.

- “CALL FROM 2000” is displayed on the LCD of 3000.

2	Extension subscriber 3000 answers.

- “CONNECT TO 2001” is displayed on both LCDs of 2000 and 3000.

- Trunk calling party/called party number display

1	In case of termination

- When calling party number is received, calling party number and the trunk line number are displayed on called party LCD. But in case of ISDN and QSIG trunks, if Calling Line identification Restriction (CLIR) is requested, “CLIR” is displayed on LCD.

- If calling party number is not received, “LINE RINGING” and the trunk line number are displayed on called party LCD.

2	In case of origination

- In case called party number is received, called party number and the trunk line number are displayed on calling party LCD. But in case of ISDN and QSIG trunks, if Connected Line Identification Restriction (COLR) is requested, “COLR” is displayed on LCD.

- If called party number is not received, calling party subscriber dialed number and the trunk line number are displayed on the calling party LCD.

- Restriction and allowance of calling/called party number by extension subscribers.

1	Off Hook.

2	Press the function code.

3	Enter option. (0 : Restrict transmission, 1: Allow transmission)

4	Function registration completion tone is heard.

5	On Hook.

Service conditions

- Types of trunk should be as below to display calling party and called party numbers of trunk.

1	ISDN and QSIG trunk call that includes calling party/called party number.

2	R2 trunk including calling party number.

3	VOIP trunk including calling party number.

- To display calling party number and called party number received from trunk call, whether to use the function should be designated in each D-Tel. Attendant shall be displayed in any case.

- In case of R2 trunk call, it is possible to designate whether to request calling party number to the opposite station by subscribers and by calls.

- In case of ISDN, QSIG trunk, it is possible to restrict or allow calling party number or called party number transmission in the whole system or by extension subscribers.

- In case of VOIP trunk call, it is not possible to restrict calling party number and called party number transmission.

- It is possible to select the types of ISDN calling party/called party number as below.

|  | Information included |
| --- | --- |
| International | Country Code + Own Area Code + DDI Prefix + Ext. No |
| National | Own Area Code + DDI Prefix + Ext. No |
| Subscriber | DDI Prefix + Ext. No |
| Unknown | Ext. No |

In case of dedicated line, the number is transmitted in unknown type irrespective of option. And in case Own PBX code is generated in the system, numbers are transmitted in Own PBX Code + Ext. No type.

Data generation

- Whether to display D-Tel calling party/called party number

[3.1 Extension line information] – Extension characteristics information  11.ANI Display Subscriber

- Whether to display DECT calling party/called party number

[9.3 DECT Information] – DECT Characteristic 11.CLI/CNI Display

- Whether to request R2 calling party number by routes

[4.9 Overall option by trunk routes] – ANI Information request.

- Whether to request R2 calling party number by subscribers

[3.1 Extension information] – Extension characteristics information  11.Malice Trace

- Whether to request R2 calling party number by calling groups

[6.4 Calling Group] – Calling group sub information R2 ANI Request

- Whether to allow/restrict ISDN trunk calling party/called party number transmission

[7.2 ISDN Option] – Line Identification Restriction

- Assign of ISDN trunk calling party/called party number type

[7.2 ISDN Option] – Line Identification Sending Type

- Whether to allow/restrict QSIG trunk calling party/called party number transmission

[7.2 QSIG Option] – Line Identification Restriction

- Assign of Country Code, Own Area Code, DDI Prefix, Own PBX Code

[1.1 System Installation Information]

- Whether to allow/restrict connection between routes

[4.2 Trunk Route Connection Allow/Restrict Option]

Interactions

- Whether to allow transit of calling party /called party number

|  | R2 | ISDN | QSIG | VOIP |
| --- | --- | --- | --- | --- |
| R2 | YES | YES | YES | YES |
| ISDN | YES | YES | YES | YES |
| QSIG | YES | YES | YES | YES |
| VOIP | YES | YES | YES | YES |

## Camp On

Functional description

The function allows a subscriber to inform a waiting call to a extension subscriber on line with other extension subscriber, and to wait for the answer from the called subscriber.

Operating procedure

- Camp On from general telephone

1	Extension subscriber (A) calls a extension subscriber (B) in busy.

2	Busy tone is heard.

3	Press Camp On Registration Feature Code (Default:6).

4	Extension subscriber (A) transmits holding tone to extension subscriber (A), and alarm tone to extension subscriber (B).

5	Extension subscriber hangs up the handset.

6	Ring back tone is transmitted to extension subscriber (A), and ring current is transmitted to extension subscriber (B).

7	Extension subscriber (B) picks up the handset.

8	Call is performed between extension subscribers (A) and (B).

- Camp On form D-Tel telephone

1	D-Tel subscriber (A) calls a extension subscriber (B) in busy.

2	Busy tone is heard as (B) subscriber is on line.

3	Press Camp On Registration Feature Code (Default:6).

4	D-Tel subscriber (A) transmits holding tone to extension subscriber (A), and alarm tone to extension subscriber (B).

5	D-Tel subscriber hangs up the handset.

6	Ring back tone is transmitted to D-Tel subscriber (A), and ring current is transmitted to extension subscriber (B).

7	(B) picks up the handset.

8	Call is performed between D-Tel subscriber (A) and (B).

- Camp On from attendant

1	Extension subscriber (A) is on line with subscriber (B).

2	Attendant is on line with extension subscriber (C) or trunk subscriber (C).

3	Attendant calls extension subscriber (A).

4	Busy tone is heard as subscriber (A) is on line.

5	Press “CAMP ON” Key.

6	Attendant is released. Holding tone is transmitted to subscriber (C) and alarm tone is transmitted to subscriber (A).

7	Subscriber (B) hangs up the handset.

8	Ring back tone is transmitted extension subscriber (A), and ring current is transmitted to extension subscriber (C).

9	If subscriber (A) picks up the handset, call is made between subscriber (A) and (B).

Service conditions

- This function is not performed when Step Call is used in the system.

- This function is not provided if Camp On function is not used by the connecting subscriber, although Camp On is available in the system.

- Camp On cannot be registered to the subscriber designated as Hot Line.

- Camp On cannot be registered to the call group.

- 20 Camp On registrations are available for a subscriber.

- If the subscriber who requested Camp On does on hook, Camp On function is cancelled.

- It is possible to designate whether to transmit alarm tone to the subscriber the Camp On is requested to.

- Alarm tone is transmitted at regular intervals to the subscriber the Camp On is requested to. This time interval is adjustable by operation work.

- In case the country code is Italy, Camp On service is not automatically performed when the opposite side is busy, but is performed only by pressing Camp On request code (Default:6).

Data generation

- Determine whether to use Step Call in the system

[2.6 System Feature Option ⇒ Step Call Use]

- Determine whether to use automatic Camp On

[2.6 System Feature Option ⇒ Auto Camp On Use]

- Determine whether to allow Camp On function when required

[3.1 Extension Information – Extension Characteristic ⇒ 33 Camp On]

- Determine whether to transmit alarm to the opposite side when Camp On is requested from station line

[2.6 System Feature Option ⇒ Send CampOn Alarm from EXT]

- Determine whether to transmit alarm to the opposite side when attendant requests Camp On

[2.6 System Feature Option ⇒ Send CampOn Alarm from ATD]

- Designate the digit that performs the same work with H/F

[2.6 System Feature Option ⇒ DP H/F Digit Type]

Interactions

- In case the subscriber who is requested of Camp On presses Hook Flash, Off-Hook or On-Hook process is performed according to [2.6 System Feature Option-Hook Flash].

- If the called subscriber is in service, the function is performed in the order of Auto GroupHunting/ AutoHold/ AutoCall Waiting/ Auto Camp On.

## Changeable System Ring Parameter

Functional description

This function allows all kinds of rings defined in the system to be operated by changing ring cadence and ring lock.

Operating procedure

Service conditions

Data generation

- System Ring Parameter determine

[2.5 System Ring Parameter]

Interactions

## Changeable System Tone Parameter

Functional description

This function allows all kinds of tones defined in the system to be operated by changing types (System basic tone, Announcement) and times (Tone lock).

Operating procedure

Service conditions

- All tones are available to substitute by announcement line.

Data generation

- System Tone Parameter determination

[2.4 System Tone Parameter]

Interactions

## Class Of Service (COS)

Functional description

Each the extension and trunk subscriber can have one of 8 classes of service by day/night (From top to bottom : Class I, Class S, Class A, Class B, Class C, Class D, Class E, and Class F). This function decides whether to allow/restrict the following services by the classes.

- Trunk originating digit allow/restrict service

Class I : Available to designate allowed/restricted trunk originating digit by the classes.

Class S : Available to designate allowed/restricted trunk originating digit by the classes.

Class A : Available to designate allowed/restricted trunk originating digit by the classes.

Class B : Available to designate allowed/restricted trunk originating digit by the classes.

Class C : Available to designate allowed/restricted trunk originating digit by the classes.

Class D : Available to designate allowed/restricted trunk originating digit by the classes.

Class E : Restrict all the trunk origination.

Class F : Restrict all the extension, trunk origination.

- Trunk origination digit allow/restrict operation mode

Origination is restricted if the digits of a subscriber originated external trunk subscriber number match the digits designated in the restriction table of the class. If the dialed digits match the maximum digits designated in the allowance table, even when the digits match the restriction table, origination is allowed.

Example) When ‘0’ is designated in deny table, and “011”, “012”, and “019” are designated in the allowance table,

Examples of restricted originating digits : 02, 03, 010, 014, 018

Examples of allowed originating digits : 0112~, 0125~, 0198~

- DID termination restriction service by classes

If the class of termination subscriber isn't higher than the class of termination allowance class of each trunk, termination of the call is restricted. In this case, the termination restricted trunk call can be processed for termination to attendant by the designated abnormal trunk I/C call process for route where the trunk is included.

- Restriction on direct call between managers

Direct call is restricted when a manager subscriber calls another manager subscriber with the class higher than himself. In this case the call is terminated to the secretary of the called manager subscriber. On the contrary, if the class of the called manager subscriber is higher than or same as the calling manager subscriber, the call is directly terminated to the called manager subscriber.

Operating procedure

Service conditions

- A subscriber can have one day class and one night class.

- Day class or night class is applied according to the day/night mode of the system.

- Attendant subscriber is not restricted of service by its class.

Data generation

- Assign of day/night class of extension subscriber

[3.1 Extension Information]

- Assign of day/night class for trunk subscribers

[4.1 Trunk Information ⇒ Day Class and Night Class]

- Origination digit allowance/restriction table by subscriber classes for trunk origination

[3.3 Class Base OG Digit Allow/Deny]

- DDI termination allowance subscriber class

[4.1 Trunk Information ⇒ IC Allow Class]

Interactions

- System Speed Dial

Incase of system speed dial, restriction is designated according to the speed dial. Therefore, it is exempted from the origination digit restriction service for trunk origination. In other words, system speed dial allowed to all subscribers excluding class E and class F can be used irrespective of the designated external number.

- ATD Through Dial

In case an extension subscriber originates through attendant subscriber, the subscriber’s class added by the designated ascending class is applied by the origination digit allowance/restriction function of subscriber class per route.

If the class of an extension subscriber is “D” and the class ascending step is “2”, origination restriction/allowance function of class “B” is applied.

## Collect Call

Functional description

When the call terminated to attendant from trunk requests charging to receiver, this function registers the related information.

Operating procedure

- Registration of receiver charging information from attendant.

1	Attendant is in line with external trunk.

2	Enter the extension subscriber number.

3	Press “CHRG” key.

4	Trunk subscriber becomes held and hears holding tone, attendant subscriber hears ringing tone, and extension subscriber hears ring current.

5	If extension subscriber answers, attendant asks whether to receive the collect call, and if extension subscriber wants to receive, presses “CHRG” key.

6	Enter the information of the collect call.

7	Enter the number of external subscriber.

8	Enter “ENT" key.

9	Attendant is released with its LCD cleared, and call is made between the trunk subscriber and the extension subscriber.

- In case the extension subscriber refuses to receive the collect call.

1	Attendant is in line with external trunk.

2	Enter the extension subscriber number.

3	Press “CHRG” key.

4	Trunk subscriber becomes held and hears holding tone, attendant subscriber hears ringing tone, and extension subscriber hears ring current.

5	If extension subscriber answers, attendant asks whether to receive the collect call, and if extension subscriber wants to receive, presses “CHRG” key.

6  Extension subscriber refuses Collect Call.

7	Press “CLR” key to make call with the trunk subscriber.

- In case of mistype, operator can press “CLR” key and start entering from beginning.

Service conditions

- Collect Call Information :

1 : Extension to Extension Collect Call

2 :	Person to Person Collect Call

3 :	Collect Call

- 11.1 Charge/Printer Option : Designate the device for charging and charging result display/print.

- 3.1 Extension Information – Extension Characteristic – 38 Charge Send to ATD : Determines whether to transfer the charging result or each subscriber to attendant.

## Conference

Functional description

This function allows an extension subscriber or a trunk subscriber on line to call other extension subscriber to have conference call.

Operating procedure

- Conference from general telephone

1	Extension subscriber (A) presses Hook Flash while on line.

2	The opposite subscriber (B) becomes hold state and hears holding tone, and extension subscriber (A) hears ringing tone.

3	Dial the code for Conference (Default:#5) and the telephone number of the other extension subscriber (C).

4	Extension subscriber (A) hears ringing tone, and extension subscriber (C) hears ring current.

5	Extension subscriber (C) answers.

6	Conference call is performed between extension subscriber (A), (B), and (C).

- Conference from D-Tel telephone.

1	D-Tel subscriber (A) presses “CONF”(Conference) button while on the line.

2	The opposite subscriber (B) becomes hold state and hears holding tone, and D-Tel subscriber (A) hears ring current.

3	Dial the telephone number of subscriber (C) to have conference with.

4	D-Tel subscriber (A) hears ringing tone, and extension subscriber (C) hears ring current.

5	Extension subscriber (C) answers.

6	Conference call is performed between D-Tel subscribers (A), (B), and (C).

- Conference from attendant

1	Attendant dials the telephone number of extension subscriber who is willing to perform conference call while on call with extension subscriber or trunk subscriber.

2	Status of dialed extension subscriber is displayed on attendant LCD.

3	Press “CONF”(Conference) key if extension subscriber is not in the service.

4	Station/trunk subscriber on line becomes hold status and hears holding tone, and ring tone is transmitted to attendant subscriber, and ring current is transmitted to other extension subscriber.

5	Extension subscriber answers.

6	Three way conference call is performed between station/trunk subscriber, attendant, and the other extension subscriber.

Service conditions

- This function is only available for the subscribers allowed for conference call.

Data generation

- Whether conference call is allowed

[3.1 Extension Information – Feature Restriction ⇒ 4. Conference]

- Minimum time required to press “Hook Flash” or “Transfer button” to transfer Consultation to Conference

[2.3 System Lock Parameter ⇒ Second Hookflash Wait Lock]

Interactions

- When require conference call, if extension subscriber (C) is in service, extension subscriber (A) hears blocking tone, and the call with opposite subscriber (B) is recovered by “Hook Flash” or “TRANS”(Transfer button).

- If conference call circuits are all engaged, extension subscriber (A) hears blocking tone, and the call with (B) is recovered by “Hook Flash” or “TRANS”(Transfer) button.

- Calls between other subscribers are maintained even when any one subscriber is released during the conference call.

- Conference call is performed with “Hook Flash”, “TRANS”(Transfer) button, or by dialing subscriber telephone number. Conference call status is maintained for other subscribers. 3

- If extension subscriber presses “Hook Flash” or “TRANS”(Transfer) button once during the conference call, Brokers Call is performed. If “Hook Flash” or “TRANS”(Transfer) button is pressed twice, conference call is performed.

- If operator presses “CLR”(Clear) key while hearing called ring tone, conference call is cancelled and previous call status is recovered.

- If a subscriber hangs up the handset while on conference call with operator, call is performed between the operator and the other subscriber.

- If the subscriber hearing holding tone is released, conference call request is cancelled, and the operator switches to a type of normal extension subscriber call.

- If the operator presses “CONF”(Conference) key while the called extension subscriber is in the service, the call is maintained and Error is displayed on attendant LCD.

- If the operator presses “RLS”(Release) key while ringing tone is being transmitted to extension subscriber, ring signal of extension subscriber finishes and conference call is cancelled. At this stage, calls of other subscribers who have heard the holding tone are released and congestion tone is transmitted.

- If the operator presses “RLS”(Release) key during the conference call, or if any one of the two subscribers hangs up the handset, it is transferred to the call status between the subscribers or between the operator and the subscriber.

- If the operator presses “CONF”(Conference) button while call with trunk by Through Dialing (by dialing trunk number), three way conference call is performed between the operator, trunk subscriber, and extension subscriber.

- If the operator presses “CONF”(Conference) button while on Attendant Consultation Transfer, three way conference call is performed between the operator and the two subscribers.

## Consultation Transfer to Conference

Functional description

This function allows an extension subscriber to hold a subscriber on line, and call other subscriber for consultation transfer, and performs three ways conference call.

Operating procedure

- Consultation Transfer to Conference Call from general telephone/D-Tel telephone.

1	Extension subscriber (A) is on line with other extension subscriber or trunk subscriber (B).

2	Subscriber (A) presses Hook Flash for transfer.

3	Subscriber (B) becomes held and hears holding tone, and subscriber (A) hears 2nd dial tone.

4	Subscriber (A) dials the number of subscriber (C).

5	Subscriber (A) receives ringing tone, and subscriber (C) receives ring current.

6	Subscriber (C) picks up the handset.

7	Subscriber (A) and subscriber (C) determine whether to perform conference call from consultation call.

8	Subscriber (A) presses “Hook Flash” or “TRANS”(Transfer) button two times.

9	Three ways conference call is performed between subscriber (A), subscriber (B), and subscriber (C).

Service conditions

- If subscriber (A) refuses to enter into conference call and hangs up the handset, general call transfer service is performed.

- Consultation transfer function key when there is not idle conference call port generates blocking tone.

- If the subscriber on consultation call presses “Hook Flash” and “TRANS”(Transfer) button once, Broker Call is performed in which the call is made with the subscriber on hold and the subscriber on line becomes held. Conference call is performed with two pressings on “Hook Flash” or “TRANS”(Transfer) button.

Data generation

- Press 2 times hook flash in predefined time for conference service (only SLT)

[2.3 System Lock Parameter ⇒ Second Hookflash Wait Lock]

Interactions

## Consultation Transfer

Functional description

This function allows extension subscribers or attendant subscribers to hold a subscriber on line, call other subscriber and perform consultation call and determine whether to transfer call, and perform call transfer.

Operating procedure

- Consultation transfer from general telephone / D-Tel telephone

1	Extension subscriber (A) is on line with other extension or trunk subscriber (B).

2	Subscriber (A) press “Hook Flash” or “TRANS”(Transfer) button.

3	Subscriber (B) becomes held and hears holding tone, and subscriber (A) hears 2nd dial tone.

4	Subscriber (A) dials the number of subscriber (C).

5	Subscriber (A) receives ringing tone, and subscriber (C) receives ring current.

6	Subscriber (C) picks up the handset.

7	Consultation call is made between subscriber (A) and subscriber (C), and decide whether to perform transfer.

8	Subscriber (A) hangs up the handset.

9	Call is made between subscriber (B) and subscriber (C).

- Consultation transfer from attendant

1	An attendant subscriber is on line with an extension subscriber (A).

2	The attendant subscriber dials the number of subscriber (B).

3	The attendant subscriber dials “*”.

4	Extension subscriber (A) hears holding tone, and subscriber (B) hears ringing tone.

5	If extension subscriber (B) replies, consultation call with attendant is performed.

6	Attendant subscriber dials “*”.

7	Call is made between attendant subscriber and the subscriber on hold, and the subscriber on line hears holding tone.

8	Attendant subscriber presses “CONNECT”(Connection) key.

9	Call is made between subscriber (A) and subscriber (B).

Service conditions

- If the subscriber (A) who tried consultation call hangs up the handset, general call transfer service is performed.

- If the subscriber on consultation call presses “Hook Flash” and “TRANS”(Transfer) button once, Broker Call is performed in which the call is made with the subscriber on hold and the subscriber on line becomes held. Conference call is performed with two pressings on “Hook Flash” or “TRANS”(Transfer) button.

- If attendant subscriber presses “RLS”(Release) key while on consultation call, all calls are released.

## Consultation Transfer

Functional description

“Same as COMMON”

Operating procedure

- Consultation transfer from general telephone / D-Tel telephone

“Same as COMMON”

- Consultation transfer from attendant

1	An attendant subscriber is on line with an extension subscriber (A).

2	The attendant subscriber dials the number of subscriber (B).

3	Extension subscriber (A) hears holding tone, and subscriber (B) hears ringing tone.

4	If extension subscriber (B) replies, consultation call with attendant is performed.

5	Attendant subscriber presses “CONNECT”(Connection) key.

6	Call is made between subscriber (A) and subscriber (B).

Service conditions

- When pressing “RLS”(Release) key during consultation call of attendant, the caller is connected to the subscriber who is hearing holding tone and the subscriber in busy is released.

## DDI Divert

Functional description

The function diverts all DDI trunk calls to other pre-designated subscriber, not connecting the calls to the called party when the system termination mode is set as night or holiday.

Operating procedure

Service conditions

- When DDI Divert service is activated, the destination call is decided by the system incoming mode..

- For description on general Direct Dialing In (DDI) functions, refer to the “Direct Inward Dialing”.

Data generation

- DDI divert designation

[4.9 Trunk Route Base All Option ⇒ Use of DDI divert function]

- Assign of termination types of system termination modes

[16.3 Night Bar]

Interactions

## DDI Incoming Digit Remove & Convert

Functional description

The digit information of the DDI trunk calls coming from the originating station is subject to the numbering plan of the station. Especially, when different stations are connected to each route, incoming numbers can not be composed of consistent digits in length. Under the circumstances, the system numbering plan is established in a restricted manner or impossible. Incoming digit remove & convert support the flexible system numbering plan, overcoming the disadvantages. Users may operate one of the two solutions or both of them at the same time, considering the characteristics of the installation areas.

- Incoming digit remove

The function is used to remove some digits or inconsistent digits from the incoming digits in the following two ways.

1	To remove digits exceeding the fixed length unconditionally

Remove the digits that exceed the fixed length from the DDI incoming digits unconditionally and analyze the incoming digits.

Example) The extension number plan is “5XXX” and the incoming digits are “02-818-5XXX”, set the length of the digits to remove as “5”.

2	To remove matched digits from the incoming digits that coincide with the planned digits

Remove matched digits from the DDI incoming digits that contain the planned digits and analyze the incoming digits.

Example) The extension number plan is “5XXX” and the incoming digits are “02-818-5XXX”, “818-5XXX” or “5XXX”, set the digits to remove as “02-818” or “818”.

- Incoming digit convert

The function is used to delete/change/add up to 6 digits from/to the incoming digits.

Example) Deletion : 12345 -> 123

Addition : 123  -> 12345

Change : 12345 -> 12367

Operating procedure

Service conditions

1	DDI incoming digit remove & convert can be operated per each trunk route.

2	In the second method of DDI incoming digit remove, the digits that are shorter than the planned digits and partially coincide with other extension numbers are not available.

Example) When “82-02-818” are designated as digits to be removed and the extension number “8202” exists

3	In the DDI incoming digit convert, the digits that are shorter than the planned digits and partially coincide with other extension numbers are not available.

Example) When 12345 should be changed to 12367 and the extension number “1234” exists

Data generation

- Trunk route separation

[4.1 Trunk Information ⇒ Trunk Route]

[4.7 Trunk Route Assign]

- Unconditional incoming digit remove of each trunk route

[4.3 Trunk Incoming Digit Remove ⇒ Unconditional Digit Remove]

- Remove when the incoming digits coincide with the planned digits

[4.3 Trunk Incoming Digit Remove ⇒ Matched Digit Remove]

- Incoming digit convert per trunk route

[4.4 Trunk Incoming Digit Convert]

Interactions

## Dial Memo

Functional description

The function enables the user to register the telephone number of another subscriber on the D-Tel while talking on the phone with one external subscriber.

Operating procedure

- To register the dial memo on the D-Tel

1	D-Tel subscriber is talking on the phone with one external subscriber.

2	Press the “MEMORY” button.

3	Press the “PROGRAM” button.

4	Dial the trunk access code.

5	Dial the telephone number of another external subscriber.

6	Press "HOLD/ENT" button.

7	The above procedure is completed, the following message is displayed on the LCD.

** COMPLETED **

REDIAL IS REGISTERED

8	Keep talking with the subscriber.

- To use the dial memo registered on the D-Tel

1	Press the “MEMORY” button.

2	The following message is displayed on the LCD.

STATION XXXX

MEMORY 09:00 am

3	Dial “#”.

4	The registered phone number is automatically dialed.

5	The ring back tone is heard and the called party hears the ring.

6	When the called external subscriber answers the phone, conversation can be made.

Service conditions

- The function can be used only in the D-Tel.

- The extension number can not be registered.

Interactions

- The same memory is used for both Saved Number Redial and Dial Memo. Therefore, if the function is registered, the telephone numbers saved by the Saved Number Redial are automatically deleted.

## Digital Direct Station : DDS

Functional description

The function expands the flexible buttons on the D-Tel connected with DDS terminal to call the registered extension subscribers with only one button touch and enable LED-based monitor of subscriber status.

Service conditions

- DDS can be connected only to DSLC-LGP and DSLC-LKD boards for use.

- Up to 64 DDSs can be connected to the system.

- Up to 4 DDS terminals can be connected to one D-Tel terminal.

- DDS can originate calls but cannot receive any incoming calls.

- When D-Tel and DDS are connected, the changed data is sent to the system.

- DDS terminal connected to D-Tel should exist in the same slot where D-Tel is equipped.

Data generation

- To create and connect DDS

1	Change 3.1 Extension Information – Terminal Type : DTEL-LGP/DTEL-LKD to DDS-DTEL.

2	Delete the existing telephone numbers.

3	Press the Change button of OMS.

4	Transmit the changed data to the system.

5	If the change is completed, place the mouse on the port and press the right button.

6	Select “D-Tel Feature Inf. View”.

7	Place the mouse on the D-Tel to which DDS is connected and enter the DDS port information (shelf No (1 digit)/slot No (2 digits)/line No (2 digits) on DDS1 Port No ~ DDS4 Port No.

8	Transmit the changed data.

Interactions

- The user can designate the trunk information/extension number/feature button/trunk access code/external subscriber number/pool loop button/digit on the flexible buttons of DDS.

## Direct Dialing In : DDI

Functional description

The function enables an external trunk subscriber to dial extension subscriber's number  without help of the operator.

Operating procedure

Service conditions

- This function may be subject to the numbering plan allocated by the opposite station connected to each route.

- ALTC-LD, ALTC-EM, DNIC-DTC, DNIC-PRI, BRIT, ITGC and QSIG are the local boards available for the DDI trunks.

- Each trunk can be operated for only incoming or for incoming and outgoing.

- Address signals supported for each DDI trunk are shown in the following table.

|  | DP | DTMF | R2-MFC | Others |
| --- | --- | --- | --- | --- |
| ALTC-LD |  |  |  |  |
| ALTC-EM |  |  |  |  |
| DNIC-DTC |  |  |  |  |
| DNIC-PRI |  |  |  |  |
| BRIT |  |  |  |  |
| ITGC |  |  |  |  |
| QSIG |  |  |  |  |

- DDI trunk incoming calls can be restricted for each extension class.

- When the opposite station is different or types of trunks of the same opposite station are different, the services provided by the system can be supplied separately to each route.

- In case abnormal trunk incoming, the route that contains the relevant trunk is designated for incoming calls.

- Inter-trunk DDI can be allowed or denied.

- It is possible to divert all the DDI trunk incoming calls to pre-designated subscribers. (Refer to DDI Divert.)

Data generation

- Designate the DDI trunk type

[4.1 Trunk Information ⇒ Type]

- Separate DDI trunk origination/termination

[4.1 Trunk Information ⇒ Mode]

- Designate DDI trunk address signal type

[4.1 Trunk Information ⇒ Dialing Type]

- Designate day/night class for each extension subscriber

[3.1 Extension Information ⇒ Day Class, Night Class]

- Designate day/night class for each trunk subscriber

[4.1 Trunk Information ⇒ Day Class, Night Class]

- Designate the DDI trunk incoming subscribers allowed

[4.1 Trunk Information ⇒ I/C Allow Class]

- Trunk route separation

[4.1 Trunk Information ⇒ Trunk Route]

[4.7 Trunk Route Assign]

- Service Assign per Day/Night Mode for Abnormal IC trunk Call

[4.10 Trunk Route Base IC Day/Night Option]

- Connection Allow/Deny Option between Routes

[4.12 Inter Route Connection Allow/Deny]

Interactions

## Direct In Line (DIL): Direct In Line of the Trunk Call

Functional description

When an incoming call is generated at the specific trunk, this function enables it to be connected directly to the designated extension subscriber without assistance of the operator.

Operating procedure

Service conditions

- The local board that can be used as the individual incoming trunk access includes APTC, ALTC-RD, ALTC-LD, ALTC-EM, DNIC-DTC, DNIC-PRI, BRIT, and ITGC.

- In case of APTC and ALTC-RD, incoming type for each system incoming mode will be applied if the individual incoming is not designated.

- In case of ALTC-LD, ALTC-EM, DNIC-DTC, DNIC-PRI, BRIT or ITGC, individual incoming service will be provided prior to DDI, if the Direct In Line is assigned to the concerned trunk.

- Assignment of the DIL can be separated and operated for each trunk.

- Assignment of the DIL can be separated and operated by night and daytime.

- Assignment of the DIL can be selectively operated among the specific extension number, paging group or external subscriber number.

- To assign the DIL as the representative number of the paging group, the related paging group should be designated as shown bellow.

- Rep. No: Should be included in the extension numbering plan.

- Service Type: Normal.

- Ring Service Type: Selectable from FIRST, UCD, MULTI or LONG IDLE.

- Rep. Name: The name that will be displayed on the LCD instead of the concerned trunk number, if the incoming subscriber is not the D-Tel or ATD.

- Other items are irrelevant.

- If the DIL assignment is the external subscriber number, connection between incoming route and outgoing route should be allowed.

Data generation

- Trunk type assignment

[4.1 Trunk Information ⇒ Type]

- DIL assignment

[6.1 DIL Incoming Assignment]

- Paging group

[6.4 Calling Group]

- Trunk route separation

[4.1 Trunk Information ⇒ Trunk Route]

[4.7 Trunk Route Assign]

- Service Assign per Day/Night Mode for Abnormal IC trunk Call

[4.10 Trunk Route Base IC Day/Night Option]

- Connection Allow/Deny Option between Routes

[4.12 Inter Route Connection Allow/Deny]

- Assign  incoming type by system incoming mode

[16.3 Night Bar]

Interaction

## Direct Inward System Access (DISA)

Functional description

To compensate for shortcomings of the DIL and DDI, this function enables the external subscriber to make a call by directly dialing the extension or trunk subscriber without assistance of the operator. The following two types of services are available. The external subscribers need additional dialing and have to get through authorization procedure (can be omitted).

- Line DISA (Method that makes up for shortcoming of DIL, which does not support direct call to the subscriber) – Used for APTC, ALTC-RD, and BRIT.

When the concerned trunk is assigned as the DISA and the incoming call is received, DISA service is provided without condition.

- DDI DISA (Method that makes up for shortcoming of DDI, which is limited by numbering plan of the outgoing station) – Used for the remaining trunk type.

DISA service is provided when the system makes a call to the designated DISA representative number through DDI.

Operating procedure

- Line DISA

[When the authorization code is not assigned]

1	The DISA line receives a call from the designated trunk.

2	The external subscriber hears the assigned DIS dial tone (can be replaced with the announcement line). If they do not match, it will be processed by the vacant number calling option of the incoming designation for abnormal trunk incoming.

3	The external subscriber makes a call to the intended extension or trunk subscriber.

4	When the wanted subscriber answers, make conversation.

[When the authorization code is assigned]

1	The DISA line receives a call from the designated trunk.

2	If the DISA authorization code exists, the external caller will hear DISA authorization dial tone (can be replaced with the announcement line).

3	The external subscriber dials the DISA authorization code that was given to him/her.

4	If the DISA authorization code matches with the code assigned to the system, the external subscriber hears the assigned DIS dial tone (can be replaced with the announcement line). If they do not match, it will be processed by the vacant number calling option of the incoming designation for abnormal trunk incoming.

5	The external subscriber makes a call to the intended extension or trunk subscriber.

6	When the wanted subscriber answers, make conversation.

- DDI DISA

[When the authorization code is not assigned]

1	Calls the DISA assigned to the system using DDI.

2	The external subscriber hears the assigned DIS dial tone (can be replaced with the announcement line). If they do not match, it will be processed by the vacant number calling option of the incoming designation for abnormal trunk call incoming.

3	The external subscriber makes a call to the intended extension or trunk subscriber.

4	When the wanted subscriber answers, make conversation.

[When the authorization code is assigned]

1	Calls the DISA assigned to the system using DDI.

2	If the DISA authorization code exists, the external caller will hear DISA authorization dial tone (can be replaced with the announcement line).

3	The external subscriber dials the DISA authorization code that was given to him/her.

4	If the DISA authorization code matches with the code assigned to the system, the external subscriber hears the assigned DIS dial tone (can be replaced with the announcement line). If they do not match, it will be processed by the vacant number calling option of the incoming call designation for abnormal trunk call incoming.

5	The external subscriber makes a call to the intended extension or trunk subscriber.

6	When the wanted subscriber answers, make conversation.

Service conditions

- For the abnormal incoming call, processing like the attendant incoming call can be processed according to the option for each route.

- Direct call service between trunks can be allowed or prohibited.

- All DISA trunk incoming calls can be diverted to the designated subscriber. (See DDI divert function)

- The DDI DISA representative number can be produced by the paging group.

1	Rep. No: Should be included in extension numbering plan.

2	Service Type: DISA

3	The remaining items are irrelevant.

Data generation

- Assign  trunk type

[4.1 Trunk Information ⇒Type item]

- Trunk route separation

[4.1 Trunk Information ⇒ Trunk Route]

[4.7 Trunk Route Assign]

- Connection Allow/Deny Option between Routes

[4.12 Inter Route Connection Allow/Deny]

- Assign  line DISA

[4.1 Trunk Information – Trunk Option ⇒ 2. DISA Line

- Assign  a DISA representative number

[6.4 Calling Group ⇒ Service Type]

- Assign  a DISA authorization code

[4.14 DISA Authorization Code]

- Changing DISA dial tone

[2.4 System tone parameter] – DISA Dial Tone item

- Changing DISA authorization code dial tone

[2.4 System tone parameter] – DISA Auto Code Dial Tone item

- Service Assign per Day/Night Mode for Abnormal IC trunk Call

[4.10 Trunk Route Base IC Day/Night Option]

Interaction

## Direct Outward Dialing (DOD)

Functional description

DOD enables the extension subscriber to make a call directly to the external subscriber without assistance of the operator. Operation methods are as follows.

- Outward dialing by dialing the trunk occupancy code.

- Outward dialing using the common speed dial.

- Outward dialing using the individual speed dial.

- Outward dialing after selecting the trunk that is assigned to the mobile button of the D-Tel.

- Outward dialing using external speed dial that is assigned to the mobile button of the D-Tel.

Operating procedure

- Outward dialing by dialing the trunk occupancy code

1	Off hook.

2	Hear the extension dial tone.

3	Dial the trunk occupancy code.

4	Hear the trunk dial tone. If the virtual dial tone is selected, the subscriber will hear the virtual dial tone instead of the trunk dial tone. (See function of the virtual dial tone.)

5	Dial the external subscriber’s number.

6	Hear the ring back tone.

7	When the external subscriber answers, make conversation.

- Outward dialing using the system speed dial (See system speed dial function)

1	Off hook.

2	Hear the extension dial tone.

3	Dial the system speed code.

4	Hear the ring back tone.

5	When the external subscriber answers, make conversation.

- Outward dialing using the individual speed dial (See individual speed dial function)

1	Off hook.

2	Hear the extension dial tone.

3	Dial the individual speed dialing function code.

4	Dial the individual speed index number.

5	Hear the ring back tone.

6	When the external subscriber answers, make conversation.

- Outward dialing after selecting the designated trunk of D-Tel flexible button. (See D-Tel flexible button function.)

1	Off hook.

2	Hear the extension dial tone.

3	Press the flexible button that is assigned with the intended trunk.

4	Hear the trunk dial tone. If the virtual dial tone is selected, the subscriber will hear the virtual dial tone instead of the trunk dial tone. (See function of the virtual dial tone.)

5	Dial external subscriber’s telephone number.

6	Hear the ring back tone.

7	When the external subscriber answers, make conversation.

- Outward dialing using external speed dial that is assigned to the D-Tel flexible button (See the D-Tel flexible button)

[When only the trunk occupancy code is registered.]

1	Off hook.

2	Hear the extension dial tone.

3	Press the flexible button.

4	Hear the trunk dial tone. If the virtual dial tone is selected, the subscriber will hear the virtual dial tone instead of the trunk dial tone. (See function of the virtual dial tone.)

5	Dial external subscriber’s telephone number.

6	Hear the ring back tone.

7	When the external subscriber answers, make conversation.

[When only trunk occupancy code and a portion of external subscriber number are registered.]

1	Off hook.

2	Hear the extension dial tone.

3	Press the related flexible button.

4	Dial remaining of the external subscriber’s phone numbers.

5	Hear the ring back tone.

6	When the external subscriber answers, make conversation.

[When the trunk occupancy code and all of external subscriber’s phone number are registered.]

1	Off hook.

2	Hear the extension dial tone.

3	Press the concerned flexible button.

4	Hear the ring back tone.

5	When the external subscriber answers, make conversation.

Service conditions

- The trunk occupancy code should be pre-produced.

- Trunks that will be sent to the concerned trunk code should be pre-assigned.

- The grade of the extension subscriber should be E class or above.

- Outgoing digit limit will be applied for trunk incoming. (See class of service)

Data generation

- Assign  outward trunk

[4.1 Trunk Information ⇒ Mode]

- Assign  trunk code numbering plan

[2.1 System Numbering Plan ⇒ Trunk numbering plan]

- Assign  trunk occupancy code

[4.2 Trunk access code]

- Assign Trunk Outgoing Line for each Trunk Access Code

[4.2 Trunk access code – ⓡTrunk OG Group View menu at each Code]

- Change extension line dial tone

[2.4 System tone parameter ⇒ Extension First Dial Tone]

- Assign  a trunk using the virtual dial tone

[4.1 Trunk Information – Trunk Option ⇒ 14. Virtual Dial Tone]

- Common speed dial numbering plan

[2.1 System Numbering Plan – Feature Code Plan

⇒ System Speed Dial Code 1 and System Speed Dial Code 2]

- Assign  a common speed dial

[16.7 System Speed Dial]

- Assign a Individual speed dial registration code

[2.1 System Numbering Plan – Feature Code Plan

⇒ Individual Speed Dial RET]

- Assign a Individual speed dial using code

[2.1 System Numbering Plan – Feature Code Plan

⇒ Individual Speed Dial USE]

Interaction

## Do Not Disturb (DND)

Functional description

When extension subscribers will not answer calls at all, they can reject all incoming calls by registering the DND(Do Not Disturb) function. They can also cancel it.

Operating procedure

- Registering DND using the general telephone

1	The extension subscriber lifts up the handset.

2	Dials the DND registration code (Default:*51) and his/her own telephone number.

3	He/she can hear the Service Set Tone as a sign of registration completion.

4	A call is generated that calls the DND registered subscriber.

5	The called subscriber will hear the closing sound

- Registering DNS using the D-Tel telephone

1	The D-Tel subscriber press the “DND/CFWD” button while in the idle state.

2	The DND state will be displayed on the D-Tel subscriber’s terminal and service will be registered.

3	The “DND/CFWD” LED will be lighted on.

- Registering DND for a single extension subscriber at the attendant.

1	The operator subscriber presses the “DND”(Do Not Disturb) key.

2	Dials the extension number to register as DND.

3	Presses the “ENT”(Registration) key.

- Registering DND for several extension subscriber at the attendant

1	The operator subscriber presses the “DND” key.

2	Dials the starting extension number to register as DND.

3	Dials “*” key.

4	Dials the ending extension number to register as DND.

5	Presses the “ENT”(Registration) key.

- Canceling DND using the general telephone

1	The extension subscriber lifts up the handset.

2	Dials the DND cancel code (Default:*50).

3	Hears the Service Set Tone as a sign of registration completion.

- Canceling DND using the D-Tel telephone

1	Presses the “DND/CFWD” button while DND is registered at the D-Tel telephone.

2	The idle state will be displayed on the D-Tel subscriber terminal and service will be cancelled.

3	The “DND/CFWD” LED will be off.

- Canceling DND for a single extension subscriber at the attendant

1	The operator subscriber presses the “DND” key.

2	Dials the extension number to cancel DND.

3	Presses the “FORC RLS” (Forced Release) key.

- Canceling DND for several extension subscriber at the attendant

1	The operator subscriber presses the “DND” key.

2	Dials the beginning extension number to cancel DND.

3	Dials “*” key.

4	Dials the ending extension number to cancel DND.

5	Presses the “FORC RLS” (Forced Release) key.

Service conditions

- When the subscriber registered in DND lifts up the handset, ACK tone for function registration will be sent for a short time and then a dial tone sent.

- The subscriber registered in DND will not be affected at all when making a call to another subscriber or using other functions.

- DND registration/cancel can be set for each subscriber.

- The operator can cancel the registered DND compulsory at his/her own discretion.

- The attendant can make a call to the DND subscriber.

- The attendant can register or cancel DND on behalf of the subscriber.

- When the subscriber registers DND, he/she will be excluded from the hunting group.

- If call forwarding service and DND service are registered at the same time, call forwarding service will be prior to DND. However, if the no-answer call forwarding service and DND service are registered at the same time, DND service will take precedence.

- Register DND in the terminal with call forwarding function can be performed using the code.

Data generation

- DND function permission status

[3.1 Extension Information – Feature Restriction ⇒ DND]

Interaction

- When DND is cancelled wile it is not registered, the subscriber will hear the lock out tone.

- Individual call park/system call park function cannot be employed using the general telephone. Instead, exclusive hold/system hold can be used.

- Once this function is registered, call forwarding function cannot be registered.

## D-Tel Ring Selection

Functional description

The function enables the D-Tel subscriber to select any ring types he wants.

Operating procedure

- To select D-Tel (LGP) ring types

1	Press the “Program” button.

2	When the subscriber dials the LGP ring selection code (Default:03), the following message is displayed on the LCD.

“Ring Type : X”    X : The ring type currently set up

3	Select one of the ring types (1~4).

4	Press “HOLD/ENT” button.

5	Hang up the handset.

- To select D-Tel (LKD) ring types

1	The extension subscriber picks up the handset.

2	When he dials the GDK ring selection code (Default:#1), the following message is displayed on the LCD.

“RING TYPE (0-2) : [X]”   X : The ring type currently set up.

3	Dial one of the ring types (0~2).

4	The relative ring of the selected ring type bells with the following message displayed on the LCD.

[RING1] Ringing Test

2000-01-01 09:00 am

5	The subscriber hangs up the handset.

Service conditions

- Four types of rings are currently provided in LGP.

- Three types of rings are currently provided in LKD.

## Emergency Call

Functional description

Emergency call enables the user to make a call regardless of service grade of the extension line even when all station lines are busy by pressing the emergency telephone number only, which is produced in advance, in case of an emergency.

Operating procedure

- emergency call.

1	Lifts up the handset.

2	Dials the emergency telephone number.

3	Dialing tone will be sent.

4	If the external subscriber answers, a call is completed.

Service conditions

- The emergency telephone number should be produced in advance.

- If all office lines are busy, station line will be released compulsory and occupied.

- The station code used by the emergency telephone number will be the access code that belongs to the same tenant of the subscriber making an emergency call.

- The emergency telephone number will be used regardless of toll check.

- If the emergency telephone number uses the same number with the extension number, it can be used only when the number of digit is less than or same as that of the extension number.

Data generation

- Emergency telephone number

[16.2 Emergency]

## Exclusive Hold

Functional description

This function holds extension/office line subscriber when the line is busy. The counterpart subscriber will wait while hearing a hold sound and only the held subscriber can make a call with the suspended subscriber.

Operating procedure

- Exclusive hold at the D-Tel telephone

1	The subscriber is on the line with the extension subscriber of extension subscriber.

2	If the [2.6 System Feature Option-Hold Preference] item is set to “EXC-HOLD”, press the “HOLD BTN”. Or, if it is set to “SYS-HOLD”, press the “HOLD BTN” twice.

3	If the exclusive hold is completed normally, one of the held flexible button on the D-Tel telephone will blink.

4	If the held D-Tel subscriber requires to resume a call with the suspended subscriber, press the flexible button.

Service conditions

- This function is not available at the general telephone.

- The hot line subscriber cannot use this function.

- The extension or office subscriber one the line can be put on the exclusive hold.

- The counterpart subscriber can be put on the exclusive hold only when he/she is already registered on the flexible button on the caller’s telephone or connected DSS, or the caller can assign the intended subscriber to hold on the Pool/Loop button if the subscriber to hold is not registered on the caller’s telephone or connected DSS.

- The counterpart subscriber can be put on the exclusive hold only when the counterpart subscriber is registered on the flexible button of the caller’s telephone, or he/she can be registered on the Pool/Loop button.

- If the "EXC-HOLD/SYS-HOLD” item is set to “EXC-HOLD” in the [2.6 System Feature Option-Hold Preference], the counterpart subscriber can be put on the exclusive hold using the “HOLD BTN”.

- If the "EXC-HOLD/SYS-HOLD” item is set toe “SYS-HOLD” in the [2.6 System Feature Option-Hold Preference], the counterpart subscriber can be put on the exclusive hold by pressing the “HOLD BTN” twice.

Data generation

- Selecting hold type

[2.6 System Feature Option ⇒ Hold Preference]

Interaction

- General telephone cannot use exclusive hold/system hold, but use exclusive call park /system call park.

- If the certain subscriber is put on the hold and released without an answer, the call will be made from the suspended subscriber.

## External Paging

Functional description

This function supports connection of multiple external paging equipment to the system and enables the user to use the paging function by selecting intended paging equipment through the telephone. When the user simply dials the access code of the assigned external paging trunk, the connected trunk line for paging will be occupied and paging will be available.

Operating procedure

Service conditions

- External equipment can be connected only to the APTC so far.

- Trunk occupancy codes for external paging trunk line will be allowed as many as occupancy codes allowed by the system.

- One occupancy code for external paging trunk line can contain several trunk lines for paging.

- External paging will be allowed to all subscribers.

- This function can be used when registered the flexible button of the D-Tel.

Data generation

- Assign the occupancy code for the external paging trunk line

[4.2 Trunk line access code ⇒ Info 1]

- Assign Paging Trunk Line for the Paging Access Code

[4.2 Trunk access code – ⓡTrunk OG Group View menu at each Code]

Interaction

## Flexible Numbering Plan

Functional description

Numbering plan (extension number, trunk line occupancy code and function code) of the system can be modified flexibility to satisfy the customer’s needs.

Operating procedure

Special Condition

- Up to 6 digits can be used for the extension number.

- Up to 6 digits can be used for the trunk occupancy code.

- Up to 4 digits can be used for the function code.

- The first digit of the function code cannot be used identically with that of the trunk line numbering plan.

Data generation

- Assign extension numbering plan

[2.1 System Numbering Plan] – Extension numbering plan item

- Assign trunk line occupancy code plan

[2.1 System Numbering Plan] – Trunk line number plan item

- Assign function code plan

[2.1 System Numbering Plan] – Function code plan item

Interaction

## Forced Release

Functional description

This function enable to select a specific extension/trunk line and to perform forced release for it at the attendant.

Operating procedure

- Forced release of the extension line at the attendant

1	Dials the extension number at the attendant.

2	The status of the selected extension line will be displayed.

3	If the selected extension line is not idle, press the “FORC RLS” (Forced Release) key.

4	The extension line and the attendant will be released.

- Forced release of the trunk line at the attendant

1	Press the “TRK SEL” (Trunk Selection) key at the attendant.

2	Dials the serial number of the attendant (4 digits).

3	The status of the selected trunk line will be displayed.

4	If the selected trunk line is not idle, press the “FORC RLS” (Forced Release) key.

5	The trunk line and the attendant will be released.

Service conditions

- If the selected trunk line is in the idle state, the attendant will occupy the trunk line.

Data generation

- Selecting forced release function of the attendant

[5.6 ATD Service ⇒ Use ATD Force Release]

## Group Hunting

Functional description

Group hunting is used to transfer a call to another subscriber belonging to the same hunting group when a subscriber is busy who will receive the transferred call from the extension or the operation. Or, when the operator tries to make a extension call but the specific extension subscriber is busy, the group hunting function can be used to call another subscriber belonging to the same group with the concerned subscriber.

Operating procedure

- Group hunting using the general telephone/D-Tel telephone

1	Makes a call to the extension subscriber.

2	The caller will hear busy tone since the receiving subscriber is busy.

3	Press the group hunting code (Default:2).

4	Sends a ring to the idle subscriber after finding a subscriber belonging to the same hunting group with the busy counterpart subscriber.

5	When the intended subscriber answers, a telephone call is completed.

- Group hunting at an attendant

1	The operator dials the phone number of the extension subscriber to whom he/she tries to transfer a call when the intended subscriber is busy.

2	The status of the extension subscriber will be displayed on the LCD screen of the attendant.

3	If the intended extension subscriber is busy, the operator presses the “GRP CALL” (Group Call) key.

4	A ring will be sent to the available extension subscriber among subscriber belonging to the same hunting group with the transferring subscriber. At this time, the original subscriber will hear a dialing tone. The LCD screen of the attendant will be cleared and the attendant will be released.

5	The operator dials a telephone number of the extension subscriber who wants to make a call in an idle state.

6	If the target extension subscriber is busy, the operator will hear busy tone.

7	While hearing the busy tone, the operator presses the “GRP CALL” (Group Call) key.

8	As the dialing signal is sent to the available extension subscriber among subscriber belonging to the same hunting group with the intended subscriber, the operator will hear dialing sound.

Service conditions

- When the step call function is used, this function cannot be used.

- There must be an idle subscriber within the same hunting group.

- If all extension subscribers of the hunting group to which the busy extension subscriber belongs or the hunting group of the extension subscriber is not assigned when the operator presses the “GRP CALL” (Group Call) key, hunting call function will not be performed but the call between the operator and the caller will be continued.

- If the operator presses the “GRP CALL” (Group Call) key when the call of the busy extension subscriber is terminated, the hunting call function will not be carried out but the call will be directly transferred.

Data generation

- Define Hunting Group

[6.7 Hunting Group]

- whether the Step call function is allowed, or not.

[2.6 System Feature Option ⇒ Step Call Use]

Interaction

- If the intended subscriber is busy, the subscriber can use the specific function by pressing the certain digit while hearing the busy tone. Functions for each digit(Default Value) are as follows.

1	“2“: Group Hunting function

2	“3”: Busy Override function

3	“4”: Call Back function

4	“5”: Call Waiting function

5	“6”: Camp On function

- When the target subscriber is busy, service will be carried out by the order of Auto Group Hunting, Auto Hold, Auto Call Waiting and Auto Camp On.

## Hot Line

Functional description

Using this function, the specific extension subscriber can make a call to another extension subscriber or registered external subscriber by lifting up a handset without dialing the phone number.

Operating procedure

- Direct call between extension subscribers.

1	The subscriber (A) designated as a hot line lifts up the handset.

2	The subscriber (A) hears the dialing tone, whereas the (B) designated as the receiver hears the calling sound.

3	The subscriber (B) answers and makes a call.

- Direct call from the extension subscriber to the external subscriber.

1	The subscriber (A) designated as an external hot line lifts up the handset.

2	The subscriber (A) dials the registered trunk line calling code and external subscriber phone number.

3	The external subscriber answers and makes a call.

Service conditions

- If all subscribers connected through the hot line are extension subscribers, calling can be made only between subscribers designated as the hot line.

- If the extension subscriber and the external subscriber are connected through the hot line, the extension subscriber can make a call to the external subscriber, whereas the external subscriber cannot it.

- The hot line subscriber cannot use a hook flash function during conversation on the phone since no dialing function is available.

- The subscriber assigned by the hot line cannot make a call to another extension, operator or trunk line.

- The hot line subscriber can be assigned only one to one.

Data generation

- Assign a hot line between extensions

[16.1 Special Phone – Hot Line]

- External hot line assignment from the extension to the external subscriber

[16.1 Special Phone – External Hot Line]

- Checks to see if the hot line is assigned

[3.1 Extension Information – Extension Characteristic ⇒ 5 Hot Line]

Interaction

One subscriber can be assigned as only one of the Hot Line/House Phone/Warm Line.

## House Phone

Functional description

This function enables the specific extension subscriber to make a call to the designated subscriber when he/she lifts up the handset without dialing.

Operating procedure

- Direct calling between extension subscribers.

1	The designated subscriber (A) lifts up the handset using the house phone.

2	The subscriber (A) hears the dialing tone, whereas the (B) designated as the receiver hears the calling sound.

3	The subscriber (B) answers and makes a call.

Service conditions

- Server house phone subscribers can select the same general subscriber as the counterpart subscriber.

- The house phone subscriber cannot use a hook flash during conversation on the phone since no dialing function is available. The general subscriber designated as the counterpart subscriber cannot transfer the house phone subscriber to other extension number.

- Other general subscriber is able to make a call to the house phone subscriber.

Data generation

- Assign a house phone between extensions

[16.1 Special Phone – House Phone]

- Checks to see if the house phone is assigned

[3.1 Extension Information – Extension Characteristic ⇒ 3 House phone]

Interaction

- One subscriber can be assigned as only one of the Hot Line/House Phone/Warm Line.

## Incoming Mode Change

Functional description

This function changes the incoming call type of the station that belongs to its own tenant group into that of the day/night. Only subscribers with pre-designated day/night office call change can apply it.

Operating procedure

- Changing to day incoming mode using the general telephone/D-Tel telephone.

1	The extension subscriber lifts up the handset.

2	Dials the change code for the day/night incoming mode (Default:*62).

3	The subscriber can hear the Service Set Tone as a sign of registration completion.

- Changing to night incoming mode using the general telephone/D-Tel telephone.

1	The extension subscriber lifts up the handset.

2	Dials the change code for the day/night incoming mode (Default:*62).

3	The subscriber can hear the Service Set Tone as a sign of registration completion.

Service conditions

- The option of incoming mode change should be allowed in the concerned terminal that the subscriber wants to change the incoming mode.

Data generation

- Permission state of the I/C Mode

[3.1 Extension Information – Extension Characteristic ⇒ 12 I/C Mode Change]

Interaction

- If the value of the [2.6 System Feature Option-TNT Grp Mode Change] item is set to “YES”, change the incoming mode of all tenants into day mode or night mode.

- If the value of the [2.6 System Feature Option-TNT Grp Mode Change] item is set to “NO”, change the incoming mode of the tenant to which the concerned terminal belongs into day mode or night mode.

- If the [2.6 System Feature Option-Auto System Class Change] item is set to “YES”, change the system class into the day class or night class.

## Individual Call Park

Functional description

This function enables the subscriber in busy to be parked individually and to resume calling with the parked subscriber with the caller wants to make a call again.

Operating procedure

- Registering an individual call park using the general telephone

1	Press the “Hook-Flash” in the conversation state.

2	Press the registration code for an individual call park (Default:#8).

3	The subscriber registering the individual call park can hear confirmation tone when if it is successful.

4	The subscriber registering the individual call park will hear suspension sound. If the suspended subscriber is using the D-Tel telephone, the following message will be displayed on the LCD screen.

HELD BY XXXX

2000 – 01 - 01 09:00 am

- Registering an individual call part using the D-Tel telephone

1	Press the “TRANS”(Transfer) Button in the busy state.

2	Press the registration code for an individual call park (Default:#8).

3	The subscriber registering the individual call park can hear confirmation sound when if it is successful, and the following message will be displayed on the LCD screen.

LINE HOLDING

LINE 10101 09:00 am

4	When function is successfully registered, the subscriber being in an individual call park will hear suspension sound, and the following message will be displayed on the LCD screen.

HELD BY XXXX

2000 – 01 - 01 09:00 am

- Responding to the individual call part using the general telephone

1	Off-hook first.

2	Press the registration code for an individual call park (Default:#8).

3	A call is connected to the subscriber being in an individual call park.

- Responding to the individual call part using the D-Tel telephone.

1	Press the registration code for an individual call park (Default:#8).

2	A call is connected to the subscriber being in an individual call park.

Service conditions

- Only one individual call park is allowed for one subscriber.

- An attendant cannot be put on the individual call park.

- The hot line subscriber cannot use this function.

- When the subscriber being in an individual call park recovers from call parking state, the individual call park will be cancelled.

- If the subscriber who requested an individual call park does not call again the subscriber being in an individual call park, the call will be directed to the subscriber who requested an individual call park. In this case, the subscriber who requested an individual call park should be in the idle state.

Data generation

- Confirmation Tone

[2.4 System Tone Parameter – EXT Camp On Holding Tone]

## Individual Speed Dial

Functional description

The function allows extension subscribers to call the frequently called external subscribers by registering individual speed dials and using them.

Operating procedure

- Registration of individual speed dial in general phone/D-Tel

1	A extension subscriber hooks off the handset and dials the individual speed dial registration code (Default:*#).

2	Dial a shortcut code between 0 and 9.

3	Dial the trunk seizure code and the external subscriber’s number.

4	Hook on the handset.

- Usage of individual speed dial in general phone/D-Tel

1	A extension subscriber hooks off the handset and dials the individual speed dial usage code (Default:#6).

2	Dial a shortcut code between 0 and 9.

3	The external subscriber is called and answers the call, and the call between the two is established.

- Registration of individual speed dial in D-Tel

1	Press the “PROGRAM” button.

2	Press the “MOMORY” button.

3	Dial a shortcut code between 0 and 9. At this time, if the information already registered exists, display it on the LCD screen.

XXXXXXXXXX

MEMORY REG. 00

XXXXXXXXXX  External subscriber’s number registered

00              Abbreviated code

4	Dial the trunk seizure code and the external subscriber’s number.

5	Press the “HOLD/SAVE” button.

6	Hear the service set tone as the signal indicating that the registration is completed. At this time, display that the registration is completed as shown below.

** COMPLETED **

MEMORY REG. XXXX

0000  Abbreviated code

- Verification/usage of individual speed dial in D-Tel

1	Press the “PROGRAM” button.

2	Press the “MEMORY” button.

3	Dial a shortcut code between 00 and 09. At this time, if the information already registered exists, display it on the LCD screen.

4	Press the “Volume Up/Down button” to search the desired information.

5	If the subscriber number desired to be called is found, press the “Hold/Input button”.

6	The external subscriber is called and answers the call, and the call between the two is established.

Service conditions

- The maximum 128 subscribers can the individual speed dial in the system.

- Each subscriber can use up to 10 individual speed dials.

- Only the subscriber to whom the individual speed dial function is allowed can use the function.

- The station number cannot be registered to the speed dial. Only the external subscriber number can be registered.

- Even though the data is already stored, the existing information is deleted when the shortcut code (0~9 or 00~09) is entered.

- The function is available only to those subscribers who can call a trunk.

- Numbers to be registered as the individual speed dial should not exceed 22 digits and cannot be the same as trunk calling numbers.

- The functions such as the long-distance call restriction and the call duration restriction also apply to calls requested by speed dial codes which correspond to numbers whose trunk number including the DDD code is registered.

Data generation

- Designate whether the individual speed dial is allowed, or not

[3.1 Extension Information – Feature Restriction ⇒ IND Speed Dial]

## Individual Trunk Access:

Functional description

The function constrains the usage of trunks produced in the system by designating them for each extension subscriber and trunk seizure code. It enables only the designated subscribers (individual trunk usage group) to whom the usage authority of designated trunk group (individual trunk group) is qualified to use the trunk. Through this function, it is possible to operate the system so that each subscriber can use other trunk group by trunk seizure code that the subscriber dials.

Operating procedure

Service conditions

- A extension subscriber can belong to only one individual trunk group.

- A trunk line can be designated to the individual trunk group by overlapping.

- For the trunk seizure code that can use the individual trunk group, there is only IND or C&I type of trunk seizure code.

- The outgoing trunk group cannot be connected directly to the IND type of trunk seizure code.

- The outgoing trunk group can be connected directly to the C&I type of trunk seizure code. If all trunks that belong to the individual trunk group connected with the related code are busy, the origination is performed through the idle line out of lines that belong to the outgoing trunk group directly connected.

- The trunk that belongs to the individual trunk group cannot be registered simultaneously to the outgoing trunk group that is directly connected to the trunk seizure code.

Data generation

- Outgoing trunk designation

[4.1 Trunk Information ⇒ Mode]

- Assign of trunk seizure code numbering plan

[2.1 System Numbering Plan ⇒ Trunk numbering plan]

- Assign of IND , C&I trunk seizure code

[4.2 Trunk Access Code ⇒ Info 1]

- Assign Trunk Outgoing Line for each Trunk Access Code

[4.2 Trunk access code – ⓡTrunk OG Group View menu at each Code]

- Assign of individual outgoing user group

[6.3 Individual outgoing group – Individual trunk outgoing user group]

- Assign of individual outgoing trunk group by trunk seizure code

[6.3 Individual outgoing group – Trunk outgoing group number by access code]

- Assign of individual outgoing trunk group

[6.3 Individual outgoing group – Trunk outgoing group]

Interactions

## Inter Position Transfer

Functional description

The function allows an operator to transfer the subscriber on the other end of the line to another operator during a call.

Operating procedure

- Transfer from attendant to attendant

1	If an operator tries to transfer the extension subscriber on the other end to another subscriber during a call, the operator presses the “Inter Position Transfer” key and enters the serial number of the attendant to transfer the call to.

2	The operator is released, the extension subscriber hears the hold tone, and the “Inter Position Transfer” LED of the called attendant flashes.

3	The called operator answers the call.

4	Then, a call between the held extension subscriber and the called operator is established.

Service conditions

- Attendant serial numbers should be defined when the system is produced.

- If the called attendant is turned off, the subscriber who performed a transfer hears the hang-up tone.

- Although the called subscriber is in the night service mode, the transfer between attendants is possible.

- The individual attendant signal may be dialed instead of the “Inter Position Transfer” key and entering the attendant serial number.

Data generation

- The ATD serial number is automatically assigned in order when the attendant is produced.

- Verify the attendant serial number

[5.7 ATD Night Subscriber]

- Verify the attendant serial number

[5.8 ATD Overflow Subscriber]

## Internal Zone Paging

Functional description

The function allows D-Tel subscribers to page through the speakers of all D-Tels in the internal zone of a certain zone-paging group.

Operating procedure

- Internal zone paging in D-Tel/SLT

1	Press the “CONF”(Conference) button or dial the paging code (Default:*90).

- Dial the zone number (00~19).

INTERNAL PAGE

ZONE ## 09:00 am

## : Zone number

3	Provide the subscriber, who is supposed to make an announcement, with the ring that informs the paging. (LGP terminal)

Blink the Ring LED in order to inform the subscriber of the paging. (LKD terminal)

4	The ring is stopped and the speech path is established.

- The subscriber who requests the paging makes an announcement.

ZONE PAGING START!!!

ZONE ## 09:00 am

6	The announcement is heard through the speakers of all D-Tel.

7	Wait the answer after the announcement is finished. If the answer is not desired, hook on the handset.

8	The subscriber who wants to answer presses the zone paging answer code (Default:*49) in D- Tel.

9	A call between the paging subscriber and the paged subscriber is established and the remained D-Tel subscribers are all released.

Service conditions

- The internal zone paging and the answer for zone paging are available only in D-Tel.

- The permission of internal zone paging can be designated.

- The answer for entire zone paging is available in all D-Tels.

- The answer for specific zone paging is available only for the designated subscriber out of related internal zone paging subscribers.

- If the answer by answer code is desired, wait it without the release of D-Tel after announcement.

Data generation

- Internal zone paging permission status

[3.1 Extension Information – Extension Characteristic ⇒ Z-Paging Use]

- Answer availability status for specific zone paging

[6.6 Zone Paging Group Using – Zone Paging Group ⇒ Answer Option]

## Last Number Redial

Functional description

The function automatically stores the last number dialed by a extension subscriber/operator subscriber and redials the number when the subscriber presses the redial code. The previously stored number is updated by the new one whenever the subscriber hooks off the handset and dials a new number. Note that special service codes are not redialed.

Operating procedure

- Last number redial in general phone/D-Tel

1	A extension subscriber hooks off the handset.

2	Presses the redial code (Default: ##).

3	The number of the last station/trunk dialed by the subscriber is redialed.

4	The called subscriber answers.

5	A call is established.

- Last number redial in D-Tel

1	Press the “MEMORY” button.

STATION XXXX

MEMORY 09:00 am

2	Dial “*”.

3	The number of the last station/trunk dialed by the subscriber is re-originated.

4	The called subscriber answers.

5	A call is established.

- Last number redial of the operator subscriber

1	An operator presses the “Function (FUNC)” key followed by “#”.

2	The last number dialed by the operator is displayed on the LCD of the attendant.

3	The operator presses the “CONNECT”(Connection) key.

4	The operator and the called subscriber receive the calling tone and the calling signal, respectively.

Service conditions

- Maximum 12 digits except for the access code can be stored

- If the digit last dialed is not an internal line/station/dedicated line calling, process it as errors.

- The new one updates the last number whose redial was registered whenever the subscriber hooks off the handset and dials a new number.

- For the LGP30D and LGP44D terminals, users can serve the function simply by pressing the Redial button.

## Loud Bell Service

Functional description

The function allows all extension subscribers of related tenant to commonly answer all trunks terminated for each tenant by using the external buzzer.

Operating procedure

1	The trunk call is automatically waited in Load Bell Queue after termination. At this time, the external subscriber hears the RBT.

2	If the designated buzzer is deactivated, turn on the buzzer and if the buzzer is activated, keep it.

3	If the extension subscriber dials the designated answer code after hooking off the handset in the active buzzer status, the incoming trunk call first waited in Load Bell Queue is connected and the call is established.

4	If there is no more waiting call in Load Bell Queue after repeating the step 3 continuously, the buzzer is automatically turned off. That is, if there is one waiting call in Load Bell Queue that is not connected to the extension subscriber, the buzzer is not turned off.

Service conditions

- The separate operation by tenant is available.

- The maximum number of incoming call simultaneously waited for each tenant is 16.

- This function is constrained to the incoming trunk call.

Data generation

- Station tenant designation

[3.2 station tenant assignment] – The incoming and outgoing tenants can be simultaneously designated.

- Incoming trunk tenant designation

[4.5 incoming trunk tenant assignment]

- Outgoing trunk tenant designation

[4.5 outgoing trunk tenant assignment]

- Loud bell designation by tenant

[16.6 loud bell]

Interactions

## LCR (Least Cost Routing)

Functional description

If there are several special category telecommunications operators who have different cost by day/time zone/terminating area, the function allows the subscriber to connect the call through the trunk paying the least cost by automatically considering the current day/time/originating digit after dialing the assigned trunk seizure code and incoming subscriber number without additional consideration.

The following three Tables are used for LCR service.

- LCR Service Information

Include the basic information required for LCR service as follows.

- Currently, is the LCR service valid?

- Will the Code Repeat be used?

- Which digit is used for Digit Sending Pause Digit?

- Which digit is used for the substitution of Authorization Code?

- Which digit is used for the substitution of station number?

- How will the Day Time Zone be divided?

- Which time zone is used for each day?

- LDT (Leading Digit Table)

This Table contains the DMT Table information to be used for each day/time zone and for each leading digit (maximum 6 digits from the first digit that the subscriber dials) that the subscriber dials. The leading digit can be designated up to 6 digits. In addition, it can be designated if the leading digit is identical but the tenant is different.

- DMT (Digit Modification Table)

This Table contains the information about how to change the digit that the subscriber dials, the outgoing trunk seizure code, and the alternative DMT Table information to be used if all designated trunks are busy.

[Cases that the LCR Service is applied]

1	Direct origination

2	Trunk transfer origination

3	Origination by the system speed dial

4	Origination by the individual speed dial

5	Origination by the flexible button speed dial

6	Origination by the external call forwarding

7	Calling between trunks

[Cases that the LCR Service is excluded]

1	Origination by selecting the extension line that is designated in the flexible button of D-Tel.

2	Origination by designating the specific line from the attendant.

Operating procedure

1	A subscriber dials the LCR service trunk seizure code designated.

2	The calling subscriber hears the LCR service dial tone.

3	The calling subscriber dials the external number.

4	After selecting the least cost trunk by analyzing the outgoing digit, the system changes and sends the outgoing digit so that the call can be established through the related special category telecommunications operator.

5	The called subscriber answers the call.

Service conditions

- The trunk seizure code for LCR service should be produced.

- The usage status of LCR service can be selected.

- In the LCR service, the ARS and AND services directly designated to the related trunk seizure code are disregarded.

- If the LCR service is not available currently, the general trunk origination service including the ARS and AND services is performed by using the trunk origination group that is directly connected to the trunk seizure code for LCR service.

- For the LCR service, the repeat status of trunk seizure code that the subscriber dials can be selected. That is, if the external number is 02-818-4114, produce the trunk seizure code for LCR service as “0” and designate the repeat function of trunk seizure code to be used. Then, the outgoing subscriber can originate the call only by dialing the “02-818-4114”.

- The charging information on the outgoing call generated from LCR service displays both the digit that the subscriber directly dials and the digit that is actually sent after changing.

- For the LCR service, the outgoing digit restriction function by station class is performed for the digit that the subscriber directly dials.

Data generation

- Assign of trunk seizure code for LCR Service

[4.2 Trunk Access Code ⇒ Info 2]

- Assign Trunk Outgoing Line for each Trunk Access Code

[4.2 Trunk access code – ⓡTrunk OG Group View menu at each Code]

- LCR Information Table designation

[12.1 LCR Information]

- LCR Leading Digit Table designation

[12.2 LCR LDT]

- LCR Digit Modification Table designation

[12.3 LCR DMT]

Interactions

- ARS (Alternative Route Selection)

If the related trunks are all busy when the subscriber attempts the trunk origination, the function allows the related outgoing call to be normally established by one-by-one auto-checking the trunks of alternative route designated and selecting the route that contains the available trunk. At this time, the repeat digit is designated to the outgoing digit and the designated digits are additionally auto-dialed when originating the call through the alternative route selected.

- AND (Automatic Network Dialing)

If dialing the trunk seizure code for trunk origination, the function allows the designated digits to be additionally auto-dialed.

## Line Lock-Out

Functional description

If a extension subscriber holds the handset without dialing for a long time, the function regards the subscriber as having no intention to dial, automatically locks out the call path, and play the lockout tone.

## Linked Station Pair

Functional description

The function allows the actual two terminals to be used as if they are one terminal by connecting two different terminals.

Operating procedure

- If ☎/☏ are designated as the linked station pair and ☎ is called, the processing is as follows.

1	☎:IDLE / ☏:IDLE  ☎, ☏ ring simultaneously. A call is connected with the first answering subscriber and the other subscriber is automatically cancelled.

2	☎:IDLE / ☏:BUSY  Processed as BUSY.

3	☎:BUSY / ☏:IDLE  Processed as BUSY.

4	☎:BUSY / ☏:BUSY  Processed as BUSY.

5	☎:IDLE / ☏:DND/ABSENCE registration  ☎ rings.

6	☎:IDLE / ☏:CFWD registration  The call forwarding port registered in ☏ rings.

7	☎:DND/ABSENCE registration / ☏:IDLE  ☏ rings.

8	☎:CFWD registration / ☏:IDLE  The call forwarding port registered in ☎ rings.

9	☎:BUSY / ☏:DND/ABSENCE registration  Processed as BUSY.

10	☎:BUSY / ☏:CFWD registration  Processed as BUSY.

11	☎:DND/ABSENCE registration / ☏:BUSY  Processed as BUSY.

12	☎:CFWD registration / ☏:BUSY  The subscriber to whom the call is forwarded rings.

Service conditions

- The attendant cannot be designated as the linked station pair.

- The DSS terminal cannot be designated as the linked station pair.

- The linked station pair designation is required.

Data generation

- Linked Station Pair designation

[16.10 Linked Station Pair]

## Manager/Secretary Calling

Functional description

In general, a general subscriber cannot make a direct call to the executive subscriber, but should go through the secretary subscriber because manager and secretary subscribers can call each other directly. Here, they can call each other by dialing the extension subscriber’s number personally or the manager/secretary calling code.

Operating procedure

- Calling from the secretary to one’s own manager

1	A secretary hooks off the handset.

2	Dial the manager’s number.

3	If the manager is in idle state, the secretary and manager hear the calling tone and calling signal, respectively. If the manager answers the calling signal, a call is established.

4	If the manager is not in idle state, the secretary subscriber hears the busy tone.

- Calling from one manager to other manager

1	The manager subscriber hooks off the handset.

2	If the option("Use class in Man to Man Call) of destination manager is "NO", the calling is performed. If the option of destination manager is "YES",  the calling is performed only in case that the calling manager class is higher than or equal to the destination manger class, and otherwise, the calling manager hears busy tone.

- Calling from the extension subscriber to the manager

1	A extension subscriber hooks off the handset.

2	Call the manager.

3	If all calls to the manager are designated so that they are forwarded to the secretary, call the secretary. Otherwise, call the manager.

4	The manager or secretary answers the calling.

5	A call is established.

Service conditions

- The manager and secretary can be assigned up to 50, respectively in the system.

- A manager can assign up to 10 secretary subscribers for use.

- A secretary can assign up to 10 manager subscribers for use.

- The assignment of manager/secretary and the option designation are required.

- If both manager and secretary subscribers are unavailable, it is not possible to originate a call, using the function, and the calling subscriber hears the busy tone.

- If a extension subscriber calls the manager, it can be designated whether forwarding it to the secretary or directly calling the manager.

- If an extension subscriber calls the manager, it can be designated whether forwarding it to the secretary or directly calling the manager.

- The cases that the secretary calls another manager/a subscriber who is not a secretary calls the manager/a subscriber who is not a manager calls the manager are processed equally with the case that a extension subscriber calls the manager.

- If all secretaries designate the DND function when a subscriber calls the secretary, it can be designated whether sending the CGT or calling the manager.

- If a subscriber calls the secretary who registers the call forwarding function, it can be designated whether forwarding the call or calling the secretary.

- The calling status can be restricted according to the class for inter-manager calling.

- If the data is produced so that a manager calls other manager according to class, the low class manager cannot call the high class manager but can call the lower class manager than himself/herself.

Data generation

- Manager group and options assignment

[6.1 Manager Group]

- Secretary assignment

[6.1 Manager Group – ⓡSecretary view]

- Processing option designation if a extension subscriber calls the manager

[6.1 Manager Group – Group Option ⇒ Manager Call From Extension]

- Processing option designation if an extension subscriber calls the manager

[6.1 Manager Group – Group Option ⇒ Manager Call From Trunk]

- Processing option designation if the secretary registers the DND (Do Not Disturb) function

[6.1 Manager Group – Group Option ⇒ All Secretary are DND]

- Processing option designation if the secretary assigns the call forwarding

[6.1 Manager Group – Group Option ⇒ Secretarys Call Forward]

- Processing option designation if a manager calls other manager

[6.1 Manager Group – Group Option ⇒ Manager to Manager Call]

- Class of manager group

[6.1 Manager Group – Group Option ⇒ Manager Class]

Interactions

- If a extension subscriber calls the manager, the call is terminated to manager/secretary according to the option.

- If an extension subscriber calls the manager, the call is terminated to manager/secretary according to the option.

## Manager/Secretary Calling

Functional description

“Same as COMMON”

Operating procedure

- Calling from the secretary of the manager to the manager

1	A secretary hooks off the handset.

2	Dial the manager’s number.

3	If the manager is in idle state, the secretary and manager hear the calling tone and calling signal, respectively. If the manager answers the calling signal, a call is established.

4	If the manager is not in idle state, the secretary subscriber hears the busy tone.

- Calling from one manager to other manager

- The manager subscriber hooks off the handset.

2	If the option("Use class in Man to Man Call) of destination manager is "NO", the calling is performed. If the option of destination manager is "YES",  the calling is performed only in case that the calling manager class is higher than or equal to the destination manger class, and otherwise, the calling manager hears busy tone.

- Calling from the extension subscriber to the manager

1	A extension subscriber hooks off the handset.

2	Call the manager.

3	If all calls to the manager are designated so that they are forwarded to the secretary, call the secretary. Otherwise, call the manager.

4	The manager or secretary answers the calling.

5	A call is established.

Service conditions

- The manager and secretary can be assigned up to 50, respectively in the system.

- A manager can assign up to 10 secretary subscribers for use.

- A secretary can assign up to 10 manager subscribers for use.

- The assignment of manager/secretary and the option designation are required.

- If both manager and secretary subscribers are unavailable, it is not possible to originate a call, using the function, and the calling subscriber hears the busy tone.

- If a extension subscriber calls the manager, it can be designated whether forwarding it to the secretary or directly calling the manager.

- If an extension subscriber calls the manager, it can be designated whether forwarding it to the secretary or directly calling the manager.

- The cases that the secretary calls another manager/a subscriber who is not a secretary calls the manager/a subscriber who is not a manager calls the manager are processed equally with the case that a extension subscriber calls the manager.

- If all secretaries designate the DND function when a subscriber calls the secretary, it can be designated whether sending the CGT or calling the manager.

- If a subscriber calls the secretary who registers the call forwarding function, it can be designated whether forwarding the call or calling the secretary.

- The calling status can be restricted according to the class for inter-manager calling.

- If the data is produced so that a manager calls other manager according to class, the low class manager cannot call the high class manager but can call the lower class manager than himself/herself.

Data generation

- Manager group and options assignment

[6.1 Manager Group]

- Secretary assignment

[6.1 Manager Group – ⓡSecretary view]

- Processing option designation if a extension subscriber calls the manager

[6.1 Manager Group – Group Option ⇒ Manager Call From Extension]

- Processing option designation if an extension subscriber calls the manager

[6.1 Manager Group – Group Option ⇒ Manager Call From Trunk]

- Processing option designation if the secretary registers the DND (Do Not Disturb) function

[6.1 Manager Group – Group Option ⇒ All Secretary are DND]

- Processing option designation if the secretary assigns the call forwarding

[6.1 Manager Group – Group Option ⇒ Secretarys Call Forward]

- Processing option designation if a manager calls other manager

[6.1 Manager Group – Group Option ⇒ Manager to Manager Call]

- Class of manager group

[6.1 Manager Group – Group Option ⇒ Manager Class]

Interactions

- If a extension subscriber calls the manager, the call is terminated to manager/secretary according to the option.

If an extension subscriber calls the manager, the call is terminated to manager/secretary according to the option.

## Message Waiting

Functional description

If the called subscriber is absent, the function allows the calling subscriber to leave a message and notify the called subscriber of the message.

Operating procedure

- Message waiting registration in the general phone

1	A extension subscriber dials another subscriber’s number.

2	The called subscriber does not answer the call since the subscriber is absent.

3	The calling subscriber presses the Hook Flash button and enters the message registration code (Default:*71).

4	The calling subscriber hears the service set tone.

5	If the terminal to which the message waiting is registered is D-Tel, the following message is displayed.

MSG EXT(1) TRK(0) VM(0)

2000 – 01 - 01 09:00 am

- Message waiting answer in the general phone

1	The subscriber for whom a message is left hooks off the handset.

2	The subscriber hears the function registration tone notifying that there is a message registered.

3	The subscriber enters the message answer code (Default:*72).

4	The subscriber getting the message receives the calling tone while the subscriber who registered a message receives the calling signal.

5	The latter subscriber above answers the calling signal.

6	A call between the two subscribers is established.

- Message waiting cancellation in the general phone

1	The extension subscriber who leaves a message hooks off the handset

2	Enter the message cancellation code (Default:*700) followed by the number of the subscriber who registered the message waiting.

3	Hear the service set tone.

- Message waiting registration in D-Tel

1	A extension subscriber dials another subscriber’s number.

2	The called subscriber does not answer the call since the subscriber is absent.

3	Press the “CALLBK”(Call Back) button.

4	The calling subscriber hears the service set tone.

5	If the terminal to which the message waiting is registered is D-Tel, the following message is displayed.

MSG EXT(1) TRK(0) VM(0)

2000 – 01 - 01 09:00 am

- Message waiting answer in D-Tel

1	The message waiting is displayed on the LCD screen of the subscriber.

2	Press the “SPEAKER” button.

3	Press the “CALLBK”(Call Back) button.

4	The subscriber getting the message receives the calling tone while the subscriber who registered a message receives the calling signal.

5	The latter subscriber above answers the calling signal.

6	A call between the two subscribers is established.

- Message waiting cancellation in D-Tel

1	A extension subscriber hooks off the handset.

2	Enter the message cancellation code (Default:*700). At this time, the following message is displayed on the LCD screen of D-Tel.

ENTER A STATION NUMBER

3	Press the number of another subscriber who got the message waiting.

4	Hear the service set tone.

- Message waiting answer rejection in D-Tel

1	Press “CALLBK”(Call Back) button.

2	Press the “DND/CFWD” button.

3	Hear the service set tone.

4	At this time, the deleted message is not displayed on the LCD screen any more.

- Message waiting registration in the external subscriber

1	An extension subscriber calls a extension subscriber.

2	The called extension subscriber does not answer the call.

3	The extension subscriber is recovered.

4	The message waiting is automatically registered to the extension subscriber according to the message waiting registration option.

MSG EXT(0) TRK(1) VM(0)

2000 – 01 - 01 09:00 am

- deletion of all message waiting information registered to D-Tel

1	Dial the deletion code (Default:*70*) of all message waiting information.

2	All information is deleted and the result is displayed on the LCD screen of D-Tel.

- Deletion of all message waiting information registered to D-Tel by the request from the station

1	Dial the deletion code (Default:*701) of all message waiting information registered by the request from the station.

2	All information is deleted and the result is displayed on the LCD screen of D-Tel.

- Deletion of all message waiting information registered to D-Tel by the request from the extension

1	Dial the deletion code (Default:*702) of all message waiting information registered by the request from the extension.

2	All information is deleted and the result is displayed on the LCD screen of D-Tel.

- Deletion of all message waiting information registered to D-Tel by the request from the VPM

1	Dial the deletion code (Default:*703) of all message waiting information registered by the request from the VPM.

2	All information is deleted and the result is displayed on the LCD screen of D-Tel.

- One by one deletion of the message waiting information registered to D-Tel

1	The Reserve LED flashes if there is any registered message.

2	Press the Waiting button while the handset is in position.

3	If there are more than two registered messages, they are displayed on the LCD of the D-Tel in the order that they were received.

MESSAGE:XXXX 18:50

2000 – 01 - 01 09:00 am

4	To delete any registered message, the subscriber should press the Reject/Forward button while the message to be deleted is displayed on the LCD.

Service conditions

- If the message is not registered normally, hear the lockout tone.

- Only one message can be registered to a general extension subscriber.

- The message waiting cannot be registered to the Hot Line subscriber.

- The message waiting cannot be registered to the calling group or representative attendant.

- Only one message can be registered to a general phone.

- If the message waiting is requested in the status that the message waiting was registered to the general phone, the request is processed as errors.

- Up to 10 message waiting can be registered to each D-Tel.

- If the subscriber already registered requests another message waiting for registering the message waiting to D-Tel, the request is disregarded.

- If the message waiting is requested from the extension to D-Tel, the number of waited messages, which are registered in the system, from the extension should not exceed 2000 messages.

- If the number of waited messages in D-Tel is 10 and the message waiting is requested from the station, the request is registered only in case that there should be message waiting from the station among the existing message waiting contents registered. Note that at this time, the first registered information among the messages that were registered by the request from the station is deleted.

- In case that the message waiting is requested from the extension to D-Tel in the status that the number of messages waited from the extension in the system is 2000, if there are messages waited from the extension to related D-Tel, the oldest information is deleted and the requested message is waited.

- If the message waiting is registered to D-Tel, the number of registered message waiting for each station/extension/VMS is displayed.

- The telephone for message waiting connected to SLC-MSW B’d can designate that it informs the subscriber of the message waiting registration by turning on the message waiting lamp for registering the message waiting,

- If the general phone or the telephone for message waiting is designated so that it does not inform the subscriber of the message waiting registration by turning on the message waiting lamp for registering the message waiting, notify the subscriber of message waiting status by periodically sending the ring to the subscriber who registered the message waiting.

- Once a call is set up by the message answer code, the registered message is automatically cancelled.

Data generation

- Designate whether displaying the message waiting registration to the general phone subscriber with ring, or lamp when registering the message waiting to the general phone

[3.1 Extension Information – Extension Characteristic ⇒ 8 Message Wait Lamp]

- Ring information used for informing the subscriber of the message waiting registration to the general phone

[2.5 System Ring Parameter ⇒ Message Wait Request Ring]

- Automatically allow the message waiting when the extension subscriber terminated from the ISDN trunk does not answer the call

[4.9 All option by trunk route ⇒ Auto Message Wait registration(CLI)

## MSN (Multiple Subscriber Number)

Functional description

If the ISDN BRI is used as trunk by connecting it with Point to Multi-Pointer mode, the function can terminate up to 8 numbers, which is not matched with the system numbering plan that is contained in the called party number from the remote station for call setup, to the extension subscriber number, calling group representation number and external subscriber number.

Operating procedure

Service conditions

- The related line should be designated as the MSN subscriber.

- If only the first B line out of 2B is designated for the function, the MSN termination designation is used for both of 2B.

- If the MSN termination and DIL termination are simultaneously produced in the identical line, the DIL termination has the precedence over the MSN for service.

- The line that is designated as MSN automatically operates with Point to Multi-Point Mode.

- The station MSN (Network Mode) is not supported.

- It is not possible to use the same trunk simultaneously with MSN and Sub Address modes.

Data generation

- Trunk designation to be used with MSN mode

[4.1 trunk information – Trunk Option ⇒ 19.ISDN BRI MSN Use]

- MSN termination designation

[7.3 MSN/Sub Address Destination Assign]

- Service Assign per Day/Night Mode for Abnormal IC trunk Call

[4.10 Trunk Route Base IC Day/Night Option]

Interactions

## Mute

Functional description

The function allows a D-Tel subscriber to receive the voice of the subscriber on the other end without sending the D-Tel subscriber’s voice during a call.

Operating procedure

- Mute in D-Tel

1	A D-Tel subscriber presses the Mute button during a call.

2	The Mute LED is turned on and the D-Tel subscriber’s voice is not sent, but the subscriber can hear the voice of the subscriber on the other end.

3	The D-Tel subscriber presses the Mute button again.

4	The Mute LED is turned off and the call is restored.

## Name Display

Functional description

If a extension subscriber registers his or her name as well as the phone number, the function displays the subscriber's name instead of his or her number when the subscriber is called/in a call/picked up.

Operating procedure

Service conditions

- The name display status should be designated.

- The name display status means whether the name designated in one’s own terminal is displayed on the other terminal.

- The name designated in the terminal of a subscriber to whom the dame display function is allowed is displayed on the target (attendant or D-Tel) terminal.

- Up to 9 digits of English character can be displayed.

Data generation

- Name display designation

[3.1 Extension Information – Feature Restriction ⇒ 8 Name Service]

- Name designation

[3.1 Extension Information ⇒ Name]

## Night Bar

Functional description

The function allows subscribers to classify the service modes of the system into day/night transfer time, Sunday and holiday so that they can restrict services according to the resultant classes as the modes automatically changed.

Service conditions

- The operation management sets up the rotation time of daytime/night and the assignment of holidays.

- It is possible to rotate the night bar as scheduled by setting up options for each tenant.

Data generation

- Automatic night bar assignment

[16.3 Night Bar]

Interactions

- It is possible to manually rotate the night bar according to attendant modes and to rotate daytime/night by changing the incoming mode of subscriber.

## Off-Hook Alarm

Functional description

The function notifies the alarm with attendant or printer when the handset is in off-hook status.

Operating procedure

- Off-hook alarm of general subscriber/D-Tel subscriber

1	The subscriber hooks off the handset.

2	A call is delivered to the subscriber assigned in advance a certain period of time after starting hearing the calling tone. Then, the calling subscriber will hear the dial tone.

3	The called subscriber hooks off the handset.

4	A call between the two subscribers is established.

Service conditions

- The device to which the result of line lock-out is reported should be assigned.

Data generation

- Assign the device to which the result of line lock-out is reported.

[2.6 System Feature Option ⇒ Bath Alarm Destination]

* 0: Howling Tone / 1: ATD / 2: PRT / 3: ATD & PRT

- Assign the time interval to automatically report the result of line lock-out to the device assigned in advance

[2.4 System Tone Parameter ⇒ Line Lock Out Tone]

Interactions

- If the result of line lock-out is reported to the attendant and the attendant answers it, the phone number of subscriber of which the line is locked out is displayed on the LCD screen.

## Other Group Call Pick-Up

Functional description

The function allows a subscriber to answer an incoming call ring to a extension subscriber in another pickup group instead of the called subscriber.

Operating procedure

- How to use the feature code

1	A extension subscriber in another pickup group rings.

2	Press the other group call pick-up code (Default:*2).

3	Enter the phone number of a subscriber who is ringing.

4	The call with the calling subscriber is established and the subscriber who is ringing is released.

- How to use the other group call pickup button

1	A extension subscriber in another pickup group rings.

2	Press the flexible button of D-Tel that is designated as other group call pick-up button.

3	Enter the phone number of a subscriber who is ringing.

4	The call with the calling subscriber is established and the subscriber who is ringing is released.

Service conditions

- The function is constrained to the subscribers who belong to the same pickup group as the subscriber who rings.

- The general phone can use how to use the feature code.

- The D-Tel can use both how to use the feature code and how to use the pickup button.

- To use the other group call pickup button, the specific flexible button should be designated as other group call pickup button.

Data generation

- How to designate the other group call pickup button

[Program + Flexible button + 3 + Function Number (12)]

- Pickup Group Definition

[6.5 Extension Pickup Group]

Interactions

- The subscriber can pick up the call of subscribers in the same pickup group by using the other group call pickup function.

## OG CDR (Outgoing Call Duration Restriction)

Functional description

If a extension subscriber is calling the outgoing trunk call, the function restricts the call duration such as forced disconnecting, warning tone by the option per class.

Operating procedure

Service conditions

- The allowed call duration, alarm tone sending and applied station class can be designated with option.

- The alarm tone and call duration restriction can separately operate by route and outgoing call type (ISD, DDD, TIE, Local).

[Determination of outgoing call type]

| ISD Call | If the maximum length from the first digit for origination is matched with the digit designated in the related menu. |
| --- | --- |
| DDD Call | If the maximum length from the first digit for origination is matched with the digit designated in the related menu and the code is different from the area code (own area code) of the site where the system is installed. |
| TIE Call | If the TIE line is designated in the related menu |
| Local Call | All calls except for ISD, DDD, TIE call |

Data generation

- Assign of Available call duration allowed without restriction

[4.13 trunk call duration restriction ⇒ Allow Time]

- Assign of call restriction mode

[4.13 trunk call duration restriction ⇒ Process Type]

- Assign of alarm tone sending cycle

[4.13 trunk call duration restriction ⇒ Alarm Periodic]

- Assign of digit deciding the international (ISD), toll (DDD), Local outgoing call

[1.1 system installation information

⇒ ISD Call Prefix, DDD Call Prefix, Own Area Code]

- Assign of trunk to be used for dedicated line

[4.1 trunk information – Trunk Option ⇒ Tie Line]

Interactions

## Outgoing Restriction

Functional description

The function allows subscribers to restrict outgoing calls by setting up their own password so that other subscribers cannot attempt outgoing calls, using the terminals not belonging to themselves.

Operating procedure

- Outgoing restriction registration

1	A extension subscriber hooks off the handset.

2	Enter the outgoing restriction registration code (*45).

3	Enter the subscriber’s own 4-digit password.

4	Assign the outgoing restriction type (0~3).

5	Hear the service set tine as the signal indicating that the registration is completed.

- Outgoing restriction cancellation in general phone/D-Tel

1	A extension subscriber hooks off the handset.

2	Enter the outgoing restriction cancellation code (*44).

3	Enter the subscriber’s own 4-digit password.

4	Hear the service set tine as the signal indicating that the registration is completed.

Service conditions

- This function can restrict outgoing calls only to general C/O trunks rather than dedicated lines.

- If a subscriber attempts an outgoing call to a C/O trunk which is registered in the function, the subscriber hears the hang-up tone.

- The outgoing restriction code and the outgoing restriction cancellation code should be specified when the system is produced.

- Outgoing restriction type: 0=Cancel, 1=LOCAL, 2=DDD, 3=ISD

Data generation

- 3.1 Extension Information – Extension Characteristic – 17 Local Restriction

- 3.1 Extension Information – Extension Characteristic – 18 DDD Restriction

- 3.1 Extension Information – Extension Characteristic – 19 ISD Restriction

Interactions

- If canceling the do not disturb function in the state that this function is not registered, the subscriber hears the hang-up tone.

- The subscriber cannot use the individual hold/system hold function but can use the individual waiting/system waiting function with the general phone.

- If this function is registered, the call forwarding function cannot be registered.

## Outgoing Trunk Queuing

Functional description

If a extension subscriber presses the extension calling code but all trunks are busy, the function allows the extension subscriber to be automatically connected to the first trunk, which becomes available by reserving a trunk call in advance.

Operating procedure

- Outgoing trunk queuing registration in the general phone

1	A extension subscriber enters the extension calling code, but all are busy at the moment.

2	The extension subscriber presses the Hook Flash button while hearing the busy tone and enters the outgoing trunk queuing code (Default:4).

3	The service set tone is sent to the extension subscriber.

4	The extension subscriber waits, hooking on the handset.

5	One of the trunks becomes available.

6	The calling signal is sent to the extension subscriber.

7	The extension subscriber answers the calling signal.

8	The extension subscriber is automatically connected to the trunk.

9	The subscriber dials an external subscriber’s number.

10	The extension subscriber and the external subscriber hear the calling tone and the calling signal, respectively.

11	The external subscriber answers the calling signal.

- Outgoing trunk queuing registration in D-Tel

1	A extension subscriber enters the extension calling code, but all are busy at the moment.

2	The extension subscriber presses the outgoing trunk queuing button or the Hook Flash button while hearing the busy tone and enters the outgoing trunk queuing code (Default:4).

3	The extension subscriber hears the service set tone and the following message is displayed on the LCD screen of D-Tel.

PLACED IN QUEUE FOR

LINE 10101 09:00 am

4	The extension subscriber waits, hooking on the handset

5	One of the trunks becomes available.

6	The calling signal is sent to the extension subscriber.

7	The extension subscriber answers the calling signal.

8	The extension subscriber is automatically connected to the trunk.

9	The subscriber dials an external subscriber’s number.

10	The extension subscriber and the external subscriber hear the calling tone and the calling signal, respectively.

11	The external subscriber answers the calling signal.

- Outgoing trunk queuing cancellation in the general phone

1	A extension subscriber who reserved an extension call hooks off the handset.

2	The subscriber dials the outgoing trunk queuing cancellation code (Default:*55).

3	The subscriber hears the service set tone which indicates that the outgoing trunk queuing is canceled.

- Outgoing trunk queuing cancellation in D-Tel

1	A extension subscriber who reserved a station call hooks off the handset.

2	The subscriber dials the “outgoing trunk queuing” button or the outgoing trunk queuing cancellation code (Default:*55).

3	The subscriber hears the service set tone which indicates that the outgoing trunk queuing is canceled.

Service conditions

- The system can register the station/extension trunk queuing simultaneously up to 100.

- If the step call is used in the system, this function cannot be used.

Interactions

- If the extension subscriber does not answer the calling signal in a certain period of time despite that one of the stations is available after the outgoing trunk reservation is registered, the outgoing trunk queuing is automatically cancelled.

- A extension subscriber is not allowed to reserve more than two trunk calls. Nevertheless, if a subscriber reserves more than two trunk calls, the previously reserved calls are all cancelled and only the last call is valid.

## Password Change

Functional description

The function allows a extension subscriber to be able to set up and change his or her own password.

Operating procedure

- Password change in the general phone

1	A subscriber hooks off the handset.

2	Dial the password change code (Default:*43).

3	Enter the current 4-digit password.

4	Enter the new 4-digit password.

5	Hook on the handset.

- Password change in D-Tel

1	Dial the password change code (Default:*43).

2	Enter the current 4-digit password.

3	Enter the new 4-digit password.

4	Press the “HOLD/SAVE” button.

Service conditions

- The initial password of all extension subscribers is “0000”.

- The initial password can be changed through the Data generation.

Data generation

- Password assignment

[3.1 Extension Information ⇒ Password]

## Privacy

Functional description

The function protects important calls or those of the subscribers engaged in data communications by preventing other subscribers from breaking into the calls.

Operating procedure

- Privacy assignment in the general phone

1	A subscriber in a call presses the Hook-Flash button.

2	The subscriber on the other end in a call hears the music on hold.

3	The subscriber who pressed the Hook-Flash button enters the privacy code (Default:*461).

4	The call between the two subscribers is resumed.

- Privacy assignment in D-Tel

1	A D-Tel subscriber in a call presses the “TRANS”(Transfer) button.

2	The subscriber on the other end in a call hears the music on hold.

3	The subscriber who pressed the “Forward” button enters the privacy code (Default:*461).

4	The call between the two subscribers is resumed.

Service conditions

- VMS subscribers should be defined as a privacy subscriber when system data are produced.

- If the subscriber registered the auto privacy function, all the calls of the subscriber are protected by the privacy function without designating the call as privacy by function code.

- If a subscriber who registered the privacy function finishes with the call, the privacy function is automatically cancelled.

Data generation

- Automatic privacy function assignment

[3.1 Extension Information – Extension Characteristic ⇒ 14 Auto Privacy]

Interactions

- The functions such as override and call waiting are unavailable once the privacy function is activated.

## Recall Button in D-Tel

Functional description

The function provides the following services by using the recall button of D-Tel.

Type 1.  The same line re-occupation after the recovery of existing call during an ACO station call

Type 2.  Transmission of the hook flash signal to the target station during an ACO station call

Type 3.  Call re-attempt after the existing call recovery of E&M, LD, RD, DTC-CAS, ISDN, QSIG, VOIP station calls

Operating procedure

- Type 1

1	A call between the D-Tel subscriber and the external subscriber is established through ACO.

2	The D-Tel subscriber presses the recall button. – At this time, the existing call is recovered and reoccupies the same line. Therefore, the D-Tel subscriber hears the station dial tone from the PX party.

3	The D-Tel subscriber dials the external subscriber’s number excluding the station access code.

4	The called subscriber answers it and a call is established.

- Type 2

1	A call between the D-Tel subscriber and the extension subscriber of target PBX connected through ACO is established.

2	The D-Tel subscriber presses the recall button. – At this time, the hook flash signal is transmitted to the target PBX party without recovering the existing call. Therefore, the subscriber hears the station dial tone from the PBX party.

3	The D-Tel subscriber dials the station number of target PBX.

4	The called subscriber answers it and a call is established.

- Type 3

1	A call between the D-Tel subscriber and the external subscriber is established through E&M, LD, RD, DTC-CAS, ISDN, QSIG, and VOIP.

2	The D-Tel subscriber presses the recall button. – At this time, the existing call is recovered and the D-Tel subscriber hears the station dial tone.

3	The D-Tel subscriber dials the external subscriber’s number including station number or station access code.

4	The called subscriber answers it and a call is established.

Service conditions

- The related service can be provided through the model where the D-Tel recall button exists.

- To use the ACO trunk with Type 2, the related trunk line should be designated as Tie line.

Data generation

- Assign of trunk to be used for Tie line

[4.1 trunk information – Trunk Line Option ⇒ Tie Line]

- Assign of hook flash time to be sent to the target station

[4.8 Trunk Lock parameter ⇒ ACO Hook Flash Duration Time]

Interactions

## Revertible Ring

Functional description

The function allows extension subscribers to check their numbers or verify whether the calling signal normally works.

Operating procedure

- Revertible ring in the general phone/D-Tel

1	A extension subscriber hooks off the handset.

2	Dial his or her number.

3	Hear the service set tone.

4	Hook on the handset.

5	Hear the phone ring.

6	Hook off the handset.

7	Hear the check tone.

## Same Group Call Pick-Up

Functional description

The function allows a subscriber to answer an incoming call ring to another extension subscriber in the same pickup group instead of the called subscriber.

Operating procedure

- How to use the feature code

1	A extension subscriber in same pickup group rings.

2	Press the same group call pick-up code (Default:**).

3	The call with the calling subscriber is established and the subscriber who is ringing is released.

- How to use the same group call pickup button

1	A extension subscriber in same pickup group rings.

2	Press the flexible button of D-Tel that is designated as same group call pick-up button.

3	The call with the calling subscriber is established and the subscriber who is ringing is released.

Service conditions

- The function is constrained to the subscribers who belong to the same pickup group as the subscriber who rings.

- The general phone can use how to use the feature code.

- The D-Tel can use both how to use the feature code and how to use the pickup button.

- To use the same group call pickup button, the specific flexible button should be designated as same group call pickup button.

Data generation

- Pickup Group designation

[6.5 Extension Pickup Group]

- How to designate the same group call pickup button

[Program BTN + Flexible button + 3 + Feature code (02)]

Interactions

- The subscriber can pick up the call of subscribers in the same pickup group by using the other group call pickup function.

## Save and Repeat

Functional description

The function allows the extension subscribers to temporarily save numbers dialed and reuse them.

Operating procedure

- Number saving in the general phone

1	A subscriber presses the “Hook-Flash” button while being connected to a station or a trunk.

2	Dial the registration code (Default:#9).

3	The subscriber’s call status holds.

- Number saving in D-Tel

1	A subscriber presses the “MEMORY” button while being connected to a station or a trunk.

MEMORY 09:00 am

2	If pressing the “MEMORY” button, the number of subscriber in busy status is saved and the following message is displayed on the LCD screen.

REDIAL IS REGISTERED

3	The subscriber’s call status holds.

- Redial of saved number in the general phone

1	A extension subscriber hooks off the handset.

2	Dial the saved number redial code (Default:#9).

3	A extension subscriber’s or external subscriber’s number that the calling subscriber saved is redialed.

4	The called subscriber answers it.

5	A call is established.

- Redial of saved number in D-Tel

1	If pressing the “MEMORY” button, the following message is displayed on the LCD screen.

STATION XXXX

MEMORY 09:00 am

2	Dial the “#” button.

3	A extension subscriber’s or external subscriber’s number that the calling subscriber saved is redialed.

4	The called subscriber answers it.

5	A call is established.

Service conditions

- The phone number of a extension subscriber or external subscriber who is verified to be in busy state by the calling of the calling subscriber can be saved.

- If the digit saved is not the station/extension/dedicated line calling, the call is processed as errors.

- The digit can be saved up to 20 numbers except for the extension/dedicated line.

- If the subscriber saves a number when another number is already saved previously, the previous number is deleted. Also, once a number is saved, it is valid until another number is saved.

- A subscriber can store only one number.

Data generation

Interactions

- This function uses the same memory as the dial memo. Therefore, if this function is designated, the number saved by the dial memo is automatically deleted.

## Station Branch

Functional description

The function allows the subscriber designated as the station branch to answer the incoming call to another subscriber in same pickup group by hooking off the handset.

Operating procedure

- Pickup in the general phone/D-Tel

1	Another subscriber in the same pickup group rings.

2	Hook off the handset and answer the calling.

3	A call with the calling subscriber is established.

Service conditions

- This function is constrained to the subscribers who belong to the same pickup group.

- The picking up subscriber should be able to use the station branch function.

- The picked up subscriber should be able to use the station branch function.

- The picked up subscriber can be only the extension subscriber.

Data generation

- Assign of Station Branch usage

[3.1 Extension Information – Characteristic ⇒ 16. Station Branch]

- Pick-Up Group designation

[6.5 Extension Pick-Up Group]

## Station Call Back

Functional description

If the line of station Subscriber A who is called is busy, the function makes a call back so that when the busy line becomes available, the calling signal is automatically sent to the calling Subscriber B for whom a station call is automatically reserved. If the Subscriber A hands off the handset after receiving the calling signal, another calling signal is sent to Subscriber B and finally a call between them is established.

Operating procedure

- Station call back registration in the general phone

1	Dial a extension subscriber’s number.

2	Hear the busy tone since the called subscriber’s line is busy.

3	Press the Hook Flash while hearing the busy tone and dial the call back code (Default:4).

4	Hear the service set tone which indicates that the call back is completed.

5	Hook on the handset and wait.

6	The call of the subscriber who a call is reserved for finishes.

7	The calling signal is sent to the subscriber who reserved a call.

8	The subscriber who reserved a call hooks off the handset.

9	The calling tone and the calling signal are sent the subscriber who reserved the call and the other subscriber, respectively.

10	The other subscriber answers the calling signal.

11	A call is established.

- Station call back registration in D-Tel

1	Dial a extension subscriber’s number.

2	Hear the busy tone since the called subscriber’s line is busy.

3	Dial the call back code(Default:4) after pressing the call back button or the Hook Flash while hearing the busy tone.

4	Hear the service set tone which indicates that the call back is completed.

The following message is displayed on the LCD screen of D-Tel.

PLACED IN QUEUE FOR

STA XXXX 09:00 am

5	Hook on the handset and wait.

6	The call of the subscriber who a call is reserved for finishes.

7	The calling signal is sent to the subscriber who reserved a call.

8	The subscriber who reserved a call hooks off the handset.

9	The calling tone and the calling signal are sent the subscriber who reserved the call and the other subscriber, respectively.

10	The other subscriber answers the calling signal.

11	A call is established.

- Call back cancellation in the general phone

1	The subscriber who reserved a call to another extension subscriber hands off the handset.

2	Dial the call back cancellation code (Default:*55).

3	Hear the service set tone which indicates that the function is cancelled.

- Call back cancellation in D-Tel

1	The subscriber who reserved a call to another extension subscriber hands off the handset.

2	Press the Call Back button or dial the call back cancellation code (Default:*55).

3	Hear the service set tone which indicates that the function is cancelled.

Service conditions

- The system can simultaneously register the station/extension call back up to 100 calls.

- If the step call is being used in the system, this function is not available.

- If a extension subscriber dials the call back cancellation code without a call back, he or she will hear the lockout tone.

- If the called subscriber receives the calling signal as the subscriber’s line becomes available after the call back registration, the call back is automatically cancelled.

Interactions

- If there is no response from the called subscriber in a certain period of time despite that a calling signal is sent to the subscriber as the subscriber’s line becomes available after the call back registration, the call back is automatically cancelled.

- A extension subscriber may not reserve more than one call. If the subscriber does so, the previously reserved call is cancelled and the last call is reserved.

## SDMA (Station Message Detail Accounting)

Functional description

The function makes the information required for call charge calculation to be displayed on the designated device for trunk origination.

- Charging system supported

[Charging system by the system timer]

The charging system starts the charging by deciding the call start time without additional information and finishes the charging when the related call is disconnected. For the trunk with answer signal, this charging system starts the charging at the time when the answer signal is received and for the trunk without answer signal, it starts the charging after the time designated by the system elapsed.

[Charging system by using the charging information received for another station] – Refer to Advice Of Charge function.

The charging system, which is constrained to ISDN or QSIG trunk, depends on the information received from another station for all information on charging.

- Charging information output device

1	PC (Personal Computer)

The PC, which displays the typical detailed charging information through SIO or LAN, is used when there is an application program that can calculate various statistics by using the charging information received in the related PC.

- For the more description, refer to the additional charging output menu.

2	Attendant: When the trunk outgoing call of designated extension subscriber is finished, the call start time, call duration, caller number and others are displayed on the LCD of attendant. If the charging information occurs, the charging button LED of attendant blinks and then, if the operator presses the charging button, the related information is displayed on the LCD. The LED of charging button continuously blinks if there is any charging information unverified.

- Charging output information

1	Calling subscriber’s number

2	Called subscriber’s number (maximum 24 digits)

3	Extension number

4	Extension calling code

5	Call start date and time

6	Call end date and time

7	Forward subscriber’s number for call forwarding

8	Others

- Charging exclusions

1	Origination through the no charge trunk

2	Origination of the no charge extension subscriber

3	No charge external number

4	In case that the origination is through dedicated line and the subscriber is not all call charge extension subscriber in the status that the dedicated line origination is designated as no charge

5	In case that the origination is local origination and the subscriber is not all call charge extension subscriber in the status that the local origination is designated as no charge

Operating procedure

Service conditions

- The charging output device can select either or both of PC and ATD.

- The charging status of dedicated line outgoing call and local outgoing call can be selected.

- When using the LCR, the detailed charging information contains the number information actually sent as well as the number that the subscriber dials.

Data generation

- Assign of charging output device

[11.1 Charging/output option ⇒ Charge Result Output Device]

- Assign of no charge trunk

[4.1 Trunk Information – Trunk Option ⇒ 15. No Metering]

- Assign of no charge extension subscriber

[3.1 Station information – Extension Characteristic ⇒ 9. No charge]

- Assign of all call charge extension subscriber

[3.1 Station information – Extension Characteristic ⇒ 9.All Call Charge]

- Assign of no charge external subscriber number

[11.2 No charge code]

- Assign of dedicated line charging

[11.1 Charging/output option ⇒ Tie Call Charge]

- Assign of local outgoing charging

[11.1 Charging/output option ⇒ Local Call Charge]

- Assign of charging information attendant display subscriber

[3.1 Station information – Extension Characteristic

⇒ 38.Charge send to ATD]

- Assign of charging start time of trunk without answer signal

[4.8 Trunk route lock parameter ⇒ Inter Digit Lock]

Interactions

## Step Call

Functional description

When the extension subscriber is listening to the busy tone after making a call to the other busy subscriber and he/she is trying to make a call by changing the last digit only, this function enables him/her to do it by dialing the last digit only.

Operating procedure

- The extension subscriber (A) dials the telephone number of the extension subscriber (B).

- The call listens to the busy tone.

- Dials the last digit (0 ~ 9) only to make a call to the other the extension subscriber (C) who has the telephone number that is different in the last digit.

- The extension subscriber (A) will hear a dialing tone, whereas the extension subscriber(C) will hear a ringing sound.

- The extension subscriber (C) answers a call.

- Then, a call between the extension subscriber (A) and extension subscriber (C) is made.

Service conditions

- This function can be used in both the general telephone/D-Tel.

- The [2.6 System Feature Option – Step Call Use] item should be set to “YES”.

- This function should be used without hook flash.

- When the unavailable number is dialed, the call will hear a closing sound.

- This function cannot be used in the dial pulse telephone.

Data generation

- Determine whether step call is used or not

[2.6 System Feature Option ⇒ Step Call Use]

Interaction

- Step Call function cannot be used when the following One Digit Busy features are used.

1	“2“: Group Hunting function

2	“3”: Busy Override function

3	“4”: Call Back function

4	“5”: Call Waiting function

5	“6”: Camp On function

## Sub Address

Functional description

When the ISDN BRI is used as a trunk line by connecting it with the Point to Multi-Pointer method, this function enables up to eight types of numbers that does not match with the system numbering plan, which is included in the Called Party Sub Address from the remote station, to be processed as the incoming call of the extension subscriber number, representative number of the paging group, external subscriber number.

Operating procedure

Service conditions

- The concerned line should be assigned as the sub address subscriber.

- When assigning the sub address incoming, both 2B will be applied when only one line is assigned, which corresponds to the first B out of 2B.

- When both the sub address incoming and DIL incoming are produced in the same line, service of the DIL incoming shall take precedence.

- The line that is assigned as the sub address will be automatically operated in the Point to Multi-Point Mode.

- The same trunk line cannot be used in the MSN and sub address method at the same time.

Data generation

- Assign a trunk like as a method

[4.1 Trunk line information – Trunk Option

⇒ 19.ISDN BRI Sub Address Use]

- Assign Sub Address Incoming

[7.3 MSN/Sub Address Destination Assign]

- Service Assign per Day/Night Mode for Abnormal IC trunk Call

[4.10 Trunk Route Base IC Day/Night Option]

Interaction

## System Hold

Functional description

The System Hold function holds the busy extension/extension subscriber in the system. The counterpart will wait while listening to a hold sound. When the intended subscriber in the system is available, conversation can be resumed.

Operating procedure

- System Hold in the D-Tel

1	The caller is talking over the phone with the extension/extension subscriber.

2	Press the “HOLD/SAVE” Button if the [2.6 System Feature Option-Hold Preference] item is set to “SYS-HOLD”. Or, if it is set to “EXC-HOLD”, press the “HOLD/SAVE” Button twice.

3	When the System Hold function is successfully carried out, all mobile buttons will blink which contains the suspended subscribers in the mobile button.

4	The multiple function subscriber who wants to continue a call with the suspended subscriber will press the blinking mobile button.

Service conditions

- This function cannot be used in the general telephone.

- The hot line subscriber cannot use this function.

- Only the busy extension subscriber or extension subscriber can be put on the System Hold.

- The counterpart subscriber can be put on the System Hold only when the subscriber is registered in his/her own telephone or the mobile button of the connected DSS.

- When the counterpart is going to be put on the System Hold, he/she must be registered on the mobile button. If not, only the Individual Hold can be carried out.

- The counterpart subscriber can be put on the System Hold using the “HOLD” button when the hold type (EXC-HOLD/SYS-HOLD) is set to “SYS-HOLD” in the [2.6 System Feature Option-Hold Preference].

- The counterpart subscriber can be put on the System Hold pressing the “HOLD” button twice when the hold type (EXC-HOLD/SYS-HOLD) is set to “EXC-HOLD” in the [2.6 System Feature Option-Hold Preference].

Data generation

- Selecting Hold Type

[2.6 System Feature Option ⇒ Hold Preference]

Interaction

- If the subscriber is not registered in his/her own telephone or the mobile button of the connected DSS, the suspended subscriber will be put on the Pool/Loop button. In this case, only the Individual Hold, not the System Hold, can be carried out.

- Individual Hold/System Hold cannot be used but the Individual Call Park/System Call Park can be used.

- If the certain subscriber is put on the hold and released without an answer, the call will be made from the suspended subscriber.

## System Call Park

Functional description

This function enables the subscriber in busy to be parked in the system and let him to resume a call with the intended subscriber later.

Operating procedure

- Registering a system call park using the general telephone

1	Press the “Hook-Flash” in the busy state.

2	Press the registration code for a system call park (Default:#7).

3	The subscriber registering the system call park can hear confirmation sound when if it is successful.

4	The subscriber registering the system call park will hear hold sound.

- Registering a system call park using the D-Tel

1	Press the “TRANS”(Transfer) button in the busy state.

2	Press the registration code for a system call park (Default:#7).

3	The subscriber registering the system call park can hear confirmation sound when if it is successful, and the following message will be displayed on the LCD screen.

LINE HOLDING

LINE 10101 09:00 am

4	When function is successfully registered, the subscriber being in a system call park will hear hold sound.

- Responding to the system call park using the general telephone

1	Off-hook first.

2	Press the response code for a system call park (Default:#7).

3	Press the extension number of the subscriber who is put on the system call park.

4	A call is connected to the subscriber being in a system call park.

- Responding to the system call park using the D-Tel

1	Press the response code for a system call park (Default:#7).

2	Press the extension number of the subscriber who is put on the system call park.

3	A call is connected to the subscriber being in a system call park.

Service conditions

- Only up to 64 subscribers can use this function in the system.

- The ATD cannot be put on the System Call Park.

- When the subscriber in the System Call Park mode release, the System Call Park will be cancelled.

- If the subscriber who requested a system call park does not call again the subscriber being in the system call park, the call will be directed to the subscriber who requested a system call park. In this case, the subscriber who requested the System Call Park should be in the idle state. It not in the idle state, the System Call Park mode will be sustained.

Data generation

- Confirmation Sound change

[2.4 System Tone Parameter – EXT Camp On Holding Tone]

## System Predetermined Night Answer (System PNA)

Functional description

This function enables the system to terminate a call to a predetermined subscriber in the following cases.

1	When the System Predetermined Night Answer is selected as incoming assignment in case of abnormal trunk incoming occurrence.

2	When the night subscriber of the attendant is not assigned when the attendant is called in the night mode.

3	When the DIL incoming is not assigned, or the System Predetermined Night Answer is selected as the applicable incoming type for each system mode in time of DDI Divert service use.

Operating procedure

Service conditions

- Assignment of the System Predetermined Night Answer can be separated and operated by each tenant.

- Assignment of the System Predetermined Night Answer can be selected from the specific extension number, paging group or external subscriber.

- When assigning the System Predetermined Night Answer with the representative number of the paging group, the related paging group should be assigned as follows.

1	Rep. No: Should be included in extension numbering plan.

2	Service Type: Normal

3	Ring Service Type: Selectable from FIRST, UCD, MULTI and LONG IDLE.

4	Rep. Name: The name of the incoming subscriber that will be displayed on the LCD instead of the concerned trunk line number when he/she is the D-Tel or ATD.

5	Other items are irrelevant

- If the telephone number of the external subscriber is assigned as an incoming, connection between the incoming route and outgoing route should be allowed.

Data generation

- Assign a System Predetermined Night Answer

[16.4 System PNA Service]

- Assign Paging Group

[6.4 Calling Group]

- Connection Allow/Deny Option between Routes

[4.12 Inter Route Connection Allow/Deny]

- Service Assign per Day/Night Mode for Abnormal IC trunk Call

[4.10 Trunk Route Base IC Day/Night Option]

- Assign Attendant Night Subscriber

[5.7 Attendant Night Subscriber]

- Assign Individual Incoming

[6.2 DIL Incoming Group]

- DDI Divert Function Option

[6.9 Trunk Route Base All Option ⇒ All DDI Divert Use]

- Assign an Incoming Type for each System Mode

[16.4 Night Bar]

Interactions

## System Speed Dial

Functional description

This function enables the subscriber to make a call to the counterpart subscriber by dialing the speed dial code only. The subscriber can save up to 2,000 speed dials and divide the user group into up to 24 groups. Each speed dial can be assigned with the applicable class of the extension subscriber. The name of the external subscriber can to registered also.

Operating procedure

- System Speed Dial Use 1.

1	Pick up the receiver (off-hook).

2	Dial the system speed dial number.

3	The call is establish if the opposite party (external subscriber) answers.

- Using after check System Speed Dial. (available only for D-Tel)

1	Press “PROGRAM” button.

2	Press “MEMORY” button.

3	Dial the system speed dial number.

4	The phone number and name of the external subscriber in accordance with the dialed system speed dial number is displayed.

5	Search the desired subscriber information using UP(),DOWN() button

6	Press “HOLD/SAVE” button, if the desired subscriber is displayed.

7	The ringing signal is sent to the opposite party and the subscriber hears the Ring Back Tone.

8	The call is establish if the opposite party answers.

Service conditions

- Speed dial code can use up to four digits and can have up to two digit numbering plan (e.g., 5XXX, 6XXX)

- Speed dial code can be allowed or denied according to each user group.

- When making a call to the external subscriber using the speed dial code, this function will be excluded from outgoing limit function of the trunk line since usage limit by the class for each speed dial code is applied.

- Additional digit dial option can be assigned to each speed dial code.

- The external telephone number that is assigned to each speed dial code can contain the Trunk Outgoing Pause Digit.

- When the CLI is received in time of IDSN trunk call incoming, the name of the concerned speed dial code will be displayed on the D-Tel or the LCD of the attendant instead of the line number of the incoming trunk, if it matches with the external number of the speed dial code.

Data generation

- Assign System Speed Dial Numbering Plan

[2.1 System Numbering Plan – Function Code Plan ⇒ System Speed Dial Code 1]

[2.1 System Numbering Plan – Function Code Plan ⇒ System Speed Dial Code 2]

- Assign System Speed Dial Code

[16.7 System Speed Dial]

- Assign System Speed Dial User Group

[16.7 System Speed Dial] – Submenu

- Additional Digit Dial Option for Each Speed Dial

[16.7 System Speed Dial] – Append Digit item in the Speed Dial Information

- Caller Name Display Option in Time of Incoming

[4.9 Entire Option for each Trunk Route ⇒ I/C Directory Service Use]

- Caller Name Display Option in Time of Incoming

[3.1 Extension Information – Extension Characteristic ⇒ 20.CLI Display]

Interactions

## Tenant Service

Functional description

Tenant service allows separation of the system into several functional groups, which is quite convenient when several companies share a single system or service should be separated by each department of the same company.

[Functions that can be separated by a tenant]

- Paging control between extensions

- Trunk outgoing control

- Trunk incoming control

- Attendant service

- System night subscriber service

- Common bell service

- Extension to Extension

- Transfer Extension to Extension

Operating procedure

Service conditions

- Depending on the type of service to separate, the extension and trunk can be separated into incoming tenant and outgoing tenant.

Data generation

- Assign the Extension Tenant - Both incoming and outgoing tenant can be assigned at the same time.

[3.2 Allocating Extension Tenant]

- Assign the Trunk Incoming Tenant

[4.5 Allocating Trunk Incoming Tenant]

- Assign the Trunk Outgoing Tenant

[4.5 Allocating Trunk Incoming Tenant]

- Assign the Trunk Occupancy Code

[4.2 Trunk Access Code]

- Assign System Night Subscriber

[16.4 System PNA Service]

- Assign Common Bell

[16.6 Common Bell]

- Assign the Extension to Extension Calling Allow/Deny Option

[2.7 Inter Tenant Connection Allow/Deny]

- Assign the Transfer Extension to Extension Allow/Deny Option

[2.7 Inter Tenant Connection Allow/Deny]

Interactions

## Through Dialing

Functional description

The operator makes an outgoing call after occupying the concerned trunk on behalf of the extension subscribers whose trunk outgoing is denied.

Operating procedure

- When the attendant provides the trunk only

1	The operator is busy with the extension subscriber.

2	The operator dials the trunk access code if the extension subscriber wants to make a trunk line call.

3	The serial number of the available trunk will be displayed on the attendant LCD and the extension subscriber will be automatically put on a hold and hear the hold sound.

4	The operator presses the “CONNECT” key.

5	The attendant LCD will be cleared, the extension subscriber hears the attendant dialing tone and the attendant is released.

6	The extension subscriber dials the telephone number of the external subscriber.

- When dialing all numbers of the external subscriber while making a call with the extension subscriber from the attendant.

1	The operator is busy with the extension subscriber.

2	The operator dials both the trunk access code and the telephone number of the external subscriber.

3	The extension subscriber will hear a hold sound, whereas the operator hears an attendant dialing tone.

4	The operator presses the “CONNECT”(Connection) key.

5	The attendant will be released. The extension subscriber on hold will make a call with the external subscriber.

Service conditions

- When the operator presses the “CLR”(Clear) button while listening to the trunk dial tone, the trunk will be released and a call with the extension subscriber will be resumed.

- When the operator presses the “RLS”(Release) button while listening to the trunk dial tone, both the trunk and extension subscriber will be released.

- When the operator checked to see if the busy counterpart subscribers are making a call after pressing the “CONNECT”(Connection) key, he/she can determine whether releasing it by pressing the “RLS”(Release) key or releasing it automatically by pressing the “CONNECT”(Connection) key instead of the “RLS”(Release) key.

Data generation

- Determines whether the attendant will press the “RLS”(Release) key or presumes to press the “RLS”(Release) key automatically in time of release after the attendant makes an outgoing call.

[5.6 ATD Service ⇒ Use ATD Force Release]

## Timed Reminder (Wake-Up)

Functional description

This function informs the time to the subscriber automatically when it arrives by sending the ring signal, if the general subscriber sets the specific time using his/her own terminal.

Operating procedure

- Registering a wake-up bell using the general telephone

1	The general subscriber lifts up the handset and hears the dial tone.

2	Dials a wake-up bell registration code (Default:*3).

3	Dials the time number (two digits).

4	Dials the minute number (two digits).

5	Dials the wake up type (one digit: 0 = today, 1 = tomorrow, 2 = everyday).

6	The general subscriber hears the Service Set Tone.

- Canceling a wake-up call using the general telephone

1	The general subscriber lifts up the handset and hears the dial tone.

2	Dials a wake-up cancellation code (Default:#3).

3	The general subscriber hears the Service Set Tone.

- Registering a wake-up bell using the D-Tel

1	When a wake-up bell registration code (Default:*3) is dialed, the current registration information will be displayed.

HH:MM:T (0/1/2)

ENTER YOUR WAKE UP TIME

2	Dials the time number (two digits).

3	Dials the minute number (two digits).

4	Dials the wake up type (one digit: 0 = today, 1 = tomorrow, 2 = everyday).

5	Press the “HOLD/SAVE” button.

6	The general subscriber hears the Service Set Tone.

- Canceling a wake-up bell using the D-Tel

1	The general subscriber lifts up the handset and hears the dial tone.

2	Dials a wake-up cancellation code (Default: #).

3	The general subscriber hears the Service Set Tone. At this time, erase status will be displayed as follows.

ERASED WAKE UP TIME

HH:MM

HH:MM  Time:Minute when registration is erased.

- Retrieving wake-up bell registration information at the attendant

1	Press the “WAKE UP” button.

2	Dials the extension number to retrieve wake-up bell registration information.

3	Wake-up bell registration information will be displayed on the LCD screen.

- Registering wake-up bell registration information at the attendant

1	Individual registration

1)	Press the “WAKE UP” button.

2)	Dials the extension number to retrieve wake-up bell registration information.

3)	Dials the time number (two digits).

4)	Dials the minute number (two digits).

5)	Dials the wake up type.

6)	Press the “ENT”(Registration) button.

2	Batch registration

1)	Press the “WAKE UP” button.

2)	Dials the first extension number to retrieve wake-up bell registration information.

3)	Dials the time number (two digits).

4)	Dials the minute number (two digits).

5)	Dials the wake up type.

6)	Dials the “*” key.

7)	The last extension number to register the wake-up.

- Canceling a wake-up bell at the attendant

1	Press the “WAKE UP” button.

2	Dials the first extension number to retrieve wake-up bell registration information.

3	Press the “FORC RLS”(Forced Release) BTN.

Service conditions

- Even though the type of wake-up bell is selected as “tomorrow”, it will be registered as “today” if the registered time is less than or equal to the current time.

- The subscriber can set the time interval of wake-up bell ringing. (When the bell will ring again.)

- Device can be selected that will be used to report the registration or cancellation statue when it normally carried out using the general telephone/D-Tel.

- The wake-up bell rings as many times as predetermined times until it is answered.

- Times of wake-up bell ringing can be set.

- When the subscriber answers to the wake-up bell call, wake-up registration information will be automatically cleared.

- When registration or cancellation using the ATD is normally carried out, its result will be reported to the PMS.

Data generation

- Defining a device that will report the result when the wake-up call is not answered.   (0 = No Service/1 = ATD)

- Defining a device that will report the result when the receiver of a wake-up call is busy. (0 = No Service/1 = ATD)

[10.4 Wakeup Service ⇒ Output Device in Line Busy]

- Defining a device that will report the result when wake-up bell registration or cancellation is normally carried out. (0 = No Service/1 = ATD)

[10.4 Wakeup Service ⇒ Output Device in Reg./Answer]

- Maximum number of times to make a wake-up call to the terminal that registered the wake-up bell.

[10.4 Wakeup Service ⇒ No Answer Re-ring Counter]

- Time interval (unit: minute) that the wake-up ring is generated again after sending a wake-up call for the first time.

[10.4 Wakeup Service ⇒ Re-ring Time Interval]

## Transfer to Attendant

Functional description

Transfers the call of the busy extension subscriber to the attendant.

Operating procedure

- Transfer of the extension subscriber to the attendant

- Press ”Hook Flash" or “TRANS”(Transfer) button on conversation.

2	The counterpart subscriber who is currently on the line will hear a hold sound and the extension subscriber will hear second dial tone.

3	When one of the “Attendant Calling Code”, “Attendant Telephone Number” or “Attendant Representative Number” is dialed, the subscriber on hold will be transferred to the attendant and hear the ringing sound, whereas the forwarding extension subscriber will hear the Service Set Tone.

4	When the attendant answers, a call between the transferred subscriber and an attendant will be made.

Service conditions

- Screened Transfer function is not provided.

Data generation

- Assign a Attendant Calling Code

[2.1 System Numbering Plan – Feature Code Plan ⇒ATD Calling Code]

- Assign a Attendant Extension Number

[3.1 Extension Information ⇒ Tel No]

- Assign a Attendant Representative Number

[5.5 ATD Representative No]

## Transit Service

Functional description

Under the following circumstances, the function enables the incoming call to be sent again to another trunk so that the call can be made while the extension subscriber is not included.

- When the trunk occupancy code is directly dialed from the DDI trunk.

- When the DIL incoming is assigned to the external telephone number and the concerned line received a call.

- When the extension subscriber who registered the external call forwarding is called from the trunk.

- When the extension subscriber who is on the line with the external subscriber transfers a call to another external subscriber.

- When the extension subscriber is released from the conference call and the call is made between two external subscribers finally.

- When the system PNA subscriber is assigned with the external telephone number and received assigned service.

- When the MSN/Sub Address incoming assignment is assigned with the external telephone number and the concerned line receives a call.

Operating procedure

- Extension and recovery procedure of a call when both trunks that are connected are not equipped with the recovery signal.

1	Two trunks are busy.

2	When three times of the assigned call time elapse that is permitted, one of the both external subscribers will hear a warning sound.

3	When the concerned external subscriber dials “*” while listening to the warning sound, the call will be maintained as long as designated period of time.

4	One of the both external subscribers will hear a warning sound again during the designated period of time (Step 3 and 4 are repeated.)

5	The concerned call will be automatically released if the external subscriber does not dial the “*” within the designated period of time.

Service conditions

- Connection between trunks should be in an allowed state.

- Allow/deny outgoing digit for each class of the incoming trunk will apply. (See class of service section).
 [Excluding case] It is excluded since allow/deny outgoing digit is applied according to class of the extension subscriber when registering the external call forwarding.

- If both trunks are not equipped with the recovery signal, forced recovery function will be applied according to the designated time period.

Data generation

- Trunk route separation

[4.1 Trunk Information ⇒ Trunk Route]

[4.7 Trunk Route Assign]

- Connection Allow/Deny Option between Routes

[4.12 Inter Route Connection Allow/Deny]

- Assign Day/Night Class for Trunk

[4.1 Trunk Information ⇒ Day Class and Night Class]

- Assign Individual Incoming

[6.2 DIL Incoming Group]

- Assign System PNA Subscriber

[16.4 System PNA Service]

- Assign ISDN MSN/Sub Address

[7.3 MSN/Sub Address Destination Assign]

- Assign Call Permission Time Between Trunks

[2.3 System Lock Parameter ⇒ CO to CO Conversation Lock]

- Selecting a Warning Sound in Time of Call between Trunks

[2.4 System Tone Parameter ⇒ CO to CO Lone Conv. Warning Tone]

Interactions

## Trunk Outgoing Pause Digit

Functional description

This function delays transmission of dialed digit for a certain period of time when sending it through a trunk. Application methods are different for the non-ISDN trunk and ISDN trunk. Generally, it is frequently used for system service that transmits many digits at a time without direct dialing from the subscriber. Usage example includes beep paging and banking service.

- Non ISDN trunk

It is limited only to the trunk with the DTMF dialing method. When the selected pause digit is dialed on the concerned route, each pause digit will delay transmission of the next digit for an assigned period of time.

- ISDN trunk

If the dialed digit contains the pause digit that is assigned to the concerned digit, the digit after the pause digit will not be transmitted in time of call setting but transmitted after changing the last digit into a DTMF when the call is made by the answer of the counterpart.

[Cases when the pause digit can be included]

- System Speed Dial

- Individual Speed Dial

- External Call Forwarding

- Mobile Button Speed Dial

- AND Digit of the Trunk Occupancy Code

- ARS Repeat Digit of the Trunk Occupancy Code

- Additional Digit in Time LCR Service

Operating procedure

Service conditions

- Different type of Pause Digit can be assigned to each route.

- In case of non-ISDN trunk outgoing, the pause time for each Pause Digit can assigned differently to each route for operation.

- Only one Pause Digit can be included in case of ISDN trunk outgoing.

Data generation

- Assign a Pause Digit for each Route

[4.9 Trunk Route Base All Option ⇒ What Digit Pause Assign]

- Assign Delay Time for each Pause Digit and Route

[4.9 Trunk Route Base All Option ⇒ Pause Time Assign].

Interactions

## Trunk Release Guard Time

Functional description

In time of station recovery, this function prevents the counterpart subscriber from occupying the concerned line when it detects the recovery signal and carries out the recovery procedure. This function is designed to remove problems that can occur when the new call is generated to the concerned trunk while the counterpart does not detect the recovery signal.

Operating procedure

Service conditions

- The followings are the case when the Guard time is applied in time of release.

1	Additional Digit in Time LCR Service

2	If it is a LD trunk outgoing.

Data generation

- Assign Guard Lock Time

[4.8 Trunk Route Lock Parameter ⇒ Guard Lock Out Confirm Lock]

Interactions

## Trunk Route Separation

Functional description

This function enables trunks generated in the system to be separated and operated independently from the tenant. The following service can be separated and applied.

- When service application should be differently operated due to a different protocol.

- When different initialization timing should be assigned to the local board because the connected remote station is different even though the protocol is same.

- When the different Allow/Deny Trunk Outgoing Digit option should be selected.

- When the different incoming should be assigned in time of abnormal trunk incoming occurrence.

- When usage of trunk-related service should be differently operated.

- When call limit in time of trunk outgoing should be differently operated.

Operating procedure

Service conditions

- A route can be separated up to eight routes for each system.

- Depending on service for each route, connection can be allowed or denied (in time of direct call, call forwarding by the extension subscriber or transmit by the attendant subscriber)

Data generation

- Trunk route separation

[4.1 Trunk Information ⇒ Trunk Route]

[4.7 Trunk Route Assign]

- Allow/Deny Outgoing Digit Table by Subscriber Class in Time of Trunk Outgoing for each Route

[3.3 Allow/Deny Outgoing Digit Table by Class]

- Service Assign per Day/Night Mode for Abnormal IC trunk Call

[4.10 Trunk Route Base IC Day/Night Option]

- Assign Call Time Limit by Route

[4.13 Trunk O/G Call Duration Restriction ⇒ Allow Time]

Interactions

## Trunk Selection

Functional description

This function enables the subscriber to select and occupy the trunk by dialing its serial number at the attendant.

Operating procedure

- Station selection at the attendant

1	Press the “TRK SEL”(Trunk Selection) key at the attendant.

2	Dial the trunk serial number (four digits).

3	The selected trunk is occupied.

Interactions

- When the busy trunk is selected, the subscriber will hear a closing sound.

- When another subscriber occupies the selected trunk, it can be released compulsory or overrode by the attendant. Refer to [5.6 ATD Service option  use ATD Force Release ?]

## Verify Busy Line Status

Functional description

Using this function, Digital Display – Station, Digital Display – Trunk, Incoming Call Identification, Called Number Display – Attendant and Call Processing Indication information will be displayed on the attendant LCD screen, when the operator calls other subscriber, or other subscriber calls the operator, or the operator is in busy with other subscriber, or the call is forwarded by the call forwarding of another subscriber.

Operating procedure

- Displaying the extension subscriber state

1	The operator dials the telephone number of the extension subscriber.

2	Various extension subscribers’ information will be displayed on the attendant LCD such as the telephone number, class, tenant number and current state.

- Displaying the trunk state

1	The operator dials the trunk serial number after pressing the trunk code or pressing the “TRK SEL” key.

2	The trunk number, route number, tenant number or trunk code of the incoming or outgoing trunk call will be displayed on the attendant.

- Displaying incoming call

1	The attendant presses the “ANSWER” key.

2	Type of incoming call will be displayed.

3	The attendant can hear the Service Set Tone as a sign of registration completion.

- Displaying forwarding information

1	A transfer call is generated at the attendant.

2	The telephone number of the subscriber who registered the call forwarding, not the original outgoing subscriber, will be displayed on the attendant LCD screen.

- Displaying call progress state

1	The progress status that is held by the attendant will be displayed on the attendant LCD screen.

- Displaying usage state

1	The operator dials the subscriber’s telephone number.

2	He/she hears the busy sound since the line is busy.

3	Presses the “BRK IN”(Break-In) key.

4	The “OVERRIDE” sound will be given to the busy subscriber and the call will be changed to a three-person conference call.

Service conditions

- In order for the extension subscriber to transfer to the attendant, he/she has to use the designated extension number. Call transfer cannot be made using the attendant page code or attendant representative number.

## Virtual Dial Tone

Functional description

When sending a call from the trunk, this function allows sending the dial tone to the dialers right after occupying the trunk line.

- When the receiving station sends the dial tone lately.

- When the receiving station does not send the dial tone.

- When the receiving station cannot send the dial tone. (R2MFC Dial Type).

Operating procedure

Service conditions

- When the AND Digit is used, the virtual dial tone will continuously sound before the subscriber dials the first digit of the external subscriber.

Data generation

- Assign the trunk that will use the virtual dial tone

[4.1 Trunk Information – Trunk Option ⇒ Virtual Dial Tone Item]

Interactions

## Walking COS (Remote DOD)

Functional description

This function enables the subscriber to use the low class telephone as if using his/her own high class telephone.

Operating procedure

- When using the Account Code in the general/D-Tel telephone

1	The extension subscriber lifts up the handset.

2	Dials the Walking COS code (Default:*78). In case of the D-Tel telephone, the following message will be displayed on the LCD screen.

ENTER A STATION NUMBER

3	Dials the extension number of the extension telephone to use. In case of the D-Tel telephone, the following message will be displayed on the LCD screen.

ENTER CURRENT PASSWORD

4	Dials the four digits password of the extension telephone to use. In case of the D-Tel telephone, the following message will be displayed on the LCD screen.

More Fet.Code/Acc. Code

5	Dials the Feature Code of the Account Code (Default:#4). In case of the D-Tel telephone, the following message will be displayed on the LCD screen.

ENTER AN ACCOUNT CODE

6	Dials the four digit Account Code. In case of the D-Tel telephone, the following message will be displayed on the LCD screen.

Enter Acc. Code + Tel No

7	When the station Access Code is dialed, the idle station will be occupied.

8	Dials the telephone number of the external subscriber.

9	When the external subscriber answers, a call will be made.

- When not using the Account Code in the general/D-Tel telephone

1	The extension subscriber lifts up the handset.

2	Dials the Walking COS code (Default:*78). In case of the D-Tel telephone, the following message will be displayed on the LCD screen.

ENTER A STATION NUMBER

3	Dials the extension number of the extension telephone to use. In case of the D-Tel telephone, the following message will be displayed on the LCD screen.

ENTER CURRENT PASSWORD

4	Dials the four digits password of the extension telephone to use. In case of the   D-Tel telephone, the following message will be displayed on the LCD screen.

More Fet.Code/Acc. Code

5	When the station Access Code is dialed, the idle station will be occupied.

6	Dials the telephone number of the external subscriber.

7	When the external subscriber answers, a call will be made.

Service conditions

- The subscriber with “Room Cut” cannot use this function.

- If the extension class currently in use is lower than “E Class”, this function cannot be used.

- When the password of the extension number is wrong or set to “0000”, this function cannot be used.

Data generation

- Assign Day Class

[3.1 Extension Information ⇒ Day Class]

- Assign Night Class

[3.1 Extension Information ⇒ Night Class]

- Assign a Password

[3.1 Extension Information ⇒ Password]

- Selecting “Room Cut” status

[10.1 Room Subscriber ⇒ Room Cut/Off]

## Warm Line

Functional description

If an emergency case occurs for the general extension subscriber that he/she cannot dial, this function enables him/her to make a call to the pre-assigned subscriber after a certain period of time by lifting up the handset only in order to notify an emergency case. Under normal condition, the subscriber can make a call by dialing the number within the specific period of time after lifting up the handset.

Operating procedure

- Direct call between extension subscribers

1	The subscriber lifts up the handset.

2	When a certain period of time elapses while hearing a dialing tone, a call is attempted to the pre-designated subscriber and the call can hear a ringing sound.

3	The counterpart subscriber lifts up the handset.

4	Makes a call with the caller.

Service conditions

- To receive this service, the subscriber should be assigned as the Warm Line subscriber.

Data generation

- Assign Warm Line

[16.1 Special Phone – Warm Line]

- Sets the time period when the automatic dialing will be made

[2.4 System Tone Parameter]

- Checks to see if the warm line is assigned

[3.1 Extension Information – Extension Characteristic ⇒ 2 Warm line]

Interactions

- One subscriber can be assigned as only one of the Hot Line/House Phone/Warm Line.

## DECT-Call Transfer

Functional description

The function allows a DECT subscriber to transfer the conversation subscriber to another extension subscriber.

Operating procedure

- Call transfer by the screened mode of the DECT subscriber.

1	The DECT subscriber is making a call with other extension or external subscriber.

2	The DECT subscriber presses the (R) button during a call. At this time, the subscriber on the other end of the line and the DECT subscriber hear the hold tone and dial tone, respectively.

3	The DECT subscriber dials the extension number to which the subscriber in busy state is to be transferred. At this time, the digits that the DECT subscriber dials are displayed on the LCD screen of the DECT terminal.

4	After the extension number is dialed, the calling tone is heard from the DECT terminal, the extension to be transferred rings, and the following message is displayed on the LCD screen of the DECT terminal.

CALL TO

STA XXXX

XXXX : Called Extension Number

5	If the extension subscriber who rings answers the call, a call is established and the following message is displayed on the LCD screen of the DECT terminal.

CONNECT TO

STA XXXX

6	If the DECT subscriber presses the TALK() button for release, a call between the subscriber in currently busy status and the subscriber held is established.

- Call transfer by the unscreened mode of the DECT subscriber.

1	The DECT subscriber is making a call with other extension or external subscriber.

2	The DECT subscriber presses the (R) button during a call. At this time, the subscriber on the other end of the line and the DECT subscriber hear the hold tone and dial tone, respectively.

3	The DECT subscriber dials the extension number to which the subscriber in busy state is to be transferred. At this time, the digits that the DECT subscriber dials are displayed on the LCD screen of the DECT terminal.

4	After the extension number is dialed, the calling tone is heard from the DECT terminal, the extension to be transferred rings, and the following message is displayed on the LCD screen of the DECT terminal.

CALL TO

STA XXXX

5	If the DECT subscriber presses the TALK() button for release, the subscriber held hears the calling tone and the ringing extension subscriber continuously rings.

6	If the subscriber who hears the calling ring answers the call, the DECT subscriber is released and a call between the subscriber receiving the calling tone and the subscriber receiving the calling ring is established.

Interactions

- If calling the wrong number or the subscriber in busy status, the DECT subscriber hears the lockout tone. In this status, if pressing the (R) button, the call transfer is cancelled and a call is resumed with the previous subscriber.

- When transferring the subscriber who is making a call with the DECT subscriber, if pressing the (R) button while the DECT subscriber hears the calling tone, the call transfer is cancelled and a call is resumed with the previous subscriber.

- If pressing INTERCOM() button during calling the subscriber to be transferred (status that the DECT subscriber hears the calling tone and the called subscriber rings), a call between the DECT subscriber and the subscriber held is established.

## DECT-DND (Do Not Disturb)

Functional description

DECT subscribers can reject all the incoming calls that they do not want to answer by enabling the function. They can also disable the function.

Operating procedure

- DND registration by using the PRE DIAL mode in the DECT telephone

1	The DECT subscriber dials the DND code (Default:*51).

2	The DECT subscriber presses the TALK() button.

3	The “Beep” sound is heard from the DECT terminal and the Phone( ) icon is displayed on the LCD screen.

4	Hear the service set tone, which indicates that the registration is finished, and the following message is displayed on the LCD screen of the DECT terminal.

DND REGIST

5	The DECT subscriber presses the TALK() button for release of the DECT terminal.

6	After that, if calling the DECT subscriber who registered the DND, hear the busy tone.

- DND registration by using the POST DIAL mode in the DECT telephone

1	The DECT subscriber presses the TALK() button.

2	The “Beep” sound is heard from the DECT terminal and the Phone) icon is displayed on the LCD screen.

3	The DECT subscriber dials the DND code (*51).

4	Hear the service set tone, which indicates that the registration is finished, and the following message is displayed on the LCD screen of the DECT terminal.

DND REGIST

5	The DECT subscriber presses the TALK() button for release of the DECT terminal.

6	After that, if calling the DECT subscriber who registered the DND, hear the busy tone.

- DND cancellation in the DECT telephone.

1	The DECT subscriber presses the TALK() button.

2	The “Beep” sound is heard from the DECT terminal and the Phone() icon is displayed on the LCD screen.

3	The DECT subscriber dials the DND cancellation code (Default:*50).

4	Hear the service set tone, which indicates that the registration is finished, and the following message is displayed on the LCD screen of the DECT terminal.

DND

CANCEL

5	The DECT subscriber presses the TALK() button for release of the DECT terminal.

6	After that, it is possible to call the DECT subscriber who cancelled the DND.

Service conditions

- If the DND function is restricted, it is not available.

Data generation

- DND permission status

[9.3 DECT Information – Feature Restriction ⇒ DND]

Interactions

- If the subscriber cancels the function when the function is not registered, the subscriber hears the lockout tone and the following message is displayed on the LCD screen of the DECT terminal.

DND

No Reg

- If the subscriber again registers the DND function when the function is already registered in the DECT terminal, the following message is displayed on the LCD screen of the DECT terminal.

ALREADY

DND REG

## DECT-Extension Call Back

Functional description

If the line of extension subscriber who is called by the DECT subscriber is busy, the function makes a call back so that when the busy line becomes available, the calling signal is automatically sent to the calling subscriber for whom a extension call is automatically reserved. If the extension subscriber hooks off the handset after receiving the calling signal, another calling signal is sent to DECT subscriber and finally a call between them is established.

Operating procedure

- Extension call back from the DECT subscriber.

1	A DECT subscriber dials the number of extension subscriber in busy status.

2	The DECT subscriber dials the call back code (Default:4) while hearing the busy tone.

3	The DECT subscriber hears the service set tone and the followings are displayed on the LCD screen of DECT terminal.

EXT C_BACK

Reg.

4	The DECT subscriber presses the TALK() button for release.

5	If the extension that the call is reserved for released, the DECT terminal rings.

6	If the DECT subscriber presses the TALK() button for answer, the extension that the call is reserved for rings and the followings are displayed on the LCD screen of the DECT terminal.

CALL FROM

STA XXXX

7	If the extension subscriber who rings answer the calling, a call is established.

- Extension call back cancellation from the DECT subscriber.

1	The DECT subscriber presses the TALK() button.

2	The DECT subscriber dials the call back cancellation (Default:*55) while hearing the dial tone.

3	The DECT subscriber hears the service set tone and the followings are displayed on the LCD screen of the DECT terminal.

C_BACK

CANCEL

4	The DECT subscriber presses the TALK() button for release.

Data generation

- Call back code designation

[16.5 One Digit Busy Service]

Interactions

- If a DECT subscriber dials the call back cancellation code without a call back, he or she will hear the lockout tone.

## DECT-Extension Calling

Functional description

The function allows the DECT subscriber to call an extension subscriber by using the TALK ON/OFF() button.

Operating procedure

- Extension subscriber calling by the PRE DIAL of the DECT subscriber.

1	The DECT subscriber dials the extension subscriber number. At this time, the digits dialed are displayed on the LCD screen of the DECT terminal.

2	The DECT subscriber presses the TALK() button.

3	The “Beep” sound is heard from the DECT terminal and the Phone() icon is displayed on the LCD screen. The following message is displayed.

CALL TO

STA XXXX

4	The DECT subscriber and the called subscriber hear the calling tone and the calling ring, respectively.

5	If the called extension subscriber answers the call, a call is established and the following message is displayed on the LCD screen of the DECT terminal.

CONNECT TO

STA XXXX

6	After a call, the DECT subscriber presses the TALK() button.

7	The Phone( ) icon disappears and the DECT terminal is released.

- Extension calling by the POST DIAL of the DECT subscriber.

1	The DECT subscriber presses the TALK() button.

2	Hear the dial tone from the DECT terminal after the “Beep” sound and the Phone( ) icon is displayed.

3	The DECT subscriber dials the extension number to be called. At this time, the digits dialed are displayed on the LCD screen.

4	The DECT subscriber and the called extension subscriber receive the calling tone and the ring, respectively.

5	If the called extension subscriber answers the call, a call is established and the following message is displayed on the LCD screen of the DECT terminal.

CALL TO

STA XXXX

6	If the called extension subscriber answers the call, a call is established.

7	After a call, the DECT subscriber presses the TALK() button.

8	The Phone( ) icon disappears and the DECT terminal is released.

Interactions

- If pressing the (R) button shortly once during dialing by the PRE DIAL mode, one digit dialed is deleted. If pressing the (R) button for two or more seconds, all digits dialed are deleted.

- After dialing the external subscriber’s number, pressing the INTERCOM() button instead of TALK() button is processed as same with pressing the TALK() button.

- If pressing the INTERCOM() button during calling the called subscriber (status that the DECT subscriber hears the calling tone and the called subscriber rings), the DECT subscriber hears the dial tone and the called subscriber is released. At this time, the DECT subscriber can dial other extension or trunk subscriber for calling.

## DECT-Internal Zone Paging

Functional description

The function allows DECT subscribers to page through the speakers of all D-Tels in the internal zone of a certain zone-paging group.

Operating procedure

- Zone paging in the DECT telephone

1	The DECT subscriber presses the TALK() button.

2	The “Beep” sound is heard from the DECT terminal and the Phone) icon is displayed on the LCD screen.

3	The DECT subscriber dials the zone-paging access code (*90).

4	The following message is displayed on the LCD screen of the DECT terminal.

ENT Zone?

5	Dial the zone number (00~19).

6	Provide the subscriber, who is supposed to make an announcement, with the ring that informs the paging. (LGP terminal)

7	Blink the Ring LED in order to inform the subscriber of the paging. (LKD terminal)

8	The ring is stopped and the speech path between the DECT terminal and the subscriber to make an announcement is established.

9	The DECT subscriber makes an announcement.

10	The announcement is heard through the speakers of all D-Tels.

11	Wait the answer without release after the announcement is finished. If the answer is not desired, hook on the handset.

12	The subscriber who wants to answer presses the zone paging answer code.

13	A call between the paging DECT subscriber and the paged subscriber is established and the remained D-Tel subscribers are all released.

Service conditions

- The usage of internal zone paging can be designated.

- The answer for entire zone paging is available in all D-Tels.

- The answer for specific zone paging is available only for the designated subscriber out of related internal zone paging subscribers.

- If a subscriber dials the zone-paging access code in the DECT terminal where the zone-paging is not allowed, the subscriber hears the lockout tone and the following message is displayed on the LCD screen of the DECT terminal.

Zpag Rej.

Data generation

- Inter zone-paging use status

[3.1 DECT Information – DECT Characteristic ⇒ Z-Paging Use]

## DECT-Last Number Redial

Functional description

The function automatically stores the last number dialed by a DECT subscriber and redials the number when the subscriber presses the redial code. The new one updates the previously stored number whenever the subscriber hooks off the handset and dials a new number. Note that special service codes are not redialed.

Operating procedure

- Last number redial in the DECT telephone

1	The DECT subscriber presses the TALK() button.

2	Press the redial code (Default: ##).

3	The number of the last station/trunk dialed by the subscriber is redialed.

4	The called subscriber answers.

5	A call is established.

Service conditions

- Maximum 12 digits except for the access code can be stored.

- If the digit last dialed is not a station/trunk/dedicated line calling, handle it as errors.

- The new one updates the last number whose redial was registered whenever the subscriber hooks off the handset and dials a new number.

## DECT-Linked Station Pair

Functional description

The function allows the actual two terminals to be used as if they are one terminal by connecting two different terminals.

Operating procedure

- If ☎/☏ are designated as the Linked Station Pair and ☎ is called, the processing is as follows.

1	☎:IDLE / ☏:IDLE  ☎, ☏ ring simultaneously. A call is connected with the first answering subscriber and the other subscriber is automatically released.

2	☎:IDLE / ☏:BUSY  Processed as BUSY.

3	☎:BUSY / ☏:IDLE  Processed as BUSY.

4	☎:BUSY / ☏:BUSY  Processed as BUSY.

5	☎:IDLE / ☏:DND/ABSENCE registration  ☎ rings.

6	☎:IDLE / ☏:CFWD registration  The Call Forwarding Port registered in ☏ rings.

7	☎:DND/ABSENCE registration / ☏:IDLE  ☏ rings.

8	☎:CFWD registration / ☏:IDLE  The Call Forwarding Port registered in ☎ rings.

9	☎:BUSY / ☏:DND/ABSENCE registration  Processed as BUSY.

10	☎:BUSY / ☏:CFWD registration  Processed as BUSY.

11	☎:DND/ABSENCE registration / ☏:BUSY  Processed as BUSY.

12	☎:CFWD registration / ☏:BUSY  The subscriber to whom the call is forwarded rings.

Service conditions

- The attendant cannot be designated as the linked station pair.

- The DSS terminal cannot be designated as the linked station pair.

- The linked station pair designation is required.

Data generation

- Linked Station Pair designation

[16.10 Linked Station Pair]

## DECT-Other Group Call Pick-Up

Functional description

The function allows a DECT subscriber to answer an incoming call ring to a extension subscriber in a pickup group other than his/her pickup group instead of the called subscriber.

Operating procedure

- Other group call pickup by using the PRE DIAL mode of the DECT subscriber.

1	A extension subscriber in another pickup group rings.

2	The DECT subscriber presses the other group call pick-up code (Default:*2).

3	Enter the phone number of a extension subscriber who is ringing.

4	The DECT subscriber presses the TALK() button.

5	The “Beep” sound is heard from the DECT terminal and the Phone() icon is displayed on the LCD screen.

6	The call between the DECT subscriber and the calling subscriber is established and the subscriber who is ringing is released.

- Other group call pickup by using the POST DIAL mode of the DECT subscriber.

1	A extension subscriber in another pickup group rings.

2	The DECT subscriber presses the TALK() button.

3	The “Beep” sound is heard from the DECT terminal and the Phone() icon is displayed on the LCD screen.

4	The DECT subscriber presses the other group call pick-up code (Default:*2).

5	Enter the station number of a extension subscriber who is ringing.

6	The call between the DECT subscriber and the calling subscriber is established and the subscriber who is ringing is released.

Service conditions

- The subscribers who belong to the same pickup group as the subscriber who rings can pick up the call by using this function.

Data generation

- Define Pickup Group

[6.5 Extension Pickup Group]

Interactions

- The subscriber can pick up the call of subscribers in the same pickup group by using the other group call pickup function.

## DECT-Outgoing Trunk Queuing

Functional description

If a extension subscriber presses the trunk access code but all trunks are busy, the function allows the extension subscriber to be automatically connected to the first trunk, which becomes available by reserving a trunk call in advance.

Operating procedure

- Outgoing trunk queuing from the DECT subscriber.

1	The DECT subscriber dials the trunk access code in busy status.

2	The DECT subscriber dials the outgoing trunk queuing code(Default:4) while hearing the busy tone.

3	The DECT subscriber hears the service set tone and the followings are displayed on the LCD screen of the DECT terminal.

TRK C_BACK

Reg.

4	The DECT subscriber presses the TALK() button for release.

5	If the trunk that a call is reserved for is in idle state after release, the DECT terminal rings.

6	If the DECT subscriber presses the TALK() button for answer, the trunk that a call is reserved for is occupied and the DECT terminal’s dial is available.

7	The DECT subscriber dials the external subscriber’s number. At this time, the digits dialed are displayed on the LCD screen of the DECT terminal.

8	If the external trunk subscriber answers the call, a call is established.

- Outgoing trunk queuing cancellation from the DECT subscriber.

1	The DECT subscriber presses the TALK() button.

2	The DECT subscriber dials the outgoing trunk queuing cancellation code (Default:*55) while hearing the dial tone.

3	The DECT subscriber hears the service set tone and the followings are displayed on the LCD screen of the DECT terminal.

C_BACK

CANCEL

4	The DECT subscriber presses the TALK() button for release.

Data generation

- Outgoing trunk queuing code assignment

[16.5 One Digit Busy Service]

Interactions

- If the subscriber cancels the outgoing trunk queuing from the DECT terminal, which did not reserve the call, the subscriber hears the lockout tone.

## DECT-Receiving an Incoming Trunk Call

Functional description

The function allows the DECT subscriber to answer the calling of external trunk subscriber.

Operating procedure

- DECT subscriber responses to an incoming trunk call.

1	A trunk is terminated to the DECT subscriber (A) and the incoming trunk ring is ringing. At this time, the following message is displayed on the LCD screen of the DECT terminal.

1/02/03

Ringing

1/02/03 : 1:Shelf No, 02:Slot No, 03:Line No

2	If the DECT subscriber presses the  button to answer the incoming trunk call, a call is established and the Phone( ) icon is displayed on the LCD screen of the DECT terminal. The following message is displayed.

CONNECT TO

LN 1/02/03

3	After a call, the DECT subscriber press the TALK() button for release.

4	The Phone( ) icon disappears and the DECT terminal is released.

Interactions

- If pressing the UP(),DOWN() button during a call, the call volume can be controlled.

- For the calls from the extension subscriber and the external trunk subscriber, individual different ring can be selected for use.  Refer to the [Incoming ring selection]

## DECT-Receiving an Station Call

Functional description

The function allows the DECT subscriber to answer the calling of extension subscriber.

Operating procedure

- DECT subscriber responses to station call.

1	An extension ring is ringing in the DECT terminal and the following message is displayed on the LCD screen of the DECT terminal.

CALL FROM

STA XXXX

XXXX : Calling Extension Number

2	If the DECT subscriber presses the  button to answer the extension call, a call is established and the Phone( ) icon is displayed on the LCD screen of the DECT terminal. The following message is displayed.

CONNECT TO

STA XXXX

3	After a call, the DECT subscriber press the TALK() button for release.

4	The Phone( ) icon disappears and the DECT terminal is released.

Interactions

- If pressing the UP(),DOWN() button during a call, the call volume can be controlled.

- For the calls from the extension subscriber and the external trunk subscriber, individual different ring can be selected for use.  Refer to the [Incoming ring selection]

## DECT-Same Group Call Pick-Up

Functional description

The function allows a DECT subscriber to answer an incoming call ring to another extension subscriber in the same pickup group instead of the called subscriber.

Operating procedure

- Same group call pickup by using the PRE DIAL mode of the DECT subscriber.

1	A extension subscriber in same pickup group rings.

2	The DECT subscriber presses the same group call pick-up code (Default:**).

3	The DECT subscriber presses the TALK() button.

4	The “Beep” sound is heard from the DECT terminal and the Phone( ) icon is displayed on the LCD screen.

5	The call between the DECT subscriber and the calling subscriber is established and the subscriber who is ringing is released.

- Same group call pickup by using the POST DIAL mode of the DECT subscriber.

1	A extension subscriber in same pickup group rings.

2	The DECT subscriber presses the TALK() button.

3	The “Beep” sound is heard from the DECT terminal and the Phone( ) icon is displayed on the LCD screen.

4	The DECT subscriber presses the same group call pick-up code (Default:**).

5	The call between the DECT subscriber and the calling subscriber is established and the subscriber who is ringing is released.

Service conditions

- The function is constrained to the subscribers who belong to the same pickup group as the subscriber who rings.

Data generation

- Define Pickup Group

[6.5 Extension Pickup Group]

Interactions

- The subscriber can pick up the call of subscribers in the same pickup group by using the other group call pickup function.

## DECT-Save and Repeat

Functional description

The function allows the extension subscribers to temporarily save numbers dialed and reuse them.

Operating procedure

- Save and repeat registration during an station call of the DECT telephone

1	The DECT subscriber is making a call with a extension subscriber after calling the extension subscriber.

2	The DECT subscriber presses the (R) button. At this time, the extension subscriber in busy status is held and hears the hold tone. In addition, the LCD screen of the DECT subscriber is cleared and then, the dial is available.

3	The DECT subscriber dials the save and repeat registration code (Default:#9).

4	The number of the extension subscriber in busy status is saved and a call between the extension subscriber held and the DECT subscriber is established.

- Save and repeat registration during a trunk call of the DECT telephone

1	The DECT subscriber is making a call with a trunk subscriber after calling the trunk subscriber.

2	The DECT subscriber presses the (R) button. At this time, the trunk subscriber in busy status is held and hears the hold tone. In addition, the LCD screen of the DECT subscriber is cleared and then, the dial is available.

3	The DECT subscriber dials the save and repeat registration code (Default:#9).

4	The number of the trunk subscriber in busy status is saved and a call between the trunk subscriber held and the DECT subscriber is established.

- Save and repeat in the DECT telephone

1	The DECT subscriber presses the TALK() button.

2	Dial the save and repeat code (Default:#9).

3	A extension subscriber’s or external subscriber’s number that the calling subscriber saved is redialed.

4	The called subscriber answers it.

5	A call is established.

Service conditions

- The phone number of a extension subscriber or external subscriber who is verified to be in busy state by the self-calling of subscriber can be saved.

- If the digit saved is not the station/trunk/dedicated line calling, the call is processed as errors.

- The digit can be saved up to 20 numbers except for the trunk/dedicated line.

- If the subscriber saves a number when another number is already saved previously, the previous number is deleted. Also, once a number is saved, it is valid until another number is saved.

- A subscriber can store only one number.

## DECT-Trunk Calling

Functional description

The function allows the DECT subscriber to call an external subscriber by using the TALK ON/OFF() button.

Operating procedure

- External subscriber calling by the PRE DIAL of the DECT subscriber.

1	The DECT subscriber dials the trunk access code.

2	The DECT subscriber dials the external subscriber’s number.

3	The digits dialed by the DECT subscriber are displayed on the LCD screen of the DECT terminal.

4	The DECT subscriber presses the TALK() button.

5	The “Beep” sound is heard from the DECT terminal and the Phone( ) icon is displayed on the LCD screen of the DECT terminal. In addition, the digits dialed by the DECT subscriber are displayed.

6	If the external subscriber answers the call, a call is established and the call duration is displayed on the second line in the LCD screen of DECT terminal.

7	After a call, the DECT subscriber presses the TALK() button.

8	The Phone( ) icon disappears and the DECT terminal is released.

- External subscriber calling by the POST DIAL of the DECT subscriber.

1	The DECT subscriber presses the TALK() button.

2	The dial tone rings after the “Beep” sound from the DECT terminal and the Phone( ) icon is displayed.

3	The DECT subscriber dials the trunk access code.

4	The DECT subscriber hears the trunk dial tone.

5	The DECT subscriber dials the external subscriber’s number. At this time, the digits dialed are displayed on the LCD screen.

6	The calling tone and the calling signal are sent to the DECT subscriber and the external subscriber, respectively.

7	If the external subscriber answers the call, a call is established and the call duration is displayed on the second line in the LCD screen of DECT terminal.

8	After a call, the DECT subscriber presses the TALK() button.

9	The Phone( ) icon disappears and the DECT terminal is released.

Service conditions

- The trunk access code should be produced.

- The trunk dialing should be available by the trunk access code.

Data generation

- Trunk access code production

[4.2 Trunk Access Code]

Interactions

- The digit can be entered up to 24 digits. If 20 or more digits are entered, the digits more than 20 are scrolled to the left for display.

- After receiving the answer from the trunk, only the first 10 digits are displayed.

- If pressing the (R) button shortly once during dialing by the PRE DIAL mode, one digit dialed is deleted. If pressing the (R) button for two or more seconds, all digits dialed are deleted.

- After dialing the external subscriber’s number, pressing the INTERCOM() button instead of TALK() button is processed as same with pressing the TALK() button.

- If pressing the INTERCOM() button during calling the called subscriber (status that the DECT subscriber hears the calling tone and the called subscriber rings), the DECT subscriber hears the dial tone and the called subscriber is released. At this time, the DECT subscriber can dial other station or trunk subscriber for calling.