TABLE OF CONTENTS

SYSTEM GENERAL	3

1.1	General Description	3

1.2	Main Specifications & Characteristics	3

1.2.1	Specifications	3

1.2.2	Characteristics	4

1.3	Services	4

1.3.1	Basic service	4

1.3.2	Supplementary service	5

1.4	SYSTEM ADVANTAGES	7

1.4.1	Reliability	7

1.4.2	Capability	7

1.4.3	Versatile integrated services	7

1.4.4	GUI (Graphic User Interface: CS-OMS Program)	7

1.4.5	Simple maintenance	8

1.4.6	Compact system	8

1.4.7	Multi-function keysets	8

1.4.8	Advanced CTI	8

1.4.9 	System Configuration	9

1.5	Slot Allotment	10

1.6	System Capacity	11

1.6.1	Line capacity	11

1.6.2	Line boards	12

1.7	Board Description	13

1.7.1	CPM2 (Central Processing Unit 2)	13

1.7.2	DTRU (DTMF Receiver Unit)	13

1.7.3	ASLC (Analog Subscriber Line Circuit)	13

1.7.4	ASLC-MW (Analog Subscriber Line Circuit – Message Wait)	13

1.7.5	DSLC-LGP (Digital Subscriber Line Circuit – LGP)	13

1.7.6	DSLC-LKD (Digital Subscriber Line Circuit – LKD)	14

1.7.7	LKDU (LKD Expansion Unit)	14

1.7.8	APTC (Analog PSTN Trunk Circuit)	14

1.7.9	DNIC (Digital Network Interface Circuit)	14

1.7.10	RMFC (R2-Multi-Frequency Controller)	14

1.7.11	BRIC (Basic Rate Interface Circuit)	14

1.7.12	BRIU (Basic Rate Interface Unit)	14

1.7.13	CLKU (Clock Unit)	15

1.7.14	ALTC (Analog Leased Trunk Circuit)	15

1.7.15	APM (Application Processing Module)	15

1.7.16	VPM (Voice Processing Module)	15

1.7.17	ITGC (Internet Telephony Gateway Circuit)	15

1.7.18	WTIC (Wireless Terminal Interface Circuit)	16

1.7.19	EXPU (Expansion Unit)	16

1.7.20	IOTU (IO Terminal Unit)	16

1.7.21	MMB (Main Mother Board)	16

1.7.22	EMB (Expansion Mother Board)	16

1.7.23	EPU-D (External Protection Unit - D)	16

1.7.24	EPU-I (External Protection Unit – I)	17

1.7.25	EPU-A (External Protection Unit –A)	17

1.7.26	PFTU (Power Failure Transfer Unit) / PFEU	17

1.8	PSU (Power Supply Unit)	18

1.8.1	PSU-A	18

1.8.2	PSU-D	19

1.8.3	RGU	20

1.8.4	TB/NF (Terminal Block / Noise Filter)	20

# SYSTEM GENERAL

## 1.1	General Description

CS1000 is a digital PBX (Private Branch Exchange) that has an open architecture providing a normal PSTN, ISDN interfaces, Internet Gateway for voice communication and multimedia service, CTI and VMS.

The platform is designed on the technologies of SPC (Stored Program Control) method and Distributed System Control and Non-Blocking Switching Matrix. And High density ASIC, PLD(Programmable Logic Device) designed by VHDL technology provides the reliable system operation and powerful OAM features.

The system’s scalability allows the administrator to configure the system with maximum 992 ports from below 100 ports with 3 building block cabinets.

## 1.2	Main Specifications & Characteristics

1.2.1	Specifications

| Item | Specification |
| --- | --- |
| Maximum capacity | 992 ports |
| CPU | 32 Bit MPC68360 Microprocessor - 25MHz, SPC |
| Switching method | Non-Blocking using S/T – Switch matrix |
| Memory | 16Mbyte DRAM / 0.5Mbyte PROM / 8Mbyte FDISK |
| I/O Interface | 3* SIO (Maximum 19.2Kbps) / 1 * LAN) port |
| Board size | 285(W)mm x 285(H) mm |

1.2.2	Characteristics

- Universal Slot (except the slots for a controller)

- Various network interface: Analog / Digital(E1/T1/BRI/PRI) PSTN, VoIP

- High density line card: Maximum 32 channels / board

- OAM through LAN

Advanced features (e.g. VMS / CTI / QSIG)

- Building block expansion: Basic system (10 slots; 288 ports), Expansion system (12 slots; 352 ports)

- System redundancy – Controller / PSU (DC power only)

- Application service – MOH, BGM(Back Ground Music), external paging

## 1.3	Services

1.3.1	Basic service

Single line telephone – Dial Pulse / DTMF type telephone

Digital keyset- LKD series (30D, 8DS, 2NS, DSS), LGP series (LGP200, LGP 30D, 44D)

Digital ATD – Digital desktop type attendant console

Analog PSTN trunk and Tie line – CO line, R/D, L/D, E&M 2W/4W

Digital PSTN trunk – E1(2.048 MHz, 30 channels), T1(1.544 MHz, 24 channels)

ISDN service – ISDN terminal, ISDN PC, G4 Fax, High speed data transmission, Screen phone

O&M terminal – System maintenance, operation, diagnosis

1.3.2	Supplementary service

Built-in type VMS service

Voice mail & Auto Attendant service – In order to play the voice mail recorded in VPM (Voice Processing Module), a user can retrieve the voice mail by dialing only the access code without mail ID.

ACD service

For the telemarketing, the system provides not only ACD function distributing the incoming calls to the agents evenly but also CTI for the connectivity with host computer and ACD Call Manager for the administration of the call handling that are necessary for ACD system operation. This feature may apply to the Customer Reception Center, Service Center and Emergency Call Reception Center.

Computer Telephony Integration service

Initialized from Call Center, the scope of CTI reaches up to maximizing the efficiency of productivity in the office by automation of office environment through interconnection of computer and communication system. CS1000 has been designed to address to these needs.

QSIG service

For the optimized interconnection with telecommunication systems – homogeneous / heterogeneous connection, CS1000 provides QSIG through ISDN PRI interface.

Hotel service

For the special customers such as Hotel, Hospital, CS1000 provides customized features - Check-In /Check-Out, Wake-Up, Room status, Call Charging.

Internet Telephony service (Voice over Internet Protocol)

As the Internet service range grows explosively, its main technology has come to get involved in the voice communication as well specially for the long distance call or international calls. The existing dedicated network for voice communication service and for data communication service can be done simply through LGIC proprietary built – in gateway.

Wireless service (DECT)

CS1000 provides mobility with the enterprise communication environment with GAP(Generic Access Protocol) standard.

## 1.4	SYSTEM ADVANTAGES

1.4.1	Reliability

CS1000 adopts the architecture that allows redundancy of control part, power supply part and signaling part and uses the upgraded CPU, even in the case of system overflow, the system performance maintains in normal state.

1.4.2	Capability

The maximum port capacity of CS1000 is 992 ports 3 stackable cabinets (BSU/ESU). And in all the line boards, there are several kinds of sub-board to be mounted onto them for small expansion. These characteristics make system suitable for wider range of customers and enable the initial investment cost stay in the lowest, which make the system categorized into small size system as well as the medium size one. Even in the view of system S/W, LGIC’s new release will be smoothly migrated into the existing system.

1.4.3	Versatile integrated services

The technical difficulties in interconnecting the different manufacturers’ products for various services have been always something unavoidable and painful. In order to minimize its time and cost, CS1000 provides built-in service modules for wireless service, voice mail service, Internet Telephony Gateway function.

1.4.4	GUI (Graphic User Interface: CS-OMS Program)

For the system operation and maintenance, CS1000 provides GUI OMS (Operating & Maintenance System) S/W running on Windows 95/98/NT instead of commands on DOS. The menu of S/W package consists of diagnosis, customer data administration, maintenance features. Also the MML(Man Machine Language) is optionally available for the expert technician.

1.4.5	Simple maintenance

For the maintenance service, CS1000 simplifies all the procedures by adopting only one controller and providing maintenance service from the remote site where the technicians are available.

The technician can make an access to the system through PSTN and get the most of the information such as system configuration, system malfunction, statistics and also change them as needed.

1.4.6	Compact system

With the latest digital technology, even the system reaches maximum 992 ports, the space for installation and power consumption remain at the level of small system. When any failure occurs during its operation, the system will isolate the failure on the spot so that the chain reaction can be minimized.

In the view of board size, by using LGIC’s proprietary ASIC controller and high density IC chips, maximum 32 channels are available in most of the line boards.

1.4.7	Multi-function keysets

To improve the productivity in the office environment and address to user’s various tastes, CS1000 suggests versatile digital keysets from the entry model to the luxurious one which are 5 variants of KD/E series and 4 variants of LKD series and 3 variants of LGP series. Using these keysets, the range of KTS function is available in CS1000.

1.4.8	Advanced CTI

Since CTI has been introduced, CTI (Computer Telephony Integration) technology has been adopted almost only for the call center application so far. However, as the telecommunication environment changes, CTI application reaches even to normal private enterprises which has eventually yielded ‘Office CTI’ concept. CS1000 provides a fast implemented inter-working path through TAPI, CSTA with computing system, which will give a customer a great potential for applications in the future.

1.4.9 	System Configuration

CS1000 is divided mainly into system (BSU/ESU1/ESU2), terminal and peripherals. The system includes BSU(Basic Shelf Unit) which consists of two control slots CPM0/CPM1, line board slots(LB1~LB10) and PSU slot, and ESU1/ESU2(Expansion Shelf Unit) which consists of line boards slots(LB1 ~ LB12) and PSU slots. Each PSU can be equipped with duplicated PSU-D and RGU or single PSU-A(AC/DC power). The peripherals(IOTU) consists of 3 units of RS-232C interfaces and one LAN port and the circuit for alarm.

Each line board is designed as universal plug-in type and in case of the cabinet expansion, flat cable to connect BSU and ESU1 or BSU and ESU1 and ESU2 is used.

[Figure - CS1000 System Configuration]

## 1.5	Slot Allotment

BSU (Basic Shelf Unit) consists of two control slots CPM0/CPM1 on the right side, 2 line board slots(LB1,LB2) that has 16 ports, 8 line board slots(LB3~LB10) that has 32 ports and one PSU slot.

ESU1/ESU2(Expansion Shelf Unit) consists of 2 line board slots(LB1,LB2) that has 16 ports, 10 line board slots(LB3~LB12) that has 32 ports and one PSU slot. Each PSU can be equipped with duplicated PSU-D (DC/DC power; PSU0/PSU1), duplicated ring generator(RGU0/RGU1) or single PSU-A(AC/DC power and Ring Generator).

System Size (Unit: mm)

| System | W | D | H |
| --- | --- | --- | --- |
| BSU | 610 | 450 | 510 |
| BSU / ESU1 | 610 | 450 | 985 |
| BSU/ESU1/ESU2 | 610 | 450 | 1460 |

## 1.6	System Capacity

1.6.1	Line capacity

| ITEM | BSU | BSU/ESU1 | BSU/ESU1/ESU2 | REMARK |
| --- | --- | --- | --- | --- |
| SLT (ASLC) | 288 | 640 | 992 |  |
| Digital keyset (DSLC-LGP) | 160 | 352 | 352 | LGP series |
| Digital keyset (DSLC-LKD) | 216 | 480 | 480 | LKD series |
| CO Line (APTC) | 160 | 352 | 544 |  |
| Analog Trunk- E&M,LD,RD (ALTC) | 120 | 264 | 408 |  |
| Digital trunk - E1/T1- (DNIC) | 240 | 480 | 480 |  |
| ISDN PRI (DNIC) | 240 | 480 | 480 |  |
| ISDN BRI -S/T- (BRIC) | 160 | 352 | 544 |  |
| Digital Attendant (DSLC-LGP) | 8 | 8 | 8 |  |
| Conference | 30 | 30 | 30 |  |
| DTMF Receiver (CPM) | 32 | 32 | 32 | Note1) |

Note1) - Additional DTMF receiver (DTRU; 8 CH) mountable on CPM.

1.6.2	Line boards

| LINE BOARD |  |  | SUB BOARD |  |  |
| --- | --- | --- | --- | --- | --- |
| NAME | MAX. Capa | DESCRIPTION | NAME | MAX. Capa | DESCRIPTION |
| ASLC | 32 | Analog extension | SLTU | 4 | Max. 8 units mountable |
| ASLC-MW | 32 | Analog extension with message wait |  |  |  |
| DSLC-LKD | 24 | Digital keyset (LKD, KD/E series) | LKDU | 12 | One unit mountable |
| DSLC-LGP | 16 | Digital keyset (LGP series) & digital ATD | N/A |  |  |
| APTC | 16 | Analog CO line | APTC | 4 | Max. 4 units mountable |
|  |  |  | CPTU | 4 | Call processing tone detection; Max. 4 units mountable |
| ALTC | 12 | Analog leased trunk | EMIU | 4(2) | E&M 2W(4W) |
|  |  |  | LDIU | 4 | Loop Dial |
|  |  |  | RDIU | 4 | Ring Down |
|  |  |  | COIU | 4 | Analog CO line |
| DNIC | 1 Link | PRI / E1 / T1 | N/A |  |  |
| RMFC | 16 | R2 signaling | N/A |  |  |
| BRIC | 16 | ISDN BRI | BRIU | 4 | So/To selectable |
| ITGC | 10 | Internet Telephony Gateway | N/A |  |  |
| WTIC | 32 | DECT | N/A |  |  |
| VPM | 4 hrs | VMS, Auto attendant (8 channels) | N/A |  |  |
| APM | N/A | 1*LAN port / 4*SIO / zip disk | N/A |  |  |

## 1.7	Board Description

1.7.1	CPM2 (Central Processing Unit 2)

CPM is a system controller that has controlling blocks for line boards for 34 LB slots. It has 8 ports DTMF receivers and carries out tone generation function, switching, system clock generation and controls the alarm out of the system and power and ring source.

An EPROM in CPM2 initializes CPU and the program in the disk will be downloaded to DRAM for system operation. The disk can be activated only by the active CPM2 when used in duplication mode.

1.7.2	DTRU (DTMF Receiver Unit)

DTRU is a sub board mountable on CPM/CPM2 that has 8 DTMF receiver per unit and the injection is done by 72 pins SIMM module.

1.7.3	ASLC (Analog Subscriber Line Circuit)

ASLC provides 32 analog extensions interface by mounting SLTU on it, it has its own CPU (ASIC) for call control for extensions and programmable CODEC filter (PEB2466H) for message communication with CPM/CPM2.

It is designed for gain & impedance control by S/W programming(OMS).

1.7.4	ASLC-MW (Analog Subscriber Line Circuit – Message Wait)

ASLC-MW provides 32 channel message wait SLT interface by mounting SLTU on it. The message wait lamp in SLT is activated by High-voltage(DC 90V). Other functions are same with ASLC board.

1.7.5	DSLC-LGP (Digital Subscriber Line Circuit – LGP)

DSLC-LGP provides 16 channels interface with LGP series LGIC proprietary digital keyset and DSS(Direct Station Selection). Maximum 4 DSS is available per board. And 8 channels out of total 16 channels can be used for digital attendant console interface.

1.7.6	DSLC-LKD (Digital Subscriber Line Circuit – LKD)

DSLC-LKD is an interface board for LKD series(LKD 30D, 8DS, 2 BTN, DSS) that has 12 ports, and it can be expanded up to 24 ports by mounting LKDU on it.

1.7.7	LKDU (LKD Expansion Unit)

LKDU enables DSLC-LKD to expand up to 24 ports from original 12 ports by adding 12 ports.

1.7.8	APTC (Analog PSTN Trunk Circuit)

APTC provides 16 channels interface for analog CO line by mounting ATCU on it.

1.7.9	DNIC (Digital Network Interface Circuit)

DNIC gives one link of digital trunk (E1 / T1) or ISDN PRI (Primary Rate Interface)

When system administrator want to change the system configuration during system operation, PRI/E1/T1 can be changed by jumpering and programming without changing the cards. And for E1/ 2.048 Mbps PRI, 120/75 can be optionally chosen by jumpering and in case of T1 or 1.544 Mbps PRI, only 100 is available.

1.7.10	RMFC (R2-Multi-Frequency Controller)

RMFC controls R2 signaling, MFR Shuttle with 16 port signaling channels. It is commonly used when using E1 / T1 trunk.

1.7.11	BRIC (Basic Rate Interface Circuit)

BRIC provides maximum 16 BRI ports (32 channels) by mounting 3 BRIUs on it. Basically, it provides 4 BRI ports without BRIC. Each 4 BRI port unit can be programmed to be used as S0 point or T0 point interface with jumper.

1.7.12	BRIU (Basic Rate Interface Unit)

BRIU is an expansion unit for BRIC that has 4 ports BRI on it.

1.7.13	CLKU (Clock Unit)

CLKU plays a role for network synchronization when using ISDN BRI interface in the system. In case that PRI is used with BRI, CLKU is not needed. However, when installing the system, the BRIC designated for master clock should be located in the MMB and CLKU be located in CN9 of MMB.

1.7.14	ALTC (Analog Leased Trunk Circuit)

ALTC provides the maximum 16 channel interfaces for analog leased trunk line, which could be E&M 2W/4W, L/D, R/D, CO line. The kind of interface depends on sub-boards to be mounted are EMIU, LDIU, RDIU, COIU.

In case of E&M 4W, one EUIU provides only 2 interfaces and each of ALTC can provide maximum 6 channels of E&M 4W.

1.7.15	APM (Application Processing Module)

APM is designed to give an advanced applications such as CTI and ACD features. It mainly consists of MC68360 for CPU and 8Mbyte flash ROM and 16Mbyte DRAM for memory and one LAN port.

1.7.16	VPM (Voice Processing Module)

VPM is a built-in type voice-processing module that provides VMS, Auto Attendant Service. It has 8 channels for voice processing and the recording time is 4 hours. By adopting DSP chip, it utilizes 8K compressed format as coding and decoding method.

1.7.17	ITGC (Internet Telephony Gateway Circuit)

ITGC is operating using RISC processor, which is integrated into system and provides 10 voice channels with the extensions and 1 LAN port for interconnecting with network. The basic technologies are H.323 for converting voice into packets and G.723.1, G.729A, FAX relay and additionally DTMF detection, echo cancellation.

1.7.18	WTIC (Wireless Terminal Interface Circuit)

WTIC is the one of peripheral cards of CS1000. The technology for the wireless communication is DECT(Digital European Cordless Telephone). Each WTIC is mainly composed of 1 CPU(MC68302), 2 FPGA (IspLSI2064), 4 ADPCMs (VP23020), 8 BMCs (PCD5043). Each WTIC supports 8 cells and each cell has 4 channels for simultaneous calls.

For the wireless service, CS1000 uses GAP(Generic Access Protocol) standard terminal and base station.

1.7.19	EXPU (Expansion Unit)

EXPU connects the BSU with ESU1/ESU2 for system expansion by flat cable. This unit is equipped on the MMB of BSU.

1.7.20	IOTU (IO Terminal Unit)

IOTU mainly provides the interfaces with I/O terminals and buzzer for system alarm. It provides 5 I/O ports that are 2 SIOs dedicated for system controller and 1 LAN port given from the left side of system cabinet and the other 2 SIOs for LB debugging.

1.7.21	MMB (Main Mother Board)

MMB is a backboard with the connectors for line boards, power and controller. IOTU & EXPU will be attached on it.

To store system S/W, 8M bytes flash disk is located on this board.

1.7.22	EMB (Expansion Mother Board)

EMB is backboard for ESU with the connectors for line boards and power supply unit.

1.7.23	EPU-D (External Protection Unit - D)

EXPU-D is an optional external surge protection unit that is used for E1/T1/PRI interface board of the system. It is equipped into the EPU box and maximum 4 units are available in one EPU box.

1.7.24	EPU-I (External Protection Unit – I)

EXPU-I is an optional external surge protection unit that is used for ISDN BRI interface board of the system. It is equipped into the EPU box and maximum 4 units are available in one EPU box.

1.7.25	EPU-A (External Protection Unit –A)

EXPU-I is an optional external surge protection unit that is used for ASLC, APTC, and ALTC of the system. It is equipped into the EPU box and maximum 4 units are available in one EPU box.

1.7.26	PFTU (Power Failure Transfer Unit) / PFEU

PFTU provides 8 ports emergency direct trunk connection with pre-assigned extensions when the power is down or when the controller fails. PFEU will provide additional 8 ports for expansion on it.

## 1.8	PSU (Power Supply Unit)

PSU is a reliable system power resource for feeding DC power to each board. CS1000 has two kinds of PSU that are PSU-D, PSU-A. PSU-D converts -54V to ±5V and operates in duplication mode. When PSU-D is used, an external rectifier to give DC -54V and RGU(Ring Generation Unit) to be equipped in the system are necessary.

PSU-A converts AC 230V/50 Hz commercial power to DC power –54V, ±5V and also provides a ring signal as well.

1.8.1	PSU-A

PSU-A works as a rectifier that converts the

Commercial AC 230V/50Hz to DC ±5V, -54V and

a ring signal source. The external battery can be

recharged by PSU-A and when the power is down,

the battery connected to PSU-A can take over the

role as the power source. In the front of PSU-A,

there are Power On LED which  shows its working

status, a power switch, a fuse and battery switch by

which the installer can disconnect the power feeding

from the battery when installing it.

[Figure - PSU-A Power]

- Specification of Input Power

| Min. input voltage | Rated input voltage | Max. input voltage |
| --- | --- | --- |
| AC 197V | AC 220V | AC 253V |

- Specification of Output Power

- DC Voltage

| Voltage | Max. currency | Output voltage limit |
| --- | --- | --- |
| -54 V | 6.0 A | ±3% |
| +5 V | 20 A | ±3% |
| -5 V | 1.0 A | ±5% |

- Output Ring

Output voltage limit: 85Vac, 25Hz Sine Wave ±5Vac

1.8.2	PSU-D

PSU-D receives DC–54V from the External

Rectifier.  And DC/DC Converter provides

DC ±5V to each board.

In the front side, there are NOR LED, ALM

LED, Test Point where the installer can check

+5V output voltage and a switch controlling

–54V input voltage.

[Figure - PSU-D Power]

- Specification of Input Voltage

| Min. input voltage | Rated input voltage | Max. input voltage |
| --- | --- | --- |
| DC - 48 V | DC –54 V | DC –60V |

- Specification of Output Voltage / Currency

| Voltage | Max. currency | Output voltage limit |
| --- | --- | --- |
| + 5 V | 20 A | ±4% |
| - 5 V | 1 A | ±4% |

1.8.3	RGU

RGU provides the ring signal with the line boards from –54V power resource fed from the external rectifier. In the front side of RGU, there are OPR LED, ALM LED, STB LED showing the status of operation of duplicated controllers and a switch that controls the input –54V.

- Specification of Input Voltage

| Min. input voltage | Rated input voltage | Max. input voltage |
| --- | --- | --- |
| DC - 48 V | DC –54 V | DC –60V |

- Specification of Input Voltage / Currency

| Item | Values | Remark |
| --- | --- | --- |
| Voltage | 85 Vac ±5 Vac |  |
| Max. output | 8 W | Impedance : 800 Ω |
| Output voltage limit | ±5 Vac |  |
| Output frequency | 25 Hz ±1 Hz | Sine wave (Distortion rate: within 5%) |

- LED Display

| Item | Status | Remark |
| --- | --- | --- |
| OPR LED | Active | Green |
| STB LED | Stand-by | Yellow |

1.8.4	TB/NF (Terminal Block / Noise Filter)

TB/NF is an interface part attached on MMB or EMB that is used only when PSU-D is used. The basic type system uses TB/NF-BS that consist of TB, NF and circuit breaker and the expansion system uses TB/NF-ES.