OPERATION & MAINTENANCE

Data Management

1	System Initial Data Setup

1.1	System Installation Information

- Functional description

This screen displays the information on the installation environment of the system and is used to query and update the data for functions such as the separation of service, the charging, and the Calling Line Identification (CLI) by country.

[Sample Screen Description]

This screen displays the followings: If the system is installed in Seoul of South Korea and the first two digits of outgoing digits are “00”, the call is recognized as the international call. If only the first digit is “0”, the call is recognized as the toll call. The DDI is “450-XXX(X)”.

[Figure  System Install Information]

- How to use

1	Query

1)	Click Query.

2)	Verify the system installation information.

2	Change

1)	Click the item to be changed.

2)	Change the data.

3)	Click Change.

4)	Click Send.

5)	Click Query to verify the contents.

※ Note: √ Default Display: When activating the “Data management” in the OMS main menu, this function selects whether the system installation information is always displayed or not through the automatic query.

√ state: After selecting this, the “System installation information” is always displayed when activating the “Data management”.

√ state: After selecting this, the “System installation information” is not displayed when activating the “Data management”.

- Contents

| Country code | Country installing the system | Used for function use order |
| --- | --- | --- |
| Area code | DDD code for the area in which the system is installed | Used for CLI/charging |
| ISD Call Type Prefix | Digit Prefix for the international call identification | Used for CLI |
| DDD Call Type Prefix | Digit Prefix for the toll call identification |  |
| DDI Prefix For 1st Route | DDI prefix for the 1ST Route assigned from the remote station |  |
| DDI Prefix For 2nd Route | DDI prefix for the 2ND Route assigned from the remote station | Used for CLI |
| DDI Prefix For 3rd Route | DDI prefix for the 3RD Route assigned from the remote station | Used for CLI |
| DDI Prefix For 4th Route | DDI prefix for the 4TH Route assigned from the remote station | Used for CLI |
| DDI Prefix For 5th Route | DDI prefix for the 5TH Route assigned from the remote station | Used for CLI |
| DDI Prefix For 6th Route | DDI prefix for the 6TH Route assigned from the remote station | Used for CLI |
| DDI Prefix For 7th Route | DDI prefix for the 7TH Route assigned from the remote station | Used for CLI |
| DDI Prefix For 8th Route | DDI prefix for the 8TH Route assigned from the remote station | Used for CLI |
| Own PBX Code | Prefix for the local station used in the private network | Used for network interactions |
| Default ISD Vender Code |  | Not used |
| Country code | Country installing the system |  |

* Note Calling Line Identification (CLI): The function that sends the originator and terminator number from ISDN, QSIG, and R2MFC trunk to the remote station (same as ANI).

2	System Data management

2.1	System Numbering Plan

- Functional description

This screen is used to query and change the extension number/trunk call code/function code plan used in the system.

[Figure 2.1  System numbering plan]

- How to use

1	Query

1)	Click Query.

2)	Verify the numbering plan.

2	Add (Applied only to the extension number and trunk numbering plans)

1)	Click the data part of item to be added.

2)	Make a blank field by pressing the down arrow key on the last data displayed.

3)	Designate the data value to be added or changed.

※ Available numbers are ‘0~9’, X(Wild Card), ‘*’, and ‘#’.

4)	Click Send.

5)	Click Query, to verify the added data.

Example) To use 2000 through 2999 for the “Extension numbering plan”,

After designating the data as shown above, click Send and then, the data is added.

※ Note: When designating the extension numbering plan, it is impossible to designate the numbers that have the same first digit.

Example) If 2XX and 2XXX are designated and the Send button is clicked, the message ‘Invalid Numbering plan’ is displayed on the Error Message Display screen.

3	Change

1)	Select the data part of item to be changed.

2)	Designate the data value to be changed.

3)	Click Send.

4)	Click Query to verify the changed data.

Example1) To change 70 of “Trunk numbering plan” to 71,

1)	Convert to the edit mode by selecting the ‘70’ data with the mouse.

2)	After entering ‘71’, click Send.

3) Verify the changed data with Query button.
Example2) To change the “ATD Calling Code” from ‘0’ to ‘00’,

1)	Convert to the edit mode by selecting the ‘0’ data of “Code” field.

2)	After changing ‘0’ to ‘00’, click Send.

3)	Verify the changed data with Query button.

4	Delete

1)	Select the data part of item to be deleted with the mouse button.

2)	Delete the data after converting it into the edit mode.

3)	Click Send.

4)	To verify the deleted data, click Query.

- Contents description

1	Extension numbering plan

Extension Code: Designate numbering plan of extension terminal to be used

Example) Designate number of 4000 ~ 4999  4XXX

Designate number of 200 ~ 299  2XX

2	Trunk numbering plan

Trunk Code: Designate numbering plan of trunk code to be used

Example) To use ‘80~89’ number as the trunk,  After designating the ‘8X’ number, click Send.

Reference) This trunk code is related to the section 4.2 Trunk Access Code.

- Function code plan

| Item | Description | Default |
| --- | --- | --- |
| ATD Calling Code | Attendant calling code | 0 |
| System Speed Dial Code 1 | Designation of system common speed dial | 6XX |
| System Speed Dial Code 2 |  |  |
| Manager/Secretary Code | Manager-to-secretary calling code | *0 |
| UNA Pick-up Code | Code for answering the incoming trunk by using the laud bell | #0 |
| Call Back Cancellation Code | Call-Back cancellation code | *55 |
| DND Cancellation Code | DND cancellation code | *50 |
| DND Registration Code | DND Registration Code | *51 |
| Call Forwarding Cancellation | Call Forwarding Cancellation | *521 |
| Wake up Cancellation Code | Wake up Cancellation Code | #3 |
| Call Pick up Code | PICK-UP code | ** |
| Broker Call Code |  | *48 |
| Save and Repeat Reg Code | Save and repeat redial code | #9 |
| Conference Code | Conference code | #5 |
| Individual Call Park Reg | Code for individual call park of busy trunk or extension subscriber | #8 |
| System Call Park Reg Code | Code for system call park of busy trunk or extension subscriber | #7 |
| Manager to Manager Call Code | Manager to Manager Call Code | *1 |
| Consultation Transfer Code | Consultation Transfer Code | *53 |
| Call Pick up Other Grp Code | Code for call pickup of extension subscribers in other group | *2 |
| Intercept Service Code |  | *60 |
| Remote DOD Code | Remote DOD Code | *78 |
| Password Modification Code | Registration code of password for using the special functions such as daytime/night class change | *43 |
| Change Service Class to Night Code | Change Service Class to Night Code | *41 |
| Change Service Class to Day Code | Change Service Class to Day Code | *42 |
| Change Incoming Mode to Night Code | Change Incoming Mode to Night Code | *61 |
| Change Incoming Mode to Day Code | Change Incoming Mode to Day Code | *62 |
| O/G Call Allow Code | O/G Call Allow Code | *44 |
| O/G Call Deny Code | O/G Call Deny Code | *45 |
| Privacy Registration Code | Privacy Registration Code | *461 |
| Wakeup Registration Code | Wakeup Registration Code | *3 |
| Group Call Code | Currently not used. | *47 |
| Absence Information REG. Code | Absence message registration code | *8 |
| REG. Message Waiting | Code for registering the message to an extension subscriber who is in absence or rejection status | *71 |
| Message Waiting Answer | Message waiting answer code | *72 |
| Cancellation Message Waiting | Message waiting cancellation code | *700 |
| Camp on Transfer REG. | Code for registering the camp on to the busy extension subscriber | *76 |
| Individual Speed Dialing REG | Individual speed dial registration code | *# |
| Individual Speed Dial USE | Code used after verifying the individual speed dial | #6 |
| Call Forwarding (All Call REG.) | All call forwarding code | *521 |
| Call Forwarding (At Busy REG.) | Busy call forwarding code | *522 |
| Call Forwarding (No Answer REG.) | No answer call forwarding code | *523 |
| Call Forwarding (Remote REG.) | Remote call forwarding code | *52# |
| Last Number Redial | Code for redialing the last outgoing number | ## |
| SLT Log ON/OFF | Currently not used. | *462 |
| SLT Not Ready | Currently not used. | *463 |
| Verify Account Code | By registering 4-digit figure before the subscriber dials the trunk code, this function code makes the trunk code to be printed out with the printer | #4 |
| Background Music Code | Background Music Code | #2 |
| REG. Call Forwarding No Answer/Busy | Call forwarding code for no answer/busy status | *524 |
| Send Hook Flash Signal to Trunk | Redial code of a general extension subscriber | #* |
| Request Zone Paging Code | Internal zone paging code | *90 |
| Answer Zone Paging Code | This function code enables an extension subscriber in paging status to answer the subscriber who is in paging status. | *49 |
| Room Status | Code for verifying the current status of rooms and registering the new status | *56 |
| Door Phone Code | Currently not used. | *58 |
| Selective DND Code | This function code allows the subscriber to selectively register the DND function according to the trunk call and extension call. | *59 |
| VPC | Currently not used. | *73 |
| CLIP (Calling Line Identification Presentation) | This function code makes the calling party to inform the called party of the originator’s number when originating the initial call. | *95 |
| CNIP (Calling Name Identification Presentation) | This function code makes the calling party to inform the called party of the originator name when originating the initial call. | *96 |
| Ext Information Delete Code | Currently not used. | *741 |
| Reg. VPM All CFWD | Call forwarding registration code to all VPM | *527 |
| Reg. VPM Busy CFWD | Call forwarding registration code to busy VPM | *528 |
| Reg. VPM No Answer.CFWD | Call forwarding registration code to no-answer VPM | *529 |
| Reg. VPM Busy/No Answer.CFWD | Call forwarding registration code to busy/no-answer | *52* |
| D-Tel(LKD) Ring Type Change | LKD Ring Type change code | #1 |

2.2	System Local Board Information

- Functional description

This screen is used to query, add, change and delete the local board data in the system.

1.Main Screen screen

3.Board Type Typeeen

2.Popup Menu  Menue

[Figure 2.2.1  Local Board Information]

- How to use

If selecting the relevant menu, the status of current local board that is produced in the system (automatic data generation or already designated data) is inquired and displayed. The local board to be used in the system can be configured by the processes such as deletion, change and addition of desired local board in this status.

Initial installation

When initially installing the system (there should be no “CS1000.DT” in the system), the local board can be configured simply by using the following method.

By deleting the existing numbering plan and subscriber information, auto-designating the various parameter, option and timing as default by the country code that is designated in the [1.1 System installation information], and entering the new information on local board from the numbering plan, the system data can be configured newly.

The use order is as follows.

1)	Turn off the system power and insert each local board to be used.

2)	Turn on the system power.

3)	It is possible to view the automatic generation of each local board through the system CRT SIO.

4)	Connect to the system by starting the OMS.

5)	Activate the data management.

6)	Enter the required data in the [1.1 System installation information]. (The country code is requisite item)

7)	Click System Initialize in the [2.1 Local board information].

8)	Click Yes in the [System Initialize].

9)	Click Yes in the [System Data All Lost. Initialize Try].

10)	The [Numbering plan information] screen is displayed.

11)	Enter the numbering plan information required. (Reference: 2.1 System numbering plan)

After finishing the processes above, the local board inserted is automatically inquired and displayed. This status is that only the numbering plan and the local board are produced. Therefore, the port information of individual local board should be produced by the following “Port information production by local board designated“ process.

When adding, changing and deleting after querying the existing data

1	Query

1)	Click Query.

2)	Verify the insertion status of board configured.

2	Add

1)		If locating the cursor in the EMPTY of local board to be added and clicking it with the right mouse button, the ‘2. Popup menu’ screen is displayed.

2)	Select the board type to be installed with the mouse after selecting the Board Exchange.

3)	Click Send.

3	Delete

1)	If locating the cursor in the local board to be deleted and clicking it with the right mouse button, the ‘2. Popup menu’ screen is displayed.

2)	Select the EMPTY with the mouse after selecting the Board Exchange.

3)	Click Send.

4	Change

1)	If locating the cursor in the local board part to be changed and clicking it with the right mouse button, the ‘2. Popup menu’ screen is displayed.

2)	Select the board type to be installed with the mouse after selecting the Board Exchange.

3)	Click Send.

Port information production by local board designated

1)	Click the mouse on the board desired to produce the data.

2)	Select the Board Data Generation.

3)	The screen configured for each board type is displayed.

- Extension/subscriber board

[Figure  Extension Data Handling Pian Dialog Window]

The screen above is the “Extension Data Handling Plan Dialog Window” that is displayed when the ASLC is produced in the Shelf 1 / LB 4. Select the data for each item, check the items to be √ in the characteristic part and then, click Verify. This screen produces the integrated data. For the setup items displayed, refer to 3.1 Extension information item.

- Trunk/dedicated line board

[Figure 3  Trunk Data Handling Plan Dialog Window]

The screen above is the “Trunk Data Handling Plan” that is displayed when the APTC is produced in the Shelf 1 / LB 1. Select the data for each item, check the items to be √ in the characteristic part and then, click Verify. This screen produces the integrated data. For the setup items displayed, refer to [4.1 Trunk information] item.

- Click Verify in [Figure 3]

The [3.1 Extension Information] and [4.1 Trunk Information] screens are displayed with added data type (yellow) as shown in the figure below. In the figure below, if moving the port information, which is marked with ““ on the left part of the upper screen, into the new port by using the arrow or mouse, the related information is automatically displayed in the extension characteristic information and Trunk Line Option part on the lower screen. That is, to configure the differential data for each port, move the port into relevant one and then, click the relevant item.

Note 1: To simultaneously change the specific field information (upper end) of all ports on the current screen, click the right mouse button after locating the cursor in the desired field and then, use the Tools -> Raw Data Change.

Note 2: To simultaneously change the extension characteristic information and Trunk Line Option of all ports on the current screen, use the Characteristic Change Toll and Trunk Line Option Toll.

[Figure  Trunk Information]

2)	If clicking Send, the following small dialog box is displayed. Until this screen disappears, do not perform any tasks. If the data production is normally sent, the data screen above is changed from yellow to gray.

3)	Click Query to verify that the data is correctly created.

Description on other mouse popup menu

- Port data view

If selecting this menu on the ‘2. Popup menu’ screen of [Figure 2.2.1], the data screed already stored for the selected board is displayed.

Example) If selecting this menu in the ASLC board, the data stored is displayed on the 3.1 Extension information screen and if selecting the DNIC (DTC), the 4.1 Trunk information screen is displayed.

- ISDN PRI mode view

This menu is activated (available state) only for the DNIC PRI board and selects whether using the User mode or Network mode.

[Figure  ISDN PRI Mode]

- How to use

1	Query

1)	Click Query.

2)	Verify the location and mode of board.

2	Change

1)	Click Query.

2)	Verify the location and mode of board.

3)	Click twice the mode of item to be changed.

4)	Change it into the desired mode.

5)	Click Change.

6)	Click Send.

7)	Click Query to verify the changed data.

※ Mode User Mode	: Used for the PRI line of a telephone office.

Network Mode     : Used if the system is designated as the upper system when connecting the dedicated line. Used for PRI of User mode and loop test.

- Trunk DIL assign view

If selecting this menu, 6.2 DIL incoming group screen is displayed.

※ Only for the trunk board (Ex, APTC, ALTC, DNIC, BRI), this menu is activated (available state). For more information, refer to 6.2 item.

- Voice coding type

This menu is only activated (available state) for the DNIC DTC, PRI board and enables selection of mode either the A-Law or -Law mode.

- How to use

1	Query

1)	Click Query.

2)	Verify the location and coding type of board.

2	Change

1)	Click Query.

2)	Verify the location and coding type of board.

3)	Click twice the coding type of item to be changed.

4)	Change it into the desired Type.

5)	Click Change.

6)	Click Send.

- Click Query to verify the changed data.

3	CRC enable / disable

This menu is only activated (available state) for the DNIC DTC, PRI board and selects whether the relevant local board enables or disables the CRC.

- How to use

1	Query

1)	Click Query.

2)	Verify the location and Enable / Disable states of board.

2	Change

1)	Click Query.

2)	Verify the location and Enable / Disable states of board.

3)	Click twice the Enable / Disable states of item to be changed.

4)	Change it into the desired state.

5)	Click Change.

6)	Click Send.

7)	Click Query to verify the changed data.

2.3	System Lock Parameter

- Functional description

This screen is used to verify and change the system Lock Time.

[Figure  System Lock Parameter]

- How to use

1	Query

1)	Click Query, and the system lock parameter is queried.

2	Change

1)	Select the Value of the item to be changed and click it (or direction key).

2)	Change the Lock Time.

3)	Click Change.

※ To restore the previous data, click Undo.

4)	Click Send

5)	Click Query to verify the changed data.

- System lock parameter

| Item | Default | Description |
| --- | --- | --- |
| R2MF Assign Wait Lock | 10 | The waiting time for the result of R2MFC port seizure request |
| CO to CO Conversation Lock | 600 | The allowed all time for the call between trunks with no call ending signal |
| Camp-On Alert Lock | 49 | The interval of the periodic alert alarm for the subscribers of Camp-on registration |
| Announcement Line Service Lock | 200 | In case of announcement Line incoming service at the time of trunk incoming, the service duration time |
| DECT Conversation Time Display Lock | 600 | The interval of conversation time displayed on DECT when DECT subscriber originates to the trunk. |
| DECT Setup Ack. Wait Lock | 600 | The maximum waiting time for Ack after the setup request to local at the time of DECT incoming |
| SLT Pulse Digit Spec | 1 | 10pps when SLT is used in DP mode |
| SLT On-Hook Detect/Release Time | 4 | On-hook detection time for SLT subscriber (Minimum time) |
| SLT Inter Digit Detect Time | 2 | The time required to distinguish digits when SLT is used in DP mode |
| SLT Digit Make Time | 5 | Pulse Make duration time when SLT is used in DP mode |
| SLT Digit Break Time | 0 | Pulse Break duration time when SLT is used in DP mode |
| SLT Off-Hook Detect/Release Time | 2 | Off-hook detection time for SLT subscriber (Minimum time) |
| DP Dial Lock | 600 | The time for SLT subscriber who uses DP to wait for the decision of the called person after the first digit |
| DT Dial Lock | 300 | The time for SLT subscriber who uses DTMF to wait for the decision of the called person after the first digit |
| Single Digit Lock | 30 | The time for waiting from the first digit and to the second digit in order to distinguish the room speed number of the room subscriber from general outgoing |
| Data Call Guard Lock | 20 | Currently not used. |
| Message Wait off Tone | 200 | Ring off time at the time of sending periodical message wait ring for Message Wait registration subscriber |
| DECT Conversation Delay Lock | 5 | Traffic channel connection delay time to remove the noise that occurs when DECT subscriber answers |
| Next Message Wait Lock | 30 | The maximum time for waiting between messages at the time of receiving Multi PB Message |
| Setup Retry Lock | 5 | The waiting time for retry setup in case QSIG fails to send the first setup |
| VPM Mail Box Check Result Wait Lock | 50 | The maximum time for waiting for the response to the request for VPM to check the mailbox |
| Trk Reseizure Lock in Emergency Call | 20 | The waiting time for re-seizure after the forced release of the seized trunk for outgoing emergency call |
| VPM Announcement Delay Lock | 10 | The response delay time to remove the phenomenon that VPM announcement is cut after the trunk incoming to VPM |
| ANCR Interval Lock | 1200 | The interval of re-outgoing when using ISDN automatic re-outgoing function |
| Second Hook-flash wait Lock | 10 | The maximum authorization time between HFs when the second HF in the second conversation causes the change into the conference call. |
| SETUP Wait after Channel Assign | 20 | The maximum waiting time for the actual call setup after receiving the channel request in ITGC, VPM |

2.4	System Tone Parameter

- Functional description

This screen is used to verify and change all the tone value and tone port used in the system.

[Figure  System Tone Parameter]

- How to use

1	Query

1)	Click Query and the Tone Parameter is queried.

2	Change

1)	Select the item to be changed and click it.

2)	Change the data of Value and Port No.

3)	Click Change.

※ When there are several data to be changed, repeat the above procedure.

※ To restore the previous data, click Undo.

4)	Click Send

- Click Query to verify the change.

- System tone parameter

| Item | Description |
| --- | --- |
| ACR RBT |  |
| Recall RBT | RBT heard in case of recall |
| Blocking Service Tone | Tone heard when calling a blocked subscriber |
| DND Service Tone | Tone heard when calling DND registration subscriber |
| Uncompleted Dialing Tone |  |
| Invalid Dialing Tone | Tone heard when dialing the number not in the numbering plan |
| Out of order Tone |  |
| Absence Service Tone | Tone heard when calling absence registration subscriber |
| Line Lock Out Tone | Tone allocated to the subscribers whose receiver is left alone |
| Vacant Tone | Tone heard when dialing a missing number |
| Break-in Tone | Break-in tone heard for busy override |
| Warning Tone | Service registration/cancellation warning tone |
| Wake-up Service Tone | Tone heard for Wake-up service |
| Camp on Alert Tone | Tone that alerts the Camp-on registration |
| Conference Alert Tone | Tone that alerts the conference call periodically |
| EXT Split & Holding Tone | Tone the extension/trunk subscriber hears in waiting or holding. |
| TRK Split & Holding Tone |  |
| TRK Camp-On Holding Tone |  |
| EXT Camp-On Holding Tone |  |
| Trunk Call Wait Holding Tone |  |
| EXT Call Wait Holding Tone |  |
| Trunk Station Holding Tone |  |
| EXT Station Holding Tone |  |
| Trunk Call Park Holding Tone |  |
| EXT Paging Holding Tone |  |
| Trunk Paging Holding Tone |  |
| EXT Paging Holding Tone |  |
| DOD Conversation Restriction Tone |  |
| DISA Dial Tone | Dial tone that outside subscribers hear for DISA function |
| Trunk C.G.T | CGT that the trunk subscribers hear |
| DISA C.G.T | CGT that outside subscribers hear for DISA function |
| Conference Break-in High Tone |  |
| Conference Break-in Low Tone |  |
| Warm Line Dial Tone | Dial tone that the Warm Line assigned subscriber hears |
| D-Tel. Admin Error Tone | Currently not used |
| AIS-54 Tone | Currently not used |
| CO to CO Long Conv. Warning Tone | In case of calls between trunks with no call ending signal, when the allowed call time expiration warning tone is heard, if the subscriber presses “*” during the tone with DTMF, additional call is allowed. The waiting time for DTMF Tone. |
| LKD Series Keyset Ring 0 Tone | LKD series Ring type 1 |
| LCR Virtual Dial Tone | Virtual dial tone when the subscribers hear using LCR |
| C.G.T Tone | CGT that internal subscribers hears |
| Auto Holding Tone |  |
| ACD Agent Ring Tone | Tone that alerts the incoming call to ACD agent |
| ACD Calling Pause Tone | Tone heard during the call delay at the time of calling ACD Split |
| LKD Series Keyset Ring 1 Tone | LKD series Ring type 2 |
| LKD Series Keyset Ring 2 Tone | LKD series Ring type 3 |
| ACD Service Tone 1~20 | The waiting announcement used for ACD service |

2.5	System Ring Parameter

- Functional description

This screen is used to verify and assign all ring time used in the system.

[Figure  System Ring Parameter]

- How to use

1	Query

1)	Click the Query, and the Ring Parameter is queried.

2	Change

1)	Select the item to be changed and click it.

2)	Enter the each Ring Lock Time.

※ (For Lock item, the time unit is sec and for other items, the time unit is msec)

3)	Click Change

※ When there are several data to be changed, repeat the above procedure.

※ To restore the previous data, click Undo.

4)	Click Send

5)	Click Query to verify the change.

Reference 1)The above ring parameter is for the general phone, D-Tel (LGP series, LKD series).

Reference 2)D-Tel of LGP series operates as other terminals do but it does not operate for 4th ON/OFF.

Reference 3)The 2nd and 3rd ON/OFF time of D-Tel of LGP series is minimum 500 msec.

Reference 4)The ring types of LKD series can be changed in [2.4 System Tone parameter].

- System ring parameter

| Item | Description |
| --- | --- |
| Extension Ring | The incoming ring when the extension subscribers in the system calls an extension subscriber |
| Trunk Ring Tie Ring | The incoming ring to the extension subscriber in case of trunk incoming |
| Tie Ring | The incoming ring to the extension subscriber in case of dedicated line incoming |
| RD Ring | Ring used for dedicated line (R/D) outgoing |
| Recall Ring | Ring used for the recall to the original subscriber when the transferred subscriber gives no answer for call transfer. |
| Call Back Ring | The ring used to call the extension subscriber when the other subscriber hangs up in case an extension subscriber uses Call-Back. |
| Wake up Ring | The ring at the turnout in case of the timed reminder registration |
| Revertible Ring | The ring when an extension subscriber calls himself and releases. |
| Call Forward Ring | Ring to B subscriber in case of a call forwarding of an extension A subscriber call to B subscriber |
| Conference Ring | Ring used to call an extension subscriber to add to a conference call. |
| Call Transfer Ring | Ring to B subscriber in case of a call transfer of an extension A subscriber call to B subscriber |
| Camp-On Ring | Ring that rings when a camp on subscriber hooks on the receiver |
| Re-Ring | Ring used to call the same extension subscriber who gives no answer |
| Message Wait Ring | Ring used to report the message wait registration periodically for the extension subscriber with Message Wait registered. |
| Laud Bell Ring | Ring used to request to switch on the buzzer to the port assigned as Laud Bel |
| Park Alarm Ring | Currently not used. |
| Internal Zone Paging Ring | Ring that reports the internal zone paging starts |
| Hand Free & Privacy Ring | Warning alert ring when using the hands-free answer back function in case of extension incoming with keyphone. |
| Message Wait Request Ring | The first ring sent to the extension subscriber who are registered for the Message Wait registration |
| Multi Ring | Currently not used. |
| No Answer Ring | When an extension subscriber uses NO Answer Call-Forward, the call forwarding service is provided after checking this ring time. |
| QSIG CC Recall Ring | Call back ring by QSIG CC (supplementary service) |
| Recover Ring | Ring used to call again an extension subscriber, when he/she releases after an abnormal call transfer |

2.6	System Option

- Functional description

This screen is used to verify and change the various functional options used in the system.

[Figure  System Feature Option Information]

- How to use

1	Query

1)	Click Query, and the system functional options are queried.

2	Change

1)	Click the ▼part of the list to be changed.

2)	Change the data in the list into the data that you want.

3)	Change the values of each functional option and click Change

4)	Click Send.

5)	Click Query to verify the change.

- System functional options

| Item | Description |
| --- | --- |
| Automatic camp on during conversation 1.Yes 2.No | Assign the automatic camp one when a busy subscriber is called by other extension 1.Camp on the busy extension. 2.Release the busy extension. |
| Handling when the transceiver is neglected 1.Howling Tone 2.ATD 3.SMDA 4.ATD & SMDA | Handling when a user neglects the transceiver 1.Send only the howing tone to the user. 2.Send the howing tone and terminates to the attendant. 3.Send the howing tone and send the content to the charging equipment. 4.Send the howing tone and send the content to the printer and the charging equipment. |
| Hook Flash 1.Discard H.F 2.On Hook 3.Process H.F | Management of hook flash of dial pulse phone 1.Discard H/F. 2.Detect as On Hook. 3.Detect as H/F. |
| On-Hook after busy extension H.P 1.Release 2.Recall EXT | Management when an extension subscriber hooks flash for call transfer and leaves the transceiver on-hook by mistake 1.Release the traffic channel. 2.Recall the disconnected extension. |
| Warning tone for Attendant camp on 1.Yes 2.No | Assign whether to give the warning tone for the attendant camp on for an extension subscriber. 1.Gives the warning tone to the applied subscriber and camps on. 2.Gives no warning tone to the applied subscriber and camps on. |
| Warning tone for extension camp on 1.Yes 2.No | Assign whether to give the warning tone for the camp on applied for subscriber. 1.Gives the warning tone to the applied subscriber and camps on. 2.Gives no warning tone to the applied subscriber and camps on. |
| DP phone hook flash digit type (0 ~ 9) | Assign the digits used for hook flash in dial pulse phone. |
| Holding type for one click of holding button 1.EXC-HOLD 2.SYS-HOLD | Assign the holing type when using the holding button in D-Tel 1.Only the holder can re-access the held trunk. 2.Other multi-functional phones can access to the held trunk. |
| The number of callback recall (1 ~ 9) | Assign the number of recall tries for the call ending in case of no answer. |
| Save the last message 1.Yes 2.No | Assign whether to save the message used last in the system. |
| Automatic system class change 1.Yes 2.No | Assign whether to change the class between day and night when the day/night incoming mode is change. 1.Change the day/night class when the incoming mode day/night is changed. 2.Not change the day/night class when the incoming mode day/night is changed. |
| The entire TNT group mode change 1.Yes 2.No | Assign the entire tenant incoming mode change when the tenant mode change subscriber changes the day/night incoming mode. 1.Change all tenant incoming modes into DAY Mode or NIGHT Mode. 2.Change the tenant incoming mode where the terminal belongs into DAY Mode or NIGHT Mode. |
| DID Allow check 1.Yes 2.No | Currently not used. |
| Item | Description |
| The busy extension seizes other trunk 1.Yes 2.No | Assign whether an extension conversing with a trunk can access other trunk 1.Can access other trunk. 2.Can not access other trunk. |
| Other key phone seizes Conference call/ Two way calling 1.No 2.Intercept 3.Override | When there is an extension conversing with a trunk, other D-Tel seizes the flexible button for which the trunk port is assigned and performs Intercept or Override 1.When not using this function, 2.When using Intercept, 3.When using Override, |
| Conversation listening when the busy override is impossible 1.Yes 2.No | Assign whether the conversation listening is possible. 1.Possible 2.Impossible |
| Conference call periodical tone transmission 1.Yes 2.No | Whether to send the break in tone when trying a conference call 1.The break in tone is sent 2.The break in tone is not sent. |
| Trunk outgoing method (Circular)? 1.Yes 2.No | Assign whether to seize the first port of the trunk group or any port by turns for the trunk outgoing. 1.Seize the trunk port by turns unconditionally for trunk outgoing. 2.Seize the first idle trunk port for trunk outgoing. |
| Display ANI information to CRT/Print? 1.No 2.CRT | Verify and assign whether to display ANI information to CRT or PRT. 1.Not display ANI information to CRT. 2.Display ANI information to CRT. |
| Step Call? 1.Yes 2.No | Assign whether to use the step call that calls a busy extension number by dialing the last digit. 1.Use Step Call. 2.Not use Step Call. |
| Whether to back up the functional data? 1.Yes 2.No | Currently not used. |
| Break-In Tone In Ext ? 1.Yes 2.No | Assign whether to send the break-in tone for the busy override of an extension subscriber. 1.Send the busy override. 2.Not send the busy override |
| Break-In Tone In ATD ? 1.Yes 2.No | Assign whether to send the break-in tone for the busy override of attendant. 1.Send the busy override. 2.Not send the busy override |
| Start Method ? 1.Cold Start 2.Warm Start 1 3.Warm Start 2 | Assign the system start mode. - This mode meams physical reset . using this mode the system restart physically. So, this mode doesn’t assure all call connection status. - This mode uses 2 type restart mode. whern the system is restarted for the first time, the system is operated on warm mode, but from the 2nd restart the system operated on cold start mode. when the 1st warm start mode, the system use another CPM board, so all call doesn’t disconnected. - This mode use always warm restart. In other say, whenever user reset the system, the system restart on warm start mode using another CPM board. so this mode also assure all call connection status. |

3	Extension Data Management

3.1	Extension Information

- Functional description

This screen is used to query, change, add, and delete the extension subscriber information (Extension types, day/night class, incoming/outgoing tenant, extension blocking, and extension name)

[Figure 3-1  Extension Information]

- How to use

1	Query

1)	Enter the range of Shelf / LB / Port or the extension number for the query conditions.

-	Enter both the start and the end of Shelf/LB/Port.

-	Enter only one extension number to be queried.

2)	Click Query.

3)	Verify all extension subscriber information.

2	Add

1)	Use the down direction key ( ↓) in the last data part to add a data part.

2)	Enter the physical address, the extension number and other information of the extension subscriber to be added in the added data part.

※ Mandatory Item; Port No, ExtNum, Ext Type, Day Class, Nigh Class, I/C Tenant, O/G Tenant

3)	Click Add.

4)	Click Send.

5)	Click Query.

3	Change

1)	Click the data part of the item to be changed with mouse.

2)	Change the data to be changed.

(Some data is not allowed for change.)

3)	Click Change.

4)	Click Send.

5)	Click Query.

4	Delete

1)	Click the data part of the item to be deleted with mouse.

2)	Click Delete.

3)	Click Send.

4)	Click Query.

Contents

| Item | Contents |  |
| --- | --- | --- |
| Port No | Physical location of the subscriber (Shelf / LB / Port). |  |
| Ext Num | Extension number |  |
| Ext Type | SLT | General phone |
|  | DTEL-LGP | LGP series D-Tel |
|  | DTEL-LKD | LKD series D-Tel |
|  | DLS-DTEL | DDS of LGP/LKD series |
|  | ACD-SLT | General phone designated according to ACD guide |
|  | ACD-DTEL | LGP series D-Tel designated according to ACD guide |
|  | LOUD BELL | General phone designated as common bell subscriber |
|  | BRIS | ISDN BRI phone subscriber |
|  | DECT | DECT subscriber |
|  | VPM | Subscriber allocated to VPM board |
|  | ATD | Attendant subscriber |
| Day Class | Designates the day class of each subscriber (The maximum class: I). |  |
| Night lass | Designates the night class of each subscriber (The maximum class: I). |  |
| I/C Tenant | Incoming tenant number (1 ~ 8) (See: Designated in 4.5 Trunk incoming tenant allocation) |  |
| OG Tenant | Outgoing tenant number (1 ~ 8) (See: Designated in 4.6 Trunk outgoing tenant allocation) |  |
| Man G | Manager group number (See: Designated in 6.1 Manager group) |  |
| Sec G | Secretary group number (See: Designated in 6.1 Manager group) |  |
| IND G | Individual trunk group (See: Designated in 6.3 Individual outgoing group.) |  |
| Speed G | Individual speed dial Index number |  |
| Call FWD Type | Types of registered call forwarding |  |
| TelNo/Trk Access Code | Registered call forwarding extension number or external number |  |
| BLK | Subscriber outgoing blocking information |  |
| Name | Subscriber name (14 digits in English) |  |
| Password | Subscriber password (4 digits) |  |

[Description for the addition or change of the extension characteristic information or feature restriction option]

The extension characteristic information or feature restriction option window in [Figure3-1] shows the current status of the extension characteristic information or the feature restriction option of the item with ▶ mark in the left.

The change in this window is the change or the addition of the extension characteristic and the extension information of the extension subscriber whom the ▶ mark indicates. (When the mouse moves to other extension subscriber, the ▶ mark moves with the mouse.)

To change and select the entire Port No. or several Port No. at a time without moving the cursor or the mouse, select Ext Character Change button in [Figure 3-1].

[Ext Character Change / Ext Feature Change Tool window description]

The following screen is displayed when Ext Characteristic Change is selected.

The following screen is displayed when Ext Feature Change Tool is selected.

- Screen description

1)	Mark " √" on the necessary information.

2)	If  is selected, the overall Port No. in [Figure 3-1] are changed and selected. (They are highlighted in blue and it has the effect of clicking the change button at the time of selection.)

3)	If  is selected, the current Port No in [Figure 3-1] is changed and selected. Then, the ▶ mark is passed to the next port. Whenever clicking the button, the selection is added. (They are highlighted in blue and it has the effect of pressing the change button at the time of selection.)

4)	If  is selected, the current port No. in [Figure 3-1] is not selected and the ▶ mark is passed to the next port No..

5)	 is used to close the window.

※ The following describes the screen that is displayed when the data part in [Figure 3-1], extension information screen is selected with the mouse Right Button.

Screen displayed when Edit is selected.

- Always Editing: This makes the data item in the screen always editing mode.

- Always Editing Cancel: This cancels the above Always Editing.

Screen displayed when Tools is selected.

- Screen displayed when Row Data Change is selected.

- All data items except Port No can be changed in this screen. To make the data of all Port No one, click All OK and to change the data step by step, click Next OK

- Use order

1)	Click the [Figure3-1] data part to be changed.

(The Ext Type part in the above figure is changed into the selected data item.)

2)	Change the data in the Edit box in this screen.

3)	Click All OK or Next OK.

- Screen displayed when Ext Character Change is selected.

Refer to “How to use Ext Character Change, Ext Feature Change Tool screen”.

- Screen displayed when Ext Feature Change Tool is selected.

Refer to “How to use Ext Character Change, Ext Feature Change Tool”.

Screen displayed when DTEL Feature Inf. View is selected.

- Functional description

This screen is used to verify and change the characteristics and DSS connection information of the multi-functional telephone.

[Figure 3-2  D-Tel Feature]

- How to use

1	Query

1)	Click Query.

2)	All information on the multi-functional telephone is queried.

2	Change

1)	Change the characteristics and DSS connection information of the multi-functional telephone.

2)	Click Change.

3)	Click Send.

4)	Click Query.

- How to set DSS to the multi-functional telephone

1)	Move  mark to the relevant port in the extension information window in [Figure 3-1] and click Change. (Highlighted in blue)

2)	First, delete the extension number (5533) of the relevant Port-No.

3)	Change the EXT-TYPE to DLS-Dtel.

4)	Click Send.

5)	Move the  mark from the digital key phone information window in [Figure 3-2] to the multi-functional phone number (port) to connect DSS to, and

designate Shelf / LB /Prt which is changed into DLS-Dtel as shown above.

- Content

| Item | Contents |
| --- | --- |
| Auto Hold | YES : Allow other incoming call for a busy D-Tel NO : Restrict other incoming call for a busy D-Tel. |
| Button Mode | RLS : Cut the currently talking subscriber and try other call. HOLD : Hold the currently talking subscriber and connect to other call. |
| Ring Type | Ring 0 : Incoming call ring type 0 of LKD D-Tel Ring 1 : Incoming call ring type 1 of LKD D-Tel Ring 2 : Incoming call ring type 2 of LKD D-Tel |
| DLS Port No | One D-Tel can be connected to maximum four DDS physical addresses (Shelf / LB / No) and DLS should be assigned to the same LB where all D-Tels are located. |

Screen displayed when DTEL Flexible Button View is selected.

- Functional description

This screen is used to verify and change the information of the flexible button of the multi-functional telephone.

Whether the relevant button is allowed to use

[Figure  D_Tel Flexible Button]

- How to use

1	Query

1)	Enter the query conditions (Shelf / LB / Port).

Enter the extension number or the physical address of the extension subscriber and both can not be entered at the same time.

2)	Click Query.

3)	Query the information on the flexible button of the multi-functional phone subscriber.

2	Change

1)	Select the data part of the item to be changed by clicking it and change the button type, button data, and O/G Allow.

2)	Click Change.

3)	Click Send.

4)	Click Query.

3	Change (e.g.)

Order for the assignment of the first button of the multi-functional phone (5531) as the extension (2000) calling button:

1)	Click the button type of the extension 5531, and the list is displayed.

2)	Select the extension button.

3)	Assign the extension number 2000 to the button data.

4)	Click Change.

5)	Click Send.

6)	Click Query.

- Contents

| Item | Contents |
| --- | --- |
| Button ID | D-Tel button No.(1 ~ 44) |
| Button Type | Reference 1 |
| Button Data | Reference 1 |
| O/G Allow | Whether the relevant button is allowed to use. |

Note 1 Button type and data

| Button type | Description | Button data sample |
| --- | --- | --- |
| Not Defined | Unused button |  |
| Trunk Button | Trunk Port No button | Trunk Port No : 1/01/01 |
| Extension Button | Extension button | Extension phone number: 2000 |
| Feature Button | Feature button | Feature code: Reference 2 |
| Trunk Group Button | Trunk code and external phone number | Trunk calling code + external phone number: 9 + 01912345678 |
| Pool / Loop Button | Holding button |  |
| Digit Button | Button speed button | When the external incoming call registration code is “*521”, *521 + 01923452345 |

Note 2 Feature button data

| Feature button data | Contents |
| --- | --- |
| 01 | Group Hunting |
| 02 | Call Pick up |
| 05 | Busy Override |
| 08 | Password Change |
| 09 | Absence Status Registration and Cancellation |
| 10 | Call Reservation |
| 11 | Timed Reminder Registration |
| 12 | Incoming Call Mode Change: System day/night mode change |
| 13 | Response Waiting |
| 14 | Other Group Call Pick up |

- Screen displayed when Telno Information View is selected.

The screen on the information of the entire extension subscriber number in the system.

[Figure  Tel No Information]

- How to use

1	Query

1)	Click Query.

※ Enter the first digit of the extension number to be queried in Tel Start No. Without entering, all extension numbers are queried.

- Contents

| Item | Contents |
| --- | --- |
| TEL NO | Extension number set |
| TYPE | Types of the extension terminal |
| DATA | The location of the board that the extension terminal is connected to. |

Screen displayed when DSS Flexible Button View is selected.

- Functional description

- How to use

1	Query

2	Contents

3.2	Extension Tenant Assign

- Functional description

This screen is used to verify and change the extension incoming/outgoing tenant groups.

[Figure  Extension Tenant Assign]

- How to use

1	Query

1)	Query conditions: Select the tenant number. (Max 8)

2)	Click Query.

2	Add

1)	Press the down direction arrow key ( ↓) in the last data part, and the row to be added is displayed.

2)	Click the data part of the mode to be added with the mouse, and the list is displayed.

Select the desired mode.

3)	Add the desired extension number.

4)	Click Add.

5)	Click Send.

6)	Click Query.

3	Change

1)	Click the data part of the mode to be changed with the mouse, and the list is displayed.

Change the value into the desired mode.

Change the value into the desired extension number.

2)	Click Change

3)	Click Send.

4)	Click Query.

4	Delete

1)	Select the data part to be deleted by clicking it.

2)	Click Delete.

3)	Click Send.

- Click Query.

- Contents

| Item | Contents |
| --- | --- |
| Mode | Select the mode of the extension number.( I/C, O/G, I/C&O/G) I/C : Used as incoming tenant. O/G : Used as outgoing tenant. I/C&O/G : Used as both the incoming and outgoing tenants. |
| Extension No | Enter the extension number. |

3.3	Class Base Outgoing Allow/Deny

- Functional description

This screen is used to verify, add, and delete the codes to be allowed or to be denied based on the class.

[Figure  O/G Digit Allow/De ny Table]

- How to use

1	Query

1)	Select √ in Deny or Allow Table and click Query.

( In the table where √ is not selected, query or addition is not available.)

2)	The information of Deny or Allow Table is displayed.

2	Add

1)	Check √ in Deny or Allow Table with the mouse.

2)	Click the last part of the data to be added with the mouse.

3)	Press the down direction arrow key ( ↓), and Add Table is displayed.

4)	Click the data part of class twice and select the desired class. ( I, S, A, B, C, D, E, F, BLK)

If clicking the Deny or Allow code Table to be added, the cursor will be displayed.

5)	Change it into the Deny/Allow Code.

6)	Click Add.

7)	Click Send.

8)	Click Query.

- Contents

| Item | Contents | Remarks |
| --- | --- | --- |
| Deny Table | The digit table whose outgoing is denied based on the class | Maximum 100 for each class |
| Allow Table | The digit table whose outgoing is allowed based on the class. | Maximum 100 for each class |
| Class | The class that denies the long distance calls. | Use only I, S, A, B, C, D. |
| Deny Code | Digits to be denied based on the class | Maximum 6 digits |
| Allow Code | Digits to be allowed based on the class. | Maximum 8 digits |

- Example

- If the denied digit of the Deny class B is ‘0’ and the Allow class B is ‘019’,

If the extension subscriber B dials ‘019’ after the trunk occupancy, the outgoing is allowed.

If the subscriber dials “02”, the trunk is restored automatically.

- To deny “700” service for class B subscribers, enter “700” for the deny code of Deny class B.

| Deny Table |  | Allow Table |  |
| --- | --- | --- | --- |
| Class | Deny Code | Class | Allow Code |
| B | 0 | B | 011 |
| B | 700 | B | 018 |
|  |  | B | 019 |

4	Trunk Data Management

4.1	Trunk Information

- Functional description

The screen is used to query, change, add, and delete the trunk port data.

[Figure 4-1  Trunk Information]

- How to use

1	Query

1)	Enter the range of Shelf / LB / Port in the query conditions.

  - Both the start and the end of Shelf/LB/Port should be keyed in.

  - To query the entire information of a local board, click  and move to 2.2 System local board.  Then place the cursor on the desired local board and select Port Data View by clicking the right button of the mouse, and the query conditions are automatically keyed in and the query is performed and the screen is displayed

2)	Click Query.

3)	The relevant trunk information is queried.

The Trunk Line Option on the bottom of the screen is the information on port with ““ mark in the far left on the bottom of screen. If the ““ mark is moved with the arrow key or the mouse, automatically the relevant information is displayed on the screen.

2	Add

1)	Add a data line with down arrow key ( ↓).

2)	Enter the Port No and other data to be added to this record.

Example)

3)	Click Add.

4)	Click Send.

5)	Click Query.

3	Change

1)	Click the data to be update with the mouse. (Or move with direction keys.)

2)	Change the data into the intended value.

3)	Click Change.

4)	Click Send.

5)	Click Query.

Trunk Line Option in the bottom of the screen is information on the port with ““ mark in the far left in the top of the screen. If the ““ mark is moved with the arrow key or the mouse, automatically the relevant information is displayed on the screen. To produce or update the different data of each port, move the mark to the relevant port and click the related items.

Note 1 To update specific field information (top) of all ports on the current screen, place the cursor on the relevant field and click the right button of the mouse and use Tools -> Raw Data Change.

Note 2 To update both the extension characteristic data and trunk line option simultaneously, use Characteristic Change Toll and Trunk Line Option Toll.

4	Delete

1)	Click the data to be deleted with the mouse. (Or move with the direction keys.)

2)	Click Delete.

Example)

3)	Click Send.

4)	Click Query.

Note 3 For Change (H)/Delete (D), select a series of plural port data and change or delete the data at a time as given below.

①Place the cursor the first port data (the last) to be changed (deleted).

②Select the data to be change or deleted by pressing ‘Shift + up*down direction keys ( ↑↓)’.

③To exclude the data in the middle, press only the direction keys without pressing ‘Shift’ Key while placing the cursor on the data.

④When all data are selected, click Change (Delete).

Note 4 The Trunk Line Option Change menu in the bottom of the screen is used to change the Trunk Line Option information of all port data displayed in the screen. The method is as follows.

Port Number where the cursor is located on.

1)	Mark" √" on the information that you want.

2)	If  is selected, all the port No. in the window screen of [Figure 2-1] are selected. (It has the effect of clicking Change upon the selection.)

3)	If  is selected, the port No in the window screen of [Figure 2-1] is selected for change by options. (It has the effect of clicking Change at the same time of the selection.)

4)	If  is selected, the current Port No is not selected and a series of following port No are displayed.

5)	is used to close the window.

- Functional description of the mouse right button in the trunk information screen

[4.1 Trunk Information] Click the right button of the mouse on the data part in the screen, and the following screen is displayed.

1	Screen displayed when Edit… is selected.

-	Always Editing	: Make the data part in the screen always in the editing mode.

-	Always Editing Cancel	: Cancel the above Always Editing.

2.	Screen displayed when Tools is selected.

2.1	Row data change

All data items can be changed in this screen. To make all the data of Port No one, Click All OK and to change the data by stages, Click Next Ok.

Use order

1)	Click the [Figure 4-1] data part to be changed with mouse.

(The Ext Type part in the above figure is changed in to the selected data item.)

2)	Change the data in the Edit box on this screen.

3)	Click All OK or Next OK.

2.2	Trunk Line Option Change

This item is displayed in Trunk Line Option Change on 4.1 Trunk Information screen.

3.	Screen displayed when I/C Tenant View is selected

Select this item with the mouse, the screen of “4.4 Trunk Incoming Tenant Allocation” is displayed. Refer to item description in 4.4 for details.

4.	Screen displayed when O/G Tenant View is selected

Select this item with the mouse, the screen of “4.6 Trunk Outgoing Tenant Allocation” is displayed. Refer to the 4.6. Item description for details.

5.	Screen displayed when Trunk Route View is selected.

Select this item with the mouse, the screen of “4.7 Trunk Route Allocation” is displayed. Refer to the item description in 4.7 for details.

6.	Screen displayed when Telno Information View is selected

Select this item with the mouse, the screen of “3.1 Extension Information” is displayed. Refer to item description in 3.1 for details.

- Contents

| Item | Contents |  |
| --- | --- | --- |
| Port No | Physical location of the relevant subscriber (Shelf / LB / Port). |  |
| Type | Trunk type |  |
|  | C/O | Line connected to the subscriber line of the telephone office |
|  | L/D | Line assigned for Loop Dial |
|  | E&M | Line assigned for Ear and Mouse |
|  | R/D | Line assigned for Ring Down |
|  | DTC-CAS | E1/T1 line used for R2MFC, DTMF, and Pulse. |
|  | DTC-PRI | E1/T1 line using ISDN PRI |
|  | BRIT | Line connected to ISDN BRI line of the telephone office |
|  | QSIG | E1 line using SDN QSIG |
|  | ITGT | Line using Internet Telephony |
| Mode | I/C | Line only for incoming call |
|  | O/G | Line only for outgoing call |
|  | BOTH | Line for incoming/outgoing call |
| Dialing Type | Select one among No dial, DP, DTMF, DP&DTMF, and R2- MFC |  |
| D-Class | Line day class: Used for Transit Toll Check |  |
| N-Class | Line night class: Used for Transit Toll Check |  |
| O/G Tenant | Outgoing tenant number |  |
| I/C Tenant | Incoming tenant number |  |
| BLK | Line manual blocking status: Outgoing limited |  |
| I/C Allow Class | The lowest class that the trunk can call |  |
| Trk Route | The route No. of line (1 ~ 8) |  |
| ITGT GrpNo |  |  |

※ Relation between the trunk type and the trunk board

| Trunk type Trunk board | C/O | L/D | E&M | R/D | DTC-CAS | DTC-PRI | BRIT | QSIG | ITGT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| APTC | √ |  |  |  |  |  |  |  |  |
| ALTC |  | √ | √ | √ |  |  |  |  |  |
| DNIC |  |  |  |  | √ | √ |  | √ |  |
| BRIC |  |  |  |  |  |  | √ |  |  |
| ITGC |  |  |  |  |  |  |  |  | √ |

√mark is the available trunk type for each board.

4.2	Trunk Access Code

- Functional description

This screen is used to verify, change, add, and delete the trunk access code information.

[Figure  Trunk Access Code Information]

- How to use

1	Query

1)	Click Query.

2)	All trunk access code information is queried.

2	Add

1)	Add a data row with the down arrow key ( ↓).

2)	Enter the trunk access code and the trunk access code information to be added. Observe the numbering plan for the call code.

3)	Click Add.

4)	Click Send.

5)	Click Query.

3	Change

1)	Click the data to be changed with the mouse.

2)	Modify each item of the trunk call code.

3)	Click Change.

4)	Click Send.

5)	Click Query.

4	Delete

1)	Select the data with the trunk access code to be deleted.

2)	Click Delete.

3)	Click Send.

4)	Click Query.

※ When the trunk access code is deleted, all the following items are automatically deleted.

- Items for which ARS code-related trunk occupancy code of other trunk occupancy code is used.

- Fixed ARS Code – Items that includes the related trunk occupancy code in the multi-functional telephone flexible button.

- Items for which IND group-related trunk occupancy code is used.

- Codes for which the related trunk occupancy code is used among the system speed dial code.

- Contents

| Item | Contents |
| --- | --- |
| Trk Acc Code | Trunk access code |
| Tenant | The tenant No. of the subscriber who can use the access code |
| Class | The lowest class of the subscriber who can use the access code. |
| Info 1 ① Blocking ② C/O ③ TIE ④ C&I ⑤ IND ⑥ Paging | ① The use of access code is blocked ② The use of access code is charged. (For the connection to the telephone office) ③ The access code set to dedicated line. ④ The access code is used as the individual trunk outgoing code. ⑤ The access code is used as only the individual trunk outgoing code. ⑥ The access code connected to the paging equipment |
| Info 2 ① None ② OWN ③ LCR | Info 2 ① Basic information ② ③ Access code using LCR function |
| O/G digit no | Currently not used |
| AND Digit | Automatic outgoing digit after trunk occupancy |
| ARS Code & Repeat Digit | When all lines assigned for the related trunk access code are occupied, other trunk access code can be occupied and can be used for outgoing. ARS Code : Already assigned Trk Acc Code Repeat Digit : Automatic outgoing digit after trunk occupancy with ARS Code |

AND: Automatic Network Dialing

ARS: Automatic Route Selection

※ [Figure 4-1] The screen displayed when the data part of the extension information screen is selected with the right button of the mouse

1.	When Edit is selected.

Always Editing		: Make data item on the screen always editing mode.

Always Editing Cancel	: Cancel the above Always Editing.

2.	When Tools is selected.

Screen displayed when Change tool view is selected

All data items except for Port No can be changed n this screen. To make all the data of all Port No one, click All OK and to change the data by stages, Click Next Ok.

Use order

1)	Click the [Figure 4-1] data part to be changed.

(The Ext Type part in the figure is changed into the selected data item.)

2)	Change the data in the Edit box on this screen.

3)	Click All OK or Next OK.

- When Trunk O/G Group View is selected

- Functional description

The screen is used to designate the trunk port range that can occupy the trunk access code.

[Figure4-2  Trunk Outgoing Group Information ]

- How to use

1	Query

1)	Click Query.

2)	The trunk port of the trunk access code is queried.

2	Add

1)	Click the data part of the Trunk Port 1 item with the mouse.

2)	Enter the data in Shelf / LB / Port format (e.g. 1/03/01)

3)	Click Add.

4)	Click Send.

5)	Click Query.

Note: When adding many data, be sure to click Query after adding one datum in order to check if the datum is added.

3	Change

1)	Click the data part to be changed.

2)	Change the data in Shelf / LB / Port format. (e.g. 1/03/03)

3)	Click Change.

4)	Click Send.

5)	Click Query.

- Contents

| Item | Description |
| --- | --- |
| Query conditions | Access Code: Enter the trunk access code to query. Tenant : Enter the tenant number to query. |
| Trunk Port 1 | Assign the trunk port range that the given trunk access code can occupy. |

- When Numbering Plan View is selected.

When this menu is selected, 2.1 System Numbering Plan is displayed on the screen. Refer to 2.1 System Numbering Plan for details.

4.3	Trunk Incoming Digit Remove

- Functional description

This screen is used to set up the options for the removal of the trunk incoming digits.

[Figure  Trunk I/C Digit Remove Information]

- How to use

1	Query

1)	Select the route number to be queried.

2) Click Query

2	Change

1)	Select the data to be changed.

e.g) Assign the incoming digit removal option list or select the incoming removal digit number.

2)	Change the data.

3)	Click Change.

4)	Click Send.

5)	Click Query.

- Contents

| Item | Contents |
| --- | --- |
| Query conditions route number | The trunk route number to query (1~8) |
| Incoming digit removal option ① Matched Digit Remove ② Unconditional Digit Remove | ① When the incoming digit is matched with the defined digit, the digit is removed. ② Unconditionally remove the digit as the defined digit length. |
| Length | The length of the digit to remove when the Unconditional Digit Remove option is selected. |
| Incoming removal digit 1~10 | The digit to removed for the Matched Digit Remove option |

This function is used mainly for the ISDN trunk.

4.4	Trunk Incoming Digit Change

- Functional description

This screen is used to change the trunk incoming digit.

[Figure  Trunk I/C Digit Change Information]

- How to use

1	Query

1)	Click Query.

2	Change

1)	Select the data of Trunk Route No. to be changed.

2)	Change the data.

3)	Click Change.

4)	Click Send.

5)	Click Query.

- Contents

| Item | Description |
| --- | --- |
| Trunk Route No | The number of the trunk routes |
| Input Digit (Original) | Digit which is sent to the trunk and to be changed |
| Output Digit (Changed) | Digit which is sent to the trunk and to be changed |

e.g.) If DDI(DID) number is 9XXX and the extension subscriber number is 3XXX, Enter “9” for the Input Digit and enter “3” for the Output Digit in order to call the extension subscribers from outside.

4.6 	Trunk Outgoing Tenant Assign

- Functional description

This screen is used to display the trunk outgoing Port No of each tenant number.

[Figure Trunk Tenant Information]

- How to use

1	Query

1)	Query conditions: Select one of the tenant numbers 1~8.

2)	Click Query.

2	Add

1)	Select the tenant number to be added.

2)	Assign the Trunk Port No to be added.

3)	Click Add.

4)	Click Send.

5)	Click Query.

※ The “tenant number” is added to the Trunk Port No. The information on this item is the mode part of 4.1 Trunk Information. For the Trunk Port No assigned as the outgoing tenant, the mode should be assigned as O/G. The O/G tenant part of 4.1 Trunk Information is automatically assigned to the added tenant number.

3	Delete

1)	Select the tenant number with the trunk port number to be deleted.

2)	Click Query and query the trunk port number that is assigned to the tenant.

3)	Select the trunk port number to be deleted with the mouse and click Delete.

4)	When all the data to be deleted are selected, click Send.

5)	Click Query.

※ Refer to the Delete (D) par of 4.5 Trunk incoming tenant allocation.

- Contents

| Item | Description |
| --- | --- |
| Query conditions: Tenant number | Select the outgoing tenant number to query. |
| Trunk Port No | Trunk port number assigned for each tenant. |

4.7	Trunk Route Base Allocation

- Functional description

This screen is used to display the port No data by trunk route.

[ Figure  Trunk Route Number Assign ]

- How to use

1	Query

1)	Query conditions: Select the route number. (Max 8)

2)	Click Query.

2	Add

1)	Select the route number to be added.

2)	Assign the Trunk Port No to be added.

3)	Click Add.

4)	Click Send.

5)	Click Query.

※ The “Route number” is added and the Trunk Port No is allocated to the added route. The information on this item is the Trk route part of 4.1 Trunk Information. The Trk Route part of 4.1 Trunk Information is assigned to the automatically added route number.

3	Delete

1)	Select the tenant number that has the trunk port number to be deleted.

2)	Click Query and query the trunk port number that is assigned to the tenant.

3)	Select the trunk Port Number to delete with the mouse and click Delete.

4)	When all the data to delete are selected, click Send.

5)	Click Query.

※ Refer to Delete function in 4.5. Trunk incoming tenant allocation.

- Contents

| Item | Description |
| --- | --- |
| Query conditions: Route number | Select the route number to query. |
| Trunk Port No | The trunk port number allocated to each route |

4.8	Trunk Route Lock Parameter

- Functional description

This screen is used to set up the time of the trunk route lock parameter.

[Figure  Trunk Route Lock Parameter]

- How to use

1	Query

1)	Query conditions: Select the route number.(Max 8)

2)	Click Query.

2	Change

1)	Select Trunk Route Lock Parameter Items to be changed.

2)	Update the data in Value item.

3)	Click Change.

4)	Click Send.

5)	Click Query.

- Contents

| Item | Default | Description |
| --- | --- | --- |
| I/C No Digit Lock | 70 | The maximum allowed time from the off-hook receiving to the decision of the person who receives the call for DP, DTMF trunk incoming |
| I/C Total Dial Lock | 100 | The maximum allowed time from the first digit receiving to the decision of the person who receives the call for DP, DTMF trunk incoming |
| O/G No Digit Lock | 70 | The maximum allowed time from the loop seizure and to the dial of first digit for trunk outgoing |
| O/G 1st Inter Digit Lock | 70 | The maximum allowed time between the first digit and the second digit for trunk outgoing |
| O/G 2nd Inter Digit Lock | 60 | The maximum allowed time between the second digit and the third digit for trunk outgoing |
| O/G 3rd Inter Digit Lock | 60 | The maximum allowed time between the third digit and the fourth digit for trunk outgoing |
| O/G 4th Inter Digit Lock | 50 | The maximum allowed time between the fourth digit and the fifth digit for trunk outgoing |
| O/G 5th Inter Digit Lock | 50 | The maximum allowed time between the fifth digit and the sixth digit for trunk outgoing |
| O/G 6th Inter Digit Lock | 40 | The maximum allowed time between the sixth digit and the seventh digit for trunk outgoing |
| O/G 7th Inter Digit Lock | 40 | The maximum allowed time between the seventh digit and the eighth digit for trunk outgoing |
| O/G nth Inter Digit Lock | 30 | The maximum allowed time between the eighth digit and the next digit for trunk outgoing |
| Switching Delay Lock | 8 | Switching Delay Lock for R2 signaling completion |
| ACO I/C Ring Lock CFM Time | 100 | The minimum allowed lock between the rings of ACO trunk |
| ACO I/C Pre-Ring Lock CFM Time | 50 | The minimum allowed lock between the ring and the pre-ring of ACO trunk |
| Guard Time Out Confirm Lock | 10 | The outgoing prevention lock for the restoration of the other side for ACO and LD trunk outgoing restoration |
| Seizure Ack Wait Lock | 50 | The maximum time for waiting for seizure Ack after Loop Seizure |
| Sub Answer Wait Lock | 600 | The maximum waiting time for the sub answer in the trunk with Sub Answer (Unauthorized copy prevention) |
| Alert Indication Wait Lock | 50 | The maximum waiting time for alert message after sending the setup message in VOIP trunk |
| Enblock Digit Lock | 40 | The maximum waiting time for the automatic outgoing of ISDN, QSIG Enblock without dialing the End Digit |
| R2 Protocol Signal Wait Lock | 60 | The maximum waiting time for the backward after sending R2 Forward |
| ISDN DTMF Sending Delay Time | 10 | The delay time for sending DTMF after receiving the response message for ISDN outgoing |
| ACO Re-seizure Delay Time | 7 | The time for call restoration by D-Tel re-outgoing button and re-seizure in ACO trunk |
| ACO Pre Ring Lock Out Time | 20 | The maximum time for waiting for receiving the ring after receiving the pre-ring for ACO trunk incoming If the ring is not received within the time, the received pre-ring is ignored. |
| ACO Receiver Ring Lock Out Time | 50 | The maximum time for waiting for receiving the next ring after receiving a ring for ACO trunk incoming. If the next ring is not received, it is regarded that the outgoing person is released. |
| ACO Seizure Ack. Waiting Time | 20 | The delay time for sending the first digit after loop seizure for ACO incoming |
| ACO Guard Off Time | 5 | Currently not used. (Use Guard Time Out Confirm Lock) |
| ACO Hook Flash Duration Time | 5 | Hook Flash Signal Sending duration time for using ACO exclusive line |
| ACO DP OG Digit Make Time | 2 | Digit Make Signal duration time for ACO Dp outoing |
| ACO DP OG Digit Break Time | 1 | Digit Break Signal duration time for ACO Dp outgoing |
| ACO DP OG Inter Dgt Pause Time | 18 | The pause time between digits for ACO Dp outgoing |
| ACO Ring Detection Time | 30 | The minimum time for ring detection for ACO trunk Ring incoming |
| DTC FWD Clear Wait Time | 30 | The minimum waiting time for the outgoing restoration signal after DTC incoming restoration |
| DTC DP Digit Finish Time | 25 | The maximum time for the authorization of the interval between digits for DTC DP mode trunk incoming |
| DTC DP IC Digit Make Time | 7 | The minimum time for the authorization of Digit Make Signal for DTC DP mode trunk incoming |
| DTC DP IC Digit Break Time | 4 | The minimum time for the authorization of Digit Break Signal for DTC DP mode trunk incoming |
| DTC Seizure Ack. Pause Time | 20 | The delay time for sending Seizure Ack Signal after receiving loop Seizure for DTC trunk incoming |
| DTC DP OG Inter Dgt Pause Time | 60 | The internal between digits for DTC Dp mode trunk outgoing |
| DTC Seizure Ack. Waiting Time | 30 | The time for waiting for the Seizure Ack after Loop Seizure for DTC trunk outgoing |
| DTC DP 1st Dgt Send Pause Time | 25 | The delay time before sending the first digit after receiving Seizure Ack Signal for DTC DP mode trunk outgoing |
| DTC DP OG Digit Make Time | 6 | Digit Make Signal duration for DTC DP mode trunk outgoing |
| DTC DP OG Digit Break Time | 4 | Digit Break Signal duration time for DTC DP mode trunk outgoing |
| DTC Seizure Ack. Detection Time | 20 | The minimum duration time for the authorization of Seizure Ack Signal for DTC trunk incoming |
| DTC Seizure Ack. Duration Time | 15 | Seizure Ack Signal sending duration time for DTC trunk incoming |
| DTC Sub Answer Detection Time | 40 | The minimum duration time for the authorization of Sub Answer Signal for DTC trunk outgoing |
| DTC T1 On Hook Detection Time | 40 | The minimum duration time for the authorization of on-hook signal for using DTC T1 trunk |
| ALTC On-Hook Detection Time | 2 | The minimum time for the detection of the authorization of ALTC trunk On Hook signal |
| ALTC Off-Hook Detection Time | 1 | The minimum time for the detection of the authorization of ALTC trunk off Hook signal |
| ALTC Seizure Ack Wait Time | 20 | The maximum time for waiting for Ack after sending Seizure signal for AFTER ALTC trunk outgoing |
| ALTC O/G Digit Make Time | 2 | Digit Make Signal duration time for ALTC DP mode trunk outgoing |
| ALTC O/G Digit Break Time | 1 | Digit Break Signal duration time for ALTC DP mode trunk outgoing |
| ALTC O/G Inter Digit Pause Time | 19 | The interval between digits for ALTC DP mode trunk outgoing |
| ALTC I/C Digit Finish Time | 3 | The maximum time for the authorization of the interval between digits for ALTC DP mode trunk incoming |
| ALTC I/C Digit Make Time | 8 | The minimum time for the authorization of Digit Make Signal for ALTC DP mode trunk incoming |
| ALTC I/C Digit Break Time | 0 | The minimum time for the authorization of Digit Break Signal for ALTC DP mode trunk incoming |
| ALTC Seizure Ack Detection Time | 8 | The minimum time for the authorization of Seizure Ack Signal for ALTC trunk outgoing |
| ALTC Seizure Ack Duration Time | 3 | The duration time for sending Seizure Ack Signal for ALTC trunk incoming |
| ALTC Pre-Ring Lock Out Time | 20 | The maximum time for waiting for receiving the ring after receiving the pre-ring for ALTC trunk incoming If the ring is not received within the time, the received pre-ring is ignored. |
| ALTC Ring Lock Out Time | 50 | The maximum time for waiting for receiving the next ring after receiving a ring for ALTC trunk incoming If the next ring is not received, it is regarded that the outgoing person is released. |
| ALTC Min. Ring-On Time | 50 | The minimum time for the ring authorization for ALTC trunk Ring incoming |

4.9	All Trunk Route Base Options

- Functional description

This screen is used to set up the entire option allocated to the trunk route.

[Figure  Trunk Route base All Option]

- How to use

1	Query

1)	Query conditions: Select the route number. (Max 8)

2)	Click Query.

2.	Change

1)	Select the list of the item to be changed.

2)	Change the data.

3)	Click Change.

4)	Click Send.

5)	Click Query.

- Contents

| Item | Description |
| --- | --- |
| Query condition route number (1~8) | Select the route number to query. |
| Release for several no answers (0~5) | This item assigns after how many rings the call is sent to the attendant, if the abnormal trunk incoming management is assigned as “Continuous Ring” and the called subscriber gives no answer for trunk incoming. |
| Possibility of unauthorized copy of the connection exclusive network | Currently not used. |
| The class allowed for the proxy outgoing (I, A, B, C, D, E, F, BLK) | The minimum class of the extension subscriber who is allowed for the proxy outgoing function in the attendant. (Reference 1) |
| How to originate VoIP | Direct Call: A call set up mode in which the system collects the digits that the subscriber dials for outgoing and sends them to ITGC at a time. Indirect Call: The system sends the digits one by one that the subscriber dials and DTMF receiver of ITGC collects the digits and sets up a call. |
| Allowed class rising stage (0~8) | Item for the application of the outgoing digit allowing/denying function by raising the class of the extension subscriber in case of the proxy outgoing by the attendant. (Reference 2) |
| Charging format for call transfer | Call Subscriber All : Charge all the first outgoing subscribers for call transfer Transfer Subscriber All : Charge all the transferred subscribers for the call transfer. IND Charge : Charge for the actual call amount. |
| Digit to use as Pause Entry (*, #, A, B, D) | Digit to use as DTMF Sending Pause Digit for trunk outgoing (Reference 3) |
| Use DDI incoming sheltering function | When the system incoming mode is night, whether to send all DDI trunk calls of each route signal not to each called subscriber but to the assigned subscriber. |
| Pause time per Pause Entry (100msec) | The time for pause per each pause digit for DTMF trunk outgoing. |
| R2 path format | Link by Link :The mode in which the system detects and repeats R2 signal for the transit service between R2 dial type trunks End to END :The mode in which the system does not detect or repeat R2 signal but only connects the call and leaves it to the outgoing/incoming terminals for the transit service between R2 dial type trunks. |
| Sender switch on Time | DTMF sending duration time for sending DTMF through DTMF Sender |
| Sender switch off Time | The interval between digits for sending several digits to DTMF Sender |
| RD ring repetition count | The number of time of right to send to the incoming side for RD trunk outgoing |
| ISDN Inband Process Info Available | Currently not used |
| Busy trunk restoration digit (0~9, *, #) | Currently not used |
| Automatic Message Wait registration (CLI) | When a call incomes to ISDN trunk, whether to use the function that registers the message wait automatically if the called subscriber gives no answer or if the originator releases. |
| Whether to send R.B.T. for R/D outgoing | Whether to send RBT to the outgoing subscriber for R/D trunk outgoing |
| The number of restarts for R2 outgoing call failure | The number of re-origination by seizing other trunk when a call set up is failed for R2MFC mode trunk outgoing |
| Whether to request ANI information | Whether to request the originator number for R2MFC mode trunk incoming |
| Whether to send ANI information | Whether to send the originator number on demand of the incoming side for R2MFC mode trunk outgoing |
| I/C Directory Service Use? | Whether to display the assigned name on D-Tel or LCD of ATD When using ISDN trunk and comparing the received originator number with the outside number in the system speed dial and when the two are same. |
| Toll Access Code Exclude in CLI - Yes 포함안함 - No 포함 함 | Decide whether include TOLL ACCESS CODE of PSTN or not . when transfer the resposed user number for I/C or O/G call of ISDN Trunk . 1. Doesn’t include TOLL ACCESS CODE 2. Include TOLL ACCESS CODE ISDN중계선로의 착/발신호에 대해 응답자번호를 보낼 때 PSTN의 TOLL ACCESS Code를 포함 시킬지에 대한 여부 |

Note 1) If the proxy outgoing allowing class is “B”, subscribers of “C, D” class can not use the proxy origination.

Note 2) If the allowed class rising stage is “1”, B class subscriber can originate to A class subscriber by using the proxy origination.

Note 3) If the digit to use as Pause Entry is “D” [16. 7 System speed dial] – OutSide Telno or [4.2 Trunk Access information] – and if Digit “D” is used for AND digit, “D” digit is not originated and the digit sending is delayed for the assigned time.

4.10	Trunk Route Base Incoming Day/Night Option

- Functional description

This screen is used to set up the options of day and night incoming of each trunk route.

[Figure  Trunk Route base I/C Day/Night Option]

- How to use

1	Query

1)	Query conditions: Select the route number and select Day/Night.

2)	Click Query.

2	Change

1)	Select the list of the item to be changed.

2)	Change the data.

3)	Click Change.

4)	Click Send.

5)	Click Query.

- Contents

| Item | Description |
| --- | --- |
| When the called extension is busy for incoming | Option in case the called extension subscriber is busy |
| When the called extension gives no answer for incoming | Option in case the called extension subscriber gives no answer. |
| When ABS registered extension is called for incoming | Option in case the called extension subscriber is absent. |
| When DND registered extension is called for incoming | Option in case the called extension subscriber is in the incoming rejection state. |
| When a vacant extension is called for incoming | Option in case a missing number is dialed in the trunk |
| When incoming restricted extension is called for incoming | Option in case the called extension subscriber is restricted from incoming |
| No Digit for incoming | Option in case there is no digit for a time after trunk seizure |
| When the transferred extension gives no answer for incoming | Option in case the called extension subscriber gives no answer to the incoming trunk transfer call. |

| Option types | Description |
| --- | --- |
| Send Busy Tone | Send Busy Tone |
| Send C.G.T | Send C.G.T |
| Transfer to PNA | Call forwarding to system PNA – Refer to [16.4] |
| Camp On | Register the automatic Camp On |
| Auto Group Hunting | Automatic group hunting |
| Force Release | Forced release |
| Announce Service | Announcement service – Refer to [16.9] |
| Transfer to ATD | Call forwarding to attendant |
| Transfer to VPM | Call forwarding to VPM |
| No Service | Maintain the current status with no service: Applied to only for the individual incoming trunk. |

※ Available options for each item

| Item Option | Busy | No answer | ABS | DND | Vacant | Incoming restriction | No Digit | Transfer no answer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Send Busy Tone | √ | √ | √ | √ | √ | √ | √ | √ |
| Send C.G.T | √ | √ | √ | √ | √ | √ | √ | √ |
| Transfer to PNA | √ | √ | √ | √ | √ | √ | √ | √ |
| Camp On | √ |  |  |  |  |  |  |  |
| Auto Group Hunting | √ | √ | √ | √ |  | √ |  | √ |
| Force Release | √ | √ | √ | √ | √ | √ | √ | √ |
| Announce Service | √ | √ | √ | √ | √ | √ | √ | √ |
| Transfer to ATD | √ | √ | √ | √ | √ | √ | √ | √ |
| Transfer to VPM | √ | √ | √ | √ | √ | √ | √ | √ |
| No Service | √ | √ | √ | √ | √ | √ | √ | √ |

Note: For all trunks assigned to DIL of [6.2], No Service option should be used.

4.12	Inter Route Connection Allow/Deny

- Functional description

This screen is used to manage the connection between the assigned trunk route.

[Figure  Inter Route Connection Allow/Deny Information]

- How to use

1	Query

1)	Click Query.

2	Change

1)	Select the Source Route and the Destination Route to be changed with the mouse.

2)	Click Change.

3)	Click Send.

4)	Click Query.

- Contents

| Item | Description |
| --- | --- |
| Source Route | Outgoing side or waiting route |
| Destination Route | Incoming side or busy route |
| Direct Access Allow/Deny Option | Option in case of the direct access from the incoming route to the outgoing route |
| Transfer by EXT Access Allow/Deny | Option in case the extension subscriber connects the incoming/outgoing route |
| Transfer by ATD Access Allow/Deny | Option in case the attendant connects the incoming/outgoing option |

4.13	Trunk Outgoing Call Duration Restriction

- Functional description

This screen is used to set up the call restriction time when using the trunk.

[Figure  Trunk O/G Call Duration Restriction]

- How to use

1	Query

1)	Query conditions: Select the route number.

2)	Click Query.

2	Change

1)	Select the list of the item to be changed.

2)	Change the data.

3)	Click Change.

4)	Click Send.

5)	Click Query.

- Contents

| Item | Setup item and description |
| --- | --- |
| Call Type ① DDD ② ISD ③ LOCAL ④ TIE | call restriction trunk call types ① Call time restriction on toll call ② Call time restriction on international call ③ Call time restriction on local call ④ Call time restriction on exclusive line call |
| Apply Class | The highest class the extension subscriber whose calling time is restricted based on the call type. When using B class, class B or less (B, C, and D) are limited. |
| Allow Time(min) | The allowed time for call restriction for using trunk. (Unit, Minute) If Time is 3, a call longer than three minutes is restricted. |
| Process Type ① One Alarm & Disconnect ② One Alarm ③ Periodic Alarm | How to restrict a call ① One alarm and the trunk is disconnected. ② Only one alarm ③ Periodical alarm |
| Alarm Periodic (Sec) | The alarm cycle if Process Type is Periodic Alarm (Unit: Second) |

4.13	DISA Authorization Code

- Functional description

This screen is used to query, delete, and add the authorization code to decide whether to allow the call of the subscribers of direct extension and trunk based on the system numbering plan in DISA trunk.

[Figure  DISA Authorization Code]

- How to use

1	Query

1)	Select First Digit. If there is no First Digit, all codes are queried.

2)	Click Query.

2	Add

1)	Select First Digit and query the data.

2)	Click the last item and press ↓.

(When entering the first data, click the first item and enter the data.)

3)	Enter the data.

4)	Click Add. The color of the item is changed.

5)	Click Send.

6)	Click Query.

3	Delete

1)	Select First Digit and query the data.

2)	Click the item to delete.

3)	Click Delete. The color of the item is changed.

4)	Click Send.

5)	Click Query.

- Contents

Authorization Code has maximum eight digits and the length of all authorization code should be the same. In other words, the length of the first added authorization code is assigned as the length of the system authorization code.

Comment is the simple description on the code.

※ DISA data

Line DISA: Mainly used in APTC trunk.

Refer to [4.2 Trunk information] – [Trunk Line Option] – [2 DISA Trunk]

DDI DISA: Mainly used in DDI (DID) trunk.

Refer to [6.4 Access group] – [Service Type] – [DISA].

5	Attendant Data Management

5.1	ATD Link

- Functional description

This screen is used to verify and change the attendant link information to inter-link the attendant.  ? 의미 불분명.

[Figure  ATD Link Information]

- How to use

1	Query

1)	Click Query.

- Query the attendant link information.

2	Add

1)	Add a table to the bottom lower item with the arrow key (↓) .

2)	Enter the ATD Port No to be added.

※ Designate the extension/ subscriber board port designated to ATD.

3)	If the inter-linked ATD exists, enter the Linked Port No.

4)	Click Add.

5)	Click Send.

6)	Click Query.

References	It is possible to designate only the DTEL-LGP extension type to ATD in the <3-1Extension information> item, before designating the ATD Port No to the attendant link information in the above procedure (2).

3	Change

1)	Select the item to change with the mouse.

2)	Enter the attendant link information.

3)	Click Change.

4)	Click Send.

5)	Click Query.

4	Delete

1)	Click the item to delete.

2)	Click Delete.

3)	Click Send.

4)	Click Query.

- Contents

| Items | Contents |
| --- | --- |
| ATD Port No | Attendant port number |
| Linked 1 | 1st Inter-linked ATD port No |
| Linked 2 | 2nd Inter-linked ATD port No |
| Linked 3 | 3rd Inter-linked ATD port No |
| Linked 4 | 4th Inter-linked ATD port No |
| Linked 5 | 5th Inter-linked ATD port No |
| Linked 6 | 6th Inter-linked ATD port No |
| Linked 7 | 7th Inter-linked ATD port No |

References> linking is done sequetially. In other word, the first ATD Port No and Linked 1 are linked. And Linked 1 and Linked 2 are linked. And following process is similar.

5.2	Tenant Representative ATD

- Functional description

This screen is used to designate the service attendant for each tenant.

[Figure  Tenant Representative ATD]

- How to use

1	Query

1)	Click Query.

2)	Query the attendant for each tenant.

2	Change

1)	Select the item to be changed with the mouse.

2)	Enter the attendant link information.

3)	Click Change.

4)	Click Send.

5)	Click Query.

3.	Delete

1)	Click the item to be deleted and click Delete.

2)	Click Send.

3)	Click Query.

- Contents

| Items | Contents |
| --- | --- |
| Tenant No | Designated tenant number |
| ATD Port No | Port number linking the attendant |

5.3	Extension Individual ATD

- Functional description

This screen is used to verify and change the individual attendant information for each extension number.

[Figure  Extension Individual ATD]

- How to use

1	Query

1)	Designate the attendant number to query to the query condition.

2)	Click Query.

3)	The individual attendant for each extension number is queried.

2	Add

1)	Add an item to the bottom lower item with the arrow key ( ↓).

2)	Click the mode to be added and enter the extension number to be added.

3)	Click Add.

4)	Click Send.

5)	Click Query.

3	Delete

1)	Click the item to be deleted.

2)	Click Delete.

3)	Click Send.

4)	Click Query.

- Contents

| Items | Contents |
| --- | --- |
| Query Condition | Select the attendant number to query. |
| Extension No | Extension number managed by the attendant |

5.4	Trunk Individual ATD

- Functional description

This screen is used to verify and change the individual attendant information for each trunk.

[Figure  Trunk Individual ATD]

- How to use

1	Query

1)	Designate the attendant number to query to the query condition.

2)	Click Query.

※ The individual attendant for each attendant number is queried.

2	Add

1)	Click the downward arrow key on the bottom lower item and an additional item appears.

2)	Click the item to be added and enter the trunk port to be added.

3)	Click Add.

4)	Click Send.

5)	Click Query.

- Contents

| Items | Contents |
| --- | --- |
| Query Condition | Select the attendant number to query. |
| Trunk PNo | Board port number linking the attendant |

5.5	ATD Representative Number

- Functional description

This screen is used to verify and designate the attendant representation telephone number.

[Figure  ATD Representative No Information]

- How to use

1	Query

1)	Click Query.

※ The attendant representation telephone number is queried.

2	Change

1)	Enter the attendant representation telephone number.

※ The attendant representation telephone number should match the extension numbering plan. It is not possible to use the extension number as the attendant representation telephone number.

2)	Click Change.

3)	Click Send.

4)	Click Query.

- Contents

Representation telephone number: Attendant representation telephone number

5.6	ATD Service

- Functional description

This screen is used to verify and change the attendant service option.

[Figure  ATD Service Option]

- How to use

1	Query

1)	Click Query.

2)	The attendant service option is queried.

2	Change

1)	Select the item to change with the mouse.

2)	Change the attendant service option value.

3)	Repeat the above procedures (1) and (2) to modify all option values to be changed.

4)	Click Change.

5)	Click Send.

6)	Click Query.

- Contents

| Items | Contents |
| --- | --- |
| Attendant Service Type | Incoming call distribution method for more than one attendant FST: Links the incoming call to the first ATD, if the first ATD is idle. UCD: Links the incoming call to all ATD alternately. |
| Night Subscriber Usage | Whether the attendant night subscriber is used |
| Send ATD Camp-On Tone | Designates whether to send an ATD camp-on tone to the subscriber on busy line when the attendant incoming call is in the camp on status to the other subscriber on busy line ( YES / NO ) |
| Normal Subscriber Inter Position Transfer | YES / NO |
| FRLS Key Using in Connection | Designates whether to use ATD forced release. (YES / NO) |
| Number of Trunk Calling Overflows | 0~9 |
| Use ATD Force Release | Designates whether to use ATD forced release (YES / NO) |
| Number of Attendant Recall Overflows | 0~9 |

5.7	ATD Night Subscriber

- Functional description

This screen is used to verify and change the night subscriber for attendant night transfer.

[Figure  ATD Night Subscriber]

- How to use

1	Query

1)	Click Query.

2)	Query the night subscriber and the operation method by the attendant serial number.

2	Change

1)	Select the night subscriber operation method by the attendant serial number and change the night subscriber and operation methods.

2)	Click Change.

3)	Click Send.

4)	Click Query.

- Contents

| Items | Descriptions |  |
| --- | --- | --- |
| ATD Sno | Attendant serial number |  |
| ATD Pno | Port number linking the attendant |  |
| Option | Not used currently |  |
| Tel No Type | Ext Tel No | If the attendant night subscriber is the extension subscriber. |
|  | Grp Tel No | If the attendant night subscriber number is the representation telephone number designated to the access group |
| Extension No | Attendant night subscriber telephone number |  |

5.8	ATD Overflow Subscriber

- Functional description

This screen is used to verify and change the extension number for temporary attendant transfer for attendant overflow.

It verifies and changes the extension number for attendant overflow

[Figure  ATD Overflow Subscriber Information]

- How to use

1	Query

1)	Click Query.

2)	Query the overflow subscriber and the operation method by the attendant serial number.

2	Change

1)	Select the overflow subscriber operation method by the attendant serial number and change the overflow subscriber.

2)	Click Change.

3)	Click Send.

4)	Click Query.

- Contents

| Items | Descriptions |  |
| --- | --- | --- |
| ATD Sno | Attendant serial number |  |
| ATD Pno | Port number linking the attendant |  |
| Option | Presently not used |  |
| Tel No Type | Ext Tel No | If the attendant overflow subscriber is the extension subscriber |
|  | Grp Tel No | If the attendant overflow subscriber number is the representation telephone number designated to the access group |
| Extension No | Attendant overflow subscriber telephone number |  |

6.	Group Data Management

6.1	Manager Group

- Functional description

This screen is used to connect two specific subscribers with the manager/secretary, and designate the mutual incoming call priority.

Icon used for setting the default value of the group option information value.

[Figure  Manager Group Information]

- How to use

1	Query

1)	Enter the groups to be queried. (Up to 50 groups.)

2)	Click Query.

3)	All manager groups in the query range are queried.

※ The option information of the manger group for the manager is displayed only for one manger at a time. Change the manager group and the option information of the manager group for the manager with the present data cursor on it appears.

4)	Place the cursor on the manager table and press the right button of the mouse. Then, items appear to show the secretary group and the extension number of the present extension subscribers.

2.	Add

1)	Enter the manager group number and the extension number of the manager subscriber.

Click

※ Place the cursor on the manager item and press the right button of the mouse. Then, items appear to show the secretary group and the extension number of the present extension subscribers.

2)	Enter the extension number of the secretary subscriber. The maximum of ten secretary subscribers is available for registration for the manager subscriber.

3)	Designate the option information of the manager group.

4)	Click Add.

5)	Click Send.

6)	Click Query.

[Figure  Security Information]

3	Change

1)	It is possible to change the manager subscriber and option information of the present manager group.

2)	Move the cursor of the manager table to enter the manger subscriber to be changed.

3)	Change the option information. To change only the option, designate the value and be sure to click Change. (The entire option part changes into blue.).

4)	Click Send.

5)	Click Query.

4	Delete

1)	To delete the secretary subscriber, move the cursor of the secretary table on the data to be deleted and click Delete. (Only the part to be deleted changes into an orange color.).

2)	To delete the manger subscriber, move the cursor of the manager table on the data to be deleted and click Delete. The entire group information of the manager subscriber is deleted.

3)	Click Send.

4)	Click Query.

※ If the manager group is deleted, the group number is rearranged. If the first group is deleted, the group number of the second group comes to 1.

- Contents

| Items | Descriptions |
| --- | --- |
| Query Condition : Group No | Manger/secretary group number to be queried |
| Executive No | Manager extension number |
| Executive Call From Extension ① Auto-Transfer ② Executive Call | Manager call from the extension number ① Call transfer to the secretary subscriber ② Call transfer to the manager |
| Executive Call From Trunk ① Auto-Transfer ② Executive Call | Manager call from the trunk ① Call transfer to the secretary subscriber ② Call transfer to the manager |
| All Secretary are DND ① C.G.T ② Executive Call | Manager calling when all secretaries register for DND ① The incoming call hears C.G.T. ② The call is transferred to the manager. |
| Secretary Call Forward ① Ignore ② Call Forward | When the secretary registers for call forwarding ① Ignores the incoming call register function and transfers the call to the secretary ② Forwards the call to incoming call subscriber by the incoming call register function |
| Use All Secretary Call | Presently not used |
| Executive to Executive Class ① NO ② YES | Manager-to-manger call ① Call restriction to manager-to-manger call ② Call permission to manager-to-manager call |
| Use Executive Class ① I,S,A,B,C,D,E,F | Call class of manager-to- manager call (Not related to the subscriber class) Call from the higher manager class to the lower manager class is permitted, while the reverse is not permitted. |

6.2	DIL Incoming Group

- Functional description

This screen is used to verify, change, add and delete the DIL group information (DIL extension information and DIL option).

2.2 Icon that show The system local board information.

[Figure 6.2.1  Trunk DIL Assign]

- How to use

1	Query

1)	Enter the trunk Shelf / LB / Port information into the query condition.

(Or, Click Local Board Information.)

2)	Click Query.

2	Change

1)	Select the item to be changed with the mouse.

2)	Modify the data to be changed.

※ Use Row Data Exchange and Column Data Exchange on the tools of the right button of the mouse to modify conveniently according to the item or the Trunk Port No.

3)	Click Change.

4)	Click Send.

5)	Click Query.

- Contents

| Items | Contents |
| --- | --- |
| Trunk Port No | Location Shelf / LB / Port of the relevant trunk |
| Day Type ① None ② Extension No ③ OutSide Ext ④ Group TelNo | Call type of the day system mode ① Ignores the incoming call ② Call forwarding to the extension subscriber ③ Trunk seizure and call forwarding to the outside subscriber ④ Call forwarding to the extension subscriber of the access group: Refer to [6.4 Access group] |
| Day Destination ① Extension No ② OutSide Ext ③ Group TelNo | Called number in the day mode ① Telephone number of the extension subscriber ② Trunk access code + telephone number of the outside subscriber (Ex.:82000) ③ Representation number of the access group |
| Night Type | Call type of the night system mode |
| Night Destination | Called number in the night mode |

Note 1) DIL incoming call data input by Hunting or Group calling,

- 1.Create a virtual number to be called in the [6.4 Access group].

- 2.Change the Ring Service Method according to the calling method in the [6.4 Access group]

- 3.Designate the Day/Night Type to the Group TelNo.

- 4.Enter the virtual number designated in the [6.4 Access group] into the Day/Night Destination.

- Mouse pop-up menu on the screen of [Figure 6.2.1]

Click the right button of the mouse in the data part of [Figure 6.2.1] and the following pop-up menu appears.

The usage of each menu is as follows.

1.	When selecting Edit …

-		Always Editing: This maintains the data item of the screen in the edit mode.

-	Always Editing Cancel: This cancels the above Always Editing function.

2.	When selecting Tools

- Screen displayed when selecting Row Data Change screen

It is possible to change data of items except for Trunk Port No in this screen. Click All OK to set up one type of data for all Port No, and click Next OK to change data by stages.

Use order

1)	Click the data to be changed in [Figure 6.2.1].

(The Day Type part in the above figure changes into the data item selected.)

2)	Change data of the Edit box on the screen.

3)	Click All OK or Next OK.

- Screen displayed when selecting Tools  Column Data Change

It is a useful menu to change data based on the line of data. Enter a proper data type into each data item or select one from the list. Then, use the buttons as follows.

1)	Use this button to move to the Port No previous to the Port No on the above screen.

Ex) Use this button to change the Port No: 1/03/04 data before Port No: 1/03/05.

2)	Use this button to move the Port No following the Port No on the above screen.

Ex) Use this button to change the Port No: 1/03/06 data following the Port No: 1/03/05.

3)	Use this button to select and change proper data in each data item of the above screen.

4)	Use this button to enter same data into all Port No after selecting and entering proper data into each data item of the above screen.

5)	Use this button to complete all the above data changing process and end the screen.

3.	When selecting Calling Group View

Select this menu and the “6.4 Access group ” screen appears. Therefore, Day Type or Night Type of the data item should be set up to Group TelNo to select this menu. Refer to “6.4 Access group” for details of Group Tel No.

4.	When selecting Tel No Information View

This menu is the same menu as the pop-up menu with the right button of the mouse on the “3.1 Extension information” screen. Refer to “[Figure 3-1] Screen by selecting the extension information screen data part with the right button of the mouse” described in 3.1 Extension information.

6.3	IND Group

- Functional description

This screen is used to set up the trunk Port and the trunk access code to be used by the extension number of the IND outgoing trunk group.

[Figure IND Trunk Service Group]

- How to use

1	Query

1)	Click the selection list to be queried and select it by √.

2)	Designate the group number of the selected list.

3)	Click Query.

2	Add

1)	Click the selection list to be queried and select it by √.

2)	Enter the Group No.

3)	Click Query. ( Query the relevant group data)

If a group is not used, an Error Message is indicated.

4)	Add data to be added by each item.

5)	Click Add.

6)	Click Send.

7)	Click Query.

3	Change

1)	Click the selection list to be queried and select it by √.

2)	Enter the Group No.

3)	Click Query. (Query the relevant group data)

If a group is not used, an Error Message is indicated.

4)	Enter data to be changed.

5)	Click Change.

6)	Click Send.

7)	Click Query.

4	Delete

1)	Click the selection list to be queried and select it by √.

2)	Enter the Group No.

3)	Click Query. (Query the relevant group data)

If a group is not used, an Error Message is indicated.

4)	Select the data item to be deleted.

5)	Click Delete.

6)	Click Send.

7)	Click Query.

- Contents

| Items | Descriptions |
| --- | --- |
| IND outgoing trunk group number ① Extension No | Designates the IND extension subscriber group to use IND trunks ① Extension subscriber telephone number to use the relevant group |
| Trunk group number by Access Code ① Access Code ② IND Trk Grp No | Designates the trunk access code for each IND group ① Trunk access code : Refer to 1) ② IND outgoing trunk group to use the trunk access code |
| Outgoing trunk group number ① Acc Method ② Trk Port No | Designates the trunk for the trunk access code for each IND group. ① How to use the circuit ;FST/UCD ② Trunk Port for the access code |
| IND O/G Extension Grp No | IND outgoing extension group number |

※ FST	: Priority trunk seizure from the first UCD	: Sequential trunk seizure

Note 1) The Info1 of the [4.2 Trunk ACCESS code information] should be designated as C&I in the trunk access code.

6.4 	Calling Group

- Functional description

This screen is used to register, change, add, delete and verify the extension number of Group Tel No.

(Reference: It is not possible to use the existing extension number as Group Tel No.)

[Figure  Calling Group]

- How to use

1	Query

1)	Enter the representation number in the query condition.

2)	Click Query.

3)	The representation number data appears.

4)	Click the right button of the mouse in the queried item to query the extension number of the representation number. Then the sub-item appears. Click Group Calling Service View among items. Then, the extension information window of the access group appears, and query starts.

2	Add

1)	Select (Click) in the bottom lower item and add the one line of data with the arrow key( ↓).

2)	Enter the representation number and the option of the representation information.

(Be sure to enter items except for Rep Name.)

3)	Click Add.

4)	Click Send.

5)	If the relevant representation number is normally added, the [Calling group service extension information] screen appears.

[Figure  Calling Group Service Ext]

6)	Add the extension number for the representation number as shown in the above screen.

7)	Click Add.

8)	Click Send.

9)	Click Query.

3	Change

1)	Click the item to be changed.

(The representation number is not changed.)

2)	Click Change.

3)	Click Send.

4)	Click Query.

4	Delete

1)	Click the item to be deleted.

2)	Click Delete.

3)	Click Send.

4)	Click Query.

※ Deletion of the representation number causes automatic deletion of the subordinate extension number.

- Contents

| Items | Descriptions |
| --- | --- |
| Rep No | Item to enter the Group Tel No (a virtual number) to add |
| Service Type ① NOT USE ② NORMAL ③ MAN ④ SEC ⑤ AUTOATD ⑥ MAILACC ⑦ BULLETIN ⑧ TOKENPALY ⑨ TOKENCHG ⑩ DISA | Incoming service type of the representation number calling ① Do not use ② Service in the Ring Service Method ③ Only the relevant secretary has access to the representation number for one manager group. ④ Only the relevant manager has access to the representation number of one secretary group. ⑤ Representation number for the automatic attendant transfer as the VPM representation number ⑥ Representation number for the voice mail service as the VPM representation number ⑦ Representation number for the voice bulletin service as the VPM representation number ⑧ Representation number for the voice token play of VPM voice bulletin ⑨ Representation number for voice token change of the VPM voice bulletin ⑩ Representation number for transit of the incoming call to other trunk It is used for TANDEM service. |
| Ring Service Method ① FIRST ② UCD ③ MULTI ④ LONG_IDLE | Representation number calling ① Call transfer to the first designated extension number of the representation number. ② Circular call transfer to the extension terminal of the access group ③ Synchronous call transfer to all extension numbers included in the representation number ④ Call transfer to the longest idle extension number among the extension numbers included in the representation number |
| Ext-Class | The minimum permission class of the extension subscriber to call the representation number |
| Trunk-Class | The minimum permission class of the trunk subscriber to call the representation number |
| Rep Name | Presently not used |
| Busy & DND Option ① Send Busy Tone ② Send C.G.T ③ Transfer to PNA ④ Continue to Ringing ⑤ Force Release ⑥ Announce Service ⑦ Transfer to ATD ⑧ Transfer to VMS ⑨ No Service | As the item applied only for the incoming trunk, follow these options instead of 4.10 Trunk route day/night incoming option for the optional handling when all extension numbers included in the representation number are busy or in the DND status. Refer to 4.10 Trunk route day/night incoming option for details. |
| No Answer Option ① Send Busy Tone ② Send C.G.T ③ Transfer to PNA ④ Auto Group Hunting ⑤ Continue to Ringing ⑥ Force Release ⑦ Announce Service ⑧ Transfer to ATD ⑨ Transfer to VMS ⑩ No Service | As the item applied only for the incoming trunk, follow these options instead of 4.10 Trunk route day/night incoming option for the optional handling, when a representation calling is transferred to the relevant group extension subscriber with the ringing tone but the extension subscriber does not respond to the incoming call. Refer to 4.10 Trunk route day/night incoming option for details. |
| R2 ANI Request | Items to designate whether to request ANI for the representation number calling by the R2 incoming trunk |

※ Screen displayed when selecting Calling Group Service View with mouse right button

- Functional description

This screen is used to display the extension number included in the access group.

[Figure  Calling Group Service Ext]

- How to use

1	Query

1)	Enter the representation number of the query condition.

2)	Click Query.

3)	Extension numbers included in the representation number is queried.

2	Add

1)	Select the data item of Extension No with the mouse.

2)	Move to the bottom lower of data with the downward arrow key ( ↓).

3)	Enter all extension numbers to be added (Enter only SLT and D-Tel. )

4)	Click Add.

5)	Click Send.

6)	Click Query

3	Delete

1)	Select the extension data of the Extension No item to be deleted.

2)	Click Delete.

3)	Click Send.

4)	Click Query.

- Contents

| Items | Descriptions |
| --- | --- |
| Representation number | Group Tel No |
| Extension No | Extension subscriber number included in Group Tel No Only the SLT, DTEL-LGP, DTEL-LKD, and VPM subscribers are available in the [3.1 Extension information] –ExtType |

6.5	Extension Pickup Group

- Functional description

This screen is used to designate the pickup group for the relevant extension number to respond to incoming call for the extension subscriber of the same group or the other group.

[Figure  Extension Pickup Group Information]

- How to use

1	Query

1)	Enter the extension number to be queried into the query condition.

2)	Select the selection items (Self-Linked Table, Next Linked Table) to be queried.

3)	Click Query.

2	Add

- At first, input Extension number at Query conditon section.

- Move the mouse into item where user want to add.

- Click the right button of the mouse

- User can see the following popup window.

- Click   Tools  Add Tool View

- Add Tool Window pops up.

- To Registrate pick-up group at same group, select the Self Liked Table, and Input Tel number to link and click   Append OK

- To Registrate other pick-up group,select the Next Liked Table , and Input Tel number to link and click   Append OK

- Click Send.

- Click Query.

3	Delete

1)	Select the item to be deleted with the mouse left button.

2)	Select the data to be deleted with the mouse left button of.

3)	Click Delete.

4)	Repeat the above procedures (2) and (3) to select all the data to be deleted.

5)	Click Send.

6)	Click Query.

- Contents

| Items | Contents |
| --- | --- |
| Query condition | Extension number |
| Shelf Linked Table | First pick up priority group |
| Next Linked Table | Second pick up priority group |
| Extension No | Extension number Only the SLT, DTEL-LGP, and DTEL-LKD subscribers are available in the [3.1 Extension information] –ExtType. |

6.6	Zone Paging Group Information

- Functional description

It is possible to register up to 32 extension subscribers for a group by using the internal zone paging function.

[Figure  Zone Paging Using Group]

- How to use

1	Query

1)	Click Query.

2)	If the extension number is designated to the internal zone paging group, √ is marked on the right side of the Zone Paging Group item.

- Contents

The group without the √ among the Zone Paging Groups during query is a group being not used.

※ Screen displayed when clicking the Zone Paging Group 1

[Figure  Zone Paging Group]

- How to use

1	Query

1)	Select the group number from the query condition.

2)	Click Query.

2	Add

1)	Click the downward arrow key in the bottom lower item and the additional item appears.

2)	Click the item to be added and enter the extension number and option to be added.

3)	Click Add.

4)	Click Send.

5)	Click Query.

3	Delete

1)	Click the mouse button the item to be deleted.

2)	Click Delete.

3)	Click Send.

4)	Click Query.

- Contents

| Items | Contents |
| --- | --- |
| Query condition | Group number |
| Extension No | Extension number to the group Only the DTEL-LGP, DTEL-LKD subscribers re available in the [3.1 Extension information] –ExtType |
| Called Option ① YES ② NO | Selects subscriber to respond to the internal zone paging. ① It is possible for the extension number to respond to the internal zone paging. ② It is not possible for the extension number to respond to the internal zone paging. |

Reference) It is possible for only the subscriber marked with √ in [3.1 Extension information] – [Extension characteristics information] – [37.Z-paging use] to call the Zone Paging Group.

6.7	Hunting Group

- Functional description

This screen is used to verify and produce the Extension Hunt Group data.

[Figure  Hunting group Information]

- How to use

1	Query

1)	Enter the extension number into the query condition.

2)	Click Query.

2	Change

1)	Select the data to be changed with the mouse.

2)	Enter the extension number to be changed.

3)	Click Change.

4)	Click Send.

5)	Click Query.

3	Add

- At first, input the Extension Number , and click the right button at Extension No to be linked . and then user can see a popup window as following.

- Select  Add Too View item. And user can see a Append Tool window as following.

At this window , user can input the telephone number. Input the Extension number of Query conditon and input the number to link at Link Telno part.

3)	Click  Add ok.

4)	Click Send.

5)	Click Query.

※ Type of Errors and Handling

| Type of Errors | Descriptions |
| --- | --- |
| No Data | No data are found in the system. |
| Invalid Extension Numbering plan | The extension number does not match the extension numbering plan. Verify the extension numbering plan on the Numbering plan menu. |
| Not Registered Extension Number | The actual extension subscriber number is not registered. Verify the extension subscriber number and use the extension subscriber number actually registered for the system. |
| Incorrect Message | Data are not entered to the data mandatory field. |

- Contents

| Items | Contents |
| --- | --- |
| Query condition: Extension number | Extension number to query the subscriber in the Hunting group Ex) Query condition to query the Hunting group subscriber extension number included in 4869: Enter ‘4869’ to the extension number. |
| Extension No | Extension subscriber number in the Hunting group Only the DTEL-LGP, and DTEL-LKD subscribers are available in the [3.1 Extension information] –ExtType |
| Master Bit | Subscriber in the Hunting group without incoming call by the Hunting service |
| Auto Group Hunting | Automatic Group Hunting services in the subscriber in-service status. |

7	ISDN Data Management

7.1	ISDN Lock-Out Time

- Functional description

This screen is used to query and change standard Timer value for ISDN service.

[Figure  ISDN Lock Out Time]

- How to use

1	Query

1)	Click Query.

2)	Verify related items.

2	Change

1)	Select data part of the item to be changed with mouse.

2)	Designate value to be changed.

3)	Click Change.

4)	Click Send.

5)	Click Query.

- Contents

See the documents below for detailed contents of Timers listed in ITEM NAME.

-	ETSI Standard: ETS 300 102-1 Chapter 9

-	ITU-T Standard: ITU-T Q.931 Chapter 92

7.2	ISND Options

- Functional description

This screen is used to query and change the necessary common options, when ISDN extension and trunk is used in the system,.

[Figure  ISDN System Opton]

- How to use

1	Query

1)	Click Query.

2)	Verify related items.

2	Change

1)	Select the item to be changed with mouse.

2)	Designate the value to be changed.

3)	Click Change.

4)	Click Send.

5)	Click Query.

- Contents

| Item | Description |
| --- | --- |
| Protocol Type ① ITU-T ② ETSI | Item to select ISDN protocol ① North America standard system ② European standard system |
| Outgoing Digits ① Overlap ② Enblock | When making a call, item to designate method to send digit ① Outgoing digits are divided and sent ② Outgoing digits are sent all at a time |
| Line Identification Restriction ① YES ② NO | Item to define whether to provide calling/called number to another party ① When making a call, do not provide calling number to called party, and when answering a incoming call, do not provide called number to calling party ② When making a call, provide calling number to called party and, when answering a incoming call, provide called number to calling party |
| Line Identification Sending Type ① International ② National ③ Subscriber ④ Unknown | When providing calling/called number to another, item to designate which type to send it (information included) ① Country Code + Own Area Code + DDI Prefix + Ext. No ② Own Area Code + DDI Prefix + Ext. No ③ DDI Prefix + Ext. No ④ Ext. No |
| AOC Type ① NOT_USE ② AOC-S (Advice Of Charge at Call Setup Time) ③ AOC-D (Advice Of Charge during the Call) ④AOC-E (Advice Of Charge at the End of the Call) | Item to define which type of charging information to be provided by another station ① In case of charging information in accordance with standard cannot be provided ② System to send charging information at the same time as the start of call ③ System to send accumulated charging information periodically and to send charging information of all calls again at the release of call ④ System to send charging information of all calls only once at the release of call |

7.3	MSN / Sub Address Destination Assign

- Functional description

If ISDN BRI is used as trunk and relevant trunk is Point to Multi-point system (in case of using one of MSN or Sub Address system), this screen is used to query, add and delete the received number and connection information of called party.

[Figure  MSN/Sub Address Destination Assign]

- How to use

1	Query

1)	Enter the range of Shelf / LB / Port in Condition.

- Though one ISDN BRI line actually has 2 channels (2 ports), it is treated in the same way in operation. So, data generation is permitted only for odd-nth ports and, even when call is terminated to the even-nth ports, the data generated in the first port is applied.

2)	Click Query.

3)	Verify received number and designated called party for each port.

2	Add

1)	In case of adding data to the relevant port for the first time, first enter the desired Shelf / LB / Port on the left screen.

2)	On the right screen, enter number to be received, called party type, called party number.

3)	When adding continually to the same port, Click ↓. Then, a new blank appears for data to be entered.

4)	Place cursor on added data and click Add.

5)	In case of adding more than one data, repeat section (4).

6)	Click Send.

7)	Click Query.

3	Delete

1)	First, query data for the desired port.

2)	Place cursor on the desired data and click Delete.

3)	Click Send.

4)	Click Query.

- Contents

| Item | Description |
| --- | --- |
| Trunk Port No | Shelf / LB / Port of ISDN BRI trunk |
| I/C Digit | In case of MSN, indicates telephone number in Called Party Information Element (called number) of incoming SETUP Message In case of Sub Address, indicates telephone number in Called Party Sub Address Information Element (sub called number) of incoming SETUP Message |
| Destination Type | Type of incoming target destination number |
| Destination | The number of called party such as extension number, representing number of call group, external number (trunk seizure code included) |

7.4	ISDN National Type IE

- Functional description

This screen is used to query, add and delete information elements designated and used for each country.

[Figure  ISDN National Type Information Element ID Assign]

- How to use

1	Query

1)	Click Query.

2)	Verify the data.

2	Add

1)	Double click Information Element Name item.

2)	Select the desired Information Element Name.

3)	Double click Code Set item.

4)	Select Code set used for the relevant Information Element item.

5)	Enter the value used for Information Element.

6)	Place cursor on added data and click Add.

7)	Click Send.

8)	Click Query.

3	Delete

1)	First, perform query process.

2)	Place cursor on data to delete and click Delete.

3)	Click Send.

4)	Click Query.

- Contents

| Item | Description |
| --- | --- |
| Information Element Name | Name of service for which the generated Information Element is used |
| Code Set | As a set of values indicating information element with certain rules, set available as National Type is Code Set 5, 6, 7. |
| ID | Given value to identify the relevant information element in actual ISDN Message |

8	QSIG Data Management

차후  내용 추가할 예정임

9	DECT Data Management

9.1	Park Code

- Functional description

Because Park Code (Portable Access Rights Key Code, System ID) has its own proper value according to board manufacturer, if the same PARK key value is assigned to different systems, problems occur in call service. Therefore, PARK Key should be managed with caution. Also, because each terminal identifies a specific system by PARK Key, one terminal may be registered in multiple systems with different PARK Keys.

[Figure  PARK Code Information]

- How to use

1	Query

1)	In “Query condition: type” item, select the device (CPM or WTIC) to be queried.

2)	Click Query.

2	Register

1)	In “Registration Type” item, select Type.

- CPM  WTIC: used to register Park Key registered in CPM to WTIC B’d

- WTIC  CPM: used to register Park Key registered in WTIC B’d to CPM

- New PARK Code: used to register Park Key to both CPM and WTIC B’d

2)	In “PARK Key” item, enter Park Key.

3)	Click Send.

4)	Click Query to verify registration.

9.2	Authentication Code

- Functional description

Authentication Code is a key used to authenticate terminal in WTIC, and this value can be used redundantly in different systems.

[Figure  Authentication Code Information]

- How to use

1	Query

1)	In “Query condition: type” item, select the device (CPM or WTIC) to query.

2)	Click Query.

2	Register

1)	In “Registration Type” item, select Type.

- CPM  WTIC: used to register Auth. Code registered in CPM to WTIC B’d

- WTIC  CPM: used to register Auth. Code registered in WTIC B’d to CPM

- New Auth. Code: used to register New Auth. Code to both CPM and WTIC B’d

2)	In “Authentication Code” item, enter Authentication Code.

3)	Click Send.

4)	Click Query to verify registration.

9.3	DECT Information

- Functional description

As a window that shows DECT ID, Phone type, DECT TelNo, classes for DECT terminal, it identifies DECT terminals in WTIC board by terminal No. in which a total of 96 terminals can be registered. Terminal can be registered randomly with regard to order and DECT ID is assigned in order of 1 through 96.

[Figure  DECT Information]

- How to use

1	Query

1)	Click Query.

2	Change

1)	Select data part of the item to be changed with mouse.

2)	Change data. But, DECT ID/Phone and type/DECT TelNo cannot be changed.

3)	Click Change.

4)	Click Send.

5)	Click Query.

3	Register

1)	In data part, create data line to be added.(using direction keys)

2)	Add data.

3)	Click Add.

4)	Click Send.

5)	When Wait Dialog window appears as below, proceed with registration procedure in terminal.

[Figure  DECT Information]

6)	If data is registered normally, Wait Dialog disappears and the relevant terminal is registered. Click Query to verify. But, in case errors occur when registering, data line to be added is indicated in red and error message is displayed.

Reference) for registration procedure in terminal, see manual of terminal to be used.

4	Clear

1)	Click data part of the item to be cleared and click Delete.

2)	Select one of clear methods below (De-subscription or Clear) and click OK.

3)	Click Send.

4)	If terminal is normally cleared by Desubscription, the relevant item is not displayed any more in OMS window and in LCD window of the relevant DECT terminal, “DESUBSCRIB PRESS PWR” message is displayed. Also, if terminal is normally cleared by Clear, the relevant item is not displayed any more on OMS window.

5)	Click Query to verify information of the relevant terminal has disappeared.

Reference) Because ‘Clear’ deletes only terminal information saved in system and board, information of terminal should be deleted separately. Because ‘Desubscription’ deletes all information of system, board and terminal, terminal should be in activated state.

All Terminals Clear

1)	When clicking Delete, the window as below is displayed

[Figure  DECT Information]

[ Figure DECT Information- All Terminals Clear ]

2)	Click YES to clear all terminals, otherwise click NO.

3)	If all terminals are normally cleared, information of all DECT terminals disappears and information of DECT terminal is not displayed on screen any more.

- Contents

| Item | Description |
| --- | --- |
| DECT ID | ID of the DECT terminal registered in system DECT (1~96) |
| Phone Type | In DECT protocols, protocol used by the relevant terminal - LG1: Standard DECT Phone. - LG2: GAP supported DECT Phone. - ETC : used in other terminals than LG1, LG2 |
| DECT Tel No | Telephone number of the relevant DECT terminal. |
| Day Class | Daily class |
| Night Class | Night class |

10	Hotel Data Management

10.1	Room Subscriber

- Functional description

This screen is used to verify, change, add and delete extension number and status of the room subscriber.

[Figure  Hotel Room Subscriber]

- How to use

1	Query

1)	Designate the range of extension number or Shelf / LB / Port in query conditions.

2)	Click Query.

2	Add

1)	In data part, create data line to be added.(using direction key)

2)	Add the data.

3)	Click Add.

4)	Click Send.

5)	Click Query.

3	Change

1)	Click data part of the item to be changed.

2)	Change the data.

3)	Click Change.

4)	Click Send.

5)	Click Query.

4	Delete

1)	Click the item to be deleted.

2)	Click Delete.

3)	Click Send.

4)	Click Query

- Contents

| Item |  | Description |
| --- | --- | --- |
| Extension |  | Extension number of hotel room subscriber |
| VIP | Non VIP | General customer |
|  | VIP | VIP customer |
| Reserved | Not Reserved | Not reserved |
|  | Reserved | In reservation |
| Room Cut/Off | No Room Cut | No room closed |
|  | Room Cut | Room closed |
| Check In/Out | Check Out | Check out |
|  | Check In | Room in use |
| Cleaning Status | Service End | Service termination state |
|  | Cleaning Needed | Room cleaning needed |
|  | Under Cleaning | Under cleaning |
|  | Cleaned Up | Cleaning completed |
|  | Repair Needed | Repair needed |
|  | Under Repair | Under repair |
|  | Repair Complete | Repair complete |
|  | Reserved | In reservation |

10.2	Room To Rome Blocking

- Functional description

This screen is used to verify and change call restriction time between subscribers designated as "Hotel Room".

[figure  Room to Room Blocking time]

- How to use

1	Query

1)	Click Query.

2	Change

1)	Enter time for restricting call.

In Edit mode, enter time.

Designate time shown with mouse drag and drop.

2)	Click Change.

3)	Click Query.

- Contents

| Item | Description |
| --- | --- |
| Start Time (Hour : Min) | Time to start call restriction for room |
| End Time(Hour : Min) | Time to end call restriction for room |

10.3	Hotel Single Digit Service

- Functional description

When the subscriber designated as "Hotel Room" presses one-figure digit and a certain time passes, it is automatically connected to the designated extension subscriber.

[Figure  Hotel Single Digit]

- How to use

1	Query

1)	Click Query.

2	Change

1)	Select extension number data part of the Single Digit Table to be changed.

2)	Designate the desired extension number.

3)	Click Change

4)	Click Send.

5)	Click Query.

- Contents

| Item | Description |
| --- | --- |
| Single Digit | Speed dial of extension number of hotel room |
| Extension No | Extension number to be designated in speed dial |

10.4	Wakeup Service  (Timed Reminder)

- Functional description

This screen is used to verify and change timed reminder service information.

[Figure  Wakeup Service]

- How to use

1	Query

1)	Click Query.

2	Change

1)	Click ▼ button of the items to be changed to select the desired list or to correct data.

2)	Click Change.

3)	Click Send.

4)	Click Query.

- Contents

| Item | Description | Remarks |
| --- | --- | --- |
| Result processing for no answer of timed reminder ① No Service ② ATD | ① Service not provided, ② Notifies no answer to attendant |  |
| Service result processing for timed reminder called ① No Service ② ATD | ① Service not provided. ② Notifies no answer to attendant |  |
| Display of contents of timed reminder registration/answer ① No Service ② ATD | ① Service not provided ② Notifies no answer to attendant |  |
| Number of recalls for no answer of timed reminder | Number of recalls after no answer of timed reminder | Default is 3 times |
| Time interval for recalling timed reminder | Time interval after recalling timed reminder | Default is 3 minutes |

11	Charging Management

11.1	Charging/Printer Option

- Functional description

This screen is used to verify and designate options for charging and display device.

[Figure  Charge/Printer Option Information]

- How to use

1	Query

1)	Click Query.

2)	Verify options for charge and display device.

2	Change

1)	Click the item to be changed.

2)	In each item, change values of list.

3)	Click Change.

4)	Click Send

5)	Click Query

- Contents

| Item | Description |
| --- | --- |
| How much for a metering Tone? | When selecting method of charge account by metering tone sent from trunk side, determines charge per metering tone |
| Charge Result Output Device ① ATD ② SMDA ① ATD&SMDA | Designates unit to display charge information. ① Attendant ② Account unit ③ Attendant and account unit |
| Printer Breakdown Alarm | Currently not in use |
| Printer Type | Currently not in use |
| Space between line | Currently not in use |
| How many line in a page | Currently not in use |
| Output format | Currently not in use |
| VIP shadowed digit | Currently not in use |
| Charge start time | In case there is no signal of elapsed time (external subscriber answer signal), means time that charge account is started after user sends last dialed number. |
| Charge Tax rates | Currently not in use |
| Local Call Charge | When displaying charge information, designates whether to display list of local calls |
| Tie Call Charge | When displaying charge information, designates whether to display list of dedicated line calls |
| Use Host PMS | Currently not in use |

Reference) In case Charge Result Output Device is attendant, charge information displayed to attendant is designated for each subscriber. [3.1 Extension information] – [Extension feature information] – [Charge Send to ATD]

11.2	No-Charge Code

- Functional description

This screen is used to verify, add and delete external subscriber number not charged.

[Figure  No Charge Code]

- How to use

1	Query

1)	Click Query.

2	Add

1)	Generate one line to be added using ↓key.

2)	Enter No Charge Code.

3)	In order of (1), (2), enter all data to be added.

4)	Click Add.

5)	Click Send.

6)	Click Query.

3	Delete

1)	Select the item to be deleted by clicking it.

2)	Click Delete.

3)	Click Send.

4)	Click Query.

- Contents

No Charge Code: External subscriber number not charged

12	LCR Data Management

- LCR functional description

In case, for day/time zone/destination area, several special carriers with different charges exist, if the subscriber dials the designated trunk seizure code and called subscriber number without any consideration, call is automatically connected through the designated trunk in consideration of the present day/time/outgoing digit.

12.1	LCR Information

This screen is used to query and change basic required information when using LCR.

[Figure  LCR information]

- How to use

1	Query

1)	Click Query.

2	Change

1)	Change the desired item.

2)	Click Change.

3)	Click Send.

4)	Click Query to verify contents changed.

Note 1 To use LCR function, trunk seizure code for LCR should be first generated. (See 4.2 Trunk Access Code menu)

- Contents

| Item | Contents |
| --- | --- |
| Use Available | Whether LCR service can be used currently (if “NO” is designated, the relevant trunk seizure code is used for general outgoing call) |
| Access Code Repeat | Whether to automatically repeat trunk seizure code for LCR |
| Pause Digit | When making a call with digit for pause digit to be used for add digit of LCR DMT, it is automatically replaced with pause digit used by the router selected. |
| Authentication Code Digit | When making a call with digit for authentication code to be used for add digit of LCR DMT, it is automatically replaced with authentication code of extension subscriber. |
| Extension Telephone No | When making a call with digit for Extension Telephone No to be used for add digit of LCR DMT, it is automatically replaced with extension subscriber telephone number. |
| Day Zone for XXXX | Day zone to be used in XXX day |
| Day Zone # Time Zone 1 Start Time | With a day divided into three time zones, start time of the first time zone of # Day Zone |
| Day Zone # Time Zone 2 Start Time | With a day divided into three time zones, start time of the second time zone of # Day Zone |
| Day Zone # Time Zone 3 Start Time | With a day divided into three time zones, start time of the third time zone of # Day Zone |

12.2	Leading Digit Table (LDT)

This screen is used to verify, change, add and delete DMT for each leading digit used in LCR service.

Note 1 Each leading digit dialed by subscriber (up to 6 digits from the first digit dialed by subscriber) has DMT table information to be used for each day/time zone. Up to 6 leading digits can be designated, and the same leading digit can be designated if its tenant is not same.

[Figure  LTD(Leading Digit Table)

- How to use

1	Query

1)	In Query item, select extension outgoing tenant to use the relevant DMT and click Query.

2)	Verify DMT data.

2	Add

1)	Select O/G tenant to be added in Query Condition.

2)	In the last data part, add one data line using ↓key.

3)	Designate DMT table index for each day zone and time zone, matching Serial No, Leading Digits and LDT

(With Day Zone as criteria, Time Zone is equally divided into three and entered.)

4)	Click Add.

5)	Click Send.

6)	Click Query.

3	Change

1)	Click data part of the item to be changed.

2)	Change the data.

3)	Click Change.

4)	Click Send.

5)	Click Query.

- Contents

| Item | Contents |
| --- | --- |
| Tenant | Outgoing tenant of extension subscriber to use the relevant LDT |
| Leading Digits | Entering digit to be dialed by subscriber |
| Look up | ALL : LCR function is applied to all Only EXT: applied only in case of extension Only TRK: applied only in case of trunk |
| D # T X | DMT index to be used for Time Zone X of Day Zone # |

Note 2 Default Data Add Set menu is used to make data addition easy. If you press the relevant menu, in the next item of the last data displayed on the screen, LDT item in which basic data is designated is automatically added, and you can change data into the desired type and then click Add and Send.

12.3	DMT (Digit Modification Table)

This screen is used to query, change and delete DMT to be used in LCR service.

[Figure  DMT(Digit Modification Table)

- How to use

1	Query

1)	In Query item, enter DMT index number to be queried.

2)	Click Query.

2	Add

1)	Designate in order of DMT index and Trunk Group to be used start location to be deleted  number of digits to be deleted  location of the digit to be added  digit to be added  DMT index to be selected next when the designated trunk is busy.

2)	Click Add.

3)	Click Send.

4)	Click Query.

3	Change

1)	Click the item to be changed.

2)	Change the data.(DMT Index cannot be changed)

3)	Click Change.

4)	Click Send

5)	Click Query.

- Contents

| Item | Contents |
| --- | --- |
| Query Condition | DMT Index No |
| Acc Code | When making a call to external number changed by the relevant DMT, trunk access code to be used |
| Del pointer | Start location to delete digit |
| Del Length | Length of digit to be deleted |
| Add pointer | Start location to add digit |
| Add Digits | Digit to be added |
| ARS DMT Index | When all designated trunks are busy, DMT index to be selected next |

13	Gain Control

13.1	Gain Value by Ports

- Functional description

This function is used to verify and change call gain for each extension/trunk.

[Sample Screen Description]

As a Gain level for each port of Shelf 1/LB4 subscriber, all initial values are “0” dB in this case.

[Figure  Port Gain Value]

- How to use

1	Query

1)	In Query condition, enter the range of Shelf / LB / Port.

For Shelf/LB/Port, enter all ranges of start and end.

2)	Click Query.

3)	Verify gain values.

2	Change

1)	Double click the item to be changed.

2)	Enter data to be changed.

3)	Click Change. The color of the relevant item is changed.

4)	Click Send.

5)	Click Query.

- Contents

| Item | Contents |
| --- | --- |
| Port No | Shelf / LB / Port |
| Ext H. Set Gain | Gain level when calling to extension subscriber (SLT, D-Tel, attendant) (0 ~ 14) |
| Ext S. Set Gain | Not in use |
| Trk H. Set Gain | Gain level when calling to trunk subscriber (0 ~ 14) |
| Trk S. Set Gain | Not in use |
| Ring Gain | Size of ring of D-Tel subscriber when call is terminated (0 ~ 14) |
| Tone Gain | Gain level when subscriber hears tone assigned to system (0 ~ 14) Dial Tone, Busy Tone and etc. |

※ Range of gain level in OMS is indicated as 0 ~ 14. (0:minimum, 14:maximum)

※ Gain level in D-Tel is indicated as 1~15.(1: minimum, 15: maximum)

※ Ring Gain is controlled only with D-Tel subscriber.

| OMS Gain level | Physical level of subscriber |
| --- | --- |
| 0 | - 12 dB |
| 1 | - 12 dB |
| 2 | - 10 dB |
| 3 | - 8 dB |
| 4 | - 7 dB |
| 5 | - 6 dB |
| 6 | - 5 dB |
| 7 | - 4 dB |
| 8 | - 3 dB |
| 9 | - 2 dB |
| 10 | - 1 dB |
| 11 | 0 dB : default |
| 12 | 2 dB |
| 13 | 3 dB |
| 14 | 4 dB |

13.2	S/W Gain Index

- Functional description

This screen is used to control call gain between groups by binding groups for control of subscriber Gain.

[Figure  S/W Value Index Table]

- How to use

1	Gain Index Query

1)	In Query Condition, enter location of the port to be queried.

2)	Click Gain Table and select with √.

3)	Click Query.

2	Gain Index Change

1)	Query with Gain Index Query.

2)	Click twice the G. Index No of the port to be changed.

3)	Enter G. Index No. (1 ~ 8): default of all subscribers is 1.

4)	Click Change. The color of the relevant item is changed.

5)	Click Send.

6)	Click Query to confirm data.

3	Gain Table Query

1)	Click Gain Table and select with √.

2)	Click Query.

4	Gain Table Change

1)	Query with Gain Table Query.

2)	Click twice the matrix to be changed.

3)	Enter Gain level. (0 ~ 14): default of all groups is 0.

4)	Click Change.

5)	Click Send.

6)	Click Query to verify data.

- Contents

1.	When connecting trunk and extension subscriber, in case of increasing call gain heard by extension subscriber

- Change G. Index No of trunk into 2.

- In Gain Table, change the value of “1” row of “2” column from 0 to 1.

- Extension subscriber can receive with gain level increased by 1dB.

2.	When connecting trunk and extension subscriber, in case of increasing call gain heard by trunk subscriber

- Change G. Index No of trunk into 2.

- In Gain Table, change the value of “2” row of “1” column from 0 to 1.

- Trunk subscriber can receive with gain level increased by 1dB.

※ Values of Gain Table are maximum values of gain levels that subscriber can change. That is, if the value is 2 in Gain Table, the maximum gain of subscriber is 2, not 0.

14	VOIP Data Management

14.1	VOIP Own Identification

- Functional description

This screen is used to set up ITGC data to access VoIP using ITGC.

[Figure  VOIP Own Identification]

- How to use

1	Query

1)	Enter the group No. (1 ~ 2) to be queried.

2)	Click Query.

2	Change

1)	Click the item and change data.

2)	Click Change.

3)	Click Send.

4)	Click Query to verify the data.

- Contents

| Item | Contents |
| --- | --- |
| Group No | There may exist a maximum of 2 VoIP groups, each of which includes an ITGC. Users can verify group numbers in [4.1 Trunk Information] ITGT Grp No of ITGC. |
| OWN IP | IP address assigned to ITGC |
| NetMask | SubNet mask of network connected to ITGC |
| Default Route IP | IP address to which outgoing calls that do not include called number prefixes to be designated are to be routed |
| Code Type ① G.723.1-53 ② G.723.1-63 ③ G.729A | Standard code type to convert voice signals into packet data ① Compression of voice into 5.3 Kbps ② Compression of voice into 6.3 Kbps ③ Compression of voice into 8 Kbps |
| Silence Compression | Whether or not background noises are included in case of voice compression |
| Frame/Packet Rate | Number of packets included in a frame |
| T301 Lock | Maximum waiting time from call origination to answer (unit : second) |
| T302 Lock | Maximum waiting time until receiving the status message from the called subscriber since the subscriber’s number is sent out (unit : second) |

14.2	VOIP O/G Prefix & Call Route

- Functional description

This screen is used to set up prefix digits and routes required to access VoIP using ITGC.

[Figure  VOIP O/G Prefix & Call Route]

- How to use

1	Query

1)	Enter the number of the group (1~2) to be queried.

2)	Click Query.

2	Add

1)	Enter the number of the group (1~2) to be added.

2)	Click Query.

3)	Click the position where the group is to be added and press the down arrow.

4)	Enter O/G prefix and route IP.

5)	Click Add.

6)	Click Send.

7)	Click Query.

3	Change

1)	Enter the number of the group (1~2) to be changed.

2)	Click Query.

3)	Double-click the data to be changed and change them.

4)	Click Change.

5)	Click Send.

6)	Click Query.

- Contents

| Item | Contents |
| --- | --- |
| Group No. | Same as Group No of [14.1 VOIP Own Identification] |
| O/G Prefix | Prefix of outgoing number e.g.) Meaning of “1” : Dial the ITGC trunk calling code and, if the dialed external number starts in “1”, the call is sent to the route IP. |
| Route IP | Termination IP address of outgoing call |

※ The maximum number of 50 O/G prefixes may be assigned to a VOIP group.

15.  VPM Data

- 15.1  Voice Tone

- Functional description

The Menu for changing or query of the related information in order to replace all tome used at the system with the audio Token ID supported by VPM board.

[Figure  Voice Tome Information]

- How to use

1  Query

1) Click Query .

2  Change

1) Click the item to change.

2) Change data.

3) Clicke Change .

4) Click Send .

5) Click Query

- Contents

| Item | Contents |
| --- | --- |
| VPM Token ID | A audio Tokens supported by VPM board. |
| System Tone ID | The temporary System TONEs which Caller can hear during VPM TOKEN are QUEUEing for service. |

- 15.2  VPM Token

- Functional description

This menu is used in order to Change or Query in detail about a VPM Token supported by VPM board.

[Figure  VPM Token Information]

- How to use

1	Query

1)	Click Query.

2	Change

1) Click the item to change.

2) Change the data .

3) Clicke Change .

4) Click Send .

5) Click Query

- Contents

| Item | Contents |
| --- | --- |
| VPM Token ID | ID of VPM Tokens supported by VPM board |
| Recording Time | The recorded time of that Token. |
| Start Port No | The start port number to be used When service is doing. |
| End Port No | The last port number to be used When service is doing |

15.3	External VMS PN Management

- Functional description

The menu is used in order to change or query voice channel which is used for a syschronization between a voice channel and message. Under the case that External VMS device is used  combined with CS system.

[ Figure  External VMS PN Management ]

- How to use

1	Query

1)	Click Query.

2	Change

1) Click the item to change.

2) Change the data .

3) Clicke Change .

4) Click Send .

5) Click Query

- Contents

| Item | Contents |
| --- | --- |
| PN No | Sequence number of a voice channel |
| Tel No | Telephone number of a voice channel |

15.4	External VMS PN Management

- Functional description

This menu is used to change or query a Voice mail Box ID informaton which will be used at CS system.

[Figure  Mail Box Management]

- How to use

1  Query

1)  Decide that the type is VPM Type or Exteranl Type.

2)  In case that the type is VPM, assign the Shelf Number and Slot Number.

3)  Click Query

2  Individual  detail  Quary

1)  Click Query

2)  Select the item among the displayed items.

3)  Click Query at the Sub-window.

3  Add

1)  Decide that the type is VPM Type or Exteranl Type.

2)  Select “Append Tool” with right button of mouse.

3)  In case that the type is VPM, assign the Shelf Number and Slot Number.

4)  Input the voice mail Box ID

5)  Input the unit

6)  Click the Append OK button

7)  Click Close

8)  Click Send

9)  Click Query

4  Change

1)  Click the item to change

2)  Change the data

3)  Click Change

4)  Click Send

5)  Click Query

5  Individual Detail Information Change

1)  Execute the “Individual Detail Query”

2)  Change the item among the displayed items

- The changible items at the internal VPM  : Unit

- The changible items at the external VPM  : Password,Unit,External Telno

3)  Click Change at the sub-window.

4)  Click Send

5)  Click Query at the sub-window.

6  Individual Passward Delet

1)  Execute the “Individual Detail Query”

2)  Click PWD Clear

3)  Click Send

4)  Click Query at the sub-window.

7  Delete

1)  Decide that the type is VPM Type or Exteranl Type.

2)  In case that the type is VPM, assign the Shelf Number and Slot Number.

3)  Click the item to delete

4)  Click Send

- Contents

| Item | Contents | Note |
| --- | --- | --- |
| VPM | Select this item , in case of using the Internal VPM board |  |
| Shelf/Slot No | Assign the position of the relavant Board. In case of Internal VPM , one more board can be used. |  |
| External VMS | Select this item , in case of using External VMS combined with CS system. |  |
| MB ID | Voice mail Box ID must be Extension number. |  |
| Password | The password of the relevant voice mail Box ID |  |
| Unit(min) | The maximum number of voice mail which subscriber can record. |  |
| External Telno | The external telephone number of a mail Box ID(internal number), this is used at the call forwarding service. | External Only |
| Reg. Msg. Count | The number of a voice mail within the current mail Box ID |  |

15.5	Voice Mail Option

- Functional description

This menu is used in order to query or change the system options for a voice mail service.

[Figure  Voice Mail Option]

- How to use

1	Query

1)	Click Query.

2	Change

1) Click the item to change.

2) Change the data .

3) Clicke Change .

4) Click Send .

5) Click Query .

- Contents

| Item | Contents | Note |
| --- | --- | --- |
| VPM Bulletin Service Menu | The digit number used at menu selection , When bulletin service. | Internal only |
| VPM Bulletin Service | Assign whether to use bulletin service or not. | Internal only |
| Whether use CFWD to VM in case NO Answer . | Decide whether use CFWD to VM in case that there is no answer for Trunk incoming call. |  |
| Whether use CFWD to VM in case Busy | Decide whether use CFWD to VM in case that the subscriber is on the telephone for Trunk incoming call. |  |
| Whether send Alert Tone to the subscriber who VM registers | Currently not used. |  |

16	Miscellaneous Information

16.1	Special Phone

- Functional description

This screen is used to query, add, change and delete subscribers such as hot line, outside hot line, warm line and house phone.

[Sample Screen Description]

1	Hot line

The extension subscribers “1000” and “1002” are connected to each other through the hot line.

2	Outside hot line

There is an outside hot line between the extension subscriber “1001” and an external subscriber whose trunk code and external number are ‘9’ and ‘03434507354’, respectively.

3	Warm line

If the extension subscriber “1005” does not dial in a given period of time hooking off the handset, another extension subscriber “2002” is called by the function.

4	House phone

When the extension subscriber “2003” hooks off the handset, another subscriber “2001” is called by the function.

[figure  Special Phone]

- How to use

1	Query

1)	Click Select Stat of an item to be queried

2)	Click Query.

(In this case, the table in which √ Select Stat is not selected is not queried or added.)

2	Add : Hot Line (All other functions are added in the same way.)

1)	Click the item to be added.

2)	Enter the extension numbers of the two extension subscribers to set up hot line over.

3)	Click Add.

4)	Click Send.

5)	Click Query.

3	Change

1)	Click an item to be changed.

2)	Change the data.

3)	Click Change.

4)	Click Send.

5)	Click Query.

4	Delete

1)	Click an item to be deleted.

2)	Click Delete.

3)	Click Send.

4)	Click Query.

- Contents

| Item |  | Description |
| --- | --- | --- |
| Hot line | Extension No 1 | Outgoing extension number under hot line active |
|  | Extension No 2 | Incoming extension number under hot line active |
| Outside hot line | Extension No | Outgoing extension number under outside hot line active |
|  | Trunk Code | Trunk access code |
|  | O/G Digit | Incoming number under outside hot line active |
| Warm line | Warm Line | Outgoing extension number under warm line active |
|  | Response No | Incoming extension number under warm line active |
| House phone | House Phone | Outgoing extension number under house phone active |
|  | Response No | Incoming extension number under house phone active |

※ Extension subscribers can designate only one of the functions, hot line, outside hot line, warm line and house phone to use.

16.2	Emergency Telephone Number

- Functional description

This function is used to verify, add and delete emergency telephone codes to be processed prior to other calls.

[Sample Screen Description]

In the following sample, an extension subscriber dials “114”, gets connected to a trunk using the trunk number “72” and calls the external number “114”.

[Figure  Emergency Telephone Number Information]

- How to use

1	Query

1)	Click Query.

2	Add

1)	Click the last item.

2)	Create an data row to be added using the direction key ↓.

3)	Enter an emergency number and trunk access code to be added.

※ Note: Emergency numbers shall be so unique as to be distinct from extension numbers, trunk numbers and function codes.

4)	Click Add.

5)	Click Send.

6)	Click Query.

3	Delete

1)	Click the item to be deleted.

2)	Click Delete.

3)	Click Send.

4)	Click Query.

- Contents

| Item | Description | Remark |
| --- | --- | --- |
| Emergency Tel No | Emergency number |  |
| Access code | Trunk access code used by emergency number |  |

16.3	Night Bar

- Functional description

This screen is used to verify and change the property of night bar by tenants.

[Figure  Night Bar]

- How to use

1	Query

1)	Click Query.

2	Change

1)	Click the tenant number to be changed.

2)	Change mode type, time information and night bar mode (activation/deactivation).

3)	Click Change.

4)	Click Send.

5)	Click Query.

- Contents

| Item | Description |
| --- | --- |
| Ten No | Tenant number designated |
| Day Mode | Day call termination mode |
| Night Mode | Night call termination mode |
| Semi Night Mode | Semi night mode |
| Holiday Mode | Holiday mode |
| D Time (HH:MM) | Start time of day mode |
| N Time (HH:MM) | Start time of night mode |
| Use Night Bar | Activation/deactivation of night bar |

※ Mode type: This applies to trunk calls only.

| Mode | Description |
| --- | --- |
| ATD Mode | Attendant call |
| Loud Bell (UNA) | Call termination to loud bell subscribers defined in [16.6 Loud Bell] |
| Announcement Mode | Hearing of announcement by routes in [16.9 Announcement Line] |
| PNA Mode | Call termination to PNA subscribers defined in [16.4 System PNA Service] |
| No Service | Mode free service. |

16.4	System PNA Service

- Functional description

This screen is used to verify and change PNA subscribers and operation method.

※ Predetermined Night Answer (PNA): This is a night service which has incoming calls to trunks or operators in the night service mode automatically transferred to predetermined subscribers.

[Sample Screen Description]

The extension subscriber “1000” is designated as a system PNA subscriber of the tenant 1.

[Figure  System PNA]

- How to use

1	Query

1)	Click Query.

2	Change

1)	Select the tenant number to be changed with the mouse.

2)	Change relevant PNA numbers. ( User doesn’t neet to input the Tel No Type, it registered automatically )

3)	Click Change.

4)	Click Send.

5)	Click Query.

References

1)	System PNA subscriber [16.4 System PNA Service]

2)	Calling group [6.4 Calling Group]

3)	Trunk-to-trunk direct access allow/restrict [4.12 Trunk Rout-Rout Connection Allow Option]

4)	Abnormal trunk call processing option in day/night mode

[4.10 Day/Night Call Processing Option by Trunk Routes]

5)	Attendant night transfer [5.7 Attendant Night Transfer]

6)	DIL [6.2 DIL Hunting Group]

7)	DDI divert [6.9 Overall Option by Trunk Routes] – DDI divert access item

8)	Call termination types by system modes [16.4 Night Bar]

- Contents

| Item | Description |  |
| --- | --- | --- |
| Tenant No | Designated tenant number |  |
| PNA Option | Currently not used |  |
| Tel No Type | Ext Tel No | PNA number designated to extension subscriber |
|  | Grp Tel No | PNA number designated as representative number of calling group |
| PNA Tel No | PNA telephone number |  |

16.5	One Digit Busy Service

- Functional description

The users can use the function defined by each digit by pressing the digit while the other extension subscriber’s line is busy. This function is unavailable while step call is active.

[Sample Screen Description]

The digit '4' is registered as the function “Call Back” and, if the other extension subscriber’s line is busy, the calling subscriber can access call back by pressing the digit '4'.

[Figure  One Digit Busy Service]

- How to use

1	Query

1)	Click Query.

2)	Functions defined by digits are queried.

2	Change

1)	Designate functions by digits by selecting relevant data using the mouse.

2)	Click Change.

3)	Click Query.

- Contents

| Item |  | Description |
| --- | --- | --- |
| Digit Item |  | Designated dial digit list (0,1,2,…9, *, #) |
| Feature | Not Used | Not used |
|  | Group Hunting | Group hunting |
|  | Override | Override |
|  | Call Back | Call back |
|  | Call Wait | Call wait |

※ This function is unavailable while step call is active.

[2.6 System Function Option] – Refer to the Step call.

16.6	Loud Bell

- Functional description

This screen is used to verify and change loud bell subscribers by tenants.

[Figure  Loud Bell Information]

- How to use

1	Query

1)	Click Query.

2	Change

1)	Select the tenant numbers to be changed with the mouse.

2)	Enter loud ports to be used by tenants.

※A tenant accommodates just a loud bell port.

3)	Click Change.

4)	Click Send.

5)	Click Query.

- Contents

| Item | Description | Remark |
| --- | --- | --- |
| Tenant | Designated tenant number |  |
| Loud Bell Port No | Loud bell port number |  |

16.7	System Speed Dial

- Functional description

This function is used to verify, change, add and delete system speed dial codes.

[Sample Screen Description]

1)	According to the numbering plan, the system speed dial code 1 is designated as 6XX

2)	The speed dial code “600” connects to the trunk code “9” and then calls the outgoing number “024567895”.

3)	The speed dial code “600” is accessible to Class “I” or higher extension subscriber.

4)	The speed dial code “600” is a chargeable service.

5)	The speed dial code “600” is available to the system speed dial code user group “1” only.

[Figure  System Speed dial Information]

- How to use

1	Query

1)	Enter the first digit of the speed dial code to be queried.

e.g.) Enter “6” if the speed dial code is 6XX.

e.g.) Enter “60” ~ “69” if the speed dial code is 6XXX.

2)	Click Query.

3)	Up to 100 speed dial codes may be queried.

2	Add

1)	Click an item or make create a data row to be added using the direction key↓.

2)	Add a new speed dial code.

3)	Enter the speed dial code followed by trunk access code, outside number and allow class.

4)	Click the Speed Dial Information.

5)	Click the Using Group Information.

6)	Click Add.

7)	Click Send.

8)	Click Query.

3	Change

1)	Click the speed dial code to be changed.

2)	Change the data of the code.

3)	Click Change.

4)	Click Send.

5)	Click Query.

4	Delete

1)	Click the speed dial code to be deleted.

2)	Click Delete.

3)	Click Send.

4)	Click Query.

- Contents

| Item | Description |
| --- | --- |
| Speed Code | Three or four digit speed dial code |
| Trunk Code | Trunk code |
| OutSide Telno | External subscriber number |
| OutSide Subscriber Name | External subscriber ID by the number |
| Allow Class | Minimum extension subscriber class to access the speed dial code to use |
| Charge | Indication of whether or not the speed dial code to be used is chargeable |
| Append Digit | Digit extension option of speed dial code dialed (Note1) |
| Emergency Code | Unavailable |
| Speed Dial Information | Extension subscriber group eligible to access the speed dial code to use (Note 2) |

(Note 1) If there are external subscribers accessing DDI (DID), the function allows the calling subscriber to enter DDD code followed by prefix without entering the end subscriber’s number in Outside Telno and dial the remaining number.

e.g.) If the end subscriber’s number is “02450XXXX”, the calling subscriber can enter “02450” in Outside Telno and dial the number “XXXX”.

(Note 2) Once a user clicks the right button of the mouse pointing at “Using Group information” (Under part of the system speed dial Informaiton Window). group number “1 ~ 24”, the items of [System Speed using Ext Group] and [Using Speed Code per EXT Group] pops u p.

System speed using Ext group

The extension subscriber numbers in the speed code group are listed and users can query, add and delete extension subscribers.

[Figure  System Speed Using Ext Group]

Using speed code per EXT group

This screen is used to view the list of speed dial codes that the selected groups can access.

[Figure  Using Speed Code per Ext Group]

16.8	BGM Channel

- Functional description

This function is used to verify, add and delete background music ports. The ports are available up to 10.

[Figure  Back Ground Music Channel]

- How to use

1	Query

1)	Click Query.

※ Then, background music serial number and their send-out ports are queried.

2	Change

1)	Enter the number background serial number and its send-out port to be changed. Up to 10 ports are available and the numbers 1 ~ 10 are used as serial number.

1)	Click Change.

2)	Click Send.

3)	Click Query.

3	Delete

1)	Select the data to be deleted with the mouse.

1)	Click Delete.

2)	Click Send.

3)	Click Query.

- Contents

| Item | Description |
| --- | --- |
| BGM Serial No | Background music channel serial number |
| BGM Port No | Port number assigned to background music channel serial number |

16.9	Announcement Line

- Functional description

This screen is used to set up the function of playing relevant voice announcements for subscriber according to functions used, instead of the tones provided by the system.

[Figure  Announcement Line]

- How to use

1	Query

1)	Click Query.

※ If there are no data stored, the error message “No Data” is displayed.

2	Change

1)	Select the data to be changed with the mouse.

2)	Enter the route number in Assign Target (as shown in the window) and the port number of ASLC board in Announcement Port No (Shelf / LB / Port).

3)	Click Change.

4)	Click Send.

5)	Click Query.

- Contents

| Item | Description |
| --- | --- |
| Announcement Line Serial No | Announcement line serial number. Numbers after 8 shall be assigned to general announcements. |
| Assign Target | No entry |
| Announcement Port No | Shelf/LB/Port of ASLC. Regarding this type of port, it is possible to enter announcement port numbers without subscriber numbers. |

16.10	Linked Station Pair

- Functional description

This function allows two extension subscribers who are connected to each other to access services as if they are one subscriber.

[Sample Screen Description]

The extension subscribers “2001” and “2000” are designated as a linked pair.

[Figure  Linked Station Fair]

- How to use

1	Query

1)	Click Query.

2	Add

1)	Create a data row to be added using the direction key.

2)	Enter the base station number followed by the extension numbers to apply linked station pair to.

3)	Click Add after entering all relevant data.

4)	Click Send.

5)	Click Query.

3	Delete

1)	Select the data to be deleted with the mouse.

2)	Click Delete.

3)	Click Send.

4)	Click Query.

- Contents

| Item | Description |
| --- | --- |
| Base Station Tel No | Master extension number of linked station pair |
| Linked Station Tel No | Slave extension number of linked station pair |

※Terminals that linked station pair is available to

| Linked Base | SLT | D-Tel | DECT |
| --- | --- | --- | --- |
| SLT | √ | √ | √ |
| D-Tel | √ | √ | √ |
| DECT |  |  |  |