## Net Warm Line

### General Description

This function enables the extension belonging in master system or slave system 1, 2 to make a call to the pre-assigned extension by picking up a handset for a certain period of time only. Under normal condition, the extension can make a call by dialing the number within the specific period of time after picking up a handset.

### Operating Procedure

- An extension A lifts the handset.

- When a certain period of time elapses while the extension A hears a dialing tone, a call is automatically attempted to the pre-assigned extension B and the extension A hears a ring back tone.

- The call is established when the extension B answers the call.

### Service Conditions

- The calling extension should be assigned as the Warm Line extension.

### Data Generation

- To assign a warm line

#### [16.1 Special Phone ⇒ Warm Line]

- To check whether the warm line is assigned

#### [3.1 Extension Information – Extension Characteristic ⇒ 2 warm Line]

- To set the time period after that the automatic dialing is made

#### [2.4 System tone parameter ⇒ Warm Line Dial Tone]

### Interactions

- One extension can be assigned as only one of the Hot Line/House Phone/Warm Line.