LG PBX System

- Software Technical Information

STI-0069

July 09, 2001

| Заголовок | #### Инструкция по установке платы VPM |
| --- | --- |
| УПАТС | CS1000 |
| Аннотация | Инструкция по установке платы голосовых сообщений устанавлеваемой в УПАТС и ее функционирование для обеспечения сервиса IVR (Интерактивное речевое взаимодействие: автооператор), сервиса VMS (голосовая почта), замещения системных тонов системы, голосового меню. |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 11.30.2000 | H.G Jung | First issue. |
| 1.1 | 05.10.2001 | H.G Jung | Third issue. |
| 1.2 | 07.09.2001 | H.G Jung | IVR service added issue. |
| 1.3 | 09.09.2001 | soo | User Defined Token issue |

Содержание

Аннотация	4

Сервис IVR	4

Голосовая почта	4

Ситемные тоны	4

Голосовое меню	5

Сообщения платы VPM	6

Тип 1 (Фиксированные)	6

Тип 2 (Определяются пользователями станции)	6

Тип 3 (Голосовое меню)	6

Установка платы VPM	7

Список фиксированных сообщений	7

Список фиксированных сообщений	7

Сообщения об ошибках	9

Месяцы	11

Дни	11

Часы	12

Минуты	13

Числа	14

Генерация базы данных для платы VPM программой OMS.	15

Генерация платы VPM	15

Присвоение номеров портам платы VPM	15

Установка программного обеспечения платы VPM	16

Список файлов	16

Генерация и компиляция сообщений, определяемых пользователями	16

Загрузка файлов	17

Service Type	26

VMS	26

Description	26

Service Method	26

Service Scenario	30

Data Management	35

Correlation	36

IVR(Interactive Voice Response : Auto Attendant)	37

Description	37

Service Method.	37

Service Scenario	38

Data Management	40

Correlation	40

Voice Tone	41

Description	41

Installation	41

Service Method	41

Data Management	43

Correlation	43

Voice Bulletin	44

Description	44

Setup Procedure	44

Service Method	45

Service Scenario	46

Data Management	48

Correlation	48

Bulletin Message Checkup	48

Trouble Shooting Guide	50

Заголовок: Установка платы VPM

# Аннотация

Плата голосовых сообщений устанавлеваемая в УПАТС предназначена для обеспечения сервиса IVR (Интерактивное речевое взаимодействие: автооператор), сервиса VMS (голосовая почта), замещения системных тонов системы, голосового меню.

## Сервис IVR

Плата VPM отвечает на входящий вызов по соединительной линии (СЛ) или от абонента станции при наборе вызывающим абонентом одного из номеров автооператора, назначенного в системе и выдает сообщение. Коммутация входящего вызова происходит в зависимости от действий абонента (набираемые в тональном режиме цифры) и заранее запрограммированных сценариев.  В зависимости от набираемых цифр возможны варианты перевода вызова:

1. В другие таблицы обработки вызова (CCR)

2. На внутренний номер абонента станции

3. Вызов в группу вызова

4. Пилотный номер группы  автоматического распределения вызова (АРВ)

5. Ячейка системного скоростного набора

6. Ячейка индивидуального скоростного набора предопределенного абонента

7. Номер сети Q-sig

8. Вызов пульта оператора

9. Вызов на сигнализирующее устройство, при переводе в системы в ночной режим (UNA)

- Каждая таблица CCR имеет до трех сообщений включительно. Каждая таблица CCR имеет свой собственный сценарий обработки вызовов для дневного/ночного режимов. Каждая таблица CCR имеет собственное время ожидания приема цифра в тональном режиме после сообщения. См. также файл

- An operator may use CCR Table within the limit of the max resources available in the system in a flexible way.

## Голосовая почта

Интерфейс голосовой почты позволяет записывать, прослушивать и стирать сообщения оставленные вызывающим абонентом, если вызываемый абонент активизировал данный сервис на своем телефоне. Каждый абонент, который может переводить вызовы в почтовый ящик может создать свое приветствие.

## Ситемные тоны

Данный сервис позволяет замещать различные тональные сигналы из платы CPM.

## Голосовое меню

Данный сервис позволяет абоненту, набравшему соответствующий номер получать различную информацию,

которая записывается непосредственно абонентами станции с телефонного аппарата.

# Сообщения платы VPM

Сообщения платы VPM подразделяются на три вида, представленных ниже.

## Тип 1 (Фиксированные)

Данные сообщения используются для сервиса IVR, VMS и речевого меню. Они создаются из файлов формата wav и преобразуются в файлы vpidx.hex & vpdat.hex и загружаются в плату VPM. Сообщения компилируются фирмой-производителем.

Время одного сообщения- не ограничено, но суммарное время сообщений не должно превышать 30 минут.

## Тип 2 (Определяются пользователями станции)

Сообщения второго типа используются для сервиса IVR и замещения системных тонов. Сообщения данного типа записываются в wav-формате системным оператором и компилируются соответствующей программой в два файла для загрузки в плату VPM: usridx.hex & usrdat.hex. Время записи одного сообщения- не ограничено, однако, суммарное время не должно превышать 25 минут. Максимальное количество сообщений данного типа- 50.

## Тип 3 (Голосовое меню)

Данный тип сообщений записывается непосредственно абонентами станции с телефонного аппарата. Максимальное время одного сообщения- 1минута.

# Установка платы VPM

## Список фиксированных сообщений

LGE поставляет  фиксированные сообщения на различных языках, которые преобразованы в файлы vpidex.hex & vpdat.hex для загрузки в плату VPM. При смене языка, на котором записаны данные сообщения,  возможно необходима смена программного обеспечения платы VPM.

### Список фиксированных сообщений

Интерактивные сообщения

| No. | Сообщения |
| --- | --- |
| VP100000 | Нажмите кнопку «один», если вы хотите оставить сообщение. Нажмите кнопку «три», если вы хотите набрать номер абонента. Нажмите кнопку «четыре» для перехода в информационное меню. |
| VP100001 | Нажмите кнопку «один», если вы хотите оставить сообщение. Нажмите кнопку «три», если вы хотите набрать номер абонента. |
| VP100002 | Нажмите кнопку «один», если вы хотите оставить сообщение. |
| VP100003 | Нажмите кнопку «три», если вы хотите набрать номер абонента. Нажмите кнопку «четыре» для перехода в информационное меню. |
| VP100004 | Нажмите кнопку «три», если вы хотите набрать номер абонента. |
| VP110000 | Извините. |
| VP110001 | Введите внутренний номер. |
| VP110002 | Соединяю. |
| VP110003 | Пожалуйста, оставьте ваше сообщение после сигнала. Нажмите кнопку 'Звездочка' по окончании сообщения. |
| VP110004 | Ваше сообщение записано. |
| VP110005 | Благодарим за звонок. |
| VP110006 | Ваше сообщение аннулировано. |
| VP110007 | Пожалуйста, подождите. |
| VP110008 | Попробуйте позже. |
| VP110009 | Для вызова секретаря нажмите '0'. |
| VP110010 | Для соединения с абонентом нажмите кнопку 'Звездочка' и введите внутренний номер. |
| VP130000 | Введите номер вашего почтового ящика. |
| VP130001 | Введите пароль из четырех цифр. |
| VP130002 | Для прослушивания вашего сообщения нажмите 'один', для входа в меню голосовой почты нажмите 'четыре'. |
| VP130003 | Для повторного прослушивания нажмите 'ноль', Для выхода нажмите кнопку 'звездочка'. Press “0” to listen this menu again or press “*” to quit. |
| VP131000 | 0.5 с - тишина |
| VP131001 | 0.5 с - тишина |
| VP131002 | 0.5 с - тишина |
| VP131003 | Сообщение записано в: |
| VP131004 | Для прослушивания следующего сообщения нажмите 'один', для повторного прослушивания нажмите 'два', для удаления нажмите 'восемь'. |
| VP131005 | Это сообщение удалено. |
| VP131006 | В вашем почтовом ящике больше нет сообщений. |
| VP131007 | В вашем почтовом ящике нет сообщений. |
| VP131008 | Извините, голосовая память переполнена. |
| VP131009 | 0.5 с - тишина |
| VP131010 | 0.5 с - тишина |
| VP131011 | Для возврата в основное меню нажмите 'ноль’. Для выхода нажмите кнопку «звездочка». |
| VP132201 | Запишите приветствие после сигнала. Нажмите кнопку 'звездочка' по окончании. |
| VP132202 | Приветствие не зарегистрировано. |
| VP133000 | Чтобы сменить пароль нажмите 'один'. Чтобы прослушать ваше приветствие нажмите 'два'. |
| VP133001 | Чтобы сменить пароль нажмите 'один'. Чтобы прослушать ваше приветствие нажмите 'два'. Чтобы зарегистрировать отсутствие нажмите 'три'. |
| VP133002 | Для возврата в предыдущее меню нажмите 'ноль'. Для выхода нажмите кнопку 'звездочка'. |
| VP133101 | Введите новый пароль из четырех цифр. |
| VP133102 | Изменения подтверждены. |
| VP133200 | Для регистрации нажмите 'один'. Для подтверждения нажмите 'два'. Для удаления нажмите 'восемь'. |
| VP133300 | Для регистрации нажмите 'один'. Для отмены нажмите 'два' |
| VP133301 | Регистрация произведена. |
| VP133302 | Отмена произведена. |
| VP140000 | Информационное меню. |
| VP140001 | Информационное меню не имеет сообщений |
| VP140002 | Для записи нажмите 'один'. Для подтверждения нажмите 'два'. Для удаления нажмите 'восемь'. |
| VP140003 | Наберите номер информационного меню, которое вы хотите записать или стереть. |
| VP140004 | Для выхода нажмите кнопку 'звездочка'. |
| ABS001 | Абонент не отвечает. |
| ABS002 | Линия занята. |
| ABS003 | Абонент не может ответить на ваш вызов. |
| ABS004 | Абонент отсутствует на рабочем месте. |
| ABS005 | Обед. |
| ABS006 | Абонент в отпуске. |
| ABS007 | Абонента нет в офисе. |
| ABS008 | Абонент в офисе, но в данный момент- недоступен. |
| ABS009 | Абонент на совещании. |
| ABS010 | Абонент будет завтра. |
| ABS011 | Абонент в командировке. |
| ABS012 | Введен несуществующий номер. |
| ABS013 | Этот номер не может быть вызван. |
| ONC00 | У вас нет сообщений |
| ONC01 | У вас одно сообщение |
| ONC02 | У вас два сообщения |
| ONC03 | У вас три сообщения |
| ONC04 | У вас четыре сообщения |
| ONC05 | У вас пять сообщений |
| ONC06 | У вас шесть сообщений |
| ONC07 | У вас семь сообщений |
| ONC08 | У вас восемь сообщений |
| ONC09 | У вас девять сообщений |
| ONC10 | У вас десять сообщений |
| ONC11 | У вас одиннадцать сообщений |
| ONC12 | У вас двенадцать сообщений |
| ONC13 | У вас тринадцать сообщений |
| ONC14 | У вас четырнадцать сообщений |
| ONC15 | У вас пятнадцать сообщений |
| ONC16 | У вас шестнадцать сообщений |
| ONC17 | У вас семнадцать сообщений |
| ONC18 | У вас восемнадцать сообщений |
| ONC19 | У вас девятнадцать сообщений |
| ONC20 | У вас двадцать сообщений |

### Сообщения об ошибках

| No. | Сообщения |
| --- | --- |
| E00001 | Вы ввели неверный номер. |
| E00002 | Пожалуйста, воспользуйтесь инструкцией и позвоните еще раз. |
| E00003 | Ваш запрос принят. Подождите. |
| E00004 | Ваш запрос отложен. Попробуйте позже. |
| E00005 | Система требует сервиса. Попробуйте позже. |
| E00006 | Извините, система перегружена. |
| E00007 | Система неисправна, попробуйте позже. |
| E00008 | Пожалуйста, проверьте пароль и попробуйте еще раз. |
| E00009 | Вы ввели неверные данные. |
| E00010 | Сообщение прослушивается другим абонентом. Вы не можете удалить его. |

### Месяцы

| No. | Сообщение | No. | Сообщение |
| --- | --- | --- | --- |
| MON00 | Месяц |  |  |
| MON01 | Января | MON07 | Июля |
| MON02 | Февраля | MON08 | Августа |
| MON03 | Марта | MON09 | Сентября |
| MON04 | Апреля | MON10 | Октября |
| MON05 | Мая | MON11 | Ноября |
| MON06 | Июня | MON12 | Декабря |

### Дни

| No. | Сообщение | No. | Сообщение |
| --- | --- | --- | --- |
| DAY00 | День | DAY16 | Шестнадцатое |
| DAY01 | Первое | DAY17 | Семнадцатое |
| DAY02 | Второе | DAY18 | Восемнадцатое |
| DAY03 | Третье | DAY19 | Девятнадцатое |
| DAY04 | Четвертое | DAY20 | Двадцатое |
| DAY05 | Пятое | DAY21 | Двадцать первое |
| DAY06 | Шестое | DAY22 | Двадцать второе |
| DAY07 | Седьмое | DAY23 | Двадцать третье |
| DAY08 | Восьмое | DAY24 | Двадцать четвертое |
| DAY09 | Девятое | DAY25 | Двадцать пятое |
| DAY10 | Десятое | DAY26 | Двадцать шестое |
| DAY11 | Одиннадцатое | DAY27 | Двадцать седьмое |
| DAY12 | Двенадцатое | DAY28 | Двадцать восьмое |
| DAY13 | Тринадцатое | DAY29 | Двадцать девятое |
| DAY14 | Четырнадцатое | DAY30 | Тридцатое |
| DAY15 | Пятнадцатое | DAY31 | Тридцать первое |

### Часы

| No. | Сообщение | No. | Сообщение |
| --- | --- | --- | --- |
| HH00 | Ноль часов |  |  |
| HH01 | Один час | HH13 | Тринадцать часов |
| HH02 | Два часа | HH14 | Четырнадцать часов |
| HH03 | Три часа | HH15 | Пятнадцать часов |
| HH04 | Четыре часа | HH16 | Шестнадцать часов |
| HH05 | Пять часов | HH17 | Семнадцать часов |
| HH06 | Шесть часов | HH18 | Восемнадцать часов |
| HH07 | Семь часов | HH19 | Девятнадцать часов |
| HH08 | Восемь часов | HH20 | Двадцать часов |
| HH09 | Девять часов | HH21 | Двадцать один час |
| HH10 | Десять часов | HH22 | Двадцать два часа |
| HH11 | Одиннадцать часов | HH23 | Двадцать три часа |
| HH12 | Двенадцать часов | HH24 | Двадцать четыре часа |

### Минуты

| No. | Сообщение | No. | Сообщение |
| --- | --- | --- | --- |
| MM00 | Ноль минут |  |  |
| MM01 | Одна минута | MM31 | Тридцать одна минута |
| MM02 | Две минуты | MM32 | Тридцать две минуты |
| MM03 | Три минуты | MM33 | Тридцать три минуты |
| MM04 | Четыре минуты | MM34 | Тридцать четыре минуты |
| MM05 | Пять минут | MM35 | Тридцать пять минут |
| MM06 | Шесть минут | MM36 | Тридцать шесть минут |
| MM07 | Семь минут | MM37 | Тридцать семь минут |
| MM08 | Восемь минут | MM38 | Тридцать восемь минут |
| MM09 | Девять минут | MM39 | Тридцать девять минут |
| MM10 | Десять минут | MM40 | Сорок минут |
| MM11 | Одиннадцать минут | MM41 | Сорок одна минута |
| MM12 | Двенадцать минут | MM42 | Сорок две минуты |
| MM13 | Тринадцать минут | MM43 | Сорок три минуты |
| MM14 | Четырнадцать минут | MM44 | Сорок четыре минуты |
| MM15 | Пятнадцать минут | MM45 | Сорок пять минут |
| MM16 | Шестнадцать минут | MM46 | Сорок шесть минут |
| MM17 | Семнадцать минут | MM47 | Сорок семь минут |
| MM18 | Восемнадцать минут | MM48 | Сорок восемь минут |
| MM19 | Девятнадцать минут | MM49 | Сорок девять минут |
| MM20 | Двадцать минут | MM50 | Пятьдесят минут |
| MM21 | Двадцать одна минута | MM51 | Пятьдесят одна минута |
| MM22 | Двадцать две минуты | MM52 | Пятьдесят две минуты |
| MM23 | Двадцать три минуты | MM53 | Пятьдесят три минуты |
| MM24 | Двадцать четыре минуты | MM54 | Пятьдесят четыре минуты |
| MM25 | Двадцать пять минут | MM55 | Пятьдесят пять минут |
| MM26 | Двадцать шесть минут | MM56 | Пятьдесят шесть минут |
| MM27 | Двадцать семь минут | MM57 | Пятьдесят семь минут |
| MM28 | Двадцать восемь минут | MM58 | Пятьдесят восемь минут |
| MM29 | Двадцать девять минут | MM59 | Пятьдесят девять минут |
| MM30 | Тридцать минут | MM60 | Шестьдесят минут |

### Числа

| No. | Сообщение | No. | Сообщение |
| --- | --- | --- | --- |
| CN000 | Ноль | ON000 | Нулевое |
| CN001 | Одно | ON001 | Первое |
| CN002 | Два | ON002 | Второе |
| CN003 | Три | ON003 | Третье |
| CN004 | Четыре | ON004 | Четвертое |
| CN005 | Пять | ON005 | Пятое |
| CN006 | Шесть | ON006 | Шестое |
| CN007 | Семь | ON007 | Седьмое |
| CN008 | Восемь | ON008 | Восьмое |
| CN009 | Девять | ON009 | Девятое |
| CN010 | Десять | ON010 | Десятое |
| CN011 | Одиннадцать | ON011 | Одиннадцатое |
| CN012 | Двенадцать | ON012 | Двенадцатое |
| CN013 | Тринадцать | ON013 | Тринадцатое |
| CN014 | Четырнадцать | ON014 | Четырнадцатое |
| CN015 | Пятнадцать | ON015 | Пятнадцатое |
| CN016 | Шестнадцать | ON016 | Шестнадцатое |
| CN017 | Семнадцать | ON017 | Семнадцатое |
| CN018 | Восемнадцать | ON018 | Восемнадцатое |
| CN019 | Девятнадцать | ON019 | Девятнадцатое |
| CN020 | Двадцать | ON020 | Двадцатое |

## Генерация базы данных для платы VPM программой OMS.

### Генерация платы VPM

OMS [2.2 Local Board Information] или [Board Information view  Click VPM Board]  [Board Data Generation] [Extension Data Handling Plan Dialog Window][Extension Information]

### Присвоение номеров портам платы VPM

Порты платы VPM присваиваются как обычные порты аналоговых абонентов. Телефонные номера портов платы VPM должны входить во внутренний план нумерации абонентов и не должны непосредственно вызываться по СЛ. [6.4 Calling Group], [Representative Number], [2.1 System Numbering PlanExtension Numbering Plan].

## Установка программного обеспечения платы VPM

### Список файлов

| Имя файла | Краткое описание |
| --- | --- |
| ram.hex | Основной программный файл |
| codec.hex codecknl.hex | Кодирование/декодирование голоса |
| dtmf.hex dtmfknl.hex | Программа анализа тональных цифр |
| vpidex.hex & vpdat.hex | Фиксированные сообщения |
| usridex.hex & usrdat.hex | Сообщения, генерируемые пользователем |

### Генерация и компиляция сообщений, определяемых пользователями

- Запишите сообщения в файлы (ZUSER01.wav ~ ZUSER50.wav) с параметрами: РСМ 8 кГц 16 бит моно .

- Установите на компьютер программу компиляции файлов (VPC.EXE and USRDEF.DAT files).

В файле USRDEF.DAT приведен список файлов, которые будут компилироваться (ZUSER01.wav ~ ZUSER50.wav).

- Скопируйте звуковые файлы в директорию, например: “E:\WORK\VPM\VPC\TOKENS”.

- Запустите программу  VPC

- Выберите директорию ‘Token Dir’ E:\WORK\VPM\VPC\TOKENS .

- Выберите ‘Hexa Dir’, где после компиляции будут находиться файлы USRDAT.HEX и USRIDX.HEX. Например: “E:\WORK\VPM\VPC\HEX”.

- Выберите режим “User Defined Token” в поле “Mode” .

- Нажмите кнопку   для проверки количества wav-файлов в файле usrdef.dat

- Нажмите кнопку  для компиляции файлов UsrDat.hex и UsrIdx.hex .

- Нажмите кнопку OK после завершения работы программы.

### Загрузка файлов

1) Подключитесь к плате VPM через последовательный порт (19200, N, 8,1) и запустите терминальную программу.

2) Назначение IP-адреса платы VPM

2.1) Запретите возможность работы платы в режиме on-line.

Для этого в терминале выполните следующие команды:

| Команда | AB 0 : Autoboot Disable AB 1 : Autoboot Enable AB : Verify Autoboot state. |
| --- | --- |
| VPMBUG $ ab [Enter] AutoBoot mode[DISABLE] Usage: ab [1 ¦ 0] VPMBUG $ VPMBUG $ ab 0 [Enter] |  |

2.2) Перезапустите плату VPM. В диалоговом окне нажмите[M], если Вы хотите изменить какие-либо данные.

2.3)  Если Вы в предыдущем пункте установили режим изменения данных, то затем нажмите “4»

How should the board boot? 1. pROBE+ stand-alone mode 2. pROBE+ waiting for host debugger via serial connection 3. pROBE+ waiting for host debugger via a network connection 4. Run the VPM Boot ROM Which one do you want? [4] 4 [Enter]

2.4) Установите сетевые параметры платы.

Do you want a LAN interface? [Y] y [Enter] This board's LAN IP address(0.0.0.0 = RARP)? [165.243.134.57] 165.243.134.11 [Enter] Subnet mask for LAN (0 for none)? [255.255.255.0] 255.255.255.0 [Enter] Should there be a default gateway for packet routing? [N] n [Enter]

2.5) Установите аппаратные параметры платы

Do you want to change the board's Ethernet address? [N] y [Enter] Bytes 0 through 2 are always 08:00:3E What should byte 3 be? [96] F3[Enter] What should byte 4 be? [3D] 86[Enter] What should byte 5 be? [45] 0B[Enter]

2.6) Установите время, по истечении которого в случае невыполнения п.2.2. невозможно изменить

пп.2.3-2.5.

How long (in seconds) should CPU delay before starting up? [1] 1 [Enter]

На экране отобразится окно п.2.2.

2.7)  Подтверждение введенных значений

VPMBUG $ ab [Enter] AutoBoot mode[DISABLE] Usage: ab [1 ¦ 0] VPMBUG $ VPMBUG $ ab 1 [Enter] VPMBUG $ ab [Enter] AutoBoot mode[ENABLE] Usage: ab [1 ¦ 0] VPMBUG $

3) Создайте на своеи компьютере директорию, например: ‘VPM’.

4) Скопируйте туда файлы usridx.hex & usrdat.hex.

5) Установите  программу TFTP поставляемую LGE.

6) Запустите  программу TFTP поставляемую LGE.

7) Выберите Configure из меню TFTP.

8) Выберите директорию с файлами usridx.hex & usrdat.hex. Например: C:\VPM

9) Запустите TFTP.

10) Подключитесь к LAN-интерфейсу платы VPM.

11) Проверьте IP-адрес

| Команда | IP |
| --- | --- |
| VPMBUG $ ip [Enter] |  |

12) Загрузка файлов.

Установите режим работы платы off-line.

12.1) Загрузка блока файлов (ram.hex, codec.hex, codecknl.hex, dtmf.hex, dtmfknl.hex, vpidex.hex, vpdat.hex) возможна только в режиме off-line.

| Команда | UPA <IP-адрес компьютера> ; На компьютере должна быть запущена программа tftp |
| --- | --- |
| VPMBUG $ upa 165.243.134.10 [Enter] Uploading "ram.hex" from HOST 165.243.134.10 / TFTP download completed Uploading "codec.hex" from HOST 165.243.134.10 / TFTP download completed Uploading "dtmf.hex" from HOST 165.243.134.10 / TFTP download completed Uploading "vpidx.hex" from HOST 165.243.134.10 / TFTP download completed Uploading "vpdat.hex" from HOST 165.243.134.10 / TFTP download completed Uploading "codecknl.hex" from HOST 165.243.134.10 / TFTP download completed Uploading "dtmfknl.hex" from HOST 165.243.134.10 / TFTP download completed VPMBUG $ |  |

12.2) Индивидуальная загрузка файлов возможна в режимах off-line и on-line.

| Команда | UPLD <имя файла> <IP-адрес компьютера> ; На компьютере должна быть запущена программа tftp |
| --- | --- |
| VPMBUG $ upld ram.hex 165.243.134.10 [Enter] Uploading "ram.hex" from HOST 165.243.134.10 \ TFTP download completed VPMBUG $ |  |

В режиме off-line- невозможно загрузить файлы usrdat.hex и usridx.hex.

12.3) Отображение имен загруженных файлов на экране.

| Команда | UPLDT |
| --- | --- |
| VPMBUG $ upldt [Enter] >>>>>>>>>>> Команда >>>>>> Upload Table Information <<<<<<<<<<<<<<<<<<< ------+-----------+----------+----------+-------------------------- ACT ¦ ram.hex ¦ 01000000 ¦ 1089864 ¦ 2000/10/01 09:10:23 ¦ codec.hex ¦ 013C0000 ¦ 13DFF1E ¦ 2000/10/01 09:11:07 ¦ dtmf.hex ¦ 013E0000 ¦ 13E7FC0 ¦ 2000/10/01 09:11:24 ------+-----------+----------+----------+-------------------------- STBY ¦ ram.hex ¦ 011E0000 ¦ 0 ¦ 0000/00/00 00:00:00 ¦codec.hex ¦ 017C0000 ¦ 0 ¦ 0000/00/00 00:00:00 ¦ dtmf.hex ¦ 017E0000 ¦ 0 ¦ 0000/00/00 00:00:00 ------+-----------+----------+----------+-------------------------- VOICE ¦vpdat.hex ¦ 01420000 ¦ 14914A8 ¦ 2000/10/01 09:12:56 PROMPT ¦vpidx.hex ¦ 01400000 ¦ 1401B18 ¦ 2000/10/01 09:11:45 ------+-----------+----------+----------+--------------------- VPMBUG $ |  |

12.4) Установка режима работы платы.

| Команда | AB 0 : режим off-line AB 1 : режим on-line AB : Отображение режима работы |
| --- | --- |
| VPMBUG $ ab 1 [Enter] VPMBUG $ ab [Enter] AutoBoot mode[ENABLE] Usage: ab [1 ¦ 0] VPMBUG $ |  |

Auto Boot Enable.

If you reset the VPM when ACTive area includes a normal file and Autoboot is enabled, the VPM_Debug> prompt will appear.

In this case, you cannot use the “upa” command because applications are running. Therefore, you can upload a file individually when the AutoBoot mode is in Enable mode. Uploading ram.hex, codec.hex, or dtmf.hex files is processed in STBY(stand-by) area so that it will not affect the currently active file.

You may also upload usrdat.hex and usridx.hex files when the AutoBoot mode is in Enable state.

Assuming that you have not uploaded usrdat.hex and usridx.hex files since the initial setup of VPM board:- Execute the ‘upldt’ command to view values in the table. Values corresponding to those files show wrong addresses. If you select user-defined token, VPM may go down. For those reasons, you must upload usrdat.hex and usridx.hex files.

| Command | UPLDT ; To verify current file status. |
| --- | --- |
| VPM_Debug> upldt [Enter] >>>>>>>>>>>>>>>>> Upload Table Information <<<<<<<<<<<<<<<<< ------+------------+----------+----------+--------------------- ACT ¦ ram.hex ¦ 20000000 ¦ 20094F48 ¦ 2001/09/09 12:43:25 ¦ codec.hex ¦ 203C0000 ¦ 203C77E0 ¦ 2001/09/07 13:30:18 ¦ dtmf.hex ¦ 203E0000 ¦ 203E3FC0 ¦ 2001/09/07 13:30:23 ------+------------+----------+----------+--------------------- STBY ¦ ram.hex ¦ 201E0000 ¦ 0 ¦ 0000/00/00 00:00:00 ¦ codec.hex ¦ 207C0000 ¦ 0 ¦ 0000/00/00 00:00:00 ¦ dtmf.hex ¦ 207E0000 ¦ 0 ¦ 0000/00/00 00:00:00 ------+------------+----------+----------+--------------------- VOICE¦ vpdat.hex ¦ 20420000 ¦ 20480D74 ¦ 2001/09/07 13:31:10 PROMPT¦ vpidx.hex ¦ 20400000 ¦ 20401968 ¦ 2001/09/07 13:30:27 ------+------------+----------+----------+--------------------- DSP ¦codecknl.hex¦ 203A0000 ¦ 203A01C0 ¦ 2001/09/07 13:31:13 ¦ dtmfknl.hex¦ 203B0000 ¦ 203B01C0 ¦ 2001/09/07 13:31:17 ------+------------+----------+----------+--------------------- USER ¦ usrdat.hex ¦ 20620000 ¦ 20626CDE ¦ 2001/09/09 13:08:40 PROMPT¦ usridx.hex ¦ 20600000 ¦ 206004B0 ¦ 2001/09/09 13:08:55 ------+------------+----------+----------+--------------------- |  |
| Command | UPLD File_Name PC_IP_Adress ; PC opened Tftp Application |
| VPM_Debug> upld ram.hex 165.243.134.10 [Enter] Uploading "ram.hex" from HOST 165.243.134.10 \ TFTP download completed VPM_Debug> upldt [Enter] >>>>>>>>>>>>>>>>> Upload Table Information <<<<<<<<<<<<<<<< ------+------------+----------+----------+--------------------- ACT ¦ ram.hex ¦ 20000000 ¦ 20094F48 ¦ 2001/09/09 12:43:25 ¦ codec.hex ¦ 203C0000 ¦ 203C77E0 ¦ 2001/09/07 13:30:18 ¦ dtmf.hex ¦ 203E0000 ¦ 203E3FC0 ¦ 2001/09/07 13:30:23 ------+------------+----------+----------+--------------------- STBY ¦ ram.hex ¦ 201E0000 ¦ 20274F48 ¦ 2001/09/09 13:49:12 ¦ codec.hex ¦ 207C0000 ¦ 0 ¦ 0000/00/00 00:00:00 ¦ dtmf.hex ¦ 207E0000 ¦ 0 ¦ 0000/00/00 00:00:00 ------+------------+----------+----------+--------------------- VOICE¦ vpdat.hex ¦ 20420000 ¦ 20480D74 ¦ 2001/09/07 13:31:10 PROMPT¦ vpidx.hex ¦ 20400000 ¦ 20401968 ¦ 2001/09/07 13:30:27 ------+------------+----------+----------+--------------------- DSP ¦codecknl.hex¦ 203A0000 ¦ 203A01C0 ¦ 2001/09/07 13:31:13 ¦ dtmfknl.hex¦ 203B0000 ¦ 203B01C0 ¦ 2001/09/07 13:31:17 ------+------------+----------+----------+--------------------- USER ¦ usrdat.hex ¦ 20620000 ¦ 20626CDE ¦ 2001/09/09 13:08:40 PROMPT¦ usridx.hex ¦ 20600000 ¦ 206004B0 ¦ 2001/09/09 13:08:55 ------+------------+----------+----------+--------------------- |  |

| Meaning of Auto Boot |  |
| --- | --- |
| Disable; Non-Operation Status Use VPM data setup before service. Files uploaded from PC will be saved to Active Table of VPM. If the data setup is completed, turn the Disable state into Enable for service and reset VPM. Enable; Operation Status VPM is in a state to provide the service. If you upload a file, it will be saved to the Standby Table of VPM that does not affect the current operation status. If you want to internally re-upload the file to Active Table of VPM, perform the following steps and reset VPM. |  |
| Command | CHGM |
| VPMBUG $ chgm file Name [Enter] ; file Name = ram.hex /codec.hex/dtmf.hex |  |

14) Initializing VPM Board completes the installation procedures of VPM Board.

# Service Type

## VMS

### Description

VMS is used to perform Mail Box Service, Mail Access Service, Call Forwarding Service.

### Service Method

      - Mail Box Service

Service Type

Three types of Mail Box Service are provided as below.

1.1) Type 1

An extension registered in Mail Box dials VMS representative number [6.4 calling GroupService type; MAILACC] to check and listen a message saved in the Mail Box assigned by his own ID.

1.2) Type 2

An extension registered in Mail Box dials VMS representative number [6.4 calling GroupService type; MAILACC] and connects to VMS for recording his voice information.

When the extension is not in the state of answering like “Busy, Absent, DND, or No-answer” in case of an outside subscriber call, he can use VMS call forwarding function by selecting a code (See 1.3.1) suitable for his present state through the phone. And the outside subscriber can hear the voice information saved by extension.

1.3.1) VMS Call forwarding registration by extension

SLT / D-Tel

OMS [2.1 System Numbering Plan  Feature Plan]

| Description | Code Use |
| --- | --- |
| All (unconditional) Call Forwarding registration. | (*527) |
| Busy Call Forwarding registration | (*528) |
| Busy or No Answer Call Forwarding registration | (*52*) |
| Busy Call Forwarding registration | (*529) |
| Note) How to select a code for VMS Call forwarding in the above SLT Ex) Off-hook  *52X  On-hook ; X = 7,8,9,* |  |

D-Tel

| Description | Code Use |
| --- | --- |
| All (unconditional) Call Forwarding registration. | MON + DND/FWD + 7 |
| Busy Call Forwarding registration | MON + DND/FWD + 8 |
| Busy or No Answer Call Forwarding registration | MON + DND/FWD + * |
| Busy Call Forwarding registration | MON + DND/FWD + 9 |
| Note) How to select a code for VMS Call forwarding in the above SLT Ex) Off-hook or MON  DND/FWD  X : X = 7,8,9,* |  |

1.4) Type 3

This type is a service for an extension registered in Mail Box to dial VMS representative number so that they may change his password by dialing on the phone.

      - Mail Access Service

This service is provided for an outside subscriber to leave his voice message when the destination (extension) is not in the state of answering the outside subscriber’s call.

1.1) Whole Capacity:

The Mail Access Service is capable of 254 Tokens of 60-second voice message in Token Category1. Assuming that Bulletin service is, the number of usable token will be reduced to 244.

1.2) Capacity of Mail Box ID for an extension

An extension registered in Mail Box can have ten voice messages.

- 2) To indicate message for D-Tel, SLT.

- 2.1) How to indicate Message arrival

- D-Tel

- Indicated on LCD

MSG EXT(0) TRK(0) VM(1)

LINE 11018       06:45 PM

- VM(1): Currently a voice mail is saved in Mail Box.

- SLT

- Message Wait Ring gives to SLT at the defined interval.

- OMS

- To adjust the interval

- [31. Extension Information  Extension Characteristics SLT MW No Ring]

- 2.2) How to check a message in Extension

- 2.2.1)  Check the message over D-Tel.

- A. Check the VM counter on LCD.

- B. Press MOM Button.

- C. Press CallBK Button.

- D. Follow VMS voice message for input.

- Check at SLT.

- Case1) When using Message Ring :

- OMS [Extension Data Management3.1 Extension

- Information=>Extension Characteristics->7.SLT M/W No

- Ring]

- Listen to Message Ring

- Go into Off-Hook mode and press *7 or VMS representative number.

- Follow VMS voice Message for input

- Note) In case of Message Ring SLT in the Check mode, access Mail Box to delete the message, or else there will be rings at a certain interval.

- Case2) when not using Message Ring

- Dial VMS (MAILACC) representative at the extension.

- Follow VMS voice Message for input.

- Check Recorded Message.

- Note) Make sure whether or not Single Line Telephone Message Ring

- [3.1 Extension InformationExtension Characteristics7.SLT M/W No Ring]

      - Call Forward Service

This service is used for an outsider subscriber to call an extension registered in Mail Box, who is not in the state of answering.

Case1) When an outside subscriber calls an extension through ATD

Case2) When an outside subscriber calls an extension in a direct way

The above both cases are given the following result:-

The outside subscriber hears the voice information, which the extension has assigned for his present state, provided by VMS. And then the subscriber may leave his message,

Note1) Exception to call forwarding) When an extension has set his phone to VMS call forwarding using feature code or function button (D-Tel) and an outside subscriber is directed to the extension. (See 4.1.2.1 Mail Box Service .12) Type 2)

Note2) Incase I-Phone calls to registered extension, this service don’t allow.

### Service Scenario

##### Mail Access Service

Mail Access

Message Wait

Answer Code ?

Mail box number exist ?

Password OK ?

C1

Retry

(VP130000)

Y

N

Input DTMF

(VP130001)

Input DTMF

Y

(ABS012)

N

Y

N

(ABS012)

(VP131000 )

(CN001, … )

(VP131001 )

Input DTMF

Are there a mail ?

(VP131007)

(VP130002)

(VP130003)

1

3

Message number > 0

Y

Y

N

N

C1

C2

C3

End

*

0

C4

“ ( ex. 1st )”  (ON001, …)

Play recorded message.

“This message has been recorded at ( time ).” (VP131003)

Input DTMF

“The message has been deleted.” (VP131005)

Next Mail ?

“There are no more messages in your mail box.”     (VP131006)

“Press ‘0’ to return top menu or press ‘*’ to quit.”   (VP131011)

Y

N

“Press “1” to listen to the next message,

press “2” to listen again,

press “8” to delete the message. ” (VP131004)

Listen Next Mail

Mail Access again

Mail cancel

1

8

MONTH XX

DAY XX

HH XX

MM XX

C3

2

End

C4

*

0

Mail Box Service

“Please press “1” to change a password and press “2” to check greetings message. ” (VP133000)

“Press”0” to return to previous menu or press”*” to quit”

(VP133002)

“Press a new password

of 4 digits.”

(VP133101)

1

2

Input Password

“The change has been

confirmed”(VP133102)

DTMF Input

C2

“Press “1” to register, press “2” to confirm, press “8” to delete. ” (VP133200)

“Press”0” to return to previous menu or press”*” to quit”(VP133002)

“Please record

greeting after beep

and press ‘*’ when

you are done.”

(VP132201)

Listen Greeting

Greeting cancel

“The Message has been

deleted.” (VP131005)

1

2

8

Greeting exist?

Y

N

“The greeting has not

been registered.”

(VP132202)

C2

C2

End

*

0

End

*

0

Forward Call Service

Forward Call

Individual Greeting ?

End

Busy : “Sorry, The line is busy” (ABS002)

No answer : “The person doesn’t answer your call.” (ABS001)

DND : “The person cannot answer your call.” (ABS003)

Absence - out to lunch  : “The person is out to lunch” (ABS005)

- on vacation   : “The person is on vacation” (ABS006)

- out of office  : “The person is out of the office on business” (ABS007)

- in office : “The person is in office, but doesn’t be connected.” (ABS008)

- in a meeting  : “The person is in a meeting.” (ABS009)

- at home         : “The person has left for the day”  ( ABS010)

- in a business trip : “The person is away on business.”  (ABS011)

Listen

Individual Greeting

Y

N

“Please leave a message after beep and

press “*” when you are done.”(VP110003)

Recording + ‘*’

“Your message has been properly recorded (VP110004)

Thanks for your call.” (VP110005)

VM Message Wait Registration

“beep~“ (BEEP10)

HOTKEY : ‘0’

In this step, if user

press ‘0’, we enter

Mai l Access call.

VM : Voice Mail

### Data Management

How to generate Code to connect to VMS

1) To access Voice Mail Box

OMS [6.4 Calling Group] (Refer to the OMS Part to system Manual)

- Input Representative Number

- Select Service Type  MAILACC

- Select Ring Ser Method  First or UCD

- Select Busy & DND Option  Send Busy Tone.

- No Answer Option  Send Busy Tone.

- Calling Group Service Extension  VPM Board의 Port Telephone Number.

2) How to assign extension Mail Box ID

OMS [15.4 Mail Box Management]  (Refer to OMS part in System Manual)

2.1) Built-in VPM

Define OMS [15.4 Mail Box ManagementVPM Check & Slot √ / MB ID/ Unit/ Recorded Msg Counter]

MB ID must be assigned with the same number as the extension.

- 2.2) Separate VMS device (See OMS Part of System manual for further)

Define OMS [15.4 Mail Box ManagementExternal VMS √ / MB ID(Mail Box ID)/ Password / Unit / External TelNo / Recorded Msg Counter]. (See OMS VMS Part of System manual for further)

MB ID must be assigned with the same number as the extension.

3) Voice Mail Option Definition

Define OMS [15.5 Voice Mail Option  Retry Count (In case Vacant Number)] Value at 2 or upper.

- OMS [15.5 Voice Mail Option CFWD to VM (Busy)] is assigned to ‘YES’.

- [15.5 Voice Mail Option CFWD to VM (No Answer)] is assigned to ‘YES’

4) Message Ring for Single Line Telephone Definition.

4.1) Interval Message Ring Parameter Handling

OMS [2.5 MSG Ring] to transfer to SLT subscriber

4.2) Whether to use Interval Message Ring to transfer to SLT subscriber or not.

OMS [3.1 Extension InformationExtension Characteristics7.SLT M/W No Ring] √ = No Ring.

### Correlation

Call processing order is as follows:-

1st  check point

OMS [15.5 Voice Mail Option CFWD to VM (Busy)/(No Answer)]

2nd  check point

OMS [4.9 Trunk route base day/night incoming option]

3rd check point

Call Forwarding code(*52X).

So if [15.5 Voice Mail Option CFWD to VM (Busy)/(No Answer)] is not assigned, the processing order is as follows:-

1st check point

OMS [4.9 Trunk route base day/night incoming option]

2nd check point

Call Forwarding code(*52X).

Or if OMS [4.9 Trunk route base day/night incoming option] is not assigned , the processing order is as follows:-

1st check point

OMS[15.5 Voice Mail Option CFWD to VM (Busy)/(No Answer)]

2nd

Call Forwarding code(*52X).

Note) If ATD was not installed in system, the clerk (ATD) call on scenario will access to System PNA extension. [16.4 System PNA].

## IVR(Interactive Voice Response : Auto Attendant)

### Description

This function is applied to the case that VPM first respond to an incoming trunk call or an extension call through Auto ATD Service Representative Number assigned to a system and then provides voice announcement to the caller. If the caller enters DTMF following the VPM voice announcement, the call will be switched to the destination extension.

### Service Method.

If there is a trunk or extension call to VPM, the caller can dial the extension (destination) number following the VPM voice announcement or enter one-digit menu index of operator-set CCR (Call Control Routing). If a scenario is composed for using CCR, an outside subscriber may be requested to enter CCR menu index in one step or more according to system supplier’s features.

[Features]

          - This function is enabled to automatically identify digits entered by a caller using Inter Digit Lock as extension number and CCR(Call Control Routing) Menu number

          - Various types of CCR destinations are assignable for use as below.

        - Different CCR Table Index

        - Extension number

        - Calling Group Representative number

        - ACD Pilot number

        - System Speed Code

        - Individual Speed Code of defined extension number

        - QSIG number

        - Internal Zone Paging

        - ATD Access Code

        - UNA

          - Every CCR Table is allowed to have three individual announcements max.

          - Every CCR Table has individual Night Bar needed to recognize Day/Night Mode.

          - Every CCR Table has Wait Time needed to enter DTMF after announcement transmission.

          - An operator may use CCR Table within the limit of the max resources available in the system in a flexible way.

### Service Scenario

Refer to the next pages.

IVR Service by CCR Start Table

IVR Service by Selected CCR Table

Can Transfer ?

One Digit Input ?

End of Auto ATD Service

CCR Table Day=?

More Digit Input ?

Retry Count > 0

A

Avail Numbering Plan ?

A

Avail CCR ?

DTMF Wait Time = 0 ?

A

Next CCR Table ?

B

B

CCR Table Announcement Play

Y

Connect to the desired number

“Sorry, the line is busy” (Note 1)

“Thank you for your call” (VP110005)

“You will be connected shortly” (VP110002)

“You have pressed the unavailable number” (VP110002)

N

“Thank you for your call” (VP110005)

Y

Y

Y

N (Time Out : No Dialing)

Increase Retry Count

N

N (Check Night Event)

Y

N

CCR Table Announcement Play

N

Y

Note 1: The recording of the failure is as follows:

- Cause-Busy: “The line is busy” (ABS002)

- No Answer: “The person doesn’t answer your call.” (ABS001)

- Absent: “The person is not in right now.” (ABS004)

- DND: “The person cannot answer your call.” (ABS003)

- Vacant: “You have pressed the unavailable number.” (ABS012)

- IC Deny Class: “The number cannot be called.” (ABS013)

- Transfer EXT No Answer : “Sorry the person doesn’t answer your call.”(ABS001)

N

N

Y

Y

N

Index n

      

### Data Management

- 1) Choosing IVR Service option

- [15.5 Voice Mail Option  Phone Book Service Use ]

- 2) Composing IVR Scenario

- [15.7 IVR]

- 3) Selecting Announcement for each CCR Table

- [15.7 IVR  User Define Ment 1,2,3]

- 4) Choosing Night Bar for each CCR Table

- [15.7 IVR  Day Start Time and Night Start Time]

- 5) Setting DTMF Wait Time for each CCR Table

- [15.7 IVR  DTMF Wait Time]

- 6) Assigning Start CCR Table per System Mode

- [15.7 IVR  Auto ATD Calling Group in Pop Up Menu by Mouse Right Button]

- 7) Setting Inter Digit Lock Time to Recognize Extension No. and CCR Index

[15.5 Voice Mail Option  Inter Digit Lock]

### Correlation

Note) If ATD was not installed in system, the clerk (ATD) call on scenario will access to System PNA extension. [16.4 System PNA].

## Voice Tone

### Description

This function is to use User Defined Token( ZUSER01.wav ~ ZUSER50.wav) recorded and compiled by an operator and included in vplst.dat & vplst.dat, instead of system-provided tone.

### Installation

The installation is in such a way that adds User Defined Token in the VPM S/W Installation item

### Service Method

Outside/extension subscribers who are provided with voice tone constantly may hear the voice tone from the start. Each system may choose two modes below that are user-customized

[Real Mode]

This mode is to access an assigned VPM port to give a tone to any subscriber, whenever called.

The merit is that the mode is activated on a real time basis. But it needs a lot of VPM ports.

[Interval Mode]

Voice tone is repeatedly played through an assigned port. Whenever there is a call made by a subscriber, the subscriber will be waiting in the Queue while hearing an assigned system tone. Then the subscriber will hear the voice tone from the start of voice tone play. The number of subscribers waiting in the Queue for the voice tone is not limited. The limitless number can be a merit but the subscribers must wait in the Queue for seconds (Voice Tone Recording Time / Number of VPM Ports) until the voice tone starts to play.

Ex. of Use) The case that a caller hears RBT waiting in the Queue while using VPM Port 3 for 30-second voice tone instead of ACD Queuing Split Tone.

2nd Port Play Start

1st Port Play Start

3rd Port Play Start

2nd Port Play Start

1st Port Play Start

3rd Port Play Start

2nd Port Play Start

1st Port Play Start

3rd Port Play Start

10sec

10sec

10sec

Outside subscriber A, B, and C make calls, get to wait in Queue, and hear RBT.

The subscribers A, B, C waiting in Queue start to hear voice tone simultaneously.

### Data Management

Selection of token ID of voice tone to be substituted for system tone

OMS [15.1 Voice Token InformationTone Type  Voice Tone Token ID]

Select the item of tone type currently provided by system. Then type the ID of the token recorded by user definition to Voice Tone Token item.

Select Message channel recorded in Token ID of voice tone to be substituted for system tone.

OMS [15.2 VPM Token Information VPM Token ID  Recoding Time  Start PortEnd Port]

Verify the recording time (Message Playing Time) needed to be played in the VPM according to its VPM Token ID, and assign port range for its voice channel.

### Correlation

4 tokens are used for ATD. Therefore 16 Tokens can be used for voice tone and announcement.

## Voice Bulletin

### Description

Voice Bulletin is a function for in-house members to check company bulletin through the telephone.

The number of Bulletin token (Token category3) is 10 in all. The recording time of each token is limited to one minute

### Setup Procedure

Step1) Assign a representative number to each service type for an extension to use Bulletin in OMS. [6.4 calling GroupService type; BULLETIN / TOKENCHG

Step2) Set Option to ‘Yes’ in [15.5 Voice Mail Option VPM Bulletin Service Use] using Bulletin Service.

Step3) Dial the TOKENCHG representative number assigned in OMS. Press “0” for the service of Bulletin Menu in the section of Input DTMF in TOKEN Change Scenario.

Step 4) According to Scenario Flow, extension assigned in OMS can record, change, and delete menu and information about Bulletin through the telephone.

(Recording Type; Announcement on pressing DTMF Digit 1 + Announcement on pressing DTMF Digit 2+ Announcement on pressing DTMF Digit 3 …………+ Announcement on pressing DTMF Digit N {N=1~9};. assign the services per Digit up to Max 9 )

Step 5) Call the BULLETIN representative number and then confirm the voice information of each item, after all the above 4).

Note) If the announcement of “There is no message for bulletin.” is displayed on calling Bulletin representative number, ensure that the voice information for the Menu of the number “0” is recorded.

### Service Method

Change Bulletin Token.

Only extensions eligible for Bulletin Token service can record/save/delete/change menu and information in the token for Bulletin service.

An extension or an outside subscriber may hear an announcement of the information recorded on item desired according to Scenario Flow using DTMF Digit.

### Service Scenario

(For example, )

“Press ‘1’ for new information announcement …….” , “press * to quit.” (VP140004)

Input DTMF

Bulletin Service

Play message.

DTMF OK ?

Y

N

“You have pressed a

wrong number.”(E00001)

“This is the bulletin service.”(VP140000)

**

Bulletin service can support max 10 token.

0    : bulletin menu

1 ~ 9 : bulletin message

If administrator record or delete bulletin message, he/she can use token change service.

Bulletin Menu.

Bulletin registration ?

Y

N

“There is no message for bulletin.”  (VP140001)

Token Change

“Please leave a message after beep and press “*” when you are finished.” (VP110003)

Recording + ‘*’

“Your message has been

properly recorded.”(VP110004)

1

Token Play

“Please press ‘1’ to record, press ‘2’ to confirm or press ‘8’ to delete.”(VP140002)

“Press * to quit”          (VP140004 )

“The message has

been deleted.”

(VP131005)

2

8

“ Press the voice bulletin number

to record or delete. “ (VP140003)

Input DTMF

End

*

### Data Management

Ensure that Bulletin Service is available.

[15.5 Voice Mail Option VPM Bulletin Service Use]

Assign a representative number used to access Bulletin Service.

- Input Representative Number

- Select Service Type  Bulletin&TOKENCHG

- Select Ring Ser Method  First or UCD

- Select Busy & DND Option  Send Busy Tone

- No Answer Option  Send Busy Tone.

Assignment of Bulletin Token Change available to extensions

OMS [3.1 Extinction Information Extension Characteristics13.Token Change],

### Correlation

If a system serves for Bulletin, the max number of leaving voice message in VMS is reduced from 254 to 244.

### Bulletin Message Checkup

Record/delete the message using token change service and confirm it in VPM as follows:-

| Command | MBOX ; Change to Token Category1 Prompt |
| --- | --- |
| Command | CHK B ; To Check Recoding Status |
| VPM_Debug> mbox [Enter] MBOX> chk b [Enter] Flash adr[0x02020000] Size[0x2594] Attr[00] 2000/10/11 10:42:44 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 Flash adr[0xFFFFFFFF] Size[0xFFFF] Attr[FF] 65535/255/255 65535:255:255 |  |

The above shows that the message of only No. “0” of Token 10 ID is recorded. If you delete it, Attr(attribute) field indicates FF

# Trouble Shooting Guide

ST1) When you mount VPM, the system goes to the Reset mode, check the PB switch of the board. In this case, all the switches must be off.

ST2) First installation in site

| Command | MBOX RSAT |
| --- | --- |
| VPM_Debug> mbox [Enter] MBOX> rsat [Enter] Used for system : 2 SCTR Used for user mail : 0 SCTR Free : 254 SCTR MBOX> |  |

In the first installation in the site, the SAT information must be displayed as above. Otherwise, go to the VPMBUG $ mode to execute the command as below.

| ###### Command | QUIT AB O CSAT AB 1 |
| --- | --- |
| MBOX> quit [Enter] VPM_Debug> ab 0 [Enter] reset VPM VPMBUG $ csat [Enter] Fixed pattern was written at 0x5000D000 Fixed pattern was written at 0x5001D000 VPMBUG $ ab 1 [Enter] |  |
| Reset VPM |  |

The “csat” command performs a function of initializing the Mail box information table. Therefore, be sure to execute the command when initially installing the board in the site. But never execute it in operation.

ST3) When a user forgot his own password.

Viewing the List of Mail Box Number and Password Created in the VPM

| Command | MBOX ML |
| --- | --- |
| VPM_Debug> mbox [Enter] MBOX> ml [Enter] [0x5000D000: 0] ID[99999999] PSWD[9999] [0x5000D100: 1] ID[ 1000] PSWD[AAAA] MBOX> |  |

The above figure shows that the password of ID 1000 has been created to be “0000.”  (the Mail box number 99999999 is the service number of Bulletin board)

Monitoring the password and listening of each Mail Box ID created in the VPM

| Command | MBOX CHK # ; # Mail Box ID |
| --- | --- |
| MBOX> chk 1000 [Enter] MBOX_ID[1000] Passwd[AAAA] MaxMAIL[3] VoxMAIL[2] CurRECD[0] Flash adr[0x02030000] Size[0x21C0] Attr[01] 2000/10/11 10:59:22 Flash adr[0x02040000] Size[0x0C4E] Attr[00] 2000/10/11 11:07:43 |  |

The above figure shows that the Mail box of ID 1000 can save max three voice messages (Max MAIL); and now it has two voice messages (VoxMAIL) recorded. In the Attr field, 00 means that the voice message is not heard yet and FF means that the voice message has been deleted already.

ST4) When Prompt indicates “pROBE+>” in the connection of VPM and PC by SIO

Cause:When Autoboot is in the Enable mode and upld table is different from Data in Flash, “pROBE+>” mode appears.

Solution :Change Autoboot into the Disable mode.

pROBE+>dm 600201d0..600201d1 600201d0 01 pROBE+>fm.c 600201d0..600201d1 00 pROBE+>dm 600201d0..600201d1 600201d0 00