SECTION 5. DECT Installation

The LDK-300 WOTS (Wireless Office Telephone System) adds wireless telephone service to the LDK-300 Digital Key Telephone System. The wireless standard of the system is DECT(Digital Enhanced Cordless Telecommunications). The LDK-300 Digital Key Telephone System is a full digital hybrid Key Telephone system, designed to meet the telecommunication needs of large and medium sized business offices for in-building or various environments.

The components of the LDK-300 WOTS are LDK-300 Digital Key Telephone system, WTIB, base stations and wireless terminals. Base stations are installed throughout a facility to relay calls between wireless terminals and the office telephone system. The GDC-330H/335H is a wireless terminal. The LDK-300 WOTS is connected to an existing telephone system and accommodates up to 24 base stations and up to 192 wireless terminal users. Each base station provides 5 channels and calling coverage of up to 190,000 squire meters in open space.

The wireless terminal users have access the right to the office telephone system (LDK-300) feature in addition to being free to make and calls away from their desks. The wireless terminal is small, light and easy to use. It also provides a display for alphanumeric information.

## 5.1  Components of the LDK-300 WOTS

The components of a LDK-300 WOTS are;

 	LDK-300 Digital Key Telephone System (wired system)

 	WTIB, WTIU(optional) cards

 	Base Station (GDC-330B)

 	Wireless Terminal (GDC-33xH) : GDC-330H & GDC-335H

 	Charger (330-SSC)

<Figure 5.1.1> shows components of LDK-300 WOTS. Others are wireless terminal, base station and charger. LDK-300 Key Telephone system includes WTIB and/or WTIU cards.

(a) LDK-300 Key Telephone System

(b) Wireless Terminal   (c) Charger(330-SSC)          (d) Base station(GDC-330B)

### <Figure 5.1.1> Components of a LDK-300 WOTS

### A.  LDK-300 Digital Key Telephone System

The LDK-300 Digital Key Telephone Systems are wired and full digital hybrid Key Telephone system, designed to meet the telecommunication needs for large and medium business offices respectively.

### B.  WTIB (Wireless Terminal Interface Board)

The WTIB provides standard interface between the LDK-300 Digital Key Telephone system and DECT network, and can be optionally equipped with WTIU for expanding base stations. The system can accommodate three WTIBs which support up to 4 base stations each.

The WTIB should be plugged into slot1 through slot8, and should be installed at the same KSU in case of installing more than one WTIB.

#### WTIU (optional)

The WTIU is an optional board for cell expansion. If it is added to the WTIB, the system can support up to 4 base stations more.

### C.  Base Station (GDC-330B)

The base station should be installed indoors and protected from surge because it is designed for indoor station.

The base station is used commonly in the GDK/LDK WOTS Series system. It has a RF(Radio Frequency) Module(transmitter and receiver) that handles communication with wireless terminals through the air link. It should be connected to a LDK-300 system via WTIB.

Each base station provides similar coverage for a particular area call as a cell, and supports five simultaneous calls (five traffic channels). However, since individual wireless terminals are not continuously in call, the system may support more than five wireless terminals:

In LDK-300 system, Maximum 192 wireless terminals can be registered.

Base stations may be mounted on walls or desktops up to 600 meters away from the LDK-300 WOTS using twisted 2-pair unshielded cable (0.5, AWG 24). They are remotely powered (DC 30 volts) by the WTIB.

The number of base stations used in a system depends on the area to be covered and the traffic density. The typical in-house coverage is a 40 meter radius. In practice the cell size may vary between 10 meter indoors in worst case situation, up to 300 meters outdoor in free space.

### D.  Wireless Terminal (GDC-33xH) : GDC-330H & GDC-335H

It is designed for ease-to-use. The LCD shows various call-information, and the keypad has a 12-dial pad and additional keys that facilitate wireless terminal.

### E.  Single Slot Charger (330-SSC)

The battery of a GDC-33xH is charged by a 330-SSC.

## 5.2	How to Make Calls in the LDK-300 WOTS

### A.  Before Making Initial Call

It is similar to other calls in a telephone system to make a call in LDK-300 WOTS, except that every call to or from a wireless terminal involves establishing a RF link to the wireless terminal through a base station. In an idle status, a wireless terminal scans a dummy bearer from base stations and locks to the base station with the strongest signal. The dummy bearer is similar to the control channel in a cellular telephone system. If the wireless terminal can not synchronize to a dummy bearer, it can not make a call.

System ID(PARK) is very important in the LDK-300 WOTS calls. The WTIB should be assigned to a unique LDK-300 WOTS ID(PARK). Installation Engineer should not use the other PARK ID as LGE assigns to the each system. Wireless terminals should be programmed with a system ID that matches a particular LDK-300 WOTS before communicating with a base station in the system.

#### Calls to Wireless Terminal

- The WTIB receives a call from an extension of the system.

- To locate the called party, the WTIB sends a request containing the ID of the called wireless terminal to all base stations.

- The called wireless terminal responds to the base station onto which it is locked.

- The RF link is established. The call then proceeds like other calls in the office telephone system.

#### Initiated Call from a Wireless Terminal

- The wireless terminal locks onto a base station and initiates a RF link.

- The base station transmits call information to the WTIB. The call then proceeds like other calls in the office telephone system.

### B.  Controlling a Call

During a call, interaction occurs mainly between the wireless terminal and a WTIB. The wireless terminal accepts input at the keypad by user and sends the information to a WTIB via base station. The WTIB handles most of signaling functions and controls the display of wireless terminal.

### C.  Terminating a Call

Call termination involves releasing a voice channel and taking a wireless terminal into a standby status. When WTIB recognizes that a call has terminated, it sends a release message to a base station. The base station and wireless terminal are released.

In some cases, the base station should initiate call termination by sending a message to the WTIB. Loss of wireless terminal power and interference with the RF signal are examples of events that cause the base station to terminate a call abruptly.

## 5.3  LDK-300 WOTS

### A.  WTIB

The WTIB provides standard interface between the LDK-300 Digital Key Telephone system and DECT network

<Figure 5.3.1> shows the layout of the WTIB of LDK-300

<Figure 5.3.1>  Layout of the LDK-300 WTIB

#### LED, Switch and Modular Connector

The 9 LEDs mounted on the LDK-300 WTIB provide diagnostic information for status of the board. <Table 5.3.1> shows the meaning of various status of the LEDs.

| LED Number | Meaning (When it is lighted) |
| --- | --- |
| LED 1 | HDLC comm. |
| LED 2 | Timer Interrupt operation |
| LED 3 | Subscription operation |
| LED 4 | Voice channel active |
| LED 5 | Inter-WTIB HDLC operation |
| LED 6 | Echo canceller operation |
| LED 7 | BMC PSCN operation |
| LED 8 | BMC Interrupt operation |
| LED 9 | Master/Slave operation |

<Table 5.3.1>  LEDs of the LDK-300 WTIB

The modular connector JK1, JK2, JK3, JK4(WTIB), JK1, JK2, JK3 and JK4(WTIU) are connected to base stations by twisted 2-pair unshielded cable. <Table 5.3.2> shows the relation between modular connectors and associated cell numbers.

| Modular Connector Number | Cell Number |
| --- | --- |
| JK1(WTIB) | Cell 1 |
| JK2(WTIB) | Cell 2 |
| JK3(WTIB) | Cell 3 |
| JK4(WTIB) | Cell 4 |
| JK1(WTIU) | Cell 5 |
| JK2(WTIU) | Cell 6 |
| JK3(WTIU) | Cell 7 |
| JK4(WTIU) | Cell 8 |

<Table 5.3.2> Relation between modular connector and cell number

| SW | On | Off | Description | Default |
| --- | --- | --- | --- | --- |
| SW1-1 | Service | Normal | WTIB Normal/Service | OFF |
| SW1-2 | Enable Echo-can. | Disable Echo-can. | Echo-canceller control | ON |
| SW1-3 | Enable Watchdog | Disable Watchdog | Watchdog control | ON |
| SW1-4 | For emulator | Normal | S/W debugging | OFF |

<Table 5.3.3> Switch and Jumper Setting Table

#### WTIB and WTIU Handling

The WTIB and WTIU contain digital circuitry which, while extremely reliable, can be damaged by exposure to excessive static electricity. When handling the WTIB and WTIU, a grounded wrist strap should be used to protect the board from static discharges with common sense for handling PCB. For example, do not place a PCB in the locations where heavy objects might fall and damage components.

### Base Station

<Figure 5.3.2> shows a base station.

<Figure 5.3.2> Base Station(GDC-330B)

Base stations may be mounted on wall or desktop up to 600 meters away from the WTIB using a twisted 2-pair unshielded cable(0.5). The system supports max. 24 base stations.

In typical office environments, each base station has a broadcast range of 70 meters and supports an area of 190,000 square meters. In settings where the absence of any obstruction provide perfect line of sight conditions between a base station and wireless terminal users and there are no atmospheric limitations, the range may increase to max.120 meters.

LEDs on the PCB of a base station provide diagnostic information for base station status.

#### Antenna Diversity

Spatial diversity at the base station is implemented by using two antennas. The antenna providing a best signal for each time slot is selected. This is performed on the basis of the RF power level and alternatively the quality information.

In DECT, the BER(Bit Error Ratio) is affected by multi-path propagation not only in terms of signal fading but in terms of time dispersion, which leads to ISI(Inter Symbol Interference). Antenna diversity not only reduces the fading probability but mitigates the effect of time dispersion.

#### Power Feeding

The base station is DC powered from the WTIB. <Figure 5.3.3> shows the phantom power feeding circuit between a WTIB and a base station.

<Figure 5.3.3> Phantom Power Feeding Circuit between WTIB and Base Station

#### Roaming

Roaming is the movement of a wireless terminal from one base station coverage area to another base station coverage area, where the base stations enable the wireless terminal to make or receive calls in both areas. The system supports roaming function.

#### Hand-over

The LDK-300 WOTS provides a seamless hand-over, almost undetectable by users as they move from one radio cell to another.

#### Digital Voice Quality

The LDK-300 WOTS uses advanced digital technology to provide high sound quality as almost equal to fixed line telephones. Speech coding uses 32kbps ADPCM(Adaptive Differential Pulse Code Modulation) conforming to CCITT recommendation G.721.

#### Multiple Simultaneous Users

The LDK-300 WOTS accommodates 96 multiple users in simultaneous conversation with no degradation in performance.

### C.  Wireless Terminal (GDC-33xH) : GDC-330H & GDC-335H

The wireless terminal is designed for ease-of-use at the desks of users or moving throughout a facility.

- It is small and light for ease-of-use.

- The LCD displays call information and the status of terminal.

- The keypad contains specially labeled keys assisting users to make calls and operate telephone features.

- <Figure 5.3.4>  Wireless Terminal (GDC-33xH)

#### Subscription

- The wireless terminals associate unique identities, to prevent unauthorized use of the PSTN line. See the User Guide of the GDC-33xH for the subscription of wireless terminals.

#### LCD Display

- The wireless terminal has a three-line display (6 icons & 20 characters) for alphanumeric information.

- <Figure 5.3.5>  The LCD of GDC-33xH

#### Security

- The wireless terminal ciphers voice information before transmitting it. It ensures that the conversations are private and inaccessible to electronic eavesdropping.

#### Ringing

- There are various types of ring signal in the wireless terminal and users can select the ring signal and control the volume of ring signal. Seven volume levels are available.

#### Key Pad Tone

- A beep is heard when the wireless terminals recognize a dial input from dial pad.

#### Low Battery Alarm

- When the battery is discharged to low battery state, the low battery message is displayed in the display LCD of the wireless terminal and warning tone is heard.

#### Removable Battery Pack

- The wireless terminal has one type of battery pack (330-SBP, 600mAh). It is standard battery pack.

#### Antenna

- The antenna of the GDC-33xH is an wire and non-retractable antenna. Do not extract it. If you try to pull it out, it may be destroyed.

#### Holes of Top and Bottom Side

- There are two holes in the top and bottom side of a GDC-33xH. The holes are special purpose and are covered with covers. Do not take off these covers.

#### Charger (330-SSC)

- The GDC-33xH comes with a charger that accommodates GDC-33xH and one type of battery pack. The SSC-330 is supplied with the adapter (330-HAP). 330-HAP adapter is used for the charger, 330-SSC. There are two types of adapter.

- It is dependent on country adaptation.

- Type -Ⅰ

- Input :	Only AC 230V ~ 50Hz 50mA

- Output :	DC 12V, 200mA  2.4VA

- Type -Ⅱ

- Input :	Both AC 220V and 110V ~ 50/60Hz

- Output :	DC 12V,  200mA

LED (red) on the charger indicates that charging is in progress.

A 330-HAP adapter is used for the charger, 330-SSC. When a battery is almost consumed during a call or in an idle state, warning tone will be heard more 3 times in every 20 second with “LOW BATT.” message on the LCD, and a low-battery icon ( ) will be twinkling at the right corner of the LCD.

The battery inside of GDC-33xH is Nickel Metal Hydride (Ni-MH) and can be charged any time. To increase the lifetime of the battery, use the wireless terminal without charging until the battery alarm is heard.

## 5.4  System Configuration

The LDK-300 WOTS is used in single cell or multi-cell(maximum 24 cells).

## 5.4.1  Before Installation

Followings are the factors that affect the quality of service for the LDK-300 WOTS.

 Location of base station

 Number of wireless terminal users within a base station range

 Obstructions between base station and wireless terminal

 Coverage area

 Desired quality of service

The purpose of this section is to provide you with information and explain tasks that you should complete to ensure the best operation of the system.

The following procedure is very helpful to check the coverage region and install the base station:

- Site survey (see the sub-clause A)

- Judge where to place bases on plan drawing (see the sub-clause B).

- For each base, check coverage tool (see the sub-clause C and CRS Tool Manual).

- Plot limit line for each base station.

- Check for shadow area.

- Adjust base position for shadow area or add base if not possible.

- Consider traffic 5 calls per cell add to plan.

- Install the base station (see the 5.4.2 Hardware Installation).

- Check with 33xH GAP handset using RSSI (displays only one base station at a time, limit is -80dbm: see the sub-clause D)

- Walk with 2 handsets, monitor for voice quality and ‘OUT OF RANGE’ message.

- If it is OK, finish it.

### A.  Site Survey

- Get building blue prints.

- Check building material and thickness.

Note the general construction materials used in walls and doors between multi-story buildings, and window coatings and coverings, if any. Also note any large metal objects, such as equipment, doors, and fluorescent lamp shades.

In general, signals are attenuated to some lower degrees when they should pass through any barrier, however, some materials such as metal attenuate much degree.

Note

- Consider high traffic areas, such as conference rooms, cafeterias, and manufacturing floors.

- Consider office locations and number of wireless terminal users within the site.

### B.  Base Station Broadcast Range and Location

- Base Station Requirements

To determine the number of base station in a system, you should consider base station broadcast range.

- Base Station Range

In a typical office environment, each base station has a broadcast range of 50 meters and supports an area of 8,000 square meters, but a broadcast range depends entirely upon office environment. Where the absence of any obstruction provides perfect line of sight conditions between the base station and wireless terminal users and if there are no atmospheric limitations, the range may increase to 250 meters.

The following base station broadcast ranges can be used as a rough guide to plan the base station positions:

- In line of sight, the base station has a range of upto 250m(open area).

- In hall, the base station has a range of 40-70m.

- In buildings, the base station has a range of about 25-40m. It assumes that walls are made of light brick, plasterboard or wallboard with metal frames. Normal electrical wiring, central heating pipes, office furniture and desktop computer equipment have no significant effect.

<Figure 5.4.1> Base Station Service Area

The characteristic antenna field pattern is a torus(doughnut) centered on the antenna.

- Base Station Location on the Plan Drawing

Determine the location of the base station on the plan drawing according to the previous information based on the base station broadcast range.

Once you have completed the steps described in Site Survey on sub-clause A and determined the number of base stations you need, you should determine where to install the base stations for optimum coverage and operation of the LDK-300 WOTS.

### Traffic Guidelines

Each base station supports five simultaneous calls, but because all users are not simultaneously in call statistically, a base station can support a greater number of wireless terminal users in practice.

LDK-300 WOTS

| Number of Cell (Base station) | Recommended Number of Registered Wireless Terminals | Number of maximum Simultaneous Wireless Conversation |
| --- | --- | --- |
| 1 | 8~192 by 8 steps | 5 |
| 2 |  | 10 |
| 3 |  | 15 |
| 4 |  | 20 |
| : |  | : |
| : |  | : |
| 23 |  | 96 |
| 24 |  | 96 |

<Table 5.4.1> Suitable Wireless Terminal Number according to Base Station Number

### D.  CRS(Cell-Coverage Region Survey) Tool

For the easy installation, LGE prepares the portable tools which consist of the base station and the wireless terminal.

This section provides general guidelines for determining location of base station in the site.

For more information, see the CRS Tool Manual.

<Figure 5.4.2> Installation Configuration using the CRS Tool

#### General Guidelines

- Place the base stations where to maximize direct line of sight between wireless terminals and base station antennas. Mounting base stations as high as possible will satisfy this guideline.

- Place the base stations where to minimize obstructions near antennas. If you mount a base station on a wall, place it higher than the average tall person and do not mount it near doors that could temporarily obstruct antennas when the door is opened.

- Antennas on wall-mounted base stations should be parallel to the wall.

- Centralize base stations within the area that you intend to cover.

- When rule 1 cannot be observed, place base stations so that signals begin their paths with a clear line of sight and then bend at the end of their travel to a receiving antenna. It limits the affect of signal fading.

- To minimize blocking in high traffic area, install the base stations required to meet the estimated traffic demand. When multiple base stations are needed in the same location, you may mount base stations as close as one meter apart (to avoid one obstruction to the other according to the rule 2).

- The base stations are designed to use frequencies that are not in use yet, so there is no problem of interference between base stations that are mounted near each other.

- In multi-story environments, it is the best to plan coverage for each floor separately. However, because RF signals can travel between floors, a single base station could support multiple floors as long as base station broadcast area, traffic requirements, and other general guidelines are kept in mind.

Usually, DECT wave is attenuated by reflection indoors. In order to minimize attenuation, when you install a base station on the wall, give at least three-wave-length intervals. The intervals should be at least 45 centimeters because the wavelength is 15 centimeters at DECT frequency.

The best communication environment is when a base station and a user are in the same height. In office environment, however, it is desirable to consider office furniture to minimize reflection, diffraction and scattering of DECT wave when you set the position of a base station.

The appropriate position for a base station is where 1.8 meter above from the floor and 0.5 meter below from the ceiling. The top of iron office furniture such as a cabinet or a desk is not a good position. Keep away from electronic equipment such as a copy machine, a printer or a computer. And the lower wall of a corridor where many people pass by, corner of a wall, and narrow indoors also should be avoided.

## 5.4.2	 Hardware Installation

### A.  Equipment Check

#### LDK-300 WOTS Equipment

- WTIB, WTIU (optional)

- Base stations (GDC-330B) , Connecting block and line cord, Ferrite core

- Wireless terminals (GDC-33xH), Single-slot charger (330-SSC), Battery (330-SBP) and Adapter (330-HAP)

<Figure 5.4.3> show the packing components of the WTIB and WTIU.

GDC-320B, GDC-33xH, Single-slot Charger(330-SSC), and Adapter(330-HAP) are shown at <Figure 5.4.4> and <Figure 5.4.5> in each.

<Figure 5.4.3.A> Packing Components of WTIB

<Figure 5.4.3.B> Packing Components of WTIU

Base station

GDC-330B

<Figure 5.4.4>  Packing Components of GDC-330B

Notes : Place the Ferrite Core close to a System(WTIB) side as close as it can be

Wireless Terminal Packing

<Figure 5.4.5>  Packing Components of GDC-33xH

### B.  Unit Installation

#### WTIB/WTIU Installation

In a LDK-300 Digital Key Telephone System, the WTIB is a base station interface board.

LDK-300 system can have three WTIB and WTIU.

If the WTIB is installed in a power-on system, the system and/or WTIB can be damaged. Thus after system power is off, the WTIB should be plugged in/out to/from the system.

If the WTIB is re-plugged into a system after extraction, the WTIB should be plugged into the original slot. Otherwise programmed data do not match the station numbers.

The WTIB can be installed in slot 1 through slot 8 of KSU, and should be installed at the same KSU in case of installing more than one WTIB.

The WTIB can have a WTIU daughter board. <Figure 5.4.6> shows the wiring diagram of the

WTIB.

<Figure 5.4.6> Wiring Diagram of WTIB

#### Base Station Installation

Before mounting base station permanently, you should determine a suitable location providing the best coverage.

##### Mounting Base Station - 1

The weight of a base station is light as approx. 460g, so it can be mounted on a masonry or dry-wall surface, wooden wall, or partition wall. The distance between mounting holes is 121mm as shown in <Figure 5.4.7>. The base station is mounted with 1 1/2 inch or longer screws. Drill pilot holes in the two locations, insert the screws and tighten leaving about 3mm gap between the wall and screw head. Mount the base station on the screws and tighten the screws securely.

<Figure 5.4.7> Wall Mounting of Base Station (without wedge)

Before mounting base station permanently, you should determine locations that provide the best coverage. The wall mounting procedure of a base station is as follows:

- Drill two 3.5 mm holes in a fixed wall. To properly position the holes, you can measure the tags on the base station or mark the surface through the tag eyelets while holding the base station in place.

- Insert anchors into the drilled holes. Then insert screws in the anchors, leaving a 3 mm gap between the wall and screw head.

- Mount the base station eyelets on the screws.

##### Mounting Base Station - 2 (Using Wedge)

Installing the Base Station using the wedge,

- Determine at first the location on wall where the wedge prepared for mounting the base station is to be fixed.

- The wedge should be settled by using two screws on the wall.

- And insert the base station pulling down into the wedge.

SCREW

<Figure 5.4.8> Wall Mounting of Base Station using Wedge

##### WTIB-Base Station Cabling

The Base Station is connected to the WTIB by a 1.152 Mbit/s transmission unshielded cable. The following rules should be obeyed:

The cable for connecting WTIB with a base station should be a twisted pair unshielded cable.

Shielded cable can be used if it can be interfered by others like AC power cable with high voltage. But it can shorten the cable length between WTIB and Base Station because its high capacitive component can degrade the shape of data. So we recommend to use unshielded cable.

The cable for connecting the WTIB with a base station shall be separately cabled. This connection shall be cabled separate to the building cabling using twisted 2-pair CAT5 unshielded cabling.

The following characteristics should be observed for all cables:

DC resistance: The resistance of a conductor should not exceed 73.4 ohm per km for a temperature of 20 degree.

Attenuation: The attenuation of any pair should not exceed 17 dB per km on condition of a temperature of 20 degree and frequency of 1 MHz.

Characteristic impedance: Characteristic impedance should remain in the range of 100 ohm _ 15% for a frequency of 1 MHz.

Crosstalk: The total crosstalk of any pair (Far-end and Near-end crosstalk), measured over a line length of 1800 metres should not exceed 40 mV.

#### Ferrite Core Installation

Ferrite core is provided in the package of Base station for EMI. Ferrite core should be installed when WTIB is installed in the key telephone system. One Ferrite core in the package of base station is for line cord between base station and each port of WTIB & WTIU.

#### Wireless Terminal Installation

To install a GDC-33xH(GAP), simply take out the handset and batteries (330-SBP), charger(330-SSC) and AC/DC adapter(330-HAP) from the package and assemble it. Before using the GDC-33xH, you should recharge the battery fully.

After inserting a battery, press the power button of GDC-33xH (GAP). <Figure 5.4.9> shows the initial LCD. The two icons in the left upside of the LCD are blinking until the GDC-33xH is subscribed.

<Figure 5.4.9> LCD Message of Unsubscribed GDC-33xH (GAP)

Before using the GDC-33xH(GAP), it should be subscribed. <Figure 5.4.10> shows the LCD message of the subscribed GDC-33xH(GAP).

<Figure 5.4.10> LCD Message of Subscribed GDC-33xH (GAP)

Refer to the 5.4.3 User subscription/de-subscription of this manual and the User Guide of the GDC –33xH for the detail information.

## 5.4.3 User subscription/desubscription

## A. System ID

- Description

System ID is given to the LDK-300 system attached the wireless GAP terminal for identification. System ID, which is PARK(Potable Access Rights Key), is written on MPB.

Authentication Code as well as system ID should be entered at only attendant station, before you begin to subscribe wireless terminal (GAP) to LDK-300 system.

- Operation

- In case of confirming system ID

- At attendant station

| [Trans/Pgm] | + | 0# | + | Flex. Button 4 |
| --- | --- | --- | --- | --- |

- If you press the [Trans/Pgm] button, [Trans/Pgm] LED will flash(60 IPM Flash) and [On/Off] LED will be turned on.. Then you can enter then user program mode.

- Dial  ‘0#’’

- Press Flexible Button 4

- Current PARK code will be displayed in LCD

- In case of entering system ID

- At attendant station

| [Trans/Pgm] | + | 0# | + | Flex. Button 6 | + | System ID(PARK) | + | [Hold/Save] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

CAUTION: Normally you should not change the system ID. If you should change it, please contact LG or local dealer in your country.

- If you press the [Trans/Pgm] button, [Trans/Pgm] LED will flash(60 IPM Flash) and [On/Off] LED will be turned on. Then you can enter then user program mode.

- Dial ‘0#’

- Press Flexible Button 6

- Enter the system ID(PARK code)

- After entering the PARK code, press [Hold/Save] button. Then you can hear confirm tone.

The key sequence of the PARK : LLP__________PC

LL       : Two digits decimal representation of PARK length (Bit count)

P______P : 11 octal digits  representation of PARK

C        : Check digit (It is calculated as the sum of each digit in the input stream multiplied by its position in the input stream , modulo 11; if the result if 10, this is represented by the “*”.)

- Conditions

- You must program for system ID when you install the system.

- If you program system ID, all data that were related to wireless features will be erased.

- The initial PARK value is 00000000000000. So the value will be display in LCD at first

## B. Authentication Code

- Description

Authentication Code is entered at only attendant station before you begin to subscribe wireless terminal(GAP) to LDK-300.

- Operation

- In case of confirming AC code

- At attendant station

| [Trans/Pgm] | + | 0# | + | Flex. Button 3 |
| --- | --- | --- | --- | --- |

- If you press the [Trans/Pgm] button, [Trans/Pgm] LED will flash(60 IPM Flash) and [On/Off] LED will be turned on.. Then you can enter then user program mode.

- Dial  ‘0#’’

- Press Flexible Button 3

- Current AC code will be displayed in LCD

- In case of entering AC code

- At attendant station

| [Trans/Pgm] | + | 0# | + | Flex. Button 3 | + | AC Code(Max 8 Digits) | + | [Hold/Save] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

The key sequence of the AC code : D______D

D_______D :  Up to 8 digits decimal representation.

- If you press the [Trans/Pgm] button, [Trans/Pgm] LED will flash(60 IPM Flash) and   [On/Off] LED will be turned on.. Then you can enter then user program mode.

- Dial  ‘0#’’

- Press Flexible Button 3

- Enter the AC code

- If you press the [Hold/Pgm] button, you will be heard confirm tone.

- Condition

- AC code is must be programmed for user subscription.

- AC code must be programmed one time after system was installed. If you change AC code under system operation, it may not operate properly.

(Ex : Subscribed terminal may not receive the incoming call or not make an outgoing call)

- If PARK value are changed, you must enter the AC code again.

- The initial AC code is 000000.

- AC code change will not affect the system operation except subscribing new wireless terminal

## C. User Subscription

- Description

This procedure is for subscribing the wireless terminal to LDK-300 system.

Operation

- Attendant station

| [Trans/Pgm] | + | 0# | + | Flex. Button 1 | + | Station Number + | Phone Type + | [Hold/Save] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

- If you press the [Trans/Pgm] button, [Trans/Pgm] LED will flash(60 IPM Flash) and [On/Off] LED will be turned on.. Then you can enter then user program mode.

- Dial  ‘0#’’

- Press Flexible Button 1

- Enter the station number.

- Enter the phone type (LG-GAP for LDK-300 system or standard GAP for any other system). In case of LG-GAP terminal,  press [1] button. Otherwise  press [2] button.

- Press [Hold/Save] button and confirmation tone is heard.

- If succeeded, the attendant station will display “ SUBSCRIBED: SUCCESS” on the LCD display.

- If failed(ether at attendant or at wireless terminal), repeat ③ ~ ⑥ steps of the attendant.

* Note : Station range of wireless terminal

How to confirm station range assigned to wireless terminal.

At attendant station,

| [Trans/Pgm] | + | 0# | + Flex. Button 9 |
| --- | --- | --- | --- |

- At the wireless terminal

| [Function] | + | 0 | + | PARK code | + | [Redial] | + AC code | + [Redial] | + FT No. + | [Function] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Press [Function()] button

- Press 0

- Enter system ID(PARK )

- Press [Redial ()] button

- Enter AC code

- Press [Redial ()] button.

- ‘1’,’2’,’3’,’4’ or ‘_’ are displayed on the LCD. A number means FT number that it is not occupied yet. The under-bar means the wireless terminal is already subscribed to the system. Therefore, the number of ‘_’ means the number of subscribed system. If 4 under-bar are displayed on the LCD, that means no subscription is allowed any more. Enter FT number by pressing numeric key if any number exists. After selecting, it is possible to re-select FT number by pressing [Flash(R)] button to cancel selection.

- Press [Function()] button

- Success or failure of the subscription is notified with confirmation tone to the attendant and the wireless terminal.

- If succeeded, the wireless terminal goes to an idle state and its station number is displayed.

- If failed, repeat ③~⑥ steps of the attendant and ①~⑧ steps of wireless terminal.

The key sequence of the PARK : LLP__________PC

LL       : Two digits decimal representation  of PARK length (Bit count)

P______P : 11 octal digits representation of PARK

C        : Check digit (It is calculated as the sum of each digit in the input stream multiplied by its Position in the input stream, modulo 11; if the result if 10, this is represented by the “*”.)

The key sequence of the AC code : D_____D

D_____D : AC value  :  Up to 8 digits decimal representation.

FTID : 1 ~ 4

Condition

- Wireless terminal must be subscribed to system for normal service.

- Only attendant can subscribe the wireless terminals(GAP).

- Attendant can subscribe another wireless terminal after one subscribing procedure.

- If attendant want to subscribe wireless terminal that was subscribed already, it will be heard error tone.

## D. User Desubscription

- Description

This procedure makes a wireless terminal loose access right to the LDK-300 system. To de-subscribe the wireless terminal, it should be in an idle state.

Operation

- Attendant station

| [Trans/Pgm] | + | 0# | + | Flex. Button 2 | + | Station Number + | [Hold/Save] |
| --- | --- | --- | --- | --- | --- | --- | --- |

- If you press the [Trans/Pgm] button, [Trans/Pgm] LED will flash(60 IPM Flash) and [On/Off] LED will be turned on.. Then you can enter then user program mode.

- Dial  ‘0#’’

- Press Flexible Button 2

- Enter the station number.

- Press [Hold/Save] button and confirmation tone is heard.

- Success or failure of the subscription is notified with confirmation tone to the attendant and the wireless terminal.

- If succeeded in de-subscription, the below LCD message will be displayed. Power-off the wireless terminal

Condition

- Only attendant can de-subscribe wireless terminal

- Attendant can de-subscribe wireless terminals those were subscribed already.(If attendant try to de-subscribe un-subscribed wireless terminal, it will be heard error tone.

- Attendant can de-subscribe another wireless terminal after one de-subscribing procedure.

- Attendant can do de-subscribing procedure only when wireless terminal is at the idle state.

* Notes : If you want to do de-subscribe procedure at wireless terminal and attendant station independently, Follow the procedure that described below.

- At attendant station

- To erase all data those were subscribed.

| [Trans/Pgm] | + | 0# | + Flex. Button 7 | + | Password(147*) + | [Hold/Save] |
| --- | --- | --- | --- | --- | --- | --- |

- Press the [Trans/Pgm] button, [Trans/Pgm] LED will flash(60 IPM Flash) and [On/Off] LED will be turned on.. Then you can enter then user program mode.

- Press 0#

- Press flexible button 7

- Enter password.

- Press [Hold/Save], you will be heard confirmation tone.

- To erase a special wireless terminal

| [Trans/Pgm] | + | 0# | + Flex. Button 8 |  | + Station Number + | [Hold/Save] |
| --- | --- | --- | --- | --- | --- | --- |

- Press the [Trans/Pgm] button, [Trans/Pgm] LED will flash(60 IPM Flash) and [On/Off] LED will be turned on.. Then you can enter then user program mode.

- Press 0# .

- Press flexible 8

- Dial station number that you want to de-subscribe.

- Press [Hold/Save], you will be heard confirmation tone.

Condition

- Wireless terminal must be de-subscribed to system for normal service.

- Only attendant can de-subscribe the wireless terminals.

- Attendant can de-subscribe another wireless terminal after one subscribing procedure.

- If attendant want to de-subscribe wireless terminal that was subscribed already, it will be heard error tone.

- At the wireless terminal

| [Function] | + | 9 | + | 0000 | + | [Function] | + | Select Park number + | [Function] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Press [Function()] button

- Press 9

- Enter User PIN number “0000”

- Press [Function()] button

- Select PARK number with UP(), DOWN() key.

- Press [Function()] button

- If the below LCD message is displayed. Power-off the wireless terminal

## 5.4.4  Summary of DECT Installation

- Check and prepare the equipment.

- Install a LDK-300 Digital Key Telephone System.

- Install the WTIB (and WTIU).

- Determine location of base stations.

- Install base stations.

- Connect the WTIB with base stations by twisted 2-pair unshielded cables.

- Install the ferrite cores on signal wires of base station line cord.

- Slot Assignments & Determination of the number of able to register wireless terminals.

- Reset the LDK-300 System.

10.  Program of the PARK at the attendant station.

11.  Register wireless terminal.

12.  Make an initial call.