6. CUSTOMER DATABASE PROGRAMMING

6.1	INTRODUCTION

The LDK Key Telephone System can be programmed to meet each customer's individual need. All programming is done at station 100 (station port # 00) using KD-36D or LKD-30/44 digital key telephone.

Upon entering the program mode, the key telephone at station 100 cannot operate as a normal telephone but as a programming instrument with all of the buttons redefined. The keys of the dial pad are used to enter the various data fields and to enter numerical information. The 24 buttons located at the top of the phone (Flexible Buttons) are used to indicate the specific data field and to enter information. Sometimes the [SPEED] button and ‘*’ of the dial pad is used to delete the data or to indicate end of data input and [REDIAL] button is used to delete one digit or character from the end of entered digits or chars.

See TABLE 1.6.1 - 1.6.35 for default data. If this pre-programming suits the customer, additional admin program is not necessary. To change admin data, the user enters the admin. programming mode and select program code. During admin programming other keysets operate normally.

When Admin programming, LCD and LEDs indicate the current programmed data and status. If the programmer enters correct data, then LCD and LEDs show the entered data and the data is stored in the temporary buffer area. Real system databases is not changed and has no effect on telephone operation unless permanent updating procedure is executed. Pressing [HOLD/SAVE] button, all data in the temporary buffer (same as LCD and LEDs show their status) is saved into permanent memory. Tones are provided to let the programmer know data entry is correct (confirmation tone) or not (error tone).

If programmer wants to parent state, press [CONF] button. When pressing [CONF] button temporary data fields are cleared.

If programmer wants to reset System, Enter PGM450 - Btn14 and press [HOLD/SAVE] button or System will be reset automatically after PGM100 – Btn1(Nation Code Assign)

## TO ENTER THE PROGRAMMING MODE

- 	Lift handset or press the [MON] button on the admin station, and hear ICM dial tone (optional).

-   Press the [TRANS/PGM] button and dial ＊# (Confirmation tone is heard).

- Enter Admin password if the password has been set. This places the station into the admin programming mode (confirmation tone heard).

- Each program is accessed by pressing the [TRANS/PGM] button and dialing the three-digit program number. If an error is made while entering data, the [TRANS/PGM] button can go the previous status. When the [TRANS/PGM] button is pressed, the LCD will display;

ENTER PROGRAM CODE

## PERMANENT UPDATE PROCEDURE

When the data has been entered, the [HOLD/SAVE] button is used to store the data permanently. If all data was entered correctly, confirmation tone is heard when pressing the [HOLD/SAVE] button. If there were any errors in the entry, then an error tone is presented and data is not stored in the permanent memory.

## 6.2	NUMBERING PLAN

The following numbering plan can be changed by Admin Programming 104-107 depending on the user's needs.

| NUMBER | ITEM | REMARK |
| --- | --- | --- |
| 100–399 | Station Number |  |
| 620–667 | Station Group Number |  |
| 501-535 | Internal Page Zones |  |
| 543 | Internal All Call Paging |  |
| 544 | Meet Me Page |  |
| 546 | External Page Zone –1 |  |
| 547 | External Page Zone –2 |  |
| 548 | External All Call Paging |  |
| 549 | All Call Paging |  |
| 550 | Enter SMDR Account Code | SLT |
| 551 | Flash Command to CO Line |  |
| 552 | Last Number Redial (LNR) |  |
| 553 | Do Not Disturb |  |
| 554 | Call Forward |  |
| 555 | Station Speed Dial |  |
| 556 | Message Wait Enable |  |
| 557 | Message Wait Answer |  |
| 558 | Speed Dial Access |  |
| 559 | Cancel DND/Call Forward/Pre-selected MSG |  |
| 563 | PGM Enter Code | SLT |
| 565 | Alarm Reset |  |
| 566 | Group Call Pick-up |  |
| 569 | Universal Night Answer (UNA) |  |
| 601-619 | Call Park Location |  |
| 7XXX | Direct Call Pick-up (XXX = Station Number) |  |
| 8XX | CO Group Access |  |
| 88XXX | CO Line Access |  |
| 89 | Tie Routing Access |  |
| 8## | Retrieve Holding CO Group | Keyset |
| 8#XXX | Retrieve Holding CO LIne | Keyset |
| 9 | Access the first CO line |  |
| 0 | Attendant Call |  |
| *0 | Print SMDR (Station Base) | Attendant |
| *1 | Print SMDR (Account Group Base) | Attendant |
| *2 | Delete SMDR (Station Base) | Attendant |
| *3 | Delete SMDR (Account Group Base) | Attendant |
| *4 | Display Call Charge on LCD | Attendant |
| *8 | VM Message Wait Enable |  |
| NUMBER | ITEM | REMARK |
| *9 | VM Message Wait Cancel |  |
| ** | Abort SMDR Printing | Attendant |
| #*1 | Door Open 1 |  |
| #*2 | Door Open 2 |  |
| #*3 | Door Open 3 |  |
| #*4 | Door Open 4 |  |
| [SPEED] + * | Last Number Redial |  |
| [SPEED] + # | Save Number Redial | Keyset |
| [SPEED] + 00-19 | Station Speed Dial Access | Keyset |
| [SPEED] + XXXX | System Speed Dial Access | XXXX : 2000 - 4999 (Bin No. of System Speed Dial) |

- To enter user programming mode, press the [TRANS/PGM] button in a keyset or dial 563 (Programming enter code) in a SLT.

- The following numbering plan is fixed, cannot be changed by Admin. Programming.

1) 	For the stations

| NUMBER | ITEM | REMARK |
| --- | --- | --- |
| 11 | Differential Ring | Keyset |
| 12 | Intercom Answer Mode (1 HF / 2 TONE / 3 PV) | Keyset |
| 21 | Station COS Change |  |
| 22 | Station COS Restore |  |
| 23 | Walking COS | Keyset |
| 31 | Authorization Code Registration |  |
| 32 | Authorization Code Change |  |
| 41 | Wake-up Time Registration (One-time/ Continuous) |  |
| 42 | Wake-up Time Cancel |  |
| 51 | Pre-selected MSG Activation | " |
| 52 | Set Custom Message |  |
| 61 | Record DVU User Greeting |  |
| 62 | Listen DVU Time & Date |  |
| 63 | Listen DVU Station Number |  |
| 64 | Listen DVU Station Status |  |
| 65 | Record DVU Page Message |  |
| 66 | Erase DVU User Greeting |  |
| 67 | Erase DVU Page Message |  |
| 71 | LCD Display Mode (English/Korean) | Keyset |
| 72 | MPB Version Display | Keyset |
| 73 | Background Music | Keyset |
| 74 | Station User Name Registration |  |
| 75 | Headset/Speakerphone Mode | Keyset |
| 78 | Serial No/SW Packages | Keyset w/ LCD |

2) 	For the attendant

| NUMBER | ITEM |  | REMARK |
| --- | --- | --- | --- |
| 0111 | Print SMDR (Station Base) |  | System Attendant |
| 0112 | Delete SMDR (Station Base) |  | System Attendant |
| 0113 | Print SMDR (Group Base) |  | System Attendant |
| 0114 | Delete SMDR (Group Base) |  | System Attendant |
| 0115 | Abort Printing |  | System Attendant |
| 0121 | Start Traffic Analysis | System Attendant |  |
| 0122 | Disable Traffic Analysis | System Attendant |  |
| 0123 | Print Traffic Analysis | System Attendant |  |
| 0124 | Abort Traffic Analysis | System Attendant |  |
| 021 | Station COS Change (COS 7) |  | Attendant |
| 022 | Station COS Restore |  | Attendant |
| 031 | Authorization Code Cancel |  | System Attendant |
| 041 | System Date/Time Setting |  | Attendant |
| 042 | Wake-up Time Registration (One-time /Continuous) |  | Attendant |
| 043 | Wake-up Time Cancel |  | System Attendant |
| 044 | LCD Date Mode Change |  | System Attendant |
| 045 | LCD Time Mode Change |  | System Attendant |
| 046 | Use PX Time & Date |  | System Attendant |
| 051 | Pre-select MSG Activation |  | Attendant |
| 052 | Pre-select MSG Deactivation |  | Attendant |
| 053 | Custom Display Message Program (11-20) |  | System Attendant |
| 054 | Erase VM MSG |  | Attendant |
| 06 | Record DVU System Greeting | System Attendant |  |
| 071 | DND/Call Forward/Pre-selected MSG Cancel |  | Attendant |
| 072 | Register Station Name |  | Attendant |
| 073 | Disable CO Outgoing |  | System Attendant |
| 074 | Automatic Day/Night Mode Program | Attendant |  |
| 075 | Door Phone Music Selection |  | Attendant |
| 076 | External Page Music -1 Assignment/Cancel | Attendant |  |
| 077 | External Page Music -2 Assignment/Cancel | Attendant |  |

3) 	Flex button programming codes

| NUMBER | ITEM |  | REMARK |
| --- | --- | --- | --- |
| 54 | [Record Conversation] BTN Assignment – With Voice Mail |  | Keyset |
| 55 | Restrict DID Ring |  | N/A |
| 56 | Restore DID Ring |  | N/A |
| 80 | [Account Code] BTN Assignment |  |  |
| 83 | [ICM Hold] BTN Assignment |  |  |
| 84 | [LOOP] BTN Assignment |  |  |
| 85 | [Camp-on] BTN Assignment |  |  |
| 86 | [ATD Override] BTN Assignment |  | System Attendant |
| 87 | [UCD DND] BTN Assignment |  | Member of UCD Group Only |
| 89 | Keypad Facility |  |  |
| 8* | [ACD] BTN Assignment |  | Keyset (Supervisor of UCD Group Only) |
| 91 | [CONF] BTN Assignment |  | 2/8 BTN Keyset |
| 92 | [CALLBK] BTN Assignment |  | 2/8 BTN Keyset |
| 93 | [DND/FWD] BTN Assignment |  | 2/8 BTN Keyset |
| 94 | [FLASH] BTN Assignment |  | 2/8 BTN Keyset |
| 95 | [MUTE] BTN Assignment |  | 2/8 BTN Keyset |
| 96 | [MON] BTN Assignment | 2/8 BTN Keyset |  |
| 97 | [REDIAL] BTN Assignment | 2/8 BTN Keyset |  |

## 6.3	ADMIN PROGRAMMING INDEX

| MAIN MENU | PGM | ITEM |
| --- | --- | --- |
| PRE-PROGRAMMED | 100 | Location Information |
|  | 101 | Slot Assignment |
|  | 102 | WTIB Port number Assign |
|  | 103 | Logical Slot Assignment |
|  | 104 | Numbering Plan Type |
|  | 105 | Flexible Number Plan – Station Number |
|  | 106 | Flexible Number Plan A |
|  | 107 | Flexible Number Plan B |
|  | 108 | IP Setting |
| STATION BASE PROGRAM | 110 | Station ID Assignment |
|  | 111 | Station Attribute I |
|  | 112 | Station Attribute II |
|  | 113 | Station Attribute III |
|  | 114 | ISDN Station Attribute |
|  | 115 | Flex Buttons Assignment |
|  | 116 | Station COS |
|  | 117 | CO Line Group Access |
|  | 118 | Internal Page Zone Access |
|  | 119 | Conference Page Zone |
|  | 120 | ICM Tenancy Group |
|  | 121 | Preset Call Forward |
|  | 122 | Hot/Warm Line Selection |
|  | 123 | CTI Attribute |
|  | 124 | SMDR Account Group |
| CO LINE BASE PROGRAM | 140 | CO Service Type |
|  | 141 | CO Line Attribute I |
|  | 142 | CO Line Attribute II |
|  | 143 | ISDN CO Line Attribute |
|  | 144 | CO Ring Assignment |
| SYSTEM BASE PROGRAM | 160 | System Attribute I |
|  | 161 | System Attribute II |
|  | 162 | ADMIN Password |
|  | 163 | Alarm Attributes |
|  | 164 | Attendant Assignment |
|  | 165 | Auto Attendant DVU Annc.# |
|  | 166 | CO-to-CO COS |
|  | 167 | DID/DISA Destination |
|  | 168 | External Control Contact |
|  | 169 | LCD Date/Time/Language Display Mode |
|  | 170 | Modem |
|  | 171 | Music |
| MAIN MENU | PGM | ITEM |
| SYSTEM BASE PROGRAM | 172 | PBX Access Code |
|  | 173 | PLA Priority |
|  | 174 | RS-232C Port Setting |
|  | 175 | Print Serial Port Selection |
|  | 176 | Pulse Dial / Speed Ratio |
|  | 177 | SMDR Attributes |
|  | 178 | System Date/Time |
|  | 179 | Linked Station Pairs Table |
|  | 180 | System Timers – I |
|  | 181 | System Timers – II |
|  | 182 | System Timers – III |
| STATION GROUP | 190 | Station Group Assign |
|  | 191 | Station Group Attribute Assign |
| ISDN SYSTEM BASE PROGRAM | 200 | ISDN Attributes 1 |
|  | 201 | COLP Table |
|  | 202 | MSN Table |
| TABLES | 220 | LCR Assignment |
|  | 221 | LCR - LDT Table |
|  | 222 | LCR - DMT Table |
|  | 223 | LCR Table Initialization |
|  | 224 | Toll Exception Table – Allow A (Entry no:01-30) |
|  |  | Toll Exception Table – Deny A (Entry no:01-30) |
|  |  | Toll Exception Table – Allow B (Entry no:01-30) |
|  |  | Toll Exception Table – Deny B (Entry no:01-30) |
|  | 225 | Canned Toll Table –Allow (Entry no:01-10) |
|  |  | Canned Toll Table –Deny (Entry no:01-10) |
|  | 226 | Emergency Code Table (Entry no:01-10) |
|  | 227 | Authorization Code Table (Entry no:001-600) |
|  | 228 | Customer Call Routing |
|  | 229 | Executive/Secretary Table |
|  | 230 | DID Digit Conversion Table |
|  | 231 | Flexible DID Table |
|  | 232 | System Speed Zone |
|  | 233 | Weekly Time Table |
|  | 234 | Voice-Mail Dialing Table |
|  | 235 | Tie Routing Table |
| Hotel | 300 |  |
| Q-SIG | 320 |  |
| VOIP | 340 |  |
| NATION SPECIFIC | 400 | DTIB Rx Gain Control |
|  | 401 | SLIB Rx Gain Control |
|  | 402 | SLIB12 Rx Gain Control |
|  | 403 | WTIB Rx Gain Control |
| MAIN MENU | PGM | ITEM |
| NATION SPECIFIC | 404 | ACOB Rx Gain Control |
|  | 405 | ACOB8 Rx Gain Control |
|  | 406 | DCOB Rx Gain Control |
|  | 407 | VMIB Rx Gain Control |
|  | 408 | DTRU Rx Gain Control |
|  | 409 | EXT Pager Rx Gain Control |
|  | 410 | CPTU Rx Gain Control |
|  | 411 | MODU Rx Gain Control |
|  | 418 | PTRP Rx Gain Control |
|  | 419 | PTRP Rx Gain Control |
|  | 420 | SYSTEM Tone Frequency |
|  | 421 | Differential Ring Frequency |
|  | 422 | Distinct CO Ring Frequency |
|  | 423 | ACNR Tone Cadence |
|  | 424 | Tie Routing Table |
|  | 425 | Singular Table(Korea Only) |
| INITIALIZATION (DB INIT) | 450 | Flexible Numbering Plan Initialization |
| PRINT DATABASE | 451 | Flexible Numbering Plan Print |