SECTION 2. GENERAL DESCRIPTION

### INTRODUCTION

The LDK-300 Digital Key Telephone System is a fully digital hybrid Key Telephone System, designed to meet the telecommunication needs of medium sized business offices.

The LDK-300 System incorporates state of the art digital technology for command processing and voice switching, utilizing a Pulse Code Modulation/Time Division Multiplexing (PCM/TDM, “A” law) distributed switching matrix.

The LDK-300 achieves a high level of flexibility by 1) employing a Universal Card Slot architecture with  the 1’st, the 2’nd and the third cabinet to house plug-in Printed Circuit Boards, and 2) providing support for different types of instrumentation.

The KSU of LDK-300 is a wall-mounted cabinet that houses the MB(Mother Board) and card slots for the CO line/Key Station/SLT/ISDN/LAN interface boards, and other useful PCBs. There are three stories KSU in the system. The first KSU is basic KSU. On the other hand, the second and the third one is the expansion KSU.  MPB should be installed in the fixed MPB slot in the first KSU. LMU1 should be installed in the fixed LMU slot in only first KSU. LMU2 can be used with a linked cable when the second or the third KSU is expended and installed in the fixed LMU slot in only expansion KSU. There are two kinds of PSUs that are installed in the fixed PSU slot in three KSU. One of them is PSU3 that is used in the GDK-100 system, the other is developed for increasing power capacity.

The system architecture has been designed to allow a high level of software control over the system's hardware. The software incorporates a vast array of features and capabilities including PC Database Administration, Auto Route Select, ACD, etc.

The LDK-300 system supports a combination of Digital Keysets (KD, KD/E, KD/S, KD/C, LKD series), various kinds of ISDN terminals, and wireless terminals as well as analogue single line devices. With the keysets, commonly used features are activated by direct button selection. Additionally, many functions may be accessed by dialing specific codes or optionally, by assigning these dial codes to Flexible Buttons on the keyset. In addition to key telephones, an array of optional terminals is available including DSS/DLS Console, Intercom/Door Box.

With the flexibility of the LDK-300 extensive feature content, and the capability to use an array of instruments, the LDK-300 can be tailored to meet the short and long term needs of the most demanding customer requirements.

### SYSTEM CAPACITY

The following Table and Chart provide system capacities and displays the configuration flexibility of the system.

### System Capacity

- KSU contains two kinds of PSU slots, nine universal slots, one MPB slot, one LMU slot, and one RAU slot.

- There are two kinds of PSUs(Power Supply Unit) . The first one is PSU3 which is used in the GDK-100 system and the other one is developed for the power capacity.

| PSU | 5V | -5V | 30V | Battery |
| --- | --- | --- | --- | --- |
| PSU3(250W) | 5A | 1A | 7A | 1A |
| New PSU(350W) | 10A | 1.5A | 8.4A | 1A |

Table 2.2.1 Power Supply Capacity

Max capacity of system is described in the table 2.2.2 max. call capacity of system, and system capacity is in the table 2.2.3 system capacity.

| KSU | Max Port (Number of Boards) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Extension |  |  | Network Side |  |  |
|  | SO (STIB | DKTU (DTIB24) | SLT (SLIBII) | PRI (PRIB) | BRI (BRIB) | LCO (LCOB) |
| 1’st KSU | 56(7) | 96(4) | 96(8) | 120(4) | 56(7) | 32(8) |
| 1’st KSU + 2’nd KSU | 112(14) | 192(8) | 19216) | 150(5) | 112(14) | 68(17) |
| 1’st KSU + 2’nd KSU + 3’rd KSU | 152(19) | 288(12) | 288(24) | 150(5) | 152(19) | 104(26) |

Table2.2.2 System Max. call capacity

| Description |  | Capacity | Description |  | Capacity |
| --- | --- | --- | --- | --- | --- |
| Time Slots (note A) | 1’st KSU | 144 | Paging Zone | External | 3 (MPB:1, MISB:2) |
|  | 2’nd KSU | 144 |  | Internal | 35 |
|  | 3’rd KSU | 96 | System Speed Dial |  | 3000(24digits) |
| Cabinet(KSU) |  | 3 | Station Speed Dial |  | 100(24digits) per station |
| Serial Port(RS232C) |  | 4(MPB:2, MISB:2) | Last Number Redial |  | 10 |
| Alarm/Door bell input |  | 3 (RAU:2,MISB:1) | Save Number Redial |  | 10 |
| External Control Contact |  | 7 (RAU:3, MISB:4) | DSS/DLS Console |  | 7 |
| Music Source Inputs |  | 2 (MPB:1, MISB:1) | SMDR |  | 5000 |
| DTMF Receivers |  | 47 (note B) | CO Line Group |  | 172 |
| Power Fail Circuit |  | 90 (30/cabinet) | Hunt/UCD Group |  | 48 |
| Intercom Group |  | 15 |  |  |  |

Table2.2.3 System capacity

* Note A ;

-. Time slots are used to allocated flexibly to support each function PSTN, ISDN, VMIB,WTIB, MOH,

Paging, Modem, DTMF detection ,etc.

-. Maximum ports of first KSU are 132 port

* Note B ;

-. Basic 4 channels : MPB

-. Option        : 2chs/DTRU, 4/DTRUII, 4/DTRU4

* Note : Not all the maximum capacities may be achieved simultaneously.

For the maximum number of cards in a system, see the section 2.4

### System Configuration Chart

### Overview

The maximum capacity of the system configured only with the wired extensions will be limited by the physical condition of the system, e.g. the quantity of slots and cards limitation. System configuration charts are ass follows;

Fig. 2.2.1 System configuration charts

### SYSTEM SPECIFICATIONS

The following Tables provide general system specifications.

### Dimension and weight

|  | Height(mm/in) | Width(mm/in) | Depth(mm/in) | Weight(kg/lbs) |
| --- | --- | --- | --- | --- |
| KSU |  |  |  |  |
| Digital Keyset | 236/9.3 | 192/7.6 | 84/3.3 | 1.5/3.3 |
| Digital DSS/DLS Console | 236/9.3 | 125/4.9 | 62/2.4 | 0.9/2.0 |
| Digital ICM/Door Box | 45/1.8 | 140/5.5 | 100/3.9 | 0.5/1.1 |
| Digital Data Module | 37/1.5 | 175/6.9 | 148/5.8 | 1.5/3.3 |
| Base Station(GDC-330B) | 170 | 220 | 57 | 0.46 |
| Wireless Terminal | 145 | 50 | 35 | 0.15 |

Table2.3.1 Dimension and weight

### Environment specification

|  | Degrees (C) | Degrees (F) |
| --- | --- | --- |
| Operation Temperature | 0~40 | 32~104 |
| Optimum Operation Temperature | 20~26 | 68~78 |
| Storage Temperature | 10~70 | 32~158 |
| Relative Humidity | 0~80% non-condensing |  |

Table2.3.2 Environment specification

### Electrical specification

### System Electrical specification

|  | Specification |
| --- | --- |
| Power Supply AC Voltage Input AC Power Heat Dissipation AC Input Fuse DC Output Voltage | 110/220 or 240 +/- 10% Volt AC @48~63Hz PSU3 : 250W, LDK-300 PSU: 350W Max. 615 BTUS 4.0 amp @ 220~240Volt AC, 6.0 amp @ 110Volt AC + 5, - 5, + 30Volt DC |
| Battery Backup PSU Input Voltage PSU Battery Fuse Charging Current | 24Volt DC 10amp @250Volt AC Max. 500mA |
| External Relay Contact | 1amp @24Volt DC |
| Music Source Input | 600ohm @0dBm |
| External Paging Port | 600ohm @0dBm, 1mw max. |

Table2.3.3 System Electrical specification

### Base Stion specification

|  | Specification |
| --- | --- |
| Power feeding | +30Vdc |
| Transmission Max Power | 250mw |
| Access Method/Duplex | TDMA/TDD |
| Frequency Band | 1880~1900MHz |
| Channel Spacing | 1.728MHz |
| Modulation | GFSK |
| Data rare | 1.152Mbps |
| Max. Base Station distance from the WTIB | 600m (twisted 2-pair cable) |

Table2.3.4 Base Station specification

### Max. Station Distance from the System

|  | AWG 22 (m/kft) | AWG 24 (m/kft) |
| --- | --- | --- |
| Digital Keyset | 500 /1.6 | 330/1 |
| Single Line Telephone SLIB24/SLIBII/SLIB4E SLIB48 | 2,500/8.2 5,000/16.5 | 1,600/5/2 3,300/10 |

Table2.3.5 Max. Station Distance from the System

### CO loop specification

|  | Specification |
| --- | --- |
| Ring Detect Sensitivity | 40Vrms @16~30Hz 30Vrms @30~37Hz |
| DTMF Dialing Frequency Deviation Signal Rise Time Tone Duration, on time Inter-digit Time | Less than +/-1 % Max. 3ms Min. 75ms Min. 75ms |
| Pulse Dialing Pulse Rate Break/Make Ratio | 10pps or 20 pps 60/40% or 67/33% |

Table2.3.6CO loop specification

### SYSTEM COMPONENTS

The following table shows the available slots in which every card can be installed. Some boards for GDK-100 can be used in LDK-300 system. Others are used in only LDK-300 system.

| No | Board Name | Number of Ports | Slot Position |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1’st KSU |  | 2’nd, 3’rd KSU |  |
| 1 | KSU |  |  |  |  | Key Service Unit |
| 2 | PSU |  | PSU |  |  | Power Supply Unit, 350W |
| 3 | PSU3 |  | PSU |  |  | Power Supply Unit, 250W |
| 4 | MPB |  | MPB |  |  | Main Processor Board |
| 5 | MISB |  | 9 only |  |  | Miscellaneous Board |
| 6 | DTIB12 | 12 | 1~9 |  |  | Digital Terminal Interface |
| 7 | DTIB24 | 24 | 1~9 |  |  | Digital Terminal Interface |
| 8 | SLIB24 | 6 | 1~9 |  |  | SLT Interface(+24V Feed) |
| 9 | SLIB48 | 6 | 1~9 |  |  | SLT Interface(-48V Feed) |
| 10 | DSIB | 12 | 1~9 |  |  | Digital Terminal(6) and SLT Interface (+24VFeed) |
| 11 | SLIBII | 12 | 1~9 |  |  | SLT Interface (-28V Feed) |
| 12 | LCOB | 4 | 1~9 |  |  | Loop Start CO Interface |
| 13 | STIB | 4 (8B) | 1~8 |  | 1~9 | ISDN So/To Interface (2B+D) |
| 14 | BRIB | 4 (8B) | 1~8 |  | 1~9 | ISDN Basic Rate Interface (2B+D) |
| 15 | PRIB | 1 (30B | 1~8 |  | 1~9 | ISDN Primary Rate Interface(30B+D) |
| 16 | VMIB | 4 | 1~8 |  | 1~9 | Voice Mail Interface Protocol |
| 17 | WTIB | 4 | 1~8 |  |  | Wireless Terminal Interface Board |
| 18 | VOIB | 4 | 1~8 |  | 1~9 | Voice Over Internet Protocol Interface Board |
| 19 | LCOB8 | 8 | 1~9 |  |  | Loop Start CO Interface |
| 20 | SLIB4E | 12 | 1~9 |  |  | SLT Interface (-42V Feed) |
| 21 | LMU1 |  | LMU |  |  | Link Module Unit (used in only 1’st KSU) |
| 22 | LMU2 |  | LMU |  |  | Link Module Unit (used in only 2’nd, 3’rd KSU) |
| 23 | RAU |  | RAU |  |  | Relay Alarm Unit (used in only Rack1) |
| 24 | MODU |  | MPB |  |  | 33.6Kbps Baud rate Internal Modem |
| 25 | SIU |  | MISB |  |  | Serial Interface Unit |
| 26 | MSGU |  | SLIB24, DSIB |  |  | Message Waiting Unit |
| 27 | MSGU48 |  | SLIB48 |  |  | Message Waiting Unit |
| 28 | MSGUII |  | SLIBII, SLIB4E |  |  | Message Waiting Unit |
| 29 | DTRU (GDK-34) |  | SLIB24,SLIB48,DSIB,LCOB,EMIB,DIDB |  |  | DTMF Receiver Circuits |
| 30 | DTRUII |  | SLIBII,SLIB4E, LCOB8 |  |  | DTMF Receiver Circuits |
| 31 | CPTU/A |  | LCOB,E1IB |  |  | Call Progress Tone Detection :305Hz~640Hz |
| 32 | CPTU/B |  | LCOB,E1IB |  |  | Call Progress Tone Detection : 350, 620, 440 and 480Hz±1% |
| 33 | CPTU4 |  | LCOB8 |  |  | Call Progress Tone Detection |
| 34 | CMU (GDK-162) |  | LCOB |  |  | Call Metering Unit (16K,12PR,50PR) |
| 37 | CMU4 |  | LCOB8 |  |  | Call Metering Unit (12/16PR,50PR) |
| 38 | FMEU |  | VMIB |  |  | Flash Memory Expansion Unit (32MB) |
| No | Board Name | Number of Ports | Slot Position |  |  | Description |
|  |  |  | 1’st KSU | 2’nd, 3’rd KSU |  |  |
| 39 | VCEU |  | VMIB |  |  | Voice Channel Expansion Unit |
| 40 | WTIU |  | WTIB |  |  | Wireless Terminal Interface Board |
| 41 | PFTU |  |  |  |  | Power Failure Transfer Unit |
| 42 | RGU5 |  |  |  |  | Ring Generator (25Hz) : Step Sine Wave, External |
| 43 | RGU6 |  |  |  |  | Ring Generator (25Hz) : Step Sine Wave, Internal |
| 44 | RGU |  |  |  |  | Ring Generator (20Hz/25Hz) : Step Sine Wave, External |
| 45 | TERM (GDK-162) |  |  |  |  | 100 ohm Termination for BRI and STI |
| 46 | GDK-TRC1 |  | PRIB/BRIB/STIB/WTIB/DVIB |  |  | Trace tool for the purpose of the maintenance |
| 47 | MOHU |  | Connected to SLT port |  |  | External Music On Hold Unit |

Table2.4.1 System Card Description

The following Fig 2.4.1 System configuration diagram shows the LDK-300 system configuration for all kinds of card in this system that includes the existing boards for the GDK-100 system and the developed new boards.

Fig 2.4.1 System Configuration

### KSU (Key Service Unit)

The KSU is a metal frame cabinets designed for wall mounting. It contains a back plane with 16 connectors which are 2 PSU slots, 9 universal slots, MPB slot, LMU slot, RAU slot , PFTU and RGU connectors. LDK-300 system is 3 story KSU which are the 1’st, 2’nd and 3’rd KSU. It’s possible to install all kinds of boards according the table 2.4.1. Cable ties are located on the front edge of the lower card rail

Fig 2.4.2 KSU structure

Fig 2.4.3  Board Configuration of MB

to allow connecting cables for the station boards, CO interface boards, and MISB to be neatly installed. The connecting cables for the stations and CO Lines exit through the outlet in the bottom of KSU and can be connected to the MDF or to a user installed termination point.

MB contains various kinds of connectors as in the Fig 2.4.3 Board Configuration of MB. Although it is possible to install either LDK-300 PSU or GDK-100 PSU3 in the CN12 or CN13, we recommend LDK-300 PSU when considering the power capacity. There are nine universal slots in the MB(CN1~CN9). Any kind of peripheral card install in the universal slot. But peripheral card that contains CPU can not be installed in the slot #9 in the first KSU. Fixed slot for MPB is CN10, LMU is CN11, RAU is CN16. RGU can be installed in the right bottom side of KSU through CN14. PFTU is connected to the CN14.

### LMU1/LMU2 (Link Module Unit)

LMU is Link Module Unit which provides link board between the KSUs. LMU1 can be installed when the second KSU is used. And LMU2 is used for the expansion of the 3rd KSU. All the signals can transmitted or received through LMU by the linked cable.

Fig 2.4.4  Board Configuration of LMU1

###### Fig 2.4.5  Board Configuration of LMU2

### PSU (Power Supply Unit)

A Power Supply Unit, PSU, is required in the Key Service Units. The PSU converts commercial AC power (110 –127 / 220 - 240 VAC @ 50-60 Hz) to DC voltages, regulates the voltages, and provides the appropriate DC voltages to the back plane for distribution to other system components. Three DC outputs are provided : ±5 VDC, +28 VDC. LEDs in the PSU front panel indicate valid outputs as well as the presence of AC input power.

The PSU includes circuitry to float charge externally connected 24 volt batteries and control operation of the battery back-up circuits. And the PSU will provide system operating voltages from the batteries if commercial AC power fails. In addition, battery back-up control circuitry is incorporated in the PSU to disconnect the batteries prior to a deep discharge or over-charged.

There are two kinds of PSUs; LDK-300 PSU and PSU3. See the clause; table 2.3.3 and the clause 3.3.5.

### RGU (Ring Generator Unit)

The Ring Generator Unit provides the Ring voltage to the SLIB circuits to ring the SLT. Also the RGU provides the input to the Message Wait source on the SLIB cards. The output of the RGU is 65VAC, 25 Hz. The RGU can support simultaneous ringing for 10SLTs.

- 25Hz pure sine wave, internal

RGU6 for internal in and RGU5 for external GDK-100 can be used, if you have.

The internal RGU is fitted on the lower right side panel inside cabinet, the external RGU is mounted suitable place outside the cabinet. And all RGUs are electrically connected to the back plane, regardless of RGU type.

### PFTU (Power Failure Transfer Unit)

The PFTU provides the relay contacts for transfer of 6 CO Lines to 6 SLTs in the event of a power or processor failure. The PFTU is installed outside of the KSU and up to 5 PFTU modules can be connected to the KSU with the cascade connection of control wire. The PFTU is equipped with a manual switch that activates the Power Failure Transfer mode for testing purpose.

A 2-pin connector with screws is fitted for the control signal. And a RJ21 type male connector is fitted for the connection between CO line, CO interface circuit, SLT interface circuit and SLT.

Fig 2.4.6 PFTU Configuration

### MPB (Main Processor Board)

Fig 2.4.7 MPB Configuration

The MPB controls and manages the communication between peripheral interface, supervises all resources in the system, controls the gain adjustment of PCM signal, generates system tone and manages call processing of the system.

The MPB, incorporates the system's RAM, master clock, 1 external MOH port, 1 internal MOH port, 1 external paging port, 2 RS-232C ports and RTC as well as the system's PCM voice processing and main micro-processor. The micro-processor is a 32 bit high speed RISC CPU which receives and transmits signaling information from/to other PCBs, controls feature activation, and PCM time-slot interchange. The MPB contains 4Mbytes of RAM which is associated with the system database and the real-time-clock are protected from power loss by a long life high energy lithium battery. On board ROM contains PCM tone, gain table etc. needed for the digital voice processing.

The 8 Mbytes of flash ROM on PMU contains basic system operating software.

32Mbytes of SDRAM(Synchronous Dynamic Random Access Memory and MODU(MODem Unit) can be installed on MPB, if required.

MPB should be installed in the MPB slot in only the first KSU.

Add-on boards: MODU, 32Mbytes of SDRAM module

### MISB (Miscellaneous Board)

The MISB incorporates the circuitry and interfaces for common optional features including:

 External Paging			2 ports

 External Control Contacts		4 contacts

 External BGM & MOH		2 ports

 Alarm/Door Detection		1 input

 RS-232C(optional SIU)		2 ports

A RJ21 type female connector is mounted on the front edge of the MISB for the connection to above circuits except RS232C serial interfaces.

One MSIB board can be installed only in only the first KSU..

Add-on board: SIU

### VMIB (Digitized Voice Interface Board)

Voice Mail Interface Board (VMIB) provides announcements for the system's ACD/UCD features as well as the System Voice Prompts and Recorded User Greetings. Following table shows the capacity of memory, ADPCM channel of VMIB, and its optional board.

| Item | Basic | Option(for expansion) | Maximum Capacity |  |
| --- | --- | --- | --- | --- |
|  |  |  | a board | a system |
| Channels | 4 channels | 4channels | 8 channels | 24 channels |
| System Voice Prompt ACD/UCD Announcement User Greeting | 300 min. | 300 min. | 600 min. | 1800 min. |

Table 2.4.2 Capacity of Memory, ADPCM channel of VMIB

- VMIB allows a FMEU or a VCEU to be expanded.

- LDK-300 allows up to 3 VMIB board to be installed.

###### Fig 2.4.8 VMIB Configuration

There are two option boards, one is VCEU for voice channel expansion and the others are FMEU for flash memory expansion.

VMIB can be installed in any of slot No. 1-8 in the 1’st KSU, any of slot No. 1-9 in the 2’nd and 3’rd.

And maximum three boards can be installed in a system.

### Extension Boards

The various types of LDK-300/GDK-100 Extension Boards have capable of supporting various types of terminals as followings,

| Board Name | Function | Remark |
| --- | --- | --- |
| DTIB12 | Provides 12 Digital Terminal interfaces | 2 wire |
| DTIB24 | Provides 24 Digital Terminal interfaces | 2 wire |
| SLIB24 | Provides 6 SLT interfaces with a line voltage of +24V. | 2 wire |
| SLIB48 | Provides 6 SLT interfaces with a line voltage of -48V. | 2 wire |
| SLIBII/SLIB4E | Provides 12 SLT interfaces with a line voltage of -28V. | 2 wire |
| DSIB | Provides 6 Digital Terminal and 6 SLT with a line voltage of +24V. | 2 wire |
| WTIB/WTIU | Be capable of accommodating up to 8 Base Stations. (up to 64 wireless terminals) | 4 wire |
| STIB | Be capable of accommodating up to 4 ISDN BRI S interfaces, 8 B channels. (Each interface is To/So switchable port) | 4 wire |

Table 2.4.3 Extension boards

### DTIB12/DTIB24 (Digital Terminal Interface Board 12/24)

DTIB allows the LGE proprietary digital terminals to be connected. DTIB provides 2-wire interfaces for terminal connection and three versions according to the capacity are available;

| Board Name | DKT interface circuits | SLT interface circuits | Remarks |
| --- | --- | --- | --- |
| DTIB12 | 12 ports | None |  |
| DTIB24 | 24 ports | None |  |
| DSIB | 6 ports | 6 ports | See clause 2.4.9.5 |

Table 2.4.4 Capacity of digital terminals

- Note:

There are KD, KD/E, KD/C, LKD series DKTU, KD and KD/E DSS, Digital Phone box, and external CTI module as LG digital terminals. KD, KD/(E)C, external CTI module and Data Module are not available now due to the production stop.

See clause 2.4.12.3 2B-module and clause 2.4.12.4 GDK-PC phone on section 2 for more details regarding 2B-module and CTI module.

Digital Interface Board provides digital voice and data communications to/from digital terminals. The board allows for either 1 or 2 bearer channels from a single hardware interface under the control of system software.

A industry standard RJ21 type female connector is mounted on the front edge of the PCB for connection to the station interfaces. In addition, one LED is mounted on the PCB to indicate the in use state of the connected terminals. It will turn on when one or more ports are busy.

DTIB12/DTIB24 can be installed in any of slot No. 1-9 in any KSU.

### SLIB24/SLIB48 (Single Line Interface Board with 6 ports)

SLIB allows single line analog devices to be connected.

Five kinds of SLIB boards are available in LDK-300/GDK-100 system;

| Board Name | Interface circuit | Feed voltage | Remark |
| --- | --- | --- | --- |
| SLIB24 | 6 ports | +24VDC battery feed |  |
| SLIB48 | 6 ports | -48VDC battery feed |  |
| SLIBII | 12 ports | -28VDC battery feed | See clause 2.4.9.3 |
| SLIB4E | 12 ports |  |  |
| DSIB | 6 ports | +24 VDC battery feed | See clause 2.4.9.5 |

Table 2.4.5 Capacity of SLT boards

- Note

DSIB provides 6 digital terminal interfaces and 6 single line analog interfaces.For SLIB II and DSIB, see the parts of SLIBII and DSIB in this section.

SLIB24/SLIB48 are capable of accommodating up to 6 single line analog devices. The boards are configured with hybrid circuit and CODEC; converts A/D and D/A signal. SLIB24/SLIB48 allow the SLT access to CO Lines connected to the system, other stations, and most features of the system through the use of dial codes. Connections to the SLIB are made through RJ21 type female connector.

The SLIB24/SLIB48 can be equipped with a Message Wait source (optional MSGU for SLIB24 or MSGU48 for SLIB48), which activate a lamp in the SLT, if so equipped. The Message Wait source sends 90 VDC signal to the SLT illuminating the MW lamp. A Ring Generator Unit is required in the cabinet where a SLIB is installed. An optional DTMF receiver unit (DTRU) can be installed on the SLIB24/SLIB48.

The SLIB24 allows a Single Line Telephone to be connected up to 2.5 Kilometers (8,200 feet, loop resistance: 490 ohm) from the system using 22 AWG wire. The SLIB48 allows a SLT to be connected up to 5 Kilometers (16,500 feet, loop resistance: 790 ohm) from the system using 22 AWG wire. See the Table 2.3.5.

In addition, one LED is mounted on the PCB to indicate the in use status of ports connected to SLIB. It will turn on when one or more ports are busy.

The SLIB24/SLIB48 can be installed in any of slot No. 1-9 in any KSU.

The maximum extension boards are limited by 300 extensions in a system.

Add-on boards: DTRU, MSGU (for SLIB24), MSGU48 (for SLIB48)

### SLIB II (Single Line Interface Board with 12 ports)

SLIB II provides the capability to connect 12 single line analog terminals to the system with appropriate A/D and D/A conversions. SLIB II is -28 VDC feeding voltage. The SLIB II allows the SLT access to CO Lines connected to the system, other stations, and most features of the system through the use of dial codes. Connections to the SLIB II are made through RJ21 type female connector.

The SLIB II is equipped with a Message Wait source, optional MSGU II, which activate a lamp in the SLT, if so equipped. The Message Wait source sends 90 VDC signal to the SLT illuminating the MW lamp. A Ring Generator Unit is required in cabinet where a SLIB II is installed.

An optional DTMF receiver unit (DTRU II) can be installed on the SLIB II.

The SLIB II allows a Single Line Telephone to be connected up to 2.5 Kilometers (8,200 feet, loop resistance: 490 ohm) from the system using 22 AWG wire. See the Table 2.3.5.

In addition, an LED is mounted on the PCB to indicate the status of ports connected to SLIB II. It will turn on when one or more ports are busy.

SLIB II can be installed in any of slot No. 1-9 in any KSU.

The maximum extension boards are limited by 300 extensions in LDK-300 system.

Add-on boards: DTRU II, MSGU II

### SLIB4E (Single Line Interface Board with 12 ports)

### DSIB (DKTU/SLT Interface Board with 12 ports)

DSIB provides 6 SLT and 6 DKT interfaces. Connections to the DSIB are made through RJ21 type female connector. An LED is mounted on the PCB to indicate the status of ports connected to SLIB. It will turn on when one or more ports are busy.

6 DKT interface circuits on this board have the same characteristics that those on DTIB12/24 do. See the part of DTIB12/DTIB24 in this section.

6 SLT interface circuits on this board also have the same characteristics that those on SLIB24 do as followings. DSIB is 24 VDC Battery feed. The DSIB is equipped with a Message Wait source, optional MSGU, which activate a lamp in the SLT, if so equipped. The Message Wait source sends 90 VDC signal to the SLT illuminating the MW lamp. A Ring Generator Unit is required in cabinet where a DSIB is installed.

The DSIB allows a Single Line Telephone to be connected up to 2.5 Kilometers (8,200 feet, loop resistance: 490 ohm) from the system using 22 AWG wire. See the Table 2.3.5. The DSIB also allows a DKTU to be connected up to 0.5 Kilometers (1600 feet) from the system using 22 AWG wire.

The DSIB can be installed in any of slot No. 1-9 in any KSU.

The maximum extension boards are limited by 300 extensions in LDK-300 system.

Add-on boards: DTRU, MSGU

### WTIB /WTIU (Wireless Terminal Interface Board / Unit)

The WTIB provides standard interface between the LDK-300 digital Key Telephone system and DECT (Digital Enhanced Cordless Telecommunications) Network. The system can accommodate three WTIBs in the same KSU, which are capable of supporting up to 24 Base Stations. The WTIB can be optionally equipped with WTIU to expand Base Stations, which can support up to additional 4 base stations. So one WTIB with WTIU allows maximum 8 Base Stations to be connected.

Following table shows the capacity of WTIB and its expansion board.

| Items | One WTIB (without WTIU/with WTIU) | Max. Capacity Three WTIB with WTIU |
| --- | --- | --- |
| Maximum Cell Number (Base Station Number) | 4/8 | 24 |
| Voice channels / Cell | 5 | 5 |
| Registrable Maximum Terminal Number | 8 – 64 by step 8 | 192 by step 8 |
| Max. Simultaneous Wireless calls | 16/32 | 96 |

Table 2.4.6 Capacity of WTIB

- Note

- Registrable Maximum Wireless Terminal numbers are programmable in Admin field 102.

- WTIB/WTIU contains RJ11 type connectors for the connection to Base Stations.

A WTIB should be installed in any of slot No. 1-8 in any KSU.

WTIB can not be operated normally in only slot No. 9 of any KSU.

WTIB for GDK-100 can not be used in the LDK-300 system.

### LCOB (Loop Start CO Line Interface Board)

LCOB provides 4 CO/PBX Loop Start CO Lines which support pulse /DTMF signal. Each Interface contains ring and loop current detection circuits, A/D and D/A conversions, and pulse and ground flash signaling circuit. LCOB contains 4 LEDs to indicate the in use status of each CO line.

RJ21 type male connector is mounted on the front edge of the LCOB for the connection to lines.

LCOB can be optionally equipped with up to 4 Call Metering Units (CMU) to monitor call charge, a Call Progress Tone Unit (CPTU) to monitor call progress tone and a DTMF Receiver Unit (DTRU) to detect DTMF.

A CMU supports call metering function and can be installed on each line. And the appropriate type of CMU should be installed on the line according to the national specification.

Each CPTU contains two tone detection circuits that are time shared under software control as system resources. These detectors are used in support of several features such as ACNR. If such features are to be used, it is recommended that at least one CPTU be provided for every LCOB. According to sensitivity of tone detector, two types of CPTU (CPTU/A, CPTU/B) are available. CPTU/A has wide tone detection range(305Hz~640Hz) and CPTU/B has narrow tone detection range (350Hz, 620Hz, 440Hz, 480Hz±1%).

LCOB can be installed in any of slot No. 1-9 in any KSU.

Analog CO interface boards are limited by maximum 104 analog lines in a system.

Add-on boards: DTRU, CPTU/A, CPTU/B, CMU50PR/CMU12PR/CMU16.

### LCOB8 (Loop Start CO Line Interface Board with 8 ports)

### ISDN Boards

LDK-300/GDK-100 ISDN Boards are capable of supporting various types of ISDN as followings,

| Board | Function | connection |
| --- | --- | --- |
| PRIB | Provides ISDN Primary Rate interfaces, 1 interface /30 B channels | 4 wire |
| BRIB | Provides ISDN Basic Rate interfaces(T), 4 interfaces /8 B channels | 4 wire |
| STIB | Provides ISDN Basic Rate interfaces(T/S switchable), 4 interfaces /8 B channels | 4 wire |

Table 2.4.7 ISDN boards

### ISDN PRIB (Primary Rate Interface Board)

PRIB (Primary Rate Interface Board) has capable of connecting to 1 PRI line, which has an interface circuit for 30 time slots with 2.048Mbps data rate speed. PRIB will be positioned at reference point Ia on ETS 300 001, that is TE slave without power feeding.  And, Data & bit clock are transmitted by the NT master and extracted by TE salve.

PRIB contains reset and watchdog circuit, address decoder, external RAM, program memory (ROM), the circuitry to control PLL device and DBID, and the board line’s specific circuitry. The line interface circuit consists of LXT304A which provides receive jitter attenuation, MT8979 which provides a complete interface between a 2.048Mbps primary rate and MITEL’s Serial Telecomm Bus (ST-BUS), and MT8920 which is provides a simple interface between ST-BUS and parallel system environments.

PRIB should be installed in any of slot No. 1-9 except only slot No. 9 of 1’st KSU.

Four PRIB can be installed in one KSU and Max 7 PRIB can be installed in LDK-300 system.

But, LDK-300 supports up to max 200 B channels for max 7 PRIB.

### ISDN BRIB (Basic Rate Interface Board: T interface only)

BRIB has specified to support four interface circuits that allow interfacing the ISDN BRI. Four interface circuits are used for the transmission of digital signals. Data & clock are transmitted by the NT master and extracted by TE salve. BRIB will be positioned at reference point Ia (T) on ETS 300 012, that is TE slave without power feeding.

BRIB contains reset and watchdog circuit, address decoder, external RAM, program memory(ROM), the circuitry to control the highway interface(DBID), and the board line’s specific circuitry. The board line’s specific circuitry is PEB2084 for Physical layer and PEB2075 for data link layer.

BRIB should be installed in any of slot No. 1-9 except only slot No. 9 of 1’st KSU .

The maximum 7 BRIBs can be installed in one KSU, but total 19 BRIBs can be installed in LDK-300 system.

### ISDN STIB (Basic Rate Interface Board: switchable S/T interface)

The S or T interfaces are based on the existing interface described in ETSI 300. 012, which is based on ITU_T Recommendations I.430 and provides modification and further requirements.

This is applied at the S or T reference points for the basic interface structure defined in ITU_T I.412. Layer 1 of the interface requires a balanced metallic transmission medium, for each direction of transmission capable of supporting 192Kbps(2B+D). For interface circuits, one per transmission direction, are used for the transmission of digital signals. Data & bit clock are transmitted by the NT master and extracted by TE slave. LDK-300 can be positioned at reference point Ia (T) or Ib (S) on ETS 300 012 that is TE-slave without power feeding or NT-master with power feeding. By selecting the jumper position on each line interface, STIB supports S-interface (Line card function)or T-interface (Trunk function).

SCC1 of MC68302 will serve the 64Kbps Monitor channel access which is use to convey message oriented local function such as software programming or S/Q channel information or access to internal register of line interface devices. SCC2 of CPU is assigned as 512Kbps HDLC communication to main processor on MPB. SCC3 of CPU is assigned as 38400bps RS232C channel. This RS232 channel will be used only in development stage for software and hardware debugging.

For physical layer interface function, SIEMENS’s PEB2084, Quadruple Transceiver for S/T interface which is implements four wire S/T interface used to link voice/data digital transmission to ISDN BRI is applied. It can handle up to 4 S/T interfaces simultaneously according to ITU-T I.430, ETSI 300.012, and ANSI T1.605 standards.

For data link layer interface function, SIEMENS’s PEB2075 ISDN D-channel exchange controller which is a serial HDLC data communication circuit with four independent channels is used. Each channel is fully independent and programmable by its own register set.

For power feeding control, PEB2025(Power feeding device from SIEMENS) is occupied. It can handle up to 4-channel with current limiting and short circuit protection.

STIB should be installed in any slot of slot No. 1-9 except only slot No. 9 of 1’st KSU.

The maximum 7 STIBs can be installed in one KSU, but total 19 STIBs can be installed in LDK-300 system.

### Add-on boards

This chapter describes add-on boards, which can be installed on various types of boards to support the proper function

| Board Name | Function | position |
| --- | --- | --- |
| PMU | Provides operating software | MPB |
| MODU | Provides a 33.6K baud modem for local access | MPB |
| SDRAM | Provides a 32 mega bytes synchronous DRAM |  |
| SIU | Provides additional two RS232 serial interfaces. | MISB |
| DTRU/DTRUII | Provides further 2/4 DTMF receivers | Note 1 |
| MSGU/MSGU48 | Provides Message Waiting signals to 6 SLT ports | Note 2 |
| MSGUII | Provides Message Waiting signals to 12 SLT ports | SLIBII |
| CPTU/A,CPTU/B | Provides Call progress detection function | Note 3 |
| CMU | Provides Call Metering signal detection | Note 4 |
| VCEU | Expands Channels up to additionally 4 channels | VMIB |
| FMEU | Expands Recording time up to additionally 300 minutes | VMIB |

Table 2.4.8 Add-on boards

- Note:

- DTRU can be installed on SLIB24, SLIB48, DSIB, LCOB, EMIB. And DTRUII can be only installed on SLIBII.

- MSGU can be installed on SLIB24 and DSIB, MSGU48 on SLIB48, and MSGUII on SLIBII.

- CPTU/A, CPTU/B can be installed on LCOB.

- Each type of CMU can be installed on the LCOB according to the local market condition and regulatory requirements.

### PMU (Programmable Memory Module)

The CPU operates on code in the flash ROMs in PMU which contains the system control, administration, and call processing code. The PMU has 8M bytes of four 70ns 16Mbit  flash ROMs which provides software downloading by BDM tool, ISDN line or LAN port.

PMU should be installed on MPB by default.

### SDRAM (Syncronous Dynamic RAM)

SDRAM is an option board for the software downloading and the other features for ethernet port.

SDRAM memory module is designed to support a 32-bit wide bank of SDRAM memory package in a standard JEDEC 168-pin SDRAM DIMM. The MPC860’s interface to the SDRAM consists of signals from the MPC860’s address bus, data bus, and control signals from it’s memory controller’s User Programmable Machine A(/CS6) and Machine B(/CS7). MPB provides four 64M-bit SDRAMs as an option.

The Samsung KMM366S424CTS is a 4M bit x 64 Synchronous Dynamic RAM high density memory module. The Samsung KMM366S424CTS consists of four CMOS 4M x 16 bit with 4banks Synchronous DRAMs in TSOP-II 400mil package and a 2K EEPROM in 8-pin TSSOP package on a 168-pin glass-epoxy substrate.

### MODU (Modem Unit)

The MODU(Modulator & Demodulator Unit) provides serial communication method of RS232 interface for remote maintenance and remote PC Admin. It supports V.34, V.32 bis, V.32, V.22bis, V.22, V.23, and V.21 Data Mode. Therefore it operates in full-duplex,asynchronous modes at line rates up to 33.6kbps.

The Modem Unit provides an asynchronous modem for access to the system database and fault reporting features from a remote site. The modem may be connected to a pre-selected CO Line through the system-switching matrix. The MODU port is independent of the SIU standard RS 232 port, allowing system database access, etc. without the need to interrupt the SMDR output.

MODU is optionally installed on the MPB

### SIU (Serial Interface Unit)

SIU provides additional two RS232C interface circuits. Two connectors are 9 pin RS232C.

Serial Interfaces on MPB and SIU The SIU is useful for system maintenance, PC based admin., SMDR print out and SMDI for PC based voice mail system.

Followings are the list of the system’s output;

- Administration database

- Off-line SMDR (on-demand)

- On-line SMDR

- Statistical Information

- SMDI (voice mail)

- System trace data

The SIU can be optionally installed on MISB.

### DTRU/DTRU II (DTMF Receiver Unit)

A DTRU contains 2 DTMF receiver circuits, and a DTRU II has 4 DTMF receiver circuits. Each receiver of DTRU /DTRU II is time-shared under the control of the system software, as a system resource regardless its position.

The DTRU can be optionally installed on SLIB24, SLIB48, DSIB, LCOB, AC15, BWDIDB, EMIB and DIDB.

The DTRU II can be optionally installed on SLIB II.

Maximum DTMF receivers are 22 in a system.

### MSGU/MSGU48/MSGU II (Message Wait Unit)

The Message Wait Unit sends 90VDC signal to the SLT illuminating the Message Wait lamp. The MSGU and MSGU48 contain 6 Circuits to support 6 SLTs. The MSGUII contains 12 circuits to support 12 SLTs.

MSGU can be optionally installed on SLIB24 and DSIB.

MSGU48 can be optionally installed on SLIB48.

MSGU II can be optionally installed on SLIB II.

### CPTU (Call Progress Tone Detection Unit; CPTU/A, CPTU/B)

The Call Progress Tone Detection Unit is used to detect specific call progress tones which are supplied from the CO or PABX. Tone detection is employed to support features such as ACNR(Automatic Called Number Redial).

CPTU provides 2 tone detection circuits that are time-shared under the control of the system software, as a system resource regardless its position.

CPTU have two versions; CPTU/A and CPTU/B. CPTU/A has wide tone detection range (305Hz - 640Hz), and CPTU/B has narrow tone detection range (350Hz, 620Hz, 440Hz, 480Hz  1% ).

Any Kind of CPTU can be optionally installed on the LCOB, BWDIDB and E1IB.

It is important to check the local specification regarding call progress tone to use CPTU/B.

### CMU (Call Metering Unit)

The Call Metering Unit detects call metering signals from the CO or PABX to monitor call duration/charges. There are six kinds of CMUs as followings,

- 12KHz

- 12KHz and Polarity Reversal

- 16KHz

- 50Hz Longitudinal

- 50Hz Longitudinal and Polarity Reversal

- Polarity Reversal

CMU can be optionally installed on the LCOB and each CMU supports one CO Line.

Each type of CMU can be installed on the LCOB according to the local market condition and national regulatory requirements.

### VCEU (Voice Channel Expansion Unit)

The VCEU allows for expansion of the four voice channels on the DVIB. VCEU consists of four(4) DSPs. One DSP is in charge of one voice channel.

VCEU can be optionally installed on the VMIB.

See clause 2.4.8 VMIB.

### FMEU (Flash Memory Expansion Unit)

The FMEU allows for expansion of the nand Flash memory on the VMIB. The FMEU expands to about 5 hours  for record and play time. FMEU consists of five(5) NAND Flash ROMs.

FMEU can be optionally installed on the VMIB

See clause 2.4.8 VMIB

### Keyset & Terminals

In addition to supporting analogue Single Line devices, by selecting appropriate interface boards, the system will support either or both the LG Digital Keysets /Terminals or the LG Electronic Keysets /Terminals.

### Digital Keysets and Terminals

Various types of digital terminals are used with LDK-300 DTIB12/DTIB24/DSIB as below:

| Model | Description | Model | Description |
| --- | --- | --- | --- |
| KD-36EXE | 24 Flexible Button Display | KD/E-36EXE | 24 Flexible Button Display |
| KD-36ENH | 24 Flexible Button Normal | KD/E-36ENH | 24 Flexible Button Normal |
| KD-24EXE | 12 Flexible Button Display | KD/E-24EXE | 12 Flexible Button Display |
| KD-24ENH | 12 Flexible Button Normal | KD/E-24ENH | 12 Flexible Button Normal |
| KD-33LD | 8 Flexible Button Large Display | KD/E-8BTN | 8 Flexible Button Normal |
| KD-DSS | 48 Button DSS/DLS Console | KD/E-36LD | 24 Flexible Button Large Display |
|  |  | KD/E DSS | 48 Button DSS/DLS Console |
| LKD-30DS | 30 Flexible Button Display | KD- Digital Phone Box | Digital Intercom Box |
| LKD-8DS | 8 Flexible Button Display | CTI module 1) | CTI module |
| LKD-2NS | 2 Flexible Button Normal |  |  |
| LKD-30LD | 30 Flexible Button Large Display |  |  |
| LKD-48DSS | 48 Button DSS/DLS Console |  |  |

Table 2.4.9 Digital Keyset and terminal

- Note:

- This type of terminal and KD/C and KD/EC series Keysets will not be supplied any more, but CTIU8/CTIU30 which are inserted on the bottom side of LKD Keysets, are supplied for CTI operation. See GDK-PC PHONE (CTI) in this section.

- Minimum software requirements for LKD operation: 3.2 or later version (except 3.4 version)

Digital Keysets

General Description

Each Digital Keyset has a standard 12 button dial-pad, color coordinated handset, an integral Wall Mount kit, a slide-out Directory Tray, an array of  “Flexible Buttons”, and 12 Fixed Feature Access keys. All of the Flexible and Fixed Buttons, except Volume, incorporate a long-life, super-bright LED to indicate the feature or circuit status. The number of “Flexible Buttons” differ from mdes in above table . The fixed feature access keys are the same for each keyset 8/2 button Keyset. Each Digital Keyset includes a RJ-11 type jack for connection to the system. See Figures 2.4.9  to 2.4.13.

Speaker Phone

Each Digital Keyset except LDK-2NS is equipped with circuitry that enables the telephone to be used hands-free in two way conversations. This circuitry provides voice-switched speaker phone operation which gives channel control to the party with the highest energy level. The speaker phone circuit is controlled by the user with the MON button at KD, KD/E and the ON/OFF button at LKD Keyset.

Volume Control

The volume control is a rocker arm button. Pressing the right side increases volume, the left decreases volume. The volume button controls the volume level of voice and ring signals received at the speaker in the keyset. Also, in the Dial-By-Name (Directory Dial) mode, the volume button scrolls the name display up and down, scrolling display information to find a telephone number in the directory.

LCD Display

The Display Keysets with EXE suffix in model name incorporate a 2-line, 48-character Liquid Crystal Display (LCD). The LCD provides alpha-numeric display of various information to assist the user in operation of features. In the idle mode, the display will show the station name or number on the top line and the time and date on the second line. And The Large Display Keysets with LD suffix also incorporate a 7-line, 112-character large Liquid Crystal Display (LCD)

Digital DSS

The Digital Direct Station Selection/Direct Line Selection (DSS/DLS) Consoles can be connected to any port of a DTIB in place of a Keyset. Up to 6 DSS/DLS Consoles can be associated with a Keyset in the system database. Each console incorporates a 4 by 12 matrix of “Flexible Buttons”, a total of 48 Buttons, each with an associated LED for status indications. The DSS/DLS Console includes a RJ-11 type jack for connection to the system.

Digital Phone Box

The Digital Phone Box provides hands-free intercom conversation from any location that generally does not require a fully functional Key Telephone. The Digital Phone Box is connected to any port of a DTIB in place of a Keyset. The Digital Phone Box includes a volume control slide switch and has 2 feature buttons with LEDs; one to call preprogrammed stations CALL and the second to block incoming calls DND.

### MOHU (Music On Hold Unit)

The MOHU provides External Music Source to the system through any kind of Single Line Interface including 2B-SLIU30 in LDK-300 System. The external music source provided through single line interface is used as music source of the system, that is, the music source through MOHU offers the same function that the other external music source of the system do. A MOHU has two interface circuits to connect 2 music sources with 2 single line interface. The system allows the maximum 5 interface circuits (restricted by the program) through the maximum 3 MOHUs to be connected. The single line interfaces connected to MOHU should be selected by Admin programming.( Field #1 and 2nd button)

The characteristics of MOHU interface are followings;

- Impedance: 600 ohm

- Transmission loss: 1 dB +/- 1dB at 1020Hz

- Maximum music source input: Max. 0 dBm @ 600 ohms

The MOHU can be connected to any kind of single line interface including 2B-SLIU30.

This module may be available to support special local market conditions.

### 2B-module

The 2B-module allows another terminal, DKTU (Digital Key Telephone Unit) or SLT (Single line Telephone) behind LKD-30DS to be connected. There are 2 kinds of 2B-modules; 2B-DTIU30 for the connection to another DKTU behind LKD-30DS, and 2B-SLIU30 for the connection to another SLT behind LKD-30DS. These 2B-modules can be only installed in LKD-30DS. See Figure 2.4.11 2B modules.

In LDK-300 system, LDK-30DS with 2B-module can be connected to every even-numbered port (0,2,4,…) of DTIB12/DTIB24/DSIB. In that case, any digital terminal can not be connected to the consecutive port (1,3,5,…). So the total capacity of DTIB12/DTIB24/DSIB is not changed by the connection of 2B-module.

The advantage of 2B module in LDK-300 system;

- Simple wiring with two terminals.

- DTIB is capable of connecting to single line device through 2B-SLIU30. It is useful for the customer who wants more single line devices even though DTIB is installed in the system.

- DTIB is capable of connecting to single line device such as Fax/Modem without any SLIB board.

2B modules are supported in the following condition

- Software: 3.2 or later version (except 3.4 version)

- Hardware: Digital Terminal Interface only, LKD-30DS, 2B-DTIU30 or 2B-SLIU30

- Line length. See Table 2.3.3 Electrical Specifications

- Note:

- In case of using 2B-DTIU30 and the secondary digital keyset, it is recommended to use the separate mains adapter, which confirms to the appropriate national or local electrical adapter for the secondary DKTU.

- The separate mains adapter can be get in local market, the recommended specification is;

- Output capacity: 24 to 30Vdc, more than 200 mA

- Size of output plug: the inside diameter - 2.1mm, the outside diameter: 5.5mm

The following table shows LKD series which the 2B module can be installed.

| DKTU | 2B-DTIU30 | 2B-DTIU30 |
| --- | --- | --- |
| LKD 30DS | Yes | Yes |
| LKD 30LDS | X | X |
| LKD 8DS | X | X |
| LKD 2NS | X | X |

Table 2.4.10  Available 2B module in LKD series

For more details, see “ Installation manual for fig.2.4.11.a 2B-DTIU30 and fig.2.4.11.4.b 2B-SLIU30”.

### GDK-PC PHONE (CTI)

GDK-PC PHONE is based on Microsoft TAPI (Telephony Application Programming Interface) Standard Specification and Windows 3.1 or Windows 95 environment. With Computer Telephony, a customer can use computer to take an advantage of a wide range of sophisticated communication features and services over a telephone line. GDK-PC PHONE includes the various value added features to integrate Computer and Telephone, to manage all the details associated with telephone business.

There are 2 kinds of hardware configurations, one is an external CTI module with KD/C or KD/EC series DKTU, the other is an internal CTI module with LKD series DKTU. See Fig. 2.4.12.a the connection of the external CTI module, Fig. 2.4.12.b the connection of the internal CTI module,.

External CTI module package

An external CTI module contains three connectors, 2 RJ-11 type jacks and a RS-232C. One of RJ-11 type jack is used for the connection to Digital Terminal Interface port of the system through 2-wire telephone line, and the other is for the connection to KD/C or KD/EC series through 6-wire telephone line. And RS-232C, comprised of 9-pin connector, is for the connection to a computer. All the line cords and RS-232 cable are provided in the same package.

External CTI module allows only KD/C or KD/EC series to be connected, but not another type of DKTU.

Internal CTI module package

An internal CTI module can be installed on the bottom side of LKD series DKTU. And the cable with RS-232C 9-pin connector and RJ11 type plug is provided for the connection to a computer in the package.

There are two kinds of internal CTI modules, CTIU30 and CTIU8. CTIU30 can be installed in LKD-30DS, and CTIU8 in LKD-8DS and LKD-2NS, but not possible in LKD-30LDS with Large Display.

| DKTU | CTIU30 | CTIU8 |
| --- | --- | --- |
| LKD 30DS | Yes | X |
| LKD 30LDS | X | X |
| LKD 8DS | X | Yes |
| LKD 2NS | X | Yes |

Table 2.4.11 Available CTI module in LKD series

For more details, see “ Installation And User’s Guide for GDK-PC PHONE”.

### Cables (5MC1, 5MC2)

There are two kinds of line connection cables, one (5MC1: 6851NF0023A) is for the connection to analog CO lines or PFTU, and the other (5MC2: 6851NF0028A) is for the connection to the terminals or miscellaneous devices. 5MC1 consists of the 25-pair cable with 5 meter line length and a RJ21 type female connector, and 5MC2 consists of the 25-pair cable with 5 meter line length and a RJ21 type male connector. One end of the cable is connected to a RJ21 type connector, and the other is open.

So the RJ21 type connector of the cable can be connected to another RJ21 type connector on the function boards according to the type of connector described above. And open wires of the cable can be connected to a user connection point, e.g. the MDF which a supplier provides.

MDF, which is in wide use in the countries, can be locally used according to the requirements of local regulation.

If necessary, above cables can be supplied from a supplier on the order.

### BRI Line Terminating Board (LDK-300/GDK-162/100 TERM: 2923NP1656A)

GDK-162/100 TERM unit provides four pairs of 100 ohm terminating resistors. The unit is capable of accommodating 4 BRA lines with RJ45 type jacks. It has metal case.

This can be connected at outside of the system. If a customer use Basic Rate (S or T) interface but the 100 ohm terminating resistors are not introduced on the interface circuit, this board will be useful.

See the parts of BRIB Installation, STIB Installation, and Terminating Resistors on Basic Rate Interfaces in section 3.

### Trace Tool (GDK-TRC1)

GDK-TRC1, Figure 2.4.13 provides a RS-232 Serial Interface for the maintenance of special cards of LDK-300 system. It provides 4-wire interface to connect to LDK-300 boards and RS-232 interface to connect to the serial port of PC. All two cables are offered in the GDK-TRC1 package.

GDK-TRC1 can be connected to the following board for access..

| System Name | Board Name |
| --- | --- |
| LDK-300 | PRIB issue 2B or later, all BRIB, STIB, WTIB, DVIB |

Table 2.4.12. Available TRC tool in the LDK-300 system

Caution: This tool should be only used by LG Electronics Inc. and its agent for the purpose of the maintenance of LDK-300 system, but not by an end user.