CONTENTS

4.	CO LINE PROGRAMMING	1

4.1	CO SERVICE TYPE  (PGM 140)	1

4.2	CO LINE ATTRIBUTE – I  (PGM 141)	4

4.3	CO LINE ATTRIBUTE – II  (PGM 142)	6

4.4	ISDN CO LINE ATTRIBUTE  (PGM 143)	9

4.5	CO RING ASSIGNMENT  (PGM 144)	11

- Admin Revision History

2. ISSUE :  0.1B

| DATE | BY | BEFORE | AFTER |
| --- | --- | --- | --- |
| 00/10/30 | LWW | 4.4. PGM 143 Btn 1 & Btn 2. Input Range 00 ~ 49, & COLP Table PGM = 202 | Input Range : 00 ~ 50 & COLP Table PGM = 201. |

# 4.	CO LINE PROGRAMMING

If CO line features are to be changed, press the [TRANS/PGM] button and dial 140-144 in Admin Programming Mode. When programming, LCD and LEDs indicate current programmed data and programming status. If the programmer enters correct data, then LCD and LEDs show the entered data and the data is stored in the temporary buffer area. To save the data permanently, press the [HOLD/SAVE] button, then all data in the temporary buffer (same as LCD/LEDs show their status) are stored into system memory.

## 4.1	CO SERVICE TYPE  (PGM 140)

In this program mode, you can program the following items.

PROCEDURE

| COL SERVICE ATT ENTER COL RANGE | - [TRANS/PGM] + 140. |
| --- | --- |
| 001-002 COL SVC F1:TYPE F2:SUB ATT | - (2) Enter CO line range (Ex. 001-002, range 001-200). LCD displays message and user can select COL service TPYE by pressing flex BTN 1. |
|  | - To program, use the TYPE as Table 4.1.2. Press digit 1-4 to select CO service type.( type 1-4 is exclusive) Flex BTN 1 is for type setting, and flex BTN 2 is used in setting sub attributes with the selected type.. |
| 001-002 SVC TYPE (1-4) NORMAL CO (1) | - To select TYPE, press flex BTN 1. Initially LCD displays current type (at start time, default value is NORMAL CO) User can select TYPE 1-4. |
| 001-002 SVC TYPE (1-4) ISDN DID / MSN (3) | - To change TYPE press another digit (EX. 3), then LCD displays the current type immediately. To save current type, press [HOLD/SAVE] button. |
| 001-002 SVC TYPE (1-4) PRESS FLEX_KEY (1-2) | - Next, to set sub attributes of the selected TYPE, press flex BTN 2. See Table 4.1.2. |
| INFO NO (01-70) : . . | - User can input number 01-70. If it is valid value, LCD shows the input. To save current value, press [HOLD/SAVE] button. |
| 001-002 SVC TYPE (1-4) F1:TYPE F2:SUB ATT | - (3) Another Example) To change TYPE for selected the CO range. Press flex BTN 1. |
| 001-002 SVC TYPE (1-4) NORMAL CO (1) | - LCD shows the current type which user preprogrammed. To change TYPE, press another digit (EX. 1). Then LCD displays the current type immediately. To save current type, press [HOLD/SAVE] button. |
| 001-002 SVC TYPE (1-4) F1:TYPE F2:SUB ATT | - Next, to set sub attributes of the selected TYPE, press flex BTN 2. See Table 4.1.2. |
| 001-002 DISA ATT DAY NIGHT WEEKEND | - In TYPE Normal CO, user can set the 3 sub attribute Day, Night, Weekend. Each attribute also has 3 sub attribute. First, choose one flex BTN.(day night weekend, 1 2 3) See Table 4.1.2. |
| 001-002 DISA ATT PRESS FLEX_KEY (1-2) | - Next, user select flex BTN 1 to set DISA Service ON/OFF. Do this procedure for other attribute. See Table 4.1.2. |
| 001-002 DISA SVC (1 : ON/ 0 : OFF) : OFF | - Next, user select flex BTN 1 to set DISA Service ON/OFF. After entering the desired value, to save current type, press [HOLD/SAVE] button. |
| 001-002 DISA ATT PRESS FLEX_KEY (1-2) | - Do this procedure for other attribute. See Table 4.1.2. |
| COL SERVICE ATT ENTER COL RANGE | (7) Press the [HOLD/SAVE] button for updating database permanently. |
| COL SERVICE ATT ENTER COL RANGE | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (2) without updating system memory. |

| ITEM | REMARK |
| --- | --- |
| Normal CO | All lines are assigned normal CO lines by default. It has DISA attribute. Each CO line in the system can be programmed as DISA (Direct Inward System Access) line and the DISA types are as follows; - Flex BTN 1 (Day) / 2 (Night) / 3 (Weekend) (Each BTN 1-3 has sub attribute) - Whether DISA Service allowed. - Voice announcement can be assigned (00-70). Default is not assigned(00) |
| ANALOG DID | Each CO line in the system can be programmed as DID (Direct Inward Dialing) line and the DID types are as follows; - 1(Immediate Start) / 2 (Wink Start) / 3 (Delayed Dial Start) (BTN 1-3 are exclusive) |
| ISDN DID/ MSN |  |
| TIE | TIE line types are as follows; - 1 (EM-C) / 2 (EM-D) / 3 (LD) / 4 (RD) |

TABLE 4.1.1  CO Service Type (PGM 140)

| BTN | TYPE | When flex BTN 2 | DEFAULT | SUB ATTR | REMARK |
| --- | --- | --- | --- | --- | --- |
| 1 | Normal CO | DISA attribute -Flex BTN 1(Day) -Flex BTN 2(Night) -Flex BTN 3(Weekend) |  | For each ITEM Flex BTN 1 - DISA Service :ON/OFF Flex BTN 2 :Voice ANNC : (00- 70) (00:not_asgn) |  |
| 2 | ANALOG DID | Flex BTN 1 : (Signal Type) Flex BTN 2 : (INFO NO) | - | Signal Type –1:Immediate Start 2:Wink Start 3:Delayed Dial Start INFO NO : 00 – 70 (00: not_asgn) |  |
| 3 | ISDN DID / MSN |  |  |  |  |
| 4 | TIE | 1:RD 2:LD 3:EM-C 4: EM-D | RD | - | digit input |

TABLE 4.1.2  Button Configuration For CO SERVICE TYPE  (PGM 140)

## 4.2	CO LINE ATTRIBUTE – I  (PGM 141)

PROCEDURE

| CO LINE ATT 1 ENTER COL RANGE | - [TRANS/PGM] + 141. |
| --- | --- |
| 001-002 CO LINE ATT1 PRESS FLEX_KEY (1-9) | - Enter CO line range (Ex. 001-002, range 001-200). LCD displays message and user can select flex BTN 1-9. |
|  | - To program, use the flex BTN as Table 4.2.1. Press BTN 1-9 and enter related data for setting , then entered data will be displayed on LCD. |
| 001-002 FLASH TYPE (1: GND/ 0: LOOP) : LOOP | - To set Flash Type, press BTN 7. |
| 001-002 FLASH TYPE (1: GND/ 0: LOOP) : LOOP | - To alter Flash Type, dial 1-0.(EX: dial 1) |
| 001-002 DISA ACCT CODE (1 : ON/ 0 : OFF) : OFF | - Press BTN 3 and DISA Account Code can be assigned. |
| 001-002 DISA ACCT CODE (1 : ON/ 0 : OFF) : OFF | - Press the [HOLD/SAVE] button for updating database permanently. |
| CO LINE ATT 1 ENTER COL RANGE | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (2) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | CO Line Group | 00-73 | 01 | Groups should be assigned according to CO type and Class-Of-Service. (00:private 73:not_used) |
| 2 | CO COS | 1-5 | 1 | -CO COS 1: no restriction -CO COS 2: Exception Table A governs -CO COS 3: Exception Table B governs -CO COS 4: restricts Long Distance Code -CO COS 5: overrides STA. COS 2,3,4 and 5, 6. |
| 3 | DISA Account Code | ON/OFF | OFF | When accessed another CO line in the system by DISA line, you should enter authorization code if this flag is set. |
| 4 | CO Line Assign | POL/LOOP | LOOP | Polarity Reverse , Loop Start |
| 5 | CO Line Type | PBX/CO | CO | When marked PBX, a 1 or 2 digit dial code may be entered after which toll restriction is applied. |
| 6 | CO Line Signal Type | DTMF/PULSE | DTMF | DTMF, Pulse |
| 7 | Flash Type | GROUND/LOOP | LOOP | Ground , Loop |
| 8 | UNA | ON/OFF | OFF | The allowance of Universal Night Answer service |
| 9 | CO Line Group Account | ON/OFF | OFF |  |

TABLE 4.2.1  Button Configuration for CO Attribute - I (PGM 141)

## 4.3	CO LINE ATTRIBUTE – II  (PGM 142)

PROCEDURE

| CO LINE ATT 2 ENTER COL RANGE | (1) [TRANS/PGM] + 142. |
| --- | --- |
| CO LINE ATT 2 PRESS FLEX_KEY (01-13) | - Enter CO line range (Ex. 001-002, range 001-200). |
|  | - To program, use the BTN as Table 4.3.1. BTN 1,4,7-11 set on and off. Press BTN 2,3,5,6,12,13 and enter related data for setting , then entered data will be displayed on LCD. |
| 001-002 CO DIST RING (0-4) : 3 | - To assign CO Distinct Ring press BTN 5. (EX) To alter RING Type dial 0-4.(EX: dial 3) |
| 001-002 CO DIST RING (0-4) : 3 | - Press the [HOLD/SAVE] button for updating database permanently. |
| 001-002 MOH(00-13) INT MUSIC (1) | - Press BTN 6 and to alter MOH, dial 00-13. |
| 001-002 MOH(00-13) INT MUSIC (1) | - Press the [HOLD/SAVE] button for updating database permanently. |
| CO LINE ATT 2 PRESS FLEX_KEY (01-13) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (2) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | CO Line Name Display | ON/OFF | OFF | If CO Line name is assigned at BTN2, and this field is ON, Co name is displayed in Co incoming. |
| 2 | CO Line Name Assign | Max 12 char | - | Max 12 character |
| 3 | Metering Unit | 00-11 | 00 | There are 7 metering signal types: - 0 : None - 1 : 50 Hz - 2 : 12 KHz - 3 : 16 KHz - 4 : Singular Polarity Reverse (SPR) - 5 : Plural Polarity Reverse (PPR) - 6 : No Polarity Reverse (NPR) - 7 : AOC-S - 8 : AOC-1 - 9 : AOC-2 - 10 : AOC-3 - 11 : AOC-4 |
| 4 | Line Drop using CPT | ON/OFF | OFF | If this field set to ON, CPT checks the incoming CO line when answered and if CPT detects dial tone, then system drops the line for toll restriction. |
| 5 | CO Distinct Ring | 0-4 | 0 | The CO can give his own ring type signal to station in system through this field. This ring type can be programmed at PGM 422. |
| 6 | CO Line MOH | 0-13 | 1 | - 0: Not assigned by this field. - 1: Internal Music - 2~4: External Music - 5~7: VMIB MOH - 8-12: SLT MOH - 13: Hold Tone |
| 7 | PABX CO Dial Tone | YES/NO | YES | - YES: In this case, PX or PABX provides dial tone. - NO: In this case PX or PABX does not provide dial tone. System provides dial tone |
| 8 | PABX CO Ring Back Tone | YES/NO | NO | - If R2 PX which does not give us tone for called party status exists, then the system provides tone according to cause value (This field is only when Cause means that Ring back is provided by PX.). - YES: PX, NO: System |
| 9 | PABX CO Error Tone | YES/NO | NO | - If R2 PX which does not give us tone for called party status exists, then the system provides tone according to cause value (This field is only when Cause means that error tone is provided by PX.). - YES: PX, NO: System |
| 10 | PABX CO Busy Tone | YES/NO | NO | - If R2 PX which does not give us tone for called party status exists, then the system provides tone according to cause value (This field is only when Cause means that busy tone is provided by PX.). - YES: PX, NO: System |
| 11 | PABX CO Announce Tone | YES/NO | NO | - If R2 PX which does not give us tone for called party status exists, then the system provides tone according to cause value (This field is only when Cause means that announcement is provided by PX, but the system provides only error tone.). - YES: PX, NO: System |
| 12 | CO Flash Timer | 000 – 300 | 050 | 10 msec base |
| 13 | Open Loop Detect Timer | 00 - 20 | 00 | 100 msec base |

- TABLE 4.3.1 Button Configuration for CO Attribute - II (PGM 142)

## 4.4	ISDN CO LINE ATTRIBUTE  (PGM 143)

PROCEDURE

| COL ISDN ATT ENTER CO RANGE | - [TRANS/PGM] + 143. |
| --- | --- |
| 001-009 COL ISDN ATT PRESS FLEX KEY (01-06) | - Enter CO line range (Ex. 01-09). |
|  | - To program, use the BTN as Table 4.4.1. BTN 6 can toggle on and off the value. Press other BTN and enter related data for setting , then it will be displayed entered data on LCD. |
| 001-009 DID REMOVE NO (00-99) : 00 | - To set DID_Remove_Number, press BTN 5. (EX) To alter DID_RN dial number.(EX: dial 21) |
| 001-009 DID REMOVE NO (00-99) : 21 | - Press the [HOLD/SAVE] button for updating database permanently. |
| 001-009 COL ISDN ATT PRESS FLEX KEY(01-06) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (2) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | COLP Table Index | 00 – 50 | Not_asgn | To make Called party number with assigned COLP Table entry. (PGM 201) |
| 2 | CLIP Table Index | 00 – 50 | Not_asgn | To make Calling party number with assigned COLP Table entry. (PGM 201) |
| 3 | CALL TYPE | 0 – 4 | 2 | - 0:Unknown - 1:INTERNATIONAL - 2:NATIONAL - 3:Not used - 4:SUBSCRIBER |
| 4 | DID CONV TYPE | 0 – 2 | 0 | - 0: convert digits by DID Dgt Conversion(PGM230) - 1: call to the valid extension. - 2:convert digits by Flex DID Table(PGM231) |
| 5 | DID REMOVE NO | 00 – 99 | Not_asgn | - Remove received digits from left as to the assigned # |
| 6 | ISDN ENBLK SEND | ON/OFF | OFF | - ON: Enblock Sending Mode - OFF: Overlap Sending Mode |

- TABLE 4.4.1 Button Configuration for ISDN CO Attribute  (PGM 143)

## 4.5	CO RING ASSIGNMENT  (PGM 144)

PROCEDURE

| CO RING ASSIGNMENT ENTER COL RANGE | - [TRANS/PGM] + 144. |
| --- | --- |
| 001-002 PRESS KEY DAY NIGHT WEEK ON-D | - Enter CO line range (Ex.001-002, range 001-200). User can select one of 4 flex BTNs. |
| 001-002 DAY CO RING DIAL TYPE(1:S/2:H/3:D) | - For the selected one of 4 mode (day, night, weekend, on-d) , input one digit (1-3) to select type. When digit is input, menu moves to the selected type setting. |
| 001-002 ENTER STA RANGE STA : 200-300 DLY : . . | - In case of DEST TYPE 1, first enter station range. If the input range is valid, LCD shows the value and system and moves delay input state with confirm tone. |
| 001-002 ENTER STA RANGE STA : 200-300 DLY : 9 | After input station range, enter desired value of delay. To save the station range and delay value, press the [HOLD/SAVE] button. |
| 001-002 DAY CO RING HUNT GRP : . . . | - In case of DEST TYPE 2, Enter desired value of hunt group. To save value, press the [HOLD/SAVE] button |
| 001-002 NIGHT CO RING VMIB MESSAGE 50 | - In case of DEST TYPE 3, Enter desired value of voice message.(ex 50) To save value, press the [HOLD/SAVE] button. |
| 001-002 NIGHT CO RING VMIB MESSAGE 50(#) | - To drop the call after VMIB message, press “#’ To save value, press the [HOLD/SAVE] button. |
| 001-002 PRESS KEY DAY NIGHT WEEK ON-D | - Press the [HOLD/SAVE] button for updating database permanently. |
| 001-002 PRESS KEY DAY NIGHT WEEK ON-D | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (2) without updating system memory. |

| BTN | ITEM | DEST TYPE | RANGE | DEFAULT |
| --- | --- | --- | --- | --- |
| 1 | DAY | TYPE 1 : STA RANGE DELAY TYPE 2 : HUNT GRP TYPE 3 : VOICE MSG | STA RANGE DELAY : 0 - 9 HUNT GRP : VOICE MSG : | - Drop : # |
| 2 | NIGHT |  |  |  |
| 3 | WEEKEND |  |  |  |
| 4 | ON-D |  |  |  |

- TABLE 4.5.1 Button Configuration for CO STATION RING Attribute  (PGM 144)