CONTENTS

12.  HOTEL(PGM 300)	2

13.  QSIG  (PGM 320)	2

14.	VOIP  (PGM 340)	2

15.	NATION SPECIFIC PROGRAMMING	3

15.1	DTIB RX GAIN CONTROL  (PGM 400)	3

15.2	SLIB RX GAIN CONTROL  (PGM 401)	4

15.3	SLIB12 RX GAIN CONTROL  (PGM 402)	5

15.4	WTIB RX GAIN CONTROL  (PGM 403)	6

15.5	ACOB RX GAIN CONTROL  (PGM 404)	7

15.6	ACOB8 RX GAIN CONTROL  (PGM 405)	8

15.7	VIMB RX GAIN CONTROL  (PGM 407)	9

15.8	DTMF RCVR RX GAIN CONTROL  (PGM 408)	10

15.9  EXT PAGE RX GAIN CONTROL  (PGM 409)	11

15.10  CPT RX GAIN CONTROL  (PGM 410)	12

15.11  MODEM RX GAIN CONTROL  (PGM 411)	13

15.12  PTRP RX GAIN CONTROL  (PGM 419)	14

15.13  SYSTEM TONE FREG  (PGM 420)	15

15.14  DIFF RING FREGUENCY  (PGM 421)	16

15.15  DISTINCT RING FREGUENCY  (PGM 422)	17

15.16  ACNR TONE CADENCE  (PGM 423)	18

16  INITIALIZATION (PGM 450)	19

17.	PRINT PROT DATABASE  (PGM 451)	20

# 12.  HOTEL(PGM 300)

Reserved

# 13.  QSIG  (PGM 320)

Reserved

# 14.	VOIP  (PGM 340)

Reserved

# 15.	NATION SPECIFIC PROGRAMMING

If system parameters are to be changed, press the [TRANS/PGM] button and dial 40-78. When programming, LCD and LEDs indicate current programmed data and programming status. The programmer enters correct data, then LCD and LEDs show the entered data and the data is stored in the temporary buffer area. After all, press the [HOLD/SAVE] button, all data in the temporary buffer (same as LCD/LEDs show their status) are stored into permanent system memory.

## 15.1	DTIB RX GAIN CONTROL  (PGM 400)

PROCEDURE

| DTIB RX GAIN PRESS FLEX KEY (01-11) | - [TRANS/PGM] + 400. |
| --- | --- |
| DTIB RX GAIN DTIB/DTIB: 26 (00-63) | - Press BTN 1. The Rx-gain of each device can be programmed. |
| DTIB RX GAIN DTIB/SLIB: 33 (00-63) | - Press BTN 2 (Select a device type to change gain). The LCD shows RX gain of the device from other devices. |
| DTIB RX GAIN DTIB/SLIB: 45 (00-63) | - To change gain, press BTN and dial new gain (00-63) and LCD shows changed gain. - (Ex: BTN 2 and dial 45.) |
| DTIB RX GAIN PRESS FLEX KEY (01-11) | - Press the [HOLD/SAVE] button for updating database permanently. Then main menu is displayed. |
| DTIB RX GAIN PRESS FLEX KEY (01-11) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system main menu without updating system memory. |

## 15.2	SLIB RX GAIN CONTROL  (PGM 401)

PROCEDURE

| SLIB RX GAIN PRESS FLEX KEY (01-11) | (1) [TRANS/PGM] + 401. |
| --- | --- |
| SLIB RX GAIN SLIB/DTIB: 12 (00-63) | - Press BTN 1. The Rx-gain of each device can be programmed. |
| SLIB RX GAIN SLIB/SLIB: 26 (00-63) | - Press BTN 2 (Select a device type to change gain). The LCD shows RX gain of the device from other devices. |
| SLIB RX GAIN SLIB/SLIB: 45 (00-63) | - To change gain, press BTN and dial new gain (00-63) and LCD shows changed gain. - (Ex: BTN 2 and dial 45.) |
| SLIB RX GAIN PRESS FLEX KEY (01-11) | - Press the [HOLD/SAVE] button for updating database permanently. Then main menu is displayed. |
| SLIB RX GAIN PRESS FLEX KEY (01-11) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system main menu without updating system memory. |

## 15.3	SLIB12 RX GAIN CONTROL  (PGM 402)

PROCEDURE

| SLIB12 RX GAIN PRESS FLEX KEY (01-11) | - [TRANS/PGM] + 402. |
| --- | --- |
| SLIB12 RX GAIN SLIB12/DTIB: 12 (00-63) | - Press BTN 1. The Rx-gain of each device can be programmed. |
| SLIB12 RX GAIN SLIB12/SLIB: 26 (00-63) | - Press BTN 2 (Select a device type to change gain). The LCD shows RX gain of the device from other devices. |
| SLIB12 RX GAIN SLIB12/SLIB: 45 (00-63) | - To change gain, press BTN and dial new gain (00-63) and LCD shows changed gain. - (Ex: BTN 2 and dial 45.) |
| SLIB12 RX GAIN PRESS FLEX KEY (01-11) | - Press the [HOLD/SAVE] button for updating database permanently. Then main menu is displayed. |
| SLIB12 RX GAIN PRESS FLEX KEY (01-11) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system main menu without updating system memory. |

## 15.4	WTIB RX GAIN CONTROL  (PGM 403)

PROCEDURE

| WTIB RX GAIN PRESS FLEX KEY (01-11) | - [TRANS/PGM] + 403. |
| --- | --- |
| WTIB RX GAIN WTIB/DTIB: 26 (00-63) | - Press BTN 1. The Rx-gain of each device can be programmed. |
| WTIB RX GAIN WTIB/SLIB: 33 (00-63) | - Press BTN 2 (Select a device type to change gain). The LCD shows RX gain of the device from other devices. |
| WTIB RX GAIN WTIB/SLIB: 45 (00-63) | - To change gain, press BTN and dial new gain (00-63) and LCD shows changed gain. - (Ex: BTN 2 and dial 45.) |
| WTIB RX GAIN PRESS FLEX KEY (01-11) | - Press the [HOLD/SAVE] button for updating database permanently. Then main menu is displayed. |
| WTIB RX GAIN PRESS FLEX KEY (01-11) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system main menu without updating system memory. |

## 15.5	ACOB RX GAIN CONTROL  (PGM 404)

PROCEDURE

| ACOB RX GAIN PRESS FLEX KEY (01-12) | - [TRANS/PGM] + 404. |
| --- | --- |
| ACOB RX GAIN ACOB/DTIB: 26 (00-63) | - Press BTN 1. The Rx-gain of each device can be programmed. |
| ACOB RX GAIN ACOB/SLIB: 37 (00-63) | - Press BTN 2 (Select a device type to change gain). The LCD shows RX gain of the device from other devices. |
| ACOB RX GAIN ACOB/SLIB: 45 (00-63) | - To change gain, press BTN and dial new gain (00-63) and LCD shows changed gain. - (Ex: BTN 2 and dial 45.) |
| ACOB RX GAIN PRESS FLEX KEY (01-12) | - Press the [HOLD/SAVE] button for updating database permanently. Then main menu is displayed. |
| ACOB RX GAIN PRESS FLEX KEY (01-12) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system main menu without updating system memory. |

## 15.6	ACOB8 RX GAIN CONTROL  (PGM 405)

PROCEDURE

| ACOB8 RX GAIN PRESS FLEX KEY (01-12) | - [TRANS/PGM] + 405. |
| --- | --- |
| ACOB8 RX GAIN LCOB8/DTIB: 26 (00-63) | - Press BTN 1. The Rx-gain of each device can be programmed. |
| ACOB8 RX GAIN LCOB8/SLIB: 37 (00-63) | - Press BTN 2 (Select a device type to change gain). The LCD shows RX gain of the device from other devices. |
| ACOB8 RX GAIN LCOB8/SLIB: 45 (00-63) | - To change gain, press BTN and dial new gain (00-63) and LCD shows changed gain. - (Ex: BTN 2 and dial 45.) |
| ACOB8 RX GAIN PRESS FLEX KEY (01-12) | - Press the [HOLD/SAVE] button for updating database permanently. Then main menu is displayed. |
| ACOB8 RX GAIN PRESS FLEX KEY (01-12) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system main menu without updating system memory. |

## 15.7	VIMB RX GAIN CONTROL  (PGM 407)

PROCEDURE

| VMIB RX GAIN PRESS FLEX KEY (1-7) | - [TRANS/PGM] + 407. |
| --- | --- |
| VMIB RX GAIN VMIB/DTIB: 21 (00-63) | - Press BTN 1. The Rx-gain of each device can be programmed. |
| VMIB RX GAIN VMIB/SLIB: 32 (00-63) | - Press BTN 2 (Select a device type to change gain). The LCD shows RX gain of the device from other devices. |
| VMIB RX GAIN VMIB/SLIB: 45 (00-63) | - To change gain, press BTN and dial new gain (00-63) and LCD shows changed gain. - (Ex: BTN 2 and dial 45.) |
| VMIB RX GAIN PRESS FLEX KEY (1-7) | - Press the [HOLD/SAVE] button for updating database permanently. Then main menu is displayed. |
| VMIB RX GAIN PRESS FLEX KEY (1-7) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system main menu without updating system memory. |

## 15.8	DTMF RCVR RX GAIN CONTROL  (PGM 408)

PROCEDURE

| DTMF RCVR RX GAIN PRESS FLEX KEY (1-3) | - [TRANS/PGM] + 408. |
| --- | --- |
| DTMF RCVR RX GAIN DTMF RC/SLIB: 37 (00-63) | - Press BTN 1. The Rx-gain of each device can be programmed. |
| DTMF RCVR RX GAIN DTMF RC/ACOB: 24 (00-63) | - Press BTN 2 (Select a device type to change gain). The LCD shows RX gain of the device from other devices. |
| DTMF RCVR RX GAIN DRMF RC/ACOB: 45 (00-63) | - To change gain, press BTN and dial new gain (00-63) and LCD shows changed gain. - (Ex: BTN 2 and dial 45.) |
| DTMF RCVR RX GAIN PRESS FLEX KEY (1-3) | - Press the [HOLD/SAVE] button for updating database permanently. Then main menu is displayed. |
| DRMF RCVR RX GAIN PRESS FLEX KEY (1-3) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system main menu without updating system memory. |

## 15.9  EXT PAGE RX GAIN CONTROL  (PGM 409)

PROCEDURE

| EXT PAGE RX GAIN PRESS FLEX KEY (1-9) | - [TRANS/PGM] + 409. |
| --- | --- |
| EXT PAGE RX GAIN EXT PAG /DTIB: 26 (00-63) | - Press BTN 1. The Rx-gain of each device can be programmed. |
| EXT PAGE RX GAIN EXT PAG /SLIB: 37 (00-63) | - Press BTN 2 (Select a device type to change gain). The LCD shows RX gain of the device from other devices. |
| EXT PAGE RX GAIN EXT PAG /SLIB: 45 (00-63) | - To change gain, press BTN and dial new gain (00-63) and LCD shows changed gain. - (Ex: BTN 2 and dial 45.) |
| EXT PAGE RX GAIN PRESS FLEX KEY (1-9) | - Press the [HOLD/SAVE] button for updating database permanently. Then main menu is displayed. |
| EXT PAGE RX GAIN PRESS FLEX KEY (1-9) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system main menu without updating system memory. |

## 15.10  CPT RX GAIN CONTROL  (PGM 410)

PROCEDURE

| CPR RX GAIN PRESS FLEX KEY (1-2) | - [TRANS/PGM] + 410. |
| --- | --- |
| CPT RX GAIN CPT /ACOB: 24 (00-63) | - Press BTN 1. The Rx-gain of each device can be programmed. |
| CPT RX GAIN CPT /DCOB: 24 (00-63) | - Press BTN 2 (Select a device type to change gain). The LCD shows RX gain of the device from other devices. |
| CPT RX GAIN CPT /DCOB: 45 (00-63) | - To change gain, press BTN and dial new gain (00-63) and LCD shows changed gain. - (Ex: BTN 2 and dial 45.) |
| CPT RX GAIN PRESS FLEX KEY (1-2) | - Press the [HOLD/SAVE] button for updating database permanently. Then main menu is displayed. |
| CPT RX GAIN PRESS FLEX KEY (1-2) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system main menu without updating system memory. |

## 15.11  MODEM RX GAIN CONTROL  (PGM 411)

PROCEDURE

| MODEM RX GAIN PRESS FLEX KEY (1-2) | - [TRANS/PGM] + 411. |
| --- | --- |
| MODEM RX GAIN MODEM /ACOB: 24 (00-63) | - Press BTN 1. The Rx-gain of each device can be programmed. |
| MODEM RX GAIN MODEM /DCOB: 24 (00-63) | - Press BTN 2 (Select a device type to change gain). The LCD shows RX gain of the device from other devices. |
| MODEM RX GAIN MODEM /DCOB: 45 (00-63) | - To change gain, press BTN and dial new gain (00-63) and LCD shows changed gain. - (Ex: BTN 2 and dial 45.) |
| MODEM RX GAIN PRESS FLEX KEY (1-2) | - Press the [HOLD/SAVE] button for updating database permanently. Then main menu is displayed. |
| MODEM RX GAIN PRESS FLEX KEY (1-2) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system main menu without updating system memory. |

## 15.12  PTRP RX GAIN CONTROL  (PGM 419)

PROCEDURE

| PTRP RX GAIN PRESS FLEX KEY (01-11) | - [TRANS/PGM] + 419. |
| --- | --- |
| PTRP RX GAIN PTRP-TX /DTIB: 01 (00-63) | - Press BTN 1. The Rx-gain of each device can be programmed. |
| PTRP RX GAIN PTRP-TX /SLIB: 01 (00-63) | - Press BTN 2 (Select a device type to change gain). The LCD shows RX gain of the device from other devices. |
| PTRP RX GAIN PTRP-TX /SLIB: 45 (00-63) | - To change gain, press BTN and dial new gain (00-63) and LCD shows changed gain. - (Ex: BTN 2 and dial 45.) |
| PTRP RX GAIN PRESS FLEX KEY (01-11) | - Press the [HOLD/SAVE] button for updating database permanently. Then main menu is displayed. |
| PTRP RX GAIN PRESS FLEX KEY (01-11) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system main menu without updating system memory. |

## 15.13  SYSTEM TONE FREG  (PGM 420)

Frequency, user entered (dial tone, ring back tone, error tone, busy tone, dummy dial tone), may be changed to the closest system frequency that provides.

PROCEDURE

| SYS-TONE FREQUENCY DIAL (1-5) | - [TRANS/PGM] + 420. |
| --- | --- |
| DIAL TONE FREQUENCY T1:0425 T2:0000 | - To change system tone frequency, dial 1-5 and press BTN 1 (T1) or BTN 2 (T2) and dial 4 digits for new frequency. |
| SYS-TONE FREQUENCY DIAL (1-5) | - Press the [HOLD/SAVE] button for updating database permanently. Then main menu is displayed. |
| SYS-TONE FREQUENCY DIAL (1-5) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system main menu without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REAMRK |
| --- | --- | --- | --- | --- |
| 1 | Dial Tone | 0000 - 9999 | T1: - T2: - | Nation specific |
| 2 | Ring Back Tone | 0000 - 9999 | T1: - T2: - | Nation specific |
| 3 | Busy Tone | 0000 – 9999 | T1: - T2: - | Nation specific |
| 4 | Error Tone | 0000 – 9999 | T1: - T2: - | Nation specific |
| 5 | Dummy Dial Tone | 0000- 9999 | T1: - T2: - | Nation specific |

TABLE 15.13.1  Button Configuration for System Tone Frequency  (PGM 420)

## 15.14  DIFF RING FREGUENCY  (PGM 421)

Frequency, user entered, may be changed to the closest system frequency that provides.

PROCEDURE

| DIFF RING FREQUENCY DIAL (1-4) | - [TRANS/PGM] + 421. |
| --- | --- |
| DIFF RING FREQ(RNG 1) T1:1000 T2:1020 | - To change system tone frequency, dial 1-4 and press BTN 1 (T1) or BTN 2 (T2) and dial 4 digits for new frequency. |
| DIFF RING FREQUENCY DIAL (1-4) | - Press the [HOLD/SAVE] button for updating database permanently. Then main menu is displayed. |
| DIFF RING FREQUENCY DIAL (1-4) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system main menu without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REAMRK |
| --- | --- | --- | --- | --- |
| 1 | Ring 1 | 0000 - 9999 | T1: - T2: - | Nation specific |
| 2 | Ring 2 | 0000 - 9999 | T1: - T2: - | Nation specific |
| 3 | Ring 3 | 0000 – 9999 | T1: - T2: - | Nation specific |
| 4 | Ring 4 | 0000 – 9999 | T1: - T2: - | Nation specific |

TABLE 15.14.1  Button Configuration for Differential Ring Frequency  (PGM 421)

## 15.15  DISTINCT RING FREGUENCY  (PGM 422)

Frequency, user entered, may be changed to the closest system frequency that provides.

PROCEDURE

| DISTINCT RING FREQUENCY DIAL (1-4) | - [TRANS/PGM] + 422. |
| --- | --- |
| DIST RING FREQ(RNG 1) T1:0480 T2:0000 | - To change system tone frequency, dial 1-4 and press BTN 1 (T1) or BTN 2 (T2) and dial 4 digits for new frequency. |
| DIST RING FREQUENCY DIAL (1-4) | - Press the [HOLD/SAVE] button for updating database permanently. Then main menu is displayed. |
| DIST RING FREQUENCY DIAL (1-4) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system main menu without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REAMRK |
| --- | --- | --- | --- | --- |
| 1 | Ring 1 | 0000 - 9999 | T1: - T2: - | Nation specific |
| 2 | Ring 2 | 0000 - 9999 | T1: - T2: - | Nation specific |
| 3 | Ring 3 | 0000 – 9999 | T1: - T2: - | Nation specific |
| 4 | Ring 4 | 0000 – 9999 | T1: - T2: - | Nation specific |

TABLE 15.15.1  Button Configuration for Distinct Ring Frequency  (PGM 422)

## 15.16  ACNR TONE CADENCE  (PGM 423)

Frequency, user entered, may be changed to the closest system frequency that provides.

PROCEDURE

| ACNR TONE CADENCE (20MS) DIAL (1-4) |  |  | - [TRANS/PGM] + 423. |  |  |
| --- | --- | --- | --- | --- | --- |
| RBACK TONE CADENCE ON:050 OFF:100 (20MS) |  |  | - To change system tone frequency, dial 1-4 and press BTN 1 (T1) or BTN 2 (T2) and dial 4 digits for new frequency. |  |  |
| ACNR TONE CADENCE (20MS) DIAL (1-4) |  |  | - Press the [HOLD/SAVE] button for updating database permanently. Then main menu is displayed. |  |  |
| ACNR TONE CADENCE (20MS) DIAL (1-4) |  |  | - Press [CONF] button instead of the [HOLD/SAVE] button, then system main menu without updating system memory. |  |  |
| BTN | ITEM | RANGE |  | DEFAULT | REMARK |
| 1 | Ring-Back Tone | 000 - 255 |  | ON: 050 / OFF: 100 | 20msec base |
| 2 | Busy-Tone | 000 - 255 |  | ON: 025 / OFF: 025 | 20msec base |
| 3 | Error-Tone | 000 - 255 |  | ON: 012 / OFF: 012 | 20msec base |
| 4 | S-Dial-Tone | 000 - 255 |  | ON: 070 / OFF: 000 | 20msec base |

TABLE 15.16.1  Button Configuration for ACNR Cadence (PGM 423)

# 16  INITIALIZATION (PGM 450)

The system has been pre-programmed with certain features which are called default data. These features are loaded into memory when the system is initialized. The system should be always initialized when installed or at any time the database has been corrupted. To initialize the system to the default values, proceed as follows.

PROCEDURE

| INITIALIZATION PRESS FLEX KEY (1-14) | - [TRANS/PGM] + 450. |
| --- | --- |
| INITIALIZATION FLEX NUM PLAN | - Press one of button (1-14) as described below table.(ex Flx1) |
| INITIALIZATION FLEX NUM PLAN | - Press the [HOLD/SAVE] button, then database is initialized with default data. Confirm Tone is heard. |

| BTN | ITEM | REMARK |
| --- | --- | --- |
| 1 | Flexible Numbering Plan Initialization |  |
| 2 | Station Database Initialization |  |
| 3 | CO Line Database Initialization |  |
| 4 | System Feature Database Initialization |  |
| 5 | Station Group Database Initialization |  |
| 6 | ISDN Tables Database Initialization |  |
| 7 | Wireless Database Initialization |  |
| 8 | System Timer Database Initialization |  |
| 9 | Toll Table Database Initialization |  |
| 10 | LCR Database Initialization |  |
| 11 | Tables Initialization |  |
| 12 | Flexible Button Program Initialization |  |
| 13 | All Database Initialization |  |
| 14 | System Reset By Software |  |

TABLE 16.1  Initialization (PGM 450)

# 17.	PRINT PROT DATABASE  (PGM 451)

In order to obtain a hard copy printout of the database, a printer must be connected to the RS-232C connector.

PROCEDURE

| PROT DATA PRINT PRESS FLEX KEY (01-14) | - [TRANS/PGM] + 451. |
| --- | --- |
| PROT DATA PRINT STATION DATA | - To print database, press one of BTN 01-15 and if there are entered data for selecting printed part. Then selected database name will be displayed on the LCD. |
| PROT DATA PRINT STATION DATA | - To print out the selected database at step (2), press the [HOLD/SAVE] button. (Ex. Sation data). After printing database, confirm tone is heard. |

| BTN | ITEM |  | Range | Default | REMARK |
| --- | --- | --- | --- | --- | --- |
| 1 | Flexible Numbering Plan Print |  |  |  |  |
| 2 | Station Database Print |  | STN_R |  |  |
| 3 | CO Line Database Print |  | CO_R |  |  |
| 4 | System Feature Database Print |  |  |  |  |
| 5 | Station Group Database Print |  |  |  |  |
| 6 | ISDN Tables Database Print |  |  |  |  |
| 7 | System Timer Database Print |  |  |  |  |
| 8 | Toll Table Database Print |  |  |  |  |
| 9 | LCR Database Print |  |  |  |  |
| 10 | Other Tables Print |  |  |  |  |
| 11 | Nation Specific Database Print |  |  |  |  |
| 12 | Flexible Button Program Print |  | STN_R |  |  |
| 13 | All Database Print |  |  |  |  |
| 14 | LCD Message Print |  |  |  |  |
|  | 1 | Language | 00 – 12 | Nation specific | 00:ENG01:KOR02:ITA 03:SWE04:NOR05:FIN 06:DUT07:SPA08:DAN 09:GER10:EST11:RUS12:POR |
|  | 2 | Stn Type | 0 – 2 | 0 | 0: NORMAL1: LG-GAP2: LARGE |

TABLE 17.1  Data Base Print (PGM 451)

(Printing Example )

This printing example is in case of GDK-100 system. In GDK-162/GDK-FPII systems, printing format is the same as GDK-100 except the system size capacities.

|  | Flexible Numbering Plan |  |  |  | FLEXIBLE NUMBERING PLAN ----------------------- FLEX STATION NUMBER 0 1 2 3 4 5 6 7 8 100 101 102 103 104 105 106 107 108 . . 63 64 65 66 67 68 69 70 71 163 164 165 166 167 168 169 170 171 STATION GROUP PILOT NUMBER : 620 - 634 SLT PGM MODE SELECT NUMBER : 3 INTERNAL PAGE ZONES NUMBER : 401 - 415 INTERNAL ALL CALL PAGE : 43 MEET ME PAGE : 44 EXTERNAL PAGE ZONE 1 : 46 EXTERNAL PAGE ZONE 2 : 47 EXTERNAL ALL CALL PAGE : 48 ALL CALL PAGE : 49 SMDR ACCOUNT CODE ENTER : 50 FLASH COMMAND TO CO LINE : 51 SLT LAST SPD_DIAL : 52 DND : 53 CALL FORWARD : 54 SPEED DIAL PROGRAM : 55 MESSAGE WAIT ENABLE : 56 MESSAGE WAIT RETURN : 57 SPEED DIAL ACCESS : 58 DND/FWD CANCEL : 59 CALL PARK LOCATIONS : 601 - 610 ALARM RESET : 65 GROUP CALL PICK UP : 66 STATION DVU : 67 NIGHT ANSWER : 69 DIRECTED CALL PICK UP : 7 CO GROUP ACCESS : 801 - 824 INDIVIDUAL CO ACCESS : 8801 - 8848 TIE ROUTING : 89 ACCESS CO IN 1ST CO GROUP : 9 ATTENDANT CALL : 0 PRINT SMDR : *0 PRINT SMDR ACCOUNT GROUP : *1 DELETE SMDR : *2 DELETE SMDR ACCOUNT G BASE : *3 DISPLAY CALL CHARGE : *4 ABORT PRINTING : ** RETRIEVE HELD CO 4 BTN : 8## RETRIEVE IND HELD CO 4 BTN : 8#01 - 8#48 DOOR OPEN RELAY 1 : #*1 DOOR OPEN RELAY 2 : #*2 DOOR OPEN RELAY 3 : #*3 DOOR OPEN RELAY 4 : #*4 |
| --- | --- | --- | --- | --- | --- |
|  | Slot Assignment |  |  |  | SLOT ASSIGNMENT --------------- 0 1 2 3 4 5 6 7 8 MPB DTIB ETIB SLIB SLIB DTIB LCOB BRIB DCOB 0 0 1 2 3 4 2 1 0 9 10 11 12 SLIB WTIB DIDB PRIB 5 6 4 3 |
|  | Station Attributes |  |  |  | STATION ATTRIBUTES ------------------ STA 100 -------- DND SPDA PAGE FWD PLA ASPK WARM SMDRH QUE WTONE OVRD AHLD N Y N N Y Y N N Y N N N DSEC MON SPK ALM IBOX COL NTA DVU VOVR DMRX PCALL N N Y Y Y Y N N N N N ID : KEYSET COS : DAY(1) / NIGHT(1) LINE GROUP ACCESS : 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 ICM TENANCE GROUP : 1 GATD : .... ACCESS GROUP : 1 PAGE : 1 PREFWD : .... IDLE LINE SELECTION : .... SMDR ACCOUNT GROUP : .. STA 101 -------- : (Printed like above for another station to range end station.) |
|  | Flex Buttons Assignment |  |  |  | FLEX BUTTON ASSIGNMENT ---------------------- STA 100 : 1 2 3 4 25 26 27 28 CO 1 CO 2 CO 3 CO 4 E102 E105 N66 N#*1 5 6 7 8 29 30 31 32 CO 5 CO 6 CO 7 CO 8 S01 S19 S200 S299 . . 21 22 23 24 45 46 47 48 COG01 COG02 COG11 LOOP USER USER USER USER STA 101 : : ( Printed like above for all keysets ) |
|  | CO Line Attributes |  |  |  | CO 1 ----- GRP COS COACT DISA DISAACT TYPE SIGNAL UNA FLASH 1 1 N U Y CO DTMF N LOOP METER FLASH LOOP TIE DID NO 5 0 NM IMDT CO LINE 1 :: DAY DELAY RING ASSIGN 100(U) 101(0) 102(U) 103(U) 104(U) 105(U) 106(U) 107(U) . . 164(U) 165(U) 166(U) 167(U) 168(U) 169(U) 170(U) 171(U) CO LINE 1 :: NIGHT DELAY RING ASSIGN 100(U) 101(0) 102(U) 103(U) 104(U) 105(U) 106(U) 107(U) . . 164(U) 165(U) 166(U) 167(U) 168(U) 169(U) 170(U) 171(U) CO 2 ----- : ( Printed like above for another CO line ) |
| System Database |  |  | NATIONALITY : KOREA (OFFICE) SYSTEM PARAMETERS ------------------ SHLD PRIV PWRN PAGE OFFR MCNF BGM MOH BOX MUSIC LANG CLIP SYS Y Y Y MUTE Y 1 1 1 INT KOR NO EXTR ACEMOD W_TIME C/F(TEMP) ISD_LENGTH ISD_CODE N LCHOICE N CELSIUS 3 00 EHR SHR XFR IRC ARC CRD PT CRG CWT CDD CPT PFT CFT APT 60 30 30 30 1 2 3 20 180 1 120 10 10 30 ADT ATT ANA ARC WRM CID PRE MRT HST HSB DNA ICM DOT FNA DRC 30 15 30 3 5 3 15 0 5 1 20 30 20 15 3 DTT IDT ART PTO RSD WKT MHF SMD DVU CDA ACD RPH BXF SAR 10 5 30 15 60 10 20 0 20 3 0 5 50 60 SMDR PASSWD MAIN-ATD 0 .... 101 .... .... .... .... < WORLD TIME > BIN # : 0 NAT_CODE : .... AREA_CODE : .... . STATION GROUPS -------------- STATION GROUPS 620: NOT ASSIGN . EXEC/SEC PAIR : 1 ..../.... 7 ..../.... . 6 ..../.... 12 ..../.... ALARM ATTRIBUTES ENAB MODE CONTACT SIGNAL N ALARM CLOSE REPT EXT CONTROL CONTACT : 1 : ........ 2 : ........ . PABX ACCESS CODES : .. .. .. .. .. LONG DIST CODES : .. .. .. .. .. AUTHORIZATION CODES : DID-RCV-DGT DID-CONV DID-2ND-DIGIT-CONVERSION 3 #*** 1234567890 -> 1234567890 PULSE DIAL SPEED RATIO : 10PPS 66/33 MODEM ASSIGN : MODE BAUD ASC_DEV NO. MODEM 02400 STA 171 DATE & TIME : 94 10 20 11:45 LCD DISPLAY : 12H DDMMYY |  |  |
| System Database |  |  |  | < VM DIALING TABLE > PREFIX SUFFIX 1 .#E E . 8 .#*6.E E VERSION : LGE/GS00P-1.0Aa OCT/95 SAVE PRT TYPE PIC DETL BAUD CRCY N N LD N Y 09600 ... COST 000FRAC 2 TIE LINE ROUTING TABLE 1 = .. .. .. .. .. .. 2 = .. .. .. .. .. .. . 29 = .. .. .. .. .. .. 30 = .. .. .. .. .. .. CUSTOM CALL ROUTING #1 #2 #3 #4 #5 #6 #7 #8 #9 .. .. .. .. .. .. .. .. .. |  |
| Toll Tables |  | EXCEPTION TABLES ---------------- ALLOW TABLE 1=E 2=E 3=E 4=E 5=E 6=E 7=E 8=E 9=E 10=E 11=E 12=E 13=E 14=E 15=E 16=E 17=E 18=E 19=E 20=E DENY TABLE A 1=E 2=E 3=E 4=E 5=E 6=E 7=E 8=E 9=E 10=E ALLOW TABLE B 1=E 2=E 3=E 4=E 5=E 6=E 7=E 8=E 9=E 10=E 11=E 12=E 13=E 14=E 15=E 16=E 17=E 18=E 19=E 20=E DENY TABLE B 1=E 2=E 3=E 4=E 5=E 6=E 7=E 8=E 9=E 10=E CANNED TOLL TABLES ------------------ ALLOW TABLE 1=011E 2=012E 3=015E 4=080E 5=E 6=E 7=E 8=E 9=E 10=E DENY TABLE 1=101E 2=115E 3=E 4=E 5=E 6=E 7=E 8=E 9=E 10=E SYSTEM SPEED GROUPING COMMON TOLL_CHK : CHECK |  |  |  |
| System Speed Dial Numbers |  | SYSTEM SPEED NUMBERS -------------------- 200=................. 201=................. : |  |  |  |
| All Data |  | COMPLETE DATABASE PRINTING -------------------------- print above all |  |  |  |