Digital Key Telephone System LDK-300 and LDK-100 Programming Manual Admin Programming

CONTENTS

1.	CUSTOMER DATABASE PROGRAMMING	1

1.1	INTRODUCTION	1

1.2	TO ENTER THE PROGRAMMING MODE	1

1.3	PERMANENT UPDATE PROCEDURE	2

1.4	NUMBERING PLAN	2

1.5	ADMIN PROGRAMMING INDEX	6

1.6	DEFAULT VALUES	8

2.	PRE-PROGRAMMED DATABASE	35

2.1	LOCATION PROGRAM  (PGM 100)	35

2.2	RACK SLOT ASSIGNMENT  (PGM 101)	37

2.3	WTIB PORT NUMBER ASSIGNMENT  (PGM 102)	39

2.4	LOGICAL SLOT ASSIGNMENT  (PGM 103)	39

2.5	NUMBERING PLAN TYPE  (PGM 104)	40

2.6	FLEXIBLE NUMBERING PLAN  (PGM 105)	41

2.7	FLEXIBLE NUMBERING PLAN  (PGM 106-107)	42

2.8	IP SETTING  (PGM 108)	44

3.	STATION PROGRAMMING	45

3.1	STATION ID  (PGM 110)	45

3.2	STATION ATTRIBUTE – I  (PGM 111)	48

3.3	STATION ATTRIBUTE - II  (PGM 112)	49

3.4	STATION ATTRIBUTE - III  (PGM 113)	51

3.5	ISDN STATION ATTRIBUTE  (PGM 114)	53

3.6	FLEX BUTTON ASSIGNMENT  (PGM 115)	55

3.7	STATION COS  (PGM 116)	57

3.8	CO LINE GROUP ACCESS  (PGM 117)	59

3.9	INTERNAL PAGE ZONE  (PGM 118)	60

3.10	CONFERENCE PAGE ZONE  (PGM 119)	61

3.11	ICM TENANCY GROUP  (PGM 120)	62

3.12	PRESET CALL FORWARD  (PGM 121)	63

3.13	HOT LINE / WARM LINE  (PGM 122)	64

3.14	CTI STATION ATTRIBUTE  (PGM 123)	65

3.15	SMDR ACCOUNT GROUP  (PGM 124)	66

4.	CO LINE PROGRAMMING	67

4.1	CO SERVICE TYPE  (PGM 140)	67

4.2	CO LINE ATTRIBUTE – I  (PGM 141)	69

4.3	CO LINE ATTRIBUTE – II  (PGM 142)	71

4.4	ISDN CO LINE ATTRIBUTE  (PGM 143)	73

4.5	CO RING ASSIGNMENT  (PGM 144)	74

4.6	CO RING ASSIGNMENT DISPLAY (PGM 145)	75

5. SYSTEM DATA PROGRAMMING	76

5.1	SYSTEM ATTRIBUTE - I  (PGM 160)	76

5.2	SYSTEM ATTRIBUTE - II  (PGM 161)	78

5.3	ADMIN PASSWORD  (PGM 162)	80

5.4	ALARM ATTRIBUTES  (PGM 163)	81

5.5	ATTENDANT ASSIGNMENT  (PGM 164)	82

5.6	AUTO ATTENDANT VMIB ANNC ASSIGNMENT  (PGM 165)	83

5.7	CO-TO-CO COS  (PGM 166)	84

5.8	DID/DISA DESTINATION  (PGM 167)	85

5.9	EXTERNAL CONTROL CONTACT  (PGM 168)	86

5.10	LCD TIME/DATE/LANGUAGE DISPLAY MODE  (PGM 169)	87

5.11	MODEM  (PGM 170)	88

5.12	MUSIC  (PGM 171)	89

5.13	PBX ACCESS CODE  (PGM 172)	91

5.14	PLA PRIORITY SETTING  (PGM 173)	91

5.15	RS-232C PORT SETTING  (PGM 174)	92

5.16	PRINT PORT SELECTION  (PGM 175)	93

5.17	PULSE DIAL RATIO  (PGM 176)	94

5.18	SMDR ATTRIBUTES  (PGM 177)	95

5.19	SYSTEM TIME/DATE SETTING  (PGM 178)	98

5.20	LINKED STATION PAIRS TABLE  (PGM 179)	99

5.21	CIDU SETTING  (PGM 185)	100

6.  SYSTEM TIMERS	101

6.1	SYSTEM TIMERS - I  (PGM 180)	101

6.2	SYSTEM TIMERS - II  (PGM 181)	103

6.3	SYSTEM TIMERS - III  (PGM 182)	105

7.	DCOB ATTRIBUTE (PGM 186 - PGM 187)	106

7.1	DCOB SYS Attribute (PGM 186)	106

7.2	DCOB CO Line Attribute  (PGM 187)	107

8.	STATION GROUP PROGRAMMING (PGM 190 - PGM 191)	108

8.1	STATION GROUP ASSIGN  (PGM 190)	109

8.2	STATION GROUP ATTRIBUTE  (PGM 191)	110

8.2.1	CIRCULAR/TERMINAL GROUP ATTRIBUTE	110

8.2.2 	UCD GROUP ATTRIBUTE	112

8.2.3	RING GROUP ATTRIBUTE	114

8.2.4	VM GROUP ATTRIBUTE	116

8.2.5	PICK-UP GROUP ATTRIBUTE	118

9.	ISDN PROGRAM	119

9.1	ISDN ATTRIBUTE  (PGM 200)	119

9.2	COLP TABLE  (PGM 201)	121

9.3	MSN TABLE  (PGM 202)	122

10.	LCR	123

10.1	LCR ATTRIBUTES  (PGM 220)	123

10.2	LEADING DIGIT TABLE  (PGM 221)	125

10.3	DIGIT MODIFICATION TABLE  (PGM 222)	127

10.4	LCR TABLE INITIALIZATION  (PGM 223)	129

11.	TOLL TABLE	131

11.1	TOLL EXCEPTION TABLE  (PGM 224)	131

11.2	CANNED TOLL TABLE  (PGM 225)	133

11.3	EMERGENCY CODE TABLE  (PGM 226)	134

12.	TABLES	135

12.1	AUTHORIZATION CODE TABLE  (PGM 227)	135

12.2	CUSTOM CALL ROUTING  (PGM 228)	136

12.3	EXECUTIVE / SECRETARY TABLE  (PGM 229)	138

12.4	DID DIGITS CONVERSION TABLE  (PGM 230)	139

12.5	FLEXIBLE DID TABLE  (PGM 231)	140

12.6	SYSTEM SPEED ZONE  (PGM 232)	142

12.7	WEEKLY TIME TABLE  (PGM 233)	143

12.8	VOICE MAIL DIALING TABLE  (PGM 234)	144

12.9	TIE ROUTING TABLE  (PGM 235)	145

13.	NETWORKING ATTRIBUTE	146

13.1	Networking Basic Attribute  (PGM 320)	146

13.2	Networking Supplementary Attribute  (PGM 321)	147

13.3	Networking CO Line Attribute  (PGM 322)	148

13.4	Networking Attendant Assignment (PGM 323)	149

13.5	Networking Routing Table (PGM 324)	150

14.	VOIB ATTRIBUTE	151

14.1	VOIP IP SETTING  (PGM 340)	151

15.	NATION SPECIFIC PROGRAMMING	152

15.1	DTIB RX GAIN CONTROL  (PGM 400)	152

15.2	SLIB RX GAIN CONTROL  (PGM 401)	153

15.3	CTR SLIB RX GAIN CONTROL  (PGM 402)	153

15.4	WTIB RX GAIN CONTROL  (PGM 403)	154

15.5	ACOB RX GAIN CONTROL  (PGM 404)	154

15.6	CTR ACOB RX GAIN CONTROL  (PGM 405)	155

15.7	DCOB RX GAIN CONTROL  (PGM 406)	155

15.8	VMIB RX GAIN CONTROL  (PGM 407)	156

15.9	DTMF RCVR RX GAIN CONTROL  (PGM 408)	156

15.10 EXT PAGE RX GAIN CONTROL  (PGM 409)	157

15.11 CPT RX GAIN CONTROL  (PGM 410)	157

15.12  MODEM RX GAIN CONTROL  (PGM 411)	158

15.13  SHORT SLIB GAIN CONTROL  (PGM 412)  ( SAF only )	159

15.14  LONG SLIB GAIN CONTROL  (PGM 413)  ( TELKOM only )	160

15.15  FAR SLIB GAIN CONTROL  (PGM 414)  ( TELKOM only )	160

15.16  SHORT ACO GAIN CONTROL  (PGM 415)   ( TELKOM only )	161

15.17  LONG ACO GAIN CONTROL  (PGM 416)  ( TELKOM only )	162

15.18  SYSTEM TONE FREQUENCY  (PGM 420)	163

15.19  DIFFERENTIAL RING FREQUENCY  (PGM 421)	164

15.20  DISTINCT RING FREQUENCY  (PGM 422)	165

15.21  ACNR TONE CADENCE  (PGM 423)	166

16.	INITIALIZATION  (PGM 450)	167

17.	PRINT PROT DATABASE  (PGM 451)	168

# 1.	CUSTOMER DATABASE PROGRAMMING

## 1.1	INTRODUCTION

The LDK Key Telephone System can be programmed to meet each customer's individual need. All programming is done at station 100 (station port # 00) using KD-36D or LKD-30/44 digital key telephone. (You cannot program with Large LCD keyset.) Additional programming stations may be assigned (PGM 113-BTN 1), but only 1 keyset can be active in programming mode at any one time.

Upon entering the program mode, the key telephone at station 100 cannot operate as a normal telephone but as a programming instrument with all of the buttons redefined. The keys of the dial pad are used to enter the various data fields and to enter numerical information. The 24 buttons located at the top of the phone (Flexible Buttons) are used to indicate the specific data field and to enter information. Sometimes the [SPEED] button and ‘*’ of the dial pad is used to delete the data or to indicate end of data input and the [REDIAL] button is used to delete one digit or character from the end of entered digits or characters.

See TABLE 1.6.1 - 1.6.27 for default data. If this pre-programming suits the customer, additional admin program is not necessary. To change admin data, the user enters the admin programming mode and select program code. During admin programming, other keysets operate normally. (While activating LDK Admin, no keyset can enter Admin mode and the reverse also.)

When Admin programming, LCD and LEDs indicate the current programmed data and status. If the programmer enters correct data, then LCD and LEDs show the entered data and the data is stored in the temporary buffer area. Real system database is not changed and has no effect on telephone operation unless permanent updating procedure is executed. Pressing the [HOLD/SAVE] button, all data in the temporary buffer (same as LCD and LEDs show their status) is saved into permanent memory. Tones are provided to let the programmer know data entry is correct (confirmation tone) or not (error tone).

To return the parent state while admin programming, press the [CONF] button. Pressing the [CONF] button, temporary data fields are cleared.

To reset the system, enter PGM 450 – BTN 15 and press [HOLD/SAVE] button. Or, the system will be reset automatically after programming PGM100 – BTN 1 (Nation Code Assign)

## 1.2	TO ENTER THE PROGRAMMING MODE

1. 	Lift handset or press the [MON] button on the admin station, and hear ICM dial tone (optional).

- Press the [TRANS/PGM] button and dial ＊# (Confirmation tone is heard).

- Enter admin password if the password has been set. This places the station into the admin programming mode (Confirmation tone is heard).

4.	Each program is accessed by pressing the [TRANS/PGM] button and dialing the three-digit program number. If an error is made while entering data, the [TRANS/PGM] button can go the previous status. When the [TRANS/PGM] button is pressed, the LCD will display;

ENTER PGM NUMBER

## 1.3	PERMANENT UPDATE PROCEDURE

When the data has been entered, the [HOLD/SAVE] button is used to store the data permanently. If all data was entered correctly, confirmation tone is heard when pressing the [HOLD/SAVE] button. If there were any errors in the entry, then an error tone is presented and data is not stored in the permanent memory.

## 1.4	NUMBERING PLAN

The following numbering plan can be changed by Admin Programming 104-107 depending on the user's needs.

| NUMBER |  | ITEM | REMARK |
| --- | --- | --- | --- |
| LDK-300 | LDK-100 |  |  |
| 100 – 399 | 100-227 | Intercom Call |  |
| 620 – 667 | 620-634 | Group Pilot Number |  |
| 501 – 535 | 501-515 | Internal Page Zone |  |
| 543 | 543 | Internal All Call Page |  |
| 544 | 544 | Meet Me Page |  |
| 545 | 545 | External Page Zone 1 |  |
| 546 | 546 | External Page Zone 2 |  |
| 547 | 547 | External Page Zone 3 |  |
| 548 | 548 | External All Call Page |  |
| 549 | 549 | All Call Page (Int & Ext) |  |
| 550 | 550 | SMDR Account Code Enter | SLT |
| 551 | 551 | Flash Command to CO Line | SLT |
| 552 | 552 | Last Number Redial | SLT |
| 553 | 553 | DND (Toggle On/Off) | SLT |
| 554 | 554 | Call Forward | SLT |
| 555 | 555 | Speed Dial Programming | SLT |
| 556 | 556 | Message Wait/Callback Enable | SLT |
| 557 | 557 | Message Wait/Callback Return | SLT |
| 558 | 558 | Speed Dial Access | SLT |
| 559 | 559 | Cancel DND/FWD/Pre-MSG | SLT |
| 560 | 560 | System Hold | SLT |
| 561 | 561 | Station Relocation Backup |  |
| 562 | 562 | Station Relocation Retrieve |  |
| 563 | 563 | Programming Mode Enter Code | SLT |
| 564 | 564 | ACD Reroute |  |
| 565 | 565 | Alarm Reset |  |
| 566 | 566 | Group Call Pickup |  |
| 568 | 568 | UCD DND |  |
| 569 | 569 | Night Answer |  |
| 601 - 619 | 601-610 | Call Parking Locations |  |
| 7 | 7 | Direct Call Pickup |  |
| 801-872 | 801-824 | CO Group Access |  |
| 88001-88200 | 8801-8840 | Individual CO Access |  |
| 8901 | 8901 | Tie Routing Access |  |

| NUMBER |  | ITEM | REMARK |
| --- | --- | --- | --- |
| LDK-300 | LDK-100 |  |  |
| 8* | 8* | Retrieve Held CO Line |  |
| 8#xxx | 8#xx | Retrieve Held Individual CO Line |  |
| 9 | 9 | Access CO Line In the 1st available CO Group |  |
| 0 | 0 | Attendant Call |  |
| #*1 | #*1 | 1st Door Open |  |
| #*2 | #*2 | 2nd Door Open |  |
| #*3 | #*3 | 3rd Door Open |  |
| #*4 | #*4 | 4th Door Open |  |
| #*5 | #*5 | 5th Door Open |  |
| #*6 | #*6 | 6th Door Open |  |
| #*7 | N/A | 7th Door Open | LDK-300 Only |
| *8 | *8 | VM Message Waiting Enable |  |
| *9 | *9 | VM Message Waiting Disable |  |

- To enter user programming mode, press the [TRANS/PGM] button in a keyset or dial 5 6 3 (Programming enter code) in a SLT.

- The following numbering plan is fixed, so it cannot be changed by Admin Programming.

- For the stations

| NUMBER | ITEM | REMARK |
| --- | --- | --- |
| 11 | Differential Ring | Keyset |
| 12 | Intercom Answer Mode (1 HF / 2 TONE / 3 PV) | Keyset |
| 21 | Station COS Down |  |
| 22 | Station COS Restore |  |
| 23 | Walking COS | Keyset |
| 31 | Authorization Code Registration |  |
| 32 | Authorization Code Change |  |
| 41 | Wake-up Time Registration (One-time/ Continuous) |  |
| 42 | Wake-up Time Cancel |  |
| 51 | Pre-selected MSG Activation |  |
| 52 | Set Custom Message |  |
| 61 | Record VMIB User Greeting |  |
| 62 | Listen VMIB Time & Date |  |
| 63 | Listen VMIB Station Number |  |
| 64 | Listen VMIB Station Status |  |
| 65 | Record VMIB Page Message |  |
| 66 | Erase VMIB User Greeting |  |
| 67 | Erase VMIB Page Message |  |
| 71 | LCD Display Mode (English/Domestic Language) | Keyset |
| 72 | MPB Version Display | Keyset |
| 73 | Background Music | Keyset |
| 74 | Station User Name Registration |  |
| 75 | Headset/Speakerphone Mode | Keyset |
| 76 | Headset Ring Mode | Keyset |
| 77 | WTU Station Number Receive | Keyset |
| 78 | Serial No/SW Packages | Keyset with LCD |

- For the attendant

| NUMBER | ITEM | REMARK |  |
| --- | --- | --- | --- |
| 0111 | Print SMDR (Station Base) | System Attendant |  |
| 0112 | Delete SMDR (Station Base) | System Attendant |  |
| 0113 | Print SMDR (Group Base) | System Attendant |  |
| 0114 | Delete SMDR (Group Base) | System Attendant |  |
| 0115 | Display Call Charge | System Attendant |  |
| 0116 | Abort Printing | System Attendant |  |
| 0117 | Print Lost Call | System Attendant |  |
| 0118 | Delete Lost Call | System Attendant |  |
| 0121 | Print All Summary | System Attendant |  |
| 0122 | Print All Periodically | System Attendant |  |
| 0123 | Abort Periodic Printing | System Attendant |  |
| 0124 | Print ATD Traffic | System Attendant |  |
| 0125 | Print Call Summary | System Attendant |  |
| 0126 | Print All Hourly | System Attendant |  |
| 0127 | Print H/W Usage | System Attendant |  |
| 0128 | Print CO Summary | System Attendant |  |
| 0129 | Print CO Hourly | System Attendant |  |
| 021 | Station COS Down (COS 7) |  | Attendant |
| 022 | Station COS Restore |  | Attendant |
| 031 | Authorization Code Cancel |  | System Attendant |
| 041 | System Date/Time Setting |  | Attendant |
| 042 | Wake-up Time Registration (One-time /Continuous) |  | Attendant |
| 043 | Wake-up Time Cancel |  | System Attendant |
| 044 | LCD Date Mode Change |  | System Attendant |
| 045 | LCD Time Mode Change |  | System Attendant |
| 046 | Use Network Time & Date |  | System Attendant |
| 051 | Pre-select MSG Activation |  | Attendant |
| 052 | Pre-select MSG Deactivation |  | Attendant |
| 053 | Custom Display Message Program (11-20) |  | System Attendant |
| 054 | Erase VM MSG |  | Attendant |
| 06 | Record VMIB System Greeting | System Attendant |  |
| 071 | DND/Call Forward/Pre-selected MSG Cancel |  | Attendant |
| 072 | Register Station Name |  | Attendant |
| 073 | Disable CO Outgoing |  | System Attendant |
| 074 | Automatic Day/Night/Weekend Mode Program | Attendant |  |
| 075 | ICM BOX BGM Channel select | Attendant |  |
| 076 | External Page Music -1 Assignment/Cancel | Attendant |  |
| 077 | External Page Music -2 Assignment/Cancel |  | Attendant |
| 078 | External Page Music -3 Assignment/Cancel | Attendant |  |
| 079 | Prepaid Call |  |  |
| 07* | LCD Display Language |  |  |
| 0# | WHTU Subscription |  |  |

- Flexible Button Programming Code

| NUMBER | ITEM | REMARK |
| --- | --- | --- |
| 11 | Differential Ring |  |
| 21 | Station COS Down |  |
| 22 | Station COS Restore |  |
| 23 | Walking COS |  |
| 31 | Authorization Code Registration |  |
| 32 | Authorization Code Change |  |
| 41 | Wake-up Time Registration (One-time /Continuous) |  |
| 42 | Wake-up Time Cancel |  |
| 51 | Pre-selected MSG Activation |  |
| 52 | Set Custom Message |  |
| 53 | CLIR Key |  |
| 54 | Two Way Recording |  |
| 61 | Record VMIB User Greeting |  |
| 64 | Listen VMIB Station Status |  |
| 66 | Erase VMIB User Greeting |  |
| 71 | LCD Display Mode (English/Domestic Language) |  |
| 73 | Background Music |  |
| 74 | Station User Name Registration |  |
| 75 | Headset/Speakerphone Mode |  |
| 76 | HeadSet Ring Mode |  |
| 80 | Account Code Activation |  |
| 81 | DID Call Wait |  |
| 83 | [ICM Hold] BTN Assignment |  |
| 84 | [LOOP] BTN Assignment |  |
| 85 | [Camp-on] BTN Assignment |  |
| 86 | [INTRUSION] BTN Assignment | System Attendant |
| 87 | [UCD DND] BTN Assignment | + Hunt Grp No. |
| 89 | Keypad Facility Key |  |
| 8* | {ACD STATUS} BTN Assignment |  |
| 91 | [CONF] BTN Assignment | 2/8 BTN Keyset |
| 92 | [CALLBK] BTN Assignment | 2/8 BTN Keyset |
| 93 | [DND/FWD] BTN Assignment | 2/8 BTN Keyset |
| 94 | [FLASH] BTN Assignment | 2/8 BTN Keyset |
| 95 | [MUTE] BTN Assignment | 2/8 BTN Keyset |
| 96 | [MON] BTN Assignment | 2/8 BTN Keyset |
| 97 | [REDIAL] BTN Assignment | 2/8 BTN Keyset |

## 1.5	ADMIN PROGRAMMING INDEX

| MAIN MENU | PGM | ITEM |
| --- | --- | --- |
| PRE-PROGRAMMED DATABASE | 100 | Location Program |
|  | 101 | Rack Slot Assignment |
|  | 102 | WTIB Port number Assignment |
|  | 103 | Logical Slot Assignment |
|  | 104 | Numbering Plan Type |
|  | 105 | Flexible Number Plan – Station Number |
|  | 106 | Flexible Number Plan A |
|  | 107 | Flexible Number Plan B |
|  | 108 | IP Setting |
| STATION BASE PROGRAM | 110 | Station ID |
|  | 111 | Station Attribute I |
|  | 112 | Station Attribute II |
|  | 113 | Station Attribute III |
|  | 114 | ISDN Station Attribute |
|  | 115 | Flex Button Assignment |
|  | 116 | Station COS |
|  | 117 | CO Line Group Access |
|  | 118 | Internal Page Zone |
|  | 119 | Conference Page Zone |
|  | 120 | ICM Tenancy Group |
|  | 121 | Preset Call Forward |
|  | 122 | Hot/Warm Line Selection |
|  | 123 | CTI Station Attribute |
|  | 124 | SMDR Account Group |
| CO LINE BASE PROGRAM | 140 | CO Service Type |
|  | 141 | CO Line Attribute I |
|  | 142 | CO Line Attribute II |
|  | 143 | ISDN CO Line Attribute |
|  | 144 | CO Ring Assignment |
|  | 145 | CO Ring Assignment Display |
| SYSTEM BASE PROGRAM | 160 | System Attribute – I |
|  | 161 | System Attribute – II |
|  | 162 | Admin Password |
|  | 163 | Alarm Attributes |
|  | 164 | Attendant Assignment |
|  | 165 | Auto Attendant VMIB Annc. Assignment |
|  | 166 | CO-to-CO COS |
|  | 167 | DID/DISA Destination |
|  | 168 | External Control Contact |
|  | 169 | LCD Date/Time/Language Display Mode |
|  | 170 | Modem |
|  | 171 | Music |
|  | 172 | PBX Access Code |
|  | 173 | PLA Priority Setting |
|  | 174 | RS-232C Port Setting |
|  | 175 | Print Port Selection |
|  | 176 | Pulse Dial Ratio |
|  | 177 | SMDR Attributes |
|  | 178 | System Date/Time Setting |
|  | 179 | Linked Station Pairs Table |

| MAIN MENU | PGM | ITEM |
| --- | --- | --- |
| SYSTEM BASE PROGRAM | 180 | System Timers – I |
|  | 181 | System Timers – II |
|  | 182 | System Timers – III |
|  | 185 | CIDU Setting |
| DCOB | 186 | DCOB System attribute |
|  | 187 | DCOB CO Line Attribute |
| STATION GROUP | 190 | Station Group Assign |
|  | 191 | Station Group Attribute |
| ISDN SYSTEM BASE PROGRAM | 200 | ISDN Attributes |
|  | 201 | COLP Table |
|  | 202 | MSN Table |
| TABLES | 220 | LCR Attributes |
|  | 221 | LCR – Leading Digit Table |
|  | 222 | LCR – Digit Modification Table |
|  | 223 | LCR Table Initialization |
|  | 224 | Toll Exception Table – Allow A (Entry no:01-30) |
|  |  | Toll Exception Table – Deny A (Entry no:01-30) |
|  |  | Toll Exception Table – Allow B (Entry no:01-30) |
|  |  | Toll Exception Table – Deny B (Entry no:01-30) |
|  | 225 | Canned Toll Table –Allow (Entry no:01-10) |
|  |  | Canned Toll Table –Deny (Entry no:01-10) |
|  | 226 | Emergency Code Table |
|  | 227 | Authorization Code Table |
|  | 228 | Customer Call Routing |
|  | 229 | Executive/Secretary Table |
|  | 230 | DID Digit Conversion Table |
|  | 231 | Flexible DID Table |
|  | 232 | System Speed Zone |
|  | 233 | Weekly Time Table |
|  | 234 | Voice Mail Dialing Table |
|  | 235 | Tie Routing Table |
| NETWORKING | 320 | Networking Basic Attribute |
|  | 321 | Networking Supplementary Attribute |
|  | 322 | Networking CO Line Attribute |
|  | 323 | Networking Attendant Assignment |
|  | 324 | Networking Routing Table |
| VOIB | 340 | VOIB IP Setting |
| NATION SPECIFIC | 400 | DTIB Rx Gain Control |
|  | 401 | SLIB Rx Gain Control |
|  | 402 | SLIB12 Rx Gain Control |
|  | 403 | WTIB Rx Gain Control |
|  | 404 | ACOB Rx Gain Control |
|  | 405 | ACOB8 Rx Gain Control |
|  | 406 | DCOB Rx Gain Control |
|  | 407 | VMIB Rx Gain Control |
|  | 408 | DTMF Receiver Rx Gain Control |
|  | 409 | EXT Page Rx Gain Control |
|  | 410 | CPTU Rx Gain Control |

| MAIN MENU | PGM | ITEM |
| --- | --- | --- |
| NATION SPECIFIC | 411 | Modem Rx Gain Control |
|  | 412 | Short SLIB Gain Control |
|  | 413 | Long SLIB Gain Control |
|  | 414 | Far SLIB Gain Control |
|  | 415 | Short ACO Gain Control |
|  | 416 | Long ACO Gain Control |
|  | 420 | System Tone Frequency |
|  | 421 | Differential Ring Frequency |
|  | 422 | Distinct CO Ring Frequency |
|  | 423 | ACNR Tone Cadence |
|  | 425 | Singular Table (Korea Only) |
| INITIALIZATION (DB INIT) | 450 | Initialization |
| PRINT DATABASE | 451 | Print Prot Database |

## 1.6	DEFAULT VALUES

#### TABLE 1.6.1  LOCATION PROGRAM

| PGM | BTN | ITEM | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 100 | 1 | Nation Code | 82 | Max 4 digits |
|  | 2 | Customer Site Name | . | Max 23 digits |

#### TABLE 1.6.2  RACK SLOT ASSIGNMENT

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 101 | - | Slot Assignment | Refer to Note 2 | Refer to Note 1 | In case of PRIB assignment, it is possible to program logical port number. |

Note.1) If the DIP switch of the manual board detection (DIP Switch 8) is ON, system will detect the installed board type automatically. If the DIP switch 8 is OFF, the board type code must be entered at each slot. After manual Rack Slot assignment, user should reset the system manually.

Note.2) Board Type Code Table:

| STA | CODE | COL | CODE | STA & COL | CODE | Etc | CODE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DTIB12 DTIB24 SLIB6 SLIB12 WTIB DSIB | 11 12 13 14 15 18 | PRIB BRIB LCOB4 LCOB8 DIDB BWDIDB TLIB EMIB AC15 DCOB VOIB NPRIB NBRIB(8) NBRIB(4) CLCOB8 RDIB EMIB8 | 31 32 33 34 35 36 37 38 39 40 41 42(LDK-100) 43(LDK-100) 44(LDK-100) 45 46 47 | STIB | 51 | VMIB MISB | 61 71 |

#### TABLE 1.6.3  WTIB PORT NUMBER ASSIGNMENT

| PGM | BTN | ITEM | RANGE |  | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LDK-300 | LDK-100 |  |  |
| 102 | - | WTIB Port Number Assignment | 008 – 192 (Multiple of 8) | 08-80 (Multiple of 8) | 8 | When LDK-100 has one rack : 08-40 |

#### TABLE 1.6.4  LOGICAL SLOT ASSIGNMENT

| PGM | BTN | ITEM | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 103 | 1 | COL Board | Refer to Note |  |
|  | 2 | STA Board | Refer to Note |  |
|  | 3 | VMIB | Not Assigned |  |

Note) If the DIP switch of the manual board detection DIP Switch 8) is ON, system will detect the logical slot assign in sequence as increase order automatically. If the DIP switch 8 is OFF, the logical slot assignment must be entered at each board type. After manual logical slot assignment, user should reset the system manually.

#### TABLE 1.6.5  NUMBERING PLAN TYPE

| PGM | BTN | ITEM | STA RANGE |  | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LDK-300 | LDK-100 |  |  |
| 104 | 1 | Number Set Type 1 | 100 – 399 | 100 – 227 | Yes | As the basic type, the 1st digit of station number should be 1 – 4. |
|  | 2 | Number Set Type 2 | 100 – 399 | 100 – 227 | No | The station number can be changed within 799. |
|  | 3 | Number Set Type 3 | 100 – 399 | 100 – 227 | No |  |
|  | 4 | Number Set Type 4 | 700 – 999 | 700 – 827 | No |  |
|  | 5 | Number Set Type 5 | 200 – 499 | 200 – 327 | No |  |
|  | 6 | Number Set Type 6 | 21 – 79 | 21 – 79 | No | Max Station Ports: 60 Stations above Max ports will be displayed “***” |
|  | 7 | Number Set Type 7 | 100 – 299 | 100 – 227 | No | Max Station Ports: 200 Stations above Max ports will be displayed “***” |
|  | 8 | Number Set Type 8 | 100 – 399 | 100 – 227 | No | The station number can be changed within 999. |

#### TABLE 1.6.6  FLEXIBLE NUMBERING PLAN

| PGM | BTN | FIELD | NUMBER SET1 | NUMBER SET2 | NUMBER SET3 | NUMBER SET4 | NUMBER SET5 | NUMBER SET6 | NUMBER SET7 | NUMBER SET8 | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 105 | - | Intercom Call | 100 - 399 | 100 - 399 (100 - 799) | 100 - 399 | 700 - 999 | 200 - 499 | 10 - 79 | 100 - 299 | 100 - 399 (100 - 999) | LDK-300 |
|  |  |  | 100 -227 | 100 - 227 (100 - 799) | 100 - 227 | 700 - 827 | 200 - 327 | 10 - 79 | 100 - 227 | 100 - 227 (100 - 999) | LDK-100 |
| 106 | 1 | Group Pilot Number | 620 - 667 | *620 - *667 | 620 - 667 | 620 - 667 | 620 - 667 | *620 - *667 | 620 - 667 | *620 - *667 | LDK-300 |
|  |  |  | 620 - 634 | *620 - *634 | 620 - 634 | 620 - 634 | 620 - 634 | *620 - *634 | 620 - 634 | *620 - *634 | LDK-100 |
|  | 2 | Internal Page Zone | 501 - 535 | *501 - *535 | #01 - #35 | #01 - #35 | #01 - #35 | *501 - *535 | 401 - 419 | *501 - *535 | LDK-300 |
|  |  |  | 501 - 515 | *501 - *515 | #01 - #15 | #01 - #15 | #01 - #15 | *501 - *515 | 401 - 415 | *501 - *515 | LDK-100 |
|  | 3 | Internal All Call Page | 543 | *543 | #5 | #7 | #5 | *543 | 43 | *543 |  |
|  | 4 | Meet Me Page | 544 | *544 | ## | ## | ## | *544 | 44 | *544 |  |
|  | 5 | External Page Zone 1 | 545 | *545 | #6 | #41 | #6 | *545 | 45 | *545 |  |
|  | 6 | External Page Zone 2 | 546 | *546 | #7 | #42 | #7 | *546 | 46 | *546 |  |
|  | 7 | External Page Zone 3 | 547 | *547 | #8 | #43 | #8 | *547 | 47 | *547 |  |
|  | 8 | External All Call Page | 548 | *548 | #9 | #5 | #9 | *548 | 48 | *548 |  |
|  | 9 | All Call Page (Int & Ext) | 549 | *549 | #00 | #6 | #00 | *549 | 49 | *549 |  |
|  | 10 | SMDR Account Code Enter | 550 | *550 | 550 | 550 | 50 | *550 | 50 | *550 | SLT |
|  | 11 | Flash Command to CO Line | 551 | *551 | 551 | 551 | 51 | *551 | 51 | *551 | SLT |
|  | 12 | Last Number Redial | 552 | *552 | 552 | 552 | 52 | *552 | 52 | *552 | SLT |
|  | 13 | DND (Toggle On/Off) | 553 | *553 | 553 | 553 | 53 | *553 | 53 | *553 | SLT |
|  | 14 | Call Forward | 554 | *554 | 554 | 554 | 54 | *554 | 54 | *554 | SLT |
|  | 15 | Speed Dial Programming | 555 | *555 | 555 | *40 | 55 | *555 | 55 | *555 | SLT |
|  | 16 | Message Wait/Callback Enable | 556 | *556 | 556 | *66 | 56 | *556 | 56 | *556 |  |
|  | 17 | Message Wait/Callback Return | 557 | *557 | 557 | *67 | 57 | *557 | 57 | *557 | SLT |
|  | 18 | Speed Dial Access | 558 | *558 | 558 | *7 | 58 | *558 | 58 | *558 | SLT |
|  | 19 | Cancel DND/FWD/Pre-MSG | 559 | *559 | 559 | 559 | 59 | *559 | 59 | *559 | SLT |
|  | 20 | System Hold | 560 | *560 | 560 | 560 | 690 | *560 | 30 | *560 | SLT |
|  | 21 | Station Relocation Backup | 561 | *561 | 561 | 561 | 691 | *561 | … | *561 |  |
|  | 22 | Station Relocation Retrieve | 562 | *562 | 562 | 562 | 692 | *562 | … | *562 |  |
|  | 23 | Programming Mode Enter Code | 563 | *563 | 563 | 563 | 693 | *3 | 33 | *3 | SLT |
|  | 24 | ACD Reroute | 564 | *564 | 564 | 564 | 694 | *4 | 34 | *4 |  |
| PGM | BTN | FIELD | NUMBER SET1 | NUMBER SET2 | NUMBER SET3 | NUMBER SET4 | NUMBER SET5 | NUMBER SET6 | NUMBER SET7 | NUMBER SET8 | REMARK |
| 107 | 1 | Alarm Reset | 565 | *565 | 565 | *565 | 695 | *565 | 35 | *565 |  |
|  | 2 | Group Call Pickup | 566 | *566 | ** | *1 | ** | *566 | 36 | *566 |  |
|  | 3 | UCD DND | 568 | *568 | 568 | 568 | 698 | *568 | 68 | *568 |  |
|  | 4 | Night Answer | 569 | *569 | 577 | 2 | 699 | *569 | 69 | *569 |  |
|  | 5 | Call Parking Locations | 601 - 619 | *601 -* 619 | 601 – 619 | 601-619 | 601 - 619 | *601 -* 619 | 601 - 619 | *601 -* 619 | LDK-300 |
|  |  |  | 601 - 610 | *601 -* 610 | 601 – 610 | 601-610 | 601 - 610 | *601 -* 610 | 601 - 610 | *601 -* 610 | LDK-100 |
|  | 6 | Direct Call Pickup | 7 | *7 | *7 | *42 | 7 | *7 | 7 | *7 |  |
|  | 7 | CO Group Access | 801-872 | 801-872 | 801-872 | 401-472 | 801-872 | 801-872 | 801-872 | #801-#872 | LDK-300 |
|  |  |  | 801-824 | 801-824 | 801-824 | 401-424 | 801-824 | 801-824 | 801-824 | #801-#824 | LDK-100 |
|  | 8 | Individual CO Access | 88001-88200 | 88001-88200 | 88001-88200 | 48001-48200 | 88001-88200 | 88001-88200 | 88001-88200 | #88001-#88200 | LDK-300 |
|  |  |  | 8801-8840 | 8801-8840 | 8801-8840 | 4801-4840 | 8801-8840 | 8801-8840 | 8801-8840 | #8801-#8840 | LDK-100 |
|  | 9 | Tie Routing Access | 8901 | 8901 | 8901 | 4901 | 89 | 8901 | 8901 | #401 |  |
|  | 10 | Retrieve Held CO Line | 8* | 8* | 8* | 4* | 8* | 8* | 8* | #8* |  |
|  | 11 | Retrieve Held Individual CO Line | 8#xxx | 8#xxx | 8#xxx | 4#xxx | 8#xxx | 8#xxx | 8#xxx | #8#xxx | LDK-300 |
|  |  |  | 8#xx | 8#xx | 8#xx | 4#xx | 8#xx | 8#xx | 8#xx | #8#xx | LDK-100 |
|  | 12 | Access CO Line In the 1st available CO Group | 9 | 9 | 9 | 1 | 0 | 9 | 9 | 0 |  |
|  | 13 | Attendant Call | 0 | 0 | 0 | 0 | 9 | 0 | 0 | #9 |  |
|  | 14 | 1st Door Open | #*1 | #*1 | #*1 | #*1 | #*1 | #*1 | #*1 | #*1 |  |
|  | 15 | 2nd Door Open | #*2 | #*2 | #*2 | #*2 | #*2 | #*2 | #*2 | #*2 |  |
|  | 16 | 3rd Door Open | #*3 | #*3 | #*3 | #*3 | #*3 | #*3 | #*3 | #*3 |  |
|  | 17 | 4th Door Open | #*4 | #*4 | #*4 | #*4 | #*4 | #*4 | #*4 | #*4 |  |
|  | 18 | 5th Door Open | #*5 | #*5 | #*5 | #*5 | #*5 | #*5 | #*5 | #*5 |  |
|  | 19 | 6th Door Open | #*6 | #*6 | #*6 | #*6 | #*6 | #*6 | #*6 | #*6 |  |
|  | 20 | 7th Door Open | #*7 | #*7 | #*7 | #*7 | #*7 | #*7 | #*7 | #*7 | LDK-300 |
|  |  |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | LDK-100 |
|  | 21 | VM Message Waiting Enable | *8 | *8 | *8 | *8 | *8 | *8 | *8 | *8 |  |
|  | 22 | VM Message Waiting Disable | *9 | *9 | *9 | *9 | *9 | *9 | *9 | *9 |  |

#### TABLE 1.6.7  IP SETTING

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 108 | 1 | IP Name | Max 16 |  |  |
|  | 2 | Server IP Address | 12 Digits |  | Skip : # |
|  | 3 | CLI IP Address | 12 Digits |  |  |
|  | 4 | Gateway Address | 12 Digits |  |  |
|  | 5 | Subnet Mask | 12 Digits | 255.255.255.0 |  |

#### TABLE 1.6.8  STATION ID ASSIGNMENT

| PGM | BTN | ITEM | RANGE |  | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LDK-300 | LDK-100 |  |  |
| 110 | 1 | ID | 01-16 | 01-11 |  |  |
|  | 2 | DSS/DLS MAP – Associate STA | STA # | STA # |  |  |

#### TABLE 1.6.9  STATION ATTRIBUTE I/II/III

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 111 | 1 | Auto Speaker Selection | ON / OFF | ON |  |
|  | 2 | Call Forward | ON / OFF | ON |  |
|  | 3 | DND | ON / OFF | ON |  |
|  | 4 | Data Line Security | ON / OFF | OFF |  |
|  | 5 | Howling Tone to SLT | ON / OFF | ON |  |
|  | 6 | ICM Box Signaling | ON / OFF | OFF |  |
|  | 7 | No Touch Answer | ON / OFF | ON |  |
|  | 8 | Page Access | ON / OFF | OFF |  |
|  | 9 | Ring Type | 1 - 4 | 1 |  |
|  | 10 | Speaker/Headset Ring | SP/HEAD/ BOTH | Speaker-Phone |  |
|  | 11 | Speaker Phone/Headset | ON/OFF | ON |  |
|  | 12 | VMIB Slot | 0 - 2 (LDK-300) 0 - 1 (LDK-100) | 0 |  |
|  | 13 | ICM Group | 01 - 15 (LDK-300) 01 - 05 (LDK-100) | 01 |  |
|  | 14 | Error Tone for TAD | ON / OFF | OFF |  |
|  | 15 | SLT Flash Drop | ON / OFF | OFF |  |
| 112 | 1 | CO Warning Tone | ON / OFF | OFF |  |
|  | 2 | Automatic Hold | ON / OFF | OFF | STA2:ON |
|  | 3 | CO Call Time Restriction | ON / OFF | OFF |  |
|  | 4 | Ind CO Line Access | EN/DIS | ENABLE |  |
|  | 5 | CO Line Queuing | EN/DIS | ENABLE |  |
|  | 6 | CO PGM | EN/DIS | DISABLE |  |
|  | 7 | PLA | EN/DIS | ENABLE |  |
|  | 8 | Prepaid Call | ON / OFF | OFF |  |
|  | 9 | Speed Dial Access | EN/DIS | ENABLE |  |
|  | 10 | Two Way Record | ON / OFF | OFF |  |
|  | 11 | Fax Mode | ON / OFF | OFF |  |
|  | 12 | OFFNET Call Mode | EXT/ALL | ALL |  |
|  | 13 | UCD Grp Service | ON / OFF | OFF |  |
|  | 14 | Ring Grp Service | ON / OFF | OFF |  |

| PGM | BTN | ITEM |  |  |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 112 | 15 | Stop Camp On Tone |  |  |  | ON / OFF | OFF |  |
|  | 16 | Line Length |  |  |  | SHORT / LONG / FAR | SHORT | SAF only (Short:0km, Long:0~3km,Far:3~7.5km) |
| 113 | 1 | Admin |  |  |  | EN/DIS | DISABLE | STA100 |
|  | 2 | VMIB Access |  |  |  | EN/DIS | DISABLE | Enable |
|  | 3 | Group Listening |  |  |  | EN/DIS | DISABLE |  |
|  | 4 | Override Privilege |  |  |  | EN/DIS | DISABLE |  |
|  | 5 | SMDR Hidden Dialed Digits |  |  |  | EN/DIS | DISABLE |  |
|  | 6 | Voice Over |  |  |  | EN/DIS | DISABLE |  |
|  | 7 | Warm Line | Hot Line / Warm Line |  |  | Warm Line |  |  |
|  | 8 | DVU MSG Retrieve Password |  |  |  | ON/OFF | OFF |  |
|  | 9 | DVU MSG Retrieve Date/Time |  |  |  | ON/OFF | ON |  |
|  | 10 | Alarm Attribute |  | LDK-300 | MISB RAU1 RAU2 | (ON/OFF) (ON/OFF) (ON/OFF) | OFF OFF OFF |  |
|  |  |  |  | LDK-100 | MPB MISB | (ON/OFF) (ON/OFF) | OFF OFF |  |

#### TABLE 1.6.10  ISDN STATION ATTRIBUTE

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 114 | 1 | CLIP LCD Display | ON / OFF | ON |  |
|  | 2 | COLP LCD Display | ON / OFF | OFF |  |
|  | 3 | CLI / REDIRECT Display | CLI/ REDIRECT | CLI |  |
|  | 4 | CLI MSG Wait | ON / OFF | OFF |  |
|  | 5 | EXT or CO ATD | ATD/EXT | EXT |  |
|  | 6 | Keypad Facility | KEYPAG/DTMF | DTMF |  |
|  | 7 | Long/Short | LONG/SHORT | SHORT |  |
|  | 8 | CPN Type | 0-2 | 0(Not used) |  |
|  | 9 | S0 Sub-address | 0-2 | 0(Not used) |  |
|  | 10 | Reserved | - | - |  |
|  | 11 | CLI Name Display | ON/OFF | OFF |  |
|  | 12 | ISDN CLI Station Number | Max 4 digits | Logical STA Number |  |
|  | 13 | Progress Indication | ON / OFF | OFF |  |
|  | 14 | ISDN CLIR | ON / OFF | OFF |  |
|  | 15 | ISDN COLR | ON / OFF | OFF |  |
|  | 16 | DID Restriction | ON / OFF | OFF |  |
|  | 17 | DID Call Wait | ON/OFF | OFF |  |

#### TABLE 1.6.11   FLEXIBLE BUTTON ASSIGNMENT

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 115 | 01-44 | Flex. Buttons Assignment | BTN 01-44 |  |  |
|  |  | 1: User Button | - |  | Not programmed |
|  |  | 2: {CO} Button | 001-200 (LDK-300) 01-40 (LDK-100) |  |  |
|  |  | 3: {CO Group} Button | 01-72 (LDK-300) 01-24 (LDK-100) |  |  |
|  |  | 4: {LOOP} Button | - |  |  |
|  |  | 5: {STA xxx} Button | STA No. |  |  |
|  |  | 6: STA PGM Button | 11 - 99 |  |  |
|  |  | 7: {STA SPD xxx} Button | STA SPD Bin No. |  |  |
|  |  | 8: {SYS SPD xxxx} Button | SYS SPD Bin No. |  |  |
|  |  | 9: Num Pln Button | Num Plan Code |  |  |

#### TABLE 1.6.12  STATION BASE PROGRAM

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 116 | 1 | Station COS : Day | 1 - 7 | 1 |  |
|  | 2 | Station COS : Night | 1 - 7 | 1 |  |
| 117 |  | CO Group Access |  |  |  |
|  | 1 | CO Line Group 01~24 |  | 01-24 |  |
|  | 2 | CO Line Group 25~48 |  | 25-48 | LDK-300 Only |
|  | 3 | CO Line Group 49~72 |  | 49-72 | LDK-300 Only |
| 118 |  | Internal Page Zone Access |  | GRP 01 |  |
|  | 1 | Internal Page Zone 01~24 | LDK-100 : 01-10 |  |  |
|  | 2 | Internal Page Zone 25~30 |  |  | LDK-300 Only |
| 119 | 1-5 | Conference Page Zone Access | 31 - 35 (LDK-300) 11 - 15 (LDK-100) |  |  |
| 120 |  | ICM Tenancy Group number |  |  |  |
|  | 1 | ICM Tenancy Group Attendant | STA No. | - |  |
|  | 2 | ICM Tenancy Access Group | 01 - 15 (LDK-300) 01 - 05 (LDK-100) | - |  |
| 121 |  | Preset Call Forward |  | - |  |
| 122 |  | Hot Line / Warm Line |  | - |  |
|  |  | 1: Flex Button | 01 – 44 | - |  |
|  |  | 2: CO Line | 001-200 (LDK-300) 01-40 (LDK-100) | - |  |
|  |  | 3: CO Group | 01-72 (LDK-300) 01-24 (LDK-100) | - |  |
|  |  | 4: Station | STA No. | - |  |
| 123 | 1 | CTI Mode | 0 – 2 | 1 | 0: Inactive 1: CTI mode 2: AT mode |
|  | 2 | CTI Baud Rate | 0 – 2 | 0 | 0: 1200 1: 2400 2: 4800 |
| 124 |  | SMDR Account Group Assign | 00 – 99(LDK-300) 00 – 23(LDK-100) | 00(Not Assigned) |  |

#### TABLE 1.6.13  CO LINE BASE PROGRAM

| PGM | BTN | ITEM |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| 140 |  | CO Service Type |  |  |  |  |
|  | 1 | CO Type |  | 1-5 | 1(Normal) | 1:Normal, 2:A_DID, 3:ISDN DID/MSN, 4: TIE, 5: DCO DID |
|  | 2 | Detailed Attribute of the type |  |  |  |  |
|  |  | DISA(D/N/W) |  |  |  | Day / Night / Weekend |
|  |  |  | DISA SVC | ON /OFF | OFF |  |
|  |  |  | VMIB ANNC | 00-70 | 00(NOT_ASG) |  |
|  |  | Analog DID |  |  |  |  |
|  |  |  | SIGNAL | 1-3 | 1 | 1: Immediate 2: Wink 3: Delayed Dial |
|  |  |  | INFO NO | 00-70 | 00(NOT_ASG) |  |
|  |  | TIE Attribute |  |  |  |  |
|  |  |  | TIE SIG | 1-5 | 1 | 1:RD 2:LD 3:EM-C 4:EM-D 5:EM-I |
| 141 |  | CO Line Attributes –I |  | 1-9 |  |  |
|  | 1 | CO Line Group Assignment |  | 00-73 (LDK-300) 00-25 (LDK-100) | 01 |  |
|  | 2 | CO Line COS |  | 1-5 | 1 |  |
|  | 3 | DISA Account Code |  | ON / OFF | OFF |  |
|  | 4 | CO Line Assign |  | POL/LOOP | LOOP | Polarity RV, Loop Start |
|  | 5 | CO Line Type |  | PBX/CO | CO |  |
|  | 6 | CO Line Signal Type |  | DTMF/PULSE | DTMF |  |
|  | 7 | Flash Type |  | GROUND/LOOP | LOOP |  |
|  | 8 | UNA |  | ON / OFF | OFF |  |
|  | 9 | CO Line Group Account |  | ON / OFF | OFF |  |
|  | 10 | Tenancy Group |  | 00-15 (LDK-300) 00-05 (LDK-100) | 01 |  |
| 142 |  | CO Line Attributes – II |  | 1-13 |  |  |
|  | 1 | CO Line Name Display |  | ON / OFF | OFF |  |
|  | 2 | CO Line Name Assign |  |  | - | Max 12 characters |
|  | 3 | Metering Unit |  | 00-06 | 0 |  |
|  | 4 | Line Drop using CPT |  | ON / OFF | OFF |  |
|  | 5 | CO Distinct Ring |  | 0-4 | 0 |  |
|  | 6 | CO Line MOH |  | 0-13 (LDK-300) 0-12 (LDK-100) | 1 |  |
|  | 7 | PABX CO Dial Tone |  | YES / NO | YES |  |
|  | 8 | PABX CO Ring Back Tone |  | YES / NO | NO |  |
|  | 9 | PABX CO Error Tone |  | YES / NO | NO |  |
|  | 10 | PABX CO Busy Tone |  | YES / NO | NO |  |
|  | 11 | PABX CO Announce Tone |  | YES / NO | NO |  |
|  | 12 | CO Flash Timer |  | 000 – 300 | 050 | 10 msec base |
|  | 13 | Open Loop Detect Timer |  | 0 – 20 | 0 | 100 msec base |
|  | 14 | Line Length |  | LONG / SHORT | SHORT | SAF only (Short:0~2km, Long:2~9Km) |

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 143 |  | ISDN CO Line Attribute | 1-6 |  |  |
|  | 1 | COLP Table Index | 00 ~ 50 | None | 00 ~ 49: PGM 201 Bin No. 50: PGM 114-BTN 5 |
|  | 2 | CLIP Table Index | 00 ~ 50 | None | 00 ~ 49: PGM 201 Bin No. 50: PGM 114-BTN 5 |
|  | 3 | Type of Calling Number | 0-4 | 2 | 0:Unknown number 1:International type 2:National number 3:Not used 4:Subscriber number |
|  | 4 | DID Conversion Type | 0 ~ 2 | 0 |  |
|  | 5 | DID Removal Number | 00-99 | 00 | 00:Do not ignore 01-99:indicate an ignored called party number (DID_RN : DID Remove number from called party information) |
|  | 6 | ISDN Enblock Sending | ON / OFF | OFF | ON:Enblock Sending Mode OFF:Overlap Sending Mode |
|  | 7 | CLI Transit | ORI(1)/CFW(0) | CFW(0) | ORI : Send CLI as the originate caller’s CLI. CFW : Send CLI as the call forwarded station’s CLI. |
| 144 |  | CO Ring Assignment |  |  | STA Range (Delay : 0 – 9), Hunt Group, VMIB Message |
|  | 1 | Day | STA_R/ HUNT/VMIB |  |  |
|  | 2 | Night | STA_R/ HUNT/VMIB |  |  |
|  | 3 | Weekend | STA_R/ HUNT/VMIB |  |  |
|  | 4 | On-demand | STA_R/ HUNT/VMIB |  |  |
| 145 |  | CO Ring Assignment Display |  |  |  |
|  | 1 | Day | STA_R/ HUNT/VMIB |  |  |
|  | 2 | Night | STA_R/ HUNT/VMIB |  |  |
|  | 3 | Weekend | STA_R/ HUNT/VMIB |  |  |
|  | 4 | On-demand | STA_R/ HUNT/VMIB |  |  |

#### TABLE 1.6.14  SYSTEM BASE PROGRAM

| PGM | BTN | ITEM |  | RANGE | DEFAULT |  |  | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 160 |  | System Attributes-I |  | 1-13 |  |  |  |  |
|  | 1 | Attendant Call Queuing RB |  | RBT/MOH | MOH |  |  | RBT:Ring Back Tone MOH:PGM171 - 2 |
|  | 2 | CAMP RBT/MOH |  | RBT/MOH | MOH |  |  |  |
|  | 3 | CO Line Choice |  | LAST/ROUND | LAST |  |  | Round-Robin, Last Choice |
|  | 4 | DISA Retry Counter |  | 1-9 | 3 |  |  |  |
|  | 5 | ICM Continuous Dial-Tone |  | CONT / DISCONT | CONT |  |  | Continuous, Discontinuous |
|  | 6 | CO Dial-Tone Detect |  | ON / OFF | OFF |  |  |  |
|  | 7 | External Night Ring |  | ON / OFF | OFF |  |  |  |
|  | 8 | Hold Preference |  | SYS/EXEC | SYS |  |  | System, Exclusive |
|  | 9 | Multi-line Conference |  | ON / OFF | ON |  |  |  |
|  | 10 | Print LCR Conversion Digit |  | ON / OFF | OFF |  |  |  |
|  | 11 | Conference Warning Tone |  | ON / OFF | ON |  |  |  |
|  | 12 | Offnet Prompt Usage |  | ON / OFF | ON |  |  |  |
|  | 13 | Offnet DTMF Tone |  | ON / OFF | ON |  |  |  |
| 161 |  | System Attributes-II |  | 1-13 |  |  |  |  |
|  | 1 | Network Time/Date Setting |  | ON / OFF | OFF |  |  |  |
|  | 2 | Off-Hook Ring Signal Type |  | MUTE/BURST | MUTE |  |  | Mute Ring,One Burst |
|  | 3 | Override 1st CO Group |  | ON / OFF | ON |  |  |  |
|  | 4 | Page Warning Tone |  | ON / OFF | ON |  |  |  |
|  | 5 | Privacy |  | ON / OFF | ON |  |  |  |
|  | 6 | Privacy Warning Tone |  | ON / OFF | ON |  |  |  |
|  | 7 | Single Ring for CO Call |  | YES/NO | NO |  |  |  |
|  | 8 | WTU Auto Release |  | ON / OFF | OFF |  |  |  |
|  | 9 | ACD PRNT Enable |  | ON / OFF | OFF |  |  | ON:10s |
|  | 10 | ACD PRNT Timer |  | 001 – 255 | 001 |  |  | 10 sec or 1 hour base |
|  | 11 | ACD Clr After Print |  | ON / OFF | OFF |  |  |  |
|  | 12 | VMIB PROMPT GAIN |  | 00 - 31 | 08 |  |  |  |
|  | 13 | New VM Type Set |  | ON / OFF | OFF |  |  |  |
|  | 14 | ACD Print Timer Unit |  | HOUR/SEC | SEC |  |  | SEC: 10 seconds |
| 162 | - | Admin Password |  | 4 Digits | - |  |  |  |
| 163 | 1 | Alarm Enable |  | ON / OFF | OFF |  |  |  |
|  | 2 | Alarm Contact Type |  | CLOSE/OPEN | CLOSE |  |  | Close, Open |
|  | 3 | Alarm Mode |  | ALARM / BELL | ALARM |  |  | Alarm, Door Bell |
|  | 4 | Alarm Signal Mode |  | RPT/ONCE | RPT |  |  | Repeat , Once |
| 164 | 1-5 | Attendant Assignment |  | STA No. | 1 : 101 |  |  | Max 5 Atds |
| 165 | - | Auto Attendant |  |  |  |  |  |  |
|  | 1 | Auto Attendant Usage |  | ON / OFF | OFF |  |  |  |
|  | 2 | Auto Attendant VMIB Annc.# |  | 00-70 | 00(not_asgn) |  |  |  |
| 166 |  | CO-to-CO COS |  | 1-7 | 1 |  |  |  |
|  | 1 | Day COS |  | 1-7 | 1 |  |  |  |
|  | 2 | Night/Weekend COS |  | 1-7 | 1 |  |  |  |
| PGM | BTN | ITEM |  | RANGE | DEFAULT |  |  | REMARK |
| 167 |  | DID/DISA Destination |  | F1-F4 |  |  |  |  |
|  | 1 | Busy Destination |  | F1-F3 | F1 |  |  | 1: Tone 2: ATD (Ring Assign) 3: FWD to Hunt Grp. |
|  | 2 | Error Destination |  | F1-F3 | F1 |  |  |  |
|  | 3 | No Answer Destination |  | F1-F3 | F1 |  |  |  |
|  | 4 | VMIB PROMPT USAGE |  | F1-F5 |  |  |  |  |
|  |  | 1 | Busy Prompt Usage | ON / OFF | ON |  |  |  |
|  |  | 2 | Error Prompt Usage | ON / OFF | ON |  |  |  |
|  |  | 3 | DND Prompt Usage | ON / OFF | ON |  |  |  |
|  |  | 4 | No Ans Prompt Usage | ON / OFF | ON |  |  |  |
|  |  | 5 | Atd Xfer Prompt Usage | ON / OFF | ON |  |  |  |
| 168 | 1 | First Contact |  | 1-5 | - |  |  | 1: LBC(STA #) 2: Door 3: Ext. 1 4: Ext. 2 5: Ext. 3 |
|  | 2 | Second Contact |  | 1-5 | - |  |  |  |
|  | 3 | Third Contact |  | 1-5 | - |  |  |  |
|  | 4 | Forth Contact |  | 1-5 | - |  |  |  |
|  | 5 | Fifth Contact |  | 1-5 | - |  |  |  |
|  | 6 | Sixth Contact |  | 1-5 | - |  |  |  |
|  | 7 | Seventh Contact |  | 1-5 | - |  |  | LDK-300 Only |
| 169 | 1 | Time Display Mode |  | 12H/ 24H | 12H |  |  |  |
|  | 2 | Date Display Mode |  | MMDD / DDMM | DDMMYY |  |  |  |
|  | 3 | Language Display Mode |  | 00-14 | 12(Korean) |  |  | 0:ENGLISH, 1:ITALIAN, 2:FINNISH, 3:DUTCH, 4:SWEDISH, 5:DANISH, 6:NORWEGIAN, 7:HEBREW, 8:GERMANY, 9:FRENCH, 10:PORTUGUESE, 11:SPANISH, 12:KOREAN, 13:ESTONIA, 14:RUSSIAN, |
| 170 |  | Modem Associated Device |  |  | STA 399 (LDK-300) STA 227 (LDK-100) |  |  | Last Station |
|  | 1 | Associated Station |  | Station Number |  |  |  |  |
|  | 2 | Associated CO |  | CO Number |  |  |  |  |
| PGM | BTN | ITEM |  | RANGE |  | DEFAULT | REMARK |  |
| 171 | 1 | BGM Type |  | 0-12 (LDK-300) 0-11 (LDK-100) |  | 1 | 0: None 1: Int. Music 2-4: Ext. Music 1-3 5-6(7): VMIB 1-2(3) 7(8)-11(12): SLT MOH |  |
|  | 2 | MOH Type |  | 0-13 (LDK-300) 0-12 (LDK-100) |  | 1 | 0: NOT_ASG 1: Int. Music 2-4: Ext. Music 1-3 5-6(7): VMIB 1-2(3) 7(8)-11(12): SLT MOH 12(13): Hold Tone |  |
|  | 3 | ICM Box Music Channel |  | 0-12 (LDK-300) 0-11 (LDK-100) |  | 1 | The same as above 1 |  |
|  | 4 | Assign MOH via SLT |  | Flex. 1-5 (+SLT STA No.) |  | - | SLT MOH 1-5 |  |
| 172 | 1 - 4 | PBX Access Code |  | Max. 2 digits |  | - | Maximum 4 PBX access code |  |
| 173 |  | PLA Priority Setting |  |  |  |  | PLA priority is set Exclusively |  |
|  | 1 | Transfer CO |  | 1 – 4 |  | 1 |  |  |
|  | 2 | Recalling CO |  | 1 – 4 |  | 2 |  |  |
|  | 3 | Incoming CO |  | 1 – 4 |  | 3 |  |  |
|  | 4 | Queued CO |  | 1 – 4 |  | 4 |  |  |
| 174 |  | RS-232 PORT Setting |  |  |  |  |  |  |
|  | 1 | Baud Rate Setting |  | 0-8 |  | 19200(6) | 0: UNKNOWN 1: UNKNOWN 2: 1200 BAUD 3: 2400 BAUD 4: 4800 BAUD 5: 9600 BAUD 6: 19200 BAUD 7: 38400 BAUD 8: 57600 BAUD |  |
|  | 2 | CTS |  | ON / OFF |  | OFF |  |  |
|  | 3 | Page Break |  | ON / OFF |  | OFF |  |  |
|  | 4 | Line Page |  | 001-199 |  | 060 |  |  |

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 175 | 1 | Off-line SMDR/Statistics Print | 01-13 (LDK-300) 01-11 (LDK-100) | COM2(02) : LDK-300 COM1(01) : LDK-100 | Value : 1-8 : LDK-300 Value : 1-6 : LDK-100 |
|  | 2 | Admin Print |  |  |  |
|  | 3 | Traffic Print |  |  |  |
|  | 4 | SMDI Print |  |  |  |
|  | 5 | CALL Info Print |  |  |  |
|  | 6 | On-line SMDR Print |  |  |  |
|  | 7 | Trace Print |  |  |  |
|  | 8 | Debug Print |  |  |  |
|  | 9 | PC_ADM |  | NET_PCADM | LDK-300 : 1-5, 9, 10 LDK-100 : 1-3, 7, 8 |
|  | 10 | PC_ATD |  | NET_PCATD | LDK-300 : 1, 2, 4, 5, 11 LDK-100 : 1, 2, 4, 9 |
|  | 11 | CTI |  | NET_CTI | LDK-300 : 1, 2, 4, 5, 12 LDK-100 : 1, 2, 4, 10 |
|  | 12 | REMOTE_DIAG |  | NET_REMOTE | LDK-300 : 1-5, 13 LDK-100 : 1-3, 11 |
| 176 | - | Pulse Dial/Speed Ratio | 66/33 / 60/40 | 66/33 | 66/33 60/40(10 PPS only) |
| 177 |  | SMDR Attributes | 1-14 |  |  |
|  | 1 | SMDR Save Enable | ON / OFF | OFF |  |
|  | 2 | SMDR Print Enable | ON / OFF | OFF |  |
|  | 3 | SMDR Recording Call Type | LD / ALL | LD | LD : Long Distance ALL: All |
|  | 4 | SMDR Long Distance Call Digit Counter | 07-15 | 07 | Considered more than this value as Long Distance Call. |
|  | 5 | Print Incoming Call | ON / OFF | OFF |  |
|  | 6 | Print Lost Call | ON / OFF | OFF |  |
|  | 7 | Records in Detail | ON / OFF | ON |  |
|  | 8 | SMDR Dial Digit Hidden | 0-9 | 0 |  |
|  | 9 | SMDR Currency | 3 English Chars | - |  |
|  | 10 | SMDR Cost Per Unit Pulse | 6 digits | - |  |
|  | 11 | SMDR Fraction | 0-5 | 0 |  |
|  | 12 | SMDR Start Timer | 0 – 250 | 0 | 1 sec base |
|  | 13 | SMDR Hidden Digit | Right/Left | Right |  |
|  | 14 | SMDR Long Distance Codes | MAX 2 digits | 0 | Max. 5 LD codes |
| 178 | 1 | System Time Setting | 4 digits | - | Hour/Min sequence |
|  | 2 | System Date Setting | 6 digits | - | Month/Day/Year sequence. |
| 179 | 1 | View |  |  | Show the linked pairs |
|  | 2 | Linked Pair Input | 2 STA# |  | Max: 64 pairs |

#### TABLE 1.6.15  SYSTEM TIMER PROGRAM

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 180 | 1 | - Attendant Recall Timer | 00 - 60 | 01 | 1 min base |
|  | 2 | - Call Park Recall Timer | 000 - 600 | 120 | 1 sec base |
|  | 3 | Camp-on Recall Timer | 000 - 200 | 030 | 1 sec base |
|  | 4 | - Exclusive Hold Recall Timer | 000 - 300 | 060 | 1 sec base |
|  | 5 | - I-Hold Recall Timer | 000 - 300 | 030 | 1 sec base |
|  | 6 | - Sys Hold Recall Timer | 000 - 300 | 030 | 1 sec base |
|  | 7 | - Transfer Recall Timer | 000 - 300 | 030 | 1 sec base |
|  | 8 | - ACNR Delay Timer | 000 - 300 | 030 | 1 sec base |
|  | 9 | - ACNR No Answer Timer | 10 - 50 | 30 | 1 sec base |
|  | 10 | - ACNR Pause Timer | 005 - 300 | 030 | 1 sec base |
|  | 11 | - ACNR Retry Counter | 01 - 30 | 3 |  |
|  | 12 | - ACNR No Tone Retry Counter | 1 - 9 | 1 |  |
|  | 13 | - ACNR Tone Detect Timer | 001-300 | 030 | 1 sec base |
|  | 14 | Automatic CO Release Timer. | 020 - 300 | 030 | 1 sec base |
|  | 15 | - CCR Inter-Digit Timer | 000 - 255 | 030 | 100 msec base |
|  | 16 | - CO Call Drop Warning Timer | 00 - 99 | 10 | 1 sec base |
|  | 17 | - CO Call Restriction Timer | 00 - 99 | 0 | 1 min base |
|  | 18 | - CO Dial Delay Timer | 00 - 99 | 01 | 100 msec base |
|  | 19 | - CO Release Guard Timer | 001 - 150 | 020 | 100 msec base |
|  | 20 | CO Ring Off Timer | 010 - 150 | 060 | 100 msec base |
|  | 21 | - CO Ring On Timer | 1 - 9 | 2 | 100 msec base |
|  | 22 | - CO Warning Tone Timer | 060 - 900 | 180 | 1 sec base |
| 181 | 1 | Call FWD No Answer Timer | 000 - 255 | 015 | 1 sec base |
|  | 2 | DID/DISA No Answer Timer | 00 - 99 | 20 | 1 sec base |
|  | 3 | VMIB User Record Timer | 010 - 255 | 020 | 1 sec base |
|  | 4 | VMIB Valid User Message Timer | 0-9 | 4 | 1 sec base |
|  | 5 | Door Open Timer | 05 - 99 | 20 | 100msec base |
|  | 6 | ICM Box Timer | 00 - 60 | 30 | 1 sec base |
|  | 7 | ICM Dial Tone Timer | 01 - 20 | 10 | 1 sec base |
|  | 8 | Inter Digit Timer | 01 - 20 | 05 | 1 sec base |
|  | 9 | MSG Wait Reminder Tone Timer | 00 - 60 | 00 | 1 min base |
|  | 10 | Paging Timeout Timer | 000 - 255 | 015 | 1 sec base |
|  | 11 | - Pause Timer | 1 - 9 | 3 | 1 sec base |
|  | 12 | - Preset Call Forward Timer | 00 - 99 | 10 | 1 sec base |
| 182 | 1 | SLT Hook Switch Bounce Timer | 01 - 25 | 01 | 100msec base |
|  | 2 | SLT Maximum Hook Flash Timer | 01-25 | 05 | 100msec base |
|  | 3 | SLT Minimum Hook Flash Timer | 000 - 250 | 020 | 10msec base |
|  | 4 | SLT Ring Phase Timer | 2 - 5 | 5 | 1 sec base |
|  | 5 | Station Auto Release Timer | 020 - 300 | 060 | 1 sec base |
|  | 6 | - Unsupervised Conference Timer | 00 - 99 | 10 | 1 min base |
|  | 7 | Wake-Up Fail Ring Timer | 00 - 99 | 20 | 1 sec base |
|  | 8 | - Warm Line Timer | 01 - 20 | 05 | 1 sec base |
|  | 9 | Wink Timer | 010 - 200 | 010 | 10msec base |
|  | 10 | Enblock Digit timer | 01-20 | 15 | 1 sec base |
|  | 11 | CCR Time Out Timer | 000-300 | 015 | 1 sec base |
|  | 12 | DID Inter Digit Timer | 01-20 | 03 | 1 sec base |

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 185 |  | CIDU Setting |  |  |  |
|  | 1 | CID Usage | ON / OFF | OFF |  |
|  | 2 | CID Name Display | Name(1) / Telephone No.(0) | Telephone No.(0) |  |
|  | 3 | Serial Port Select | 1-4 | - |  |
|  | 4 | CID/CO Line Port Mapping | 000-063 | - |  |
|  | 5 | Initialize CID Data |  |  |  |

#### TABLE 1.6.16  DCOB ATTRIBUTE

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 186 |  | DCOB System Attributes | 1-12 |  |  |
|  | 1 | DCOB CO Type | 0-2 | 2 | 0:Sweden/Cyprus 1:Italy 2:Korea/Australia |
|  | 2 | Metering Type | 0-1 | 0 |  |
|  | 3 | R2 OUT Manage Timer | 01-50 | 14 | 1 sec |
|  | 4 | R2 IN Manage Timer | 01-50 | 14 | 1 sec |
|  | 5 | R2 Disappear Timer | 01-50 | 14 | 1 sec |
|  | 6 | R2 Pulse Timer | 01-30 | 7 | 20 msec |
|  | 7 | R2 Ready Timer | 000-500 | 7 | 20 msec |
|  | 8 | Dial Tone Delay Timer | 01-30 | 20 |  |
|  | 9 | Line Status | 1-9 | 6 | Free Line |
|  | 10 | Calling Category | 1-9 | 1 | User no priority |
|  | 11 | DNIS Service | ON/OFF | OFF |  |
|  | 12 | CLI Digit Num | 01-10 | 4 | Reserved |
| 187 |  | DCOB CO Line Attributes | CO Line range |  |  |
|  | 1 | IN Digit Type | 0-2 | 2 | 0:PULSE 1:DTMF |
|  | 2 | OUT Digit Type | 0-2 | 2 | 2:R2MFC |
|  | 3 | CLI Digit Num | 01-15 | 10 |  |

#### TABLE 1.6.17  STATION GROUP ASSIGNMENT

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 190 |  | Station Group Number | STA Grp # | - |  |
|  | 1 | Group Type | 0-6 | 0 | 0: Not Assign 1: Circular 2: Terminal 3: UCD 4: Ring 5: VM 6: Pick up |
|  | 2 | Pick-up Attribute | ON/OFF | OFF |  |
|  | 3 | Member Assignment | Not Assigned | - | Group type should be assigned |

#### TABLE 1.6.18  STATION GROUP PROGRAM

| PGM | ITEM | BTN | SUB ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| 191 | Circular Group | 1 | VMIB Announce 1 Timer | 000-999 | 015 | 1 sec base |
|  |  | 2 | VMIB Announce 2 Timer | 000-999 | 000 | 1 sec base |
|  |  | 3 | VMIB Announce Location 1 | 00-70 | 00(not_asgn) |  |
|  |  | 4 | VMIB Announce Location 2 | 00-70 | 00(not_asgn) |  |
|  |  | 5 | VMIB Announce 2 Repeat | 000-999 | 000 | 1 sec base |
|  |  | 6 | VMIB Announce 2 Repeat E/D | ON / OFF | OFF |  |
|  |  | 7 | Overflow Destination | STA #/HUNT#/ VMIB #/SYS SPD# | - |  |
|  |  | 8 | Overflow Timer | 000-600 | 180 | 1 sec base |
|  |  | 9 | Wrap-up Timer | 002-999 | 002 | 1 sec base |
|  |  | 10 | No Answer Timer | 00-99 | 15 | 1 sec base |
|  |  | 11 | Pilot Hunt | ON / OFF | ON |  |
|  |  | 12 | Alt If No Member | ON / OFF | OFF |  |
|  |  | 13 | Music Source | 00 - 12 (LDK-300) 00 - 11 (LDK-100) | 00 |  |
|  | Terminal Group | 1 | VMIB Announce 1 Timer | 000-999 | 015 | 1 sec base |
|  |  | 2 | VMIB Announce 2 Timer | 000-999 | 000 | 1 sec base |
|  |  | 3 | VMIB Announce Location 1 | 00-70 | 00(not_asgn) |  |
|  |  | 4 | VMIB Announce Location 2 | 00-70 | 00(not_asgn) |  |
|  |  | 5 | VMIB Announce 2 Repeat | 000-999 | 000 | 1 sec base |
|  |  | 6 | VMIB Announce 2 Repeat E/D | ON / OFF | OFF |  |
|  |  | 7 | Overflow Destination | STA #/HUNT#/ VMIB #/SYS SPD# | - |  |
|  |  | 8 | Overflow Timer | 000-600 | 180 | 1 sec base |
|  |  | 9 | Wrap-up Timer | 002-999 | 002 | 1 sec base |
|  |  | 10 | No Answer Timer | 00-99 | 15 | 1 sec base |
|  |  | 11 | Pilot Hunt | ON / OFF | ON |  |
|  |  | 12 | Alt If No Member | ON / OFF | OFF |  |
|  |  | 13 | Music Source | 00 - 12 (LDK-300) 00 - 11 (LDK-100) | 00 |  |

| PGM | ITEM | BTN | SUB ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| 191 | UCD Group | 1 | VMIB Announce 1 Timer | 000-999 | 015 | 1 sec base |
|  |  | 2 | VMIB Announce 2 Timer | 000-999 | 000 | 1 sec base |
|  |  | 3 | VMIB Announce Location 1 | 00-70 | 00(not_asgn) |  |
|  |  | 4 | VMIB Announce Location 2 | 00-70 | 00(not_asgn) |  |
|  |  | 5 | VMIB Announce 2 Repeat | 000-999 | 000 | 1 sec base |
|  |  | 6 | VMIB Announce 2 Repeat E/D | ON / OFF | OFF |  |
|  |  | 7 | Overflow Destination | STA #/HUNT#/ VMIB #/SYS SPD# | - |  |
|  |  | 8 | Overflow Timer | 000-600 | 180 | 1 sec base |
|  |  | 9 | Wrap Up Timer | 002-999 | 002 | 1 sec base |
|  |  | 10 | Alt If No Member | ON / OFF | OFF |  |
|  |  | 11 | Music Source | 00 - 12 (LDK-300) 00 - 11 (LDK-100) | 00 |  |
|  |  | 12 | ACD Warning Tone | ON / OFF | ON |  |
|  |  | 13 | Alternate Destination | STA #/STA GRP# |  |  |
|  |  | 14 | Supervisor Timer | 000-999 | 030 | 1 sec base |
|  |  | 15 | Supervisor Call Count | 00-99 | 00 |  |
|  |  | 16 | ACD Queued Call | ON / OFF | OFF |  |
|  |  | 17 | MAX Que Call Count | 00-99 | 00 |  |
|  |  | 18 | Supervisor | STA # | - |  |
|  |  | 19 | UCD hunt Stations' Priority | 0-9 | 0 | For each member |
|  | Ring Group | 1 | VMIB Announce 1 Timer | 000-999 | 015 | 1 sec base |
|  |  | 2 | VMIB Announce 2 Timer | 000-999 | 000 | 1 sec base |
|  |  | 3 | VMIB Announce Location 1 | 00-07 | 00(not_asgn) |  |
|  |  | 4 | VMIB Announce Location 2 | 00-07 | 00(not_asgn) |  |
|  |  | 5 | VMIB Announce 2 Repeat | 000-999 | 000 | 1 sec base |
|  |  | 6 | VMIB Announce 2 Repeat E/D | ON / OFF | OFF |  |
|  |  | 7 | Overflow Destination | STA #/HUNT#/ VMIB #/SYS SPD# | - |  |
|  |  | 8 | Overflow Timer | 000-600 | 180 | 1 sec base |
|  |  | 9 | Wrap Up Timer | 002-999 | 002 | 1 sec base |
|  |  | 10 | Music Source | 00 - 12 (LDK-300) 00 - 11 (LDK-100) | 00 |  |
|  |  | 11 | Max. Queued Call Count | 00-99 | 00 |  |
|  | VM Group | 1 | Wrap-up Timer | 002-999 | 002 | 1 sec base |
|  |  | 2 | Put Mail Index | 1-4 | 1 |  |
|  |  | 3 | Get Mail Index | 1-4 | 2 |  |
|  |  | 4 | Hunt Type | Cir/Term | Term |  |
|  |  | 5 | SMDI Port | 01-13 (LDK-300) 01-11 (LDK-100) | 02(COM2) 01(COM1) |  |
|  |  | 6 | Overflow Timer | 000-600 | 180 | 1 sec base |
|  |  | 7 | Overflow Destination | STA #/HUNT#/ VMIB #/SYS SPD# | - |  |
|  | Pick-up Group | 1 | Auto Pick-up | ON / OFF | OFF |  |
|  |  | 2 | All Group Member Ringing | ON / OFF | OFF |  |

#### TABLE 1.6.19  ISDN ATTRIBUTES

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 200 | 1 | Advice of Charge | 0-6 | 0 | 0: Do not service AOC 1: Italy and Spain 2: Finland 3: Australia 4: Belgium 5: Standard 6: Netherlands |
|  | 2 | CO ATD Code | Max 2 digits | - |  |
|  | 3 | Incoming Prefix Code Insertion | ON / OFF | OFF |  |
|  | 4 | Outgoing Prefix Code Insertion | ON / OFF | ON |  |
|  | 5 | A/u-Law Line Installed | U/A | A-Law | ON: u-Law OFF: A-Law |
|  | 6 | CLI Print | ON/OFF | OFF |  |
|  | 7 | International Access Code | Max 4 digits | - |  |
|  | 8 | Calling Sub-address | ON/OFF | OFF | If this field is ON, station number will be filled in calling party number sub-address IE in setup. |
|  | 9 | My Area Code | Max 6 digits | - |  |
|  | 10 | My Area Prefix Code | Max 4 digits | - |  |
| 201 | - | COLP Table | Entry No. (00-49) | - | Max 10 digits |
| 202 | - | MSN Table | Entry No. (000-249) | - |  |
|  | 1 | CO Line number | 001-200 (LDK-300) 01-40 (LDK-100) | - |  |
|  | 2 | Index | 000-999 | - |  |
|  | 3 | SUB Number | 0-9 | - |  |
|  | 4 | MSN number | 20 digits | - |  |

#### TABLE 1.6.20  LCR TABLE ASSIGNMENT

| PGM | BTN | ITEM |  |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 220 | 1 | LCR Access Mode |  |  | M00/M01/M02/M11/M12/M13 | M00 |  |
|  | 2 | Set the Day of week zone |  |  |  | 1234567 |  |
|  |  | 1 |  | MON | 1 – 3 | 1 |  |
|  |  | 2 |  | TUE | 1 – 3 | 1 |  |
|  |  | 3 |  | WED | 1 – 3 | 1 |  |
|  |  | 4 |  | THUR | 1 – 3 | 1 |  |
|  |  | 5 |  | FRI | 1 – 3 | 1 |  |
|  |  | 6 |  | SAT | 1 – 3 | 1 |  |
|  |  | 7 |  | SUN | 1 – 3 | 1 |  |
|  | 3 | Set the Time Zone of Day zone 1 |  |  |  |  |  |
|  |  | 1 | Time Zone1 |  | 00 – 24 | 0024 |  |
|  |  | 2 | Time Zone2 |  | 00 – 24 | - |  |
|  |  | 3 | Time Zone3 |  | 00 – 24 | - |  |
|  | 4 | Set the Time Zone of Day zone 2 |  |  |  |  |  |
|  |  | 1 |  | Time Zone1 | 00 – 24 | 0024 |  |
|  |  | 2 |  | Time Zone2 | 00 – 24 | - |  |
|  |  | 3 |  | Time Zone3 | 00 – 24 | - |  |
|  | 5 | Set the Time Zone of Day zone 3 |  |  |  |  |  |
|  |  | 1 |  | Time Zone1 | 00 – 24 | 0024 |  |
|  |  | 2 |  | Time Zone2 | 00 – 24 | - |  |
|  |  | 3 |  | Time Zone3 | 00 – 24 | - |  |
| 221 |  | Leading Digit Table |  |  | 000-249 |  |  |
|  | 1 | LCR Type |  |  | 1 – 3 | 3 | 1:int 2:col 3: both |
|  | 2 | Code (leading digit) |  |  | Max 12 digits | - |  |
|  | 3 | Day Zone 1 DMT |  |  | 6digits |  | Time1: 2digit |
|  | 4 | Day Zone 2 DMT |  |  | 6digits |  | Time2: 2digit |
|  | 5 | Day Zone 3 DMT |  |  | 6digits |  | Time3: 2digit |
| 222 |  | Digit Modification Table |  |  | 00-99 |  |  |
|  | 1 | Added Digit |  |  | Max 25 digits |  |  |
|  | 2 | Removal Position |  |  | 1 – 12 | 1 |  |
|  | 3 | Number Of Remove |  |  | 1 – 12 | 0 |  |
|  | 4 | Add Position |  |  | 1 – 13 | 1 |  |
|  | 5 | CO Group |  |  | 1 – 72 (LDK-300) 1 – 24 (LDK-100) | 1 |  |
|  | 6 | Alt Index |  |  | 0 – 99 | - |  |
| 223 |  | LCR Table Initialization |  |  |  |  |  |
|  | 1 | DMT Of Day_zone_1 |  |  | 6 digits |  | Time1: 2digit |
|  | 2 | DMT Of Day_zone_2 |  |  | 6 digits |  | Time2: 2digit |
|  | 3 | DMT Of Day_zone_3 |  |  | 6 digits |  | Time3: 2digit |
|  | 4 | CO Grp Init |  |  | 1 – 72 (LDK-300) 1 – 24 (LDK-100) |  |  |
|  | 5 | Alt Index Init |  |  | 0 - 99 |  |  |
|  | 6 | Init All LCR |  |  |  |  |  |

#### TABLE 1.6.21  TOLL TABLE ASSIGNMENT

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 224 |  | Toll Table |  |  |  |
|  | 1 | Allow Table A (01-30) | Max 14 digits | - |  |
|  | 2 | Deny Table A (01-30) | Max 14 digits | - |  |
|  | 3 | Allow Table B (01-30) | Max 14 digits | - |  |
|  | 4 | Deny Table B (01-30) | Max 14 digits | - |  |
| 225 |  | Canned Toll Table |  |  |  |
|  | 1 | Allow Table (01-20) | Max 14 digits | - |  |
|  | 2 | Deny Table (01-20) | Max 14 digits | - |  |
| 226 |  | Emergency Code Table (01-10) | Max 14 digits |  |  |

#### TABLE 1.6.22  OTHER TABLES

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 227 |  | Author Code Table | 001 - 600 (LDK-300) 001 - 164 (LDK-100) |  |  |
|  |  | Table entry (001-600) | Max 5 digits |  |  |
| 228 |  | CCR Table | 1-70 |  |  |
|  | 1 | Station | STA # | - |  |
|  | 2 | Hunt Group | HUNT # | - |  |
|  | 3 | VMIB | Announce # | - |  |
|  | 4 | VMIB Drop | Announce # |  |  |
|  | 5 | System Speed | 2000-4999 (LDK-300) 2000-3499 (LDK-100) | - |  |
|  | 6 | Internal Page | 1 - 30 (LDK-300) 1 - 10 (LDK-100) | - |  |
|  | 7 | External Page | 1 – 3 | - |  |
|  | 8 | All Call Page | 1 – 3 | - | 1:INT 2:EXT 3:All |
| 229 |  | Exec/Sec Table | 01-36 (LDK-300) 01-12 (LDK-100) |  |  |
|  |  | Table entry(01-36) | STA #/STA # |  |  |
| 230 |  | DID Digit Conversion Table |  |  |  |
|  | 1 | Receive Number | 2 – 4 | 3 |  |
|  | 2 | Dgt Mask | Max 5 digits | #*** |  |
| 231 | - | Flexible DID Table | Entry No.(000-999) | - |  |
|  | 1 | DID Name | Max 11 chars. | - |  |
|  | 2 | Day Destination | 1-9 | - | 1(sta), 2(hunt), 3(VMIB) |
|  | 3 | Night Destination | 1-9 | - | 4(VMIB drop), 5(spd), |
|  | 4 | Weekend Destination | 1-9 | - | 6(int. page), 7(ext. page) |
|  | 5 | Reroute Destination | 1-6 | - | 8(all page), 9(net call) |
| 232 |  | System Speed Zone | 01-10 |  |  |
|  | 1 | Speed Bin Range in Zone | 2200-4999 (LDK-300) 2200-3499 (LDK-100) | - |  |
|  | 2 | Station Range | Station Range | - |  |
|  | 3 | Toll Checking | On/Off | ON |  |

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 233 |  | Weekly Time Table | 1 – 7 | - | Day of Week |
|  | 1 | Day Start Time | 0000 – 2359 | 0900 |  |
|  | 2 | Night Start Time | 0000 – 2359 | 1800 |  |
|  | 3 | Weekend Start Time | 0000 – 2359 |  |  |
| 234 |  | Voice Mail Dial-Table | 1 - 9 |  |  |
|  | 1 | Prefix Index | 12 Digits | - |  |
|  | 2 | Suffix Index | 12 Digits | - |  |
| 235 |  | TIE Line Routing Table | CO number | - |  |

#### TABLE 1.6.23  NETWORKING ATTRIBUTE

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 320 |  | Networking Basic Attributes | F1-F7 |  |  |
|  | 1 | Networking Enable | ON / OFF | OFF |  |
|  | 2 | Networking Retry Count | 00 – 99 | 00 |  |
|  | 3 | Networking CNIP Enable | ON / OFF | OFF |  |
|  | 4 | Networking CONP Enable | ON / OFF | OFF |  |
|  | 5 | Networking Signal Method | FAC / UUS | UUS |  |
|  | 6 | Networking CAS Enable | ON / OFF | OFF |  |
|  | 7 | Networking VPN Enable | ON / OFF | OFF |  |
| 321 |  | Supplementary Attributes | F1-F7 |  |  |
|  | 1 | Networking Transfer Mode | RERT / JOIN | JOIN |  |
|  | 2 | TCP port | 4 digits | 9000 |  |
|  | 3 | UDP port | 4 digits | 9001 |  |
|  | 4 | Gatekeeper IP Address | 12 digits | 0.0.0.0 |  |
|  | 5 | Duration of BLF status | 01 ~ 20 sec | 02 |  |
|  | 6 | Multicast IP Address | 12 digits | 0.0.0.0 |  |
|  | 7 | Net Trans Recall Timer | 1 ~ 300 | 10 |  |
| 322 |  | Networking CO Line Attributes | CO Line range |  |  |
|  | 1 | Networking CO Group | 00 – 24 | 00 |  |
|  | 2 | Networking CO Out Send | ON / OFF | OFF |  |
|  | 3 | Networking CO Out Trans | ON / OFF | OFF |  |
| 323 |  | CAS/VPN CO Group Assign | F1-F3 |  |  |
|  | 1 | CAS Number Table Index | 00-71 | 00 |  |
|  | 2 | VPN CO Group | 00-71 | 00 |  |
|  | 3 | CAS Prefix Code | 8 digits | - |  |
| 324 |  | Networking Routing Table | 00-71 |  |  |
|  | 1 | Numbering Plan Code | 16 digits | - |  |
|  | 2 | Numbering Plan CO Group | 00-24 | - |  |
|  | 3 | System Usage | VOIP(1) / QSIG(0) | QSIG(0) |  |
|  | 4 | CPN Information | 16 digits | - |  |
|  | 5 | Alternate Speed Bin | 2000-4999 | - |  |
|  | 6 | MPB LAN IP Address | IP address | - |  |

#### TABLE 1.6.24  VOIB NET ATTRUBUTE

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 340 |  | VOIB IP SETTING | F1-F3 (LDK-300) F1-F2 (LDK-100) |  |  |
|  |  | VOIB NET SETTING | F1-F5 |  |  |
|  | 1 | IP Addressing(SKIP:#) |  |  |  |
|  | 2 | GATEWAY Addressing (SKIP:#) |  |  |  |
|  | 3 | SUBNET Mask(SKIP:#) |  |  |  |
|  | 4 | DNS Addressing (SKIP:#) |  |  |  |
|  | 5 | TRACE Password |  |  | Max 10 Digits |
|  | 6 | Default CODEC | 1 – 4 | 1 |  |
|  | 7 | Default GAIN | 1 - 62 | 31 |  |
|  | 8 | NO Delay (TOS) | ON / OFF | OFF |  |
|  | 9 | Throughput (TOS) | HIGH / NORMAL | NORMAL |  |
|  | 10 | Reliability (TOS) | HIGH / NORMAL | NORMAL |  |

#### TABLE 1.6.25  NATION SPECIFIC

| PGM | FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 400 |  | DTIB RX Gain |  |  | Korean version |
|  | 1 | DTIB/DKT | 00 – 63 | 26 |  |
|  | 2 | DTIB/SLT | 00 – 63 | 33 |  |
|  | 3 | DTIB/CTR SL | 00 – 63 | 22 |  |
|  | 4 | DTIB/WTU | 00 – 63 | 26 |  |
|  | 5 | DTIB/ACO | 00 – 63 | 33 |  |
|  | 6 | DTIB/CTR CO | 00 – 63 | 22 |  |
|  | 7 | DTIB/DCO | 00 – 63 | 33 |  |
|  | 8 | DTIB/VMIB | 00 – 63 | 29 |  |
|  | 9 | DTIB/DTMF | 00 – 63 | 8 |  |
|  | 10 | DTIB/TONE | 00 – 63 | 32 |  |
|  | 11 | DTIB/MUSIC1 | 00 – 63 | 29 |  |
|  | 12 | DTIB/MUSIC2 | 00 – 63 | 29 |  |
|  | 13 | DTIB/MUSIC3 | 00 – 63 | 29 |  |
| 401 |  | SLIB RX Gain |  |  |  |
|  | 1 | SLIB/DKT | 00 – 63 | 12 |  |
|  | 2 | SLIB/SLT | 00 – 63 | 23 | LDK-100 : Default 27 |
|  | 3 | SLIB/CTR SL | 00 – 63 | 12 | LDK-100 : Default 16 |
|  | 4 | SLIB/WTU | 00 – 63 | 12 |  |
|  | 5 | SLIB/ACO | 00 – 63 | 21 |  |
|  | 6 | SLIB/CTR CO | 00 – 63 | 12 |  |
|  | 7 | SLIB/DCO | 00 – 63 | 24 |  |
|  | 8 | SLIB/VMIB | 00 – 63 | 20 |  |
|  | 9 | SLIB/DTMF | 00 – 63 | 8 |  |
|  | 10 | SLIB/TONE | 00 – 63 | 18 |  |
|  | 11 | SLIB/MUSIC1 | 00 – 63 | 20 |  |
|  | 12 | SLIB/MUSIC2 | 00 – 63 | 20 |  |
|  | 13 | SLIB/MUSIC3 | 00 – 63 | 20 |  |
| PGM | FLEX | ITEM | RANGE | DEFAULT | REMARK |
| 402 |  | CTR SLIB RX Gain |  |  |  |
|  | 1 | CTRSL2/DKT | 00 – 63 | 32 |  |
|  | 2 | CTRSL2/SLT | 00 – 63 | 43 | LDK-100 : Default 47 |
|  | 3 | CTRSL2/ CTR SL | 00 – 63 | 32 | LDK-100 : Default 36 |
|  | 4 | CTRSL2/WTU | 00 – 63 | 32 |  |
|  | 5 | CTRSL2/ACO | 00 – 63 | 41 |  |
|  | 6 | CTRSL2/ATR CO | 00 – 63 | 32 |  |
|  | 7 | CTRSL2/DCO | 00 – 63 | 44 |  |
|  | 8 | CTRSL2/VMIB | 00 – 63 | 40 |  |
|  | 9 | CTRSL2/DTMF | 00 – 63 | 28 |  |
|  | 10 | CTRSL2/TONE | 00 – 63 | 38 |  |
|  | 11 | CTRSL2/MUSIC1 | 00 – 63 | 40 |  |
|  | 12 | CTRSL2/MUSIC2 | 00 – 63 | 40 |  |
|  | 13 | CTRSL2/MUSIC3 | 00 – 63 | 40 |  |
| 403 |  | WTIB RX Gain |  |  |  |
|  | 1 | WTIB/DKT | 00 – 63 | 26 |  |
|  | 2 | WTIB/SLT | 00 – 63 | 33 |  |
|  | 3 | WTIB/CTR SL | 00 – 63 | 22 |  |
|  | 4 | WTIB/WTU | 00 – 63 | 26 |  |
|  | 5 | WTIB/ACO | 00 – 63 | 38 |  |
|  | 6 | WTIB/CTR CO | 00 – 63 | 29 |  |
|  | 7 | WTIB/DCO | 00 – 63 | 33 |  |
|  | 8 | WTIB/VMIB | 00 – 63 | 29 |  |
|  | 9 | WTIB/DTMF | 00 – 63 | 8 |  |
|  | 10 | WTIB/TONE | 00 – 63 | 37 |  |
|  | 11 | WTIB/MUSIC1 | 00 – 63 | 29 |  |
|  | 12 | WTIB/MUSIC2 | 00 – 63 | 29 |  |
|  | 13 | WTIB/MUSIC3 | 00 – 63 | 29 |  |
| 404 |  | ACOB RX Gain |  |  |  |
|  | 1 | ACOB/DKT | 00 – 63 | 26 |  |
|  | 2 | ACOB/SLT | 00 – 63 | 37 |  |
|  | 3 | ACOB/CTR SL | 00 – 63 | 27 |  |
|  | 4 | ACOB/WTU | 00 – 63 | 26 |  |
|  | 5 | ACOB/ACO | 00 – 63 | 36 |  |
|  | 6 | ACOB/STR CO | 00 – 63 | 27 |  |
|  | 7 | ACOB/DCO | 00 – 63 | 33 |  |
|  | 8 | ACOB/VMIB | 00 – 63 | 32 |  |
|  | 9 | ACOB/DTMF | 00 – 63 | 32 |  |
|  | 10 | ACOB/TONE | 00 – 63 | 32 |  |
|  | 11 | ACOB/MUSIC1 | 00 – 63 | 32 |  |
|  | 12 | ACOB/MUSIC2 | 00 – 63 | 32 |  |
|  | 13 | ACOB/MUSIC3 | 00 – 63 | 32 |  |
|  | 14 | ACOB/MODEM | 00 – 63 | 37 |  |

| PGM | FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 405 |  | CTR ACOB RX Gain |  |  |  |
|  | 1 | CTRCO8/DKT | 00 – 63 | 28 |  |
|  | 2 | CTRCO8/SLT | 00 – 63 | 43 |  |
|  | 3 | CTRCO8/CTR SL | 00 – 63 | 32 |  |
|  | 4 | CTRCO8/WTU | 00 – 63 | 31 |  |
|  | 5 | CTRCO8/ACO | 00 – 63 | 41 |  |
|  | 6 | CTRCO8/CTR CO | 00 – 63 | 32 |  |
|  | 7 | CTRCO8/DCO | 00 – 63 | 38 |  |
|  | 8 | CTRCO8/VMIB | 00 – 63 | 37 |  |
|  | 9 | CTRCO8/DTMF | 00 – 63 | 37 |  |
|  | 10 | CTRCO8/TONE | 00 – 63 | 37 |  |
|  | 11 | CTRCO8/MUSIC1 | 00 – 63 | 37 |  |
|  | 12 | CTRCO8/MUSIC2 | 00 – 63 | 37 |  |
|  | 13 | CTRCO8/MUSIC3 | 00 – 63 | 37 |  |
|  | 14 | CTRCO8/MODEM | 00 – 63 | 44 |  |
| 406 |  | DCOB RX Gain |  |  |  |
|  | 1 | DCOB/DKT | 00 – 63 | 26 |  |
|  | 2 | DCOB/SLT | 00 – 63 | 37 |  |
|  | 3 | DCOB/CTR SL | 00 – 63 | 26 |  |
|  | 4 | DCOB/WTU | 00 – 63 | 26 |  |
|  | 5 | DCOB/ACO | 00 – 63 | 24 |  |
|  | 6 | DCOB/CTR CO | 00 – 63 | 15 |  |
|  | 7 | DCOB/DCO | 00 – 63 | 32 |  |
|  | 8 | DCOB/VMIB | 00 – 63 | 32 |  |
|  | 9 | DCOB/DTMF | 00 – 63 | 32 |  |
|  | 10 | DCOB/TONE | 00 – 63 | 32 |  |
|  | 11 | DCOB/MUSIC1 | 00 – 63 | 32 |  |
|  | 12 | DCOB/MUSIC2 | 00 – 63 | 32 |  |
|  | 13 | DCOB/MUSIC3 | 00 – 63 | 32 |  |
|  | 14 | DCOB/MODEM | 00 – 63 | 37 |  |
| 407 |  | VMIB RX Gain |  |  |  |
|  | 1 | VMIB/DKT | 00 – 63 | 21 |  |
|  | 2 | VMIB/SLT | 00 – 63 | 32 |  |
|  | 3 | VMIB/CTR SL | 00 – 63 | 21 |  |
|  | 4 | VMIB/WTU | 00 – 63 | 26 |  |
|  | 5 | VMIB/ACO | 00 – 63 | 32 |  |
|  | 6 | VMIB/CTR CO | 00 – 63 | 23 |  |
|  | 7 | VMIB/DCO | 00 – 63 | 32 |  |
|  | 8 | VMIB/MUSIC1 | 00 – 63 | 32 |  |
|  | 9 | VMIB/MUSIC2 | 00 – 63 | 32 |  |
| 408 |  | DTMF RC Gain |  |  |  |
|  | 1 | DTMF/SLT | 00 – 63 | 28 |  |
|  | 2 | DTMF/CTR SL | 00 – 63 | 17 |  |
|  | 3 | DTMF/ACO | 00 – 63 | 24 |  |
|  | 4 | DTMF/CTR CO | 00 – 63 | 15 |  |
|  | 5 | DTMF/DCO | 00 – 63 | 24 |  |

| PGM | FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 409 |  | EXT PAGE Gain |  |  |  |
|  | 1 | EXT PAGE/DKT | 00 – 63 | 26 |  |
|  | 2 | EXT PAGE/SLT | 00 – 63 | 37 |  |
|  | 3 | EXT PAGE/CTR SL | 00 – 63 | 26 |  |
|  | 4 | EXT PAGE/WTU | 00 – 63 | 26 |  |
|  | 5 | EXT PAGE/ACO | 00 – 63 | 37 |  |
|  | 6 | EXT PAGE/CTR CO | 00 – 63 | 28 |  |
|  | 7 | EXT PAGE/DCO | 00 – 63 | 37 |  |
|  | 8 | EXT PAGE/VMIB | 00 – 63 | 37 |  |
|  | 9 | EXT PAGE/MUSIC1 | 00 – 63 | 37 |  |
|  | 10 | EXT PAGE/MUSIC2 | 00 – 63 | 37 |  |
|  | 11 | EXT PAGE/MUSIC3 | 00 – 63 | 37 |  |
| 410 |  | CPT Gain |  |  |  |
|  | 1 | CPT/ACO | 00 – 63 | 24 |  |
|  | 2 | CPT/CTR CO | 00 – 63 | 15 |  |
|  | 3 | CPT/DCO | 00 – 63 | 24 |  |
| 411 |  | MODEM Gain |  |  |  |
|  | 1 | MODEM/ACO | 00 – 63 | 24 |  |
|  | 2 | MODEM/CTR CO | 00 – 63 | 20 |  |
|  | 3 | MODEM/DCO | 00 – 63 | 24 |  |
| 412 |  | Short SLIB Gain |  |  | SAF only |
|  | 1 | Shot ACO | 00 – 63 | 31 |  |
|  | 2 | Long ACO | 00 – 63 | 31 |  |
| 413 |  | Long SLIB Gain |  |  | SAF only |
|  | 1 | Shot ACO | 00 – 63 | 37 |  |
|  | 2 | Long ACO | 00 – 63 | 37 |  |
| 414 |  | Far SLIB Gain |  |  | SAF only |
|  | 1 | Shot ACO | 00 – 63 | 45 |  |
|  | 2 | Long ACO | 00 – 63 | 45 |  |
| 415 |  | Short ACO Gain |  |  | SAF only |
|  | 1 | Short SLIB | 00 – 63 | 35 |  |
|  | 2 | Long SLIB | 00 – 63 | 41 |  |
|  | 3 | Far SLIB | 00 – 63 | 47 |  |
| 416 |  | Long ACO Gain |  |  | SAF only |
|  | 1 | Short SLIB | 00 – 63 | 39 |  |
|  | 2 | Long SLIB | 00 – 63 | 45 |  |
|  | 3 | Far SLIB | 00 – 63 | 51 |  |
| 420 |  | System Tone Frequency |  |  |  |
|  | 1 | Dial Tone | 4digits | 0425, 0000 |  |
|  | 2 | Ring Back Tone | 4digits | 0425, 0000 |  |
|  | 3 | Busy Tone | 4digits | 0425, 0000 |  |
|  | 4 | Error Tone | 4digits | 0620, 000 |  |
|  | 5 | Dummy Dial Tone | 4digits | 0350, 440 |  |
| 421 |  | Differential Ring Frequency |  |  |  |
|  | 1 | Ring 1 | 4digits | 1000, 1020 |  |
|  | 2 | Ring 2 | 4digits | 0890, 0910 |  |
|  | 3 | Ring 3 | 4digits | 1260, 1280 |  |
|  | 4 | Ring 4 | 4digits | 0800, 0820 |  |

| PGM | FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 422 |  | Distinct Ring Frequency |  |  |  |
|  | 1 | Ring 1 | 4digits | 0480, 0000 |  |
|  | 2 | Ring 2 | 4digits | 0400, 0000 |  |
|  | 3 | Ring 3 | 4digits | 0620, 0000 |  |
|  | 4 | Ring 4 | 4digits | 0770, 0000 |  |
| 423 |  | ACNR Tone Cadence |  |  |  |
|  | 1 | Ring-Back Tone | 0-255 | 050, 100 | 20msec base |
|  | 2 | Busy Tone | 0-255 | 025, 025 | 20msec base |
|  | 3 | Error Tone | 0-255 | 012, 012 | 20msec base |
|  | 4 | S –Dial Tone | 0-255 | 070, 000 | 20msec base |

#### TABLE 1.6.26  INITIALIZATION

| PGM | FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 450 |  | Initialization |  |  |  |
|  | 1 | Flexible Numbering Plan Initialization |  |  |  |
|  | 2 | Station Database Initialization |  |  |  |
|  | 3 | CO Line Database Initialization |  |  |  |
|  | 4 | System Feature Database Initialization |  |  |  |
|  | 5 | Station Group Database Initialization |  |  |  |
|  | 6 | ISDN Tables Database Initialization |  |  |  |
|  | 7 | Reserved |  |  | Reserved |
|  | 8 | System Timer Database Initialization |  |  |  |
|  | 9 | Toll Table Database Initialization |  |  |  |
|  | 10 | LCR Database Initialization |  |  |  |
|  | 11 | Tables Initialization |  |  |  |
|  | 12 | Flexible Button Program Initialization |  |  |  |
|  | 13 | Networking Database Initialization |  |  |  |
|  | 14 | All Database Initialization |  |  |  |
|  | 15 | System Reset By Software |  |  |  |

#### TABLE 1.6.27  PRINT PROT DATABASE

| PGM | FLEX | ITEM |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| 451 |  | Print Prot Data |  |  |  |  |
|  | 1 | Flexible Numbering Plan Print |  |  |  |  |
|  | 2 | Station Database Print |  | STN_R |  |  |
|  | 3 | CO Line Database Print |  | CO_R |  |  |
|  | 4 | System Feature Database Print |  |  |  |  |
|  | 5 | Station Group Database Print |  |  |  |  |
|  | 6 | ISDN Tables Database Print |  |  |  |  |
|  | 7 | System Timer Database Print |  |  |  |  |
|  | 8 | Toll Table Database Print |  |  |  |  |
|  | 9 | LCR Database Print |  |  |  |  |
|  | 10 | Other Tables Print |  |  |  |  |
|  | 11 | Nation Specific Database Print |  |  |  |  |
|  | 12 | Flexible Button Program Print |  | STN_R |  |  |
|  | 13 | Networking Data Print |  |  |  |  |
|  | 14 | All Database Print |  |  |  |  |
|  | 15 | LCD Message Print |  |  |  |  |
|  |  | 1 | Language | 00 – 12 | Nation specific | 00:ENG 01:ITA 02:FIN 03:DUT 04:SWE 05:DAN 06:NOR 07:HUN 08:GER 09:FRE 10:POR 11:SPA 12:KOR |
|  |  | 2 | Station Type | 0 – 2 | 0 | 0: NORMAL 1: LG-GAP 2: LARGE |
|  | 16 | Print Port Data |  |  |  |  |

# 2.	PRE-PROGRAMMED DATABASE

This section describes the pre-programming of the data. When installed, user must program some database (Ex. Nationality, Numbering Plan Type, and Slot Assignment) first, otherwise system may not operate properly.

If you want to change nationality of the system, then use PGM 100. To assign the board in each rack, proceed PGM 101-102. After assigning rack slot, you must adjust PGM 103, Logical Slot Assignment. If you want to use the station number range flexibly, you can select a proper numbering plan type at PGM 104. If you want to change station numbers or feature codes, then use PGM 105~107. If you need more than 4 digits as a station number for VPN or QSIG, use the station prefix code at PGM 200-BTN 1.

PROCEDURE FOR PRE-PROGRAMMING

- Set Nationality (See Section 2.1)

- Set DIP switch 8 as ‘ON’, and reset the system (PGM 450–BTN 15). Then slot assignment is configured.

- Set DIP switch 8 as ‘OFF’ to retain current configuration.

- Set Rack Slot Assignments (If configuration is not equal to the system default slot assignments, you must program it manually. See Section 2.2)

- Set Logical Slot Assignment. (After Logical Slot assignment or Rack Slot assignment, the system must be reset manually. See Section 2.4)

- Set Numbering Plan Type. (See Section 2.5)

- Set Flexible Numbering Plan. (See Section 2.6)

## 2.1	LOCATION PROGRAM  (PGM 100)

PROCEDURE

| LOCATION PROGRAM PRESS FLEX_KEY (1-2) | - [TRANS/PGM] + 100. |
| --- | --- |
| NATION CODE 82 | - Press Flex. BTN 1~2. (Ex. Press Flex. BTN 1) LCD shows default or saved value of NATION CODE. Enter desired nation code in the TABLE 2.1.2. To save the changed nation code in memory, press the [HOLD/SAVE] button. |
| LOCATION PROGRAM PRESS FLEX_KEY (1-2) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Nation Code |  | 82 | Max 2 digits |
| 2 | Site Name |  | - | Max 23 digits (Refer to English Character Set Table 2.1.3) |

TABLE 2.1.1	  Location Program (PGM 100)

| NATION | CODE | NATION | CODE | NATION | CODE |
| --- | --- | --- | --- | --- | --- |
| America | 1 | Argentina | 54 | Australia | 61 |
| Bahrain | 973 | Bangladesh | 880 | Belgium | 32 |
| Bolivia | 591 | Brazil | 55 | Brunei | 673 |
| Burma | 95 | Cameroon | 237 | Chile | 56 |
| China (Taiwan) | 886 | CIS | 7 | Colombia | 57 |
| Costa Rica | 506 | Cyprus | 357 | Czech | 42 |
| Denmark | 45 | Ecuador | 593 | Egypt | 20 |
| El Salvador | 503 | Ethiopia | 251 | Fiji | 679 |
| Finland | 358 | France | 33 | Gabon | 241 |
| Germany | 49 | Ghana | 233 | Greece | 30 |
| Guam | 671 | Guatemala | 502 | Guyana | 592 |
| Haiti | 509 | Honduras | 504 | Hong Kong | 852 |
| India | 91 | Indonesia | 62 | Iran | 98 |
| Iraq | 964 | Ireland | 353 | Israel | 972 |
| Italy | 39 | Japan | 81 | Jordan | 962 |
| Kenya | 254 | Korea | 82 | Kuwait | 965 |
| Liberia | 231 | Libya | 218 | Luxembourg | 352 |
| Malaysia | 60 | Malta | 356 | Mexico | 52 |
| Monaco | 377 | Morocco | 212 | Netherlands | 31 |
| New Zealand | 64 | Nigeria | 234 | Norway | 47 |
| Oman | 968 | Pakistan | 92 | Panama | 507 |
| P.N.G | 675 | Paraguay | 595 | Peru | 51 |
| Philippines | 63 | Portugal | 351 | Qatar | 974 |
| Saudi Arabia | 966 | Senegal | 221 | Singapore | 65 |
| South Africa | 27 | Spain | 34 | Sri Lanka | 94 |
| Swaziland | 268 | Sweden | 46 | Switzerland | 41 |
| TELKOM | *27 | Thailand | 66 | Tunisia | 216 |
| Turkey | 90 | U.A.E. | 971 | United Kingdom | 44 |
| Uruguay | 598 | Venezuela | 58 | Y.A.R. | 967 |

TABLE 2.1.2  Nation & National Code (PGM 100)

| Q – 11 Z – 12 . – 13 1 – 10 | A - 21 B - 22 C - 23 2 - 20 | D - 31 E - 32 F - 33 3 - 30 |
| --- | --- | --- |
| G – 41 H - 42 I - 43 4 – 40 | J - 51 K - 52 L - 53 5 - 50 | M - 61 N - 62 O - 63 6 - 60 |
| P - 71 R - 72 S - 73 Q - 7* 7 – 70 | T - 81 U - 82 V - 83 8 - 80 | W - 91 X - 92 Y - 93 Z - 9# 9 - 90 |
| *1 - Blank *2 - : *3 - , | 0-00 | # |

TABLE 2.1.3  English Character Set

## 2.2	RACK SLOT ASSIGNMENT  (PGM 101)

Rack slot assignment can be performed automatically and manually. If the DIP switch of the manual board detection is ON, system will detect the installed board type automatically. If the DIP switch is OFF, the board type code should be entered at each slot. After manual rack slot assignment, user must reset the system manually. To change the number of WTIB channels, program PGM102(WTIB port number assign) and reset manually.

PROCEDURE

| BOARD ASSIGNMENT ENTER SLOT NUMBER | - [TRANS/PGM] + 101 |
| --- | --- |
| SLOT 01 (F1:ID F2:DEVS) ID : DTIB12 DEVS: 12 | - To program the slot assignment, dial 2 digits as a slot number. If the slot is already assigned, related message will be displayed on the LCD. |
| SLOT 01 (F1:ID F2:DEVS) ID : SLIB6 DEVS: 6 | - To change pre-assigned board, press Flex BTN 1 and dial 2 digits as the board type code. Refer to Table 2.2.1 for each board type code. Then entered board type code will be displayed on the LCD. |
| SLOT 01 (F1:ID F2:DEVS) ID : PRIB DEVS: 30 | Cf.) In case of PRIB, logical port assign is possible. To program a logical port number, after entering board type code, press BTN 2 and dial 2 digits as logical port number between 01 and maximum board port capacity. If logical port is not entered, maximum port will be assigned .(After programming logical port number, program PGM 103.) |
| SLOT 01 (F1:ID F2:DEVS) ID : PRIB DEVS: 30 | - Press the [HOLD/SAVE] button for saving database permanently. |
| BOARD ASSIGNMENT ENTER SLOT NUMBER | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| STA | CODE | COL | CODE | STA & COL | CODE | Etc | CODE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DTIB12 DTIB24 SLIB6 SLIB12 WTIB DSIB | 11 12 13 14 15 18 | PRIB BRIB LCOB4 LCOB8 DIDB BWDIDB TLIB EMIB AC15 DCOB VOIB NPRIB NBRIB(8) NBRIB(4) CLCOB8 RDIB EMIB8 | 31 32 33 34 35 36 37 38 39 40 41 42(LDK-100) 43(LDK-100) 44(LDK-100) 45 46 47 | STIB | 51 | VMIB MISB | 61 71 |

TABLE 2.2.1   Board Type Code Table  (PGM 101)

## 2.3	WTIB PORT NUMBER ASSIGNMENT  (PGM 102)

This procedure determines the number of registrable wireless terminals. The available number of wireless terminal is 008-192 for LDK-300 and 08-80(08-40 for 1 rack) for LDK-100, and default value is 008.

PROCEDURE

| WTIB PORT NO (008–192) 008 (MULTIPLE OF 8) | - [TRANS/PGM] + 102. In LDK-100, (008-192) will be like (08-80). If LDK-100 has only one rack, it will be like (08-40). |
| --- | --- |
|  | - Press the number of registrable wireless terminals. The number should be in the available range, and be the multiple of 8. |
| WTIB PORT NO (008–192) 032 (MULTIPLE OF 8) | - Press the [HOLD/SAVE] button for updating database permanently. |

## 2.4	LOGICAL SLOT ASSIGNMENT  (PGM 103)

Logical slot assignment can be performed automatically and manually. If the DIP switch of the manual board detection is ON, system will detect the logical slot assign in sequence as increase order automatically. If the DIP switch is OFF, the logical slot assignment should be entered at each board type. After manual logical slot assignment, user must reset the system manually. For the VMIB, it is impossible to make logical slot assignment. So user must assign the VMIB slot number manually. In case of WTIB it is detected as one board by logical slot assignment. So you must assign just one slot number regardless of the number of installed WTIBs.

PROCEDURE

| LOGICAL SLOT ASSIGN COL STA VMIB | - [TRANS/PGM] + 103 |
| --- | --- |
| 02 03 .. .. .. .. .. .. .. .. .. .. .. .. .. .. | - Press BTN regarding the slot type and dial slot numbers with increasing order for logical slot numbers. |
| 02 03 05 07 .. .. .. .. .. .. .. .. .. .. .. .. | - Press the [HOLD/SAVE] button for saving database permanently. |
|  | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | DEFAULT | REMARK |
| --- | --- | --- | --- |
| 1 | COL Board | - | DIP Switch OFF: Manual slot assignment DIP Switch ON: Automatic slot assignment |
| 2 | STA Board | - | DIP Switch OFF: Manual slot assignment DIP Switch ON: Automatic slot assignment |
| 3 | VMIB | - | Should be programmed manually |

TABLE 2.4.1	  Button Configuration for Slot Assignment  (PGM 103)

## 2.5	NUMBERING PLAN TYPE  (PGM 104)

PROCEDURE

| NUMBERING PLAN (1-8) 1 | - [TRANS/PGM] + 104. |
| --- | --- |
| NUMBERING PLAN (1-8) 2 | - To assign the numbering plan type, press one digit between 1-8. |
| NUMBERING PLAN (1-8) 2 | - Press the [HOLD/SAVE] button for saving database permanently. |

| BTN | ITEM | INTERCOM RANGE |  | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
|  |  | LDK-300 | LDK-100 |  |  |
| 1 | Number Set Type 1 | 100 – 399 | 100 – 227 | Yes | As the basic type, the 1st digit of station number should be 1 – 4. |
| 2 | Number Set Type 2 | 100 – 399 | 100 – 227 | No | The station number can be changed within 799. |
| 3 | Number Set Type 3 | 100 – 399 | 100 – 227 | No |  |
| 4 | Number Set Type 4 | 7700-7999 | 7700-7827 | No |  |
| 5 | Number Set Type 5 | 200 – 499 | 200 – 327 | No |  |
| 6 | Number Set Type 6 | 10 – 79 | 10 – 79 | No | Max Station Ports: 70 |
| 7 | Number Set Type 7 | 100 – 299 | 100 – 227 | No | Max Station Ports: 200 (LDK-300) |
| 8 | Number Set Type 8 | 100 – 399 | 100 – 227 | No | The station number can be changed within 999. |

TABLE 2.5.1	  Flexible Numbering Plan  (PGM 104)

## 2.6	FLEXIBLE NUMBERING PLAN  (PGM 105)

You can change each station number in this field. Station number of the system can be assigned flexibly via the system programming. Station Numbering Code length should be in the range of 2 digits through 4 digits.

PROCEDURE

| 000 001 002 003 100 101 102 103 | - [TRANS/PGM] + 105. |
| --- | --- |
| 000 001 002 003 100 101 102 103 | - Station Number Assign. You will see the 4 station numbers corresponding to the 4 port numbers. Station number length is in the range of 2 digits through 4 digits. There are two methods for changing station number. - Dial two station numbers - Range start station number & range end station number, then LCD shows dialed range value. Press the [HOLD/SAVE] button, then station numbers changed from the first station number on current LCD to range end (All LEDs of BTNs are off.). - Press one of BTN 1-4 (Each BTN 1-4 is assigned to station number 1- 4 on the current LCD), then LED of pressed Flex button is steady on. Dial new station number and press the [HOLD/SAVE] button, or press other Flex button to assign station number to other station without saving (The LED of pressed Flex. button is on.). - If you want to delete all station numbers, press the [SPEED] button and press [HOLD/SAVE] button, then all station numbers are cleared. - If you want to change next 4 station numbers then press [▼] button. If you want to change previous 4 station numbers, then press [▲] button. |
| 000 001 002 003 100 400 102 103 | - Press the [HOLD/SAVE] button for saving database permanently. (Ex: Press Flex. BTN 2, dial 400 and press [HOLD/SAVE] button.) |

## 2.7	FLEXIBLE NUMBERING PLAN  (PGM 106-107)

Feature codes of the system can be assigned flexibly via the system programming. Feature Code length should be in the range of 1(one) digit through 4 digits. Let's say that a feature code conflict has occurred in such a case that a feature code string matches with other longer feature code string, checking from the first digit of the code. For example, features Code 53 and 536 have a feature code conflict. The system will not allow any feature code conflict.

PROCEDURE

| FLEX NUMBERING PLAN A PRESS FLEX KEY (01-24) | - [TRANS/PGM] + 106. You can program the 24 Flex. Numbers. Table 2.7.1 illustrates the programmable list of the 24 flexible Numbering plan used by PGM 106 and Table 2.7.2 illustrates the list for PGM 107. |
| --- | --- |
| STA GRP PILOT NUMBER START & END #(620-667) | - To change a numbering plan, press the related flexible button. Each code length is in 1~4 digits.(To enter station range, 2 ~ 8 digits) For example, press Flex. BTN 1 to change Station Group pilot number. In LDK-100, the LCD display will be different with the left. see below table for more. Dial the first station group pilot number and last station group pilot number, and press the [HOLD/SAVE] button. Then you can see the changed station group pilot numbers on the LCD. If you press other Flex. BTN, you can assign other numbering plan. |
| STA GRP PILOT NUMBER START & END #(620-667) | - If you want to save all changed flexible numbers to system memory, press the [HOLD/SAVE] button. There are no errors in the Flexible Numbers, then confirmation tone will be heard. |
|  | - If some errors are detected, then error tone will be heard without updating system memory. |

| BTN | LCD DISPLAY | ITEM |
| --- | --- | --- |
| 1 | STA GRP PILOT NUMBER START & END #(620-667) : LDK-300 STA GRP PILOT NUMBER START & END #(620-634) : LDK-100 | Station Group Pilot Number |
| 2 | INT PAGE ZONES START & END #(501-535) : LDK-300 INT PAGE ZONES START & END #(501-515) : LDK-100 | Internal Page Zone Number |
| 3 | INT ALL CALL ENTER NEW #(543) | Internal All Call Page |
| 4 | MEET ME PAGE ENTER NEW #(544) | Meet Me Page |
| 5 | EXT PAGE ZONE 1 ENTER NEW #(545) | External Page Zone - 1 |
| 6 | EXT PAGE ZONE 2 ENTER NEW #(546) | External Page Zone - 2 |
| 7 | EXT PAGE ZONE 3 ENTER NEW #(547) | External Page Zone - 3 |
| 8 | EXT ALL CALL ENTER NEW #(548) | External All Call Page |
| 9 | ALL CALL PAGE ENTER NEW #(549) | All Call Page (Internal/External) |
| 10 | SMDR ACT CODE ENTER ENTER NEW #(550) | SMDR Account Code Enter |
| 11 | FLASH CMD TO CO ENTER NEW #(551) | Flash Command to CO Line |
| BTN | LCD DISPLAY | ITEM |
| 12 | SLT LAST SPD DIAL ENTER NEW #(552) | Last Number Redial (LNR) |
| 13 | DND ENTER NEW #(553) | Do-Not-Disturb |
| 14 | CALL FWD ENTER NEW #(554) | Call Forward |
| 15 | SPD DIAL PGM ENTER NEW #(555) | Speed Dial Program |
| 16 | MSG WAIT ENABLE ENTER NEW #(556) | MSG Wait/Call-Back Enable |
| 17 | MSG WAIT RETURN ENTER NEW #(557) | MSG Wait/Call-Back Answer |
| 18 | SPD DIAL ACCESS ENTER NEW #(558) | Speed Dial Access |
| 19 | DND/FWD CANCEL ENTER NEW #(559) | Cancel DND/CFW/Pre-selected MSG Features |
| 20 | SYSTEM HOLD ENTER NEW #(560) | System Hold |
| 21 | STA RELOC BACKUP ENTER NEW #(561) | Backup STA Relocation |
| 22 | STA RELOC RETRIEVE ENTER NEW #(562) | Retrieve STA Relocation |
| 23 | SLT PGM MODE ENTER ENTER NEW #(563) | SLT Program Mode Select |
| 24 | ACD REROUTE ENTER NEW #(564) | ACD Reroute |

TABLE 2.7.1  Flex Numbering Plan A  (PGM 106)

| BTN | LCD DISPLAY | ITEM |
| --- | --- | --- |
| 1 | ALARM RESET ENTER NEW #(565) | Alarm Reset |
| 2 | GROUP CALL PKUP ENTER NEW #(566) | Group Call Pick-Up |
| 3 | UCD DND ENTER NEW #(568) | UCD Group DND |
| 4 | NIGHT ANSWER ENTER NEW #(569) | Night Answer |
| 5 | CALL PARK LOCATIONS START & END #(601-619) : LDK-300 CALL PARK LOCATIONS START & END #(601-610) : LDK-100 | Call Park Locations |
| 6 | DIRECT CALL PKUP ENTER NEW #(7) | Direct Call Pick-Up |
| 7 | ACCESS CO GROUP FEAT START & END #(801-872): LDK-300 ACCESS CO GROUP FEAT START & END #(801-824): LDK-100 | Access CO Group |
| 8 | ACCESS IND CO FEAT START&END #(88001-88200) : LDK-300 ACCESS IND CO FEAT START&END #(8801-8840) : LDK-100 | Access Individual CO Line |
| 9 | TIE ROUTING ACCESS ENTER NEW #(8901) | Tie Routing Access |
| 10 | ACCESS HELD CO FEAT ENTER NEW #(8*) | Access Held CO Group |
| 11 | ACCESS HELD IND CO FEAT START&END # (8#001- 8#200) ACCESS HELD IND CO FEAT START&END # (8#01- 8#40) | Access Held Individual CO Line |
| 12 | ACCESS CO IN 1ST CO GRP ENTER NEW #(9) | Access to CO line in the 1st available CO Group |
| 13 | ATTENDANT CALL ENTER NEW #(0) | Attendant Call |
| 14 | DOOR OPEN 1 ENTER NEW #(#*1) | Door Open – 1 |
| 15 | DOOR OPEN 2 ENTER NEW #(#*2) | Door Open – 2 |
| 16 | DOOR OPEN 3 ENTER NEW #(#*3) | Door Open – 3 |
| 17 | DOOR OPEN 4 ENTER NEW #(#*4) | Door Open – 4 |
| 18 | DOOR OPEN 5 ENTER NEW #(#*5) | Door Open – 5 |
| 19 | DOOR OPEN 6 ENTER NEW #(#*6) | Door Open – 6 |
| 20 | DOOR OPEN 7 ENTER NEW #(#*7) : LDK-300 Only | Door Open – 7 : LDK-300 Only |
| 21 | VM MSG WAIT ENABLE ENTER NEW #(*8) | VM MSG Wait Enable |
| 22 | VM MSG WAIT CANCEL ENTER NEW #(*9) | VM MSG Wait Cancel |

TABLE 2.7.2  Flex Numbering Plan B  (PGM 107)

## 2.8	IP SETTING  (PGM 108)

IP address, Subnet mask, and GW address should be programmed for trace, remote upgrade, PC Admin, PC Attendant, etc via network.

PROCEDURE

| IP NET SETTING PRESS FLEX_KEY (1-5) | - [TRANS/PGM] + 108. |
| --- | --- |
| IP NAME LGICKEYPHONE | - For IP Name assignment, press Flex BTN 1, and enter the character. |
| SERVER IP ADDR(SKIP:#) 165.147. 3. 1 | - For Server IP assignment, press Flex BTN 2, and enter the 12 digit. - To skip entering digit, press # button. |
| SERVER IP ADDR(SKIP:#) 165.147. 3. 1 | - Press the [HOLD/SAVE] button for saving database permanently. |
| IP NET SETTING PRESS FLEX_KEY (1-5) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | INTERCOM RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | IP Name | 16 Chars | - | IP Name |
| 2 | Server IP Address | 12 Digits | - | Sever Address |
| 3 | CLI IP Address | 12 Digits | - | CLI IP Address |
| 4 | Gateway Address | 12 Digits | - | Gateway Address |
| 5 | Subnet mask | 12 Digits | - |  |

TABLE 2.8.1	  IP Setting  (PGM 108)

# 3.	STATION PROGRAMMING

If station features are to be changed, press the [TRANS/PGM] button and dial 110-124 in Admin Programming mode. When programming, LCD and LEDs indicate current programmed data and programming status. If the programmer enters correct data, then LCD and LED's show the entered data and the data is stored in the temporary buffer area. To save data permanently, press the [HOLD/SAVE] button, then all data in the temporary buffer (same as LCD/LED's status) are stored into system memory.

## 3.1	STATION ID  (PGM 110)

When the system is initialized, all the stations’ ID will be assigned to default value as Table 3.1.3.

PROCEDURE

| STATION ID ASSIGN ENTER STA RANGE | - [TRANS/PGM] + 110. |
| --- | --- |
| (F1: ID F2 :ASC) DKTU | - Enter station range (Ex. 100-110). LCD shows the station range. First press the 1st Flex. BTN and dial two digits to choose type of the station as Table 3.1.1. - Note: DSS Map Type is only valid with the Flex. BTN 2. |
| (F1: ID F2 :ASC) SLT (DTMF) | - Dial digit 12(07 for LDK-100), and SLT (DTMF) will be assigned to the selected station range. Press the [HOLD/SAVE] button for updating database permanently. |
| (F1: ID F2 :ASC) DSS MAP2 : STA . . . . | - In case of DSS Map type, enter the station range (Ex. 110-110) in the step (1). At first press the 1st Flex. BTN and enter 2-digit (Ex. 03: DSS Map 2) and press the [HOLD/SAVE] button for updating database. Next, press the 2nd Flex. BTN to assign the associating station number. Enter the station number and press the [HOLD/SAVE] button for updating database permanently. - Note: When identifying a station as a DSS, you must enter the station number of the key station the DSS is associated with, not station number that the DSS is using. See also TABLE 3.1.2 for DSS default button configuration. |

| DGT | LDK-300 |  | LDK-100 |  |
| --- | --- | --- | --- | --- |
|  | ITEM | FLEX KEY 2 | ITEM | FLEX KEY 2 |
| 1 | DKTU | Invalid | DKTU | Invalid |
| 2 | DSS MAP 1 | Associated STA No. | DSS MAP 1 | Associated STA No. |
| 3 | DSS MAP 2 | Associated STA No. | DSS MAP 2 | Associated STA No. |
| 4 | DSS MAP 3 | Associated STA No. | DSS MAP 3 | Associated STA No. |
| 5 | DSS MAP 4 | Associated STA No. | ICM BOX | Invalid |
| 6 | DSS MAP 5 | Associated STA No. | WHTU | Invalid |
| 7 | DSS MAP 6 | Associated STA No. | SLT (DTMF) | Invalid |
| 8 | DSS MAP 7 | Associated STA No. | SLT (PULSE) | Invalid |
| 9 | DSS MAP 8 | Associated STA No. | SLT with MSG Wait Lamp for DTMF | Invalid |
| 10 | ICM BOX | Invalid | SLT with MSG Wait Lamp for Pulse | Invalid |
| 11 | WHTU | Invalid | ISDN phone | Invalid |
| 12 | SLT (DTMF) | Invalid |  |  |
| 13 | SLT (PULSE) | Invalid |  |  |
| 14 | SLT with MSG Wait Lamp for DTMF | Invalid |  |  |
| 15 | SLT with MSG Wait Lamp for Pulse | Invalid |  |  |
| 16 | ISDN phone | Invalid |  |  |

TABLE 3.1.1  Button Configuration for Station ID  (PGM 110)

| ITEM | DEFAULT | REMARK |
| --- | --- | --- |
| DSS/DLS MAP 1 | First 12 Buttons Button 1 : Intrusion Button 2 : All Call Page Button 3 : Call Park 01 Button 4 : Station Group 1 Button 5 : Camp-On Button 6 : Internal All Call Page Button 7 : Call Park 02 Button 8 : Station Group 2 Button 9 : Grp Call Pickup Button 10 : External All Call Page Button 11 : Call Park 03 Button 12 : Station Group 3 |  |
| DSS/DLS MAP 2 | Station Ports 136 – 183 |  |
| DSS/DLS MAP 3 | Station Ports 184 – 231 (184-227 for LDK-100) |  |
| DSS/DLS MAP 4 | Station Ports 232 – 279 | LDK-300 Only |
| DSS/DLS MAP 5 | Station Ports 280 – 327 |  |
| DSS/DLS MAP 6 | CO Line 01 – 48 |  |
| DSS/DLS MAP 7 | CO Line 49 – 96 |  |
| DSS/DLS MAP 8 | CO Line 97 – 144 |  |

TABLE 3.1.2  Initial Button Configurations for DSS/DLS Map  (PGM 110)

| DGT | LDK-300 |  | LDK-100 |  |
| --- | --- | --- | --- | --- |
|  | ITEM | DEFAULT ID | ITEM | DEFAULT ID |
| 1 | DKTU | DKTU | DKTU | DKTU |
| 2 | DSS MAP 1 | DKTU | DSS MAP 1 | DKTU |
| 3 | DSS MAP 2 | DKTU | DSS MAP 2 | DKTU |
| 4 | DSS MAP 3 | DKTU | DSS MAP 3 | DKTU |
| 5 | DSS MAP 4 | DKTU | ICM BOX | ICM BOX |
| 6 | DSS MAP 5 | DKTU | WHTU | WHTU |
| 7 | DSS MAP 6 | DKTU | SLT (DTMF) | SLT (DTMF) |
| 8 | DSS MAP 7 | DKTU | SLT (PULSE) | SLT (DTMF) |
| 9 | DSS MAP 8 | DKTU | SLT with MSG Wait Lamp for DTMF | SLT (DTMF) |
| 10 | ICM BOX | ICM BOX | SLT with MSG Wait Lamp for Pulse | SLT (DTMF) |
| 11 | WHTU | WHTU | ISDN Phone | ISDN |
| 12 | SLT (DTMF) | SLT (DTMF) |  |  |
| 13 | SLT (PULSE) | SLT (DTMF) |  |  |
| 14 | SLT with MSG Wait Lamp for DTMF | SLT (DTMF) |  |  |
| 15 | SLT with MSG Wait Lamp for Pulse | SLT (DTMF) |  |  |
| 16 | ISDN Phone | ISDN |  |  |

TABLE 3.1.3  Default Value of Station ID  (PGM 110)

## 3.2	STATION ATTRIBUTE – I  (PGM 111)

PROCEDURE

| STATION ATT 1 ENTER STA RANGE | - [TRANS/PGM] + 111. |
| --- | --- |
| 100-110 STATION ATT 1 PRESS FLEX_KEY (01-15) | - Enter station range (Ex. 100-110). |
|  | - To program, use the Flex. BTN as Table 3.2.1. Press Flex. BTN 1-15 for setting each value. After pressing one flex BTN, the revised value can be set by entered digit. |
| 100-110 AUTO SPKER (1 : ON / 0 : OFF) : ON | Ex) When pressing Flex. BTN 1, ‘Auto Speaker Selection’ attribute value will be displayed. |
| 100-110 AUTO SPKER (1 : ON / 0 : OFF) : OFF | User can change value by pressing digit 0. LCD shows the changed attribute value. |
| 100-110 AUTO SPKER (1 : ON / 0 : OFF) : OFF | - Press the [HOLD/SAVE] button for updating database permanently. |
| STATION ATT 1 PRESS FLEX KEY (01-15) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Auto Speaker Selection | ON/OFF | ON | Allowance to access a CO line or place a DSS call by pressing appropriate {CO} or {DSS} button without lifting handset or pressing the [MON] button. |
| 2 | Call Forward | ON/OFF | ON | Enables Call Forward to be activated by the station. |
| 3 | DND | ON/OFF | ON | Enables DND to be activated by the station. |
| 4 | Data Line Security | ON/OFF | OFF | The allowance to protect from override and camp-on, when busy state. |
| 5 | Howling Tone to SLT | ON/OFF | ON | The allowance to give howling tone to SLT |
| 6 | ICM Box Signaling | ON/OFF | OFF | The allowance to receive ICM box signal. |
| 7 | No Touch Answer | ON/OFF | ON | The allowance to connect the transferred CO line automatically when station mode is H/P. |
| 8 | Page Access | ON/OFF | OFF | Allows access to paging by the station. |
| 9 | Ring Type | 0 – 4 | 0 | The station can give own ring type signal to another station in system through this field calling party centric. |
| 10 | Speaker Ring | (1:S /2:H /3:BOTH) | SPKR | Station rings through Speaker, Headset or Both (speaker and headset). |
| 11 | Speakerphone | ON/OFF | ON | Operate with speakerphone. |
| 12 | VMIB Slot | 0-2 | 0 | Assign VMIB logical slot with station base |
| 13 | ICM Group | 01-15(LDK-300) 01-05(LDK-100) | 01 | Assign ICM Tenancy Group which is belonged to the station |
| 14 | Error Tone for TAD | ON/OFF | OFF | With Answering Device instead of SLT, busy tone will be provided instead of error tone. |
| 15 | SLT Flash Drop | ON/OFF | OFF | In SLT, Pressing [FLASH] Key or Hook Flashing will drop the CO Call |

TABLE 3.2.1  Button Configuration  (PGM 111)

## 3.3	STATION ATTRIBUTE - II  (PGM 112)

PROCEDURE

| STATION ATT 2 ENTER STA RANGE | - [TRANS/PGM] + 112. |
| --- | --- |
| 100-110 STATION ATT 2 PRESS FLEX_KEY (01-16) | - Enter station range (Ex. 100-110). |
|  | - To program, use the BTN as TABLE 3.3.1. Press Flex. BTN 1-16 for setting each value. After pressing a Flex. BTN, the revised value can be set by entered digit. |
| 100-110 CO WARN TN (1 : ON/ 0 : OFF ) : OFF | Ex) When pressing Flex. BTN 1, ‘CO Warning Tone’ attribute value will be displayed. |
| 100-110 CO WARN TN (1 : ON/ 0 : OFF ) : ON | - User can change value by pressing digit 1. LCD shows the changed attribute value. |
| 100-110 CO WARN TN (1 : ON/ 0 : OFF ) : ON | - Press the [HOLD/SAVE] button for updating database permanently. |
| STATION ATT 2 PRESS FLEX_KEY (01-16) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | CO Warning Tone | ON/OFF | OFF | The allowance to receive warning tone in order to remind the call elapse time in case of outgoing CO line conversation. (PGM 180-BTN 22) |
| 2 | Automatic Hold | ON/OFF | OFF | While on a CO line, the station user seizes another CO line by depressing the {CO} button. The first CO line goes on hold automatically. (STA2: ON) |
| 3 | CO Call Time Restriction | ON/OFF | OFF | If this flag is set to YES, station's outgoing CO call may be disconnected when CO call restriction timer (PGM 180-BTN 17) expires. |
| 4 | Ind CO Line Access | ENABLE /DISABLE | ENABLE | The allowance to access individual CO line by dialing. |
| 5 | CO Line Queuing | ENABLE /DISABLE | ENABLE | The allowance of queuing for a busy CO/group of lines. |
| 6 | CO PGM | ENABLE /DISABLE | DISABLE | Determines that each station user can program CO button or not. |
| 7 | PLA | ENABLE /DISABLE | ENABLE | The allowance to answer calls by simply lifting handset or pressing the [MON] button with the answering priority. (PGM173) |
| 8 | Prepaid Call | ON/OFF | OFF | The allowance to use Prepaid CO Call feature. (Refer to PGM 180-BTN 16) |
| 9 | Speed Dial Access | ENABLE /DISABLE | ENABLE | Allows access to system speed dial by the station. |
| 10 | Two-way Record | ON/OFF | OFF | During incoming or outgoing Call, user can record the conversation. |
| 11 | Fax Mode | ON/OFF | OFF | In Fax mode, single ring is provided and Attendant recall is not operated. |
| 12 | Off-net Call Mode | EXT/ALL | ALL | ALL: Internal Off-net Call Fwd and External Off-net Call Fwd are allowed. EXT: External Off-net Call Fwd is only allowed |
| 13 | UCD Group Service | ON/OFF | OFF | When DID/DISA call destination is STA, ON: ring to UCD Grp which the station belongs to. OFF: ring to the station. |
| 14 | Ring Group Service | ON/OFF | OFF | When DID/DISA call destination is STA, ON: ring to Ring Grp which the station belongs to. OFF: ring to the station. |
| 15 | Stop Camp-on Tone | ON/OFF | OFF | Make Camp on Tone not to be heard. |
| 16 | Line Length | Short/Long/Far | Short | Line Lengh . (SAF only) |

TABLE 3.3.1  Button Configuration  (PGM 112)

## 3.4	STATION ATTRIBUTE - III  (PGM 113)

PROCEDURE

| STATION ATT 3 ENTER STA RANGE | - [TRANS/PGM] + 113. |
| --- | --- |
| 100-110 STATION ATT 3 PRESS FLEX_KEY (1-10) | - Enter station range (Ex. 100-110). |
|  | - To program, use the BTN as Table 3.4.1. Press flex BTN 1-10 for setting each value. After pressing one flex BTN, the revised values can be set by input digit. |
| 100-110 ADMIN (1 : EN/ 0 : DIS ) : DISABLE | Ex) Pressing Flex. BTN 1, ‘ADMIN’ attribute value will be displayed on the LCD. |
| 100-110 ADMIN (1 : EN/ 0 : DIS ) : ENABLE | - User can select value by pressing digit 1, LCD shows the changed attribute value. |
| 100-110 ADMIN (1 : EN/ 0 : DIS ) : ENABLE | - Press the [HOLD/SAVE] button for updating database permanently. |
| STATION ATT 3 PRESS FLEX KEY (1-10) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Admin | ENABLE /DISABLE | DISABLE | The allowance the station to program Admin Database. This feature is available at only DKTU. (STA 100 : Enable as default) |
| 2 | VMIB Access | ENABLE /DISABLE | DISABLE | The allowance to access Digital Voice Unit. |
| 3 | Group Listening | ENABLE /DISABLE | DISABLE | The allowance to use group listening (While you are talking on handset, by pressing the [MON] button, other people around you may hear the conversation through the speaker of the keyset.). |
| 4 | Override Privilege | ENABLE /DISABLE | DISABLE | The allowance to override CO line to gain access to the conversation. |
| 5 | SMDR Hidden Dialed Digits | ENABLE /DISABLE | DISABLE | The allowance to hide CO dialing number on SMDR printing. |
| 6 | Voice Over | ENABLE /DISABLE | DISABLE | The allowance to use Voice Over feature |
| 7 | Warm Line | HOT/WRM | WARM | This field is determined that Warm Line (OFF) or Hot Line (ON) in PGM 122. |
| 8 | DVU MSG Retrieve Password | ON/OFF | OFF | When retrieving DVU Message, User must enter password or not |
| 9 | DVU MSG Retrieve Date/Time | ON/OFF | ON | When retrieving DVU Message, Date and time will be heard or not |
| 10 | Alarm Attribute | Flex BTN 1 ON/OFF | OFF | Alarm MISB (LDK-300) Alarm MPB (LDK-100) |
|  |  | Flex BTN 2 ON/OFF | OFF | Alarm RAU 1 (LDK-300) Alarm MISB (LDK-100) |
|  |  | Flex BTN 3 ON/OFF | OFF | Alarm RAU 2 : LDK-300 Only |

TABLE 3.4.1  Button Configuration - III   (PGM 113)

## 3.5	ISDN STATION ATTRIBUTE  (PGM 114)

PROCEDURE

| STATION ATT 4 ENTER STA RANGE | - [TRANS/PGM] + 114. |
| --- | --- |
| 100-110 STATION ATT 4 PRESS FLEX_KEY (01-17) | - Enter station range (Ex. 100-110). |
|  | - To program, use the BTN as Table 3.5.1. Press Flex. BTN 1-17 for setting each value. After pressing a Flex. BTN, the revised value can be set by input digit. |
| 100-110 CLIP DISP (1 : ON / 0 : OFF ) : OFF | Ex) Pressing Flex. BTN 1, ‘CLIP LCD Display’ attribute value will be displayed. |
| 100-110 CLIP DISP (1 : ON / 0 : OFF ) : ON | User can change value by pressing digit 1, LCD shows the changed attribute value. |
| 100-110 CLIP DISP (1 : ON / 0 : OFF ) : ON | - Press the [HOLD/SAVE] button for updating database permanently. |
| STATION ATT 4 PRESS FLEX_KEY (01-17) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | - CLIP LCD Display | ON/OFF | ON | This field is determined that a station displays CLIP or not. |
| 2 | - COLP LCD Display | ON/OFF | OFF | This field is determined that a station displays COLP or not. |
| 3 | - CLI / Redirect Display | RED/CLI | CLI | To select original CLI or redirected CLI. ON: Original CLI, OFF: Redirected CLI |
| 4 | - CLI MSG Wait | ON/OFF | OFF | - This field is determined that a station receives CO message wait or not. ON:YES, OFF:NO |
| 5 | - EXT or CO ATD | ATD/EXT | EXT | - To select EXT(extension number) or CO ATD to make outgoing CLI or COLP information |
| 6 | - Keypad Facility | KEYPAD /DTMF | DTMF | - This field determines that ISDN station sends digit in DTMF or keypad facility after connected. |
| 7 | Long/Short | LONG /SHORT | SHORT | This field determines that ISDN station acts in Short passive mode or not. |
| BTN | ITEM | RANGE | DEFAULT | REMARK |
| 8 | CPN Type | 0-2 | 0 | This field indicates how the CPN IE is filled in SETUP message. 0: Do not send CPN (Called Party Number) to S0. In this case, all S0 STAs of the S port will be ringing. 1: Send station number as CPN 2: Bypass CPN from the network. (In the case of 1 & 2, only one specific STA will be ringing) |
| 9 | S0 Sub Address | 0-2 | 0 | This field indicates how the sub-address used in SETUP message. 0: Station sub-address not used. 1: Sub-address is filled in the CPN field of SETUP message. 2: Sub-address is filled in the CPSN (Called Party Sub-address Number) field of SETUP. |
| 10 | Reserved | - | - | - |
| 11 | CLI Name Display | ON/OFF | OFF | If this field is ON, the system checks whether the received CLI is matched with the speed dial data or not. If it is matched, the speed dial name is displayed. |
| 12 | ISDN CLI STA | Max. 4 digit | Logical STA No. | If outgoing CLI is active and CLI type is EXT, this field used when making outgoing CLI. |
| 13 | Progress Indication | ON/OFF | OFF | If this field ON and a SLT seizes an ISDN line, the progress indication IE indicating the originator is non-ISDN device is made in SETUP message. |
| 14 | ISDN CLIR | ON/OFF | OFF | If this field is ON, the system will not send CLI information and restrict PX send it. |
| 15 | ISDN COLR | ON/OFF | OFF | If this field is ON, the system will not send COLI information and restrict PX send it. |
| 16 | DID Restriction | ON/OFF | OFF | Restrict the DID Call. |
| 17 | DID Call Wait | ON/OFF | OFF | New DID call waiting indicate. |

TABLE 3.5.1  Button Configuration for ISDN Station Attributes  (PGM 114)

## 3.6	FLEX BUTTON ASSIGNMENT  (PGM 115)

Each Flex. Button in a key station can be assigned identified as one of the followings;

| No. | Type | RANGE |  | REMARK |
| --- | --- | --- | --- | --- |
|  |  | LDK-300 | LDK-100 |  |
| 1 | User Button | - | - | User can program by button programming procedure. (empty) |
| 2 | {CO xx} Button | 001 – 200 | 01 – 40 | CO Line |
| 3 | {CO Grp xx} | 01 – 72 | 01 – 24 | CO Group |
| 4 | {LOOP} |  |  |  |
| 5 | {STAxxxx} | 100 – 399 | 100 – 227 | Station No. |
| 6 | STA PGM Button | 11 – 99 | 11 – 99 |  |
| 7 | {STA SPDxx} | 00 – 99 | 00 – 99 | Station Speed Bin |
| 8 | {SYS SPDxxxx} | 2000 –4999 | 2000 –3499 | System Speed Bin |
| 9 | Num Pln Button | Num Plan Code | Num Plan Code |  |
| 10 | Net DSS Button |  |  | When using Networking feature |

TABLE 3.6.1  Button Type for Flexible Button Assignment  (PGM 115)

PROCEDURE

| FLEX BUTTON ASSIGN ENTER STA RANGE | - [TRANS/PGM] + 115. |
| --- | --- |
| 100-110 BTN ASSIGN PRESS FLEX_KEY (01-44) | - Enter station range (Ex.100 -110). |
| 100-110 DIAL 1 – 10 BTN 01 = CO 001 | - To program Flexible Button, press the Flex. BTN which you want to program (Ex. BTN 1). The LCD will display current BTN assignment of the first station in the range, and LED of the BTN (Ex. BTN 1) will be lit. (See Table 3.6.3). And input one digit to choose type. If needed, enter data, then entered data will be displayed on the LCD. (See Table 3.6.3) |
| 100-110 BTN ASSIGN PRESS FLEX_KEY(01-44) | - Press the [HOLD/SAVE] button for updating database permanently. |
| 100-110 BTN ASSIGN PRESS FLEX_KEY(01-44) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (2) without updating system memory. |

| BTN | 12-Button (Digital) | 24-Button (Digital) |
| --- | --- | --- |
| 1 | {CO 1} | {CO 1} |
| 2 | {CO 2} | {CO 2} |
| 3 | {CO 3} | {CO 3} |
| 4 | {CO 4} | {CO 4} |
| 5 | {CO 5} | {CO 5} |
| 6 | {CO 6} | {CO 6} |
| 7 | {CO 7} | {CO 7} |
| 8 | {CO 8} | {CO 8} |
| 9 | {CO 9} | {CO 9} |
| 10 | {CO 10} | {CO 10} |
| 11 | {CO 11} | {CO 11} |
| 12 | {LOOP} | {LOOP} |
| 13 - 24 |  | - |

TABLE 3.6.2  Initial Button Configuration  (PGM 115)

|  | BTN | TYPE | DATA |
| --- | --- | --- | --- |
| Button 01 - 24 | 1 - 24 | 1 : User Button 2 : CO 3 : CO GRP 4 : LOOP 5 : STA … 6 : STA PGM (11-99) 7 : SPD (00-99) 8 : SYS SPD 9 : Num Plan Code | - CO Line CO Group - Station No. Station Programming Code Speed Bin No. System Speed Bin No. Num Plan Code |

TABLE 3.6.3   Button Configuration for Flexible Button Assignments (PGM 115)

## 3.7	STATION COS  (PGM 116)

All stations’ COS for day and night operation are 1 as default. For a particular call, the CO COS is combined with station COS to determine restriction. Each station must be assigned a class of service which governs that station's toll restriction for the day and night operation. The weekend COS is same as night COS. (Table 3.7.1 and Table 3.7.2)

| STA COS 1 | No restrictions are placed at the station for dialing. |
| --- | --- |
| STA COS 2 | The assignments in the Exception Table A are monitored for allow and deny numbers. |
| STA COS 3 | The assignments in the Exception Table B are monitored for allow and deny numbers. |
| STA COS 4 | The assignments in both Exception Tables A & B are monitored for allow and deny numbers. |
| STA COS 5 | The leading digit dialed can not be a long distance code. The dialed digits can be longer than 7 digits. There is no restriction for the number in Canned Toll Table. |
| STA COS 6 | The leading digits can not be a Long Distance code. Only eight digits maximum can be dialed. There is no restriction for the number in Canned Toll Table. |
| STA COS 7 | Intercom and paging calls are allowed. No dialing allowed on CO lines. ICM boxes are assigned with this COS. |

TABLE 3.7.1  Station Class-Of-Service  (PGM 116)

|  | CO COS 1 | CO COS 2 | CO COS 3 | CO COS 4 | CO COS 5 |
| --- | --- | --- | --- | --- | --- |
| STA COS 1 | No Restriction Applied | No Restriction Applied | No Restriction Applied | Restricts Long Distance code. Only within 7 digits. Possible to dial the number in Canned Toll Table. | No restriction applied |
| STA COS 2 | Exception Table A governs the dialing | Exception Table A governs the dialing | No Restriction Applied | Restricts Long Distance code. Only within 7 digits. Possible to dial the number in Canned Toll Table. | No restriction applied |
| STA COS 3 | Exception Table B governs the dialing | No Restriction Applied | Exception Table B governs the dialing | Restricts Long Distance code. Only within 7 digits. Possible to dial the number in Canned Toll Table. | No restriction applied |
| STA COS 4 | Exception Table A&B governs the dialing | Exception Table A governs the dialing | Exception Table B governs the dialing | Restricts Long Distance code. Only within 7 digits. Possible to dial the number in Canned Toll Table. | No restriction applied |
| STA COS 5 | Restricts Long Distance code. | Restricts Long Distance code. | Restricts Long Distance code. | Restricts Long Distance code. Only within 7 digits. | No restriction applied |
| STA COS 6 | Restricts Long Distance code. Only within 7 digits. Possible to dial the number in Canned Toll Table. | Restricts Long Distance code. Only within 7 digits. Possible to dial the number in Canned Toll Table. | Restricts Long Distance code. Only within 7 digits. Possible to dial the number in Canned Toll Table. | Restricts Long Distance code. Only within 7 digits. Possible to dial the number in Canned Toll Table. | No restriction applied |
| STA COS 7 | In-house dialing only | In-house dialing only | In-house dialing only | In-house dialing only | In-house dialing only |

TABLE 3.7.2  Toll Checking Table  (PGM 116)

PROCEDURE

| STATION COS ENTER STA RANGE | - [TRANS/PGM] + 116. |
| --- | --- |
| 100-110 STATION COS DAY=1 NIGHT=1 | - Enter station range (Ex. 100-110). |
|  | - To program, use the BTNs as Table 3.7.3. To change the COS for day operation, press Flex. BTN 1 and dial COS (1 digit) and to change the COS for night operation, press Flex. BTN 2 and dial COS (1 digit). Then changed COS will be displayed on the LCD. (Ex. day = 5, night =3) |
| 100-110 STATION COS DAY=5 NIGHT=3 | - Press the [HOLD/SAVE] button for updating database permanently. |
| STATION COS ENTER STA RANGE | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | DEFAULT | RANGE | REMARK |
| --- | --- | --- | --- |
| 1 | 1 | 1 – 7 | Day Class-Of-Service |
| 2 | 1 | 1 – 7 | Night / Weekend Class-Of-Service |

TABLE 3.7.3  Button Configuration for Station COS  (PGM 116)

## 3.8	CO LINE GROUP ACCESS  (PGM 117)

If you want to change CO line group access of some stations, program this item. Default gives to access CO Line Group 1 for all stations.

PROCEDURE

| CO GROUP ACCESS ENTER STA RANGE | - [TRANS/PGM] + 117. |
| --- | --- |
| SELECT CO GROUP RANGE PRESS FLEX_KEY (1-3) | - Enter station range (Ex 100-110). In case of LDK-300 press the Flex. BTN 1~3 to program access authority of the CO line Group. |
|  | - LEDs of BTN 01-24 show current CO line group 01-24 access of the first station in the range. To program CO line group 01-24 access authority, press BTN 01-24 for toggle setting at the CO line group 01-24 Access Programming mode. (LED ON: Station can access the CO line group. / LED OFF: Station cannot access the CO line group.) |
| 100-110 CO GRP (01-24) PRESS FLEX_KEY | - Press the [HOLD/SAVE] button for updating database permanently. |
|  | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | DEFAULT | RANGE | REMARK |
| --- | --- | --- | --- |
| 1 | - | 1-24 | CO line Group 01-24 (Toggle) |
| 2 | - | 1-24 | CO line Group 25-48 (Toggle) : LDK-300 Only |
| 3 | - | 1-24 | CO line Group 49-72 (Toggle) : LDK-300 Only |

TABLE 3.8.1  CO Line Group Access  (PGM 117)

## 3.9	INTERNAL PAGE ZONE  (PGM 118)

Each station is assigned to internal paging zone. A station can be in any or no zone at all. Internal all call is defined as the sum of all zones. If station is not in any internal zone, it will not receive any internal page announcement. In LDK-300(100) system supports 30(10) internal paging zones.

Default assigns all stations to Internal Page Zone 1.

PROCEDURE

| INTERNAL PAGE ZONE ENTER STA RANGE | - [TRANS/PGM] + 118. |
| --- | --- |
| SELECT PAGE ZONE RANGE F1: 1-24 F2: 25-30 | - Enter station range (Ex 100-110). In case of LDK-300 press the Flex. BTN 1~2 to program access authority of the Internal Page Group. Suppose BTN 1 pressed. |
| 100-110 (ZONE 01-24) PRESS FLEX_KEY (01-24) | - LEDs of BTNs show currently assigned page zones of the first station in range. To assign Internal page zone to the stations, press one of BTNs for toggle setting. Each button means Internal Page Zone 01-24. (LED ON: Stations are in the internal page zones. LED OFF: Stations are not in the internal page zones.) |
| 100-110 (ZONE 01-24) PRESS FLEX_KEY (01-24) | - Press the [HOLD/SAVE] button for updating database permanently. |
|  | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | DEFAULT | RANGE | REMARK |
| --- | --- | --- | --- |
| 1 | 1 | 1 - 24 (LDK-300) 1 - 10 (LDK-100) | Internal Page Zone 01 – 24(10) (Toggle) |
| 2 | - | 1 - 6 | Internal Page Zone 25 - 30 (Toggle) : LDK-300 Only |

TABLE 3.9.1  Internal Page Zone Access  (PGM 118)

## 3.10	CONFERENCE PAGE ZONE  (PGM 119)

Each station is assigned to conference paging zone. A station can be in any or no zone at all. LDK-300 and LDK-100 support 5 conference paging zones.

Default assigns all stations to None.

PROCEDURE

| CONFERENCE PAGE ZONE ENTER STA RANGE | - [TRANS/PGM] + 119. |
| --- | --- |
| 100 - 110 ( ZONE 31 - 35 ) PRESS FLEX_KEY (1 – 5) | - Enter station range (Ex 100-110). LEDs of BTNs show currently assigned page zones of the first station in range. In LDK-100, ZONE 11-15 will be displayed instead of ZONE 31-35. |
|  | - To assign Conference Page zone to the stations, press one of BTNs for toggle setting. Each button means Conference Page Zone 1-5. (LED ON: Stations are in the conference page zones. LED OFF: Stations are not in the conference page zones.) |
| CONFERENCE PAGE ZONE ENTER STA RANGE | - Press the [HOLD/SAVE] button for updating database permanently. |
| CONFERENCE PAGE ZONE ENTER STA RANGE | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | DEFAULT | RANGE | REMARK |
| --- | --- | --- | --- |
|  | - | 1-5 | Conference Page Zone (Toggle) |

TABLE 3.10.1  Conference Page Zone Access  (PGM 119)

## 3.11	ICM TENANCY GROUP  (PGM 120)

A station belongs to only one ICM tenancy group. A station in ICM Tenancy Group (A) can call another station in other ICM Tenancy Group (B) if the station in the ICM Tenancy Group (A) is programmed to be allowed to access ICM Tenancy Group (B).

LDK-300(100) system supports 15(5) ICM Tenancy Groups and Tenancy ATDs.

PROCEDURE

| ICM TENANCY GROUP ENTER GRP NUMBER(01-15) | - [TRANS/PGM] + 120. In LDK-100, the range will be 1-5. |
| --- | --- |
| ICM TENANCY GRP 01 F1: ATD F2:ACCESS | - Enter the group number (Ex. 01). |
|  | - To program ICM Tenancy Group, use the buttons as Table 3.11.1. |
| ICM TENANCY GRP 01 ATD : …. | - To assign attendant station of the ICM tenancy group, press BTN 1 and enter the station number to be assigned as attendant, then assigned attendant station number will be displayed on the LCD. |
| ICM TENANCY GRP 01 ENTER ACCESS GROUP(1-15) | - To assign accessible ICM tenancy groups for the group, press BTN 2. Then the LCD will show the status and LEDs of BTNs show current accessible ICM tenancy groups. In LDK-100, the range is 1-5. To change ICM tenancy groups access, press the Flex. BTN for toggle setting. (LED ON: stations have the authority to access the ICM tenancy group. LED OFF: stations have not the authority to access the ICM tenancy group.). Pressing the [CONF] button, the system will goes to step (2) without updating database. |
| ICM TENANCY GRP 01 F1: ATD F2:ACCESS | - Press the [HOLD/SAVE] button for updating database permanently. |
| ICM TENANCY GRP 01 F1: ATD F2:ACCESS | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (2) without updating system memory. |

| BTN | DEFAULT | RANGE | REMARK |
| --- | --- | --- | --- |
| 1 | - | STA No. | Attendant station of assigned ICM tenancy group |
| 2 | GROUP 01 | BTN 01-15(LDK-300) BTN 1-5 (LDK-100) | ICM tenancy groups allow to access for assigned group |

TABLE 3.11.1  Button Configuration of ICM Tenancy Group  (PGM 120)

## 3.12	PRESET CALL FORWARD  (PGM 121)

A station can be programmed so that incoming CO lines can be forwarded to a preset station or station group if the first station(or station group) does not answer after Preset Call Forward timer.

No station is assigned as default.

PROCEDURE

| CALL FWD PRESET ENTER STA NUMBER | - [TRANS/PGM] + 121. |
| --- | --- |
| (1:STN /2:HUNT GRP) FROM 101 TO .... | - Enter the station number to which you want to forward (Ex.101). LCD shows current status of the station. Dial 1 if you want to forward the call to station, or dial 2 if want to forward to hunt group. (Ex. dial 2.) |
| ENTER FWD HUNT GRP NO. FROM 101 TO HUNT .... | - Enter the preset hunt group number which first station forward to. (Ex.620) |
| CALL FWD PRESET FROM 101 TO HUNT 620 | - To clear the Preset Call Forward station, press the [SPEED] button. |
| CALL FWD PRESET ENTER STA NUMBER | - Press the [HOLD/SAVE] button for updating database permanently. |
| CALL FWD PRESET ENTER STA NUMBER | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

## 3.13	HOT LINE / WARM LINE  (PGM 122)

This feature lets a station perform a pre-assigned feature as soon as lifting handset or pressing the [MON] button as if a station selects the feature (Hot Line). On the other hand, Idle Line Selection for a station which is assigned to warm line, is activated when takes no action for Warm Line Timer after lifting handset or pressing the [MON] button (Warm Line). Warm line is programmable at PGM 113.

By default, all stations are not assigned any Idle Line Selection.

PROCEDURE

| IDLE LINE SELECTION ENTER STA RANGE | - [TRANS/PGM] + 122. |
| --- | --- |
| 100-110 IDLE LINE NOT ASSIGNED | - Enter station range (Ex.100-110). User can enter one digit (1-4) and LCD shows the default value of idle line selection. |
|  | - To program Idle Line Selection, use the Table 3.13.1. To assign Idle Line Selection, dial one digit (1-4) and enter related data. Then selected value and the related data will be displayed on LCD. Otherwise, to delete any Idle Line Selection, press the [SPEED] button, then Idle Line Selection assignment will be deleted. |
| IDLE LINE SELECTION ENTER STA RANGE | - Press the [HOLD/SAVE] button for updating database permanently. |
| IDLE LINE SELECTION ENTER STA RANGE | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| DGT | ITEM | RANGE | REMARK |
| --- | --- | --- | --- |
| 1 | Flex. BTN | 01 - 44 | To activate a feature on a flex button as if pressed. |
| 2 | CO Line | 001 – 200 (LDK-300) 01 – 40 (LDK-100) | To seize a CO Line |
| 3 | CO Group | 01 – 72 (LDK-300) 01 – 24 (LDK-100) | To seize a CO Line Group |
| 4 | Station | 100 – 399 (LDK-300) 100 – 227 (LDK-100) | To call an another station |

TABLE 3.13.1  Button Configuration for Hot Line/Warm Line (PGM 122)

## 3.14	CTI STATION ATTRIBUTE  (PGM 123)

This sets the features/modes used when CTIU8/30 or CTI module is connected at a key station. User can use a CTI keyset with PC when the CTI station mode is set to CTI mode. (Please refer to the User’s Guide of TAPI-NT.)

PROCEDURE

| CTI STATION ATT ENTER STA RANGE | - [TRANS/PGM] + 123. |
| --- | --- |
| CTI STATION ATT PRESS FLEX_KEY (1-2) | - Enter station range (Ex.100 - 110). User can select Flex. BTN 1or 2. |
| 100-110 CTI MODE (0-2) CTI MODE (2) | - To program, use the Flex BTNs as Table 3.14.1. To program the CTI station's mode, press Flex. Button 1 and dial CTI mode 0-2. By default, CTI station's mode is CTI mode (1). |
| 100-110 STA BAUD(0-2) 1200 (1) | - To program the CTI station's baud rate, press Flex. Button 2 and dial baud rate 0-2. By default, CTI station's baud rate is 1200 (0). |
| 100-110 STA BAUD(0-2) 1200 (1) | - Press the [HOLD/SAVE] button for updating database permanently. |
| CTI STATION ATT PRESS FLEX_KEY (1-2) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (2) without updating system memory. |

| BTN | ITEM | DEFAULT | RANGE | REMARK |
| --- | --- | --- | --- | --- |
| 1 | CTI Station Mode | 1 | 0-2 | Determines the CTI keyset mode 0: Inactive, 1: CTI Mode, 2: At Mode |
| 2 | CTI Station's Baud Rate | 0 | 0-2 | Determines the baud rate of the CTI keyset 0: 1200, 1: 2400, 2: 4800 |

TABLE 3.14.1  Button Configuration for CTI Station Attribute  (PGM 123)

## 3.15	SMDR ACCOUNT GROUP  (PGM 124)

Stations can be assigned as member of call account group on SMDR. A station belongs to only one call account group. In LDK-300(100), system supports 99(23) SMDR Account Groups.

All stations are not assigned as member of any Call Account Group by default.

PROCEDURE

| SMDR ACCOUNT GROUP ENTER STA RANGE | - [TRANS/PGM] + 124. |
| --- | --- |
| 100-110 SMDR ACCT GRP (00 – 99) : 01 | - Enter station range (Ex.100-110). LCD shows current account group status of the first station in the range. In LDK-100, the range is 00-23. |
|  | - To assign Call Account Group: enter group number, then assigned Call Account Group Number will be displayed on the LCD. Otherwise, to cancel assignment of Call Account Group, press the [SPEED] button then it will be displayed on the LCD. |
| SMDR ACCOUNT GROUP ENTER STA RANGE | - Press the [HOLD/SAVE] button for updating database permanently. |
| SMDR ACCOUNT GROUP ENTER STA RANGE | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

# 4.	CO LINE PROGRAMMING

If CO line features are to be changed, press the [TRANS/PGM] button and dial 140-144 in Admin Programming Mode. When programming, LCD and LEDs indicate current programmed data and programming status. If the programmer enters correct data, then LCD and LEDs show the entered data and the data is stored in the temporary buffer area. To save the data permanently, press the [HOLD/SAVE] button, then all data in the temporary buffer (same as LCD/LEDs show their status) are stored into system memory.

## 4.1	CO SERVICE TYPE  (PGM 140)

In this program mode, you can program the following items;

PROCEDURE

| COL SERVICE ATT ENTER COL RANGE |  | - [TRANS/PGM] + 140. |
| --- | --- | --- |
| 001-002 COL SVC F1:TYPE F2:SUB ATT |  | - (2) Enter CO line range. User can select COL Service Type by pressing Flex. BTN 1. |
| 001-002 SVC TYPE (1-5) NORMAL CO (1) |  | - To program, select the desired type as Table 4.1.2. Press digit 1-5 to select CO service type (Type 1-5 is exclusive and the default value is NORMAL CO.). |
| 001-002 SVC TYPE (1-5) ISDN DID / MSN (3) |  | - To change the type, press another digit (Ex. 3), then LCD displays the select type immediately. To save the current type, press the [HOLD/SAVE] button. |
| 001-002 COL SVC PRESS FLEX_KEY (1-2) |  | - To set the sub-attributes of the selected type, press Flex. BTN 2. See Table 4.1.2. |
| 001-002 DISA ATT F1:DAY F2:NIGHT F3: WEND |  | - In Normal CO type, user can set the 3 sub-attributes; Day, Night, or Weekend. Each attribute also has 3 sub attributes. At first, choose a Flex. BTN. (Day/Night/Weekend: 1, 2, 3) See Table 4.1.2. |
| 001-002 DISA ATT F1: SVC F2: VMIB |  | - User can set the DISA attributes by pressing Flex. BTN 1~2. Do this procedure for other attribute. See Table 4.1.2. |
| 001-002 DISA SVC (1 : ON/ 0 : OFF) : OFF |  | - Press Flex. BTN 1 to set DISA Service ON/OFF. After entering the desired value, press the [HOLD/SAVE] button to save the value. |
| 001-002 VMIB ANNC VMIB MSG .. (00-70) |  | - Press Flex. BTN 2 to select VMIB Announcement No. - When VMIB Announcement number is assigned and CCR table (PGM 228) matched with VMIB Announcement No is programmed, CCR feature is activated. |
| COL SERVICE ATT ENTER COL RANGE |  | - Press the [HOLD/SAVE] button for updating database permanently. |
| COL SERVICE ATT ENTER COL RANGE |  | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |
| ITEM | REMARK |  |
| Normal CO | All lines are assigned as normal CO lines as default. Each CO line in the system can be programmed as DISA (Direct Inward System Access) line and the DISA types are as follows; - Flex BTN 1 (Day) / 2 (Night) / 3 (Weekend) - Each DISA type (BTN 1-3) has sub-attribute F1: DISA Service On/Off F2: VMIB Message No. (Voice announcement (VMIB Message) can be assigned (00-70 and it is not assigned (00) as default.) |  |
| Analog DID | Each CO line in the system can be programmed as DID (Direct Inward Dialing) line and the DID types are as follows; - 1(Immediate Start) / 2 (Wink Start) / 3 (Delayed Dial Start) (BTN 1-3 are exclusive) |  |
| ISDN DID/ MSN | If ISDN board (BRIB, PRIB) is assigned for operating with DID type. |  |
| TIE | TIE line types are as follows; - / 1 (RD) / 2 (LD) / 3 (EM-C) / 4 (EM-D) / 5 (EM-I) |  |
| DCO DID | If R2DCOB (E1 R2) board is assigned for operating with DID type. |  |

TABLE 4.1.1  CO Service Type  (PGM 140)

| BTN | TYPE | FLEX BTN 2 | DEFAULT | SUB ATTR | REMARK |
| --- | --- | --- | --- | --- | --- |
| 1 | Normal CO | DISA Attributes -Flex BTN 1 (Day) -Flex BTN 2 (Night) -Flex BTN 3 (Weekend) |  | For each Item; Flex BTN 1 - DISA Service: ON/OFF Flex BTN 2 – VMIB MSG (00- 70) (00: not assigned) |  |
| 2 | ANALOG DID | Flex BTN 1 : (Signal Type) Flex BTN 2 : (INFO NO) | - | Signal Type –1: Immediate Start 2: Wink Start 3: Delayed Dial Start INFO NO : 00 – 70 (00: not assigned) |  |
| 3 | ISDN DID / MSN |  |  |  |  |
| 4 | TIE | 1:RD 2:LD 3:EM-C 4:EM-D 5:EM-I | RD | - | Enter digit |
| 5 | DCO DID |  |  |  |  |

TABLE 4.1.2  Button Configuration for CO Service Type  (PGM 140)

## 4.2	CO LINE ATTRIBUTE – I  (PGM 141)

PROCEDURE

| CO LINE ATT 1 ENTER COL RANGE | - [TRANS/PGM] + 141. |
| --- | --- |
| 001-002 CO LINE ATT1 PRESS FLEX_KEY (1-10) | - Enter CO line range. |
|  | - To program, press the Flex. BTN as Table 4.2.1. Press BTN 1-10 and enter related data, then entered data will be displayed on the LCD. |
| 001-002 DISA ACCT CODE (1 : ON/ 0 : OFF) : OFF | - Press Flex. BTN 3 and assign DISA Account Code. |
| 001-002 FLASH TYPE (1: GND/ 0: LOOP) : LOOP | - To set Flash Type, press Flex. BTN 7. |
| 001-002 FLASH TYPE (1: GND/ 0: LOOP) : LOOP | - To alter Flash Type, dial 1-0 .(Ex. dial 1) |
| 001-002 DISA ACCT CODE (1 : ON/ 0 : OFF) : OFF | - Press the [HOLD/SAVE] button for updating database permanently. |
| CO LINE ATT 1 ENTER COL RANGE | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | CO Line Group | 00-73 (LDK-300) 00-25 (LDK-100) | 01 | Groups should be assigned according to CO type and Class-Of-Service. (00:private, 73(25) : not used) |
| 2 | CO COS | 1-5 | 1 | -CO COS 1: no restriction -CO COS 2: Exception Table A governs -CO COS 3: Exception Table B governs -CO COS 4: restricts Long Distance Code -CO COS 5: overrides STA. COS 2,3,4 and 5, 6. |
| 3 | DISA Account Code | ON/OFF | OFF | When accessed another CO line in the system by DISA line, you should enter authorization code if this flag is set. |
| 4 | CO Line Assign | POL/LOOP | LOOP | Polarity Reverse, Loop Start |
| 5 | CO Line Type | PBX/CO | CO | When marked PBX, a 1 or 2 digit dial code may be entered after which toll restriction is applied. |
| 6 | CO Line Signal Type | DTMF/PULSE | DTMF | DTMF, Pulse |
| 7 | Flash Type | GROUND/LOOP | LOOP |  |
| 8 | UNA | ON/OFF | OFF | The allowance of Universal Night Answer service |
| 9 | CO Line Group Account | ON/OFF | OFF |  |
| 10 | CO Tenancy Group | 01-15(LDK-300) 01-05(LDK-100) | 01 | Tenancy Group of CO line. |

TABLE 4.2.1  Button Configuration for CO Attribute - I (PGM 141)

## 4.3	CO LINE ATTRIBUTE – II  (PGM 142)

PROCEDURE

| CO LINE ATT 2 ENTER COL RANGE | (1) [TRANS/PGM] + 142. |
| --- | --- |
| CO LINE ATT 2 PRESS FLEX_KEY (01-14) | - Enter CO line range. |
|  | - To program, use the Flex. BTNs as Table 4.3.1. Flex. BTN 1, 4, 7-11,14 may set On/Off. Press BTN 2, 3, 5, 6, 12, 13 and enter related data for setting, then entered data will be displayed on the LCD. |
| 001-002 CO DIST RING (0-4) : 3 | - To assign CO Distinct Ring, press Flex. BTN 5. (Ex. To change Ring Type, dial 0-4. (Ex: dial 3)) |
| 001-002 CO DIST RING (0-4) : 3 | - Press the [HOLD/SAVE] button for updating database permanently. |
| 001-002 MOH(00-14) INT MUSIC (1) | - To change MOH type, press Flex. BTN 6, and dial 00-13. In LDK-100, the range is 00-12 |
| 001-002 MOH(00-14) INT MUSIC (1) | - Press the [HOLD/SAVE] button for updating database permanently. |
| CO LINE ATT 2 PRESS FLEX_KEY (01-14) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (2) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | CO Line Name Display | ON/OFF | OFF | If CO line name is assigned at BTN2, and this field is ON, CO name is displayed when a CO line call is received. |
| 2 | CO Line Name Assign | Max 12 char | - | Max 12 characters |
| 3 | Metering Unit | 00-06 | 00 | There are 7 metering signal types: - 0 : None - 1 : 50 Hz - 2 : 12 KHz - 3 : 16 KHz - 4 : Singular Polarity Reverse (SPR) - 5 : Plural Polarity Reverse (PPR) - 6 : No Polarity Reverse (NPR) |
| 4 | Line Drop using CPT | ON/OFF | OFF | If this field set to ON, CPT checks the incoming CO line when answered and if CPT detects dial tone, then system drops the line for toll restriction. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 5 | CO Distinct Ring | 0-4 | 0 | The CO line can give his own ring type signal to station in system through this field. This ring type can be programmed at PGM 422. |
| 6 | CO Line MOH | 0-13 (LDK-300) 0-12 (LDK-100) | 1 | - 0: Not assigned by this field. - 1: Internal Music - 2~4: External Music - 5~6(7): VMIB MOH - 7(8)-11(12): SLT MOH - 12(13): Hold Tone |
| 7 | PABX CO Dial Tone | YES/NO | YES | - YES: PX or PABX provides dial tone. - NO: PX or PABX does not provide dial tone and the LDK system provides dial tone. |
| 8 | PABX CO Ring Back Tone | YES/NO | NO | - If R2 PX which does not give you tone for called party status exists, then the system provides tone according to cause value (This field is only when Cause means that Ring back is provided by PX.). - YES: PX, NO: System |
| 9 | PABX CO Error Tone | YES/NO | NO | - If R2 PX which does not give you tone for called party status exists, then the system provides tone according to cause value (This field is only when Cause means that error tone is provided by PX.). - YES: PX, NO: System |
| 10 | PABX CO Busy Tone | YES/NO | NO | - If R2 PX which does not give you tone for called party status exists, then the system provides tone according to cause value (This field is only when Cause means that busy tone is provided by PX.). - YES: PX, NO: System |
| 11 | PABX CO Announce Tone | YES/NO | NO | - If R2 PX which does not give you tone for called party status exists, then the system provides tone according to cause value (This field is only when Cause means that announcement is provided by PX, but the system provides only error tone.). - YES: PX, NO: System |
| 12 | CO Flash Timer | 000 – 300 | 005 | 10msec base |
| 13 | Open Loop Detect Timer | 00 - 20 | 00 | 100msec base |
| 14 | Line Length | LONG/SHORT | SHORT | Line Length of CO.(SAF only.) |

- TABLE 4.3.1  Button Configuration for CO Attribute - II  (PGM 142)

## 4.4	ISDN CO LINE ATTRIBUTE  (PGM 143)

PROCEDURE

| COL ISDN ATT ENTER CO RANGE | - [TRANS/PGM] + 143. |
| --- | --- |
| 001-009 COL ISDN ATT PRESS FLEX KEY (01-07) | - Enter CO line range. |
|  | - To program, press the Flex. BTN as Table 4.4.1. BTN 6 can toggle for the value. Press other BTNs and enter related data for setting, then it will be displayed entered data on the LCD. |
| 001-009 DID REMOVE NO (00-99) : 00 | - To set DID Remove Number, press Flex. BTN 5. (Ex. To alter DID_RN dial number, dial 21.) |
| 001-009 DID REMOVE NO (00-99) : 21 | - Press the [HOLD/SAVE] button for updating database permanently. |
| 001-009 COL ISDN ATT PRESS FLEX KEY(01-07) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (2) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | COLP Table Index | 00 – 50 | Not Assigned | To make called party number with assigned COLP Table entry. (PGM 201) 00~49: PGM 201 Bin No. / 50: PGM 114-BTN 5 |
| 2 | CLIP Table Index | 00 – 50 | Not Assigned | To make calling party number with assigned CLIP Table entry. (PGM 201) 00~49: PGM 201 Bin No. / 50: PGM 114-BTN 5 |
| 3 | Call Type | 0 – 4 | 2 | - 0: Unknown - 1: International - 2: National - 3: Not used - 4: Subscriber |
| 4 | DID CONV Type | 0 – 2 | 0 | - 0: convert digits by DID Dgt Conversion (PGM230) - 1: call to the valid extension. - 2:convert digits by Flex DID Table (PGM231) |
| 5 | DID Remove No. | 00 – 99 | Not Assigned | - Remove received digits from the left as to the assigned # |
| 6 | ISDN Enblock Send | ON/OFF | OFF | - ON: Enblock Sending Mode - OFF: Overlap Sending Mode |
| 7 | CLI Transit | ORI(1)/CFW(0) | CFW(0) | ORI : Send CLI as the originate caller’s CLI. CFW : Send CLI as the call forwarded station’s CLI. |

- TABLE 4.4.1 Button Configuration for ISDN CO Attribute  (PGM 143)

## 4.5	CO RING ASSIGNMENT  (PGM 144)

PROCEDURE

| CO RING ASSIGNMENT ENTER COL RANGE | - [TRANS/PGM] + 144. |
| --- | --- |
| 001-002 PRESS KEY DAY NIGHT WEEK ON-D | - Enter CO line range. User can select one of 4 Flex. BTNs. |
| 001-002 DAY CO RING DIAL TYPE (1:S/2:H/3:D) | - For the selected one of 4 modes (day, night, weekend, on-d), enter one digit (1-3) to select a type. When entering a digit, menu moves to the selected type setting. |
| 001-002 ENTER STA RANGE STA :100-150 DLY : . . | - In case of DEST TYPE 1, first enter station range. If the entered range is valid, LCD shows the value and system moves delay input state with confirmation tone. |
| 001-002 ENTER STA RANGE STA : 100-150 DLY : 9 | - After entering station range, enter desired value of delay. To save the station range and delay value, press the [HOLD/SAVE] button. |
| 001-002 DAY CO RING HUNT GRP : . . . | - In case of DEST TYPE 2, enter desired value of hunt group. To save the value, press the [HOLD/SAVE] button |
| 001-002 NIGHT CO RING VMIB MESSAGE 50 | - In case of DEST TYPE 3, Enter desired value of voice message. (Ex. 50) To save the value, press the [HOLD/SAVE] button. |
| 001-002 NIGHT CO RING VMIB MESSAGE 50(#) | - To drop the call after VMIB message, press “#’ after entering voice message number. |
| 001-002 PRESS KEY DAY NIGHT WEEK ON-D | - Press the [HOLD/SAVE] button for updating database permanently. |
| 001-002 PRESS KEY DAY NIGHT WEEK ON-D | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (2) without updating system memory. |

| BTN | ITEM | DEST TYPE | RANGE | DEFAULT |
| --- | --- | --- | --- | --- |
| 1 | Day | TYPE 1 : STA Range + Delay TYPE 2 : Hunt Group TYPE 3 : Voice Message | STA Range Delay : 0 - 9 Hunt GRP : 620 – 6XX Voice Message : 01~70 | - Not assigned () - Not assigned - Not assigned - Drop : # |
| 2 | Night |  |  |  |
| 3 | Weekend |  |  |  |
| 4 | ON-Demand |  |  |  |

- TABLE 4.5.1 Button Configuration for CO Station Ring Attribute  (PGM 144)

## 4.6	CO RING ASSIGNMENT DISPLAY (PGM 145)

PROCEDURE

| CO RING ASSIGN DISPALY ENTER COL RANGE | (1) [TRANS/PGM] + 145. |
| --- | --- |
| 001-002 PRESS KEY DAY NIGHT WEEK ON-D | (2) Enter CO line range. User can select one of 4 Flex. BTNs. |
| C001(D): 100(0) 101(0) 102(1) 103(1) 104(1) |  If DAY/NIGHT ring is assigned to the station, You can see the delay value also. |
| C001(W) VMIB MSG 02 (00-70) | You can move to the other (Day/Night/Weekend/On-demand) modes by press Flex button. |

| BTN | ITEM | REMARK |
| --- | --- | --- |
| 1 | Day | - When there are too many stations to see, you can scroll data using volume up/down key. |
| 2 | Night |  |
| 3 | Weekend |  |
| 4 | ON-Demand |  |

- TABLE 4.6.1 Button Configuration for CO Station Ring Attribute  (PGM 145)

# 5. SYSTEM DATA PROGRAMMING

If system parameters are to be changed, press the [TRANS/PGM] button and dial 160-179. When programming, LCD and LEDs indicate current programmed data and programming status. The programmer enters correct data, then LCD and LEDs show the entered data and the data is stored in the temporary buffer area. Pressing the [HOLD/SAVE] button, all data in the temporary buffer (same as LCD/LEDs show their status) are stored into permanent system memory.

## 5.1	SYSTEM ATTRIBUTE - I  (PGM 160)

PROCEDURE

| SYSTEM ATT 1 PRESS FLEX_KEY (1-13) | - [TRANS/PGM] + 160. The system attributes fields are shown in TABLE 5.1.1. |
| --- | --- |
| ATD CALL QUE RB TONE (1 : RBT/ 0 :MOH ) : MOH | - To program the Attendant Call Queuing Ring back Tone, press the Flex. BTN 1. Entering a desired value, the LCD shows the changed value. |
| CAMP RBT/MOH (1 : RBT/ 0 :MOH ) : MOH | - To select the Camp Ring back tone or MOH, press the Flex. BTN 2. Entering a desired value, the LCD shows the changed value. |
| CO LINE CHOICE (1: LAST/ 0: ROUND) : LAST | - To program the CO Line Choice, press the Flex. BTN 3. Entering a desired value, the LCD shows the changed value. - (1: Last Choice, 0: Round Robin) |
| DISA RETRY CNT ( 1 – 9 ) : 3 | - To program DISA Retry Count, press the Flex. BTN 4 and dial 1 digit (1-9). Entering a desired value, the LCD shows the changed value. |
| ICM CONT DIAL TONE (1: CONT/ 0: DISC) : CONT | - To program the Intercom Continuous dial tone, press the Flex. BTN 5. Entering a desired value, the LCD shows the changed value. (1: Continuous, 0: Discontinuous) |
| CO DIAL TONE DET (1 : ON/ 0 : OFF) : OFF | - To program the CO dial Tone Detect, press the Flex. BTN 6. Entering a desired value, the LCD shows the changed value. |
| EXT NIGHT RING (1 : ON/ 0 : OFF) : OFF | - To program the External Night ring, press the Flex. BTN 7. Entering a desired value, the LCD shows the changed value. |
| HOLD PREFENCE (1: SYS/ 0: EXC) : SYS | - To program the Hold Preference, press the Flex. BTN 8. Entering a desired value, the LCD shows the changed value. (1: System, 0: Exclusive) |
| MULTI LINE CONF (1 : ON/ 0 : OFF) : ON | - To program the Multi Line Conference, press the Flex. BTN 9. Entering a desired value, the LCD shows the changed value. |
| PRT LCR CONV DGT (1: ON/ 0: OFF) : OFF | - To program Print LCR Conversion Digit, press the Flex. BTN 10. Entering a desired value, the LCD shows the changed value. |
| CONF WARNING TONE (1 : ON/ 0 : OFF) : ON | - To program Conference Warning Tone, press the Flex. BTN 11. Entering a desired value, the LCD shows the changed value. |
| CONF WARNING TONE (1 : ON/ 0 : OFF) : ON | - Press the [HOLD/SAVE] button for updating database permanently when setting value in each flex BTN. |
| SYSTEM ATT 1 PRESS FLEX_KEY (1-13) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Attendant Call Queuing Ring back Tone | - RBT/MOH | - MOH | - RBT: The station will be presented ring back tone when calling busy attendant station. - MOH: The station will be presented MOH, hold tone or VMIB-MOH by system database (PGM171-BTN2) |
| 2 | CAMP RBT/MOH | - RBT/MOH | - MOH | - MOH or Ring back tone is heard in camp-on. |
| 3 | CO Line Choice | LAST/ ROUND | LAST | The method of a CO line seizing on CO line group access (Last Choice/Round-robin) |
| 4 | DISA Retry Counter | 1-9 | 3 | When the DISA user fails to call a station or access a feature, then DISA user can retry other calls or features within this retry counter. If DISA user cannot access appropriately within this counter, the DISA line will be disconnected automatically. |
| 5 | ICM Continuous Dial-Tone | CONT/ DISCONT | CONT | This field sets whether ICM dial tone is continuous or not. |
| 6 | CO Dial-Tone Detect | - ON/OFF | OFF | When the speed dial is activated, system detects dial tone using CPT instead of pause timer. |
| 7 | External Night Ring | - ON/OFF | OFF | When CO lines are marked to UNA, ringing will be sent to LBC1 when an incoming call is received on those lines during night service. |
| - 8 | Hold Preference | SYS/ EXEC | SYS | System hold or exclusive hold |
| - 9 | - Multi-line Conference | - ON/OFF | ON | The system allows a conference with multi-CO lines. |
| - 10 | - Prt LCR Conv Dgt | - ON/OFF | OFF | Print dialed digits or LCR conversed digits in LCD and SMDR. |
| - 11 | - Conference Warning Tone | - ON/OFF | ON | When a new member enters a conference, other members will hear warning tone. |
| - 12 | - Offnet Prompt Usage | - ON/OFF | ON | In case of Offnet call forward, offnet prompt will be heard.(It is only applied to CO-to-CO Transfer) |
| - 13 | - Offnet DTMF Tone | - ON/OFF | ON | In case of Offnet call forward, DTMF tone will be heard.(It is only applied to CO-to-CO Transfer) |

TABLE 5.1.1  Button Configuration for System Attribute - I (PGM 160)

## 5.2	SYSTEM ATTRIBUTE - II  (PGM 161)

PROCEDURE

| SYSTEM ATT 2 PRESS FLEX_KEY (1-14) | - [TRANS/PGM] + 161. |
| --- | --- |
| NETWORK TIME/DATE (1 : ON/ 0: OFF) : OFF | - To program, use the Flex. BTNs as Table 5.2.1. Press one of the Flex. BTNs 1-14. (EX. Flex. BTN 1) Entering a desired value, the LCD shows the changed value. |
| NETWORK TIME/DATE (1 : ON/ 0: OFF) : OFF | - Press the [HOLD/SAVE] button for updating database permanently when setting values in each Flex. BTN. |
| SYSTEM ATT 2 PRESS FLEX_KEY (1-14) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Network Time/Date Setting | - ON/OFF | - OFF | - If this field is ON, the system time/date are set by the network time/date. |
| 2 | Off-Hook Ring Type | MUTE/ BURST | MUTE | The off-hook ring type in the system can be set to mute or one burst ring. |
| 3 | Override 1st CO Group | - ON/OFF | ON | If there is no available CO line in the 1st CO group, system can access the next accessible CO group when this field is ON. |
| 4 | Page Warning Tone | - ON/OFF | ON | If desired, page warning tone can be suppressed. |
| 5 | Auto Privacy | - ON/OFF | ON | The system can be programmed to override CO line call to gain access to the conversation. If privacy is disabled, a station privileged to override in PGM113-BTN 4 joins an existing call in progress. |
| 6 | Privacy Warning Tone | - ON/OFF | ON | If desired, privacy warning tone can be suppressed. |
| - 7 | Single Ring for CO Call | - YES/NO | NO | Changes a cadence of ICM or incoming CO ring. In case of NO, ICM: 1sec on/ 4sec off CO : 0.4s on/ 0.2s off/ 0.4s on/ 4sec off In case of YES, the cadence is the reverse. |
| 8 | WTU Auto Release | ON/OFF | OFF | Enable or disable auto release of WTU |
| 9 | ACD Print Enable | 1:10s/ 0:OFF | OFF | Enable or disable ACD Print feature |
| 10 | ACD Print Timer | 001 –255 (3 digits) | 001 | Determines the amount of time between repeated ACD database prints. (10 sec or 1 hour base) |
| 11 | Clear ACD Database after Print | ON/OFF | OFF | Determines that initialize ACD database after print-out. |
| 12 | VMIB Prompt Gain | 00-31 | 08 | To control prompt gain level. |
| 13 | New VM Type Set | ON / OFF | OFF | When Voice Mail information printed through RS232 port by SMDI, if this is ‘ON’, system use new Type 1 print format (CLI added, see RS232 Spec.) instead of Type 2. |
| 14 | ACD Print Timer Unit | HOUR/ SEC | SEC | Determines the unit of ACD Print timer of Flex Btn 10 (1 hour or 10 seconds). |

TABLE 5.2.1  Button Configuration for System Attribute - II  (PGM 161)

## 5.3	ADMIN PASSWORD  (PGM 162)

Admin password can be assigned to enter Admin Programming mode for only administrator who knows the Admin Password. It is not assigned by default.

PROCEDURE

| ADMIN PASSWORD .... |  | - [TRANS/PGM] + 162. |
| --- | --- | --- |
| ADMIN PASSWORD 1234 |  | - To assign Admin Password, enter 4 digits number, then entered admin password will be displayed on the LCD. Otherwise to delete the admin password, press the [SPEED] button. (Ex. 1234) |
| ADMIN PASSWORD .... |  | - Press the [HOLD/SAVE] button for updating database permanently. |

## 5.4	ALARM ATTRIBUTES  (PGM 163)

PROCEDURE

| SYSTEM ALARM ATT PRESS FLEX_KEY (1-4) | - [TRANS/PGM] + 163. |
| --- | --- |
| ALARM ENABLE (1 : ON/ 0: OFF) : OFF | - To program, use the Flex. BTNs as Table 5.4.1. Press one of the Flex. BTNs 1-4. (Ex. Flex. BTN 1) Entering a desired value, the LCD shows the changed value. - Note: An SLT must not be assigned to receive signals for either the alarm or door bell. |
| ALARM ENABLE (1 : ON/ 0: OFF) : OFF | - Press the [HOLD/SAVE] button for updating database permanently when setting value in each Flex. BTN. |
| SYSTEM ALARM ATT PRESS FLEX_KEY (1-4) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Alarm Enable | ON/OFF | OFF |  |
| 2 | Alarm Contact Type | CLOSE/OPEN | CLOSE | Close, Open |
| 3 | Alarm Mode | ALARM / BELL | ALARM | Alarm, Door Bell |
| 4 | Alarm Signal Mode | RPT/ONCE | RPT | Repeat , Once |

TABLE 5.4.1  Button Configuration for Alarm Attribute  (PGM 163)

## 5.5	ATTENDANT ASSIGNMENT  (PGM 164)

- Maximum 5 Attendants can be assigned including the Main Attendants and System Attendant. The system attendant is different with main attendant in aspect of the call handling and system management priority. The system attendant has more powerful priority than main attendant. The system and main attendants can be assigned each 1 and maximum 4. So the sum of system and main attendants should be less than 5. As default, the System Attendant is assigned to Station 101, and others are not assigned.

PROCEDURE

| ATTENDANT ASSIGNMENT 101 . . . . . . . . . . . . . . . . | - [TRANS/PGM] + 164. |  |
| --- | --- | --- |
| ATTENDANT ASSIGNMENT 101 . . . . . . . . . . . . . . . . | - To assign System Attendants (one of 1-5), press the Flex. BTN 1. Enter the station number, then assigned system attendant station number will be displayed on the LCD. Otherwise to delete any system attendant, press the Flex. BTN, which want to delete and press the [SPEED] button. - Note: It is impossible to delete the first System Attendant. |  |
| MAIN ATD ASSIGN 101 . . . . . . . . . . . . . . . . | - To assign Main Attendants (one of 2-5), press the Flex.BTN 2. And the procedure is the same as system attendant assignment. |  |
| ATTENDANT ASSIGNMENT 101 . . . . . . . . . . . . . . . . | - Press the [HOLD/SAVE] button for updating database permanently. |  |
| ATTENDANT ASSIGNMENT 101 . . . . . . . . . . . . . . . . |  | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory |

## 5.6	AUTO ATTENDANT VMIB ANNC ASSIGNMENT  (PGM 165)

User may set the number of the VMIB announcement for auto attendant.

PROCEDURE

| AUTO ATTENDANT PRESS FLEX_KEY (1-2) | - [TRANS/PGM] + 165. |
| --- | --- |
| AUTO ATTENDANT VMIB ANNC : 55 (00-70) | - To program the number of VMIB announcement for auto attendant, Press Flex Key2 and dial the number of VMIB announcement location. (Ex. dial 55) Then the dialed number will be displayed on the LCD. |
| AUTO ATTENDANT VMIB ANNC : 55 (00-70) | - Press the [HOLD/SAVE] button for updating database permanently. |
| AUTO ATTENDANT PRESS FLEX_KEY (1-2) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | AUTO ATD USAGE | ON/OFF | OFF |  |
| 2 | VMIB ANNC | 00 - 70 | 00 |  |

## 5.7	CO-TO-CO COS  (PGM 166)

When an external user of DID/DISA/TIE line tries to access another CO line in the system, CO-to-CO COS is applied. The attributes of CO-to-CO COS are the same as the station COS.

PROCEDURE

| CO TO CO COS (1 – 7) DAY: 1 NIGHT/WEEKEND: 1 | - [TRANS/PGM] + 166. |
| --- | --- |
|  | - To program, use the Flex. BTNs as Table 5.7.1. To change the COS for Day operation, press Flex. BTN 1 and dial the COS (1 digit) and to change the COS for Night/Weekend operation, press Flex. BTN 2 and dial COS (1 digit). (Ex. Day:2, N/W:3) Then changed COS will be displayed on the LCD. |
| CO TO CO COS (1 – 7) DAY: 2 NIGHT/WEEKEND:3 | - Press the [HOLD/SAVE] button for updating database permanently. |
| CO TO CO COS (1 – 7) DAY: 1 NIGHT/WEEKEND:1 | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Day COS | 1-7 | 1 | Day Class-of-Service |
| 2 | Night/Weekend COS | 1-7 | 1 | Night/Weekend Class-of-Service |

- TABLE 5.7.1  Button Configuration for CO-to-CO COS  (PGM 166)

## 5.8	DID/DISA DESTINATION  (PGM 167)

When a station receives a DID/DISA call which is busy, invalid or vacant, the call may be sent to Attendant, forwarded to Hunt group or tone is presented to the called party by Admin programming.

Selecting Attendant as the DID/DISA destination, the call will follow ring assignment at first. If there is no ring assigned station, the call will be sent to Attendant.

PROCEDURE

| DID/DISA DEST PRESS FLEX_KEY (1-4) | - [TRANS/PGM] + 167. One of BTN 1-4 show assigned DID Busy/Error/No Answer destination. To see VMIB Prompt Usage Press Flex BTN4 |
| --- | --- |
| BUSY DESTINATION TONE (F1 – F3) | - To assign Busy Destination, press Flex. BTN 1. |
| BUSY DESTINATION ATD (RING ASGN) | - Ex. Press Flex BTN 2 to assign Attendant as Busy Destination. To save, press the [HOLD/SAVE] button. |
| BUSY DESTINATION HUNT: 620 | - Ex. Press Flex BTN 3 and dial Hunt # to assign Hunt Group 620 as Busy Destination. To save, press the [HOLD/SAVE] button. |
| BUSY DESTINATION HUNT: 620 (F1-F3) | - Press the [HOLD/SAVE] button for updating database permanently. |
| VMIB PROMPT USAGE PRESS FLEX_KEY (1-5) | - To assign VMIB Prompt Usage, Press Flex BTN 4 |
| BUSY PROMPT USAGE (1:ON / 0:OFF) : ON | - Ex. Press Flex BTN 1 to change Busy Prompt Usage. To save, press the [HOLD/SAVE] button. |
| DID/DISA DEST PRESS FLEX_KEY (1-4) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory |

| BTN | ITEM |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 1 | Busy Destination |  | F1-F3 | 1 | 1: Tone 2: Attendant (Ring Assign) 3: Forward to Hunt Group |
| 2 | Error Destination |  | F1-F3 | 1 |  |
| 3 | No Answer Destination |  | F1-F3 | 1 |  |
| 4 | VMIB PROMPT USAGE |  | F1-F5 |  | If the field is set as “OFF”, each Prompt is not supplied to the calling party. This field affects only DID service CO line. DISA line doesn’t affect this field. |
|  | 1 | Busy Prompt Usage | ON / OFF | ON |  |
|  | 2 | Error Prompt Usage | ON / OFF | ON |  |
|  | 3 | DND Prompt Usage | ON / OFF | ON |  |
|  | 4 | No Ans Prompt Usage | ON / OFF | ON |  |
|  | 5 | Atd Xfer Prompt Usage | ON / OFF | ON |  |

TABLE 5.8.1  Button Configuration for DID/DISA Destination  (PGM 167)

## 5.9	EXTERNAL CONTROL CONTACT  (PGM 168)

By default, External control contacts are not assigned at all.

PROCEDURE

| EXT CONTROL CONTACT PRESS FLEX_KEY (1-X) | - [TRANS/PGM] + 168. |  |
| --- | --- | --- |
| EXT CONTROL CONTACT NO 1 : .… ( 1 – 5 ) | - Select one of External Control Contacts with Flex. BTN 1-7(1-6 for LDK-100). Then the pressing BTN’s LED will be lit and currently assigned External Control Contact will be displayed on the LCD. (Ex. External Control Contact 1: Flex. BTN 1). |  |
| EXT CONTROL CONTACT NO 1 : LBC 150 | - To assign Loud Bell Control to External Control Contact, dial 1 and enter station number. Then entered data will be displayed on the LCD. (Ex. STA 150). |  |
| EXT CONTROL CONTACT NO 1 : DOOR OPEN | - To assign Door to External Control Contact, dial 2. |  |
| EXT CONTROL CONTACT NO 1 : EXT_1 | - To assign External Control Device 1 to External Control Contact, dial 3. In LDK-300/100, only 3 External Control Devices are allowed. |  |
| EXT CONTROL CONTACT NO 2 : EXT_2 | - To assign External Control Device 2 to External Control Contact, dial 4. |  |
| EXT CONTROL CONTACT NO 1 : ... | - To delete the assignment of External Control Contact, press the [SPEED] button. |  |
| EXT CONTROL CONTACT PRESS FLEX_KEY (1-7) | - Press the [HOLD/SAVE] button for updating database permanently when setting value in each Flex. BTN. |  |
| EXT CONTROL CONTACT PRESS FLEX_KEY (1-7) |  | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | First Contact | 1 – 5 | - | 1: LBC(STA #) |
| 2 | Second Contact | 1 – 5 | - | 2: Door |
| 3 | Third Contact | 1 – 5 | - | 3: Ext. 1 |
| 4 | Forth Contact | 1 – 5 | - | 4: Ext. 2 |
| 5 | Fifth Contact | 1 – 5 | - | 5: Ext. 3 |
| 6 | Sixth Contact | 1 – 5 | - |  |
| 7 | Seventh Contact | 1 – 5 | - | LDK-300 Only |

TABLE 5.9.1  External Control Contact  (PGM 168)

## 5.10	LCD TIME/DATE/LANGUAGE DISPLAY MODE  (PGM 169)

The LCD Time/Date/Language display formats can be set. Two LCD Time formats are Ordinary (12-hour)/Military (24-hour) mode and two LCD date formats are Day/Month/Year (DDMMYY) or Month/Day/Year (MMDDYY) mode. The LCD language format can be set, too.

PROCEDURE

| LCD DISPLAY MODE PRESS FLEX_KEY (1-3) | - [TRANS/PGM] + 169. |
| --- | --- |
| LCD TIME MODE (1: 12H/ 0: 24H) : 12H | - To program the LCD time display mode, press the Flex. BTN 1. Then user can select the desired value with LCD display. - 1: 12-Hour mode, 0 : 24-Hour mode |
| LCD DATE MODE (1: MMDD/ 0: DDMM) : DDMMYY | - To program LCD date display mode, press the Flex. BTN 2. Then user can select the desired value with LCD display. - 1: MM-DD-YY, 0: DD-MM-YY |
| LCD LANGUAGE (00-24) ENGLISH (00) | - To program LCD language format, press the Flex. BTN 3 and dial 2-digit as language format. (See TABLE 5.10.1) The LCD will be changed to the current value. |
| LCD LANGUAGE (00-24) ENGLISH (00) | - Press the [HOLD/SAVE] button for updating database permanently. |
| LCD DISPLAY MODE PRESS FLEX_KEY (1-3) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | LCD Time Display Mode | 12H/24H | 12H | 12-Hour Mode 24-Hour Mode |
| 2 | LCD Date Display Mode | MMDDYY / DDMMYY | DDMMYY | Month/Day/Year Day/Month/Year |
| 3 | LCD Language Display Mode | - 00-14 | - 00 - (English) | - 00: English, - 01: Italian, - 02: Finnish, - 03: Dutch, - 04: Swedish, - 05: Danish, - 06: Norwegian, - 07: Hebrew, - 08: Germany, - 09: French, - 10: Portuguese, - 11: Spanish, - 12: Korean, - 13: Estonia, - 14: Russian |

TABLE 5.10.1  Button Configuration for LCD Time/Date/Language Display Mode  (PGM 169)

## 5.11	MODEM  (PGM 170)

PROCEDURE

| MODEM ASC DEVICE STA : 399 (F1: STA F2: CO) | - [TRANS/PGM] + 170. In LDK-100, ‘STA:227’ will be displayed. |
| --- | --- |
| MODEM ASC DEVICE STA : 399 (F1: STA F2: CO) | - To program MODEM Attributes, press the one of Flex. BTN 1-2 as Table 5.11.1. - To set modem associated device for station, press Flex. BTN 1 and digit the number. Then entered data will be displayed on LCD. To save, press the [HOLD/SAVE] button. - To set modem associated device for CO, press Flex. BTN 2 and digit the number. Then entered data will be displayed on LCD. To save, press the [HOLD/SAVE] button. |
| MODEM ASC DEVICE CO : . . . (F1: STA F2: CO) |  |
| MODEM ASC DEVICE CO : 010 (F1: STA F2: CO) | - Press the [HOLD/SAVE] button for updating database permanently. |
| MODEM ASC DEVICE STA : 399 (F1: STA F2: CO) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory |

| BTN | ITEM | RANGE |  | DEFAULT |  | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | LDK-300 | LDK-100 | LDK-300 | LDK-100 |  |
| 1 | STA No. | 100 – 399 | 100 – 227 | STA 399 | STA 227 | Last Station |
| 2 | CO No | 001 – 200 | 01 – 40 | - |  |  |

TABLE 5.11.1  Button Configuration for Modem Assignment  (PGM 170)

## 5.12	MUSIC  (PGM 171)

PROCEDURE

| MUSIC ASGN PRESS FLEX_KEY (1-4) | - [TRANS/PGM] + 171. |
| --- | --- |
|  | - To program, use the Flex. BTNs as Table 5.12.1. Press BTN 1-4 and enter the related data. Then entered data will be displayed on the LCD. |
| BGM TYPE (00-12) INT MUSIC (1) | - To program the background music type, press the Flex. BTN 1. To change the BGM type, press two digits of 00-12. In LDK-100, the range is 00-11. Then entered data and related BGM type will be displayed on the LCD. |
| BGM TYPE (00-12) EXT MUSIC 2 ( 3) | - Ex. Dial 3 as a BGM type. |
| MOH TYPE (00-13) INT MUSIC (1) | - To program the MOH type, press the Flex. BTN 2. To change the MOH type, press two digits of 00-13. In LDK-100, the range is 00-12. Then entered data and related MOH type will be displayed on the LCD. |
| ICM BOX MUSIC CH (00-12) INT MUSIC (1) | - To program the ICM box music channel, press the Flex. BTN 3. To change the ICM box music channel, press two digits of 00-12. In LDK-100, the range is 00-11. Then entered data and related ICM box music channel will be displayed on the LCD. |
| ASSIGN SLT MOH .... .... .... .... .... | - To program the SLT MOH, press the Flex. BTN 4. To assign the SLT MOH, press a flexible button between F1-F5 and enter the SLT station number. To save the data, press the [HOLD/SAVE] button whenever entering each station number. The LCD will indicate each MOH channel assigned at the SLT station. |
| MUSIC ASGN PRESS FLEX KEY (1-4) | (7) Press the [HOLD/SAVE] button for updating database permanently. |
| MUSIC ASGN PRESS FLEX KEY (1-4) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory |

| BTN | ITEM | RANGE |  | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
|  |  | LDK-300 | LDK-100 |  |  |
| 1 | BGM Type | 00-12 | 00-11 | 01 | 0: None 1: Int. Music 2-4: External Music 1-3 5-6(7): VMIB BGM1-2(3) 7(8)-11(12): SLT MOH |
| 2 | MOH Type | 00-13 | 00-12 | 01 | 0: None 1: Int. Music 2-4: External Music 1-3 5-6(7): VMIB BGM1-2(3) 7(8)-11(12): SLT MOH 12(13) : Hold Tone |
| 3 | ICM Box Music Channel | 00-12 | 00-11 | 01 | 0: None 1: Int. Music 2-4: External Music 1-3 5-6(7): VMIB BGM1-2(3) 7(8)-11(12): SLT MOH |
| 4 | Assign SLT MOH | - |  | Flex. 1-5 (+ SLT STA No.) | SLT MOH 1-5 |

TABLE 5.12.1  Button Configuration for System Attribute - II  (PGM 171)

## 5.13	PBX ACCESS CODE  (PGM 172)

Maximum 4 PABX Access Codes are assignable. PABX Access Code is 1 or 2-digit number. By default, PABX Access Codes are not assigned at all.

PROCEDURE

| PABX ACCESS CODE PRESS FLEX_KEY (1-4) | - [TRANS/PGM] + 172. LCD shows currently assigned PABX Access codes. |
| --- | --- |
| PABX ACCESS CODE 1 9 | - To assign PABX Access code, press Flex. BTN 1-4 and do one of followings: - Dial 2 digits or 1 digit number to assign PBX Access Code. - Press the [SPEED] button to delete PBX Access Code. - For example, pressing Flex. BTN 1 and dialing 9, the changed value will be displayed on the LCD. |
| PABX ACCESS CODE PRESS FLEX_KEY (1-4) | - Press the [HOLD/SAVE] button for updating database permanently. |
| PABX ACCESS CODE PRESS FLEX_KEY (1-4) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory |

## 5.14	PLA PRIORITY SETTING  (PGM 173)

The PLA priority can be programmable with CO recalling, transfer call, incoming call and queued call.

PROCEDURE

| XFR REC INC QUE 1 2 3 4 | - [TRANS/PGM] + 173. |
| --- | --- |
| XFR REC INC QUE 2 1 4 3 | - To change the PLA priority, first choose one of 4 Flex. BTNs as XFR REC INC QUE, and press one digit of 1-4. Then entered digit and related data will be displayed on the LCD. (Ex. Flex. BTN 2 and dial 1) |
| XFR REC INC QUE 2 1 4 3 | - Press the [HOLD/SAVE] button for updating database permanently. |
| XFR REC INC QUE 1 2 3 4 | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | XFER (Transfer Call) | 1 – 4 | 1 | PLA priority is set exclusively |
| 2 | REC (Recall) | 1 – 4 | 2 |  |
| 3 | INC (Incoming Call) | 1 – 4 | 3 |  |
| 4 | QUE (Queued Call) | 1 – 4 | 4 |  |

TABLE 5.14.1  PLA Priority Setting  (PGM 173)

## 5.15	RS-232C PORT SETTING  (PGM 174)

PROCEDURE

| RS232 PORT SETTING PRESS FLEX_KEY (1-5) | - [TRANS/PGM] + 174. To select a RS232C port, use the Flex. BTNs as Table 5.15.1. In LDK-100, the range (1-3) will show. |
| --- | --- |
| COM1 PORT SETTING F1:BD F2:CTS F3:BK F4:P | - To program RS232 Port 1, press Flex. BTN 1. Then the entered COM port and related data will be displayed on LCD. |
| COM1 BAUDRATE BAUDRATE: 19200 | - To program BAUDRATE, press Flex. BTN 1 and press one digit of 0-7 Then related data will be displayed on the LCD. Press the [HOLD/SAVE] button to save it. |
| COM1 CTS/RTS (1:ON/0:OFF):OFF | - To program CTS/RTS, press Flex. BTN 2 and press one digit of 0-1. Then the related data will be displayed on the LCD. Press the [HOLD/SAVE] button to save it. |
| COM1 PAGE BRK (1:ON/0:OFF):OFF | - To program PAGE BREAK, press Flex. BTN 3 and press one digit of 0-1. Then the entered data will be displayed on the LCD. Press the [HOLD/SAVE] button to save it. |
| COM1 LINE PAGE (001-199) : 060 | - To program LINE PAGE, press Flex. BTN 4 and press three digit of 001-199. Then the entered data will be displayed on LCD. Press the [HOLD/SAVE] button to save it. |
| COM1 PORT SETTING F1:BD F2:CTS F3:BK F4:P | - Press the [HOLD/SAVE] button for updating database permanently. |
| COM1 PORT SETTING F1:BD F2:CTS F3:BK F4:P | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | COM1 Port Setting | Flex. BTN 1-4 |  |  |
| 2 | COM2 Port Setting | Flex. BTN 1-4 |  |  |
| 3 | COM3 - MODU Port Setting | Flex. BTN 1-4 |  |  |
| 4 | COM4 - MISB Port Setting | Flex. BTN 1-4 |  | LDK-300 Only |
| 5 | COM5 - MISB Port Setting | Flex. BTN 1-4 |  | LDK-300 Only |

TABLE 5.15.1  Button Configuration for RS-232 Port Setting  (PGM 174)

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | BAUDRATE | 0-7(Note1) | 19200 | 0: Unknown 1: Unknown 2: 1200 Baud 3: 2400 Baud 4: 4800 Baud 5: 9600 Baud 6: 19200 Baud 7: 38400 Baud |
| 2 | CTS/RTS | ON/OFF | OFF |  |
| 3 | P-BREAK | ON/OFF | OFF |  |
| 4 | LPP | 001-199 | 060 |  |

Note1) Only COM2 port can use 57600-baud rate with setting the baud rate value ‘8’.

TABLE 5.15.2  Button Configuration for COM Port (PGM 174)

## 5.16	PRINT PORT SELECTION  (PGM 175)

PROCEDURE

| PRINT PORT SELECTION PRESS FLEX_KEY (01-12) | - [TRANS/PGM] + 175. To select print port, use the BTNs as Table 5.16.1. |
| --- | --- |
| OFF LINE SMDR (01-13) COM2 (02) | - To program Off-line SMDR/STAT print mode, press Flex. BTN 1 and press two digits of 01-13. In LDK-100, the range is 00-11. Then the entered digit and related data will be displayed on the LCD. Press the [HOLD/SAVE] button to save it. |
| ADMIN DATA (01-13) COM2 (02) | - To program Admin data print mode, press Flex. BTN 2 and press two digits of 01-13. In LDK-100, the range is 00-11. Then the entered digit and related data will be displayed on the LCD. Press the [HOLD/SAVE] button to save it. |
| SMDI (01-13) COM2 (02) | - To program SMDI print mode, press Flex. BTN 4 and press two digits of 01-13. In LDK-100, the range is 00-11. Then the entered digit and related data will be displayed on the LCD. Press the [HOLD/SAVE] button to save it. |
| ONLINE SMDR (01-13) COM2 (02) | - To program On-line SMDR mode, press Flex. BTN 6 and press two digits of 01-13. In LDK-100, the range is 00-11. Then the entered digit and related data will be displayed on the LCD. Press the [HOLD/SAVE] button to save it. |
| TRACE (01-13) COM2 (02) | - To program TRACE mode, press Flex. BTN 7 and press two digits of 01-13. In LDK-100, the range is 00-11. Then the entered digit and related data will be displayed on LCD. Press the [HOLD/SAVE] button to save it. |
| PRINT PORT SELECTION PRESS FLEX_KEY (01-12) | - Press the [HOLD/SAVE] button for updating database permanently. |
| PRINT PORT SELECTION PRESS FLEX_KEY (01-12) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory |

| BTN | ITEM | RANGE |  | DEFAULT |  | REMARK |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LDK-300 | LDK-100 | LDK-300 | LDK-100 | LDK-300 | LDK-100 |
| 1 | Off-line SMDR/ Statistics Print | 01-13 | 01-11 | COM2 | COM1 | 01: COM1 02: COM2 03: COM3-MODU 04: COM4-MISB 05: COM5-MISB 06: TELNET 1 07: TELNET 2 08: TELNET 3 09: ISDN 10: NET_PCADM 11: NET_PCATD 12: NET_CTI 13: NET_REMOTE | 01: COM1 02: COM2 03: COM3-MODU 04: TELNET 1 05: TELNET 2 06: TELNET 3 07: ISDN 08: NET_PCADM 09: NET_PCATD 10: NET_CTI 11: NET_REMOTE |
| 2 | Admin Print | 01-13 | 01-11 | COM2 | COM1 |  |  |
| 3 | Traffic | 01-13 | 01-11 | COM2 | COM1 |  |  |
| 4 | SMDI Print | 01-13 | 01-11 | COM2 | COM1 |  |  |
| 5 | Call Information | 01-13 | 01-11 | COM2 | COM1 |  |  |
| 6 | Info/On-line SMDR | 01-13 | 01-11 | COM2 | COM1 |  |  |
| 7 | Trace | 01-13 | 01-11 | COM2 | COM1 |  |  |
| 8 | Debug | 01-13 | 01-111 | COM2 | COM1 |  |  |
| 9 | PC Admin | 01-13 | 01-11 | NET_PCADM |  |  |  |
| 10 | PC Attendant | 01-13 | 01-11 | NET_PCATD |  |  |  |
| 11 | CTI | 01-13 | 01-11 | NET_CTI |  |  |  |
| 12 | Remote Diagnostic | 01-13 | 01-11 | NET_REMOTE |  |  |  |

TABLE 5.16.1  Button Configuration for Print Port Selection  (PGM 175)

## 5.17	PULSE DIAL RATIO  (PGM 176)

In LDK-300/100, pulse dial speed ratio is set only for 10 PPS.

PROCEDURE

| PULSE DIAL RATIO (1:66/33 0:60/40) : 66/33 | - [TRANS/PGM] + 176. |
| --- | --- |
| PULSE DIAL RATIO (1:66/33 0:60/40) : 60/40 | - To assign Pulse Dial Ratio, press the one digit 0-1 as TABLE 5.17.1. Then the selected value will be displayed on the LCD. (Ex. digit 0: 60/40) |
| PULSE DIAL SPD RATIO (1:66/33 0:60/40) : 60/40 | - Press the [HOLD/SAVE] button for updating database permanently. |

| DGT | PULSE DIAL SPEED RATIO | REMARK |
| --- | --- | --- |
| 1 | 10 PPS 66/33 % | Defaults |
| 0 | 10 PPS 60/44 % |  |

TABLE 5.17.1  Button Configuration for Pulse Dial Speed Ratio  (PGM 176)

## 5.18	SMDR ATTRIBUTES  (PGM 177)

Station Message Detail Recording (SMDR) will provide details on both incoming and outgoing calls. As an assignable database option, if All Call record type is selected, incoming and outgoing local and long distance calls are all provided. If only Long Distance is selected, then only outgoing calls that meet the toll check status requirements listed below are provided.

PROCEDURE

| SMDR ATTRIBUTES PRESS FLEX_KEY (01-14) | - [TRANS/PGM] + 177. To program SMDR attributes, use the BTNs as TABLE 5.18.1. |
| --- | --- |
| SMDR SAVE (1 : ON/ 0: OFF) : OFF | - To program SMDR save mode, press the Flex. BTN 1. The LCD will show the current related field status. User can change the value by pressing one digit. Press the [HOLD/SAVE] button to save it. |
| SMDR PRINT (1 : ON/ 0: OFF) : ON | - To program SMDR print mode, press the Flex. BTN 2. The LCD will show the current related field status. User can change the value by pressing one digit. Press the [HOLD/SAVE] button to save it. |
| RECORD TYPE (1 : LD/ 0: ALL) : LD | - To program SMDR recording call type, press the Flex. BTN 3. The LCD will show the current related field status. User can change the value by pressing one digit. Press the [HOLD/SAVE] button to save it. |
| LD CALL DGT CNT (07-15) : 07 | - To program SMDR long distance call digit counter, press the Flex. BTN 4 and dial 2 digits. (Ex. 07) Then the LCD will be changed to the current related field status. Press the [HOLD/SAVE] button to save it. |
| PRINT INCOMING CALL (1 : ON/ 0: OFF) : OFF | - To program SMDR incoming call print mode, press the Flex. BTN 5. Then the LCD will show the current related field status. User can change the value by pressing one digit. Press the [HOLD/SAVE] button to save it. |
| PRINT LOST CALL (1 : ON/ 0: OFF) : ON | - To program SMDR lost call print mode, press the Flex. BTN 6. Then the LCD will show the current related field status. User can change the value by pressing one digit. Press the [HOLD/SAVE] button to save it. |
| RECORD IN DETAIL (1 : ON/ 0: OFF) : ON | - To program the record detailed SMDR, press the Flex. BTN 7. Then the LCD will show the current related field status. User can change the value by pressing one digit. Press the [HOLD/SAVE] button to save it. |
| HIDDEN DIALED DGT ( 0 – 9 ) : 7 | - To program SMDR dial digit hidden, press the Flex. BTN 8 and dial one digit. (Ex. 7) Then the LCD will be changed to the current related field status. Press the [HOLD/SAVE] button to save it. |
| SMDR CURRENCY UNIT ABC | - To assign currency unit, press the Flex. BTN 9. Then the LCD will be changed to the current related field status. Refer to the English Character Set to enter currency unit. Press the [HOLD/SAVE] button to save it. |
| COST PER PULSE (6DGT) 000000 | - To program SMDR cost per unit pulse, press the Flex. BTN 10 and dial 6 digits. Then the LCD will be changed to the current related field status. Press the [HOLD/SAVE] button to save it. |
| SMDR FRACTION (0-5) : 0 | - To program SMDR fraction, press the Flex. BTN 11 and dial one digit. Then the LCD will be changed to the current related field status. Press the [HOLD/SAVE] button to save it. |
| LONG DISTANCE CODE 0 .. .. .. .. | - To program SMDR long distance codes, press the Flex. BTN 14. Then the current related field status on the LCD. Press 1-5 Flex. BTN and dial maximum 2 digits. Press the [HOLD/SAVE] button to save it. |
| SMDR ATTRIBUTES PRESS FLEX_KEY (1-14) | - Press the [HOLD/SAVE] button for updating database permanently. |
| SMDR ATTRIBUTES PRESS FLEX_KEY (1-14) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | SMDR Save Enable | ON/OFF | OFF | The system can be set to save SMDR record or not. |
| 2 | SMDR Print Enable | ON/OFF | OFF | The system can be set to real time print. |
| 3 | Long Distance / All Call Recorded | LD/All Call | LD | The system can be set to record either all outgoing calls or only long distance calls, exceeding time limit set by SMDR Start Timer. The long distance calls are identified by programmed SMDR long distance code (BTN 14). |
| 4 | SMDR Long Distance Call Digit Counter | 07-15 | 07 | If SMDR digit counter is more than this value, it is considered as long distance call. |
| 5 | Print Incoming Call | ON/OFF | OFF | If this option (PIC) is set to Enable, all incoming calls are printed with either all outgoing calls or long distance calls. |
| 6 | Print Lost Call | ON/OFF | ON | If this option (PLC) is set to Enable, all lost calls are printed with either unanswered or not. |
| 7 | Records in Detail | ON/OFF | ON | Due to limited system memory size, in places where many calls take place, the SMDR record buffer can easily saturated. So, if the customer doesn't need the detailed call information but total call, total metering count and total cost for individual station, then it is possible to save only the total accumulation, rather than the whole detailed records. |
| 8 | SMDR Dial Digit Hidden | 0-9 | 0 | According to this value, SMDR dial digit will be hidden and ‘*’ symbol will be displayed instead of the hidden digits. |
| 9 | SMDR Currency Unit | 3 Chars | - | For easy identification of call cost, the currency unit can be entered with 3 alphabet characters to be printed in front of call charge amount. |
| 10 | SMDR Cost Per Unit Pulse | 6 digits | - | This is the call cost unit per cost metering pulse, which is sent from the Central Office. |
| 11 | SMDR Fraction | 0-5 | 0 | This value means the decimal position point of the cost per unit pulse. |
| 12 | SMDR Start Timer | 000-250 | 000 | 1 sec base |
| 13 | SMDR Hidden Digit | Right/ Left | Right | Hide digits from right or left |
| 14 | SMDR Long Distance Codes | Flex. BTN 1 – 5 | 0 | Max. 5 SMDR long distance codes are available. SMDR long distance code is 1 or 2 digit number. By default, SMDR long distance code is 0. |

TABLE 5.18.1  Button Configuration for SMDR Attributes  (PGM 177)

## 5.19	SYSTEM TIME/DATE SETTING  (PGM 178)

In LDK-300/100, date and time can be set by Admin programming, and it will be shown in the LCD of stations.

PROCEDURE

| SET SYSTEM TIME/DATE PRESS FLEX_KEY (1 - 2) | - [TRANS/PGM] + 178. |
| --- | --- |
| SET SYSTEM TIME/DATE TIME 12:30 (HH:MM) | - To program the system time, press the Flex. BTN 1 and enter Hour/Minute (HHMM) in 24-hour format. Then the LCD will be changed to the current related field status. |
| SET SYSTEM TIME/DATE DATE: 05/10/00 (MMDDYY) | - To program the system date, press the Flex. BTN 2 and enter Month/Date/Year (MMDDYY) format. Then the LCD will be changed to the current related field status. |
| SET SYSTEM TIME/DATE PRESS FLEX_KEY (1-2) | - Press the [HOLD/SAVE] button for updating database permanently. |
| SET SYSTEM TIME/DATE PRESS FLEX_KEY (1-2) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory |

| - BIN | - ITEM | - RANGE | - DEFAULT | - REMARK |
| --- | --- | --- | --- | --- |
| 1 | System Time | 4 digits | - - | - Hour/Min in sequence |
| 2 | System Date | 6 digits | - - | - Month/Day/Year in sequence |

TABLE 5.19.1  System Time/Date Setting  (PGM 178)

## 5.20	LINKED STATION PAIRS TABLE  (PGM 179)

PROCEDURE

| - LINKED STA_PAIR - F1 : VIEW F2 : INPUT | (1) [TRANS/PGM] + 179. |
| --- | --- |
| - LINKED STA_PAIR - 100 / 200 | (2) Press the Flex. BTN 2 and dial two station numbers sequentially to enter a linked station pair. - The station number entered at left side will be master station, and the other will be the slave. |
| - LINKED STA_PAIR - 110 / 120 | - If entered station has linked station already, then linked station number will be displayed automatically. (Ex. When 110,120 is already linked pair, dial 110) |
| - LINKED STA_PAIR - . . . . / . . . . | (3) Press the [HOLD/SAVE] button for updating/saving current linked pair permanently. As this procedure, linked pairs can be assigned continuously. - To delete linked station pair, press the [SPEED] button and the [HOLD/SAVE] button. |
| - 100 108 150 152 - 110 155 151 160 | (4) When linked pairs are assigned, press Flex. BTN 1 to see the linked station pairs. Two station numbers of each column means linked pair (Ex. Master 100 & slave 110, 108 & 155, 150 & 151, 152 & 160). Press [] button to view the next 4 linked pairs. To enter another linked pair, press Flex. BTN 2, then the system goes to step (2). |
| - ALL LINKED PAIRS DELETE - PRESS [HOLD/SAVE] BTN | (5) To delete all linked pairs, press the [SPEED] button. If you press the [SPEED] button, confirmation message will be displayed. Press the [HOLD/SAVE] button, and all linked pairs will be deleted |
| - LINKED STA_PAIR - F1 : VIEW F2 : INPUT | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory |

| - BIN | - ITEM | - RANGE | - DEFAULT | - REMARK |
| --- | --- | --- | --- | --- |
| - 1 | - Review Linked Station Pairs | - 100 – 399 (LDK-300) - 100 – 227 (LDK-100) | - None | - All Linked Station Pairs - are displayed. |
| - 2 | - Enter Linked Pair | - 2 STA # | - - | - STA # / STA # |

- TABLE 5.20.1  Linked Station Pairs  (PGM 179)

- Note: If you assign linked pair with wired station (DKT or SLT) and wireless station (WHTU), please assign wired station to be the master station.

## 5.21	CIDU SETTING  (PGM 185)

- To use CIDU(Analog CO Line CLI Decoder), this admin program must be set.

PROCEDURE

| - CIDU SETTING - PRESS FLEX_KEY (1-5) | - [TRANS/PGM] + 185. |
| --- | --- |
| - CID USAGE - (1:ON/0:OFF):OFF | - Press the flexible button 1, and enter the “CID USAGE” set field. Press dial ‘1’ to enable the CID usage. |
| - CID NAME DISPLAY - (1:NAME/0:TEL) : TEL | - Press the flexible button 2, and enter the “CID NAME DISPLAY” set field. Press dial ‘1’ to display the caller name, and press dial ‘0’ to display ther caller telephone number. |
| - SERIAL PORT SEL - (1-4) : COM1 | - Press the flexible button 3, and enter the “SERIAL PORT SEL” set field. Press dial 1~5 to set the serial port for CIDU connection.In LDK-100, the range of the serial ports is 1-2. |
| - 000 001 002 003 - 001 002 003 004 | - To mapping the CIDU port and the analog CO line port, press the flexible button 4. Upper line of LCD messages is present the CIDU port number and the lower line is present the analog CO line port number. |

| - BIN | - ITEM | - RANGE | - DEFAULT | - REMARK |
| --- | --- | --- | --- | --- |
| - 1 | CID Usage | ON / OFF | OFF | Set the CID usage enable. |
| - 2 | CID Name Display | Name (1) / Telephone No.(0) | Telephone No.(0) | Set the LCD display mesage between the character name or the telephone number. |
| - 3 | Serial Port Select | 1-4 (LDK-300) 1-2 (LDK-100) | COM1 | Set the serial port for CIDU connection. |
| - 4 | CID/CO Line Port Mapping | 000-063 | - | Set the CIDU port and the analog CO line port mapping. |
| - 5 | Initialize CID Data |  |  | Initialize the CIDU admin. |
| - 6 | CID type II Usage | ON / OFF | OFF | Set the CID type II usage |

- TABLE 5.21.1  CIDU Setting  (PGM 185)

# 6.  SYSTEM TIMERS

## 6.1	SYSTEM TIMERS - I  (PGM 180)

PROCEDURE

| SYSTEM TIMER 1 PRESS FLEX KEY (01-22) | - [TRANS/PGM] + 180. |
| --- | --- |
|  | - To program, use the BTNs as TABLE 6.1.1. Press one of BTN 1-22 and enter related data. (LED of selected Flex. BTN will be lit.) |
| - ATD RECALL TMR(min) - (00 – 60) : 01 | - To program the Attendant Recall Timer, press the Flex. BTN 1. The current value will be displayed on the LCD and user can enter a desired value. Then LCD shows the changed value. |
| - SYSTEM TIMER 1 - PRESS FLEX KEY (01-22) | - Press the [HOLD/SAVE] button for updating database permanently. |
| SYSTEM TIMER 1 PRESS FLEX KEY (01-22) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| - BTN | - ITEM | - RANGE | - DEFAULT | - REMARK |
| --- | --- | --- | --- | --- |
| - 1 | - Attendant Recall Timer | - 00 – 60 - (2 Digits) | - 01 - (min) | - Determines the amount of time before system disconnects the call |
| - 2 | - Call Park Recall Timer | - 000 – 600 - (3 Digits) | - 120 - (sec) | - Determines the amount of time before a call placed in a call park location will recall the station placing the park. |
| - 3 | Camp-on Recall Timer | 000 – 200 (3 Digits) | 030 (sec) | If a station transfers to busy station and hang up, this recall timer is assigned. |
| - 4 | - Exclusive Hold Recall Timer | - 000 – 300 - (3 Digits) | - 060 - (sec) | - Determines the amount of time before a call placed on exclusive hold will recall the station placing the hold. |
| - 5 | - I-Hold Recall Timer | - 000 – 300 - (3 Digits) | - 030 - (sec) | - Determines the amount of time before a call recalls the attendant. |
| - 6 | - Sys Hold Recall Timer | - 000 - 300 - (3 Digits) | - 030 - (sec) | - Determines the amount of time before a call placed on system hold will recall the station placing the hold. |
| - 7 | - Transfer Recall Timer | - 000 - 300 - (3 Digits) | - 030 - (sec) | - Determines the amount of time a transferred call will ring at the station receiving the transfer and how long it will recall the station transferring the call. |
| - 8 | - ACNR Delay Timer | - 000 - 300 - (3 Digits) | - 030 - (sec) | - When ACNR Pause Timer expires and there is no available CO Line in the group, this timer is invoked. - When ACNR Delay Timer expired, - - Invoke ACNR Pause Timer if is no available CO line. Still, ACNR is activated. |
| - 9 | - ACNR No Answer Timer | - 10 - 50 - (2 Digits) | - 30 - (sec) | - This timer is invoked after system detects CO ring back tone or voice from CO party. After this timer, system retries ACNR. |
| - BTN | - ITEM | - RANGE | - DEFAULT | - REMARK |
| - 10 | - ACNR Pause Timer | - 005 - 300 - (3 Digits) | - 030 - (sec) | - When expired, ACNR is activated. |
| - 11 | - ACNR Retry Counter | 01 – 30 - (2 Digit) | - 03 | - This is decreased every time station retries ACNR, ACNR is canceled if it set to 0. |
| - 12 | - ACNR No Tone Retry Counter | - 1 – 9 - (1 Digit) | - 1 | - The number of retry count to detect tone. |
| - 13 | - ACNR Tone Detect Timer | 000 –300 (3 Digits) | - 030 - (sec) | - This timer is invoked upon completion of dialing and system considers the CO party is busy when the CPTU cannot detect the valid tone type until this timer expires. |
| - 14 | Automatic CO Release Timer | 020 –300 (3 Digits) | 030 (sec) | Uncompleted CO line call will be automatically released after this timer. |
| - 15 | - CCR Inter-digit Timer | - 000 - 255 - (3 Digits) | - 030 - (100ms) | - This field is used for the CCR inter-digit timer in the DISA/DID CO line. In DID type 2, it is used for DID inter-digit timer. |
| - 16 | - CO Call Drop Warning Timer | - 00 - 99 - (2 Digits) | - 10 - (sec) | - If prepaid money is going to expire during a CO line conversation, system will give warning tone and after this time the call will be disconnected. This timer is also used for call drop warning in Unsupervised Conference. |
| - 17 | - CO Call Restriction Timer | - 00 - 99 - (2 Digits) | 0 (min) | - Outgoing CO line call is allowed for this time. |
| - 18 | - CO Dial Delay Timer | - 00 - 99 - (2 Digits) | - 01 - (100ms) | - Voice connection to the outside party will be made after this timer. This can be used to prevent illegal dialing in case of slow response from the Central Office Line or PBX. |
| - 19 | - CO Release Guard Timer | - 001 – 150 - (3 Digits) | - 020 - (100ms) | - This timer controls the time necessary to guarantee idle loop state when the line is released. |
| - 20 | CO Ring Off Timer | 010 -150 (3 Digits) | 060 (100ms) | This timer is to secure time interval between incoming ringing signals so that the active ringing can be lasted in the system until this timer is expired. |
| - 21 | - CO Ring On Timer | - 1 – 9 - (1 Digit) | - 2 - (100ms) | - This timer controls the time necessary to detect an outside line as ringing into the system. |
| - 22 | - CO Warning Tone Timer | - 060 – 900 - (3 Digits) | - 180 - (sec) | - Determines the amount of time before receiving warning tone in order to remind the call elapsed time in case of outgoing CO line conversation (Only for Korea). |

TABLE 6.1.1  Button Configuration for System Timers - I  (PGM 180)

## 6.2	SYSTEM TIMERS - II  (PGM 181)

PROCEDURE

| SYSTEM TIMER 2 PRESS FLEX KEY (01-12) | - [TRANS/PGM] + 181. |
| --- | --- |
|  | - To program, use the BTNs as TABLE 6.2.1. Press one of BTN 1-12 and enter related data. (LED of selected Flex. BTN will be lit.) |
| - CFW NO ANS TMR(sec) - (000 – 255) : 015 | - To program the Call Forward No Answer Timer, press the Flex. BTN 1. The current status will be displayed on the LCD and user can enter a desired value. Then LCD shows the changed value |
| - SYSTEM TIMER 2 - PRESS FLEX KEY (01-12) | - Press the [HOLD/SAVE] button for updating database permanently. |
| SYSTEM TIMER 2 PRESS FLEX KEY (01-12) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| - BTN | - ITEM | - RANGE | - DEFAULT | - REMARK |
| --- | --- | --- | --- | --- |
| - 1 | Call Forward No Answer Timer | 000 -255 (3 Digits) | 015 (sec) | The Call Forward busy/no answer feature will occur with this timer. If this timer has a non-zero value and an extension is set at busy, no answer forward by station user then the extension will ring for this timer and take place a forward to the next. |
| - 2 | DID/DISA No Answer Timer | 00 -99 (2 Digits) | 20 (sec) | A DID call will be forwarded attendant if the station is busy or does not answer within this time. |
| - 3 | VMIB User Record Timer | 010 – 255 (3 Digits) | 20 (sec) | The time duration of VMIB user greeting |
| - 4 | VMIB Valid User Message Timer | 0-9 (1 Digits) | 4 (sec) | The time duration of valid VMIB user message In case of 0, No message can be recorded. |
| - 5 | Door Open Timer | 05 –99 (2 Digits) | 20 (100ms) | Determines the length of time that is needed to activate door open relay for the setting time. |
| - 6 | ICM Box Timer | 00 -60 (2 Digits) | 30 (sec) | Determines the amount of time programmed stations will ring when ICM box user presses the [CALL] button. |
| - 7 | ICM Dial Tone Timer | 01-20 (2 Digits) | 10 (sec) | If action is not taken within ICM dial tone timer, user will hear error-tone. |
| - 8 | Inter Digit Timer | 01-20 (2 Digits) | 05 | The time between digits cannot exceed Inter-digit timer, or error tone is received. |
| - 9 | MSG Wait Reminder Tone Timer | 00 -60 (2 Digits) | 00 | Determines the amount of time between repeated reminder tones to a key telephone with a message waiting. |
| - BTN | - ITEM | - RANGE | - DEFAULT | - REMARK |
| - 10 | Paging Timeout Timer | 000 –255 (3 Digits) | 15 | Determines the maximum time of a page. The system will automatically disconnect the page at the end of this time unless the caller has hung up earlier. |
| - 11 | - Pause Timer | - 1 – 9 - (1 Digit) | - 3 | - Determines the length of the pause for use with automatically sent digits or other speed dialing. |
| - 12 | - Preset Call Forward Timer | - 00 – 99 - (2 Digits) | - 10 | - Determines the amount of time an outside line will ring before being forwarded to a predetermined station. This entry works with Preset Forward Assignments in station attributes. More than one station can be forwarded to the same destination. |

TABLE 6.2.1  Button Configuration for System Timers - II  (PGM 181)

## 6.3	SYSTEM TIMERS - III  (PGM 182)

PROCEDURE

| SYSTEM TIMER 3 PRESS FLEX KEY (01-12) | - [TRANS/PGM] + 182. |
| --- | --- |
|  | - To program, use the BTNs as TABLE 6.3.1. Press one of BTN 1-12 and enter related data. (LED of selected Flex. BTN will be lit.) |
| - STA AUTO RLS TMR (sec) - (020 – 300) : 060 | - To program the Station Auto Release Timer, press the Flex. BTN 5. The current status will be displayed on the LCD and user can enter a desired value. Then LCD shows the changed value. |
| - SYSTEM TIMER 3 - PRESS FLEX KEY (01-12) | - Press the [HOLD/SAVE] button for updating database permanently. |
| SYSTEM TIMER 3 PRESS FLEX KEY (01-12) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| - BTN | - ITEM | - RANGE | - DEFAULT | - REMARK |
| --- | --- | --- | --- | --- |
| - 1 | SLT Hook Switch Bounce Timer | 01-25 (2 Digits) | 01 (100ms) | Determines the length of time that is needed to regard as a valid on-hook or off-hook (for SLT). |
| - 2 | SLT Maximum Hook Flash Timer | 01-25 (2 Digits) | 05 (100ms) | Determines how long the user could depress the hook switch in order for it to be considered a FLASH (Timed-Break Recall) (for SLT). |
| - 3 | SLT Minimum Hook Flash Timer | 000 -250 (3 Digits) | 020 (10ms) | The minimum bound time that system considers as hook flash for SLT. |
| - 4 | SLT Ring Phase Timer | 2 – 5 (1 Digit) | 5 (sec) | Determines the ring phase of SLT. (5 SEC: 1SEC ON / 4SEC OFF) |
| - 5 | Station Auto Release Timer | 020 – 300 (3 Digits) | 060 (sec) | If a station hears ring back tone and no action is taken, this timer is assigned. When this timer is expired, the station is released. |
| - 6 | - Unsupervised Conference Timer | - 00 - 99 - (2 Digits) | - 10 - (min) | - Determines the amount of the time an unsupervised conference can continue after the initiator of the conference has exited the conference. |
| - 7 | Wake-Up Fail Ring Timer | - 00 - 99 - (2 Digits) | 20 (sec) | After a Wake-up fail ring invokes on system attendant, the alarm ring exists during this timer. Then if this timer expires, the alarm ring will be disappeared. |
| - 8 | - Warm Line Timer | 010 - 200 - (2 Digits) | - 05 - (sec) | - User takes no action after lifting handset or pressing the [MON] button and warm line timer is expired, then idle line selection for warm line is activated. |
| - 9 | Wink Timer | 010 -200 (3 Digits) | 010 (10ms) | The time duration of seize acknowledge signal to DID line. |
| - 10 | Enblock Digit timer | 01-20 (2 Digits) | 15 (sec) | After timer is expired, setup is sent. |
| - 11 | CCR Time Out Timer | 000-300 | 015(sec) | When this timer is expired, CCR is activated. |
| - 12 | DID Inter Digit Timer | 01-20 | 03 | In DID type2, used as digit timer |

- TABLE 6.3.1  Button Configuration for System Timers - III  (PGM 182)

# 7.	DCOB ATTRIBUTE (PGM 186 - PGM 187)

## 7.1	DCOB SYS Attribute (PGM 186)

PROCEDURE

| DCOB SYS ATTRIBUTE PRESS FLEX KEY(01-12) | - [TRANS/PGM] + 186 |
| --- | --- |
| R2 OUT MANAG TMR(sec) (01 – 50) : 14 | - To program DCOB System Attribute, Press the Flex Btn 1-11. Related message will be displayed on LCD. (EX: Btn3) |
| R2 OUT MANAG TMR(sec) (01 – 50) : 30 | - To change the value, dial the value in the range (EX:30) |
| R2 OUT MANAG TMR(sec) (01 – 50) : 30 | - Press the [HOLD/SAVE] button for saving database permanently. |
| DCOB SYS ATTRIBUTE PRESS FLEX KEY(01-12) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | DCOB CO Type | 0-2 | 2 | 0:Sweden/Cyprus 1:Italy 2:Korea/Australia |
| 2 | Metering Type | 0-1 | 0 | 0:Not used 1:When received the Metering signal |
| 3 | R2 OUT Manage Timer | 01-50 | 14 | In R2 signaling, maximum time for waiting for forward signal from PX (1 sec) |
| 4 | R2 IN Manage Timer | 01-50 | 14 | In R2 signaling, maximum time for waiting for forward signal from PX (1 sec) |
| 5 | R2 Disappear Timer | 01-50 | 14 | 1 sec |
| 6 | R2 Pulse Timer | 01-30 | 7 | In R2 signaling, time duration to send pulse typed R2 signal (20 msec) |
| 7 | R2 Ready Timer | 000-500 | 7 | 20 msec |
| 8 | Dial Tone Delay Timer | 01-30 | 20 |  |
| 9 | Line Status | 1-9 | 6 | Free Line |
| 10 | Calling Category | 1-9 | 1 | User no priority |
| 11 | DNIS Service | ON/OFF | OFF | ON: Caller ID Service |
| 12 | CLI Digit Num | 01-10 | 4 | reserved |

- TABLE 7.1.1  DCOB System Attribute  (PGM 186)

## 7.2	DCOB CO Line Attribute  (PGM 187)

PROCEDURE

| DCOB COLINE ATTR ENTER COL RANGE | - [TRANS/PGM] + 187. |
| --- | --- |
| 001-002 DCOB CO ATTR PRESS FLEX_KEY (1-3) | - Enter CO line range. |
|  | - To program, Press BTN 1-3 and enter related data for setting, then entered data will be displayed on the LCD. |
| 001-002 IN DGT TYPE (0 – 2) : R2MFC | - To assign Incoming Digit Type, press Flex. BTN 1 and dial Digit Type.(Ex: dial 2) |
| 001-002 IN DGT TYPE (0 – 2) : R2MFC | - Press the [HOLD/SAVE] button for updating database permanently. |
| 001-002 DCOB CO ATTR PRESS FLEX_KEY (1-3) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (2) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | IN Digit Type | 0-2 | 2 | Default: R2MFC (2) To set type. [0 : PULSE, 1 : DTMF, 2 : R2MFC] |
| 2 | OUT Digit Type | 0-2 | 2 | Default: R2MFC(2) To set type. [0 : PULSE, 1 : DTMF, 2 : R2MFC] |
| 3 | CLI Digit Number | 01-15 | 10 | Set the digit numbers received for CLI |

- TABLE 7.2.1  DCOB CO Line Attribute (PGM 187)

# 8.	STATION GROUP PROGRAMMING (PGM 190 - PGM 191)

Stations in the system can be grouped so that incoming calls will search (hunt) for an idle station in the group. Three hunting processes can be assigned; Circular, Terminal, or UCD (Uniform Call Distribution). Each of the system's groups is assigned as a function; Call Pick-Up Group and/or Hunt Group, Voice Mail Group, and Ring Group. The available group number and station number in a group is as follows:

| System | LDK – 300 | LDK – 100 |
| --- | --- | --- |
| No. of Group | 48 | 15 |
| STA No. in a Group | 64 | 32 |

- A station can belong to any number of Pickup groups, but can only belong to one Station Hunt group, Voice mail group or Ring group.

- When assigning a station group to any type of hunt group or voice mail group, ring, pick up group, the system initializes hunt attributes by default value for it's own function. It can be programmed to meet each customer's individual need.

## 8.1	STATION GROUP ASSIGN  (PGM 190)

PROCEDURE

| STATION GRP ASSIGN ENT HUNT NO (620 – 667) |  | - [TRANS/PGM] + 190. In LDK-100, the range is 620-634. |
| --- | --- | --- |
| STATION GRP 620 F1:TYPE F2:PKUP F3:MEM |  | - Enter Station Group Number(Ex 620). Then user can set the type, pick-up feature and member of the selected group. First, press Flex. BTN 1 and set Type. (see TABLE 8.1.1) |
| STATION GRP 620 NOT ASSIGNED (0-6) |  | - To select Type, press one digit of 0 – 6 (Ex. 1). |
| STATION GRP 620 CIRCULAR GROUP (0-6) |  | - To save, press the [HOLD/SAVE] button for updating database. |
| GROUP 620 PICK-UP (1:ON/0:OFF) : OFF |  | - To set pick_up feature of the Circular Group, press Flex. BTN 2 and enter the desired value. |
| STATION GRP 620 F1:TYPE F2:PKUP F3:MEM |  | - To assign member of the Circular Group, press Flex. BTN 3. |
| CIRCULAR 620 . . . . . . . . . . . . . . . . |  | - Choose one of 4 Flex. BTNs, and enter station number step by step or enter the station range (Ex: 100120). |
|  | STATION GRP 620 F1:TYPE F2:PKUP F3:MEM | - Press the [HOLD/SAVE] button for updating database permanently. User can set another group by going step (1). |
|  | STATION GRP 620 F1:TYPE F2:PKUP F3:MEM | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| DGT | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Group Type | 0-6 | 0 | 0: Not Assigned 1: Circular 2: Terminal 3: UCD 4: Ring 5: VM 6: Pick up |
| 2 | Pick-up Attribute | ON/OFF | OFF | OFF |
| 3 | Member assignment | Not Assigned | - | First, group type should be assigned. |

TABLE 8.1.1  Button Configuration for Station Group Type  (PGM 190)

## 8.2	STATION GROUP ATTRIBUTE  (PGM 191)

### 8.2.1	CIRCULAR/TERMINAL GROUP ATTRIBUTE

The features of Terminal Group are same as that of Circular Group.

PROCEDURE

| STATION GRP ASSIGN ENT HUNT NO (620 – 667) |  | (1) [TRANS/PGM] + 191. In LDK-100, 620-634 is displayed. - Note: To program this, first, assign a station group to Circular/ Terminal hunt group by PGM 190. |
| --- | --- | --- |
| CIRC GRP 621 PRESS FLEX_KEY (01-13) |  | - Enter Station Group Number (Ex. 621, If user sets 621 to Circular or Terminal group). Press one of 13 Flex. BTNs and enter desired value of the group feature. |
| CIRC 621 ANNC 1 TMR (000 - 999) : 015 |  | - To assign VMIB Announce 1 Timer, press Flex. BTN 1 and enter three digits. Press the [HOLD/SAVE] button to save it. |
| CIRC 621 ANNC1 LOC VMIB MSG … (00 – 70) |  | - To assign VMIB Announce Location 1, press Flex. BTN 3 and enter two digits (VMIB Msg Number). Press the [HOLD/SAVE] button to save it. |
| CIRC 621 ANNC1 LOC VMIB MSG 065 (#) (00 – 70) |  | - To drop the call after VMIB announcement, dial digit ’#’. Press the [HOLD/SAVE] button to save it. |
| CIRC 621 ANNC2 RPT TMR (000 - 999) : 000 |  | - To assign VMIB Announce 2 Repeat Timer, press Flex. BTN 5 and enter three digits. Press the [HOLD/SAVE] button to save it. |
| CIRC 621 OVFLOW DEST STA/HUNT/VMIB (1-4) |  | - To assign Overflow Destination, press Flex. BTN 7 and enter one digit. - Dial ‘1’ to enter station number, - Dial ‘2’ to enter Hunt group number, - Dial ‘3’ to enter VMIB number - Dial ‘4’ to enter SYS SPD number - Press the [SPEED] button for deleting. Press the [HOLD/SAVE] button to save it. |
| CIRC 621 OVERFLOW DEST STA 105 |  | - For example, enter digit 1, and enter the station number (Ex. 105) as Overflow Destination. Press the [HOLD/SAVE] button to save it. |
| CIRC 621 OVERFLOW TMR (000 - 600) : 180 |  | - To assign Overflow Timer, press Flex. BTN 8 and enter three digits. Press the [HOLD/SAVE] button to save it. |
| CIRC 621 NO ANS TMR (00 – 99) : 15 |  | - To assign No Answer Timer, press Flex. BTN 10 and enter two digits (00-99). Press the [HOLD/SAVE] button to save it. |
| CIRC 621 PILOT HUNT (1 : ON/ 0: OFF) : OFF |  | - To assign Pilot Hunt, press Flex. BTN 11 and enter desired value. Press the [HOLD/SAVE] button to save it. |
| CIRC 621 ALT IF NO MBR (1 : ON/ 0: OFF) : OFF |  | - To assign Alternative if No Member, press Flex. BTN 12 and enter desired value. Press the [HOLD/SAVE] button to save it. |
| CIRC 621 MUSIC SRC (00 – 12) : 00 |  | - To assign Music Source, press Flex. BTN 13 and enter two digits. In LDK-100, the range is 00-11. Press the [HOLD/SAVE] button to save it. |
| CIRC GRP 621 PRESS FLEX_KEY (01-13) | - Terminal Group feature is just same as Circular group. Press the [HOLD/SAVE] button for updating database permanently. For another group’s setting, first assign type of group number by PGM 190 and go step (1) and continue this process. |  |
| CIRC GRP 621 PRESS FLEX_KEY (01-13) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |  |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | VMIB Announce 1 Timer | 000-999 | 015 (sec) | If this timer expires after a call is received in the group, the system announces the greeting if exists. |
| 2 | VMIB Announce 2 Timer | 000-999 | 000 (sec) | The second announcement can be provided if the call continues to wait beyond the 2nd announcement timer. |
| 3 | VMIB Announce Location 1 | 00-70 | 00 (Not Assigned) | This is used to announce greeting when the VMIB announce 1 timer is expired. |
| 4 | VMIB Announce Location 2 | 00-70 | 00 (Not Assigned) | This is used to announce greeting when the VMIB announce 2 timer is expired if assigned. |
| 5 | VMIB Announce 2 Repeat | 000-999 | 000 (sec) | This is used to repeat VMIB announce 2 when the timer is expired. (000: Not assigned) |
| 6 | VMIB Announce 2 Repeat Enable/Disable | ON/OFF | OFF | This is used to enable or disable VMIB Announce 2 Repeat. |
| 7 | Overflow Destination | STA #./ HUNT #./ VMIB # / SYS SPD # |  | The call to a station in the group will continue to route until answered or each station in the group has been tried. The call will remain at the last station in the group or will be passed to this overflow station/group/ VMIB/System Speed bin. |
| 8 | Overflow Timer | 000-600 | 180 (sec) | If this timer expires after a call is received in the group, the call is routed to the overflow destination. |
| 9 | Wrap-up Timer | 002-999 | 002 (sec) | A station in a hunt group is maintained in a busy state for min. 2 seconds after any call and for hunt group calls for the assigned wrap-up time. |
| 10 | No Answer Timer | 00-99 | 15 (sec) | In circular/terminal hunt, calls to a station in the group will go to the station, if unavailable or unanswered in this no answer time, the call is directed to the next station in the group. |
| 11 | Pilot Hunt | ON/OFF | ON | A circular/terminal hunt group can be assigned with a pilot number (the station group) so that only calls to the pilot number will hunt. |
| 12 | Alt If No Member | ON/OFF | OFF | If there is no member on duty, ICM call will be dropped or CO incoming call will be routed to ATD. |
| 13 | Music Source | 00-11(12) | 00 (Not Assigned) | If music source is assigned, calling user will be heard music instead of ring back tone. - 00: Not assigned - 01: Internal Music - 02~04: External Music - 5~6(7): VMIB BGM - 7(8)-11(12): SLT MOH |

TABLE 8.2.1.1  Button Configuration for Circular/Terminal Group Attribute (PGM 191)

### 8.2.2 	UCD GROUP ATTRIBUTE

PROCEDURE

|  | STATION GRP ASSIGN ENT HUNT NO (620 – 667) |  |  | - [TRANS/PGM] + 191. In LDK-100, 620-634 is displayed. - Note: To program this, first, assign a station group to UCD hunt group by PGM 190. |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | UCD GRP 623 PRESS FLEX_KEY (01-19) |  |  | - Enter Station Group Number (Ex 623, If user set 623 to UCD group at PGM 190). Press one of 19 Flex. BTNs and enter desired value of the group feature. |  |  |
|  | UCD 623 ANNC 1 TMR (000 – 999) : 000 |  |  | - To assign VMIB Announce 1 Timer, press Flex. BTN 1 and enter three digits. Press the [HOLD/SAVE] button to save it. |  |  |
|  | UCD 623 ANNC1 LOC VMIB MSG . . . (00 – 70) |  |  | - To assign VMIB Announce Location 1, press Flex. BTN 3 and enter two digits (VMIB MSG Number). Press the [HOLD/SAVE] button to save it. |  |  |
|  | UCD 623 ANNC2 RPT TMR (000 – 999) : 000 |  |  | - To assign VMIB Announce 2 Repeat, press Flex. BTN 5 and enter three digits. Press the [HOLD/SAVE] button to save it. |  |  |
|  | UCD 623 ACD WARN TONE (1 : ON/ 0: OFF) : OFF |  |  | - To assign ACD Warning Tone, press Flex. BTN 12 and enter desired value. Press the [HOLD/SAVE] button to save it. |  |  |
|  | UCD 623 ALTER DEST (STA/HUNT) |  |  | - To assign Alternate Destination, press Flex. BTN 14 and enter one digit. - Dial '1' and enter station number, - Dial ‘2’ and enter Hunt group number, - Press the [SPEED] button for deleting. Press the [HOLD/SAVE] button to save it. |  |  |
|  | UCD 623 SUPERVISOR STA … |  |  | - To assign Supervisor, press Flex. BTN 17 and enter station number. - Press the [SPEED] button for deleting. Press the [HOLD/SAVE] button to save it. |  |  |
|  | 100 110 123 124 0 0 0 0 |  |  | - To assign UCD priority, press Flex. BTN 18 and press Flex. BTN 1-4 to enter the priority (0 – 9). |  |  |
|  | UCD GRP 623 PRESS FLEX_KEY (01-19) |  |  | - Press the [HOLD/SAVE] button for updating database permanently. For another group’s setting, first assign the type of the group number by PGM 190 and go step (1) and continue this process. |  |  |
|  | UCD GRP 623 PRESS FLEX_KEY (01-19) |  |  | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |  |  |
| BTN |  | ITEM | RANGE |  | DEFAULT | REMARK |
| 1 |  | VMIB Announce 1 Timer | 000 – 999 (3 Digits) |  | 015 (sec) | If all stations in the group are busy when a call is received for the group, the call may continue to wait (queue) for an available station in the group. If queued, the call may be sent to a UCD announcement when the queue period exceeds the 1st announcement Timer. If the timer is set to 0 the call will receive the full first announcement prior to the hunting process (guaranteed announcement). |
| 2 |  | VMIB Announce 2 Timer | 000 - 999 (3 Digits) |  | 000 (sec) | The second announcement can be provided if the call continues to wait beyond the 2nd announcement timer. |
| 3 |  | VMIB Announce Location 1 | 00-70 |  | 00 (Not Assigned) | Each station hunt group can be assigned an announcement, which is played when the call is first received. The announcement may be assigned as VMIB. |
| 4 |  | VMIB Announce Location 2 | 00-70 |  | 00 (Not Assigned) | The second announcement can be provided after VMIB Announce 2 Timer. |
| 5 |  | VMIB Announce 2 Repeat | 000-999 |  | 000 (sec) | This is used to announce VMIB announce 2 when the timer is expired. |
| 6 |  | VMIB Announce 2 Repeat Enable/Disable | ON/OFF |  | OFF | This is used to enable or disable VMIB Announce 2 Repeat. |
| 7 |  | Overflow Destination | STA #./ STA Grp #/ VMIB# / SYS SPD # |  | - | The queued call may be taken out of the group and directed to an overflow destination (Station/Hunt/VMIB/ System Speed bin). |
| 8 |  | Overflow Timer | 000 - 600 (3 Digits) |  | 180 (sec) | If this timer expires after a call is received in the group, the call is routed to the overflow destination. |
| 9 |  | Wrap-up Timer | 002 - 999 (3 Digits) |  | 002 (sec) | A station in a hunt group is maintained in a busy state for min. 2 seconds after any call for the assigned wrap-up time. |
| 10 |  | Alt If No Member | ON/OFF |  | OFF | If there is no member on duty, ICM call will be dropped or CO incoming call will be routed to ATD |
| 11 |  | Music Source | 00-11(12) |  | 00 | If music source is assigned, calling user will be heard music instead of ring back tone. - 00: Not assigned - 01: Internal Music - 02~04: External Music - 5~6(7): VMIB BGM 7(8)-11(12): SLT MOH |
| 12 |  | ACD Warning Tone | ON/OFF |  | ON | Determines that the ACD supervisor monitors an agent with warning tone or without warning tone |
| 13 |  | Alternate destination | STA No/ HUNT # |  | - | When a call is received in the group and there is no available station in the group, then the call will be routed to this destination if assigned. |
| 14 |  | Supervisor Timer | 000 – 999 (3 Digits) |  | 030 (sec) | When the queued timer is longer than this timer, the number of queued lines will be displayed onto supervisor's LCD. |
| 15 |  | Supervisor Call Count | 00 - 99 (2 Digits) |  | 00 | If the number of queued calls is more than this call count, the supervisor timer will be started. |
| 16 |  | ACD Queued Call | ON/OFF |  | OFF | Show the number of queued call to supervisor keyset |
| 17 |  | Max Que Call Cnt | 00-99 |  | 00 |  |
| 18 |  | Supervisor | STA # |  | - | Station No. of Supervisor |
| 19 |  | UCD hunt Stations' Priority | 0 - 9 (1 Digit) |  | 0 | UCD group member's priority |

TABLE 8.2.2.1  Button Configuration for UCD Group Attribute  (PGM 191)

### 8.2.3	RING GROUP ATTRIBUTE

PROCEDURE

| STATION GRP ASSIGN ENT HUNT NO (620 – 667) |  | - TRANS/PGM] + 191. In LDK-100, 620-634 is displayed. - Note: To program this, first, assign a station group to Ring hunt group by PGM 190. |
| --- | --- | --- |
| RING GRP 624 PRESS FLEX_KEY (1-10) |  | - Enter Station Group Number (Ex. 624, If user set 624 to Ring group). Press one of 10 Flex. BTNs and enter desired value of the group feature. |
| RING 624 ANNC 1 TMR (000 – 999) : 015 |  | - To assign VMIB Announce 1 Timer, press Flex. BTN 1 and enter three digits. |
| RING 624 ANNC1 LOC VMIB MSG . . . ( 00 – 70 ) |  | - To assign VMIB Announce Location 1, press BTN 3 and enter VMIB number. Press the [HOLD/SAVE] button to save it. |
| RING 624 ANNC 2 RPT (1 : ON/ 0: OFF) : OFF |  | - To assign VMIB Announce 2 Repeat E/D, choose flex BTN 6 and enter desired value. Press the [HOLD/SAVE] button to save it. |
| RING 624 MUSIC SRC (00 – 12) : 00 |  | - To assign Music Source, choose flex BTN 10 and enter two digits. In LDK-100, the range is 00-11. Press the [HOLD/SAVE] button to save it. |
| RING GRP 624 PRESS FLEX_KEY (1-10) |  | - Press the [HOLD/SAVE] button for updating database permanently. For another group’s setting, first assign type of a group number by PGM 190 and go step (1) and continue this process. |
|  | RING GRP 624 PRESS FLEX_KEY (1-10) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | VMIB Announce 1 Timer | 000-999 | 015 (sec) | If this timer expires after a call is received in the group, the system announces the greeting if exists. |
| 2 | VMIB Announce 2 Timer | 000-999 | 000 (sec) | The second announcement can be provided if the call continues to wait beyond the 2nd announcement timer. |
| 3 | VMIB Announce Location 1 | 00-70 | 00 (Not Assigned) | This is used to announce the greeting when the VMIB announce 1 timer is expired. |
| 4 | VMIB Announce Location 2 | 00-70 | 00 (Not Assigned) | This is used to announce the greeting when the VMIB announce 2 timer is expired. |
| 5 | VMIB Announce 2 Repeat | 000-999 | 000 (sec) | This is used to announce the greeting 2 when the timer is expired. (1 Sec Base) |
| 6 | VMIB Announce 2 Repeat Enable/ Disable | ON/OFF | OFF | This is used to enable or disable VMIB Announce 2 Repeat. |
| 7 | Overflow Destination | STA#./ HUNT#/ VMIB# / SYS SPD # |  | The call to a station in the group will continue to route until answered or each station in the group has been tried. The call will remain at the last station in the group or will be passed to this overflow destination (Station/Hunt/VMIB/System Speed bin). |
| 8 | Overflow Timer | 000-600 | 180 (sec) | If this timer expires after a call is received in the group, the call is routed to the overflow destination. |
| 9 | Wrap Up Timer | 002-999 (3 digits) | 002 (sec) | A station in a hunt group is maintained in a busy state for min. 2 seconds after any call for the assigned wrap-up time. |
| 10 | Music Source | 00-11(12) | 00 | If music source is assigned, calling user will be heard music instead of ring back tone. - 00: Not assigned - 01: Internal Music - 02~04: External Music - 5~6(7): VMIB BGM 7(8)-11(12): SLT MOH |
| 11 | Max. Queued Call Count | 00-99 | 00 | Maximum number of queued call in the ring group |

TABLE 8.2.3.1  Button Configuration for Ring Group Attribute (PGM 191)

### 8.2.4	VM GROUP ATTRIBUTE

PROCEDURE

| STATION GRP ASSIGN ENT HUNT NO (620 – 667) | - [TRANS/PGM] + 191. In LDK-100, 620-634 is displayed. - Note : To program this, first, assign a station group to VM group by PGM 190. |
| --- | --- |
| VM GRP 626 PRESS FLEX_KEY (1-7) | - Enter Station Group number (Ex. 626, If user sets 626 to VM group). Press one of 7 Flex. BTNs and enter desired value of the group feature. |
| VM 626 WRAP UP TMR (006 – 999) : 002 | - To assign Wrap-up Timer, press Flex. BTN 1 and enter three digits. Press the [HOLD/SAVE] button to save it. |
| VM 626 PUT MAIL INDEX (1 – 4) : 1 | - To assign Put Mail Index, press Flex. BTN 2 and enter one digit (1-4). Press the [HOLD/SAVE] button to save it. |
| VM 626 HUNT TYPE (1 : CIRC/ 0 : TERM) : TERM | - To assign Hunt Type, press Flex. BTN 4 and enter desired value. Press the [HOLD/SAVE] button to save it. |
| VM 626 OVERFLOW DEST STA/HUNT/VMIB (1-4) | - To assign Overflow Destination, press Flex. BTN 7 and enter one digit. - Dial ‘1’ to enter station number, - Dial ‘2’ to enter Hunt group number, - Dial ‘3’ to enter VMIB number, - Dial ‘4’ to enter System Speed number. - Press the [SPEED] button for deleting. Press the [HOLD/SAVE] button to save it. |
| VM 626 OVERFLOW DEST STA 147 | - For example, dial 1 and enter station number 147. Press the [HOLD/SAVE] button to save it. |
| VM GRP 626 PRESS FLEX_KEY (1-7) | - Press the [HOLD/SAVE] button for updating database permanently. For another group’s setting, first assign type of a group number by PGM 190 and go step (1) and continue this process. |
| VM GRP 626 PRESS FLEX_KEY (1-7) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Wrap-up Timer | 002-999 (3 Digits) | 002 (sec) | A station in a hunt group is maintained in a busy state for min. 2 seconds after any call and for hunt group calls for the assigned wrap-up time. |
| 2 | Put Mail Index | 1 -4 | 1 | This index is one of the voice mail dialing tables |
| 3 | Get Mail Index | 1 -4 | 2 | This index is one of the voice mail dialing tables |
| 4 | Hunt Type | CIRC /TERM | TERM | 1: Circular Hunt Group 0: Terminal Hunt Group |
| 5 | SMDI Port | 01-13 (LDK-300) 01-11 (LDK-100) | 02 (LDK-300) 01 (LDK-100) |  |
| 6 | Overflow Timer | 000 -600 (3 Digits) | 180 (sec) | If this timer expires after a call is received in the group, the call is routed to the overflow destination. |
| 7 | Overflow Destination | STA #/ HUNT #/ VMIB# / SYS SPD # | - | The call to the group will continue to be reroute until reaching the last station in the group where the call will remain or can be sent to this overflow destination (Station/Hunt group/VMIB/ System Speed bin). |

TABLE 8.2.4.1  Button Configuration for Voice Mail Group Attribute (PGM 191)

### 8.2.5	PICK-UP GROUP ATTRIBUTE

PROCEDURE

| STATION GRP ASSIGN ENT HUNT NO (620 – 667) | - [TRANS/PGM] + 191. In LDK-100, 620-634 is displayed. - Note: To program this, first, assign a station group to Pick Up group by PGM 190. |
| --- | --- |
| PICK UP GRP 625 PRESS FLEX_KEY (1-2) | - Enter Station Group number (Ex. 625, If user sets 625 to Pick-up group). Press one of 2 Flex BTNs and enter desired value of the group feature. |
| PICK UP 625 AUTO PICK UP (1 : ON/ 0: OFF) : OFF | - To assign Auto Pick-up, press Flex. BTN 1 and enter desired value. Press the [HOLD/SAVE] button to save it. |
| PICK UP 625 ALL RING (1 : ON/ 0: OFF) : OFF | - To assign All Ring, press Flex. BTN 2 and enter desired value. Press the [HOLD/SAVE] button to save it. |
| PICK UP GRP 625 PRESS FLEX_KEY (1-2) | - Press the [HOLD/SAVE] button for updating database permanently. For another group’s setting, first assign type of a group number by PGM 190 and go step (1) and continue this process. |
| PICK UP GRP 625 PRESS FLEX_KEY (1-2) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT (LED) | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Auto Pickup | ON/OFF | OFF | If a hunt member is ringing, another hunt member can pickup the call automatically only by pressing [MON] button or off-hook. |
| 2 | All Ring | ON/OFF | OFF | When a hunt member that is Tone mode is ringing, all other stations are ringing also. Auto Pickup feature must be set before All Ring is set. |

TABLE 8.2.5.1  Button Configuration for Pick-up Group Attribute  (PGM 191)

# 9.	ISDN PROGRAM

If ISDN system parameters are to be changed, press the [TRANS/PGM] button and dial 200-202. When programming, LCD and LEDs indicate current programmed data and programming status. The programmer enters correct data, then LCD and LEDs show the entered data and the data is stored in the temporary buffer area. Pressing the [HOLD/SAVE] button, all data in the temporary buffer (same as LCD/LEDs show their status) are stored into permanent ISDN memory.

## 9.1	ISDN ATTRIBUTE  (PGM 200)

PROCEDURE

| SYSTEM ISDN ATT PRESS FLEX KEY (1-10) | - [TRANS/PGM] + 200. If user wants to program the ISDN system attributes press one of Flex. BTNs of 1-7. |
| --- | --- |
| - ADVICE OF CHARGE (0-6) - FINLAND(2) | - To program AOC type, press the Flex. BTN 1 and dial one digit. Then the current status will be displayed on the LCD. |
| - CO ATD CODE (2DGT) - . . | - To program the CO Attendant code, press the Flex. BTN 2 and dial max. 2 digits. |
| - IN PREFIX CODE INS - (1:ON/0:OFF) : OFF | - To program the Incoming Prefix Code Insertion, press the Flex. BTN 3. Then the current status will be displayed on the LCD. |
| - ISDN LINE TYPE - (1:U/0:A): A_LAW | - To select ISDN Line Type, press the Flex. BTN 5 and dial 0 or 1. (1: -LAW 0: A-LAW) |
| - INTERNATION ACC CODE 001 | - To program the International Access Code, press the Flex. BTN 7. Then the current status will be displayed on the LCD. To change the data, dial the desired value (Max. 4 digits). |
| SYSTEM ISDN ATT - PRESS FLEX KEY (1-10) | - Press the [HOLD/SAVE] button for updating database permanently. |
| SYSTEM ISDN ATT - PRESS FLEX KEY (1-10) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Advice of Charge | 0-6 | - 0 | 0: Do not service AOC 1: Italy and Spain 2: Finland 3: Australia 4: Belgium 5: Standard 6: Netherlands |
| 2 | CO ATD Code | Max. 2 Digits | - - | According to PGM114 – BTN 5, CO ATD code or station number can be contained to CLI, COLP message. |
| 3 | Incoming prefix code Insertion | ON/OFF | OFF(NO) | If this field is ON, prefix code will be attached in front of incoming phone number. |
| 4 | Outgoing prefix code Insertion | ON/OFF | ON (YES) | If this field is ON, prefix code will be attached in front of outgoing phone number. |
| - 5 | - ISDN Line Type | -Law/ A-Law | A-Law (OFF) | Installed ISDN Back bone type |
| 6 | CLI print | ON/OFF | OFF (NO) | If this field is ON, the CLI will be sent to RS-232C port regardless setting the CLIP. |
| - 7 | - International Access Code | Max. 4 Digits | - | International Access Code Assign |
| - 8 | - Calling Sub-address | ON/OFF | OFF (NO) |  |
| - 9 | - My Area Code | Max. 6 Digits | - | Local area code |
| - 10 | - My Area Prefix Code | Max. 4 Digits | - | Prefix code of local area code |

TABLE 9.1.1  ISDN Attributes  (PGM 200)

## 9.2	COLP TABLE  (PGM 201)

PROCEDURE

| COLP TABLE ENTRY ENTER BIN NO ( 00 – 49 ) | - [TRANS/PGM] + 201. |
| --- | --- |
| - COLP TABLE 05 - ………. | - To program COLP table, dial Bin No (00 – 49). |
| COLP TABLE ENTRY - ENTER BIN NO ( 00 – 49 ) | - Press the [HOLD/SAVE] button for updating database permanently. |
| COLP TABLE ENTRY - ENTER BIN NO ( 00 – 49 ) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | DEFAULT | RANGE | REMARK |
| --- | --- | --- | --- | --- |
|  | COLP | - - | Max. 10 digits | In CO PGM part, PGM 143-BTN1, BTN3 indicate an entry of this COLP table index. |

TABLE 9.2.1  COLP Table  (PGM 201)

## 9.3	MSN TABLE  (PGM 202)

PROCEDURE

| MSN TABLE ATT ENTER BIN NO ( 000 – 249 ) | - [TRANS/PGM] + 202. |
| --- | --- |
| - MSN TABLE 125 - PRESS FLEX_KEY ( 1 – 4 ) | - To enter MSN table, dial the digits (Ex. 125). |
| - MSN TABLE 125 - COL NO : 001 | - To enter CO line number, press Flex. BTN 1 and the dial digits (Ex. 001). |
| MSN TABLE 125 INDEX : 100 | - To enter index number of PGM 231, press Flex. BTN 2, and dial the index number (Ex. 100). |
| - MSN TABLE 125 - SUB NO : 1 | - To enter Sub-address number, press Flex. BTN 3, and dial Sub-address number (Ex. 1). |
| TABLE 125 MSN NUMBER …………………….. | - To enter Incoming MSN telephone number, press Flex. BTN 4 and dial MSN number that is provided by PX. |
| MSN TABLE ATT ENTER BIN NO ( 000 – 249 ) | - Press the [HOLD/SAVE] button for updating database permanently. |
| MSN TABLE ATT ENTER BIN NO ( 000 – 249 ) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| - 1 | - CO Line No. | - 001-200 (LDK-300) - 01-40 (LDK-100) | - None |  |
| - 2 | - Index of Flexible - DID Table | - 000-999 | - None | - If Incoming CO line number and MSN number are matched or only MSN number is matched with Table entry, it will follow the assigned Flexible DID Table. |
| - 3 | - Sub Number | - 0-9 | - None | - MSN Subscriber number |
| - 4 | - MSN Number | - 20 Digits | - None | - ISDN Incoming MSN number |

- TABLE 9.3.1  Button Configuration of MSN Table  (PGM 202)

# 10.	LCR

To program LCR, press the [TRANS/PGM] button and dial 220 - 223. There are 4 parts in the LCR table. User can program general database, LCR access mode, day zone and time zone by PGM 220. PGM 221 is for Leading Digit Table and PGM 222 is for Digit Modification Table. The stored database for LCR may be initialized by PGM 223.

## 10.1	LCR ATTRIBUTES  (PGM 220)

PROCEDURE

| LCR CONTROL ATTRIBUTES PRESS FLEX_KEY (1-5) | - [TRANS/PGM] + 220. |
| --- | --- |
| LCR ACCESS MODE (1-6) (M00) DISABLE LCR | - Flex BTN 1: Change LCR access mode Dialing digit 1-6, the LCD will show the changed value M00, M01, M02, M11, M12 or M13. See the Table 10.1.1. |
| LCR ACCESS MODE (1-6) (M02) INT AND LOOP LCR | - For example, dial 3 and press the [HOLD/SAVE] button to save the changed data. |
| DAY ZN 1:1234567 2: 3: M1 T2 W3 T4 F5 SA6 SU7 | - Flex BTN 2: Set the day-of-week zone To set the day zone, dial the associated zone number following the Flex. BTN related with weekday. (BTN 1 for MON, BTN 2 for TUE, BTN 3 for WED, etc.) For example, if you want to set Saturday as zone 2, then press BTN 6, dial 2 and press the [HOLD/SAVE] button. |
| DAY ZN 1:123457 2:6 3: MON : ZONE 3 (1–3 ) | - Confirmation tone is heard and the LCD will be changed. Ex) To set the Monday as zone 3, press BTN 1. LCD shows current zone of Monday. And dial the desired zone 3. The changed value will be displayed on the LCD. |
| DAY ZN 1:23457 2:6 3:1 M1 T2 W3 T4 F5 SA6 SU7 | - Press the [HOLD/SAVE] button. Confirmation tone is heard and LCD will be changed. |
| TIME ZONE 1 1:00-24 2: . .-. . 3: . .-. . | - Flex BTN 3: Set the time-of-day zone 1 |
| TOD1 (24HOUR FORM) 1:00-24 2:..-.. 3:..-.. | - To enter value, choose the zone by pressing Flex. BTN and dial the time range (BTN1 for zone 1, BTN 2 for zone 2, BTN 3 for zone 3). For example, if you want to set time zone as Zone 1: 08-18 Zone 2: 18-24 Zone 3: 24-08, then, press BTN 1, dial 0818 and press the [HOLD/SAVE] button, BTN 2, dial 1824 and [HOLD/SAVE] button, and press BTN 3, dial 2408, and [HOLD/SAVE] button. |
| TOD1 (24HOUR FORM) 1:08-18 2:18-24 3:..-.. | - Pressing the [HOLD/SAVE] button, LCD will be changed in order with confirmation tone. |
| TIME ZONE 2 1:00-24 2: . .-. . 3: . .-. . | - Flex BTN 4: Set the time-of-day zone 2 |
| TOD1 (24HOUR FORM) 1:00-24 2:..-.. 3:..-.. | - To enter value, choose zone by pressing Flex. BTN and dial the time range. |
| TIME ZONE 3 1:00-24 2: . .-. . 3: . .-. . | - Flex BTN 5: Set the time-of-day zone 3. To enter value, choose zone by pressing Flex. BTN and dial the time range. |
| LCR CONTROL ATTRIBUTES PRESS FLEX_KEY (1-5) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| - BTN | - ITEM | - RANGE | - DEFAULT | - REMARK |
| --- | --- | --- | --- | --- |
| - 1 | - LCR Access | M00/01/02/11/12/13 | - Disable - (M00) | - LCR Access Mode 00 (M00): Disable LCR - LCR Access Mode 01 (M01): only Loop LCR. - LCR Access Mode 02 (M02): Internal and Loop LCR. - LCR Access Mode 11 (M11) : Loop and Direct CO LCR - LCR Access Mode 12 (M12): Internal, Loop and Direct CO LCR - LCR Access Mode 13 (M13): Internal, Loop, Direct CO and Direct Loop LCR |
| - 2 | - Day Zone | Zone : 3 - Day : 1 - 7 | - Belongs to Zone 1 | - First, select day and choose zone |
| - 3 | - Time Zone | - Time : 00 – 24 | - Belongs to Zone 1 | - LDK accepts it as same value for 00 and 24 and changes to “00”, if input is 24 as starting value and vice versa. - *Note : The time not belonging to any zone will be considered as zone 1 - *Note : 10 - 13 means 10:00:00 - 12:59:59 |

- TABLE 10.1.1  LCR Table  (PGM 220)

## 10.2	LEADING DIGIT TABLE  (PGM 221)

PROCEDURE

| LDT TABLE ENTER LDT BIN (000) | - [TRANS/PGM] + 221. Dial bin number 000 – 249. (000 means that LDT is empty.) |  |
| --- | --- | --- |
| 000 BOTH CD: . . . . . . . . . . . . . . DMT: . . . . . . . . . . . . . . . . . . | - Program the Leading Digit Table - Press BTN 1 to select one of BOTH, INT, COL. - CD: Dial the leading digits following BTN 2 - DMT (dependent to day/time zone) For Day zone 1 : Press BTN 3 and 6 digits (6 digits: each pair (2 digits) means the index to the DMT for the each time zone 1/2/3) For Day zone 2: Press BTN 4 and 6 digits For Day zone 3: Press BTN 5 and 6 digits (The [SPEED] button is used to erase the entered data in BTN 2, BTN 3 BTN 4 and BTN 5. Press the [SPEED] button and press the [HOLD/SAVE] button to erase the data.) Note: - If the entered data is correct, system will sort the data set by some criteria. (Code/LCR Type/DMT Index) So, when user programs PGM 221 to enter LDT values, the first available bin will be displayed. - When user enters data, the updated data will be displayed on the LCD. But the time that actually LDK system accepts and sort the data is when the condition (described in Note.1) is satisfied. - The modified one may be stored into a different bin, since LDK ascending sort for the faster lookup. - Refer to the Index of DMT (See the Table 9.2.1) |  |
| LDT 000 : LCR TYPE LCR MODE : COL (3) | - Flex BTN 1: Change LCR access mode Dial one digit 1-3 to select INT(1), COL(2) or BOTH(3). Press the [HOLD/SAVE] button to save the changed data. |  |
| 000 COL CD: . . . . . . . . . . . . . . DMT: . . . . . . . . . . . . . . . . . . | - Confirmation tone is heard and updated data will be displayed on the LCD. |  |
| LDT 000 : LCR CODE 1234567 | - Flex BTN 2: Dial Leading digits Press the [HOLD/SAVE] button to save the changed data. |  |
| 000 BOTH CD: 1234567E DMT: . . . . . . . . . . . . . . . . . . | - Confirmation tone is heard and updated data will be displayed on the LCD. |  |
| LDT 000 : DAY ZONE 1 DMT INDEX(6DGT): . . . . . . | - Flex BTN 3: Set the time-of-day zone 1 To enter each time zone (1/2/3), dial 6 digits continuously. |  |
| LDT 000 : DAY ZONE 1 DMT INDEX(6DGT): 00 99 55 | - Press the [HOLD/SAVE] button to update the data. |  |
| 000 BOTH CD: 1234567E DMT: 009955 . . . . . . . . . . . . | - Confirmation tone is heard and updated data will be displayed on the LCD. |  |
| LDT 000 : DAY ZONE 2 DMT INDEX(6DGT): . . . . . . | - Flex BTN 4: Set the time-of-day zone 2 To enter each time zone (1/2/3), dial 6 digits continuously. |  |
| LDT 000 : DAY ZONE 1 DMT INDEX(6DGT): 12 34 56 | - Press the [HOLD/SAVE] button to update the data. |  |
| 000 BOTH CD: 1234567E DMT: 009955 123456 . . . . . . | - Confirmation tone is heard and updated data will be displayed on the LCD. |  |
| LDT 000 : DAY ZONE 2 DMT INDEX(6DGT): . . . . . . | - Flex BTN 5: Set the time-of-day zone 3 To enter each time zone (1/2/3), dial 6 digits continuously. |  |
| LDT 000 : DAY ZONE 1 DMT INDEX(6DGT): 22 33 44 | - Press the [HOLD/SAVE] button to update the data. |  |
| 000 BOTH CD: 1234567E DMT: 009955 123456 223344 | - Confirmation tone is heard and updated data will be displayed on the LCD. |  |
| 011 BOTH CD: 1234567E DMT: 009955 123456 223344 | Note: When pressing the [HOLD/SAVE] button in individual program mode, if system already has LDT data sets and input data can be accepted and sorted, the system will return another bin after sorting the data set. |  |
| LDT TABLE ENTER LDT BIN (000) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |  |

| - BTN | - ITEM | - RANGE | - DEFAULT | - REMARK |
| --- | --- | --- | --- | --- |
| - 1 | - LCR Type | - INT - COL - (3) BOTH | - BOTH | - INT: look up this entry only for internal dialing - COL: look up this entry only after dialing CO Access Code - BOTH: look up this entry for both INT and COL. |
| - 2 | - CD | - 12 digits | - None | - To be compared with the dialed digits by user. |
| - 3 | - DMT index | - 00 - 99 | - None | - Day Zone 1, 2, 3 has 3 time zone DMT index (6 digits) |

- TABLE 10.2.1  Leading Digit Table  (PGM221)

## 10.3	DIGIT MODIFICATION TABLE  (PGM 222)

PROCEDURE

| DMT TABLE ENTER DMT BIN (00-99) | - [TRANS/PGM] + 222. Dial bin number 00 – 99 |  |
| --- | --- | --- |
| 55 A: . . . . . . . . . . . . . . . . . . . . . RP01 NR00 AP01 CG01 AD . . | - A (Added digit stream) Dial the digit stream following BTN 1. - RP (Removal Position) Dial xx following BTN 2. - RN (Removal Number of digits of code in each table) Dial xx following BTN 3. - AP (Add Position) Dial xx following BTN 4. - CG (CO group) Dial CO group xx following BTN 5. - AD (Alternative DMT index) Dial xx following BTN 6. (See Table 9.3.1) |  |
| DMT 55 : ADDED DGT . . . . . . . . . . . . . . . . . . . . . . . . | - Flex BTN 1: Set the Added digit stream |  |
| DMT 55 : ADDED DGT 1234567890 | - Dial digits (to 25 digits) To erase the entered data, press the [SPEED] button. Press the [HOLD/SAVE] button to update the changed data. |  |
| 55 A: 1234567890E RP01 NR00 AP01 CG01 AD . . | - Confirmation tone is heard and updated data will be displayed on the LCD. |  |
| DMT 55 REMOVAL POSITION (01-12) : 01 | - Flex BTN 2: Set the Removal Position Dial 2 digits (01-12). |  |
| 55 A: 1234567890E RP10 NR00 AP01 CG01 AD . . | - For example, dial 10 and press the [HOLD/SAVE] button. Confirmation tone is heard and updated data will be displayed on the LCD. |  |
| DMT 55 NUM OF REMOVE DIGITS (01-12) : 00 | - Flex BTN 3: Set the Removal Number of digits of code in each table. Dial 2 digits (01-12). |  |
| 55 A: 1234567890E RP10 NR07 AP01 CG01 AD . . | - For example, dial 07, and press the [HOLD/SAVE] button. Confirmation tone is heard and updated data will be displayed on the LCD. |  |
| DMT 55 ADD POSITION (01-13) : 01 | - Flex BTN 4: Set the Add Position Dial 2 digits (01-13). |  |
| 55 A: 1234567890E RP10 NR07 AP05 CG01 AD . . | - For example, dial 05, and press the [HOLD/SAVE] button. Confirmation tone is heard and updated data will be displayed on the LCD. |  |
| DMT 55 CO GROUP (01-72) : 01 | - Flex BTN 5: Set the CO group. In LDK-100, the range is 01-24. Dial 2 digits. |  |
| 55 A: 1234567890E RP10 NR07 AP05 CG10 AD . . | - For example, dial 10, and press the [HOLD/SAVE] button. Confirmation tone is heard and updated data will be displayed on the LCD. |  |
| DMT 55 ALT INDEX (00-99) : . . | - Flex BTN 6: Set the Alternative DMT index. Dial 2 digits (00-99). |  |
| 55 A: 1234567890E RP10 NR07 AP05 CG10 AD 77 | - For example, dial 77, and press the [HOLD/SAVE] button. Confirmation tone is heard and updated data will be displayed on the LCD. |  |
| DMT TABLE ENTER DMT BIN (00-99) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. - To erase the value in every step, press the [SPEED] button and press the [HOLD/SAVE] button. |  |

| - BTN | - ITEM | - RANGE | - DEFAULT | - REMARK |
| --- | --- | --- | --- | --- |
| - 1 | - Added Digit Stream | - 20 digits | - None | - Normal digits (0 .. 9, * , #) - Special characters [CALLBK]: Pause [DND/FOR]: Dial-tone-detection instead of pause [FLASH]: Billing code (Station Number) |
| 2 | Removal Position | 01-12 | 01 | Index to CD stream in Lead table to be removed |
| 3 | Number of digits to be removed | 00-12 | 00 | Removes digits in CD stream up to this count. |
| 4 | Add Position | 01-13 | 01 | Determines the position of CD stream after removal, where the stream will be inserted. |
| 5 | CO Group | 01-72 (LDK-300) 01-24 (LDK-100) | 01 | Determines which CO group is used for LCR dialing |
| 6 | Alternative DMT Index | 00-99 | None | Determines alternative DMT index when there is no idle CO line in the CO group. |

TABLE 10.3.1  Digit Modification Table  (PGM222)

## 10.4	LCR TABLE INITIALIZATION  (PGM 223)

PROCEDURE

| INITIALIZE LCR DB PRESS FLEX_KEY (1-6) | - [TRANS/PGM] + 223. |
| --- | --- |
| ENTER DMT INIT VAL(6) DAY ZONE 1: . . . . . . | - Flex. BTN 1: Initialize all DMT index of Day Zone 1 in LDT. |
| ENTER DMT INIT VAL(6) DAY ZONE 1: 00 11 22 | - Dial 6 digits continuously. And press the [HOLD/SAVE] button to initialize the data. |
| INITIALIZE LCR DB PRESS FLEX_KEY (1-6) | - Confirmation tone will be heard and the system goes to previous menu. |
| ENTER DMT INIT VAL(6) DAY ZONE 2: . . . . . . | - Flex. BTN 2: Initialize all DMT index of Day Zone 2 in LDT. |
| ENTER DMT INIT VAL(6) DAY ZONE 2: 55 77 99 | - Dial 6 digits continuously. And press the [HOLD/SAVE] button to initialize the data. |
| ENTER DMT INIT VAL(6) DAY ZONE 3: . . . . . . | - Flex BTN 3: Initialize all DMT index of Day Zone 3 in LDT. |
| ENTER DMT INIT VAL(6) DAY ZONE 3: 88 44 22 | - Dial 6 digits continuously. And press the [HOLD/SAVE] button to initialize the data. |
| INITIALIZE LCR DB PRESS FLEX_KEY (1-6) | - Confirmation tone will be heard and the system goes to previous menu. |
| ENTER CO GRP INIT VAL . . | - Flex BTN 4: Initialize all CO group of DMT. |
| ENTER CO GRP INIT VAL 23 | - Dial 2 digits(01-72 in LDK-300, 01-24 in LDK-100). (Ex. Dial 23) And press the [HOLD/SAVE] button to initialize the data |
| INITIALIZE LCR DB PRESS FLEX_KEY (1-6) | - Confirmation tone will be heard and the system goes to previous menu. |
| ENTER ALT INDEX INIT VAL . . | - Flex BTN 5: Initialize alternative DMT index. |
| ENTER ALT INDEX INIT VAL 77 | - Dial two digits (00-99). (Ex. Dial 77) And press the [HOLD/SAVE] button to initialize the data |
| INITIALIZE LCR DB PRESS FLEX_KEY (1-6) | - Confirmation tone will be heard and the system goes to previous menu. |
| - INITIALIZE ALL LCR ? | - Flex BTN 6: Initialize all LCR. Press the [HOLD/SAVE] button to initialize all LCR. |
| INITIALIZE LCR DB PRESS FLEX_KEY (1-6) | - All LCR database are set to default values. Confirmation tone will be heard and the system goes to previous menu. |
| INITIALIZE LCR DB PRESS FLEX_KEY (1-6) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory - To erase the value in every step, press the [SPEED] button and press the [HOLD/SAVE] button. |

# 11.	TOLL TABLE

To program Toll Tables, press the [TRANS/PGM] button and dial 224 - 226. Toll tables are used to have access to certain toll free calls as well as being denied certain calls for the stations assigned Station COS. Exception table A & B allow the station that is programmed in STA COS 2, 3 & 4 to have access to certain toll free calls as well as being denied certain calls.

## 11.1	TOLL EXCEPTION TABLE  (PGM 224)

The Allow/Deny Tables are organized into 2 sets of tables to support 2 different toll plans at one installed site. Each allow/deny table may contain up to 30 number strings. All bins of allow and deny tables have no entries by default. Each number string can contain up to 14 entries including any number 0-9, *, #, "Don't care".

The following rules should be remembered when setting up the Allow/Deny Tables:

- If the tables have no entries, no restriction is applied.

- If entries are made in the allow table and only there, then only those numbers are allowed.

- If entries are made in the deny table and only there, then only those numbers are denied.

- If there are entries in both tables, the allow table is searched at first and if number is found, it is allowed. If not found, the deny table is searched and if number is found, it is denied. If it is not found in either table, it is allowed.

| RULE | ENTRY |  | CONDITIONS & RESULT |  |
| --- | --- | --- | --- | --- |
|  | ALLOW | DENY | ALLOW TABLE | DENY TABLE |
| 1 | Not Exist | Not Exist | No Restriction | No Restriction |
| 2 | Exist | Not Exist | Found - allowed Not found - denied | - |
| 3 | Not Exist | Exist | - | Found - denied Not found - allowed |
| 4 | Exist | Exist | Found - allowed Not found – check deny table | Found - denied Not Found - allowed |

TABLE 11.1.1  Allow/Deny Rules  (PGM 224)

PROCEDURE

| TOLL EXCEPTION TABLES PRESS FLEX_KEY (1-4) | - [TRANS/PGM] + 224. |
| --- | --- |
|  | - To program Toll Exception Tables, use the Flex. BTNs as TABLE 11.1.2. |
| ALLOW TABLE A ENTER BIN NO (01-30) | - Press a Flex. BTN to program a table. (Ex. Press BTN 1: Allow Table A) |
| ALLOW TABLE A BIN 01: . . . . . . . . . . . . . . | - Dial bin number. Refer to TABLE 10.1.3 and enter data. Each entry can be entered up to 14 digits |
|  | - To delete exception number, press the [SPEED] button and the [HOLD/SAVE] button. |
| ALLOW TABLE A ENTER BIN NO (01-30) | - Press the [HOLD/SAVE] button for updating database permanently. |
| ALLOW TABLE A ENTER BIN NO (01-30) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | ENTRY | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | ALLOW A | 01 – 30 | - | Max 14 digits |
| 2 | DENY A | 01 – 30 | - | Max 14 digits |
| 3 | ALLOW B | 01 – 30 | - | Max 14 digits |
| 4 | DENY B | 01 – 30 | - | Max 14 digits |

TABLE 11.1.2  Button Configuration for Toll Tables  (PGM 224)

| VALID DATA | FUNCTION | LCD DISPLAY |
| --- | --- | --- |
| 0 - 9, *, # | Numbers | as dialed |
| [DND/FWD] | Don’t Care | 'D' |

TABLE 11.1.3  Data Entry of Toll Table  (PGM 224)

## 11.2	CANNED TOLL TABLE  (PGM 225)

In addition to the basic toll restrictions, stations with a COS 5 or 6 are subject to dial restrictions based on the “Canned” Allow and Deny Tables. This program permits entries in the Canned Toll Tables. Both the Allow and Deny table have 20 bins up to 14 digits.

PROCEDURE

| CANNED TOLL TABLES PRESS FLEX_KEY (1-2) | - [TRANS/PGM] + 225. |
| --- | --- |
|  | - To program Canned Toll Tables, use the BTNs as TABLE 11.2.1. |
| ALLOW TABLE ENTER BIN NO (01-20) | - Press a Flex. Button to program a table. (Ex. Press BTN 1: Allow Table) |
| ALLOW TABLE BIN 01: . . . . . . . . . . . . . . | - Dial bin number. Refer to TABLE 10.2.2 and enter data. Each entry can be up to 14 digits. |
|  | - To delete the number, press the [SPEED] button and the [HOLD/SAVE] button. |
| ALLOW TABLE ENTER BIN NO (01-20) | - Press the [HOLD/SAVE] button for updating database permanently. |
| ALLOW TABLE ENTER BIN NO (01-20) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | ENTRY | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | ALLOW | 01 – 20 | - | Max. 14 digits |
| 2 | DENY | 01 – 20 | - | Max. 14 digits |

TABLE 11.2.1  Button Configuration for Canned Toll Table  (PGM 225)

| VALID DATA | FUNCTION | LCD DISPLAY |
| --- | --- | --- |
| 0 - 9, *, # | Number | as dialed |
| [DND/FWD] | Don’t Care | 'D' |

TABLE 11.2.2  Data Entry of Canned Toll Table  (PGM 225)

## 11.3	EMERGENCY CODE TABLE  (PGM 226)

The emergency code table is used for emergency call service. All stations, regardless of COS, can dial the emergency codes in this table.

PROCEDURE

| EMERGENCY SVC CALL ENTER BIN NO (01 - 10) | - [TRANS/PGM] + 226. |
| --- | --- |
| EMERGENCY SVC CALL BIN 01: . . . . . . . . . . . . . . | - Dial 2 digits of 01-10. (Ex. 01) |
| EMERGENCY SVC CALL BIN 01: 00119 | - To assign emergency call number, dial the desired digits. (Ex. 00119) |
| EMERGENCY SVC CALL BIN 01: 00119 | - Press the [HOLD/SAVE] button for updating database permanently. |
| EMERGENCY SVC CALL ENTER BIN NO (01 - 10) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | ENTRY | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Emergency Code Table | 01 – 10 | - | Max digit: 14 |

TABLE 11.3.1  Button Configuration for Emergency Code Table  (PGM 226)

# 12.	TABLES

To program TABLES, press the [TRANS/PGM] button and dial 227 - 235.

## 12.1	AUTHORIZATION CODE TABLE  (PGM 227)

Trunk groups can be marked to deny access until a matched Authorization code is entered. In this case, DND warning tone is provided when the trunk group access code is dialed. If the dialed Authorization code is verified, you will hear CO dial tone. Otherwise, you will hear error tone and cannot access the group. The authorization codes can be entered by stations or admin programming. Authorization code is fixed 5 digits. Administrator can see and change station's password. There can be no duplicate entries. By default, Authorization Codes are not assigned at all. In LDK-300(100) system, the total number of Authorization Codes is 600(164) entries.

PROCEDURE

| AUTHOR CODE ENTER BIN NO (001 - 600) |  | - [TRANS/PGM] + 227. In LDK-100, 001-164 is displayed. |
| --- | --- | --- |
| AUTHOR CODE 001 : . . . . . |  | - Enter entry number of authorization code. (Ex. 001) LCD shows current assigned Authorization code. |
| AUTHOR CODE 001 : 12345 |  | - To assign authorization code, enter 5-digits code. If there are no code duplication in the system, then entered code will be displayed on the LCD, otherwise error tone will be given. To delete authorization code, press the [SPEED] button. (Ex. 12345) |
| AUTHOR CODE ENTER BIN NO (001 -600) |  | - Press the [HOLD/SAVE] button for updating database permanently. |
| AUTHOR CODE ENTER BIN NO (001 -600) |  | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

## 12.2	CUSTOM CALL ROUTING  (PGM 228)

PROCEDURE

| CUSTOM CALL ROUTING SELECT CCR TABLE (01-70) | - [TRANS/PGM] + 228. - To select CCR Table number, dial 01~70. |
| --- | --- |
| CUSTOM CALL ROUTING : 01 PRESS FLEX KEY (1-10) | - To select each entry number of CCR table, press Flex. BTN 1~10, then the entered data will be displayed on the LCD. |
| CUSTOM CALL ROUTING : 01 INPUT 1 : ... | - To program call routing destination, dial destination type as follow; 1 – STA 2 – HUNT 3 – VMIB 4 – VMIB DROP 5 – SYSTEM SPEED DIAL 6 – INTERNAL PAGE 7 – EXTERNAL PAGE 8 – ALL CALL PAGE |
| CUSTOM CALL ROUTING : 01 INPUT 1 : STA 100 | - To program station number, dial the station number. (Ex. Dial station number '100') |
| CUSTOM CALL ROUTING : 01 INPUT 1 : HUNT GRP 621 | - To program Hunt Group, dial a hunt group number. (Ex. Dial Hunt group 621.) |
| CUSTOM CALL ROUTING : 01 INPUT 1 : VMIB ANNC 1 | - To program system announcement, dial a VMIB system announcement number. |
| CUSTOM CALL ROUTING : 01 INPUT 1 : VMIB ANNC 10 (#) | - To program system drop the CO line after providing the announcement, dial a VMIB system announcement number and dial “#”. |
| CUSTOM CALL ROUTING : 01 INPUT 1 : SYS SPD 2000 | - To program System Speed Dial, dial a system speed bin number. (Ex. Dial Speed bin 2000.) |
| CUSTOM CALL ROUTING : 01 INPUT 1 : INT PAGE 4 | - To program Internal Page, dial an internal page number. (Ex. Dial 4.) |
| CUSTOM CALL ROUTING : 01 INPUT 1 : EXT PAGE 2 | - To program External Page, dial an external page number. (Ex. Dial 2.) |
| CUSTOM CALL ROUTING : 01 INPUT 1 : ALL 1 | - To program All Call Page, dial an all call page number. (Ex. Dial 1.) |
| CUSTOM CALL ROUTING : 01 INPUT 1 : … | - To delete the content of entry, press the [SPEED] button. |
| CUSTOM CALL ROUTING : 01 INPUT 1 : VMIB ANNC 1 | - Press the [HOLD/SAVE] button for updating database permanently. |
| CUSTOM CALL ROUTING SELECT CCR TABLE (01-70) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| TYPE (DIGIT) | TYPE | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Station | STA # | - |  |
| 2 | Hunt Group | HUNT # | - |  |
| 3 | VMIB | Announce # | - |  |
| 4 | VMIB Drop | Announce # |  |  |
| 5 | System Speed | 2000-4999 (LDK-300) 2000-3499 (LDK-100) | - |  |
| 6 | Internal Page | 1 – 30 | - |  |
| 7 | External Page | 1 – 3 | - |  |
| 8 | All Call Page | 1 – 3 | - | 1: INT All Page 2: EXT All Page 3: All Page |

TABLE 12.2.1   Custom Call Routing Table  (PGM 228)

## 12.3	EXECUTIVE / SECRETARY TABLE  (PGM 229)

There are a number of Executive/Secretary pairs available for assignment so that when the executive designated station is in DND state, intercom calls and transfers will be automatically routed to the designated secretary station. By default, Executive/Secretary Pairs are not assigned at all. In LDK-300(100), system supports 36(12) Executive/Secretary pairs.

PROCEDURE

| EXEC/SEC PAIRS ENTER BIN NO (01-36) |  | - [TRANS/PGM] + 229. In LDK-100, 01-12 is displayed. |
| --- | --- | --- |
| EXEC/SEC PAIRS PAIR 1 . . . ./. . . . |  | - Select one of Exec/Sec pairs number. (Ex Pair 1: Dial 01 for bin number 1). |
| EXEC/SEC PAIRS PAIR 01 101/105 |  | - To assign Exec/Sec pair, Dial Executive station number and Secretary station number sequentially, then Exec/Sec pair will be displayed on the LCD. (Ex. Exec/Sec:101/105). To delete Exec/Sec pair, press the [SPEED] button. |
| EXEC/SEC PAIRS ENTER BIN NO (01-36) |  | - Press the [HOLD/SAVE] button for updating database permanently. |
| EXEC/SEC PAIRS ENTER BIN NO (01-36) |  | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
|  | Executive/Secretary Table | STA # | - |  |

TABLE 12.3.1  Executive/Secretary Table  (PGM 229)

## 12.4	DID DIGITS CONVERSION TABLE  (PGM 230)

With Direct Inward Dialing (DID), a user dials through a public telephone network. To complete the connection, the DID trunk receives two, three or four digits from the central office, and routes the call to the proper corresponding station. The DID dialed digits may be modified by stripping up to two leading digits, by substitution of two leading digits (after stripping).

PROCEDURE

| DID DIGIT CONVERSION F1:RCV_NO F2:DGT_CONV |  | - [TRANS/PGM] + 230. LCD shows current status of DID Digit Conversion. To program, use the Flex. Buttons of Table 12.4.1. |
| --- | --- | --- |
| DID DIGIT CONVERSION RCV_NO : 3 (2 - 4) |  | - To assign received digit number from PBX, press Flex. BTN 1 and enter data. Then entered data will be displayed on the LCD. |
| DID DGT MASK #*** |  | - To assign converted DID 4 digits, press Flex. BTN 2 and dial 4 digits (combination of 0-9, *, #), then entered 4 digits will be displayed on the LCD. |
| DID DIGIT CONVERSION F1:RCV_NO F2:DGT_CONV |  | - Press the [HOLD/SAVE] button for updating database permanently. |
| DID DIGIT CONVERSION F1:RCV_NO F2:DGT_CONV |  | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | DID Received Digit No. from PX | 2 - 4 | 3 |  |
| 2 | DID Digit Conversion | 4 digits (d, *, #) | #*** | d : digit (0 - 9) # : ignore digits * : any kind of digit |

TABLE 12.4.1  Button Configuration for DID Digit Conversion  (PGM 230)

## 12.5	FLEXIBLE DID TABLE  (PGM 231)

This table is for flexible DID table service.

PROCEDURE

| FLEX DID CONV TABLE ENTER BIN NO (000 – 999) |  |  | - [TRANS/PGM] + 231. To enter flexible DID table, dial digits (000-999). (Ex. 001) |  |
| --- | --- | --- | --- | --- |
| - TALBE BIN 001 - PRESS FLEX KEY (1-5) |  |  | - To select each item of flexible DID table, press Flex. BTN 1-5, then the entered data will be displayed on the LCD. |  |
| - TABLE 001 NAME - ………... |  |  | - To enter DID Name, press Flex BTN 1 and enter the name. |  |
| - TABLE 000 DAY DEST - NONE (1-9) |  |  | - To enter Day destination, press Flex BTN 2 and dial destination type and destination number as follow; 1 – STA: dial type number 1 and dial station number. 2 – HUNT: dial type number 2 and dial hunt group number. 3 – VMIB: dial type number 3 and dial VMIB announcement number. 4 – VMIB (#) (after The VMIB announcement is heard and the CO line is dropped.) : dial type number 4 and dial VMIB announcement number. 5 – SPD: dial type number 5 and dial Speed bin number. 6 – INT PAGE: dial type number 6 and dial Page number 7 – EXT PAGE: dial type number 7 and dial Page number 8- ALL PAGE: dial type number 8 and dial Page number 9 – NET NUM: dial type number 9 and dial the net station number. |  |
| - TABLE 001 NIGHT DEST - NONE (1-9) |  |  | - To enter Night Destination, Flex. BTN 3 and dial destination type and destination number. |  |
| - TABLE 001 WEEKEND DEST - NONE (1-9) |  |  | - To enter Weekend Destination, Flex. BTN 4 and dial destination type and destination number. |  |
| - TABLE 001 DAY DEST STA 100 |  |  |  | - Press the [HOLD/SAVE] button for updating database permanently. |
| - DID CONV TABLE 001 - PRESS FLEX KEY (1-5) |  |  |  | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | DID Name | 1 – 11 Chars | None | Max. 11 characters |
| 2 | Day Destination | STA # / Hunt # / VMIB # / VMIB # drop SPD Int Page Ext Page All Page Net Num | STA # Or Null | 00 – 70 (00: Not assigned) 00 – 70 (00: Not assigned) 2000-4999 (LDK-300), 2000-3499 (LDK-100) 1-30 (LDK-300), 1-10 (LDK-100) 1 – 3 1 – 3 Network station number. |
| 3 | Night Destination | STA # / Hunt # / VMIB # / VMIB # drop SPD Int Page Ext Page All Page Net Num | ATD STA # | 00 – 70 (00: Not assigned) 00 – 70 (00: Not assigned) 2000-4999 (LDK-300), 2000-3499 (LDK-100) 1-30 (LDK-300), 1-10 (LDK-100) 1 – 3 1-3 Network station number. |
| 4 | Weekend Destination | STA # / Hunt # / VMIB # / VMIB # drop SPD Int Page Ext Page All Page Net Num | ATD STA # | 00 – 70 (00: Not assigned) 00 – 70 (00: Not assigned) 2000-4999 (LDK-300), 2000-3499 (LDK-100) 1-30 (LDK-300), 1-10 (LDK-100) 1 – 3 1 – 3 Network station number |
| 5 | Reroute Destination | STA # / Hunt # / VMIB # / VMIB # drop SPD | ATD STA # | 00 – 70 (00: Not assigned) 00 – 70 (00: Not assigned) 2000-4999 (LDK-300), 2000-3499 (LDK-100) |

TABLE 12.5.1  Button Configuration of Flexible DID Table  (PGM 231)

## 12.6	SYSTEM SPEED ZONE  (PGM 232)

PROCEDURE

| SYSTEM SPD ZONE PGM ENTER ZONE NO (01-10) | - [TRANS/PGM] + 232. |
| --- | --- |
| SYSTEM SPD ZONE 01 F1:ZN F2:STA F3:TCHK | - Enter Zone Number (01 - 10). |
| ENTER NEW ZONE RANGE ZONE 01 : xxxx- xxxx | - To enter speed zone range, press Flex. BTN 1 and dial related new zone range. |
| ENTER STA RANGE ZONE 01 : xxx – xxx | - To enter the station range, press Flex. BTN 2 and dial related new station range. |
|  | - To toggle speed zone toll check, press Flex. BTN 3, then LED 3 shows toll check status of current zone. |
| SYSTEM SPD ZONE PGM ENTER ZONE NO (01-10) | - Press the [HOLD/SAVE] button for updating database permanently. |
| SYSTEM SPD ZONE PGM ENTER ZONE NO (01-10) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Speed Bin Range in Zone |  | 2200 – 4999 (LDK-300) 2200 – 3499 (LDK-100) | Each zone is exclusive (2000 – 2199: Toll Free Zone) |
| 2 | Station Range to Access Zone | STA No. | 100 – 399 (LDK-300) 100 – 227 (LDK-100) |  |
| 3 | Toll Checking | YES / NO | YES (ON) |  |

TABLE 12.6.1  Button Configuration for System Speed Dial Zone  (PGM 232)

## 12.7	WEEKLY TIME TABLE  (PGM 233)

PROCEDURE

| WEEKLY TIME TBL DIAL DIGIT (00-15) |  | - [TRANS/PGM] + 233. |
| --- | --- | --- |
| WEEKLY TIME TBL DIAL DIGIT (00-15) |  | - To program each Weekly Time Table, dial 2 digits of 00-15 (Ex. 07). Table of index 00 is used for the Main Attendant Auto Ring mode. Tables of index 01~15 are used for the ICM Tenancy Attendants Auto Ring mode. |
| WEEKLY TIME TBL 07 PRESS FLEX_KEY (1-7) |  | - Each time table has 1-7 day mode as TABLE 12.7.1. Press one of Flex. BTNs. (Ex. BTN 5: Friday) |
| WEEKLY TBL : FRI D:09:00 N: W: | - Pressing Flex. BTN 3, the start time of Day, Night and Weekend may be changed. Press Flex. BTN 1 to edit start time of Day mode, and enter 4 digits continuously. (Ex. 0900 for 09:00) |  |
| WEEKLY TBL : FRI D:09:00 N:18:00 W: | - Press Flex. BTN 2 to edit start time of Night mode, and enter 4 digits continuously. (e.g. 1800 for 18:00). Press Flex. BTN 3 for Weekend mode time. |  |
| WEEKLY TIME TBL 07 PRESS FLEX_KEY (1-7) | - Press the [HOLD/SAVE] button. Then, system goes to up level. Press one of Flex. BTNs (1-7) for another day. Continue step (3). |  |
| WEEKLY TIME TBL DIAL DIGIT (00-15) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |  |

| BTN | ITEM | REMARK |
| --- | --- | --- |
| 1 | Monday | Refer Table 12.7.2 |
| 2 | Tuesday |  |
| 3 | Wednesday |  |
| 4 | Thursday |  |
| 5 | Friday |  |
| 6 | Saturday |  |
| 7 | Sunday |  |

TABLE 12.7.1  Weekly Time Table - 1  (PGM 233)

| BTN | ITEM | DEFAULT | REMARK |
| --- | --- | --- | --- |
| 1 | Day |  | Day ring mode start time (HH:MM) |
| 2 | Night |  | Night ring mode start time (HH:MM) |
| 3 | Weekend |  | Weekend ring mode start time (HH:MM) |

TABLE 12.7.2  Weekly Time Table - 2  (PGM 233)

## 12.8	VOICE MAIL DIALING TABLE  (PGM 234)

PROCEDURE

| VOICE MAIL DIALING TBL DIAL DIGIT (1-9) |  |  | - [TRANS/PGM] + 234. |
| --- | --- | --- | --- |
| VOICE MAIL 1 PREFIX OR SUFFIX (0-1) |  |  | - To program each dialing table, dial digit as TABLE 12.8.1. |
| VOICE MAIL 1 PRE XXXXXXXXXXXX |  |  | - Enter digits (0-9, *, #, Pause) within 12 characters. To enter the pause, press the [CALLBK] button. |
| VOICE MAIL DIALING TBL DIAL DIGIT (1-9) |  | - Press the [HOLD/SAVE] button for updating database permanently. |  |
| VOICE MAIL DIALING TBL DIAL DIGIT (1-9) |  | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |  |

| DIGIT | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | VM Table 1 |  | Prefix : P# Suffix : - | Put Mail |
| 2 | VM Table 2 |  | Prefix : P## Suffix : - | Get Mail |
| 3 | VM Table 3 |  | Prefix : P#*3P Suffix : - | Busy Table |
| 4 | VM Table 4 |  | Prefix : P#*4P Suffix : - | No Answer Table |
| 5 | VM Table 5 |  | Prefix : P#*5P Suffix : - | Error Table |
| 6 | VM Table 6 |  | Prefix : P#*6P Suffix : - | DND Table |
| 7 | VM Table 7 |  | Prefix : Suffix : - |  |
| 8 | VM Table 8 |  | Prefix : Suffix : - |  |
| 9 | VM Table 9 |  | ***** | Disconnect Table |

TABLE 12.8.1  Voice Mail Table  (PGM 234)

## 12.9	TIE ROUTING TABLE  (PGM 235)

Maximum 30 Tie Line routings can be programmed. Maximum 6 CO lines are assignable to each Routing. By default, Tie Line Routings are not assigned at all.

PROCEDURE

| TIE LINE ROUTING DIAL ROUTING NO 01-30 | - [TRANS/PGM] + 235. |
| --- | --- |
| TIE LINE ROUTING 01 ... ... ... ... ... ... | - Enter two digits for routing number (01-30). (Ex. 01) |
|  | - To assign Tie Line Routing, press Flex. BTN 1-6 and enter CO line number. See TABLE 12.9.1. |
|  | - To delete any of code, press Flex. BTN 1-6 and press the [SPEED] button. See TABLE 11.9.1. |
| TIE LINE ROUTING DIAL ROUTING NO 01-30 | - Press the [HOLD/SAVE] button for updating database permanently. |
| TIE LINE ROUTING DIAL ROUTING NO 01-30 | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
|  | Tie Routing Table (1-30) | 001 – 200 (LDK-300) 01 – 40 (LDK-100) | - |  |

TABLE 12.9.1  Tie Routing Table  (PGM 235)

# 13.	NETWORKING ATTRIBUTE

To program NETWORKING ATTRIBUTE, press the [TRANS/PGM] button and dial 320 - 324.

## 13.1	Networking Basic Attribute  (PGM 320)

PROCEDURE

| NET BASIC ATTRIBUTE PRESS FLEX KEY(1-5) | - [TRANS/PGM] + 320 |
| --- | --- |
| NET RETRY COUNT (00 – 99) : 25 | - To program Networking Basic Attribute, Press the Flex Btn 1-5. Related message will be displayed on LCD. (EX: Btn2) |
| NET RETRY COUNT (00 – 99) : 30 | - To change the value, dial the value in the range (EX:30) |
| NET RETRY COUNT (00 – 99) : 30 | - Press the [HOLD/SAVE] button for saving database permanently. |
| NET BASIC ATTRIBUTE PRESS FLEX KEY(1-5) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Networking Enable | ON / OFF | OFF | Enable Networking function |
| 2 | Networking Retry Count | 00 - 99 | 00 | No need at direct connection between LDK systems. This field is available at connection through the public network. |
| 3 | Networking CNIP Enable | ON / OFF | ON | The name of calling station is sent to the called system between LDK systems. CNIP is displayed at called party stations display based on the programming. If the CNIP and CLI are received together, CNIP is prior to CLI. |
| 4 | Networking CONP Enable | ON / OFF | OFF | Reserved for future usage |
| 5 | Networking Signal Method | FAC / UUS | FAC | Select the information element type for Networking supplementary service message. |
| 6 | Networking CAS Enable | ON/OFF | OFF | Enable Centralized attendant In master system, CAS should be disabled. |
| 7 | Networking VPN Enable | ON/OFF | OFF | Reserved for future usage |

TABLE 13.1.1  Networking Basic Attribute  (PGM 320)

## 13.2	Networking Supplementary Attribute  (PGM 321)

PROCEDURE

| NET SUPPLIMENTARY ATTR PRESS FLEX KEY(1-1) | - [TRANS/PGM] + 321 |
| --- | --- |
| NET TRANSFER MODE (1:RERT/ 0:JOIN): JOIN | - To program Networking Supplementary Attribute, Press the Flex Btn 1-1. Related message will be displayed on LCD. |
| NET TRANSFER MODE (1:RERT/ 0:JOIN): RERT | - Press the [HOLD/SAVE] button for saving database permanently. |
| NET SUPPLIMENTARY ATTR PRESS FLEX KEY(1-5) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Networking Transfer Mode | RERT/JOIN | REROUT | Only Transfer by Rerouting is possible |
| 2 | TCP port | 4 digits | 9000 | TCP port for BLF message |
| 3 | UDP port | 4 digits | 9001 | UDP port for BLF message |
| 4 | BLF Manager IP Address | 12 digits | 0.0.0.0 | IP Address of BLF manager for BLF service |
| 5 | Duration of BLF status | 01 ~ 20 sec | 02 | Duration of BLF status message |
| 6 | Multicast IP Address | 12 digits | 0.0.0.0 | IP address of Multicast for BLF service |
| 7 | Net Trans Fault Recall Timer | 1 ~ 300 | 10 | Network transfer fault recall timer. |

TABLE 13.2.1  Networking Supplementary Attribute  (PGM 321)

## 13.3	Networking CO Line Attribute  (PGM 322)

PROCEDURE

| NET COLINE ATTRIBUTE ENTER COL RANGE | - [TRANS/PGM] + 322 |
| --- | --- |
| 001-002 NET COL PGM PRESS FLEX_KEY (1-3) | - Enter CO line range. |
|  | To program, press the Flex. BTN. Press BTN 1-3 and enter related data, then entered data will be displayed on the LCD. |
| 001-002 NET CO GRP (00 – 24) : 00 | - Press Flex. BTN 1 and assign Networking CO Group. |
| 001-002 NET CO GRP (00 – 24) : 01 | - Press the [HOLD/SAVE] button for saving database permanently. |
| 001-002 NET COL PGM PRESS FLEX_KEY (1-3) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (2) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Networking CO Group | 00 - 24 | 00 | - Networking CO group programming for Networking call between LDK systems. |
| 2 | Networking CO OutSend | ON / OFF | OFF | - Set about the sharing CO line at slave system. |
| 3 | Networking CO OutTrans | ON / OFF | OFF | - Set about the PSTN CO line at master system. |

TABLE 13.3.1  Networking CO Line Attribute  (PGM 322)

## 13.4	Networking Attendant Assignment (PGM 323)

PROCEDURE

| CAS / VPN CO GROUP ASG PRESS FLEX_KEY (1-3) | (1). [TRANS/PGM] + 323 |  |
| --- | --- | --- |
| CAS NUM TBL INDEX (00-71) : 00 | (2). To assign CO group for Centralized Attendant, press the Flex BTN 1. Enter the table index number of CO group which is connected with CAS system. (Refer to PGM 324.) |  |
| VPN CO GROUP (00-24) : 00 | (3). Press the Flex. BTN 2. And the procedure is the same as CAS CO group assignment. |  |
| VPN CO GROUP (00-24) : 05 | ● Press the [HOLD/SAVE] button for updating database permanently. |  |
| CAS / VPN CO GROUP ASG PRESS FLEX_KEY (1-3) |  | ● Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | CAS Num table index | 00 - 71 | 00 | - Networking Net bin number for CAS. |
| 2 | VPN CO Group | 00 – 24 | 00 | - Reserved for future usage |
| 3 | Prefix for CAS | 8 digits | - | - Prefix for CAS Call |

TABLE 13.4.1  Centralized Attendant & VPN CO Group Assignment (PGM 323)

## 13.5	Networking Routing Table (PGM 324)

PROCEDURE

| NET NUM PLAN TABLE ENTER BIN NO (00 – 71) | - [TRANS/PGM] + 324. |  |
| --- | --- | --- |
| 00 NET NUM PLAN TBL PRESS FLEX KEY (1 – 6) | - To assign Networking Numbering Plan Table, Dial the Table number user want to program. |  |
|  | To program, press the Flex. BTN. Press BTN 1-6 and enter related data, then entered data will be displayed on the LCD. |  |
| 00 NUM PLAN CODE . . . . . . . . . . . . . . . . | - Press Flex Btn 1 and dial the networking numbering plan code. |  |
| 00 NUM PLAN CO GRP (00-24) : . . | - Press Flex Btn 2 and dial the networking CO group number. |  |
| 00 SYSTEM USAGE (1:VOIP/0:QSIG): QSIG | - Press Flex Btn 3 and dial the type of networking CO group. |  |
| 00 CPN INFORMATION . . . . . . . . . . . . . . . . | - In case of VOIP, press Flex Btn 4 and dial the CPN information. |  |
| 00 NET NUM PLAN TBL PRESS FLEX KEY (1 – 6) |  | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Net Numbering Code | 16 digits | -- | ‘*’ means any digits can be inserted between 0 ~ 9. The digits followed by ‘#’ is a internal station number. |
| 2 | Net Number CO Group | 00 – 24 | -- | ‘00’ means an internal net station number. |
| 3 | System Usage | VOIP / QSIG | QSIG | Select Routing Table Usage |
| 4 | CPN or IP Information | 16 digits | -- | CPN for ISDN, IP address for VoIP |
| 5 | Alternate Dial Bin | 2000 – 4999 (LDK-300) 2000 – 3499 (LDK-100) | - | Alternative Dial Number(System SPD Bin) when the networking path has a fatal problem. |
| 6 | Destination MPB IP | IP Address | -- | IP Address of destination system to support DECT mobility service. |

TABLE 13.5.1  Networking Routing Table  (PGM 324)

# 14.	VOIB ATTRIBUTE

## 14.1	VOIP IP SETTING  (PGM 340)

PROCEDURE

| VOIP IP SETTING F1:BRD1 F2:BRD2 F3:BRD3 | - [TRANS/PGM] + 340. |
| --- | --- |
|  | To program the first VOIB, Press Flex Btn 1 |
| VOIB 1 NET SETTING PRESS FLEX KEY (1-10) | - To program IP Address of VOIB 1, Press Flex Btn 1 |
| IP ADDR(SKIP:#) 165.147. 3. 1 | - To skip entering digit, press #. |
| IP ADDR(SKIP:#) 165.147. 3. 1 | - Press the [HOLD/SAVE] button for saving database permanently. |
| VOIB 1 NET SETTING PRESS FLEX KEY (1-10) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | INTERCOM RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | IP Address(SKIP:#) | 12 Digits | 0.0.0.0. |  |
| 2 | GATEWAY Address (SKIP:#) | 12 Digits | 0.0.0.0. |  |
| 3 | SUBNET Mask(SKIP:#) | 12 Digits | 0.0.0.0. |  |
| 4 | DNS Address (SKIP:#) | 12 Digits | 0.0.0.0. |  |
| 5 | TRACE Password | 10 Digits | . . . . |  |
| 6 | Default Codec | 0 – 3 | 0 |  |
| 7 | Default Gain | 1 – 62 | 31 |  |
| 8 | No Delay (TOS) | ON / OFF | OFF |  |
| 9 | Throughput (TOS) | HIGH / NORMAL | NORMAL |  |
| 10 | Reliability (TOS) | HIGH / NORMAL | NORMAL |  |

TABLE 14.1.1  VOIB IP SETTING  (PGM 340)

# 15.	NATION SPECIFIC PROGRAMMING

If system parameters are to be changed, press the [TRANS/PGM] button and dial 400-423. When programming, LCD and LEDs indicate current programmed data and programming status. The programmer enters correct data, then LCD and LEDs show the entered data and the data is stored in the temporary buffer area. Pressing the [HOLD/SAVE] button, all data in the temporary buffer (same as LCD/LEDs show their status) are stored into permanent system memory.

## 15.1	DTIB RX GAIN CONTROL  (PGM 400)

PROCEDURE

| DTIB RX GAIN PRESS FLEX KEY (01-13) | - [TRANS/PGM] + 400. |
| --- | --- |
| DTIB RX GAIN DTIB/DTIB: 26 (00-63) | - Press one of Flex. BTNs (1~13) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows RX gain of the device from other devices. |
| DTIB RX GAIN DTIB/SLIB: 33 (00-63) | - For example, press the Flex. BTN 2 to change the Rx gain of DTIB from SLIB. |
| DTIB RX GAIN DTIB/SLIB: 45 (00-63) | - To change the gain dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| DTIB RX GAIN PRESS FLEX KEY (01-13) | - Press the [HOLD/SAVE] button for updating database permanently. |
| DTIB RX GAIN PRESS FLEX KEY (01-13) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

## 15.2	SLIB RX GAIN CONTROL  (PGM 401)

PROCEDURE

| SLIB RX GAIN PRESS FLEX KEY (01-13) | (1) [TRANS/PGM] + 401. |
| --- | --- |
| SLIB RX GAIN SLIB/DTIB: 12 (00-63) | - Press one of Flex. BTNs (01~13) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows RX gain of the device from other devices. |
| SLIB RX GAIN SLIB/SLIB: 26 (00-63) | - For example, press the Flex. BTN 2 to change the Rx gain of SLIB from the other SLIB. |
| SLIB RX GAIN SLIB/SLIB: 45 (00-63) | - To change the gain dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| SLIB RX GAIN PRESS FLEX KEY (01-13) | - Press the [HOLD/SAVE] button for updating database permanently. |
| SLIB RX GAIN PRESS FLEX KEY (01-13) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory.. |

## 15.3	CTR SLIB RX GAIN CONTROL  (PGM 402)

PROCEDURE

| SLIB12 RX GAIN PRESS FLEX KEY (01-13) | - [TRANS/PGM] + 402. |
| --- | --- |
| SLIB12 RX GAIN SLIB12/DTIB: 12 (00-63) | - Press one of Flex. BTNs (01~13) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows RX gain of the device from other devices. |
| SLIB12 RX GAIN SLIB12/SLIB: 26 (00-63) | - For example, press the Flex. BTN 2 to change the Rx gain of SLIB12 from SLIB. |
| SLIB12 RX GAIN SLIB12/SLIB: 45 (00-63) | - To change the gain dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| SLIB12 RX GAIN PRESS FLEX KEY (01-13) | - Press the [HOLD/SAVE] button for updating database permanently. |
| SLIB12 RX GAIN PRESS FLEX KEY (01-13) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

## 15.4	WTIB RX GAIN CONTROL  (PGM 403)

PROCEDURE

| WTIB RX GAIN PRESS FLEX KEY (01-13) | - [TRANS/PGM] + 403. |
| --- | --- |
| WTIB RX GAIN WTIB/DTIB: 26 (00-63) | - Press one of Flex. BTNs (01~13) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows RX gain of the device from other devices. |
| WTIB RX GAIN WTIB/SLIB: 33 (00-63) | - For example, press the Flex. BTN 2 to change the Rx gain of WTIB from SLIB. |
| WTIB RX GAIN WTIB/SLIB: 45 (00-63) | - To change the gain dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| WTIB RX GAIN PRESS FLEX KEY (01-13) | - Press the [HOLD/SAVE] button for updating database permanently. |
| WTIB RX GAIN PRESS FLEX KEY (01-13) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

## 15.5	ACOB RX GAIN CONTROL  (PGM 404)

PROCEDURE

| ACOB RX GAIN PRESS FLEX KEY (01-14) | - [TRANS/PGM] + 404. |
| --- | --- |
| ACOB RX GAIN ACOB/DTIB: 26 (00-63) | - Press one of Flex. BTNs (01~14) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows RX gain of the device from other devices. |
| ACOB RX GAIN ACOB/SLIB: 37 (00-63) | - For example, press the Flex. BTN 2 to change the Rx gain of ACOB from SLIB. |
| ACOB RX GAIN ACOB/SLIB: 45 (00-63) | - To change the gain dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| ACOB RX GAIN PRESS FLEX KEY (01-14) | - Press the [HOLD/SAVE] button for updating database permanently. |
| ACOB RX GAIN PRESS FLEX KEY (01-14) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

## 15.6	CTR ACOB RX GAIN CONTROL  (PGM 405)

PROCEDURE

| ACOB8 RX GAIN PRESS FLEX KEY (01-14) | - [TRANS/PGM] + 405. |
| --- | --- |
| ACOB8 RX GAIN ACOB8/DTIB: 26 (00-63) | - Press one of Flex. BTNs (01~14) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows RX gain of the device from other devices. |
| ACOB8 RX GAIN ACOB8/SLIB: 37 (00-63) | - For example, press the Flex. BTN 2 to change the Rx gain of LCOB8 from SLIB. |
| ACOB8 RX GAIN ACOB8/SLIB: 45 (00-63) | - To change the gain dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| ACOB8 RX GAIN PRESS FLEX KEY (01-14) | - Press the [HOLD/SAVE] button for updating database permanently. |
| ACOB8 RX GAIN PRESS FLEX KEY (01-14) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

## 15.7	DCOB RX GAIN CONTROL  (PGM 406)

PROCEDURE

| DCOB8 RX GAIN PRESS FLEX KEY (01-14) | (1) [TRANS/PGM] + 406. |
| --- | --- |
| DCOB8 RX GAIN DCOB8/DTIB: 26 (00-63) | (2) Press one of Flex. BTNs (01~14) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows RX gain of the device from other devices. |
| DCOB8 RX GAIN DCOB8/SLIB: 37 (00-63) | (3) For example, press the Flex. BTN 2 to change the Rx gain of DCOB from SLIB. |
| DCOB8 RX GAIN DCOB8/SLIB: 45 (00-63) | (4) To change the gain dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| DCOB8 RX GAIN PRESS FLEX KEY (01-14) | (5) Press the [HOLD/SAVE] button for updating database permanently. |
| DCOB8 RX GAIN PRESS FLEX KEY (01-14) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

## 15.8	VMIB RX GAIN CONTROL  (PGM 407)

PROCEDURE

| VMIB RX GAIN PRESS FLEX KEY (1-9) | - [TRANS/PGM] + 407. |
| --- | --- |
| VMIB RX GAIN VMIB/DTIB: 21 (00-63) | - Press one of Flex. BTNs (1~9) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows RX gain of the device from other devices. |
| VMIB RX GAIN VMIB/SLIB: 32 (00-63) | - For example, press the Flex. BTN 2 to change the Rx gain of VMIB from SLIB. |
| VMIB RX GAIN VMIB/SLIB: 45 (00-63) | - To change the gain, dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| VMIB RX GAIN PRESS FLEX KEY (1-9) | - Press the [HOLD/SAVE] button for updating database permanently. |
| VMIB RX GAIN PRESS FLEX KEY (1-9) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

## 15.9	DTMF RCVR RX GAIN CONTROL  (PGM 408)

PROCEDURE

| DTMF RCVR RX GAIN PRESS FLEX KEY (1-5) | - [TRANS/PGM] + 408. |
| --- | --- |
| DTMF RCVR RX GAIN DTMF RC/SLIB: 37 (00-63) | - Press one of Flex. BTNs (1~5) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows RX gain of the device from other devices. |
| DTMF RCVR RX GAIN DTMF RC/ACOB: 24 (00-63) | - For example, press the Flex. BTN 3 to change the Rx gain of DTMF Receiver from ACOB. |
| DTMF RCVR RX GAIN DRMF RC/ACOB: 45 (00-63) | - To change the gain dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| DTMF RCVR RX GAIN PRESS FLEX KEY (1-5) | - Press the [HOLD/SAVE] button for updating database permanently. |
| DTMF RCVR RX GAIN PRESS FLEX KEY (1-5) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

## 15.10 EXT PAGE RX GAIN CONTROL  (PGM 409)

PROCEDURE

| EXT PAGE RX GAIN PRESS FLEX KEY (1-11) | - [TRANS/PGM] + 409. |
| --- | --- |
| EXT PAGE RX GAIN EXT PAG /DTIB: 26 (00-63) | - Press one of Flex. BTNs (1~11) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows RX gain of the device from other devices. |
| EXT PAGE RX GAIN EXT PAG /SLIB: 37 (00-63) | - For example, press the Flex. BTN 2 to change the Rx gain of External Page from SLIB. |
| EXT PAGE RX GAIN EXT PAG /SLIB: 45 (00-63) | - To change the gain dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| EXT PAGE RX GAIN PRESS FLEX KEY (1-11) | - Press the [HOLD/SAVE] button for updating database permanently. |
| EXT PAGE RX GAIN PRESS FLEX KEY (1-11) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

## 15.11 CPT RX GAIN CONTROL  (PGM 410)

PROCEDURE

| CPT RX GAIN PRESS FLEX KEY (1-3) | - [TRANS/PGM] + 410. |
| --- | --- |
| CPT RX GAIN CPT/ACOB: 24 (00-63) | - Press one of Flex. BTNs (1~3) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows RX gain of the device from other devices. |
| CPT RX GAIN CPT/DCOB: 24 (00-63) | - For example, press the Flex. BTN 3 to change the Rx gain of CPT from DCOB. |
| CPT RX GAIN CPT/DCOB: 45 (00-63) | - To change the gain dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| CPT RX GAIN PRESS FLEX KEY (1-3) | - Press the [HOLD/SAVE] button for updating database permanently. |
| CPT RX GAIN PRESS FLEX KEY (1-3) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

## 15.12  MODEM RX GAIN CONTROL  (PGM 411)

PROCEDURE

| MODEM RX GAIN PRESS FLEX KEY (1-3) | - [TRANS/PGM] + 411. |
| --- | --- |
| MODEM RX GAIN MODEM /ACOB: 24 (00-63) | - Press one of Flex. BTNs (1~3) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows RX gain of the device from other devices. |
| MODEM RX GAIN MODEM /DCOB: 24 (00-63) | - For example, press the Flex. BTN 3 to change the Rx gain of Modem from DCOB. |
| MODEM RX GAIN MODEM /DCOB: 45 (00-63) | - To change the gain dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| MODEM RX GAIN PRESS FLEX KEY (1-3) | - Press the [HOLD/SAVE] button for updating database permanently. |
| MODEM RX GAIN PRESS FLEX KEY (1-3) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

## 15.13  SHORT SLIB GAIN CONTROL  (PGM 412)  ( SAF only )

PROCEDURE

| SHORT SLIB GAIN PRESS FLEX KEY (1-2) | (1) [TRANS/PGM] + 412. |
| --- | --- |
| SHORT SLIB GAIN S_SLIB/S_ACO: 32 (00-63) | (2) Press one of Flex. BTNs (1~5) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows Short SLIB gain of the device from other devices. |
| SHORT SLIB GAIN S_SLIB/S_ACO: 45 (00-63) | (3) To change the gain, dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| SHORT SLIB GAIN PRESS FLEX KEY (1-2) | (4) Press the [HOLD/SAVE] button for updating database permanently. |
| SHORT SLIB GAIN PRESS FLEX KEY (1-2) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | SHORT ACO | 00 – 63 | 31 |  |
| 2 | LONG ACO | 00 – 63 | 31 |  |

TABLE 15.13.1  Button Configuration for Short SLIB Gain  (PGM 412)

## 15.14  LONG SLIB GAIN CONTROL  (PGM 413)  ( SAF only )

PROCEDURE

| LONG SLIB GAIN PRESS FLEX KEY (1-2) | (1) [TRANS/PGM] + 413. |
| --- | --- |
| LONG SLIB GAIN L_SLIB/S_ACO: 32 (00-63) | (2) Press one of Flex. BTNs (1~5) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows Long SLIB gain of the device from other devices. |
| SHORT SLIB GAIN L_SLIB/S_ACO: 45 (00-63) | (3) To change the gain, dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| LONG SLIB GAIN PRESS FLEX KEY (1-2) | (4) Press the [HOLD/SAVE] button for updating database permanently. |
| LONG SLIB GAIN PRESS FLEX KEY (1-2) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

## 15.15  FAR SLIB GAIN CONTROL  (PGM 414)  ( SAF only )

PROCEDURE

| FAR SLIB GAIN PRESS FLEX KEY (1-2) | (1) [TRANS/PGM] + 414. |
| --- | --- |
| FAR SLIB GAIN F_SLIB/S_ACO: 32 (00-63) | (2) Press one of Flex. BTNs (1~5) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows Far SLIB gain of the device from other devices. |
| FAR SLIB GAIN F_SLIB/S_ACO: 45 (00-63) | (3) To change the gain, dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| FAR SLIB GAIN PRESS FLEX KEY (1-2) | (4) Press the [HOLD/SAVE] button for updating database permanently. |
| FAR SLIB GAIN PRESS FLEX KEY (1-2) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

## 15.16  SHORT ACO GAIN CONTROL  (PGM 415)   ( SAF only )

PROCEDURE

| SHORT ACO GAIN PRESS FLEX KEY (1-3) | (1) [TRANS/PGM] + 415. |
| --- | --- |
| SHORT ACO GAIN S_ACO/S_SLIB: 32 (00-63) | (2) Press one of Flex. BTNs (1~5) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows Short ACO gain of the device from other devices. |
| SHORT ACO GAIN S_ACO/S_SLIB: 45 (00-63) | (3) To change the gain, dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| SHORT ACO GAIN PRESS FLEX KEY (1-3) | (4) Press the [HOLD/SAVE] button for updating database permanently. |
| SHORT ACO GAIN PRESS FLEX KEY (1-3) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | SHORT SLIB | 00 – 63 | 35 |  |
| 2 | LONG SLIB | 00 – 63 | 41 |  |
| 3 | FAR SLIB | 00 – 63 | 41 |  |

TABLE 15.13.1  Button Configuration for Short SLIB Gain  (PGM 412)

## 15.17  LONG ACO GAIN CONTROL  (PGM 416)  ( SAF only )

PROCEDURE

| LONG ACO GAIN PRESS FLEX KEY (1-3) | (1) [TRANS/PGM] + 416. |
| --- | --- |
| LONG ACO GAIN L_ACO/S_SLIB: 32 (00-63) | (2) Press one of Flex. BTNs (1~5) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows Long ACO gain of the device from other devices. |
| LONG ACO GAIN L_ACO/S_SLIB: 45 (00-63) | (3) To change the gain, dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| LONG ACO GAIN PRESS FLEX KEY (1-3) | (4) Press the [HOLD/SAVE] button for updating database permanently. |
| LONG ACO GAIN PRESS FLEX KEY (1-3) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

## 15.18  SYSTEM TONE FREQUENCY  (PGM 420)

Frequency, user entered (dial tone, ringback tone, error tone, busy tone, dummy dial tone), may be changed to the closest system frequency that provides.

PROCEDURE

| SYS-TONE FREQUENCY DIAL (1-5) | - [TRANS/PGM] + 420. |
| --- | --- |
| DIAL TONE FREQUENCY T1:0425 T2:0000 | - To change system tone frequency, dial 1-5 and press Flex. BTN 1 (T1) or BTN 2 (T2) and dial 4 digits to enter new frequency. |
| SYS-TONE FREQUENCY DIAL (1-5) | - Press the [HOLD/SAVE] button for updating database permanently. |
| SYS-TONE FREQUENCY DIAL (1-5) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REAMRK |
| --- | --- | --- | --- | --- |
| 1 | Dial Tone | 0000 - 9999 | T1: - T2: - | Nation specific |
| 2 | RingBack Tone | 0000 - 9999 | T1: - T2: - | Nation specific |
| 3 | Busy Tone | 0000 – 9999 | T1: - T2: - | Nation specific |
| 4 | Error Tone | 0000 – 9999 | T1: - T2: - | Nation specific |
| 5 | Dummy Dial Tone | 0000-9999 | T1: - T2: - | Nation specific |

TABLE 15.18.1  Button Configuration for System Tone Frequency  (PGM 420)

## 15.19  DIFFERENTIAL RING FREQUENCY  (PGM 421)

Frequency for 4 differential rings may be changed to the closet ring frequency that provides. Station can change its own ring type signal by PGM 111-BTN 9.

PROCEDURE

| DIFF RING FREQUENCY DIAL (1-4) | - [TRANS/PGM] + 421. |
| --- | --- |
| DIFF RING FREQ(RNG 1) T1:1000 T2:1020 | - To change ring frequency of calling party, dial 1-4 and press Flex. BTN 1 (T1) or BTN 2 (T2) and dial 4 digits to enter new frequency. |
| DIFF RING FREQUENCY DIAL (1-4) | - Press the [HOLD/SAVE] button for updating database permanently. |
| DIFF RING FREQUENCY DIAL (1-4) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REAMRK |
| --- | --- | --- | --- | --- |
| 1 | Ring 1 | 0000 - 9999 | T1: - T2: - | Nation specific |
| 2 | Ring 2 | 0000 - 9999 | T1: - T2: - | Nation specific |
| 3 | Ring 3 | 0000 – 9999 | T1: - T2: - | Nation specific |
| 4 | Ring 4 | 0000 – 9999 | T1: - T2: - | Nation specific |

TABLE 15.19.1  Button Configuration for Differential Ring Frequency  (PGM 421)

## 15.20  DISTINCT RING FREQUENCY  (PGM 422)

Frequency for 4 distinct rings may be changed to the closet ring frequency that provides. The CO line can give its own ring type signal to the station in the system. It is assigned at PGM 142-BTN 5.

PROCEDURE

| DISTINCT RING FREQUENCY DIAL (1-4) | - [TRANS/PGM] + 422. |
| --- | --- |
| DIST RING FREQ(RNG 1) T1:0480 T2:0000 | - To change distinct CO ring frequency of called party, dial 1-4 and press BTN 1 (T1) or BTN 2 (T2) and dial 4 digits to enter new frequency. |
| DIST RING FREQUENCY DIAL (1-4) | - Press the [HOLD/SAVE] button for updating database permanently. |
| DIST RING FREQUENCY DIAL (1-4) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REAMRK |
| --- | --- | --- | --- | --- |
| 1 | Ring 1 | 0000 - 9999 | T1: - T2: - | Nation specific |
| 2 | Ring 2 | 0000 - 9999 | T1: - T2: - | Nation specific |
| 3 | Ring 3 | 0000 – 9999 | T1: - T2: - | Nation specific |
| 4 | Ring 4 | 0000 – 9999 | T1: - T2: - | Nation specific |

TABLE 15.20.1  Button Configuration for Distinct Ring Frequency  (PGM 422)

## 15.21  ACNR TONE CADENCE  (PGM 423)

Ring back tone, busy tone, error tone, or secondary dial tone may be programmed for ACNR.

PROCEDURE

| ACNR TONE CADENCE (20MS) DIAL (1-4) | - [TRANS/PGM] + 423. |
| --- | --- |
| RBACK TONE CADENCE ON:050 OFF:100 (20MS) | - To change ACNR tone cadence, dial 1-4 and press BTN 1 (T1) or BTN 2 (T2) and dial 4 digits to enter new cadence. |
| ACNR TONE CADENCE (20MS) DIAL (1-4) | - Press the [HOLD/SAVE] button for updating database permanently. |
| ACNR TONE CADENCE (20MS) DIAL (1-4) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Ring-Back Tone | 000 - 255 | ON: 050 / OFF: 100 | 20msec base |
| 2 | Busy Tone | 000 - 255 | ON: 025 / OFF: 025 | 20msec base |
| 3 | Error Tone | 000 - 255 | ON: 012 / OFF: 012 | 20msec base |
| 4 | S-Dial Tone | 000 - 255 | ON: 070 / OFF: 000 | 20msec base |

TABLE 15.21.1  Button Configuration for ACNR Cadence  (PGM 423)

# 16.	INITIALIZATION  (PGM 450)

The system has been pre-programmed with certain features which are called default data. These features are loaded into memory when the system is initialized. The system should be always initialized when installed or at any time the database has been corrupted. To initialize the system to the default values, proceed as follows;

PROCEDURE

| INITIALIZATION PRESS FLEX KEY (1-15) |  | - [TRANS/PGM] + 450. |
| --- | --- | --- |
| INITIALIZATION FLEX NUM PLAN |  | - Press one of Flex. BTNs (1-15) as described below table (Ex. Flex. BTN 1). |
| INITIALIZATION FLEX NUM PLAN |  | - Press the [HOLD/SAVE] button, then database is initialized with default data. Confirmation tone is heard. |

| BTN | ITEM | REMARK |
| --- | --- | --- |
| 1 | Flexible Numbering Plan Initialization | PGM105, PGM106, PGM107 |
| 2 | Station Database Initialization | PGM110, PGM111, PGM112, PGM113, PGM114, PGM 116, PGM117, PGM118, PGM119, PGM121, PGM122, PGM123, PGM124, PGM179 |
| 3 | CO Line Database Initialization | PGM140, PGM141, PGM142, PGM143, PGM144 |
| 4 | System Feature Database Initialization | PGM160 – PGM 177, PGM108 |
| 5 | Station Group Database Initialization | PGM190, PGM191 |
| 6 | ISDN Tables Database Initialization | PGM201, PGM202, PGM230, PGM231 |
| 7 | Reserved | None(Reserved) |
| 8 | System Timer Database Initialization | PGM180 – PGM182 |
| 9 | Toll Table Database Initialization | PGM224, PGM225 |
| 10 | LCR Database Initialization | PGM220 – PGM222 |
| 11 | Tables Initialization | PGM227 – PGM229, PGM232 – PGM235 |
| 12 | Flexible Button Program Initialization | PGM115 |
| 13 | Networking Database Initialization | PGM 320, PGM321,PGM322,PGM323, PGM 324 |
| 14 | All Database Initialization | Above All |
| 15 | System Reset By Software |  |

TABLE 16.1  Initialization  (PGM 450)

# 17.	PRINT PROT DATABASE  (PGM 451)

In order to obtain a hard copy printout of the database, a printer should be connected to the RS-232C connector.

PROCEDURE

| PROT DATA PRINT PRESS FLEX KEY (01-16) | - [TRANS/PGM] + 451. |
| --- | --- |
| PROT DATA PRINT STATION DATA | - To print database, press one of Flex. BTNs 01-15 and if there are entered data for selecting printed part, then selected database name will be displayed on the LCD. |
| PROT DATA PRINT STATION DATA | - To print out the selected database at step (2), press the [HOLD/SAVE] button. (Ex. Station Database). After printing database, confirmation tone is heard. |

| BTN | ITEM |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 1 | Flexible Numbering Plan Print |  |  |  |  |
| 2 | Station Database Print |  | STN_R |  |  |
| 3 | CO Line Database Print |  | CO_R |  |  |
| 4 | System Feature Database Print |  |  |  |  |
| 5 | Station Group Database Print |  |  |  |  |
| 6 | ISDN Tables Database Print |  |  |  |  |
| 7 | System Timer Database Print |  |  |  |  |
| 8 | Toll Table Database Print |  |  |  |  |
| 9 | LCR Database Print |  |  |  |  |
| 10 | Other Tables Print |  |  |  |  |
| 11 | Nation Specific Database Print |  |  |  |  |
| 12 | Flexible Button Program Print |  | STN_R |  |  |
| 13 | Networking Data Print |  |  |  |  |
| 14 | All Database Print |  |  |  |  |
| 15 | LCD Message Print |  |  |  |  |
|  | 1 | Language | 00 – 12 | Nation specific | 00:ENG 01:ITA 02:FIN 03:DUT 04:SWE 05:DAN 06:NOR 07:HUN 08:GER 09:FRE 10:POR 11:SPA 12:KOR |
|  | 2 | Station Type | 0 – 2 | 0 | 0: NORMAL 1:LG-GAP 2:LARGE |
| 16 | Print Port Data |  |  |  |  |

TABLE 17.1  Database Print  (PGM 451)

(Printing Example applying to LDK-300)

|  | Flexible Numbering Plan |  |  | STN Flex Numbering PGM 106 Flexible Numbering Plan A STA GRP PILOT NUMBER : 620-667 INT PAGE ZONES : 501-535 INT ALL CALL : 543 MEET ME PAGE : 544 EXT PAGE ZONE : 545 EXT PAGE ZONE : 546 EXT PAGE ZONE : 547 EXT ALL : 548 ALL CALL PAGE : 549 SMDR ACT CODE ENTER : 550 FLASH CMD TO CO : 551 SLT LAST SPD DIAL : 552 DND : 553 CALL FWD : 554 SPD DIAL PGM : 555 MSG WAIT ENABLE : 556 MSG WAIT RETURN : 557 SPD DIAL ACCESS : 558 DND/FWD CANCEL : 559 SLT_HOLD : 560 STA RELOC BACKUP : 561 STA RELOC RETRIEVE : 562 SLT PGM MODE ENTER : 563 ACD REROUTE : 564 PGM 107 Flexible Numbering Plan B ALARM RESET : 565 GROUP CALL PKUP : 566 UCD DND : 568 NIGHT ANSWER : 569 CALL PARK LOCATIONS : 601-619 DIRECT CALL PKUP : 7 ACCESS CO GROUP FEAT : 801-872 ACCESS IND CO FEAT : 88 TIE ROUTING ACCESS : 8901 ACCESS HELD CO FEAT : 8* ACCESS HELD IND CO FEAT : 8# ACCESS CO IN 1ST CO GRP : 9 ATTENDANT CALL : 0 DOOR OPEN 1 : #*1 DOOR OPEN 2 : #*2 DOOR OPEN 3 : #*3 DOOR OPEN 4 : #*4 DOOR OPEN 5 : #*5 DOOR OPEN 6 : #*6 DOOR OPEN 7 : #*7 VM MSG WAIT ENABLE : *8 VM MSG WAIT CANCEL : *9 |
| --- | --- | --- | --- | --- |
|  | Station Attributes |  |  | Station Attributes ------------------ Station 100 Attribute ====================== Station ID : KEYSET STATION ATTR1 (PGM111) AUTO SPKR :ON CALL FWD :OFF DND :OFF DATA SEC :OFF HOWLING :ON I-BOX SGNL:ON NO TCH ANS:ON PAGE ACC :OFF RING TYPE :0 SPK RING :HEAD SPK PHONE :ON VMIB SLOT :0 ICM GROUP :1 ERR TONE TAD:ON FLASH DROP:OFF STATION ATTR2 (PGM112) CO WARN :OFF AUTO HOLD :OFF TIME REST :OFF CO ACCESS :ENABLE CO QUEUE :ENABLE CO PGM :DISABLE PLA :ENABLE PREPAID :OFF SPD ACC :ENABLE TWOWAY RED:OFF FAX MODE :OFF OFFNET MOD:ALL UCD GRP SVC :OFF RING GRP SVC:OFF STATION ATTR3 (PGM113) ADMIN :ENABLE VMIB ACC :DISABLE GRP LISTN :DISABLE OVERRIDE :DISABLE SMDR HDN :DISABLE VOICE OVR :DISABLE WARM LINE :WARM ALARM MISB:OFF ALARM RAU1:OFF ALARM RAU2:OFF STATION ATTR4 (PGM114) CLIP DISP :ON COLP DISP :OFF CLI/REDIRT:CLI CLI M-WAIT:OFF EXT OR ATD:EXT KEYPAD FAC:DTMF LONG/SHORT:SHORT SUB ADDR :NOT_USED AUTO TEI :FIXED CLI NAME D:OFF CLI OUT NUM:100 PROG IND :OFF ISDN CLIR D:OFF ISDN COLR D:OFF DID REST : OFF STATION COS (PGM116) DAY COS : 1 NIGHT COS : 1 : (Printed like above for another station to range end station.) |
|  | Flex Buttons Assignment |  |  | Flex Button Assignment Station 101 Flex Button ======================== BTN 1 BTN 2 BTN 3 BTN 4 BTN 5 BTN 6 BTN 7 BTN 8 CO 1 CO 2 CO 3 CO 4 CO 5 CO 6 CO 7 LOOP BTN 9 BTN 10 BTN 11 BTN 12 BTN 13 BTN 14 BTN 15 BTN 16 EMPTY EMPTY EMPTY EMPTY EMPTY EMPTY EMPTY EMPTY BTN 17 BTN 18 BTN 19 BTN 20 BTN 21 BTN 22 BTN 23 BTN 24 EMPTY EMPTY EMPTY EMPTY EMPTY EMPTY EMPTY EMPTY BTN 25 BTN 26 BTN 27 BTN 28 BTN 29 BTN 30 BTN 31 BTN 32 EMPTY EMPTY EMPTY EMPTY EMPTY EMPTY EMPTY EMPTY BTN 33 BTN 34 BTN 35 BTN 36 BTN 37 BTN 38 BTN 39 BTN 40 EMPTY EMPTY EMPTY EMPTY EMPTY EMPTY EMPTY EMPTY BTN 41 BTN 42 BTN 43 BTN 44 BTN 45 BTN 46 BTN 47 BTN 48 EMPTY EMPTY EMPTY EMPTY EMPTY EMPTY EMPTY EMPTY : ( Printed like above for all keysets ) |
|  | CO Line Attributes |  |  | CO Line Attribute CO Line Ring Assignment ================================= CO 001 Ring Assignment DAY : STA101(0) NIGHT: STA101(0) ON-D : STA101(0) WEEK : STA101(0) Coline 1 Attribute ====================== Coline Attr1 (PGM141) CO GRP :1 CO COS :1 DISA ACCT :OFF CO ASGN TYPE :LOOP COLINE TYPE :CO OUT SGNL TYPE :DTMF FLASH TYPE :LOOP UNA :OFF CO GRP ACCT :OFF Coline Attr2 (PGM142) NAME DISPLAY :OFF CO NAME : SMDR METER :NONE LINE DROP(CPN):OFF DIST RING TYPE:0 MOH TYPE :INT MUSIC DIAL TONE :ON RING_BACK TONE:OFF ERROR TONE :OFF BUSY TONE :OFF ANNC TONE :OFF CO FLASH TMR :5 OPEN LOOP TMR :0 Coline Attr3 (PGM143) COLP TBL INDEX :NOT_ASG CLIP TBL INDEX:NOT_ASG CALL TYPE :NATIONAL DID CONV TYPE :0 DID RM NO :0 ENBLOCK SEND :OFF ( Printed like above for another CO line ) |
| System Database |  | PGM 100: Location Information Nation Code: 82 (KOREA) Site Name: Area Code: Station Prefix Code: .... PGM 101: Slot Information Slot# Board ID DEVS ----- ------------------------- ---------------- 1 DTIB12 12 STA devices 2 UNKNOWN 0 devices 3 UNKNOWN 0 devices 4 UNKNOWN 0 devices 5 UNKNOWN 0 devices 6 PRIB 30 COL devices 7 STIB 4 STA devices, 4 COL devices 8 UNKNOWN 0 devices 9 UNKNOWN 0 devices 10 UNKNOWN 0 devices 11 UNKNOWN 0 devices 12 UNKNOWN 0 devices 13 UNKNOWN 0 devices 14 UNKNOWN 0 devices 15 UNKNOWN 0 devices 16 UNKNOWN 0 devices 17 UNKNOWN 0 devices 18 UNKNOWN 0 devices 19 UNKNOWN 0 devices 20 UNKNOWN 0 devices 21 UNKNOWN 0 devices 22 UNKNOWN 0 devices 23 UNKNOWN 0 devices 24 UNKNOWN 0 devices 25 UNKNOWN 0 devices 26 UNKNOWN 0 devices 27 UNKNOWN 0 devices PGM 160 : System Attributes ATD CALL QUE RB TONE : OFF CAMP MOH/RBT : MOH CO LINE CHOICE : LAST DISA RETRY CNT : 3 ICM CONT DIAL TONE : CONT CO DIAL TONE DET : OFF EXT NIGHT RING : OFF HOLD PREFERENCE : SYS MULTI LINE CONF : ON PRT LCR CONV DGT : OFF CONF WARN TONE : ON PGM 161 : System Attributes NETWORK TIME/DATE SET : OFF OFF_HOOK RING SIG : MUTE OVRIDE 1ST CO GRP : OFF PAGE WARN TONE : ON AUTO PRIVACY : ON PRIVACY WARN TONE : ON SINGLE RING FOR CO : NO WTU AUTO RLS : OFF ACD PRN ENABLE : OFF ACD PRINT TMR : ON ACD CLR AFTER PRN : OFF ACD PRINT TMR UNIT : SEC |  |  |
| System Database |  |  | Other System Attributes ALARM ENABLE : OFF ALARM CONTACT : CLOSE ALARM MODE : ALARM ALARM SIGNAL MODE : ON CO2CO DAY COS : 1 CO2CO NIGHT COS : 1 BUSY DESTINATION : TONE ERROR DESTINATION : TONE NO ANS DESTINATION : TONE DIAL PULSE BRK RATIO : 66/33 EXT CNT(1):... EXT CNT(2):... EXT CNT(3):... EXT CNT(4):... EXT CNT(5):... EXT CNT(6):... EXT CNT(7):... RS232_PORT_1 BAUDRATE : 19200 RS232_PORT_1 CTS_RTS : OFF RS232_PORT_1 PAGE_BEAK : OFF RS232_PORT_1 LINE PAGE : 60 RS232_PORT_2 BAUDRATE : 19200 RS232_PORT_2 CTS_RTS : OFF RS232_PORT_2 PAGE_BEAK : OFF RS232_PORT_2 LINE PAGE : 60 RS232_PORT_3 BAUDRATE : 19200 RS232_PORT_3 CTS_RTS : OFF RS232_PORT_3 PAGE_BEAK : OFF RS232_PORT_3 LINE PAGE : 60 RS232_PORT_4 BAUDRATE : 19200 RS232_PORT_4 CTS_RTS : OFF RS232_PORT_4 PAGE_BEAK : OFF RS232_PORT_4 LINE PAGE : 60 RS232_PORT_5 BAUDRATE : 19200 RS232_PORT_5 CTS_RTS : OFF RS232_PORT_5 PAGE_BEAK : OFF RS232_PORT_5 LINE PAGE : 60 LCD TIME MODE : 12H LCD DATE MODE : DDMMYY SMDR Attributes SMDR SAVE : OFF SMDR PRINT : OFF RECORD TYPE : LD LD CALL DGT CNT : 7 PRINT INCOMING CALL : OFF PRINT LOST CALL : OFF RECORD IN DETAIL : ON HIDDEN DIALED DGT : 0 SMDR CURRENCY UNIT : COST PER PULSE : 0 SMDR FRACTION : 0 SMDR START TIMER(1sec): 0 SMDR HIDE DGT : RIGHT SMDR LD CODE: 0 ISDN System Attributes ADVICE OF CHARGE : NO SERVICE CO ATD CODE : .... IN PREFIX CODE INSERT : OFF OUT PREFIX CODE INSERT : ON A_U_LAW LINE INSTALLED : A_LAW CLI PRINT : OFF INTERNATIONAL ACCS CODE: CALLING SUB_ADDRESS : OFF |  |

| ISDN Tables | COLP Table Entry ================================================ COLP TABLE 00 : 12345 ------------------------ COLP TABLE 01 : 4536799 ------------------------ COLP TABLE 02 : ------------------------ COLP TABLE 03 : ------------------------ COLP TABLE 04 : ------------------------ COLP TABLE 05 : ------------------------ COLP TABLE 06 : : MSN Table Entry ================================================ MSN TABLE 0 ------------------------ COL_NO : 001. FLEX_DID_NO : .230 SUB_NO : 9 MSN_TEL_NO : 26303621 MSN TABLE 1 ------------------------ COL_NO : ... FLEX_DID_NO : ... SUB_NO : . MSN_TEL_NO : MSN TABLE 2 ------------------------ COL_NO : ... FLEX_DID_NO : ... SUB_NO : . MSN_TEL_NO : MSN TABLE 3 ------------------------ COL_NO : ... FLEX_DID_NO : ... SUB_NO : . MSN_TEL_NO : MSN TABLE 4 ------------------------ COL_NO : ... FLEX_DID_NO : ... SUB_NO : . MSN_TEL_NO : : Flexible Did Conv Table Entry ================================================ DID CONV TABLE 0 ------------------------ COL NAME : DAY DESTINATION : STA230 NIGHT DESTINATION : VMIB(#) 50 WEEKEND DESTINATION :SPD 2500 DID CONV TABLE 1 ------------------------ COL NAME : DAY DESTINATION : .... NIGHT DESTINATION : .... WEEKEND DESTINATION : .... DID CONV TABLE 2 ------------------------ COL NAME : DAY DESTINATION : .... NIGHT DESTINATION : .... WEEKEND DESTINATION : .... : |
| --- | --- |
| System Timers | System Timer Assignment ======================= System Timer 1 ATD RCL TIMER(min) :1 CALL PARK TIMER(sec) :120 CAMPON TRNS RCL TIMER(sec) :30 EXCL HOLD RCL TIMER(sec) :60 I-HOLD RCL TIMER(sec) :30 SYS HOLD RCL TIMER(sec) :30 TRANSFER RCL TIMER(sec) :30 ACNR DELAY TIMER(sec) :30 ACNR NO ANS TIMER(sec) :30 ACNR PAUSE TIMER(sec) :30 ACNR RETRY CNT :3 ACNR NO TONE RTY CNT :1 ACNR TONE DCT TIMER(sec) :30 CO AUTO RLS TIMER(sec) :30 CCR INT DGT TIMER(100ms) :30 CALL DROP WARN TIMER(sec) :10 CALL RESTRICT TIMER(min) :0 CO DIAL DELAY TIMER(100ms) :1 CO RLS GUARD TIMER(100ms) :2 RING OFF TIMER(100ms) :60 RING ON TIMER(100ms) :2 CO WARN TONE TIMER(sec) :180 System Timer 2 CFW NO ANS TIMER(sec) :15 DISA-DID NO ANS TIMER(sec) :20 VMIB USER RECORD TMR(sec) :20 VMIB VALID MSG TIMER(sec) :4 DOOR OPEN TIMER(100ms) :20 ICM BOX TIMER(sec) :30 DIAL TONE TIMER(sec) :10 INTER DGT TIMER(sec) :5 MSG WAIT REM TONE TMR(min) :0 PAGE TIMEOUT TIMER(sec) :15 PAUSE TIMER(sec) :3 PRESET CFW TIMER(sec) :10 System Timer 3 SLT HOOK BOUNCE TMR(100ms) :1 SLT MAX HOOK FLASH(100ms) :5 SLT MIN HOOK FLASH(10ms) :20 SLT RING PHASE(sec) :5 STA AUTO RLS TIMER(sec) :60 UNSUPER CONF TMR(min) :10 WAKE UP FAIL TIMER(sec) :20 WARM LINE TIMER(sec) :5 PP WINK TIMER(10ms) :10 ENBLOCK INT DGT TIMER(sec) :10 CCR TIME OUT TIMER(sec) :15 |

| Toll Data | TOLL Table Data Entry ================================================ Allow TABLE A Bin 1 : 012 Bin 2 : Bin 3 : Bin 4 : Bin 5 : : Allow TABLE B Bin 1 : 015 Bin 2 : Bin 3 : Bin 4 : Bin 5 : Deny TABLE A Bin 1 : 011 Bin 2 : 080 Bin 3 : 070 Bin 4 : Bin 5 : Deny TABLE B Bin 1 : 001 Bin 2 : 002 Bin 3 : Bin 4 : Bin 5 : Canned TOLL Table Data Entry ================================================ Canned Allow TABLE Bin 1 : 080 Bin 2 : 012 Bin 3 : 015 Bin 4 : Bin 5 : Canned Deny TABLE Bin 1 : 115 Bin 2 : Bin 3 : Bin 4 : Bin 5 : |
| --- | --- |

| LCR Data | LCR Table Data Entry ================================================ LCR Control Data ================================================ LCR_ACCESS_MODE : (M00)DISABLE LCR MON : DAY_ZONE(1) TUE : DAY_ZONE(1) WED : DAY_ZONE(1) THU : DAY_ZONE(1) FRI : DAY_ZONE(1) SAT : DAY_ZONE(1) SUN : DAY_ZONE(1) DAY_ZONE 1 TIME_ZONE_1 : 08 - 18 TIME_ZONE_2 : 19 - 24 TIME_ZONE_3 : 00 - 07 DAY_ZONE 2 TIME_ZONE_1 : 00 - 24 TIME_ZONE_2 : ... - ... TIME_ZONE_3 : ... - ... DAY_ZONE 3 TIME_ZONE_1 : 00 - 24 TIME_ZONE_2 : ... - ... TIME_ZONE_3 : ... - ... LCR Table LDT(Leading Digit Table) Entry ================================================ LDT Table (000) CODE : Working Mode : BOTH DMT_INDEX 0 :.. 00 00 00 DMT_INDEX 1 :.. 12 23 22 DMT_INDEX 2 :.. .. .. LDT Table (001) CODE : Working Mode : BOTH DMT_INDEX 0 :.. .. .. DMT_INDEX 1 :.. .. .. DMT_INDEX 2 :.. .. .. LDT Table (002) CODE : Working Mode : BOTH DMT_INDEX 0 :.. .. .. DMT_INDEX 1 :.. .. .. DMT_INDEX 2 :.. .. .. : |
| --- | --- |

| Other Tables | Emergency Code Data Entry ================================================ Entry 0 : 119 Entry 1 : 911 Entry 2 : 00911 Entry 3 : Entry 4 : Entry 5 : Entry 6 : Entry 7 : Entry 8 : Entry 9 : Author Code Data Entry ================================================ Entry 1 : 12345 Entry 2 : 34567 Entry 3 : 98765 Entry 4 : Entry 5 : Entry 6 : Entry 7 : CCR(Customer Call Routing) Table Entry ================================================ DVU Index : 1 ------------------------ CCR Entry 1 : HUNT 620 CCR Entry 2 : STA 101 CCR Entry 3 : SPD 2500 CCR Entry 4 : INT PAGE 1 CCR Entry 5 : .... CCR Entry 6 : .... CCR Entry 7 : .... CCR Entry 8 : .... CCR Entry 9 : .... CCR Entry 10 : .... DVU Index : 2 ------------------------ CCR Entry 1 : .... CCR Entry 2 : .... CCR Entry 3 : .... CCR Entry 4 : .... CCR Entry 5 : .... CCR Entry 6 : .... CCR Entry 7 : .... CCR Entry 8 : .... CCR Entry 9 : .... CCR Entry 10 : .... Exec/Sec Data Entry ================================================ Entry 1 : .... / .... Entry 2 : .... / .... Entry 3 : .... / .... Entry 4 : .... / .... Entry 5 : .... / .... Entry 6 : .... / .... Entry 7 : .... / .... Entry 8 : .... / .... |
| --- | --- |

| STN Group | Station Group Assignment STA GRP PILOT NUMBER : 620-667 ================================================================ Station Group : 620 Group Type: CIRCULAR GROUP ============================================= Group Member --------------------------------------------- 100 101 102 .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... .... --------------------------------------------- ANNC1 TIMER :15 ANNC2 TIMER :0 ANNC1 LOC :VMIB(NOT_ASG) ANNC2 LOC :VMIB(NOT_ASG) ANNC2 RPT TIMER :0 ANNC 2 REPEAT :OFF OVERFLOW DEST :NOT ASSIGNED OVERFLOW TIMER :180 WRAP UP TIMER :2 NO ANS TIMER :15 PILOT HUNT :ON REPORT NO MEM :OFF MUSIC SOURCE :0 Station Group : 621 Group Type: NOT ASSIGNED ============================================= |
| --- | --- |
| Nation Specific | NATION GAIN PRINT =============================================== DTIB/DTIB:26 DTIB/SLIB:33 DTIB/WTIB:26 DTIB/ACOB:33 DTIB/DCOB:33 DTIB/VMIB:29 DTIB/DTMF:08 DTIB/TONE:32 DTIB/MUSIC1:29 DTIB/MUSIC2:29 DTIB/MUSIC3:29 SYSTEM TONE FREQ ============================================== DIAL TONE FREQUENCY: (T1:0425 / T2:0000) RBACK TONE FREQUENCY: (T1:0425 / T2:0000) BUSY TONE FREQUENCY: (T1:0425 / T2:0000) ERROR TONE FREQUENCY: (T1:0620 / T2:0000) DDIAL TONE FREQUENCY: (T1:0350 / T2:0440) DIFFERENTIAL RING FREQ ============================================== DIFF RING FREQ(1): (T1:1000 / T2:1020) DIFF RING FREQ(2): (T1:0890 / T2:0910) DIFF RING FREQ(3): (T1:1260 / T2:1280) DIFF RING FREQ(4): (T1:0800 / T2:0820) DISTINCT RING FREQ ============================================== DIST RING FREQ(1): (T1:0480 / T2:0000) DIST RING FREQ(2): (T1:0400 / T2:0000) DIST RING FREQ(3): (T1:0620 / T2:0000) DIST RING FREQ(4): (T1:0770 / T2:0000) TONE CADENCE ============================================== RBACK TONE CADENCE: (T1:0050 / T2:0100) BUSY TONE CADENCE: (T1:0025 / T2:0025) ERROR TONE CADENCE: (T1:0012 / T2:0012) S_DIAL TONE CADENCE: (T1:0070 / T2:0000) |
| All Data | COMPLETE DATABASE PRINTING -------------------------- print above all |