- WIT-400H

- User Manual – Upgrade Part (WLAN)

THIS DOCUMENT CONTAINS PROPRIETARY INFORMATION AND MAY NOT BE REPRODUCED OR COPIED WITHOUT EXPRESS WRITTEN PERMISSION OF A DULY AUTHORIZED REPRESENTATIVE OF LG-Ericsson Co.Ltd.

Oct. 7-2010

# Appendix.	S/W Upgrade

WIT-400H can be upgraded via WLAN.

## A.1	Network configuration for upgrade

WLAN Upgrade Configuration

1. Connect PC LAN port to the network.

2. Connect WIT-400H to wireless network.

* The address of PC and WIT-400H should be within same network.

## A.2	WEB Server Settings

1. Create “C:\WIT400H” directory and copy HTTP File Server (hts.exe) to the directory

2. Run hfs.exe

3. Add file to upgrade

4. New image file is added

Server Port #

S/W image file

## A.3	Starting Upgrade

- When WIT-400H is boot up completely, connect WLAN network.

2. Press “#14563*” + “Send” button.

3. Set the server IP, port and file name.

1) Server IP: IP address of Web Server

2) Server Port: Web Server Port#

3) File Name: New S/W image of WIT-400H

4. When you select “DOWN LOAD”, popup will be displayed. Press “YES” to start upgrade.

5. The process has three steps as follows.

1) 1st step : S/W download

2) 2nd step : apply new S/W to the flash

3) 3rd step : reboot after upgrade

<Download>                 <Upgrade>

# 별첨.	S/W 업그레이드

WIT-400H의 S/W는 무선랜 접속으로 업그레이드를 할 수 있습니다.

## A.1	단말 업그레이드를 위한 환경 설정 방법

무선 업그레이드 환경

1. PC를 AP의 LAN포트에 연결을 한다.

2. 단말을 PC와 연결된 AP에 등록한다.

(SIP 서버에 등록되지 않아도 됨)

* 단말과 PC는 같은 네트워크에 있어야 합니다.

## A.2	WEB 서버 설정 방법

1. C: 에 WIT400H 폴더를 만들고, HTTP File Server (hfs.exe) 파일을 복사한다

2. hfs.exe를 실행한다.

3. WIT-400H 신규 이미지 파일을 공유 파일로 추가합니다.

4. WIT-400H 이미지 파일이 추가되었습니다.

## A.3	단말기에서 업그레이드 실행 방법

- 완전히 부팅된 단말과 PC를 무선랜으로 연결한다.

2. WIT-400H 단말에서 “#14563*#” + 통화버튼을 누른다.

3. 아래 설정값을 지정한다.

1) Server IP: Web Server PC IP 주소

2) Server Port: Web Server 포트 번호

3) File Name: WIT-400H에 복사한 신규 S/W 파일명

4. “확인”버튼을 누르면 아래와 같은 팝업창이 뜨고 “예”를 선택하면,

S/W 업그레이드를 시작합니다.

5. 아래와 같은 단계로 S/W 업그레이드를 진행합니다.

1) 1단계 : S/W 다운로드

2) 2단계 : S/W 업그레이드 진행

3) 3단계 : 단말 재 시작

<다운로드>           <업그레이드>