VOIM WANU Configuration Guide

Revision History

| DATE | BY | Revision | Changed |
| --- | --- | --- | --- |
| 30 Dec 2009 | J.W.Park | Initial release |  |
| 11 Mar 2010 | J.W.Park |  |  |
| 27 Aug 2010 | J.W.Park | E.0Bd | Add DNS address filed |
| 06 Sep 2010 | J.W.Park | E.0Be | Add firewall address field |
| 09 Nov 2010 | J.W.Park | E.0Bg | Add secondary SIP proxy address |

- Outline

The WANU feature of VOIM connects the iPECS-LIK system to the Internet like as commercial network routers. With setting up the port range and port forwarding, the iPECS-LIK system can be connected by using a single IP address. Using this feature, all system function such as web programming, telnet, remote IP phones, and other features are accessed by single IP address. Also the ALG functions of SIP and H.323 signals in the WANU enables the multiple IP GW call to Internet by using single IP address. Basically the WANU uses the static routing to connect the system to Internet and it enables the PC based program such as billing server connecting to Internet. The WANU is the additional function of existing VOIM, so the basic functions of VOIM are performed regardless of this WANU feature.

Internet

Web Admin & Telnet

Remote Phones

IP System

20.20.20.3

10.10.10.4

30.30.30.1

40.40.40.1

50.50.50.1

MFIM

VOIM

LIP

10.10.10.2

10.10.10.3

10.10.10.5

VOIMW

iPECS LIK

20.20.20.1

IP Signal

Media

Maintenance

< Figure 1 WANU Configuration >

Figure 1 illustrates the system configuration using the WANU. All messages toward the Internet are transmitted through the WANU.

VOIM

VOIM

MFIM Port Ranges

Local Port Ranges

VSF Port Ranges

PC Port Ranges

Other Forwarding Port Ranges

Other Forwarding Port Ranges

SIP Port Ranges

H.323 Port Ranges

ALG

SIP

H.323

MFIM

VSF

PC

VOIM

Media Port Ranges

WAN

WANU

< Figure 2 Block Diagram of Port Forwarding >

Figure 2 illustrates how the port forwarding is implemented in WANU. Some port related information should be set up in WANU to make the connection to Internet.

  - MFIM Port Ranges: IP address and TCP/UDP ports information for MFIM

  - VSF Port Ranges: VSF GW’s IP and ports information for message backup to remote phone.

  - PC Port Ranges: PC IP address for temporary Internet connectivity.

  - Other Forwarding Port Ranges: Additional port forwarding information to make the Internet connectivity.

  - SIP/H.323 Port Ranges: VOIM GW’s IP and signal ports information

  - Media Port Ranges : VOIM GW’s media ports information

- Glossary of Document

  - VOIMW: VOIM that the WANU feature is enabled.

  - LAN1: Ethernet port of VOIMW connected to system network.

  - LAN2: Ethernet port of VOIMW connected to Internet.

  - GW: iPECS-LIK Gateway.

  - ALG: Application-Level Gateway, it allows customized NAT traversal filters to be plugged into the gateway to support address and port translation for certain application layer "control/data" protocols such as SIP and H.323.

- Allocation of Physical Ethernet Port

VOIM have two physical Ethernet ports, the port marked "WAN" used to communicate with system for compatible with the existing method. And the port marked "LAN" used to communicate with Internet.

< Figure 3 Ethernet Port Usage >

For System Network (LAN1)

For Internet (LAN2)

- Change of System Configuration by WANU Function

Some of changes are needed in system setting to connect system to Internet by using the WANU feature.

  - MFIM

The IP address of MFIM should be set within the system network address of PGM102. The router address should be set with LAN1 IP address of VOIMW. And the firewall IP address should be set with LAN2 IP address of VOIMW. Some of TCP/UDP port ranges should be set and applied to WANU feature of VOIM if the internal VoIP GW is connecting to Internet with SIP or H.323 IP call. The port ranges of internal VSF GW should applied to WANU feature of VOIM if the internal VSF GW is connecting to Internet.

  - VOIM

The IP address of VOIM should be set within the system network address of PGM102. The router address should be set with LAN1 IP address of VOIMW. And the firewall IP address should be set with LAN2 IP address of VOIMW. Some of TCP/UDP port ranges should be set and applied to WANU feature of VOIM if the VOIM GW is connecting to Internet with SIP or H.323 IP call.

  - VMIM

The IP address of VMIM should be set within the system network address of PGM102. The router address should be set with LAN1 IP address of VOIMW. And the firewall IP address should be set with LAN2 IP address of VOIMW. The port ranges of VMIM GW should applied to WANU feature of VOIM if the VMIM GW is connecting to Internet.

  - VOIMW

The IP address of LAN1 should be set within the system network address of PGM102. The router address of VOIMW LAN1 itself should be set with IP address of LAN1. And the firewall IP address should be set with LAN2 IP address of VOIMW. Some of TCP/UDP port ranges should be set and applied to WANU feature of VOIM if the VOIM GW is connecting to Internet with SIP or H.323 IP call.

The above some of GW's port ranges and other additional setting should be applied to VOIM's WANU feature to enable the transmit the IP calls and media stream from system GWs to Internet.

- System Configuration 1 (MFIM’s Network Setting, PGM102-103)

The router and firewall IP addresses are set such as following example. In the example below, the VOIMW's LAN1/LAN2 IP addresses are 10.150.75.1/150.150.150.80.

  - Router IP Address

It should bet set to VOIMW’s LAN1 address.

  - Firewall IP Address

It should bet set to VOIMW’s LAN2 address. In case that system is behind NAT router, this address should be set to router’s firewall IP address.

- System Configuration 2 (VOIM/VOIMW Network Setting, PGM132)

The router and firewall IP addresses are set such as following example. In the example below, the VOIMW's LAN1/LAN2 IP addresses are 10.150.75.1/150.150.150.80. Here in case that system is behind NAT router, “Firewall IP Address” should be set to router’s firewall IP address.

- Configuration of Port Ranges (VoIP/VOIM/VOIMW)

All of port ranges of each GW should be re-allocated without overlap with other GW’s to make a correct IP calls to Internet. The port ranges can be set in maintenance mode of each GW including the internal VoIP GW of MFIM. And it also can be set in web interface of VOIM and VOIMW GW. After setting up the port ranges in each GW, the GW should be restarted to reflect the changing. There are three types of port ranges in which have to be re-allocated. These three ranges are for Data Sharing, IP Calls (SIP and H.323), and Media Stream. In the MFIM's internal VoIP GW, the Data Sharing is not supported. To avoid the confusing of usage of port ranges, we recommend dividing the port ranges with each characteristic as following.

  - Data Sharing Ports: start from 40000

  - IP Call: start from 41000

  - Media Stream: start from 42000

Compatible versions of the interface to set up the port ranges are as follows.

- MFIM VoIP: E.0Gk or later

- VOIM: D.3Db or later

- VOIMW: E.0Ac or later

Setting up the port ranges are performed as follows.

1) Maintenance Set (VoIP/VOIM/VOIMW)

[Data Sharing Port Settings]

maint> proxy dataports <start-port-in-decimal>

[H.323 port settings]

maint> proxy h323ports <start-port-in-decimal>

[Media Port Settings]

maint> proxy rtpports <start-port-in-decimal>

[Settings Check]

maint> proxy get

The figure shown below is an example VOIM8 GW which the ports for Data Sharing, IP Call, and Media Stream are set to 10000, 15000, and 20000.

[Clear settings]

maint> proxy reset

2) Web settings (VOIM/VOIMW)

The port ranges can be set in the "Relay" sub-menu of the "System" menu via web interface of GW. The title "Signal/RTP Port Range Configuration" is for set up the port ranges. The input fields are used as following.

[Data Sharing Port Settings]

Input the starting port number at left box and fill 0 at the rest right box.

[H.323 port settings]

Input the start port number at left box of "Q.931 signal ports" and fill 0 at the rest boxes.

[Media Port Settings]

Input the starting port number at left box of "RTP P1 ports" and fill 0 at the rest boxes.

After entering, click the "Save Setting" button. The figure showm below is an example of VOIM8 GW which the ports for Data Sharing, IP Call, and Media Stream are set to 10000, 15000, and 20000. The second figure displays the actual port ranges that GW is using.

3) Port setting with previous revision of MFIM’s VOIP GW

Configuring the port ranges with previous revision can be done in maintenance mode and some different commands have to be run. The range setting for H.323 port needs three commands with starting and ending port numbers. And for Media port needs one command with IP address, starting, and ending port numbers. Next is the how to run commands.

[H.323 port settings]

maint> proxy q931 <start-port-in-decimal> <end-port-in-decimal>

maint> proxy h245 <start-port-in-decimal> <end-port-in-decimal>

maint> proxy ras <start-port-in-decimal> <end-port-in-decimal>

Here, the size of port range for q931 and h245 parameters should be greater than 50. And the size of port range for RAS parameters should be greater than 10. The base number of three ranges can be used with same value such as 1xxxx. And normally you can set up the size of each range using the following formula.

Size-of-RAS = Size-of-Q931 + Size-of-H245

Here, the type of port q931 and h245 is a TCP. And the type of RAS port is UDP.

[Media Port Settings]

maint> proxy mcli <ip-address> <start-port-in-decimal> <end-port-in-decimal>

Here, the <ip-address> parameter is the IP address of MFIM’s. And other two port number is the range of media stream. The size of media port range should be greater than 112.

The figure shown below is an example which the ports for IP Call and Media Stream are set to 15000 and 20000.

- Changing Ports of Web and Remote Maintenance (VOIM/VOIMW)

The port numbers for web and maintenance of each GW have to be changed if there is an availability of access from the Internet for the debugging or maintenance. These two ports number can be changed in "Trace" sub-menu of the "System" menu. The field "Web access port" is to change web access port and the "Remote maintenance port" is to change the maintenance port number. After changing the web access port number, you have to type the new port number at URL window of web-browser for re-connection. The figure shown below is an example of VOIM8 GW for two ports are changed to 5008 and 8008.

1

2

- Configuring LAN2 Network (VOIMW)

The Internet IP address fields should be set up to make a connection to the Internet. The firewall address is used when system is behind the NAT router device and informs the real public address to WANU. The DNS Server Configuration is used when there is a SIP proxy server address with domain name in WANU/VOIP menu. In this configuration VOIMW needs DNS server address to detect IP address of that SIP proxy server. Here, “query interval” in seconds is used to send DNS query message to DNS server when there was no response in previous request.

The address fields can be set at "WAN" sub-menu of "WANU" menu. See the picture below.

- Configuring WANU Function 1 (VOIMW)

This part is for setting up the WANU feature and composed of the following configurations. Click the "Config" sub-menu of "WANU".

  - WANU Control: Enables the overall feature of the WANU. This should be set to ON to make connectivity to the Internet.

  - Accepting WAN side Access: Controls whether to allow access of below protocols from the Internet.

    - ICMP (ping): Accepting of Ping message

    - Web: Accepting of web programming

    - Maintenance: Accepting of remote maintenance

  - Accepting LAN Side Access: Controls whether to allow access of below protocols from the system network.

ICMP (ping): Accepting of Ping message

  - VOIM Function: Enables or disables of basic VOIM functions. The VOIM does not register to the system (MFIM) and just do WANU function if this field is selected to OFF.

- Configuring WANU Function 2 (VOIMW)

This part is for setting up the ALG function which is related to the IP calls of SIP and H.323 protocols. The ALG function enables the connection of IP calls to the Internet. And there is two part with SIP and H.323 in this configuration. Two parts are programmed independently.

  - SIP/H.323 On/Off: Enables the ALG functions of each protocol.

  - SIP/H.323 Seize: This is to select which method is used to connect the incoming IP call from the Internet. The "First" method connects the incoming IP call to IP GWS by using the order of GW information of ALG configuration. And the "Round" method connects the calls by load balancing. In most case the default value "Round" is used.

  - SIP/H.323 Poll: This is set the polling time to check whether the IP GW are keep alive. This polling is used to which IP GW is connected with call from the Internet. The check of keep alive is not performed if this polling timer is set to zero.

  - H.323 GK: This is to configure the H.323 call over the Internet is Gatekeeper routed call.

- Configuring MFIM Information (VOIMW)

This part is to configure the ports forwarding about that MFIM is using. All of ports related to MFIM are listed at one page. Click the "MFIM" sub-menu of "WANU" menu. The meaning of each field follows. Just type the MFIM IP address. Port numbers are used with default values.

  - IP address: MFIM IP address

  - 5588: Port number for remote GW is connecting to MFIM.

  - 80: Web Port

  - 69: TFTP upgrade Port for remote GW

  - 21/9700..9709: FTP upgrade Port for remote GW

  - 25: Port number for SMDR reporting transfer

  - 161: Port number for SNMP server

- Configuring VSF Information (VOIMW)

This part is to configure the ports forwarding about that VSF or VMIM are using to connect to the Internet, all of ports related to VMIM are listed at one page. Web and Maintenance fields are not used to MFIM's internal VSF GW. Click the "VSF" sub-menu of "WANU" menu.

- Configuring IP GW Information (VOIMW)

This part is to configure the GW information to make the IP call connection to Internet. The port ranges which are previously arranged and other information should be input in this part. The meaning of each fields are followed and the configuration is performed in "VOIP" sub-menu of "WANU".

  - VOIP IP Address: GW IP address

  - SIP Proxy Address: Enter SIP proxy server address if there is exist. Type the 255.255.255.255 if there is no SIP proxy server. And also domain name is acceptable in this field. DNS server address should be set in WANU/WAN menu if one of this filed is set with domain name. If this field is specified all incoming SIP calls from proxy server are forwarded to this GW. There is also field of secondary SIP proxy address to communicate with backed up server. In this new field IP address or subnet mask can be acceptable.

  - SIP Server Port: SIP UDP server port number. Currently it is fixed with 5060.

  - SIP Server TCP Port: SIP TCP server port number. Currently it is fixed with 5060.

  - SIP Server TLS Port: SIP securing server port number. Currently it is fixed with 5061.

  - H.323 Proxy IP Address: Reserved for the future purpose.

  - H.323 Server Port: H.323 server port number. Currently it is fixed in 1720.

  - Q.931/H.245/RAS Ports: This field is to set up the port range which each GW is using to transmit the H.323 call signals. Both starting and ending port numbers defined at GW "H.323 port settings" are needed at first field. And rest two fields are set to zero. The figure shown below is the screen when its starting and ending ports are set up to 15000 and 15031.

  - RTP Ports: This field is to set up the port range which each GW is using to transmit the media stream. Both starting and ending port numbers defined at GW "Media Port Settings" are needed at first field. and rest three fields are set to zero. The figure shown below is the screen when its starting and ending ports are set up to 20000 and 20999.

.

  - Remote Maintenance Port: Maintenance port number of each GW.

  - Web Server Port: Web server port number of each GW.

  - Data Relay Ports: This field is to set up the port range which each GW is using to transmit the data packet. Both starting and ending port number from GW "Data Sharing Port Settings" are needed at field.

  - ALG Total Channels: This field is to set up the maximum channel number of each IP GW. This information is used to manage the entire call traffic in the ALG function.

  - ALG Max SIP Channels: This field is to set up the maximum SIP channel number of each IP GW. The value of “CO VoIP Mode” in PGM140-142 should be respected here.

  - ALG Max H.323 Channels: This field is to set up the maximum H.323 channel number of each IP GW. The value of “CO VoIP Mode” in PGM140-142 should be respected here.

  - ALG SIP/H..323 Order: This field is to set up the priority number of each GW. And it is used to scheduling which IP GW is allocated to process the incoming call from the Internet. The number 0 is the highest priority and the default value -1 is the lowest priority.

  - ALG SIP/H.323 Call Direction: This field is to setup the direction of IP calls. The Tx means the IP call toward the Internet and Rx means the IP call from the Internet. The incoming call from Internet will not be allocated to an IP GW which the direction was specified to Tx only.

  - ALG H.323 Gatekeeper: This field is to set up the information that the H.323 call of this GW is routed by Gatekeeper.

The following is an example of setting VOIM8 for ALG.

13) The following is an example of the Overview.

- Configuring PC Information (VOIMW)

This configuration is to provide a temporary access to the Internet using a laptop when a system manager is visiting site. Caution is needed in use this setting because there is no traffic bandwidth limitation. This configuration can be used when FMC handset which is registered to LiK system needs to connect to Internet. Use the "PC" sub-menu and just type the IP address of the laptop.

< Single IP Input >

The following picture is an example of input IP range.

< Range IP Input >

The following picture is an example to permit all private hosts to connect to Internet.

< All Local Hosts >

- Configuring Port Forwarding (VOIMW)

Here the port forwarding can be done like that the commercial network router is providing. The third party's PC-based applications, such as billing server can communicate to the Internet by setting its port ranges here. Use the "Port Forward" sub-menu and type the IP addesss, protocol, and port range.

- Start and Stop WANU Function

The new configuration or changing of WANU feature is applied through this sub-menu. Button "Update" is to apply the current setting. And button "Stop" is to stop the WANU function without changing of current setting.

- Up/Download Configuration Files (VOIMW)

There are two configuration files in VOIM, one for its local DB file and the other is for storing the WANU configuration. With this feature, two configuration files can be up and downloaded by suing the submenu “Update”. At this menu “Normal Config” means the local DB file. And “WANU Config” means the WANU configuration file. So you have to choice the combo box what you want to download. The local DB file is stored with file extension “.cfg”. And WANU configuration file is stored with file extension “.adm”. The following picture is a snap shot of up and download feature.

- Configuring Flow to Implement the IP Call Binding

Below is the brief sequence to configure the IP call binding.

  - Register all IP GWs to MFIM.

  - Set up the MFIM's IP, router, and firewall address using the VOIMW's network information.

  - Set up the IP GW's router and firewall address using the VOIMW's network information.

  - Set up all port ranges in each IP GW.

  - Set up the VOIMW's LAN2 address.

  - Set up the MFIM and VSF information at VOIMW.

  - Set up the VOIP information which defined in step 4.

  - Enable the ALG and WANU functions.

  - Finally, apply the changed settings at "Update" sub-menu.

- Network Configuration Example

  - When system is behind the NAT router.

- local network: 192.168.1.1/24

- gateway IP: 192.168.1.254

- firewall IP: 20.30.40.50

- system network: 192.168.1.1/24

- system gateway IP: 192.168.1.1

- VOIMW local IP: 192.168.1.1

- VOIMW WAN, IP: 192.168.1.2

- VOIMW WAN, gateway IP: 192.168.1. 254

- MFIM or VOIM IP: 192.168.1.x

- system firewall IP: 20.30.40.50

- Port Forwarding in NAT router: all ports for system should be assigned to IP 192.168.1.2

  - When system have a public IP address

- local network: 10.10.10.1/24

- gateway IP: 10.10.10.254 (alive or not)

- Public IP : 20.30.40.50

- system network: 10.10.10.1/24

- system gateway IP: 10.10.10.1

- VOIMW local IP: 10.10.10..1

- VOIMW WAN, IP: 20.30.40.50

- VOIMW WAN, gateway IP: 20.30.40.x

- system firewall IP: 20.30.40.50

  - …

- VOIMW Upgrade

The VOIMW S/W will be released by divided in two files with suffix numbers such as "_1" and "_2". The file having suffix "_1" should be upgraded in first.