SPECIFICATION

Of Unified 1.2

Revision History

| DATE | BY | BEFORE | AFTER |
| --- | --- | --- | --- |
| 08Jan2015 | C.H.Wei | Initial release | 0.1 |
| 10Mar2015 | C.H.Wei |  | Add LCD Display, operation for SLT |
| 07Apr2015 | C.H.Wei |  | Call-Wait Integration, Normalization for all Country |

Table of Contents

1	Multiple ICM/CO call waiting functionality button	4

2	Call Parking of Internal Call	8

3	Alternative Route Selection(ARS) for Digit conversion table	9

# Multiple ICM/CO call waiting functionality button

Description

User Request

- User wants to hold multiple internal/external calls waiting on per a button without busy-return to caller if there are available buttons.

- User want to see indication of LED on button and caller name/number of waiting calls on LCD.

- Automatic hold/retrieve of calls by switchover of buttons

- Automatic transfer of talking call to a making call on a button

A new type of LOOP button named "Unified-LOOP(U-LOOP)" to hold waiting calls of internal and external calls.

Audible Muted(Off Hook)-Ring is presented for waiting calls.

Visual LED indication and LCD display is presented sequentially for waiting calls.

For SLT, maximum 2 normal calls and additional one camp-on call can be served for call-wait unconditionally.

|  |  | 1st cond. | 2nd cond. | Max. waiting | Audible | LED | LCD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LIP /LDP | Normal Incoming | P.113(LIK-P.111) Station Attributes {Call Wait = ON} | available empty U-LOOP buttons, or Station/CO(GRP)/ (CO)LOOP buttons | U-LOOP(48), Other Button(as available Flexible Button) | Muted-Ring | Green steady/held/recall, Amber Normal/Fast flashing Red Normal/Fast flashing | Name /Number of Waiting Call for a few second, back to original |
|  | Ring/ACD Group Call Queue Indication | P.191 Group Attr. {Indication = ON} | With or Without Button | N/A |  |  |  |
|  | Auto Hold : Station Data > Common Attributes(111) > “Automatic Hold = OFF/ON” Auto Transfer : Station Data > Common Attributes(111) > “Auto Transfer By Button = OFF/ON” |  |  |  |  |  |  |
| SLT | Normal Incoming | Call Wait = ON | Unconditionally | 2 calls | OffHook Signal Tone | No | Not Yet |
|  | Camp-On | When busy after 2 normal calls |  | 1 call |  |  |  |
|  | Pickup 2nd Incoming call, Switch between 2 picked-up Calls : 2 times of [Hook Flash] |  |  |  |  |  |  |
|  | One time of [Hook Flash] : Transfer make a new call => after connected => Broker Call by 1 time of [Hook Flash] / Conference by Fast 2 times of [Hook Flash] |  |  |  |  |  |  |

Operation

"Unified-LOOP(U-LOOP)" button creation : make self-coverage button(s)

- Web admin : select "U-Loop” Type and [SAVE]

- Keyset : [PGM] + flexible button + "call-coverage numbering plan" + "my station number"

Example) 671001 ('67' is call-coverage feature code, '1001' is my station number)

Call Wait Condition

- “Call Wait = ON/OFF” per a station (LIK:P.111> 43.Call Wait ; UCP P.113> 1.Call Wait)

“ON” : multiple waiting-call on multiple buttons

“OFF” : single waiting-call on single button

- Button per a waiting call : U-LOOP, or other Station/CO/CO-Grp./(CO)LOOP buttons

- Making call using an idle button

Call Wait Operation

- One Unified-LOOP(U-LOOP) button can keep one incoming/making/talking/held internal/external call.

- An incoming call is acknowledged to be waited if there is an idle Unified-LOOP(U-LOOP) button. First call to a station is acknowledged even without an idle button. From second call, there need to be an idle U-LOOP or Station/CO(Grp.)/(CO)LOOP button to accept the incoming call without reject it to ‘busy’.

- To make a call, user can dial internal or external number with or without press of the button.

If user presses a button to make a call then the button will be used for the call.

If user does not press a button then an idle button is automatically seized for the call.

Make of internal call: [button] + dial station number

Make of external call: [button] + co access code + external number

- Automatic call switching between buttons

  - Auto-Hold: a talking in a button is automatically sent to being held when press the other button.

  - Auto-Transfer: can transfer a talking call to a new making call by press idle button to make a call (ex. Operation: user is talking a call => press an empty U-LOOP button and make a call => complete the transfer by on-hook).

Call Waite Indication

- Audible Muted Ring (Continuous/One-Burst/Silence by P.111 Station Data Programming > “Off-Hook Ring Type”

- LCD Display

  - Display Caller Name & Number of Waiting Calls

  - Sequential Display : Talking Call => 1st waiting call (for 6 seconds) => 2nd waiting call (for 6 seconds) => … => back to talking call if there is no more waiting call

- LED Flashing

  - Talking/Making : Green Steady

  - Held : Green Normal-Flash

  - Held Recall : Green Fast-Flash, or Red Fast-Flash to distinguish from 1st waiting’ Red Normal-Flash

  - Incoming Waiting :

      - Single waiting : Amber Normal-Flash

      - Multiple waiting : 1st waiting(Red Normal-Flash), from-2nd (Amber Normal-Flash)

  - Differentiation of current LCD displayed Name &Number by LED : Amber Fast-Flash(for synchronization of Display and Flashing : Amber Fast-Flash button is displaying now)

Call Switching

- Automatic Hold : Station Data > Common Attributes(111) > Automatic Hold

- Automatic Transfer : Station Data > Common Attributes(111) > Auto Transfer By Button

(LIK system : System Data P.160-161 “Transfer ICM by Button : not used any more)

SLT

- Off-Hook Signaling Tone is presented when call comes in during a talking.

- Pick-up 2nd call during a talking state: Hook-Flash x 2 times of normal interval between Hook-Flashes. 1st Flash(transfer) + 2nd Flash(pickup waiting call or switch to the other call).

- Switch calls between one talking and the other one waiting call: Hook-Flash x 2 times of normal interval between Hook-Flashes.

- Camp-On Call : Can accept maximum 2 of normal incoming calls, and the 3rd call is returned to ‘busy’ and then caller can indicate ‘Camp-On’ by press ‘*’. The SLT user will hear Off-Hook signaling tone and pick up the camp-on call by 2 times of Hook-Flash on normal interval.

User can switch between 3 calls (2 normal incoming and 1 camp-on call) by press 2 times of Hook-Flash.

- Transfer a talking call : 1 time of Hook-Flash and then make a new call

- Switch of calls between on transferred call and a connected call(made) : 1 time of Hook-Flash

- Conference between SLT, transferred and a connected call(made) call : 2 times of Hook-Flash on fast interval(within 2 or 3 seconds).

- Recommended(by self-test) SLT Hook Flash Timer (P.182) : SLT Hook Flash Bounce Timer(1*100ms) / SLT Maximum Hook Switch Flash Timer(5*100ms) / SLT Minimum Hook Flash Timer(7*10ms)

Station Group Call-Queued Indication

- Regardless of “Call Wait” option

- Ring Group Call Ringing Indication : LIK(System Data P.160-161 > RING-GROUP INDICATION = OFF/ON), UCP(P.191 Group Attributes)

- ACD Group Call Queuing Indication : LIK(System Data P.160-161 > ACD-Group Queuing Call Indication = OFF/ON(Ring and LED)/ON(LED Only) ), UCP(P.191 Group Attributes)

- The Indication is served by a new incoming call waiting on a button or by LED/LCD only even though there is not an available/idle button.

Conditions

Introduction of various types of call-queuing (incoming/making /talking/held call) buttons

- DSS button :

- Assignment of a specific station number

- Call-queuing from/to the specific station number

- Presence of the station status

- CO channel button :

- Assignment of a specific CO channel

- Call-queuing for an external calling/receiving/held call via the the specific CO channel

- Presence of the CO channel status

- CO-Group button :

- Assignment of a specific CO-group number

- Call-queuing for an external calling/receiving/held call via the the specific CO-Group

- Presence of the queued call on the button

- CO-LOOP button :

- Assignment of a special type 'LOOP' for non-specific/random/whole-range of CO channels

- Call-queuing for an external calling/receiving/held call via the button

- Presence of the queued call on the button

- MSN-LOOP button :

- Assignment of a external DID telephone number

- Call-routing of an incoming external call with called party number to stations who have MSN LOOP buttons with same DID telephone number as the incoming called party number

- Call-queuing and presence of status of the incoming external call

- Call-queuing and presence of status of an outgoing external call which was created on the button when make a call by pressing the button.

- Multiple buttons assignment with same or different MSN telephone number

- An option : MSN Wait = OFF/ON (second incoming MSN routing call allow = yes/not)

- Unified-LOOP button :

- Assignment of coverage of my station number

- Multiple buttons assignment

- A button can have an receiving/making/held call for both type of internal and external call

- DSS/CO/CO-Group/CO-LOOP/MSN-LOOP button goes first than Unified-LOOP button

- (An incoming call is presented on related DSS/CO/CO-Group/CO-LOOP/MSN-LOOP button at first, the call is presented on an Unified-LOOP button at second priority)

- Do not need to enable "call-coverage-mode" feature for utilization of this button.

Combination with exist DSS/CO-Individual/CO-Group/CO-LOOP/MSN-LOOP buttons

- Priority

  - For internal call: DSS > Unified-LOOP

  - For external call: CO-Individual > MSN-LOOP > CO-Group > CO-LOOP > Unified-LOOP

- An incoming internal call

  - The call is acceptable if there is appropriate DSS button or an idle Unified-LOOP button.

  - Via DSS button first if there is associated DSS button.

- An incoming external call

  - The call is acceptable if there is an appropriate CO-Individual or idle MSN-LOOP or idle CO-Group or idle CO-LOOP or idle Unified-LOOP button.

  - Via one of above buttons according to priority.

- Making of internal call

  - In idle state, if dial a station number by dial pad

      - One of DSS button or Unified-LOOP button is seized automatically for the call

  - Seize DSS button and dial : the DSS button is seized

  - Seize Unified-LOOP button and dial

      - If there is DSS button, the DSS button is seized.

      - Or, the pressed Unified-LOOP button is seized.

- Making of internal call

  - In idle state, if dial CO-access-code and external number by dial pad

      - One of CO-Individual/CO-Group/CO-LOOP/Unified-LOOP button is seized automatically for the call

  - Seize CO-Individual/CO-Group/CO-LOOP button and dial : the CO-Individual/CO-Group/CO-LOOP button is seized.

  - Seize Unified-LOOP button and dial.

      - If there is CO-Individual/CO-Group/CO-LOOP button, the CO-Individual/CO-Group/CO-LOOP button is seized.

      - Or, the pressed Unified-LOOP button is seized.

- Auto-hold and auto-transfer are implemented also using previous DSS/CO-Individual/CO-Group/CO-LOOP/MSN-LOOP buttons

  - On internal/external talking state, the call is transferred to the right-next call of making by pressing of CO-Individual/CO-Group/CO-LOOP/MSN-LOOP/Unified-LOOP

Program

Refer to "Unified-LOOP(U-LOOP)" button creation

# Call Parking of Internal Call

Description

Previously, parking of only external call has been able to be made.

Now, parking of internal call can be made to same range of parking area as external.

Operation

- Transfer an internal/external call to a specific parking area in a talking state.

Press [TRANS] + “Parking Area Code”

Or, Press [TRANS] + “Parking Area Saved Flexible Button”

Or, Press “Parking Area Saved Flexible Button” without press [TRANS]

- Retrieve internal/external call from a specific parking area.

Press the “Parking Area Code” which is parking a call.

Or, Press flashing “Parking Area Saved Flexible Button” which is parking a call.

SLT

Press ‘Hook-Flash’ for [TRANS] function key

Conditions

Program

  - Numbering Plan : Call Park Locations (range)

  - Flexible Button Programming

# Alternative Route Selection(ARS) for Digit conversion table

Description

This feature is enabled if there are several paths in order to connect toward a destination System when a user use digit conversion feature.

If a selected path is not available for some reason (All Busy, Line Fault, etc) after digit conversion, Alternative Route Selection (ARS) will connect calls using predefined designated path in digit conversion table.

Operation

If set, Alternative Route Selection operation is automatic.

,

.

Conditions

- ARS is optional and ARS digit in digit conversion table must be programmed to be operational.

- ARS digits should be contained in the CO Group Access Code

- ARS can be used in coordination with Last Number Redial, Station Speed Dial, and System Speed Dial.

Program

Keyset Admin.

| SYSTEM | Digit conversion Table (PGM 270) |
| --- | --- |
| STATION | Digit conversion table index(114-Button 24-Button10) |
| CO | Digit conversion table index (141-Button20) |

Web Admin.

| TABLE DATA | Digit conversion Table (PGM 270) |
| --- | --- |
| STATION DATA | Common Attribute Digit conversion table |
| CO LINE DATA | Common Attribute Digit conversion table index |