# Lab - Implement Route Redistribution Between Multiple Protocols (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

# Topology

# Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| R1 | G0/0/0 | 10.1.12.1 | 255.255.255.0 |
| R1 | G0/0/1 | 10.1.11.1 | 255.255.255.0 |
| R1 | Loopback 0 | 10.1.1.1 | 255.255.255.0 |
| R2 | G0/0/0 | 10.1.12.2 | 255.255.255.0 |
| R2 | G0/0/1 | 10.1.23.2 | 255.255.255.0 |
| R3 | G0/0/0 | 10.1.23.3 | 255.255.255.0 |
| R3 | G0/0/1 | 10.1.32.3 | 255.255.255.0 |
| R3 | Loopback 0 | 10.3.3.3 | 255.255.255.0 |
| D1 | G1/0/11 | 10.1.11.2 | 255.255.255.0 |
| D1 | Loopback 0 | 198.51.100.1 | 255.255.255.128 |
| D2 | G1/0/11 | 10.1.32.2 | 255.255.255.0 |
| D2 | Loopback 0 | 209.165.201.1 | 255.255.255.128 |

# Objectives

Part 1: Build the Network and Configure Basic Device Settings

Part 2: Configure and Verify Two-Way Redistribution on R1

Part 3: Configure and Verify Two-Way Redistribution on R3

Part 4: Filter and Verify Redistribution using a Route Map

# Background / Scenario

Every routing protocol has a unique redistribution behavior. The default redistribution behavior for EIGRP, OSPF, and BGP is as follows:

External routes redistributed into EIGRP have a seed metric of infinity and EIGRP routes set with infinity are not installed into the EIGRP topology table.

External routes redistributed into OSPF by default, are Type 2 (E2) external. Routes sourced from BGP will have a seed metric of 1, while other routing protocols will have a seed metric of 20. Only classful networks are redistributed, not subnets.

External routes redistributed into BGP have the origin set to incomplete (?), the multi-exit discriminator (MED) is set to the IGP metric and the weight is set to 32,768. By default, BGP does not redistribute internal BGP routes.

In this lab, you will configure mutual or two-way redistribution between multiple EIGRP and OSPF on R1. Then you will configure two-way redistribution between OSPF and BGP on R3. Finally, a route map will be used to selectively redistribute routes.

Note: This lab is an exercise in configuring and verifying two-way route redistribution on router’s R1 and R3. Route redistribution in this lab does not reflect networking best practices.

Note: The routers used with CCNP hands-on labs are Cisco 4221 with Cisco IOS XE Release 16.9.4 (universalk9 image). The switches used in the labs are Cisco Catalyst 3650 with Cisco IOS XE Release 16.9.4 (universalk9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs.

Note: Make sure that all the devices have been erased and have no startup configurations. If you are unsure, contact your instructor.

Instructor Note: Refer to the Instructor Lab Manual for the procedures to initialize and reload devices.

# Required Resources

3 Routers (Cisco 4221 with Cisco IOS XE Release 16.9.4 universal image or comparable)

2 Switches (Cisco 3650 with Cisco IOS XE release 16.9.4 universal image or comparable)

1 PC (Windows with terminal emulation program)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

# Instructions

## Build the Network and Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings.

### Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram, and cable as necessary.

### Configure basic settings for each device.

Console into each device, enter global configuration mode, and apply the basic settings for the lab. Initial configurations for each device are listed below.

Open configuration window

Router R1

hostname R1

no ip domain lookup

banner motd # R1, Configure BGP Route Redistribution #

line con 0

exec-timeout 0 0

logging synchronous

exit

interface g0/0/0

ip address 10.1.12.1 255.255.255.0

no shutdown

exit

interface g0/0/1

ip address 10.1.11.1 255.255.255.0

no shutdown

exit

interface loopback 0

ip address 10.1.1.1 255.255.255.0

ip ospf network point-to-point

ip ospf cost 15

no shutdown

exit

router eigrp 64512

eigrp router-id 1.1.1.1

network 10.1.11.0 0.0.0.255

exit

router ospf 1

router-id 1.1.1.1

network 10.1.1.0 0.0.0.255 area 1

network 10.1.12.0 0.0.0.255 area 0

exit

end

Router R2

hostname R2

no ip domain lookup

banner motd # R2, Configure BGP Route Redistribution #

line con 0

exec-timeout 0 0

logging synchronous

exit

interface g0/0/0

ip address 10.1.12.2 255.255.255.0

no shutdown

exit

interface g0/0/1

ip address 10.1.23.2 255.255.255.0

no shutdown

exit

router ospf 1

router-id 2.2.2.2

network 10.1.12.0 0.0.0.255 area 0

network 10.1.23.0 0.0.0.255 area 0

end

Router R3

hostname R3

no ip domain lookup

banner motd # R3, Configure BGP Route Redistribution #

line con 0

exec-timeout 0 0

logging synchronous

exit

interface g0/0/0

ip address 10.1.23.3 255.255.255.0

no shutdown

exit

interface g0/0/1

ip address 10.1.32.3 255.255.255.0

no shutdown

exit

interface loopback 0

ip address 10.3.3.1 255.255.255.0

ip ospf network point-to-point

no shutdown

exit

router ospf 1

router-id 3.3.3.3

network 10.3.3.0 0.0.0.255 area 0

network 10.1.23.0 0.0.0.255 area 0

router bgp 64532

bgp router-id 3.3.3.3

no bgp default ipv4-unicast

neighbor 10.1.32.2 remote-as 64532

address-family ipv4

neighbor 10.1.32.2 activate

neighbor 10.1.32.2 next-hop-self

exit-address-family

end

Switch D1

hostname D1

no ip domain lookup

ip routing

banner motd # D1, Configure BGP Route Redistribution #

line con 0

exec-timeout 0 0

logging synchronous

exit

interface range g1/0/1-24

shutdown

exit

interface g1/0/11

no switchport

ip address 10.1.11.2 255.255.255.0

no shutdown

exit

interface loopback 0

ip address 198.51.100.1 255.255.255.128

no shutdown

exit

router eigrp 64512

eigrp router-id 11.11.11.11

network 10.1.11.0 0.0.0.255

network 198.51.100.0 0.0.0.127

end

Switch D2

hostname D2

no ip domain lookup

ip routing

banner motd # D2, Configure BGP Route Redistribution #

line con 0

exec-timeout 0 0

logging synchronous

exit

interface range g1/0/1-24

shutdown

exit

interface g1/0/11

no switchport

ip address 10.1.32.2 255.255.255.0

no shutdown

exit

interface loopback 0

ip address 209.165.201.1 255.255.255.128

no shutdown

exit

router bgp 64532

bgp router-id 22.22.22.22

no bgp default ipv4-unicast

neighbor 10.1.32.3 remote-as 64532

address-family ipv4

network 209.165.201.0 mask 255.255.255.128

neighbor 10.1.32.3 activate

exit-address-family

end

Set the clock on all devices to UTC time.

Save the running configuration to startup-config on all devices.

Close configuration window

### Verify EIGRP on R1.

Verify that R1 has one EIGRP neighbor with D1.

Open configuration window

R1# show ip eigrp neighbors

EIGRP-IPv4 Neighbors for AS(64512)

H   Address                 Interface              Hold Uptime   SRTT   RTO  QSeq

(sec)         (ms)       CntNum

0   10.1.11.2               Gi0/0/1                  10 00:04:08    3   100  032

Next, issue the show ip route eigrp command, as shown, and notice the internal EIGRP route is from D1, Loopback 0.

R1# show ip route eigrp | begin Gateway

Gateway of last resort is not set

198.51.100.0/25 is subnetted, 1 subnets

D        198.51.100.0

[90/130816] via 10.1.11.2, 00:07:43, GigabitEthernet0/0/1

### Verify OSPF on R1.

Verify that R1 has one OSPF neighbor with R2.

- R1# show ip ospf neighbor

Neighbor ID     Pri   State           Dead Time   Address         Interface

2.2.2.2           1   FULL/BDR        00:00:39    10.1.12.2       GigabitEthernet0/0/0

Next, on R1 issue the show ip route ospf command, as shown. Notice the two OSPF intra–area routes.

- R1# show ip route ospf | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 8 subnets, 2 masks

O        10.1.23.0/24 [110/2] via 10.1.12.2, 00:35:32, GigabitEthernet0/0/0

O        10.3.3.0/24 [110/3] via 10.1.12.2, 00:35:32, GigabitEthernet0/0/0

Close configuration window

### Verify OSPF on R3.

Verify that R3 has one OSPF neighbor with R2 using the show ip ospf neighbor command.

Open configuration window

R3# show ip ospf neighbor

Neighbor ID     Pri   State           Dead Time   Address         Interface

2.2.2.2           1   FULL/DR         00:00:36    10.1.23.2       GigabitEthernet0/0/0

Next, issue the show ip route ospf command, as shown. Notice the first route is an OSPF inter–area route from Area 1 on R1 with an OSPF cost of 17. Notice the other route is an OSPF intra–area prefix with an OSPF cost of 2.

- R3# show ip route ospf | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 6 subnets, 2 masks

O IA     10.1.1.0/24 [110/17] via 10.1.23.2, 06:07:43, GigabitEthernet0/0/0

O        10.1.12.0/24 [110/2] via 10.1.23.2, 06:16:33, GigabitEthernet0/0/0

### Verify BGP on R3.

Issue the show bgp ipv4 unicast neighbors command, as shown. Notice the “established” BGP peer at 10.1.32.2, D2.

R3# show bgp ipv4 unicast neighbors | include BGP

BGP neighbor is 10.1.32.2,  remote AS 64532, internal link

BGP version 4, remote router ID 22.22.22.22

BGP state = Established, up for 00:11:11

< some output omitted >

Next, issue the show bgp ipv4 unicast command and notice the 209.165.201.0/25 prefix is learned via internal BGP (iBGP).

R3# show bgp ipv4 unicast | begin Network

Network          Next Hop            Metric LocPrf Weight Path

*>i  209.165.201.0/25 10.1.32.2                0    100      0 i

Close configuration window

## Configure Two-Way Redistribution on R1

In this part of the lab, you will perform mutual EIGRP-to-OSPF and OSPF-to-EIGRP redistribution on R1.

### Redistribute EIGRP 64512 into OSPF.

By default, EIGRP routes redistributed into OSPF will be seen as external Type 2 (E2) routes. In this step, you will change the external Type 2 (E2) routes to external Type 1 (E1) routes and specify the subnets keyword.

Note: Best practice suggests always entering the keyword subnets. However, depending on the IOS version, the keyword subnets may automatically be appended to the redistribute command in OSPFv2.

Open configuration window

R1(config)# router ospf 1

R1(config-router)# redistribute eigrp 64512 metric-type 1 subnets

R1(config-router)# exit

Close configuration window

### Verify One-Way Redistribution on R3.

Issue the show ip route ospf on R3 to see the external OSPF routes are Type 1 with a cost of 22. Both E1 routes originated from EIGRP AS 64512.

Open configuration window

- R3# show ip route ospf | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 9 subnets, 2 masks

O IA     10.1.1.0/24 [110/17] via 10.1.23.2, 08:19:58, GigabitEthernet0/0/0

O E1     10.1.11.0/24 [110/22] via 10.1.23.2, 01:21:42, GigabitEthernet0/0/0

O        10.1.12.0/24 [110/2] via 10.1.23.2, 08:28:48, GigabitEthernet0/0/0

198.51.100.0/25 is subnetted, 1 subnets

O E1     198.51.100.0 [110/22] via 10.1.23.2, 01:21:42, GigabitEthernet0/0/0

Close configuration window

### Redistribute OSPF into EIGRP 64512.

By default, external EIGRP routes are given an administrative distance of 170 and a seed metric of infinity, which prevents the installation of the redistributed routes into the EIGRP topology table. This default path metric can be changed from infinity to a specific value for bandwidth, delay, reliability, load, and maximum transmission unit (MTU).

Redistribute OSPF into EIGRP 64512 and set the EIGRP K values as shown.

Open configuration window

R1(config)# router eigrp 64512

R1(config-router)# redistribute ospf 1 metric 1000000 10 255 1 1500

R1(config-router)# end

close configuration window

### Verify Two-Way Redistribution on D1.

Issue the show ip route eigrp | begin Gateway on D1 to see four external EIGRP routes from OSPF.

Open configuration window

D1# show ip route eigrp | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 6 subnets, 2 masks

D EX     10.1.1.0/24 [170/5376] via 10.1.11.1, 00:00:12, GigabitEthernet1/0/11

D EX     10.1.12.0/24

[170/5376] via 10.1.11.1, 00:00:12, GigabitEthernet1/0/11

D EX     10.1.23.0/24

[170/5376] via 10.1.11.1, 00:00:12, GigabitEthernet1/0/11

D EX     10.3.3.0/24 [170/5376] via 10.1.11.1, 00:00:12, GigabitEthernet1/0/11

Close configuration window

## Configure Two-Way Redistribution on R3

In this part of the lab you will perform OSPF-to-BGP and BGP-to-OSPF redistribution on R3.

### Redistribute OSPF into BGP.

By default, when configuring redistribution of OSPF into BGP without any keywords, only OSPF intra–area and inter–area routes are redistributed. Your BGP configuration determines where the redistribute command is entered. When using an address family, the redistribute command is entered in the address family configuration mode, otherwise it is entered under the BGP process.

In this lab, the redistribute command is configured under the BGP IPv4 address family as shown. Notice that no additional keywords or sub-commands are configured.

Open configuration window

R3(config)# router bgp 64532

R3(config-router)# address-family ipv4

R3(config-router-af)# redistribute ospf 1

R3(config-router-af)# exit

close configuration window

### Verify redistribution on D2.

Issue the show bgp ipv4 unicast on D2 to see the default behavior of OSPF being redistributed into BGP. Notice that only intra-area and inter-area routes are redistributed. All routes redistributed into BGP have the origin code set to incomplete (?) and the weight set to 32,768. Additionally, the MED / Metric value was set based on the OSPF cost on R3. The 10.1.1.0/24 was the inter-area route and shows a Metric of 17. The 10.1.12.0/24 route was an intra-area route with a Metric of 2. The 10.1.23.0/24 and 10.3.3.0/24 routes were directly connected via OSPF and show a Metric of 0.

Open configuration window

D2# show bgp ipv4 unicast | begin Origin

Origin codes: i - IGP, e - EGP, ? - incomplete

RPKI validation codes: V valid, I invalid, N Not found

Network          Next Hop            Metric LocPrf Weight Path

*>i  10.1.1.0/24      10.1.23.2               17    100      0 ?

*>i  10.1.12.0/24     10.1.23.2                2    100      0 ?

*>i  10.1.23.0/24     0.0.0.0                  0    100      0 ?

*>i  10.3.3.0/24      0.0.0.0                  0    100      0 ?

*>   209.165.201.0/25 10.1.32.2                0         32768 i

Close configuration window

Next, on R3 issue the redistribute command again, and add the keyword match to redistribute internal and external Type 1 OSPF routes into BGP.

Open configuration window

R3(config)# router bgp 64532

R3(config-router)# address-family ipv4

R3(config-router-af)# redistribute ospf 1 match internal external 1

R3(config-router-af)# exit

close configuration window

Issue the show bgp ipv4 unicast on D2, as shown, to see the two external OSPF routes redistributed into BGP. Notice the metric of 22 and origin code of incomplete (?). Remember that both prefixes originated in EIGRP AS 64512.

Open configuration window

D2# show bgp ipv4 unicast | begin Network

Network          Next Hop            Metric LocPrf Weight Path

*>i  10.1.1.0/24      10.1.32.3               17    100      0 ?

*>i  10.1.11.0/24     10.1.32.3               22    100      0 ?

*>i  10.1.12.0/24     10.1.32.3                2    100      0 ?

*>i  10.1.23.0/24     10.1.32.3                0    100      0 ?

*>i  10.3.3.0/24      10.1.32.3                0    100      0 ?

*>i  198.51.100.0/25  10.1.32.3               22    100      0 ?

*>   209.165.201.0/25 0.0.0.0                  0         32768 i

close configuration window

### Redistribute BGP into OSPF.

In this step of the lab, you will redistribute BGP into OSPF.

Note: BGP is designed to support a large routing table, whereas, IGP’s are not. Redistribution of BGP into an IGP on a router with a larger BGP routing table (for example the internet table with 800,000 plus routes) should use selective route redistribution. Otherwise the IGP can become unstable in the routing domain, which can lead to packet loss.

When redistributing BGP into OSPF, internal BGP routes are not redistributed, by default.

On R3 redistribute BGP into OSPF as shown, adding the subnets keyword and leaving the default OSPF external Type 2.

Open configuration window

R3(config)# router ospf 1

R3(config-router)# redistribute bgp 64532 subnets

R3(config-router)# exit

close configuration window

### Verify Redistribution of BGP into OSPF on R1.

Issue the show ip route ospf on R1. Notice the internal BGP prefix 209.165.201.0/25 was not redistributed and missing from the OSPF routing table. Normally only external BGP routes are redistributed. However, in this lab there are no eBGP routes.

Open configuration window

- R1# show ip route ospf | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 9 subnets, 2 masks

O        10.1.23.0/24 [110/2] via 10.1.12.2, 03:49:00, GigabitEthernet0/0/0

O        10.3.3.0/24 [110/3] via 10.1.12.2, 03:49:00, GigabitEthernet0/0/0

close configuration window

### Allow iBGP routes to be redistributed into OSPF.

To allow internal BGP routes to be redistributed into OSPF requires the bgp redistribute-internal command. This command is issued within the BGP address family process for IPv4, as shown.

Open configuration window

R3(config)# router bgp 64532

R3(config-router)# address-family ipv4

R3(config-router-af)# bgp redistribute-internal

R3(config-router-af)# end

close configuration window

### Verify Redistribution of iBGP into OSPF on R1.

Issue the show ip route ospf on R1 to see the OSPF external Type 2 (E2) route from BGP. Notice the default seed metric of 1 for BGP routes redistributed into OSPF.

Open configuration window

- R1# show ip route ospf | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 9 subnets, 2 masks

O        10.1.23.0/24 [110/2] via 10.1.12.2, 04:12:12, GigabitEthernet0/0/0

O        10.3.3.0/24 [110/3] via 10.1.12.2, 04:12:12, GigabitEthernet0/0/0

209.165.201.0/25 is subnetted, 1 subnets

O E2     209.165.201.0 [110/1] via 10.1.12.2, 00:00:06, GigabitEthernet0/0/0

close configuration window

From D2 ping the 198.51.100.1 address on D1 using the Loopback 0 address on D2. The ping should be successful. This verifies full end-to-end connectivity and successful redistribution on R1 and R3.

Open configuration window

- D2# ping 198.51.100.1 source 209.165.201.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 198.51.100.1, timeout is 2 seconds:

Packet sent with a source address of 209.165.201.1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 2/2/4 ms

close configuration window

## Filter and Verify Redistribution using a Prefix List and Route Map

In this part of the lab, you will use a prefix list and router map on R3 to filter specific OSPF prefixes from being redistributed into BGP.

### Create a prefix list named LOOPBACK and specify the action for each statement.

Permit only the Loopback addresses on D1, R1 and R3, as shown. The last sequence 20 statement filters all other prefixes. If not explicitly set, the deny statement is implied similar to using an ACL.

Open configuration window

R3(config)# ip prefix-list LOOPBACK seq 5 permit 198.51.100.0/25

R3(config)# ip prefix-list LOOPBACK seq 10 permit 10.1.1.0/24

R3(config)# ip prefix-list LOOPBACK seq 15 permit 10.3.3.0/24

R3(config)# ip prefix-list LOOPBACK seq 20 deny 0.0.0.0/0 le 32

### Apply the IP prefix list using a route map.

Create a route map named OSPF-into-BGP. Next, apply the prefix-list LOOPBACK to the route map which allows redistribution of prefixes into BGP. Any prefixes matching the named prefix list LOOPBACK with a permit statement, will be redistributed into BGP.

R3(config)# route-map OSPF-into-BGP permit 10

R3(config-route-map)# match ip address prefix-list LOOPBACK

R3(config-route-map)# exit

### Apply the route map to the redistribute command.

Apply the route map named OSPF-into-BGP at the end of the redistribute command, as shown.

R3(config)# router bgp 64532

R3(config-router)# address-family ipv4

R3(config-router-af)# redistribute ospf 1 match internal external 1 route-map OSPF-into-BGP

R3(config-router-af)# end

### Verify Redistribution Filtering.

Issue the show ip prefix-list detail command on R3 to verify the hit count for each sequence in the prefix list. Notice in our example each Loopback address has 2 hits, while the deny statement has 6 hits.

R3# show ip prefix-list detail

Prefix-list with the last deletion/insertion: LOOPBACK

ip prefix-list LOOPBACK:

count: 4, range entries: 1, sequences: 5 - 20, refcount: 3

seq 5 permit 198.51.100.0/25 (hit count: 2, refcount: 1)

seq 10 permit 10.1.1.0/24 (hit count: 2, refcount: 1)

seq 15 permit 10.3.3.0/24 (hit count: 2, refcount: 2)

seq 20 deny 0.0.0.0/0 le 32 (hit count: 6, refcount: 1)

close configuration window

Issue the show bgp ipv4 unicast to verify filtering of OSPF prefixes into BGP. Notice only the Loopback addresses on D1, R1 and R3 are redistributed into BGP.

Open configuration window

D2# show bgp ipv4 unicast | begin Network

Network          Next Hop            Metric LocPrf Weight Path

*>i  10.1.1.0/24      10.1.32.3               17    100      0 ?

*>i  10.3.3.0/24      10.1.32.3                0    100      0 ?

*>i  198.51.100.0/25  10.1.32.3               22    100      0 ?

*>   209.165.201.0/25 0.0.0.0                  0         32768 i

From D2 ping the 198.51.100.1 address on D1. The ping should not be successful.

D2# ping 198.51.100.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 198.51.100.1, timeout is 2 seconds:

.....

Success rate is 0 percent (0/5)

From D2 ping the 198.51.100.1 address on D1 using the Loopback 0 address on D2. The ping should be successful. This verifies full end-to-end connectivity and successful redistribution on R1 and R3 as well as redistribution filtering on R3 using a prefix list and route map.

- D2# ping 198.51.100.1 source 209.165.201.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 198.51.100.1, timeout is 2 seconds:

Packet sent with a source address of 209.165.201.1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 2/2/4 ms

close configuration window

# Reflection Questions

Why does the ping to 198.51.100.1 fail when you do not specify the source Loopback 209.165.201.1 on D2?

Type your answers here.

The ping from D2 uses the source address of 10.1.32.2 which is not an iBGP nor eBGP route. Therefore, this prefix is not being redistributed into OSPF. The ICMP echo reaches D1 but the ICMP echo-reply is being dropped by D1 since the 10.1.32.0/24 prefix in not in the routing table on D1.

By default, routes redistributed into BGP have the origin code, weight, and MED have which values?

Type your answers here.

The origin code is set to incomplete or ?, the weight is 32,768 and the MED is based on the IGP metric unless directly connected, which is given a MED value of 0.

By default, which OSFP prefixes are redistributed into BGP using the redistribute ospf 1 command?

Type your answers here.

Only classful intra-area and inter-area prefixes are redistributed into BGP.

Redistributed routes into OSPF have a metric of 20, with the exception of redistributed BGP routes which has a seed metric of _____ ?

Type your answers here.

one

# Router Interface Summary Table

| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| --- | --- | --- | --- | --- |
| 1800 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 1900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2801 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |
| 2811 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 4221 | Gigabit Ethernet 0/0/0 (G0/0/0) | Gigabit Ethernet 0/0/1 (G0/0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |
| 4300 | Gigabit Ethernet 0/0/0 (G0/0/0) | Gigabit Ethernet 0/0/1 (G0/0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |

Note: To find out how the router is configured, look at the interfaces to identify the type of router and how many interfaces the router has. There is no way to effectively list all the combinations of configurations for each router class. This table includes identifiers for the possible combinations of Ethernet and Serial interfaces in the device. The table does not include any other type of interface, even though a specific router may contain one. An example of this might be an ISDN BRI interface. The string in parenthesis is the legal abbreviation that can be used in Cisco IOS commands to represent the interface.

End of document

# Device Configs – Final

# Router R1

R1# show running-config

Building configuration...

Current configuration : 1943 bytes

!

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

no platform punt-keepalive disable-kernel-core

!

hostname R1

!

boot-start-marker

boot-end-marker

!

no aaa new-model

!

no ip domain lookup

!

login on-success log

!

subscriber templating

!

multilink bundle-name authenticated

!

diagnostic bootup level minimal

!

spanning-tree extend system-id

!

redundancy

mode none

!

interface Loopback0

ip address 10.1.1.1 255.255.255.0

ip ospf network point-to-point

ip ospf cost 15

!

interface GigabitEthernet0/0/0

ip address 10.1.12.1 255.255.255.0

negotiation auto

!

interface GigabitEthernet0/0/1

ip address 10.1.11.1 255.255.255.0

negotiation auto

!

interface Serial0/1/0

no ip address

!

interface Serial0/1/1

no ip address

!

router eigrp 64512

network 10.1.11.0 0.0.0.255

redistribute ospf 1 metric 1000000 10 255 1 1500

eigrp router-id 1.1.1.1

!

router ospf 1

router-id 1.1.1.1

redistribute eigrp 64512 metric-type 1 subnets

network 10.1.1.0 0.0.0.255 area 1

network 10.1.12.0 0.0.0.255 area 0

!

ip forward-protocol nd

no ip http server

ip http secure-server

!

control-plane

!

banner motd ^C R1, Configure BGP Route Redistribution ^C

!

line con 0

exec-timeout 0 0

logging synchronous

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

!

end

# Router R2

R2# show running-config

Building configuration...

Current configuration : 1585 bytes

!

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

no platform punt-keepalive disable-kernel-core

!

hostname R2

!

boot-start-marker

boot-end-marker

!

no aaa new-model

!

no ip domain lookup

!

login on-success log

!

subscriber templating

!

multilink bundle-name authenticated

!

diagnostic bootup level minimal

spanning-tree extend system-id

!

redundancy

mode none

!

interface GigabitEthernet0/0/0

ip address 10.1.12.2 255.255.255.0

negotiation auto

!

interface GigabitEthernet0/0/1

ip address 10.1.23.2 255.255.255.0

negotiation auto

!

router ospf 1

router-id 2.2.2.2

network 10.1.12.0 0.0.0.255 area 0

network 10.1.23.0 0.0.0.255 area 0

!

ip forward-protocol nd

no ip http server

ip http secure-server

!

control-plane

!

banner motd ^C R2, Configure BGP Route Redistribution ^C

!

line con 0

exec-timeout 0 0

logging synchronous

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

!

end

# Router R3

R3# show running-config

Building configuration...

Current configuration : 2367 bytes

!

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

no platform punt-keepalive disable-kernel-core

!

hostname R3

!

boot-start-marker

boot-end-marker

!

no aaa new-model

!

no ip domain lookup

!

login on-success log

!

Subscriber templating

!

multilink bundle-name authenticated

!

diagnostic bootup level minimal

!

spanning-tree extend system-id

!

redundancy

mode none

!

interface Loopback0

ip address 10.3.3.1 255.255.255.0

ip ospf network point-to-point

!

interface GigabitEthernet0/0/0

ip address 10.1.23.3 255.255.255.0

negotiation auto

!

interface GigabitEthernet0/0/1

ip address 10.1.32.3 255.255.255.0

negotiation auto

!

interface Serial0/1/0

no ip address

!

interface Serial0/1/1

no ip address

!

router ospf 1

router-id 3.3.3.3

redistribute bgp 64532 subnets

network 10.1.23.0 0.0.0.255 area 0

network 10.3.3.0 0.0.0.255 area 0

!

router bgp 64532

bgp router-id 3.3.3.3

bgp log-neighbor-changes

no bgp default ipv4-unicast

neighbor 10.1.32.2 remote-as 64532

!

address-family ipv4

bgp redistribute-internal

redistribute ospf 1 match internal external 1 route-map OSPF-into-BGP

neighbor 10.1.32.2 activate

neighbor 10.1.32.2 next-hop-self

exit-address-family

!

ip forward-protocol nd

no ip http server

ip http secure-server

!

ip prefix-list LOOPBACK seq 5 permit 198.51.100.0/25

ip prefix-list LOOPBACK seq 10 permit 10.1.1.0/24

ip prefix-list LOOPBACK seq 15 permit 10.3.3.0/24

ip prefix-list LOOPBACK seq 20 deny 0.0.0.0/0 le 32

!

route-map OSPF-into-BGP permit 10

match ip address prefix-list LOOPBACK

!

control-plane

!

banner motd ^C R3, Configure BGP Route Redistribution ^C

!

line con 0

exec-timeout 0 0

logging synchronous

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

!

end

# Switch D1

D1# show running-config

Building configuration...

Current configuration : 4117 bytes

!

version 16.9

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

! Call-home is enabled by Smart-Licensing.

service call-home

no platform punt-keepalive disable-kernel-core

!

hostname D1

!

vrf definition Mgmt-vrf

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

no aaa new-model

switch 1 provision ws-c3650-24ps

!

ip routing

!

no ip domain lookup

!

login on-success log

!

license boot level ipservicesk9

!

diagnostic bootup level minimal

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

redundancy

mode sso

!

transceiver type all

monitoring

!

class-map match-any system-cpp-police-topology-control

description Topology control

class-map match-any system-cpp-police-sw-forward

description Sw forwarding, L2 LVX data, LOGGING

class-map match-any system-cpp-default

description DHCP Snooping, EWLC control, EWCL data

class-map match-any system-cpp-police-sys-data

description Learning cache ovfl, Crypto Control, Exception, EGR Exception, NFL SAMPLED DATA, RPF Failed

class-map match-any system-cpp-police-punt-webauth

description Punt Webauth

class-map match-any system-cpp-police-l2lvx-control

description L2 LVX control packets

class-map match-any system-cpp-police-forus

description Forus Address resolution and Forus traffic

class-map match-any system-cpp-police-multicast-end-station

description MCAST END STATION

class-map match-any system-cpp-police-multicast

description Transit Traffic and MCAST Data

class-map match-any system-cpp-police-l2-control

description L2 control

class-map match-any system-cpp-police-dot1x-auth

description DOT1X Auth

class-map match-any system-cpp-police-data

description ICMP redirect, ICMP_GEN and BROADCAST

class-map match-any system-cpp-police-stackwise-virt-control

description Stackwise Virtual

class-map match-any non-client-nrt-class

class-map match-any system-cpp-police-routing-control

description Routing control

class-map match-any system-cpp-police-protocol-snooping

description Protocol snooping

class-map match-any system-cpp-police-system-critical

description System Critical and Gold

!

policy-map system-cpp-policy

!

interface Loopback0

ip address 198.51.100.1 255.255.255.128

!

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

speed 1000

negotiation auto

!

interface GigabitEthernet1/0/1

shutdown

!

interface GigabitEthernet1/0/2

shutdown

!

interface GigabitEthernet1/0/3

shutdown

!

interface GigabitEthernet1/0/4

shutdown

!

interface GigabitEthernet1/0/5

shutdown

!

interface GigabitEthernet1/0/6

shutdown

!

interface GigabitEthernet1/0/7

shutdown

!

interface GigabitEthernet1/0/8

shutdown

!

interface GigabitEthernet1/0/9

shutdown

!

interface GigabitEthernet1/0/10

shutdown

!

interface GigabitEthernet1/0/11

no switchport

ip address 10.1.11.2 255.255.255.0

!

interface GigabitEthernet1/0/12

shutdown

!

interface GigabitEthernet1/0/13

shutdown

!

interface GigabitEthernet1/0/14

shutdown

!

interface GigabitEthernet1/0/15

shutdown

!

interface GigabitEthernet1/0/16

shutdown

!

interface GigabitEthernet1/0/17

shutdown

!

interface GigabitEthernet1/0/18

shutdown

!

interface GigabitEthernet1/0/19

shutdown

!

interface GigabitEthernet1/0/20

shutdown

!

interface GigabitEthernet1/0/21

shutdown

!

interface GigabitEthernet1/0/22

shutdown

!

interface GigabitEthernet1/0/23

shutdown

!

interface GigabitEthernet1/0/24

shutdown

!

interface GigabitEthernet1/1/1

!

interface GigabitEthernet1/1/2

!

interface GigabitEthernet1/1/3

!

interface GigabitEthernet1/1/4

!

interface Vlan1

no ip address

!

router eigrp 64512

network 10.1.11.0 0.0.0.255

network 198.51.100.0 0.0.0.127

eigrp router-id 11.11.11.11

!

ip forward-protocol nd

ip http server

ip http secure-server

!

control-plane

service-policy input system-cpp-policy

!

banner motd ^C D1, Configure BGP Route Redistribution ^C

!

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

line vty 5 15

login

!

end

# Switch D2

D2# show running-config

Building configuration...

Current configuration : 4222 bytes

!

version 16.9

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

! Call-home is enabled by Smart-Licensing.

service call-home

no platform punt-keepalive disable-kernel-core

!

hostname D2

!

vrf definition Mgmt-vrf

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

no aaa new-model

switch 1 provision ws-c3650-24ps

!

ip routing

!

no ip domain lookup

!

logon on-success log

!

license boot level ipservicesk9

!

diagnostic bootup level minimal

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

redundancy

mode sso

!

transceiver type all

monitoring

!

class-map match-any system-cpp-police-topology-control

description Topology control

class-map match-any system-cpp-police-sw-forward

description Sw forwarding, L2 LVX data, LOGGING

class-map match-any system-cpp-default

description DHCP Snooping, EWLC control, EWCL data

class-map match-any system-cpp-police-sys-data

description Learning cache ovfl, Crypto Control, Exception, EGR Exception, NFL SAMPLED DATA, RPF Failed

class-map match-any system-cpp-police-punt-webauth

description Punt Webauth

class-map match-any system-cpp-police-l2lvx-control

description L2 LVX control packets

class-map match-any system-cpp-police-forus

description Forus Address resolution and Forus traffic

class-map match-any system-cpp-police-multicast-end-station

description MCAST END STATION

class-map match-any system-cpp-police-multicast

description Transit Traffic and MCAST Data

class-map match-any system-cpp-police-l2-control

description L2 control

class-map match-any system-cpp-police-dot1x-auth

description DOT1X Auth

class-map match-any system-cpp-police-data

description ICMP redirect, ICMP_GEN and BROADCAST

class-map match-any system-cpp-police-stackwise-virt-control

description Stackwise Virtual

class-map match-any non-client-nrt-class

class-map match-any system-cpp-police-routing-control

description Routing control

class-map match-any system-cpp-police-protocol-snooping

description Protocol snooping

class-map match-any system-cpp-police-system-critical

description System Critical and Gold

!

policy-map system-cpp-policy

!

interface Loopback0

ip address 209.165.201.1 255.255.255.128

!

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

speed 1000

negotiation auto

!

interface GigabitEthernet1/0/1

shutdown

!

interface GigabitEthernet1/0/2

shutdown

!

interface GigabitEthernet1/0/3

shutdown

!

interface GigabitEthernet1/0/4

shutdown

!

interface GigabitEthernet1/0/5

shutdown

!

interface GigabitEthernet1/0/6

shutdown

!

interface GigabitEthernet1/0/7

shutdown

!

interface GigabitEthernet1/0/8

shutdown

!

interface GigabitEthernet1/0/9

shutdown

!

interface GigabitEthernet1/0/10

shutdown

!

interface GigabitEthernet1/0/11

no switchport

ip address 10.1.32.2 255.255.255.0

!

interface GigabitEthernet1/0/12

shutdown

!

interface GigabitEthernet1/0/13

shutdown

!

interface GigabitEthernet1/0/14

shutdown

!

interface GigabitEthernet1/0/15

shutdown

!

interface GigabitEthernet1/0/16

shutdown

!

interface GigabitEthernet1/0/17

shutdown

!

interface GigabitEthernet1/0/18

shutdown

!

interface GigabitEthernet1/0/19

shutdown

!

interface GigabitEthernet1/0/20

shutdown

!

interface GigabitEthernet1/0/21

shutdown

!

interface GigabitEthernet1/0/22

shutdown

!

interface GigabitEthernet1/0/23

shutdown

!

interface GigabitEthernet1/0/24

shutdown

!

interface GigabitEthernet1/1/1

!

interface GigabitEthernet1/1/2

!

interface GigabitEthernet1/1/3

!

interface GigabitEthernet1/1/4

!

interface Vlan1

no ip address

!

router bgp 64532

bgp router-id 22.22.22.22

bgp log-neighbor-changes

no bgp default ipv4-unicast

neighbor 10.1.32.3 remote-as 64532

!

address-family ipv4

network 209.165.201.0 mask 255.255.255.128

neighbor 10.1.32.3 activate

exit-address-family

!

ip forward-protocol nd

ip http server

ip http secure-server

!

control-plane

service-policy input system-cpp-policy

!

banner motd ^C D2, Configure BGP Route Redistribution ^C

!

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

line vty 5 15

login

!

end