# Lab - Implement a GRE Tunnel (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

# Topology

# Addressing Table

| Device | Interface | IPv4 Address | IPv6 Address | IPv6 Link-Local |
| --- | --- | --- | --- | --- |
| R1 | G0/0/0 | 10.1.2.1/24 | 2001:db8:acad:12::1/64 | fe80::1:1 |
| R1 | Loopback 0 | 192.168.1.1/24 | 2001:db8:acad:1::1/64 | fe80::1:2 |
| R1 | Loopback 1 | 172.16.1.1/24 | 2001:db8:acad:1721::1/64 | fe80::1:3 |
| R2 | G0/0/0 | 10.1.2.2/24 | 2001:db8:acad:12::2/64 | fe80::2:1 |
| R2 | G0/0/1 | 10.2.3.2/24 | 2001:db8:acad:23::2/64 | fe80::2:1 |
| R3 | G0/0/0 | 10.2.3.3/24 | 2001:db8:acad:23::3/64 | fe80::3:1 |
| R3 | Loopback 0 | 192.168.3.1/24 | 2001:db8:acad:3::1/64 | fe80::3:2 |
| R3 | Loopback 1 | 172.16.3.1/24 | 2001:db8:acad:1723::1/64 | fe80::3:3 |

# Objectives

Part 1: Build the Network and Configure Basic Device Settings

Part 2: Configure and Verify GRE Tunnels with Static Routing

Part 3: Configure and Verify GRE Tunnels with Dynamic Routing

# Background / Scenario

Overlay networks allow you to insert flexibility into existing topologies. An existing physical topology is referred to as an underlay network. Generic Routing Encapsulation (GRE) protocol, which was originally developed by Cisco, is a very useful tool that allows you to create overlay networks to support many different purposes. GRE is very flexible and works with IPv4 and IPv6 in an underlay network. In this lab you will deploy basic GRE tunnels over both IPv4 and IPv6 underlay networks.

Note: This lab is an exercise in configuring and verifying various implementations of GRE tunnels and does not reflect networking best practices.

Note: The routers used with CCNP hands-on labs are Cisco 4221 with Cisco IOS XE Release 16.9.4 (universalk9 image). Other routers and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs.

Note: Ensure that the routers’ startup configurations have been erased and the devices reloaded if necessary. If you are unsure contact your instructor.

Instructor Note: Refer to the Instructor Lab Manual for the procedures to initialize and reload devices.

# Required Resources

3 Routers (Cisco 4221 with Cisco IOS XE Release 16.9.4 universal image or comparable)

1 PC (Choice of operating system with a terminal emulation program installed)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

# Instructions

## Build the Network and Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings.

### Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram, and cable as necessary.

### Configure basic settings for each switch.

Console into each router, enter global configuration mode, and apply the basic settings for the lab. Initial configurations for each device are listed below.

Open configuration window

Router R1

hostname R1

no ip domain lookup

ipv6 unicast-routing

banner motd # R1, Implement a GRE Tunnel #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

router eigrp EIGRP-IPv4_GRE_LAB

address-family ipv4 unicast autonomous-system 68

eigrp router-id 1.1.1.1

network 10.1.2.0 255.255.255.0

network 192.168.1.0 255.255.255.0

network 172.16.1.0 255.255.255.0

exit

exit

router eigrp EIGRP-IPv6_GRE_LAB

address-family ipv6 unicast autonomous-system 68

eigrp router-id 1.1.1.1

exit

exit

interface g0/0/0

ip address 10.1.2.1 255.255.255.0

ipv6 address fe80::1:1 link-local

ipv6 address 2001:db8:acad:12::1/64

no shutdown

exit

interface loopback 0

ip address 192.168.1.1 255.255.255.0

ipv6 address fe80::1:2 link-local

ipv6 address 2001:db8:acad:1::1/64

no shutdown

exit

interface loopback 1

ip address 172.16.1.1 255.255.255.0

ipv6 address fe80::1:3 link-local

ipv6 address 2001:db8:acad:1721::1/64

exit

Router R2

hostname R2

no ip domain lookup

ipv6 unicast-routing

banner motd # R2, Implement a GRE Tunnel #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

router eigrp EIGRP-IPv4_GRE_LAB

address-family ipv4 unicast autonomous-system 68

eigrp router-id 2.2.2.2

network 10.1.2.0 255.255.255.0

network 10.2.3.0 255.255.255.0

exit

exit

router eigrp EIGRP-IPv6_GRE_LAB

address-family ipv6 unicast autonomous-system 68

eigrp router-id 2.2.2.2

exit

exit

interface g0/0/0

ip address 10.1.2.2 255.255.255.0

ipv6 address fe80::2:1 link-local

ipv6 address 2001:db8:acad:12::2/64

no shutdown

exit

interface g0/0/1

ip address 10.2.3.2 255.255.255.0

ipv6 address fe80::2:2 link-local

ipv6 address 2001:db8:acad:23::2/64

no shutdown

exit

Router R3

hostname R3

no ip domain lookup

ipv6 unicast-routing

banner motd # R3, Implement a GRE Tunnel #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

router eigrp EIGRP-IPv4_GRE_LAB

address-family ipv4 unicast autonomous-system 68

eigrp router-id 3.3.3.3

network 10.2.3.0 255.255.255.0

network 192.168.3.1 255.255.255.0

network 172.16.3.0 255.255.255.0

exit

exit

router eigrp EIGRP-IPv6_GRE_LAB

address-family ipv6 unicast autonomous-system 68

eigrp router-id 3.3.3.3

exit

exit

interface g0/0/0

ip address 10.2.3.3 255.255.255.0

ipv6 address fe80::3:1 link-local

ipv6 address 2001:db8:acad:23::3/64

no shutdown

exit

interface loopback 0

ip address 192.168.3.1 255.255.255.0

ipv6 address fe80::3:2 link-local

ipv6 address 2001:db8:acad:3::1/64

no shutdown

exit

interface loopback 1

ip address 172.16.3.1 255.255.255.0

ipv6 address fe80::3:3 link-local

ipv6 address 2001:db8:acad:1723::1/64

no shutdown

exit

Set the clock on each device to UTC time.

Save the running configuration to startup-config.

Close configuration window

## Configure and Verify GRE Tunnels with Static Routing

In Part 2, you will configure and verify a GRE tunnel between R1 and R3. You will use static routes for overlay reachability and dynamic routing for underlay reachability. You will configure two tunnels, one for IPv4 traffic and one for IPv6 traffic. GRE tunnels are extremely flexible, and there are many options for implementation beyond what is being done in this lab.

### Verify reachability between R1 and R3.

From R1, ping R3 interface Loopback 0 using IPv4. All pings should be successful.

Open configuration window

R1# ping 192.168.3.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.3.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/1 ms

From R1, ping R3 interface Loopback 0 using IPv6. All pings should be successful.

R1# ping 2001:db8:acad:3::1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 2001:DB8:ACAD:3::1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/2/7 ms

### Create an IPv4-based GRE tunnel between R1 and R3.

On R1, create interface Tunnel 0, by specifying the IP address 100.100.100.1/30, a tunnel source of Loopback0, and a tunnel destination of 192.168.3.1.

R1(config)# interface tunnel 0

R1(config-if)# ip address 100.100.100.1 255.255.255.252

R1(config-if)# tunnel source loopback 0

R1(config-if)# tunnel destination 192.168.3.1

R1(config-if)# exit

On R1, create a static route to 172.16.3.0/24 via interface Tunnel 0.

R1(config)# ip route 172.16.3.0 255.255.255.0 tunnel 0

Close configuration window

On R3, create interface Tunnel 0, by specifying the IP address 100.100.100.2/30, a tunnel source of Loopback0, and a tunnel destination of 192.168.1.1.

Open configuration window

R3(config)# interface tunnel 0

R3(config-if)# ip address 100.100.100.2 255.255.255.252

R3(config-if)# tunnel source loopback 0

R3(config-if)# tunnel destination 192.168.1.1

R3(config-if)# exit

On R3, create a static route to 172.16.1.0/24 via interface Tunnel 0.

R3(config)# ip route 172.16.1.0 255.255.255.0 tunnel 0

Close configuration window

On R1, issue the show interface tunnel 0 command and examine the output.

Open configuration window

R1# show interface tunnel 0

Tunnel0 is up, line protocol is up

Hardware is Tunnel

Internet address is 100.100.100.1/30

MTU 9976 bytes, BW 100 Kbit/sec, DLY 50000 usec,

reliability 255/255, txload 1/255, rxload 1/255

Encapsulation TUNNEL, loopback not set

Keepalive not set

Tunnel linestate evaluation up

Tunnel source 192.168.1.1 (Loopback0), destination 192.168.3.1

Tunnel Subblocks:

src-track:

Tunnel0 source tracking subblock associated with Loopback0

Set of tunnels with source Loopback0, 1 member (includes iterators), on interface <OK>

Tunnel protocol/transport GRE/IP

Key disabled, sequencing disabled

Checksumming of packets disabled

Tunnel TTL 255, Fast tunneling enabled

Tunnel transport MTU 1476 bytes

Tunnel transmit bandwidth 8000 (kbps)

Tunnel receive bandwidth 8000 (kbps)

Last input never, output never, output hang never

Last clearing of "show interface" counters 00:02:45

Input queue: 0/375/0/0 (size/max/drops/flushes); Total output drops: 0

Queueing strategy: fifo

Output queue: 0/0 (size/max)

5 minute input rate 0 bits/sec, 0 packets/sec

5 minute output rate 0 bits/sec, 0 packets/sec

0 packets input, 0 bytes, 0 no buffer

Received 0 broadcasts (0 IP multicasts)

0 runts, 0 giants, 0 throttles

0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored, 0 abort

0 packets output, 0 bytes, 0 underruns

0 output errors, 0 collisions, 0 interface resets

0 unknown protocol drops

0 output buffer failures, 0 output buffers swapped out

From R1, ping 172.16.3.1. The pings should be successful.

### Create an IPv6-based GRE tunnel between R1 and R3.

On R1, create interface Tunnel 1, by specifying the IPv6 address 2001:db8:ffff::1/64, a tunnel source of Loopback 0, a tunnel destination of 2001:db8:acad:3::1, and the tunnel mode GRE IPv6.

R1(config)# interface tunnel 1

R1(config-if)# ipv6 address 2001:db8:ffff::1/64

R1(config-if)# tunnel source loopback 0

R1(config-if)# tunnel destination 2001:db8:acad:3::1

R1(config-if)# tunnel mode gre ipv6

R1(config-if)# exit

On R1, create a static route to 2001:db8:acad:1723::/64 via interface Tunnel 1.

R1(config)# ipv6 route 2001:db8:acad:1723::/64 tunnel 1

Close configuration window

On R3, create interface Tunnel 1, by specifying the IPv6 address 1002:db8:ffff::2/64, a tunnel source of Loopback 0, a tunnel destination of 2001:db8:acad:1::1 and the tunnel mode of GRE IPv6.

Open configuration window

R3(config)# interface tunnel 1

R3(config-if)# ipv6 address 2001:db8:ffff::2/64

R3(config-if)# tunnel source loopback 0

R3(config-if)# tunnel destination 2001:db8:acad:1::1

R3(config-if)# tunnel mode gre ipv6

R3(config-if)# exit

On R3, create a static route to 2001:db8:acad:1721::/64 via interface Tunnel 1.

R3(config)# ipv6 route 2001:db8:acad:1721::/64 tunnel 1

Close configuration window

On R1, issue the show interface tunnel 1 command and examine the output.

Open configuration window

R1# show interface tunnel 1

Tunnel1 is up, line protocol is up

Hardware is Tunnel

MTU 1456 bytes, BW 100 Kbit/sec, DLY 50000 usec,

reliability 255/255, txload 255/255, rxload 255/255

Encapsulation TUNNEL, loopback not set

Keepalive not set

Tunnel linestate evaluation up

Tunnel source 2001:DB8:ACAD:1::1 (Loopback0), destination 2001:DB8:ACAD:3::1

Tunnel Subblocks:

src-track:

Tunnel1 source tracking subblock associated with Loopback0

Set of tunnels with source Loopback0, 2 members (includes iterators),on interface <OK>

Tunnel protocol/transport GRE/IPv6

Key disabled, sequencing disabled

Checksumming of packets disabled

Tunnel TTL 255

Path MTU Discovery, ager 10 mins, min MTU 1280

Tunnel transport MTU 1456 bytes

Tunnel transmit bandwidth 8000 (kbps)

Tunnel receive bandwidth 8000 (kbps)

Last input 00:00:31, output 00:01:01, output hang never

Last clearing of "show interface" counters 00:06:58

Input queue: 0/375/0/0 (size/max/drops/flushes); Total output drops: 0

Queueing strategy: fifo

Output queue: 0/0 (size/max)

5 minute input rate 367000 bits/sec, 395 packets/sec

5 minute output rate 367000 bits/sec, 395 packets/sec

246335 packets input, 28574884 bytes, 0 no buffer

Received 0 broadcasts (0 IP multicasts)

0 runts, 0 giants, 0 throttles

0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored, 0 abort

246336 packets output, 28575000 bytes, 0 underruns

0 output errors, 0 collisions, 0 interface resets

0 unknown protocol drops

0 output buffer failures, 0 output buffers swapped out

From R1, ping 2001:db8:acad:1723::1. The pings should be successful.

Close configuration window

## Configure and Verify GRE Tunnels with Dynamic Routing

In Part 3, you will configure and verify GRE tunnels between R1 and R3. You will use dynamic routing for overlay reachability and static routing for underlay reachability. You will configure two tunnels, one for IPv4 traffic and one for IPv6 traffic.

### Remove the tunnel interfaces and dynamic routing on R1 and R3.

Open configuration window

Issue the no interface tunnel 0 and no interface tunnel 1 command on R1 and R3. This will also remove the static routes.

On R1, R2, and R3, remove EIGRP with the no router eigrp EIGRP-IPv4_GRE_LAB and no router eigrp EIGRP-IPv6_GRE_LAB commands.

### Replace the EIGRP configuration on R1, R2, and R3 with static routing.

On R1 and R3, create IPv4 and IPv6 static default routes that point to R2.

R1(config)# ip route 0.0.0.0 0.0.0.0 GigabitEthernet0/0/0 10.1.2.2

R1(config)# ipv6 route ::/0 2001:DB8:ACAD:12::2

R3(config)# ip route 0.0.0.0 0.0.0.0 GigabitEthernet0/0/0 10.2.3.2

R3(config)# ipv6 route ::/0 2001:DB8:ACAD:23::2

On R2, create IPv4 and IPv6 static routes that point to the R1 and R3 Loopback 0 networks.

R2(config)# ip route 192.168.1.0 255.255.255.0 10.1.2.1

R2(config)# ip route 192.168.3.0 255.255.255.0 10.2.3.3

R2(config)# ipv6 route 2001:db8:acad:1::/64 2001:db8:acad:12::1

R2(config)# ipv6 route 2001:db8:acad:3::/64 2001:db8:acad:23::3

Verify that R1 can reach Loopback 0 on R3 with pings that use the source address of the R1 Loopback 0 address.

R1# ping 192.168.3.1 source loopback 0

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.3.1, timeout is 2 seconds:

Packet sent with a source address of 192.168.1.1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/2 ms

R1# ping 2001:db8:acad:3::1 source loopback 0

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 2001:DB8:ACAD:3::1, timeout is 2 seconds:

Packet sent with a source address of 2001:DB8:ACAD:1::1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/1 ms

### Create an IPv4-based GRE tunnel between R1 and R3.

On R1, create interface Tunnel 0, by specifying the IP address 100.100.100.1/30, bandwidth of 4000 kbps, a tunnel source of Loopback 0, and a tunnel destination of 192.168.3.1.

R1(config)# interface tunnel 0

R1(config-if)# ip address 100.100.100.1 255.255.255.252

R1(config-if)# bandwidth 4000

R1(config-if)# ip mtu 1400

R1(config-if)# tunnel source loopback 0

R1(config-if)# tunnel destination 192.168.3.1

R1(config-if)# exit

On R1, configure classic EIGRP for IPv4 with router-id 1.1.1.1 and AS 68. The network statements should include Tunnel 0 and Loopback 1.

R1(config)# router eigrp 68

R1(config-router)# eigrp router-id 1.1.1.1

R1(config-router)# network 100.100.100.0 255.255.255.252

R1(config-router)# network 172.16.1.0 255.255.255.0

R1(config-router)# exit

On R3, create interface Tunnel 0, by specifying the IP address 100.100.100.2/30, bandwidth of 4000 kbps, a tunnel source of Loopback0, and a tunnel destination of 192.168.1.1.

R3(config)# interface tunnel 0

R3(config-if)# ip address 100.100.100.2 255.255.255.252

R3(config-if)# bandwidth 4000

R3(config-if)# ip mtu 1400

R3(config-if)# tunnel source loopback 0

R3(config-if)# tunnel destination 192.168.1.1

R3(config-if)# exit

On R3, configure classic EIGRP for IPv4 with router-id 3.3.3.3 and AS 68. The network statements should include Tunnel 0 and Loopback 1.

R3(config)# router eigrp 68

R3(config-router)# eigrp router-id 3.3.3.3

R3(config-router)# network 100.100.100.0 255.255.255.252

R3(config-router)# network 172.16.3.0 255.255.255.0

R3(config-router)# end

On R1, issue the show interface tunnel 0 command and examine the output.

R1# show interface tunnel 0

Tunnel0 is up, line protocol is up

Hardware is Tunnel

Internet address is 100.100.100.1/30

MTU 9976 bytes, BW 4000 Kbit/sec, DLY 50000 usec,

reliability 255/255, txload 1/255, rxload 1/255

Encapsulation TUNNEL, loopback not set

Keepalive not set

Tunnel linestate evaluation up

Tunnel source 192.168.1.1 (Loopback0), destination 192.168.3.1

Tunnel Subblocks:

src-track:

Tunnel0 source tracking subblock associated with Loopback0

Set of tunnels with source Loopback0, 1 member (includes iterators), on interface <OK>

Tunnel protocol/transport GRE/IP

Key disabled, sequencing disabled

Checksumming of packets disabled

Tunnel TTL 255, Fast tunneling enabled

Tunnel transport MTU 1476 bytes

Tunnel transmit bandwidth 8000 (kbps)

Tunnel receive bandwidth 8000 (kbps)

Last input 00:00:01, output 00:00:04, output hang never

Last clearing of "show interface" counters 00:06:11

Input queue: 0/375/0/0 (size/max/drops/flushes); Total output drops: 0

Queueing strategy: fifo

Output queue: 0/0 (size/max)

5 minute input rate 0 bits/sec, 0 packets/sec

5 minute output rate 0 bits/sec, 0 packets/sec

23 packets input, 2064 bytes, 0 no buffer

Received 0 broadcasts (0 IP multicasts)

0 runts, 0 giants, 0 throttles

0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored, 0 abort

58 packets output, 6784 bytes, 0 underruns

0 output errors, 0 collisions, 0 interface resets

0 unknown protocol drops

0 output buffer failures, 0 output buffers swapped out

On R1, issue the show ip route eigrp | begin Gateway command and verify that 172.16.3.0/24 appears in the routing table as an EIGRP route.

R1# show ip route eigrp | begin Gateway

Gateway of last resort is 10.1.2.2 to network 0.0.0.0

172.16.0.0/16 is variably subnetted, 3 subnets, 2 masks

D        172.16.3.0/24 [90/2048000] via 100.100.100.2, 00:02:01, Tunnel0

From R1, ping 172.16.3.1. The pings should be successful.

### Create an IPv6-based GRE tunnel between R1 and R3.

On R1, create interface Tunnel 1, by specifying the IPv6 address 2001:db8:ffff::1/64, bandwidth of 4000 kbps, a tunnel source of Loopback 0, and a tunnel destination of 2001:db8:acad:3::1.

R1(config)# interface tunnel 1

R1(config-if)# ipv6 address 2001:db8:ffff::1/64

R1(config-if)# bandwidth 4000

R1(config-if)# tunnel source loopback 0

R1(config-if)# tunnel destination 2001:db8:acad:3::1

R1(config-if)# tunnel mode gre ipv6

R1(config-if)# exit

On R1, configure classic EIGRP for IPv6 with router-id 1.1.1.1 and AS 68. Add the ipv6 eigrp 68 command to the Tunnel 1 and Loopback 1 interfaces.

R1(config)# ipv6 router eigrp 68

R1(config-rtr)# eigrp router-id 1.1.1.1

R1(config-rtr)# exit

R1(config)# interface tunnel 1

R1(config-if)# ipv6 eigrp 68

R1(config-if)# exit

R1(config)# interface loopback 1

R1(config-if)# ipv6 eigrp 68

R1(config-if)# end

On R3, create interface Tunnel 1, by specifying the IPv6 address 1002:db8:ffff::2/64, bandwidth of 4000 kbps, a tunnel source of Loopback 0, and a tunnel destination of 2001:db8:acad:1::1.

R3(config)# interface tunnel 1

R3(config-if)# ipv6 address 2001:db8:ffff::2/64

R3(config-if)# bandwidth 4000

R3(config-if)# tunnel source loopback 0

R3(config-if)# tunnel destination 2001:db8:acad:1::1

R3(config-if)# tunnel mode gre ipv6

R3(config-if)# exit

On R3, configure classic EIGRP for IPv6 with router-id 3.3.3.3 and AS 68. Add the ipv6 eigrp 68 command to the Tunnel 1 and Loopback 1 interfaces.

R3(config)# ipv6 router eigrp 68

R3(config-rtr)# eigrp router-id 3.3.3.3

R3(config-rtr)# exit

R3(config)# interface tunnel 1

R3(config-if)# ipv6 eigrp 68

R3(config-if)# exit

R3(config)# interface loopback 1

R3(config-if)# ipv6 eigrp 68

R3(config-if)# exit

On R1, issue the show interface tunnel 1 command and examine the output.

R1# show interface tunnel 1

Tunnel1 is up, line protocol is up

Hardware is Tunnel

MTU 1456 bytes, BW 4000 Kbit/sec, DLY 50000 usec,

reliability 255/255, txload 1/255, rxload 1/255

Encapsulation TUNNEL, loopback not set

Keepalive not set

Tunnel linestate evaluation up

Tunnel source 2001:DB8:ACAD:1::1 (Loopback0), destination 2001:DB8:ACAD:3::1

Tunnel Subblocks:

src-track:

Tunnel1 source tracking subblock associated with Loopback0

Set of tunnels with source Loopback0, 2 members (includes iterators),on interface <OK>

Tunnel protocol/transport GRE/IPv6

Key disabled, sequencing disabled

Checksumming of packets disabled

Tunnel TTL 255

Path MTU Discovery, ager 10 mins, min MTU 1280

Tunnel transport MTU 1456 bytes

Tunnel transmit bandwidth 8000 (kbps)

Tunnel receive bandwidth 8000 (kbps)

Last input 00:00:09, output 00:00:04, output hang never

Last clearing of "show interface" counters 00:04:20

Input queue: 0/375/0/0 (size/max/drops/flushes); Total output drops: 0

Queueing strategy: fifo

Output queue: 0/0 (size/max)

5 minute input rate 0 bits/sec, 0 packets/sec

5 minute output rate 0 bits/sec, 0 packets/sec

31 packets input, 4048 bytes, 0 no buffer

Received 0 broadcasts (0 IP multicasts)

0 runts, 0 giants, 0 throttles

0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored, 0 abort

46 packets output, 5864 bytes, 0 underruns

0 output errors, 0 collisions, 0 interface resets

0 unknown protocol drops

0 output buffer failures, 0 output buffers swapped out

On R1, issue the show ipv6 route eigrp | section D command and verify that 2001:db8:acad:1723::/64 appears in the routing table as an OSPF route.

R1# show ipv6 route eigrp | section D

I2 - ISIS L2, IA - ISIS interarea, IS - ISIS summary, D - EIGRP

EX - EIGRP external, ND - ND Default, NDp - ND Prefix, DCE - Destination

NDr - Redirect, RL - RPL, O - OSPF Intra, OI - OSPF Inter

D   2001:DB8:ACAD:1723::/64 [90/2048000]

via FE80::2FC:BAFF:FE94:29B0, Tunnel1

From R1, ping 2001:db8:acad:1723::1. The pings should be successful.

Close configuration window

# Router Interface Summary Table

| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| --- | --- | --- | --- | --- |
| 1800 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 1900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2801 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |
| 2811 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 4221 | Gigabit Ethernet 0/0/0 (G0/0/0) | Gigabit Ethernet 0/0/1 (G0/0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |
| 4300 | Gigabit Ethernet 0/0/0 (G0/0/0) | Gigabit Ethernet 0/0/1 (G0/0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |

Note: To find out how the router is configured, look at the interfaces to identify the type of router and how many interfaces the router has. There is no way to effectively list all the combinations of configurations for each router class. This table includes identifiers for the possible combinations of Ethernet and Serial interfaces in the device. The table does not include any other type of interface, even though a specific router may contain one. An example of this might be an ISDN BRI interface. The string in parenthesis is the legal abbreviation that can be used in Cisco IOS commands to represent the interface.

End of document

# Device Configs - Final

# Router R1

R1# show run

Building configuration...

Current configuration : 2231 bytes

!

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

no platform punt-keepalive disable-kernel-core

!

hostname R1

!

boot-start-marker

boot-end-marker

!

no aaa new-model

!

no ip domain lookup

!

login on-success log

!

subscriber templating

!

ipv6 unicast-routing

multilink bundle-name authenticated

!

spanning-tree extend system-id

!

redundancy

mode none

!

interface Loopback0

ip address 192.168.1.1 255.255.255.0

ipv6 address FE80::1:2 link-local

ipv6 address 2001:DB8:ACAD:1::1/64

!

interface Loopback1

ip address 172.16.1.1 255.255.255.0

ipv6 address FE80::1:3 link-local

ipv6 address 2001:DB8:ACAD:1721::1/64

ipv6 eigrp 68

!

interface Tunnel0

bandwidth 4000

ip address 100.100.100.1 255.255.255.252

ip mtu 1400

tunnel source Loopback0

tunnel destination 192.168.3.1

!

interface Tunnel1

bandwidth 4000

no ip address

ipv6 address 2001:DB8:FFFF::1/64

ipv6 eigrp 68

tunnel source Loopback0

tunnel mode gre ipv6

tunnel destination 2001:DB8:ACAD:3::1

tunnel path-mtu-discovery

!

interface GigabitEthernet0/0/0

ip address 10.1.2.1 255.255.255.0

negotiation auto

ipv6 address FE80::1:1 link-local

ipv6 address 2001:DB8:ACAD:12::1/64

!

interface GigabitEthernet0/0/1

no ip address

negotiation auto

!

interface Serial0/1/0

no ip address

!

interface Serial0/1/1

no ip address

!

router eigrp 68

network 100.100.100.0 0.0.0.3

network 172.16.1.0 0.0.0.255

eigrp router-id 1.1.1.1

!

ip forward-protocol nd

no ip http server

ip http secure-server

ip route 0.0.0.0 0.0.0.0 GigabitEthernet0/0/0 10.1.2.2

!

ipv6 route ::/0 2001:DB8:ACAD:12::2

ipv6 router eigrp 68

eigrp router-id 1.1.1.1

!

control-plane

!

banner motd ^C R1, Implement a GRE Tunnel ^C

!

line con 0

exec-timeout 0 0

logging synchronous

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

exec-timeout 0 0

privilege level 15

password cisco123

logging synchronous

login

!

end

# Router R2

R2# show run

Building configuration...

Current configuration : 1960 bytes

!

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

no platform punt-keepalive disable-kernel-core

!

hostname R2

!

boot-start-marker

boot-end-marker

!

no aaa new-model

!

no ip domain lookup

!

login on-success log

!

subscriber templating

!

ipv6 unicast-routing

multilink bundle-name authenticated

!

spanning-tree extend system-id

!

redundancy

mode none

!

interface GigabitEthernet0/0/0

ip address 10.1.2.2 255.255.255.0

negotiation auto

ipv6 address FE80::2:1 link-local

ipv6 address 2001:DB8:ACAD:12::2/64

!

interface GigabitEthernet0/0/1

ip address 10.2.3.2 255.255.255.0

negotiation auto

ipv6 address FE80::2:2 link-local

ipv6 address 2001:DB8:ACAD:23::2/64

!

ip forward-protocol nd

no ip http server

ip http secure-server

ip route 192.168.1.0 255.255.255.0 10.1.2.1

ip route 192.168.3.0 255.255.255.0 10.2.3.3

!

ipv6 route 2001:DB8:ACAD:1::/64 2001:DB8:ACAD:12::1

ipv6 route 2001:DB8:ACAD:3::/64 2001:DB8:ACAD:23::3

!

control-plane

!

banner motd ^C R2, Implement a GRE Tunnel ^C

!

line con 0

exec-timeout 0 0

logging synchronous

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

exec-timeout 0 0

privilege level 15

password cisco123

logging synchronous

login

!

end

# Router R3

R3# show run

Building configuration...

Current configuration : 2262 bytes

!

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

no platform punt-keepalive disable-kernel-core

!

hostname R3

!

boot-start-marker

boot-end-marker

!

no aaa new-model

!

no ip domain lookup

!

login on-success log

!

subscriber templating

!

vtp domain TSHOOT

vtp mode off

!

ipv6 unicast-routing

multilink bundle-name authenticated

!

spanning-tree extend system-id

!

redundancy

mode none

!

interface Loopback0

ip address 192.168.3.1 255.255.255.0

ipv6 address FE80::3:2 link-local

ipv6 address 2001:DB8:ACAD:3::1/64

!

interface Loopback1

ip address 172.16.3.1 255.255.255.0

ipv6 address FE80::3:3 link-local

ipv6 address 2001:DB8:ACAD:1723::1/64

ipv6 eigrp 68

!

interface Tunnel0

bandwidth 4000

ip address 100.100.100.2 255.255.255.252

ip mtu 1400

tunnel source Loopback0

tunnel destination 192.168.1.1

!

interface Tunnel1

bandwidth 4000

no ip address

ipv6 address 2001:DB8:FFFF::2/64

ipv6 eigrp 68

tunnel source Loopback0

tunnel mode gre ipv6

tunnel destination 2001:DB8:ACAD:1::1

tunnel path-mtu-discovery

!

interface GigabitEthernet0/0/0

ip address 10.2.3.3 255.255.255.0

negotiation auto

ipv6 address FE80::3:1 link-local

ipv6 address 2001:DB8:ACAD:23::3/64

!

interface GigabitEthernet0/0/1

no ip address

negotiation auto

!

interface Serial0/1/0

no ip address

!

interface Serial0/1/1

no ip address

!

router eigrp 68

network 100.100.100.0 0.0.0.3

network 172.16.3.0 0.0.0.255

eigrp router-id 3.3.3.3

!

ip forward-protocol nd

no ip http server

ip http secure-server

ip route 0.0.0.0 0.0.0.0 GigabitEthernet0/0/0 10.2.3.2

!

ipv6 route ::/0 2001:DB8:ACAD:23::2

ipv6 router eigrp 68

eigrp router-id 3.3.3.3

!

control-plane

!

banner motd ^C R3, Implement a GRE Tunnel ^C

!

line con 0

exec-timeout 0 0

logging synchronous

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

exec-timeout 0 0

privilege level 15

password cisco123

logging synchronous

login

!

end