Part II

An NT example: li wb_maf_full_replays

Problem: the SPEC benchmark "li" encounters many 
“wb_maf_full_replays" on the pca56.  
It has already been measured on the pca56; 
for comparison, we want to measure it on an ev56.

Note the different cache structures of the systems:

|  | pca56 | ev56 |
| --- | --- | --- |
| L1 | 16+8KB I+D on chip | 8+8KB I+D on chip |
| L2 | 1mb off chip | 96KB on chip |
| L3 | none | 8mb off chip |

Here's a sample portion of the pca56 output:

Cycle=cycles

PDry=pipe_dry

MfRe=wb_maf_full_replays

IMis=icache_miss

DMi=dcache_miss

Ld=loads

LMe=loads_merged

BWri=bcache_write

BWHi=bcache_write_hit

xlsave:

Address     Instruction         Cycle  PDry MfRe IMis DMi   Ld LMe BWri BWHi

00406480     xor sp,zero,sp      4024   520    5    5        3      411  950

00406484     ldq t2,0x18(sp)     7353   936    6    2 195 8325   7  662 2337

00406488     stl v0,0(s0)        8417  2967 2534        1  486      723 2811

0040648C     stl t1,0(v0)       14511  6781 5173    5   1  712   1 1216 5079

Note the large number of wb_maf_full_replays on the 3rd and 4th instructions above.

Install Perl

On the ev56, the first order of business is to install the NT resource kit,

to get perl:

- Insert the CD

- Click on "Install Resource Kit"

- Take the defaults

E:\li> dir e:\ntreskit\perl\perl.exe

Volume in drive E is nt4sp2vc5rc12

Volume Serial Number is 9457-96EB

Directory of e:\ntreskit\perl

03/01/96  12:00a               124,928 PERL.EXE

1 File(s)        124,928 bytes

1,196,438,528 bytes free

E:\li> perl -e "print 123456;"

The name specified is not recognized as an

internal or external command, operable program or batch file.

New windows have the path automatically included, but windows

that existed before the kit was installed do not.  Add to path.

E:\li> set path=%path%;e:\ntreskit\perl

E:\li> perl -e "print 123456;"

123456

E:\li>

Finding the Resource Kit

The NT Resource Kit used here was from the book

"Microsoft Windows NT Workstation Resource Kit"

Microsoft Press

ISBN 1-57231-343-9

$69.95 at Barnes & Noble

$47 when ordered via "Stream" part # 276206

VTX PCSOFTWARE

choice 6 US Orders

<return> 13 times to save instructions

Save it, fill it out, email it.

------------------------------------------------------------------

|STREAM PART # |            DESCRIPTION                   |QUANTITY|

------------------------------------------------------------------

276206         MS Win NT Resource Kit V4.0 Workstation        4

Other useful utilities on the NT resource kit include tools such as "sleep", "kill", "ls", "wc", “vi”, and "timethis":

E:\li> timethis sleep 2

The name specified is not recognized as an

internal or external command, operable program or batch file.

E:\li> set path=%path%;e:\ntreskit

E:\li> timethis sleep 1

TimeThis :  Command Line :  sleep 1

TimeThis :    Start Time :  Mon Jul 07 10:26:04 1997

TimeThis :  Command Line :  sleep 1

TimeThis :    Start Time :  Mon Jul 07 10:26:04 1997

TimeThis :      End Time :  Mon Jul 07 10:26:05 1997

TimeThis :  Elapsed Time :  00:00:01.086

Populate a directory

For the benchmark to be tested, start with an empty directory

E:\> cd li

E:\li> dir

Volume in drive E is nt4sp2vc5rc12

Volume Serial Number is 9457-96EB

Directory of E:\li

07/07/97  09:44a        <DIR>          .

07/07/97  09:44a        <DIR>          ..

2 File(s)              0 bytes

1,236,467,200 bytes free

Add the benchmark, its input files (*.lsp), and a batch control file

E:\li> dir

Volume in drive E is nt4sp2vc5rc12

Volume Serial Number is 9457-96EB

Directory of E:\li

07/07/97  10:23a        <DIR>          .

07/07/97  10:23a        <DIR>          ..

07/07/97  10:22a                 1,919 au.lsp

07/07/97  10:22a                13,498 boyer.lsp

07/07/97  10:22a                 4,060 browse.lsp

07/07/97  10:22a                   573 ctak.lsp

07/07/97  10:22a                 2,289 dderiv.lsp

07/07/97  10:22a                 1,209 deriv.lsp

07/07/97  10:22a                 1,411 destru0.lsp

07/07/97  10:22a                 1,411 destru1.lsp

07/07/97  10:22a                 1,411 destru2.lsp

07/07/97  10:22a                 1,563 destrum0.lsp

07/07/97  10:22a                 1,563 destrum1.lsp

07/07/97  10:22a                 1,563 destrum2.lsp

07/07/97  10:22a                 1,279 div2.lsp

07/07/97  10:25a                   158 do_li.bat

07/07/97  10:23a               231,936 li.exe

07/07/97  10:22a                 5,192 puzzle0.lsp

07/07/97  10:22a                 5,192 puzzle1.lsp

07/07/97  10:22a                   623 tak0.lsp

07/07/97  10:22a                   623 tak1.lsp

07/07/97  10:22a                   623 tak2.lsp

07/07/97  10:22a                20,808 takr.lsp

07/07/97  10:22a                 2,072 triang.lsp

07/07/97  10:22a                    29 xit.lsp

25 File(s)        301,005 bytes

1,196,128,256 bytes free

Build it the right way…

We are using an existing executable, and not rebuilding it.  But it is important to notice how it was built:

/link /debug /debugtype:coff

which is essential in order to be able to pick up addresses from the image.  Support for other image types is a highly likely feature for later this summer, but in the meantime remember to use coff.

E:\li>

E:\li> type do_li.bat

timethis ".\li.exe *.lsp 1>tmp.out 2>tmp.err" > time.this

echo off

echo           d

echo            o

echo             n

echo              e

echo on

The funny printing of "done" is so I can see it from 6 feet away.

Try it, to get a base time:

E:\li> do_li

E:\li>timethis ".\li.exe *.lsp 1>tmp.out 2>tmp.err"  1>time.this

E:\li>echo off

d

o

n

e

E:\li> move time.this no_iprobe.times

1 file(s) moved.

E:\li> type no_iprobe.times

TimeThis :  Command Line :  .\li.exe *.lsp 1>tmp.out 2>tmp.err

TimeThis :    Start Time :  Mon Jul 07 10:26:45 1997

TimeThis :  Command Line :  .\li.exe *.lsp 1>tmp.out 2>tmp.err

TimeThis :    Start Time :  Mon Jul 07 10:26:45 1997

TimeThis :      End Time :  Mon Jul 07 10:29:21 1997

TimeThis :  Elapsed Time :  00:02:36.094

Get IPROBE

Install IPROBE (using the newest experimental version, which has the "-command" feature):

E:\li> cd \

E:\> mkdir iprobe_newest_7jul

E:\> cd iprobe_newest_7jul

E:\iprobe_newest_7jul>ftp

ftp> op 16.31.144.83

Connected to 16.31.144.83.

220 perf.zko.dec.com FTP server (Digital UNIX Version 5.60) ready.

User (16.31.144.83:(none)): anonymous

331 Guest login ok, send ident as password.

Password:

ftp> cd pub

ftp> dir

total 3

drwxr-xr-x   2 9246     512          512 Jul  7 09:37 IprobeKits

drwxr-xr-x   2 9139     512          512 Apr 11 07:12 gaertner

drwxr-xr-x   2 6562     15           512 Jun 17 05:45 henning

ftp> cd IprobeKits

ftp> dir

total 7111

-rw-r--r--   1 9246  512      57856 Apr 10 14:49 Api.doc

-rw-r--r--   1 9246  512      42242 Apr  3 09:03 IprNew.mod

-rw-r--r--   1 9246  512     272356 Apr  3 08:59 Iprobe.ps

-rw-r--r--   1 9246  512     580608 Apr  3 08:59 Iprobe020.a

-rw-r--r--   1 9246  512      96768 Apr  3 09:03 Iprobe020ProgrammingKit.bck

-rw-r--r--   1 9246  512    1075614 Apr  3 08:59 Iprobe021Osf.tar.Z

-rw-r--r--   1 9246  512     692920 Apr  3 08:59 Iprobe0221Unix40.tar.Z

-rw-r--r--   1 9246  512     548352 Apr  3 09:01 IprobeVms021.a

-rw-r--r--   1 9246  512     677376 May  5 13:02 IprobeVms022.a

-rw-r--r--   1 9246  512     735003 Apr  3 09:01 Nt35IprobeT21.zip

-rw-r--r--   1 9246  512     304899 Apr  3 09:01 Nt35IprobeT21Update.zip

-rw-r--r--   1 9246  512     496452 Apr  3 09:02 Nt40IprobeT22Ev4.zip

-rw-r--r--   1 9246  512     505099 Jun 12 12:37 Nt40IprobeT23Ev5.zip

-rw-r--r--   1 9246  512     346312 Jun 10 06:42 TurboLaserBusMonitorUnix.tar.Z

-rw-r--r--   1 9246  512     185344 Apr  3 09:02 UnzipAxp.exe

-rw-r--r--   1 9246  512       4629 Jun 10 07:08 WhatsHere.txt

-rw-r--r--   1 9246  512     506252 Jul  7 09:37 nt40_iprobet2_3_ev5_experimental.zip

ftp> bin

200 Type set to I.

ftp> get UnzipAxp.exe

ftp> get  nt40_iprobet2_3_ev5_experimental.zip

506252 bytes received in 0.69 seconds (735.83 Kbytes/sec)

ftp> bye

221 Goodbye.

Install it

E:\iprobe_newest_7jul>dir

Directory of E:\iprobe_newest_7jul

07/07/97  10:46a        <DIR>          .

07/07/97  10:46a        <DIR>          ..

07/07/97  10:46a               506,252 nt40_iprobet2_3_ev5_experimental.zip

07/07/97  10:45a               185,344 UnzipAxp.exe

4 File(s)        691,596 bytes

1,195,430,400 bytes free

E:\iprobe_newest_7jul> unzipaxp *zip

Archive:  nt40_iprobet2_3_ev5_experimental.zip

inflating: install.bat

inflating: ipreduce.exe

inflating: iprkrnl.ini

inflating: iprkrnl.sys

inflating: iprobe.exe

inflating: iprobe_ps.exe

inflating: iprshr.dll

inflating: iprshr.exp

inflating: iprshr.lib

inflating: PSAPI.DLL

inflating: read1st.txt

inflating: regini.exe

inflating: rep.exe

E:\iprobe_newest_7jul> install

Copying Iprobe device driver (IprKrnl.sys) to E:\WINNT\system32\drivers ...

1 file(s) copied.

Copying Iprobe API library (IprShr.dll) to E:\WINNT\system32 ...

1 file(s) copied.

Copying Iprobe command and control user interface (Iprobe) to E:\WINNT\system32...

1 file(s) copied.

Copying Iprobe data reduction program (Ipreduce) to E:\WINNT\system32 ...

1 file(s) copied.

Copying Iprobe read entry points program (Rep) to E:\WINNT\system32 ...

1 file(s) copied.

Copying Iprobe show running processes program (Iprobe_PS) to E:\WINNT\system32 ...

1 file(s) copied.

Copying support libraries to E:\WINNT\system32 ...

1 file(s) copied.

Installing IprKrnl service in the registry ...

**********************************************************************

*                                                                    *

*                   Installation complete.                           *

*                                                                    *

* You need to reboot the system for the installation to take effect. *

*                                                                    *

**********************************************************************

E:\iprobe_newest_7jul>

I think the message above is in error, that ever since the IPROBE driver was made loadable you don't really need to reboot.  But just to be obedient, we'll reboot, and try iprobe.

Test That IPROBE was Installed

E:\li>iprobe -h

Unable to load library "E:\WINNT\System32\iprshr.dll"

Error = 45a

We forgot to wake up the driver.

- Double click "My Computer"

- Double click "Control Panel"

- Double click "Devices"

- Scroll to "IprKrnl" and click "Start"

- Close the dialog boxes

Test again

E:\li>iprobe -help

...

Events defined on the current system -- select up to one event from each column:

issues                  single_issue_cycles     long_stalls

cycles                  dual_issue_cycles       branch_mispr

triple_issue_cycles     pc_mispr

quad_issue_cycles       icache_miss

split_issue_cycles      dcache_miss

pipe_dry                dtb_miss

pipe_frozen             loads_merged

replay_trap             ldu_replays

branches                cycles

cond_branches           itb_miss

jsr_ret                 wb_maf_full_replays

integer_ops             external

float_ops               mem_barrier_cycles

loads                   load_locked

stores                  scache_write

icache_access           scache_miss

dcache_access           scache_read_miss

scache_access           scache_write_miss

scache_read             scache_sh_write

scache_write            bcache_miss

scache_victim           sys_inv

bcache_hit              sys_read_req

bcache_victim

sys_req

Note that the events defined here are NOT the same as the events on the pca56.  So we'll measure a slightly different set of events.  Here's a batch file to do a whole bunch of measurements:

E:\li>type do_a_bunch.bat

iprobe -method sample -command "do_li.bat" -output cyc_dry_maf.dat cycles

pipe_dry wb_maf_full_replays

move time.this cyc_dry_maf.times

iprobe -method sample -command "do_li.bat" -output icache_miss.dat icache_miss

move time.this icache_miss.times

iprobe -method sample -command "do_li.bat" -output dcache_miss.dat

loads dcache_miss

move time.this dcache_miss.times

iprobe -method sample -command "do_li.bat" -output loads.dat loads loads_merged

move time.this loads.times

iprobe -method sample -command "do_li.bat" -output bcache.dat bcache_hit

bcache_miss

move time.this bcache.times

iprobe -method sample -command "do_li.bat" -output scache.dat scache_write

scache_write_miss

move time.this scache.times

Do the measurements

E:\li>do_a_bunch

E:\li>iprobe -method sample -command "do_li.bat" -output cyc_dry_maf.dat cycles

pipe_dry wb_maf_full_replays

Node name            : PRF07

OS                   : Microsoft Windows NT Version 4.0

CPU count            : 1

Model                : DEC-00Alcor

Page count           : 81916

Pagelength           : 8192

Counter count        : 3

cycles               : Low frequency skip 0 interrupts between samples

pipe_dry             : Low frequency skip 0 interrupts between samples

wb_maf_full_replays  : Low frequency skip 0 interrupts between samples

Current time         : Mon Jul 07 12:16:22 1997

Start time:          : immediate

Duration             : 0 (until user interrupts)

Interval             : 1

Method               : sample

Measured Modes       : all modes

Measured Data        : ps pc pid ctr

Output file          : cyc_dry_maf.dat

Buffer_count         : 3

Buffer_size          : 8192

Start of sampling

Started PID 0x0000006f via command line

E:\li>timethis ".\li.exe *.lsp 1>tmp.out 2>tmp.err"  1>time.this

E:\li>echo off

d

o

n

e

End of sampling

Buffers written : 3092

Partial buffers written: 1

E:\li>move time.this cyc_dry_maf.times

...

Confirm file sizes and run times

E:\li> dir *.dat

Volume in drive E is nt4sp2vc5rc12

Volume Serial Number is 9457-96EB

Directory of E:\li

07/07/97  12:29p                73,844 bcache.dat

07/07/97  12:19p            25,329,780 cyc_dry_maf.dat

07/07/97  12:24p             5,242,996 dcache_miss.dat

07/07/97  12:21p             1,024,116 icache_miss.dat

07/07/97  12:26p             4,218,996 loads.dat

07/07/97  12:32p             1,417,332 scache.dat

6 File(s)     37,307,064 bytes

1,156,445,184 bytes free

E:\li> findstr laps *times

bcache.times:TimeThis :  Elapsed Time :  00:02:34.422

cyc_dry_maf.times:TimeThis :  Elapsed Time :  00:02:44.922

dcache_miss.times:TimeThis :  Elapsed Time :  00:02:38.429

icache_miss.times:TimeThis :  Elapsed Time :  00:02:35.344

loads.times:TimeThis :  Elapsed Time :  00:02:39.852

no_iprobe.times:TimeThis :  Elapsed Time :  00:02:36.094

scache.times:TimeThis :  Elapsed Time :  00:02:36.039

The times running with and without IPROBE are quite close to each other.

Pick up John's data reduction harness...

E:\li> ftp 16.31.144.83

Connected to 16.31.144.83.

220 perf.zko.dec.com FTP server (Digital UNIX Version 5.60) ready.

User (16.31.144.83:(none)): anonymous

331 Guest login ok, send ident as password.

Password:

230 Guest login ok, access restrictions apply.

ftp> cd pub

250 CWD command successful.

ftp> dir

200 PORT command successful.

150 Opening ASCII mode data connection for /bin/ls (16.31.144.16,1036).

total 3

drwxr-xr-x   2 9246     512          512 Jul  7 09:37 IprobeKits

drwxr-xr-x   2 9139     512          512 Apr 11 07:12 gaertner

drwxr-xr-x   2 6562     15           512 Jun 17 05:45 henning

226 Transfer complete.

202 bytes received in 0.27 seconds (0.74 Kbytes/sec)

ftp> cd henning

250 CWD command successful.

ftp> dir

200 PORT command successful.

150 Opening ASCII mode data connection for /bin/ls (16.31.144.16,1037).

total 24

-rwxr-xr-x   1 6562     15         23686 Jul  7 11:56 harness.pl

226 Transfer complete.

76 bytes received in 0.02 seconds (3.30 Kbytes/sec)

ftp> get harness.pl

200 PORT command successful.

150 Opening ASCII mode data connection for harness.pl (16.31.144.16,1038) (23686

bytes).

226 Transfer complete.

24422 bytes received in 0.13 seconds (195.38 Kbytes/sec)

ftp> bye

221 Goodbye.

E:\li>

Hanging rep: same workaround as Unix

Unfortunately the harness hangs when the rep command is issued:

E:\li>perl harness.pl -x li.exe -e bcache_miss -d bcache -s

Os=NT

Running rep to create addresses.resolved

rep li.exe

^C at harness.pl line 70.

As in Unix, the workaround is to rep by PID:

E:\li>start

.\li.exe *.lsp

1>tmp.out

2>tmp.err

- Press control-alt-delete and select Task Manager.

- Go to the "Processes" tab and get the pid.

E:\li>rep -pid 117

PID: 0x00000075 Base: 00400000 Size: 0x00038000 Name: li.exe

** VA range: 0x00400000:0x00438000 (000229376) Symbols: 0000757 Name: E:\li\li.exe

** VA range: 0x77f00000:0x77f96000 (000614400) Symbols: 0000990 Name: E:\WINNT\System32\ntdll.dll

** VA range: 0x77e60000:0x77ef4000 (000606208) Symbols: 0000682 Name: E:\WINNT\

system32\KERNEL32.dll

** Base VA 0x80080000 Symbols: 0001035 Name: \WINNT\System32\ntoskrnl.exe

. . .

Reduce a small event

Let's try the harness on a sample event - picking the smallest .dat file for a quick test.

Use "-s" to only produce a summary report

E:\li> perl harness.pl -x li.exe -e bcache_miss -d bcache -s

Os=NT

Generating top-level report for bcache_miss

ipreduce -input_file bcache.dat -output_file bcache_miss.rpt -event bcache_miss

-pthresh 1

No reports for counter 2 (Bcache Miss)

Reason: no samples matched with specifiedcommand line switches

Filtered sample count is zero, all reports suppressed

can't open bcache_miss.rpt at harness.pl line 265.

Apparently there were no Bcache misses.  OK, let's look at hits-

E:\li> perl harness.pl -x li.exe -e bcache_hit -d bcache -s

Os=NT

Generating top-level report for bcache_hit

ipreduce -input_file bcache.dat -output_file bcache_hit.rpt -event bcache_hit

-pthresh 1

E:\li> dir *hot*

Volume in drive E is nt4sp2vc5rc12

Volume Serial Number is 9457-96EB

Directory of E:\li

07/07/97  02:32p                   593 bcache_hit.hot_routines

1 File(s)            593 bytes

1,154,143,744 bytes free

E:\li> cat *hot*

Hot Routines for bcache_hit -pthresh 1

Events  % Routine           Image            Addr

1890 44 mark              E:\li\li.exe     405590:40580F

1180 27 xlminit           E:\li\li.exe     405AC0:405E1F

233  5 xlsave            E:\li\li.exe     4063F0:40681F

195  5 xleval            E:\li\li.exe     405E20:405EFF

142  3 cons              E:\li\li.exe     4048B0:40495F

91  2 xlabind           E:\li\li.exe     406150:4063EF

53  1 xlygetvalue       E:\li\li.exe     40E750:40E7AF

43  1 consd             E:\li\li.exe     404A00:404A9F

Try to look at routines

The summary level information looks good.  Re-enter the same command without the "-s", and the harness proceeds to disassemble the benchmark and add detailed reports by routines.

E:\li>perl harness.pl -x li.exe -e bcache_hit -d bcache

Os=NT

ipreduce -o bcache_hit_mark.rpd -d pc -event bcache_hit -input_file bcache.dat

-pc 405590:40580F

Generating disassembly of E:\li\li.exe

dumpbin/disasm E:\li\li.exe > E:\li\li.asm

Extracting mark from E:\li\li.asm

Annotating mark

ipreduce -o bcache_hit_xlminit.rpd -d pc -event bcache_hit -input_file

bcache.dat -pc 405AC0:405E1F

Extracting xlminit from E:\li\li.asm

Annotating xlminit

ipreduce -o bcache_hit_xlsave.rpd -d pc -event bcache_hit -input_file

bcache.dat -pc 4063F0:40681F

Extracting xlsave from E:\li\li.asm

Annotating xlsave

ipreduce -o bcache_hit_xleval.rpd -d pc -event bcache_hit -input_file

bcache.dat -pc 405E20:405EFF

Extracting xleval from E:\li\li.asm

Annotating xleval

ipreduce -o bcache_hit_cons.rpd -d pc -event bcache_hit -input_file

bcache.dat -pc 4048B0:40495F

Extracting cons from E:\li\li.asm

Annotating cons

ipreduce -o bcache_hit_xlabind.rpd -d pc -event bcache_hit -input_file

bcache.dat -pc 406150:4063EF

Extracting xlabind from E:\li\li.asm

Annotating xlabind

ipreduce -o bcache_hit_xlygetvalue.rpd -d pc -event bcache_hit -input_file

bcache.dat -pc 40E750:40E7AF

Extracting xlygetvalue from E:\li\li.asm

Annotating xlygetvalue

ipreduce -o bcache_hit_consd.rpd -d pc -event bcache_hit -input_file

bcache.dat -pc 404A00:404A9F

Extracting consd from E:\li\li.asm

Annotating consd

Throw away .dis files without fear

Everything seems to be in working order.  But the annotated disassemblies will now have bcache_hit as the first column, since that was the first event we reduced.  It seems preferable to have cycles be the first event, so throw away the annotated disassemblies.

E:\li>del *dis

Note: when the harness says "Extracting mumble", it creates mumble.dib, which contains the instructions for that routine WITHOUT any event annotations.  When events are added, the .dis files are created or updated.  We are not deleting the extracts, just the annotations.

E:\li>dir *.dib

Volume in drive E is nt4sp2vc5rc12

Volume Serial Number is 9457-96EB

Directory of E:\li

07/07/97  02:33p                 1,857 cons.dib

07/07/97  02:33p                 1,693 consd.dib

07/07/97  02:33p                 6,612 mark.dib

07/07/97  02:33p                 6,944 xlabind.dib

07/07/97  02:33p                 2,348 xleval.dib

07/07/97  02:33p                 2,188 xlminit.dib

07/07/97  02:33p                 2,184 xlsave.dib

07/07/97  02:33p                 1,041 xlygetvalue.dib

8 File(s)         24,867 bytes

1,152,613,888 bytes free

Let's go for the big event: cycles

E:\li>perl harness.pl -x li -e cycles -d cyc_dry_maf

Os=NT

Generating top-level report for cycles

ipreduce -input_file cyc_dry_maf.dat -output_file cycles.rpt -event cycles

-pthresh 1

Ipreduce really wants you to appreciate just how hard it’s working, and it lets you know about every furlong of memory allocated J.

It’s “on the list” to add a quiet mode.

100 buffers...

200 buffers...

300 buffers...

400 buffers...

500 buffers...

600 buffers...

700 buffers...

800 buffers...

900 buffers...

1000 buffers...

1100 buffers...

1200 buffers...

1300 buffers...

1400 buffers...

1500 buffers...

1600 buffers...

1700 buffers...

1800 buffers...

1900 buffers...

2000 buffers...

2100 buffers...

2200 buffers...

2300 buffers...

2400 buffers...

2500 buffers...

2600 buffers...

2700 buffers...

2800 buffers...

2900 buffers...

3000 buffers...

ipreduce -o cycles_mark.rpd -d pc -event cycles -input_file cyc_dry_maf.dat

-pc 405590:40580F

Annotating mark

ipreduce -o cycles_xlsave.rpd -d pc -event cycles -input_file cyc_dry_maf.dat

-pc 4063F0:40681F

Annotating xlsave

ipreduce -o cycles_xlygetvalue.rpd -d pc -event cycles -input_file

cyc_dry_maf.dat -pc 40E750:40E7AF

^C at harness.pl line 359, <HOT> line 5.

Oops.  Note that the first ipreduce command used "-pthresh 1".  That's the default, to include routines that generate at least 1% of the events; but it seems preferable to use 5% this time.  Kill the script, throw away the top level (1%) reports and start again.

Reduce all counters - from the top with a new ptrhesh

Delete all top-level reports and hot summaries.

Q. What have we forgotten to delete?

E:\li> del *rpt

E:\li> del *hot*

E:\li> perl harness.pl -x li -p 5 -e cycles -d cyc_dry_maf

Os=NT

Generating top-level report for cycles

ipreduce -input_file cyc_dry_maf.dat -output_file cycles.rpt -event cycles

-pthresh 5

ipreduce -o cycles_mark.rpd -d pc -event cycles -input_file cyc_dry_maf.dat

-pc 405590:40580F

Annotating mark

ipreduce -o cycles_xlsave.rpd -d pc -event cycles -input_file cyc_dry_maf.dat

-pc 4063F0:40681F

Annotating xlsave

ipreduce -o cycles_xlygetvalue.rpd -d pc -event cycles -input_file

cyc_dry_maf.dat -pc 40E750:40E7AF

Annotating xlygetvalue

ipreduce -o cycles_xlminit.rpd -d pc -event cycles -input_file cyc_dry_maf.dat

-pc 405AC0:405E1F

Annotating xlminit

ipreduce -o cycles_xleval.rpd -d pc -event cycles -input_file cyc_dry_maf.dat

-pc 405E20:405EFF

Annotating xleval

ipreduce -o cycles_xlxgetvalue.rpd -d pc -event cycles -input_file

cyc_dry_maf.dat -pc 40E690:40E74F

Extracting xlxgetvalue from E:\li\li.asm

Annotating xlxgetvalue

E:\li> perl harness.pl -x li -p 5 -d cyc_dry_maf -e pipe_dry

Os=NT

Generating top-level report for pipe_dry

ipreduce -input_file cyc_dry_maf.dat -output_file pipe_dry.rpt -event pipe_dry

-pthresh 5

ipreduce -o pipe_dry_mark.rpd -d pc -event pipe_dry -input_file cyc_dry_maf.dat

-pc 405590:40580F

Annotating mark

ipreduce -o pipe_dry_xlsave.rpd -d pc -event pipe_dry -input_file

cyc_dry_maf.dat -pc 4063F0:40681F

Annotating xlsave

ipreduce -o pipe_dry_xlygetvalue.rpd -d pc -event pipe_dry -input_file

cyc_dry_maf.dat -pc 40E750:40E7AF

Annotating xlygetvalue

ipreduce -o pipe_dry_xlxgetvalue.rpd -d pc -event pipe_dry -input_file

cyc_dry_maf.dat -pc 40E690:40E74F

Annotating xlxgetvalue

ipreduce -o pipe_dry_xleval.rpd -d pc -event pipe_dry -input_file

cyc_dry_maf.dat -pc 405E20:405EFF

Annotating xleval

ipreduce -o pipe_dry_xlminit.rpd -d pc -event pipe_dry -input_file

cyc_dry_maf.dat -pc 405AC0:405E1F

Annotating xlminit

E:\li> perl harness.pl -x li -p 5 -d cyc_dry_maf -e pipe_dry

Os=NT

Generating top-level report for pipe_dry

ipreduce -input_file cyc_dry_maf.dat -output_file pipe_dry.rpt -event

pipe_dry -pthresh 5

ipreduce -o pipe_dry_mark.rpd -d pc -event pipe_dry -input_file cyc_dry_maf.dat

-pc 405590:40580F

Annotating mark

ipreduce -o pipe_dry_xlsave.rpd -d pc -event pipe_dry -input_file

cyc_dry_maf.dat -pc 4063F0:40681F

Annotating xlsave

ipreduce -o pipe_dry_xlygetvalue.rpd -d pc -event pipe_dry -input_file

cyc_dry_maf.dat -pc 40E750:40E7AF

Annotating xlygetvalue

ipreduce -o pipe_dry_xlxgetvalue.rpd -d pc -event pipe_dry -input_file

cyc_dry_maf.dat -pc 40E690:40E74F

Annotating xlxgetvalue

ipreduce -o pipe_dry_xleval.rpd -d pc -event pipe_dry -input_file

cyc_dry_maf.dat -pc 405E20:405EFF

Annotating xleval

ipreduce -o pipe_dry_xlminit.rpd -d pc -event pipe_dry -input_file

cyc_dry_maf.dat -pc 405AC0:405E1F

Annotating xlminit

E:\li> E:\li>perl harness.pl -x li -p 5 -d icache_miss

Os=NT

Generating top-level report for icache_miss

ipreduce -input_file icache_miss.dat -output_file icache_miss.rpt -event

icache_miss -pthresh 5

ipreduce -o icache_miss_xlsave.rpd -d pc -event icache_miss -input_file

icache_miss.dat -pc 4063F0:40681F

Annotating xlsave

ipreduce -o icache_miss_xlxgetvalue.rpd -d pc -event icache_miss -input_file

icache_miss.dat -pc 40E690:40E74F

Annotating xlxgetvalue

ipreduce -o icache_miss_xlobgetvalue.rpd -d pc -event icache_miss -input_file

icache_miss.dat -pc 40A930:40AB6F

Extracting xlobgetvalue from E:\li\li.asm

Annotating xlobgetvalue

ipreduce -o icache_miss_xleval.rpd -d pc -event icache_miss -input_file

icache_miss.dat -pc 405E20:405EFF

Annotating xleval

ipreduce -o icache_miss_xlevarg.rpd -d pc -event icache_miss -input_file

icache_miss.dat -pc 40DEB0:40DF2F

Extracting xlevarg from E:\li\li.asm

Annotating xlevarg

ipreduce -o icache_miss_xlygetvalue.rpd -d pc -event icache_miss -input_file

icache_miss.dat -pc 40E750:40E7AF

Annotating xlygetvalue

E:\li> perl harness.pl -x li -p 5 -d dcache_miss

Os=NT

Generating top-level report for dcache_miss

ipreduce -input_file dcache_miss.dat -output_file dcache_miss.rpt -event

dcache_miss -pthresh 5

ipreduce -o dcache_miss_mark.rpd -d pc -event dcache_miss -input_file

dcache_miss.dat -pc 405590:40580F

Annotating mark

ipreduce -o dcache_miss_xlminit.rpd -d pc -event dcache_miss -input_file

dcache_miss.dat -pc 405AC0:405E1F

Annotating xlminit

ipreduce -o dcache_miss_xlsave.rpd -d pc -event dcache_miss -input_file

dcache_miss.dat -pc 4063F0:40681F

Annotating xlsave

ipreduce -o dcache_miss_xleval.rpd -d pc -event dcache_miss -input_file

dcache_miss.dat -pc 405E20:405EFF

Annotating xleval

ipreduce -o dcache_miss_xlygetvalue.rpd -d pc -event dcache_miss -input_file

dcache_miss.dat -pc 40E750:40E7AF

Annotating xlygetvalue

ipreduce -o dcache_miss_xlxgetvalue.rpd -d pc -event dcache_miss -input_file

dcache_miss.dat -pc 40E690:40E74F

Annotating xlxgetvalue

ipreduce -o dcache_miss_xlobgetvalue.rpd -d pc -event dcache_miss -input_file

dcache_miss.dat -pc 40A930:40AB6F

Annotating xlobgetvalue

E:\li> perl harness.pl -x li -p 5 -d loads

Os=NT

Generating top-level report for loads

ipreduce -input_file loads.dat -output_file loads.rpt -event loads -pthresh 5

ipreduce -o loads_xlsave.rpd -d pc -event loads -input_file loads.dat -pc

4063F0:40681F

Annotating xlsave

ipreduce -o loads_mark.rpd -d pc -event loads -input_file loads.dat -pc

405590:40580F

Annotating mark

ipreduce -o loads_xlygetvalue.rpd -d pc -event loads -input_file loads.dat

-pc 40E750:40E7AF

Annotating xlygetvalue

ipreduce -o loads_xlxgetvalue.rpd -d pc -event loads -input_file loads.dat

-pc 40E690:40E74F

Annotating xlxgetvalue

ipreduce -o loads_xleval.rpd -d pc -event loads -input_file loads.dat -pc

405E20:405EFF

Annotating xleval

E:\li> perl harness.pl -x li -p 5 -d loads -e loads_merged

Os=NT

Generating top-level report for loads_merged

ipreduce -input_file loads.dat -output_file loads_merged.rpt -event

loads_merged -pthresh 5

ipreduce -o loads_merged_xlxgetvalue.rpd -d pc -event loads_merged

-input_file loads.dat -pc 40E690:40E74F

Annotating xlxgetvalue

ipreduce -o loads_merged_xlobgetvalue.rpd -d pc -event loads_merged

-input_file loads.dat -pc 40A930:40AB6F

Annotating xlobgetvalue

ipreduce -o loads_merged_xleval.rpd -d pc -event loads_merged -input_file

loads.dat -pc 405E20:405EFF

Annotating xleval

ipreduce -o loads_merged_xlsave.rpd -d pc -event loads_merged -input_file

loads.dat -pc 4063F0:40681F

Annotating xlsave

ipreduce -o loads_merged_xlevlist.rpd -d pc -event loads_merged -input_file

loads.dat -pc 406050:40611F

Extracting xlevlist from E:\li\li.asm

Annotating xlevlist

ipreduce -o loads_merged_mark.rpd -d pc -event loads_merged -input_file

loads.dat -pc 405590:40580F

Annotating mark

E:\li>perl harness.pl -x li -p 5 -d bcache -e bcache_hit

Os=NT

Generating top-level report for bcache_hit

ipreduce -input_file bcache.dat -output_file bcache_hit.rpt -event bcache_hit

-pthresh 5

ipreduce -o bcache_hit_mark.rpd -d pc -event bcache_hit -input_file

bcache.dat -pc 405590:40580F

Annotating mark

ipreduce -o bcache_hit_xlminit.rpd -d pc -event bcache_hit -input_file

bcache.dat -pc 405AC0:405E1F

Annotating xlminit

ipreduce -o bcache_hit_xlsave.rpd -d pc -event bcache_hit -input_file

bcache.dat -pc 4063F0:40681F

Annotating xlsave

E:\li> perl harness.pl -x li -p 5 -d bcache -e bcache_miss

Os=NT

Generating top-level report for bcache_miss

ipreduce -input_file bcache.dat -output_file bcache_miss.rpt -event

bcache_miss -pthresh 5

No reports for counter 2 (Bcache Miss)

Reason: no samples matched with specifiedcommand line switches

Filtered sample count is zero, all reports suppressed

can't open bcache_miss.rpt at harness.pl line 265.

E:\li> perl harness.pl -x li -p 5 -d scache -e scache_write

Os=NT

Generating top-level report for scache_write

ipreduce -input_file scache.dat -output_file scache_write.rpt -event

scache_write -pthresh 5

ipreduce -o scache_write_xlsave.rpd -d pc -event scache_write -input_file

scache.dat -pc 4063F0:40681F

Annotating xlsave

ipreduce -o scache_write_mark.rpd -d pc -event scache_write -input_file

scache.dat -pc 405590:40580F

Annotating mark

ipreduce -o scache_write_xleval.rpd -d pc -event scache_write -input_file

scache.dat -pc 405E20:405EFF

Annotating xleval

ipreduce -o scache_write_xlminit.rpd -d pc -event scache_write -input_file

scache.dat -pc 405AC0:405E1F

Annotating xlminit

E:\li> perl harness.pl -x li -p 5 -d scache -e scache_write_miss

Os=NT

Generating top-level report for scache_write_miss

ipreduce -input_file scache.dat -output_file scache_write_miss.rpt -event

scache_write_miss -pthresh 5

ipreduce -o scache_write_miss_xlminit.rpd -d pc -event scache_write_miss

-input_file scache.dat -pc 405AC0:405E1F

Annotating xlminit

ipreduce -o scache_write_miss_xlsave.rpd -d pc -event scache_write_miss

-input_file scache.dat -pc 4063F0:40681F

Annotating xlsave

ipreduce -o scache_write_miss_xlevlist.rpd -d pc -event scache_write_miss

-input_file scache.dat -pc 406050:40611F

Annotating xlevlist

ipreduce -o scache_write_miss_xlabind.rpd -d pc -event scache_write_miss

-input_file scache.dat -pc 406150:4063EF

Annotating xlabind

ipreduce -o scache_write_miss_xleval.rpd -d pc -event scache_write_miss

-input_file scache.dat -pc 405E20:405EFF

Annotating xleval

ipreduce -o scache_write_miss_cons.rpd -d pc -event scache_write_miss

-input_file scache.dat -pc 4048B0:40495F

Annotating cons

ipreduce -o scache_write_miss_xlbind.rpd -d pc -event scache_write_miss

-input_file scache.dat -pc 40E5E0:40E62F

Extracting xlbind from E:\li\li.asm

Annotating xlbind

Files created by harness.pl

E:\li>ls *asm

E:\li\*asm

li.asm

1434624 (1434374) bytes in 1 files

E:\li> ls *rpt

E:\li\*rpt

bcache_hit.rpt           loads.rpt                scache_write_miss.rpt

cycles.rpt               loads_merged.rpt         wb_maf_full_replays.rpt

dcache_miss.rpt          pipe_dry.rpt

icache_miss.rpt          scache_write.rpt

29184 (26539) bytes in 10 files

E:\li> ls *hot*

E:\li\*hot*

bcache_hit.hot_routines           loads_merged.hot_routines

cycles.hot_routines               pipe_dry.hot_routines

dcache_miss.hot_routines          scache_write.hot_routines

icache_miss.hot_routines          scache_write_miss.hot_routines

loads.hot_routines                wb_maf_full_replays.hot_routines

6144 (4416) bytes in 10 files

E:\li> ls *xlsave*

E:\li\*xlsave*

bcache_hit_xlsave.rpd           pipe_dry_xlsave.rpd

cycles_xlsave.rpd               scache_write_miss_xlsave.rpd

dcache_miss_xlsave.rpd          scache_write_xlsave.rpd

icache_miss_xlsave.rpd          wb_maf_full_replays_xlsave.rpd

loads_merged_xlsave.rpd         xlsave.dib

loads_xlsave.rpd                xlsave.dis

62976 (60038) bytes in 12 files

Harness.pl creates:

- address.res - if rep works from the harness, otherwise create by pid

- <exe>.asm - disassembly of entire program

- <event>.rpt - top level report, from ipreduce

- <event>.hot_routines - summary of above

- <event>_<routine>.rpd - detail report about a specific routine

- <routine>.dib - disassembly without any event annotations

- <routine>.dis - disassembly with annotations

Hot Routines

Done.  Let's have a look at the hot routine for the various events (using the NT-resource-kit supplied utility "cat"):

E:\li>cat *hot*

Hot Routines for bcache_hit -pthresh 5

Events  % Routine           Image            Addr

1890 44 mark              E:\li\li.exe     405590:40580F

1180 27 xlminit           E:\li\li.exe     405AC0:405E1F

233  5 xlsave            E:\li\li.exe     4063F0:40681F

Hot Routines for cycles -pthresh 5

Events  % Routine           Image            Addr

289813 23 mark              E:\li\li.exe     405590:40580F

213822 17 xlsave            E:\li\li.exe     4063F0:40681F

164149 13 xlygetvalue       E:\li\li.exe     40E750:40E7AF

115423  9 xlminit           E:\li\li.exe     405AC0:405E1F

89642  7 xleval            E:\li\li.exe     405E20:405EFF

86298  7 xlxgetvalue       E:\li\li.exe     40E690:40E74F

Hot Routines for dcache_miss -pthresh 5

Events  % Routine           Image            Addr

17495 25 mark              E:\li\li.exe     405590:40580F

8218 12 xlminit           E:\li\li.exe     405AC0:405E1F

7556 11 xlsave            E:\li\li.exe     4063F0:40681F

6628 10 xleval            E:\li\li.exe     405E20:405EFF

5647  8 xlygetvalue       E:\li\li.exe     40E750:40E7AF

5112  7 xlxgetvalue       E:\li\li.exe     40E690:40E74F

4520  7 xlobgetvalue      E:\li\li.exe     40A930:40AB6F

Hot Routines for icache_miss -pthresh 5

Events  % Routine           Image            Addr

11227 18 xlsave            E:\li\li.exe     4063F0:40681F

9401 15 xlxgetvalue       E:\li\li.exe     40E690:40E74F

5497  9 xlobgetvalue      E:\li\li.exe     40A930:40AB6F

5326  9 xleval            E:\li\li.exe     405E20:405EFF

4676  7 xlevarg           E:\li\li.exe     40DEB0:40DF2F

3373  5 xlygetvalue       E:\li\li.exe     40E750:40E7AF

Hot Routines for loads -pthresh 5

Events  % Routine           Image            Addr

48619 19 xlsave            E:\li\li.exe     4063F0:40681F

48561 19 mark              E:\li\li.exe     405590:40580F

45121 18 xlygetvalue       E:\li\li.exe     40E750:40E7AF

25131 10 xlxgetvalue       E:\li\li.exe     40E690:40E74F

20119  8 xleval            E:\li\li.exe     405E20:405EFF

Hot Routines for loads_merged -pthresh 5

Events  % Routine           Image            Addr

2180 32 xlxgetvalue       E:\li\li.exe     40E690:40E74F

1929 29 xlobgetvalue      E:\li\li.exe     40A930:40AB6F

623  9 xleval            E:\li\li.exe     405E20:405EFF

396  6 xlsave            E:\li\li.exe     4063F0:40681F

364  5 xlevlist          E:\li\li.exe     406050:40611F

338  5 mark              E:\li\li.exe     405590:40580F

Hot Routines for pipe_dry -pthresh 5

Events  % Routine           Image            Addr

57989 18 mark              E:\li\li.exe     405590:40580F

56530 18 xlsave            E:\li\li.exe     4063F0:40681F

41431 13 xlygetvalue       E:\li\li.exe     40E750:40E7AF

24491  8 xlxgetvalue       E:\li\li.exe     40E690:40E74F

19962  6 xleval            E:\li\li.exe     405E20:405EFF

15910  5 xlminit           E:\li\li.exe     405AC0:405E1F

Hot Routines for scache_write -pthresh 5

Events  % Routine           Image            Addr

30027 35 xlsave            E:\li\li.exe     4063F0:40681F

11698 14 mark              E:\li\li.exe     405590:40580F

8585 10 xleval            E:\li\li.exe     405E20:405EFF

7003  8 xlminit           E:\li\li.exe     405AC0:405E1F

Hot Routines for scache_write_miss -pthresh 5

Events  % Routine           Image            Addr

605 43 xlminit           E:\li\li.exe     405AC0:405E1F

151 11 xlsave            E:\li\li.exe     4063F0:40681F

122  9 xlevlist          E:\li\li.exe     406050:40611F

116  8 xlabind           E:\li\li.exe     406150:4063EF

96  7 xleval            E:\li\li.exe     405E20:405EFF

82  6 cons              E:\li\li.exe     4048B0:40495F

70  5 xlbind            E:\li\li.exe     40E5E0:40E62F

Hot Routines for wb_maf_full_replays -pthresh 5

Events  % Routine           Image            Addr

497 46 xleval            E:\li\li.exe     405E20:405EFF

177 16 xlsave            E:\li\li.exe     4063F0:40681F

75  7 mark              E:\li\li.exe     405590:40580F

68  6 cons              E:\li\li.exe     4048B0:40495F

55  5 xlminit           E:\li\li.exe     405AC0:405E1F

Let's look at the routine of interest:

E:\li> type xlsave.dis

Cycle=cycles

Cycle=cycles

PDry=pipe_dry

MfR=wb_maf_full_replays

IMi=icache_miss

DMis=dcache_miss

Ld=loads

LMe=loads_merged

BHi=bcache_hit

SWri=scache_write

SWM=scache_write_miss

xlsave:

Address     Instruction        Cycle Cycle PDry MfR IMi DMis   Ld LMe BHi SWri SWM

004063F0     lda sp,0xFFA0(sp)  2248  2248  433      84        10       2  133

004063F4     stq a0,0x30(sp)    2039  2039  126  14             1       8   31

004063F8     stq a1,0x38(sp)    2022  2022  463  20                     8   54

004063FC     stq a2,0x40(sp)    1943  1943   49      21                 2   47

00406400     stq a3,0x48(sp)    2055  2055   69  17   8             1   1  181   1

00406404     stq a4,0x50(sp)    1900  1900   29       1    1            3  248

00406408     stq s0,0(sp)       1839  1839   24   1                     1  887

0040640C     stq s1,8(sp)       2209  2209   60  28                     6  405   2

. . .

Well there's a pain in the neck - "Cycles" are reported in both the first and second event columns because we ran the harness twice for that event (remember the ^C and switch to a 5% threshold?).  But this gives us a chance to demonstrate a feature of the harness.  Throw away the annotated disassemblies and regenerate them:

E:\li> del *.dis

E:\li> perl harness.pl -x li -p 5 -d cyc_dry_maf -e cycles

Os=NT

Annotating mark

Annotating xlsave

Annotating xlygetvalue

Annotating xlminit

Annotating xleval

Annotating xlxgetvalue

Note that ALL ipreduce calls are skipped, because the reports already exist.  The .dib files also are not re-generated; only the .dis files, which are created very quickly.

E:\li> perl harness.pl -x li -p 5 -d cyc_dry_maf -e pipe_dry

Os=NT

Annotating mark

Annotating xlsave

Annotating xlygetvalue

Annotating xlxgetvalue

Annotating xleval

Annotating xlminit

E:\li>perl harness.pl -x li -p 5 -d cyc_dry_maf -e wb_maf_full_replays

Os=NT

Annotating xleval

Annotating xlsave

Annotating mark

Annotating cons

Annotating xlminit

E:\li>perl harness.pl -x li -p 5 -d icache_miss

Os=NT

Annotating xlsave

Annotating xlxgetvalue

Annotating xlobgetvalue

Annotating xleval

Annotating xlevarg

Annotating xlygetvalue

E:\li>perl harness.pl -x li -p 5 -d dcache_miss

Os=NT

Annotating mark

Annotating xlminit

Annotating xlsave

Annotating xleval

Annotating xlygetvalue

Annotating xlxgetvalue

Annotating xlobgetvalue

E:\li>perl harness.pl -x li -p 5 -d loads

Os=NT

Annotating xlsave

Annotating mark

Annotating xlygetvalue

Annotating xlxgetvalue

Annotating xleval

E:\li>perl harness.pl -x li -p 5 -d loads -e loads_merged

Os=NT

Annotating xlxgetvalue

Annotating xlobgetvalue

Annotating xleval

Annotating xlsave

Annotating xlevlist

Annotating mark

E:\li>perl harness.pl -x li -p 5 -d bcache -e bcache_hit

Os=NT

Annotating mark

Annotating xlminit

Annotating xlsave

E:\li>perl harness.pl -x li -p 5 -d bcache -e bcache_miss

Os=NT

Generating top-level report for bcache_miss

ipreduce -input_file bcache.dat -output_file bcache_miss.rpt -event bcache_miss -pthresh 5

No reports for counter 2 (Bcache Miss)

Reason: no samples matched with specifiedcommand line switches

Filtered sample count is zero, all reports suppressed

can't open bcache_miss.rpt at harness.pl line 265.

E:\li>perl harness.pl -x li -p 5 -d scache -e scache_write

Os=NT

Annotating xlsave

Annotating mark

Annotating xleval

Annotating xlminit

E:\li>perl harness.pl -x li -p 5 -d scache -e scache_write_miss

Os=NT

Annotating xlminit

Annotating xlsave

Annotating xlevlist

Annotating xlabind

Annotating xleval

Annotating cons

Annotating xlbind

Done.  Here's the annotated ev56 xlsave disassembly:

E:\li> type xlsave.dis

Cycle=cycles            PDry=pipe_dry      MfR=wb_maf_full_replays

IMi=icache_miss         DMis=dcache_miss   Ld=loads

LMe=loads_merged        BHi=bcache_hit     SWri=scache_write

SWM=scache_write_miss

Address     Instruction          Cycle PDry MfR IMi DMis   Ld LMe BHi SWri SWM

004063F0     lda sp,0xFFA0(sp)    2248  433      84        10       2  133

004063F4     stq a0,0x30(sp)      2039  126  14             1       8   31

004063F8     stq a1,0x38(sp)      2022  463  20                     8   54

004063FC     stq a2,0x40(sp)      1943   49      21                 2   47

00406400     stq a3,0x48(sp)      2055   69  17   8             1   1  181   1

00406404     stq a4,0x50(sp)      1900   29       1    1            3  248

00406408     stq s0,0(sp)         1839   24   1                     1  887

0040640C     stq s1,8(sp)         2209   60  28                     6  405   2

00406410     stq a5,0x58(sp)      1883   28       8                 5  386

00406414     stq ra,0x10(sp)      1819   23   3                     5  705   2

00406418    ldah s0,0x43

0040641C     lda s0,0xF6C8(s0)    2083   26                         2   40

00406420     lda t0,0x30(sp)      1995   30       9    1           15   25

00406424    ldah s1,0x43

00406428     ldl a0,0(s0)         1876   17                         4 1778

0040642C     ldl v0,0x30(sp)

00406430     stl t0,0x18(sp)      2015   55  15        4    4       5    9   1

00406434     mov 8,t0

00406438     stl t0,0x1C(sp)      2088   30       3                    984   8

0040643C     lda s1,0xF790(s1)

00406440     stl a0,0x20(sp)      2538   40       7  323  150       3  204   3

00406444     beq v0,004064A8       901    7          407  169       7  238   5

00406448     ldl a0,0(s0)        14929 1676       2   15 4858   4  11 4810   1

0040644C     ldl t0,0(s1)            4    1                 4            3

00406450  cmpule a0,t0,t0         8875 3807   1      707  496       3 3350  17

00406454     beq t0,00406464

00406458    ldah a0,0x43

0040645C     lda a0,0xA3B8(a0)

00406460     bsr ra,xlabort

00406464     ldl t0,0x1C(sp)      3448 1609   1                        616   2

00406468     ldl v0,0(s0)

0040646C     ldl t1,0x30(sp)      3817 1640   1                         93   1

00406470    addl t0,8,t0          5540  855       1  826 7132   3   3 1757   2

00406474     lda v0,0xFFFC(v0)       3

00406478     stl t0,0x1C(sp)      3805   78   5             2       1   29

0040647C     nop

00406480     xor sp,zero,sp       3965   81                            254

00406484     ldq t2,0x18(sp)      7330  111   1      197 9952   8   1  820

00406488     stl v0,0(s0)         3628   40   1                     2  197

0040648C     stl t1,0(v0)         4123   91  21            39   1   2   73

00406490    addl t2,t0,t0         3440   40       1                 1   61   1

00406494     ldl t3,0x30(sp)      7237   65           15 3586   2   1   79

00406498     stl zero,0(t3)       7510  147  32   1   13    3          886   3

0040649C     ldl t0,0xFFF8(t0)    3595   36                           2299

004064A0     stl t0,0x30(sp)      7420  198   7       99 3479       1   75

004064A4     bne t0,00406448

004064A8     ldq ra,0x10(sp)      4831  568            3  529         1016

004064AC     ldq s0,0(sp)

004064B0     ldq s1,8(sp)         3423  561            3 1395   1   1 1188   1

004064B4     ldl v0,0x20(sp)

004064B8     lda sp,0x60(sp)      2024  560                           1583   2

004064BC     ret                     1

And here's the (previously generated) pca56 xlsave

Cycle=cycles          PDry=pipe_dry       MfRe=wb_maf_full_replays

IMis=icache_miss      DMi=dcache_miss     Ld=loads

LMe=loads_merged      BWri=bcache_write   BWHi=bcache_write_hit

Address     Instruction          Cycle  PDry MfRe IMis DMi   Ld LMe BWri BWHi

004063F0     lda sp,0xFFA0(sp)    3943  3565   16   92       14      166  393

004063F4     stq a0,0x30(sp)      2747   911  239    1        1       78  390

004063F8     stq a1,0x38(sp)      5149  3150 1732    1               270 1057

004063FC     stq a2,0x40(sp)      2237   324    2                     26  191

00406400     stq a3,0x48(sp)      8009  4775 3046   27        1      425 1904

00406404     stq a4,0x50(sp)      2205   607    5                     52  186

00406408     stq s0,0(sp)         1881   588    7                     76  224

0040640C     stq s1,8(sp)        17143  9729 7568   30   1    1      802 4876

00406410     stq a5,0x58(sp)      1369  1328    5        2            82  850

00406414     stq ra,0x10(sp)      5524  3239 1632                    586 1913

00406418    ldah s0,0x43

0040641C     lda s0,0xF6C8(s0)    1655  1239    6                    255  662

00406420     lda t0,0x30(sp)      2417  1304        26               233  572

00406424    ldah s1,0x43

00406428     ldl a0,0(s0)         1652   419    4    1        1       92  604

0040642C     ldl v0,0x30(sp)

00406430     stl t0,0x18(sp)     10004  4626 3965   30 247 1407   3  819 2889

00406434     mov 8,t0

00406438     stl t0,0x1C(sp)      1362   767    7    1               170 1365

0040643C     lda s1,0xF790(s1)

00406440     stl a0,0x20(sp)      6172   987   14   20 219  109      428 1319

00406444     beq v0,004064A8      4547   129   14    4 243   96      342 1386

00406448     ldl a0,0(s0)        14281  4842   27    3     3236   2  861 5190

0040644C     ldl t0,0(s1)                       1

00406450  cmpule a0,t0,t0        14648  4200   37   13 683  456   1 1381 4470

00406454     beq t0,00406464

00406458    ldah a0,0x43

0040645C     lda a0,0xA3B8(a0)

00406460     bsr ra,xlabort

00406464     ldl t0,0x1C(sp)      4256  1722    9   10        6      309  686

00406468     ldl v0,0(s0)           82    16             8    7

0040646C     ldl t1,0x30(sp)      3425  1284    6    1               331 1359

00406470    addl t0,8,t0         10876   911   28   11 738 7201  10  625 1936

00406474     lda v0,0xFFFC(v0)       9     7

00406478     stl t0,0x1C(sp)      7281  1794 1992        1 1620   1  525 3797

0040647C     nop

00406480     xor sp,zero,sp       4024   520    5    5        3      411  950

00406484     ldq t2,0x18(sp)      7353   936    6    2 195 8325   7  662 2337

00406488     stl v0,0(s0)         8417  2967 2534        1  486      723 2811

0040648C     stl t1,0(v0)        14511  6781 5173    5   1  712   1 1216 5079

00406490    addl t2,t0,t0         3505  1154    6                    402 2120

00406494     ldl t3,0x30(sp)      5448   274   10          2287   1  397 1881

00406498     stl zero,0(t3)      14369  6426 3792    6      843     1131 4915

0040649C     ldl t0,0xFFF8(t0)    3950  1363    7                    682 1021

004064A0     stl t0,0x30(sp)     18845  7422 5722    6  78 4115   5 1755 7247

004064A4     bne t0,00406448

004064A8     ldq ra,0x10(sp)      5006  1956    4    1      501      300 1376

004064AC     ldq s0,0(sp)

004064B0     ldq s1,8(sp)         3526  2213    2           563      290 1736

004064B4     ldl v0,0x20(sp)

004064B8     lda sp,0x60(sp)      1802  1343    5                    176 1181

004064BC     ret                     4     1    1

Observations - 1

Using Map Network Drive the ev56 reports are currently mounted on drive G: and the pca56 reports on drive H:.  Comparing the two, we can make the following observations and hypotheses:

- pca56 takes 60% more cycles than ev56, and the pipeline is dryer.

E:\>findstr "Cycles.*[0-9]" g:\li\cycles.rpt h:\li\cycles.rpt

g:\li\cycles.rpt:    Cycles                     1254830

h:\li\cycles.rpt:    Cycles                     1984890

E:\>findstr "Dry.*[0-9]" g:\li\pipe_dry.rpt h:\li\pipe_dry.rpt

g:\li\pipe_dry.rpt:    Pipe Dry                    314420

h:\li\pipe_dry.rpt:    Pipe Dry                    743731

The findstr regular expression looks for the summary line in the top-level  IPROBE report, searching for the event name and a count.

The pca56 system incurs about 48B more cycles than the ev56 ((1984890-1254830) cycle counter overflows * 2^16 cycles per counter event) or about 1.6x.

The pca56 spends about 28B more cycles dry than the ev56 ((743731-314420)*2^16)

The ev56 is about 25% dry (314 thousand pipe dry counter overflows / 1255 thousand cycle counter overflows) and the pca56 is 37% dry (744/1985).

Observations - 2

2. MAF replays contribute about 40% of the additional pca56 dry time.

E:\>findstr "Maf.*[0-9]" g:\li\*maf*.rpt h:\li\*maf*.rpt

g:\li\wb_maf_full_replays.rpt:    Wb Maf Full Replays           1078

h:\li\wb_maf_full_replays.rpt:    Wb Maf Full Replays         100751

MAF replays are expected to consume 7 cycles.  Each report of a pipe dry cycle counter overflow represents 2^16 events and each report of a Maf Replay represents 2^14 events:

G:\li>findstr /c:"One sample " *rpt

bcache_hit.rpt:    * One sample = 65536 events *

cycles.rpt:    * One sample = 65536 events *

dcache_miss.rpt:    * One sample = 16384 events *

icache_miss.rpt:    * One sample = 16384 events *

loads.rpt:    * One sample = 65536 events *

loads_merged.rpt:    * One sample = 16384 events *

pipe_dry.rpt:    * One sample = 65536 events *

scache_write.rpt:    * One sample = 65536 events *

scache_write_miss.rpt:    * One sample = 16384 events *

wb_maf_full_replays.rpt:    * One sample = 16384 events *

Therefore we have (743731-314420)*2^16 = 28B extra dry cycles which include 7*(100751-1078)*2^14 = 11B cycles for Maf Replays.  About 40% (11/28) of the extra dry time is due to Maf Replays.

Observations - 3

- Icache misses are higher on the pca56, even though it has a larger Icache, and the imisses account for the other 60% of the dry increase.

g:\li\icache_miss.rpt:    Icache Miss                  63000

h:\li\icache_miss.rpt:    Icache Miss                 100406

If icache misses each cost, say, 8 cycles on an ev56 and 16 cycles on this pca56, then the above would account for an additional 18B cycles on the pca56 (100406*2^14*16 - 63000*2^14*8).

A latency estimate of 8 cycles for ev56's on-chip S-cache is based on http://www.digital.com/info/DTJH09/DTJH09SC.TXT.  The pca56 estimate of 16 cycles is based on the fact that the pca56 had its bcache latency set to 8 cycles, which is assumed to be added to on-chip time of about the same amount as ev56.  Emperical evidence for 8 and 16 might be indicated by the Dependent Load measurements with the Nix memtest (gem-alpha-perf note 331.33).

This roughly accounts for the other 60% (18/28) of the dry time.

Observations - 4

- Write misses are similar between the two systems.

E:\>findstr "cache.*[0-9]" g:\li\*scache_write.rpt

h:\li\*bcache_write.rpt

g:\li\scache_write.rpt:    Scache Write                 86159

g:\li\scache_write.rpt:    Scache Write Miss             1418

h:\li\bcache_write.rpt:    Bcache Write                 70251

h:\li\bcache_write.rpt:    Bcache Write Hit            280824

Remember to always check the units for the counters you are using:

E:\>findstr /c:"One sample" g:\li\*scache*.rpt h:\li\*bcache*.rpt

g:\li\scache_write.rpt:    * One sample = 65536 events *

g:\li\scache_write_miss.rpt:    * One sample = 16384 events *

h:\li\bcache_write.rpt:    * One sample = 65536 events *

h:\li\bcache_write_hit.rpt:    * One sample = 16384 events *

Therefore we find that the ev56 writes miss the scache less than 1% of the time ((1418*16384)/(86159*65536)) and the pca56 writes hit the bcache over 99% of the time ((280824*16384)/(70251*65536)).

Observations - 5 (uh-oh)

- The data reduction harness isn't picking up enough of the routines when it creates the .dis files.  Note that ipreduce sees xlsave as the hot routine for imisses:

Begin    End                         Sample Image Total

Address  Address   Name               Count   Pct   Pct

-------  -------   ----               -----   ---   ---

004063F0-0040681F  xlsave             21809  21.8  21.7

But no instruction in the pca56 xlsave.dis is listed as getting more than 92 icache_miss events!  The problem is probably due to the fact that the harness recognizes routine boundaries in the .asm disassembly by the occurence of a label followed by a colon, and stops extracting lines at 4064bc instead of continuing on to 40681f.

004064B4: A01E0020 ldl           v0,0x20(sp)

004064B8: 23DE0060 lda           sp,0x60(sp)

004064BC: 6BFA8001 ret

evalhook:

004064C0: 23DEFFE0 lda           sp,0xFFE0(sp)

004064C4: 47FF0413 clr           a3

004064C8: B75E0000 stq           ra,0(sp)

Look for a future version of harness.pl to extract by address rather than by label.  In the meantime, hand-extracts have been posted to http://tlg-www.zko.dec.com/~henning/li, where it can be noted that the icache misses are happening in evform and evfun.

Observations - 6

- The reason for the added icache misses is probably bcache latency.  Noting differences such as the following in the postings from #5:

Cycle=cycles        IMi=icache_miss      PDry=pipe_dry     MfR=wb_maf_full_replays

Address     Instruction                Cycle  IMi  PDry  MfR

ev56  evfun:

00406720     lda sp,0xFFC0(sp)          1395  698   613

00406724     clr a3

00406728     stq s0,0(sp)                192        168

0040672C     stq s1,8(sp)                206        212

00406730     stq ra,0x10(sp)             194    3   184

00406734     stl a0,0x24(sp)             218   34   185

00406738     lda a0,0x20(sp)

0040673C     stl a1,0x28(sp)             210          2

00406740     stl a2,0x2C(sp)             228    5    26

00406744     lda a1,0x18(sp)

00406748     lda a2,0x1C(sp)             188          1

0040674C     bsr ra,xlsave

00406750     ldl t0,0x24(sp)            1413    4   835

00406754     stl v0,0x30(sp)             209        193    1

00406758     ldl s0,8(t0)                230    2   206

0040675C     beq s0,0040676C             738    2   532

00406760    ldbu t2,0(s0)                182    1    69

pca56  evfun:

00406720     lda sp,0xFFC0(sp)          4997  691  4194    9

00406724     clr a3

00406728     stq s0,0(sp)                390        210

0040672C     stq s1,8(sp)                 62    3   235   25

00406730     stq ra,0x10(sp)             367  758   350    1

00406734     stl a0,0x24(sp)             461   10    25    8

00406738     lda a0,0x20(sp)

0040673C     stl a1,0x28(sp)              25          2    2

00406740     stl a2,0x2C(sp)             516  724   256    2

00406744     lda a1,0x18(sp)

00406748     lda a2,0x1C(sp)             404         11

0040674C     bsr ra,xlsave

00406750     ldl t0,0x24(sp)            1593    2   882    2

00406754     stl v0,0x30(sp)              96        196    1

00406758     ldl s0,8(t0)                518        238    2

0040675C     beq s0,0040676C            1537    5   619    7

00406760    ldbu t2,0(s0)                434    1   146    1

00406764   cmpeq t2,3,t2                1244        215    2

Both processors start off evfun with similar icache misses charged to the first instruction.  Both processors are presumably prefetching the following instructions, but ev56 prefetches from the on-chip S-cache and pca56 prefetches from the off-chip B-cache.  It continues to incur about 700 icache miss events for two more fetch blocks, whereas ev56 has successfully prefetched them, and avoided most of the misses.