MMDIAG.EXE will load the appropriate tests for the type of MicroMaster board

being used (based on the obtained board ID) and the system environment (running

from the original CPU or from the MicroMaster "side".  If the Board ID is

found to be invalid,  All tests are loaded,  but only Board ID tests are

selected.

To start The Diagnostics:

1.) Run MMDIAG

2.) Read the help screen,  and then press any key.

The following options are now available:

========================================

A.) To run the default tests:

1.) Press the "A" Key.	 (A = ALL tests)

B.) Tests which take a long time to run default to deselected as shown

in the menu window.  You may wish to activate these deselected tests

or turn off some default selected tests.  Follow these steps:

1.) Position the green highlight bar (using the up and down cursor keys)

so that the highlight bar is over the test you wish to change the

selection of.

2.) Press the "S" key. (S = Select)  Sel appears in the menu window,

to indicate the test has been selected.  Dsel appears in the menu

window to indicate that a test has been deselected.  The "S" Key

toggles between Selected and deselected.

3.) To execute All the tests as selected Press the "A" Key.

D.) To LOOP on tests multiple times:

1.) Position the green highlight bar (using the up and down cursor keys)

so that the highlight bar is over the test you wish to loop on.

2.) Press the "L"  key.   (L = Loop).  LOOP appears in the menu window,

to indicate looping has been selected.  L toggles between looping ON,

or OFF.

3.) repeat 1 and 2 until all tests which you wish to loop on have been

selected.  (a single or multiple tests may be selected)

4.) To execute all the tests as selected Press the "A" key.

5.) To interrupt looping tests,  strike any key.  Tests will terminate

at the completion of the current "pass".

NOTE:  Since a test is to be looped on,  any tests _without_ loop attributes

on become deselected.  It will not be possible to select a test ("S"

Key) if any other test has its loop attribute enabled.

C.) To Run any single test 1 time:

1.) Position the green highlight bar (using the up and down cursor keys)

so that the highlight bar is over the test you wish to run.

2.) Press the "R" key.   (R = Run the highlighted test)

The following command line switches exist in MMDIAG.EXE:

(They are case insensitive)

/ISFAST        = Returns errorlevel 1 or 0.  1 if is running faster.  usefull

perhaps for batch file conditional branching,  based on

presance or absense of MicroMaster card.

/LOGFILE       = Sends most error/status messages to a logfile named

MMDIAG.LOG.

/LOCKON        = Attempts to lock onto intermittant errors.  Should a test

fail while this switch is used,  other tests are de-selected

and the failing test aquires a looping attribute.  The failed

test is then repeatedly tested.

/TESTPAL       = Tests U707 for rev 3 part on type A16? MicroMaster 386 boards.