# 1.  Product Description

This manual contains the information required to install and use the EMC-FDM floppy drive expansion module.

The EMC-FDM is a floppy disk unit and also includes a standard male 9-pin serial port and a standard female 25-pin parallel port.

Due to the size of the floppy drive unit, the EMC-FDM uses a double-wide EXM form factor. The EMC-FDM is the same size as the EMC-PS50 unit, and is designed to mount on the right side (opposite the EMC-PS50) of an EMC chassis.  One such chassis available from RadiSys is the EMC-CH6D.

The EMC-FDM is fully software configurable via the setup screen allowing the user to enable or disable the card and configure the I/O address and interrupt level for each port.

## Specifications

The following table defines the environmental and electrical specifications of the 
EMC-FDM.

| Characteristic |  | Value |
| --- | --- | --- |
| Environmental |  |  |
| Temperature | operating | 4° to 45° C (* see below) |
|  | storage | -40° to 65° C |
| Humidity | operating | 20% - 80% (non-condensing) |
|  | storage | 5% - 95% (non-condensing) |
| Vibration | operating | .015" P-P 0.6g (max) 5-2000 Hz |
|  | storage | .030" P-P 5g (max) 5-2000 Hz |
| Shock | operating | 5g 11 msec duration |
|  | storage | 50g 11 msec duration |
| Electrical |  |  |
|  | maximum | +5V @ 1.7 A (with floppy drive running) |
|  | typical | +5V @ 1.0 A (floppy drive idle) |

Table 1. EMC-FDM Environmental and Electrical Specifications

* Upper temperature limit degrades 2° C per 1000 ft. elevation.  Maximum  
   elevation 10,000 ft.