# 2.  Installation

Before installing the EMC-FDM, unpack and inspect it for shipping  damage.

	do not remove the module from its anti-static bag unless you are in a static-free environment. the 
emc-fdm, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

	ensure that the installation process as described herein is also performed in a static-free environment.

## Insertion in an EMC Chassis

Insertion of the EMC-FDM into an EMC chassis is straightforward.  Locate the double-wide open card guides at the far right of the carrier (opposite the power supply) and insert the EMC-FDM into the backplane connector.  Firmly press the EMC-FDM front panel to ensure that the module is properly seated in the backplane, then secure it with the supplied screws and nylon washers.

	make sure that power to your system is off.  the            emc-fdm is not designed to be inserted or removed from a live system.

	when inserting the emc-fdm, avoid touching the circuit board, and make sure the environment is static-free.

	insert it with adequate continuous force rather than tapping or hammering on it.

## Configuring the BIOS Setup

The EMC-FDM is electrically an EXM device, and is configured like any other EXM module.

The configuration data in the EPC to which the EMC-FDM is connected needs to be modified to recognize and enable the module.  After installing the module, turn on the system and boot it up. From the command line, invoke the BIOS setup function by pressing the CTRL-ALT-ESC keys simultaneously.

### Enabling the EMC-FDM module

Once in the setup function, a menu displays specifying which function keys are available for further configuration.  Press the F2 function key to invoke the EXM Setup Screen.  Information similar to the screen shown in Figure 1 below displays:

ID OB1 OB2       
 Slot 0    FF  00  00
      1    EC  01  00
      2    7C  E1  00
      3    DC  F5  00
      4    F5  05  00
      5    FA  25  00

Figure 1.  EXM Setup Screen

This screen displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory. The EPC uses this information at power-up to recognize and configure each installed EXM.  The displayed data shows SLOT, ID, OB1 and OB2 information. These are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See Figure 2 below to determine which slot the EXM occupies.

EXM Slots

Figure 2. EXM Slot Numbering

The EMC-FDM occupies the two open slots on the far right of the chassis.

ID 	is a hard-wired ID value.  Each EXM type has a unique ID value.

OB1/OB2 	are two bytes of option information.

Note that all slots are listed even if the system configuration does not have all the  possible EXM slots.  All slots not occupied by an EXM module should show an ID of FF and OB1/OB2 of   00  00  indicating that no EXM is present.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EMC-FDM should be set to 7C.

OB1

OB1 is defined as follows:

| Parallel Port I/O Enable (bit 7) | Parallel Port Enable (bit 6) | Printer Port Select (bit 5) | COM Port Interrupt (bits 4,3) | COM Port Base Address (bits 2,1) | Card Enable (bit 0) |
| --- | --- | --- | --- | --- | --- |
| 1=Enableoutput 0=Enable input | 1=Enable 0=Disable | 1=378 IRQ7 0=278 IRQ5 | 00=Disabled 01=IRQ3 10=IRQ4 11=IRQ9(2) | 00=Disabled 01=2F8h 10=3E8h 11=2E8h | 1=Enable card 0=Disable card, IRQs |

Table 2. OB1 Register.

Parallel port In/Out - Normally set to 1 to enable output to the parallel port. Set to 0 to input data from a parallel device such as a scanner. Note that the DIR bit, which is bit 5 of register 27Ah or 37Ah, must be set to 1 via application software before the parallel port accepts input. Bit 7 has no effect when bit 6 = 0.

Parallel port enable - If 0, disables the parallel port registers and interrupt. A value of 1 enables the port.

Printer port select - If set to 1, selects a base address for the parallel port of 378h (LPT1:) and interrupt IRQ7.  If clear, selects 278h and IRQ5.

COM port interrupt select -  Encoded as follows:

00	Disabled. COM port uses no interrupt but can be written to.

01	Use IRQ3 (this is normally COM2)

10	Use IRQ4 (this is normally COM1)

11	Use IRQ9

COM port base address and enable -  Encoded as follows:

00	Disabled. COM port will not respond.

01	Base address 2F8. This is normally COM2.

10	Base address 3E8. This is normally COM3.

11	Base address 2E8. This is normally COM4.

Card enable -  CDEN must be set to 1 to enable the EMC-FDM.  Note that setting CDEN to 0 disables all access to the registers, as well as disabling all interrupts.

OB2

OB2 is defined as follows:

| Bits 7-2 | Reserved (bit 1) | Floppy drive enable (bit 0) |
| --- | --- | --- |
| 1 1 1 1 1 1 | 1= default | 1 = Enable floppy drive, DRQ and IRQ 0 = Disable floppy drive |

Table 3. OB2 Register

Bit 1 - Reserved for future use.

Floppy Drive Enable - Use to enable or disable the floppy drive. Please note that only one floppy drive controller per system can be enabled at any one time.  If the EMC system requires an EXM-16 or an EXM-MX running with the EXM-FDM, disable all but one floppy drive.

Floppy Drive Setup

Refer to the pertinent EPC manual for a detailed description of how to change the configuration information stored in battery-backed RAM. In most cases, invoke the BIOS setup function by using CTRL-ALT-ESC from the DOS command prompt. Be sure to define drive A: as a  1.44M 3.5 inch drive.

Port Conflicts

Avoid interrupt and address conflicts due to multiple serial or parallel ports with the same address or interrupt. Be sure to either disable unused serial or parallel ports, or set them to a value that does not conflict with other ports in the system.

Note that in general attaching a mouse to a serial port only works on COM1 or COM2.

NOTES