# 3. Operation

The EMC-FDM provides a 3.5" floppy disk drive and controller and both a standard 25-pin parallel port and a 9-pin DTE serial port.  Both ports are implemented using a single VLSI VL16C452 chip.  The VL16C552 provides two serial channels, channel 0 and channel 1.  Channel 0 is not used.

This chip does not support HDLC and SDLC protocols.  For detailed programming information, see the appropriate VLSI Technology data manual.

The floppy disk is controlled by a 37C65C floppy disk controller, identical to other RadiSys products.

The floppy drive supports both 1.44 Mbyte and 720 Kbyte 3.5" diskettes. The drive select is set as the A: drive.

### Connectors

The parallel printer port is a standard female DB-25 with the pin-out defined as follows:

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | Strobe | 14 | Auto line feed |
| 2 | D0 | 15 | Error |
| 3 | D1 | 16 | Initialize printer |
| 4 | D2 | 17 | Select in |
| 5 | D3 | 18 | Signal ground |
| 6 | D4 | 19 | Signal ground |
| 7 | D5 | 20 | Signal ground |
| 8 | D6 | 21 | Signal ground |
| 9 | D7 | 22 | Signal ground |
| 10 | Acknowledge | 23 | Signal ground |
| 11 | Busy | 24 | Signal ground |
| 12 | Paper end | 25 | Signal ground |
| 13 | Select |  |  |

Table 4. DB-25 Parallel Port Pin-out.

The serial port is a standard male DB-9 connector.  The pin-out of the serial port is shown below.

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | CD | 6 | DSR |
| 2 | RxD | 7 | RTS |
| 3 | TxD | 8 | CTS |
| 4 | DTR | 9 | RI |
| 5 | Gnd |  |  |

Table 5. DB-9 Serial Port Pin-out.

NOTES