# 1.  Product Description

The EPC-21 and EPC-22 are PC/AT compatible CPU modules in a very small form factor (3" x 5.9") based on the Intel386 SL chip set.  These EPCs have been designed to meet stringent safety and low EMI standards (UL-1950).  All front panel accessible ports have filter networks for reduced EMI and increased ESD protection.

Throughout this manual, the term EPC is used to designate both the EPC-21 and the EPC-22.  Where necessary, differences between the EPC-21 and EPC-22 are specified separately.

Each processor module contains the following:

·	The EPC-21 contains a 16 MHz Intel386 SL processor

·	The EPC-22 contains a 25 MHz Intel386 SL processor and an 80387SX math coprocessor

·	The EPC-22 contains 64 Kbytes of SRAM cache

·	Up to 20 MBytes of DRAMXE "DRAM"§ memoryXE "memory"§

·	Keyboard interface

·	1 standard 9-pin DTE serial portXE "serial ports"§ (COM1XE "Com1"§XE "Com2"§)

·	1 RJ-45 serial port (COM2)

·	Time-of-day clock with user replaceable battery

·	Award BIOS

·	Watchdog Timer

·	EXM expansion interface (electrically similar to the 16-bit PC/AT ISA bus)

## Specifications

| EnvironmentalXE "Environmentals"§ |  |  |  |  |
| --- | --- | --- | --- | --- |
| Temperature | operating |  | 0° to 60° C (* see below) |  |
|  | storage |  | -40° to 125° C |  |
| Humidity | operating |  | 0 - 95% (non-condensing) |  |
|  | storage |  | 0 - 95% (non-condensing) |  |
| Vibration | operating |  | .015"PP 2.5g (max) 5-2000 Hz |  |
|  | storage |  | .030"PP 5g (max) 5-2000 Hz |  |
| Shock | operating |  | 30g 11 msec duration |  |
|  | storage |  | 50g 11 msec duration |  |
| Electrical |  |  |  |  |
| EPC-21 | maximum |  | +5V @ 850 mA |  |
|  | typical |  | +5V @ 650 mA |  |
| EPC-22 | maximum |  | +5V @ 1.25 A |  |
|  | typical |  | +5V @ 1.1 A |  |

* Upper temperature limit degrades 2° C per 1000 ft. elevation.  Maximum  
   elevation 10,000 ft.

See Appendix A for mechanical dimensions.