2.  Configuring the

BIOS SetupXE "BIOS setup"§

## Power-On Screen Display

Whenever a hardware reset of the EPC occurs (power-on or front panel reset), the system performs a power-on self-testXE "power-on self-test"§ (POSTXE "POST"§) which will display information showing the status of the BIOS self test if a monitor is attached via a video adapterXE "BIOS selftest"§.  If everything proceeds normally, the screen image will appear approximately as shown below.

386SL Modular BIOS V3.06abd.

Copyright (c) 1984-90 Award Software Inc.
Copyright 1992 RadiSys Corporation  BIOS V3.06

TESTING INTERRUPT CONTROLLER #1 	OK
TESTING INTERRUPT CONTROLLER #2 	OK
TESTING CMOS BATTERY 	OK
TESTING CMOS CHECKSUM 	OK

SIZING SYSTEM MEMORY 	 640K	FOUND
TESTING SYSTEM MEMORY 	 640K	OK

CHECKING UNEXPECTED INTERRUPTS AND STUCK NMI 	OK
TESTING PROTECTED MODE 	OK

SIZING EXPANSION MEMORY 	 3072K	FOUND
TESTING MEMORY IN PROTECTED MODE 	 3712K	FOUND

TESTING PROCESSOR EXCEPTION INTERRUPTS 	OK
TESTING ONBOARD SERIAL PORT #1 	OK
TESTING ONBOARD SERIAL PORT #2 	OK

RadiSys EPC Setup Program

The values shown in the two lines SIZING EXPANSION MEMORY and TESTING MEMORY IN PROTECTED MODE may be different than shown above depending on the memory configuration of each system.

If errors occur during the power-on self-test, the BIOS will display the errorXE "BIOS error"§ on the appropriate line of the screen display and attempt to continue.  For instance, if a failure is discovered in COM1, the BIOS will disable COM1, display the error on the line "TESTING ONBOARD SERIAL PORT #1 ....." and then continue as if the error did not occur.  It is important to watch the POST display to verify that no errors occur.

If error messages are displayed during or after the POST display, see chapter 5,  Troubleshooting & Error Messages on page 27.

## BIOS Setup Screen

The EPC-21/22 BIOS contains a setup function to display and alter the system configuration.  This configuration information is maintained in the EPC's battery-backed CMOS RAMxe "CMOS RAM"§ and is used by the BIOS to initializeXE "initialization"§ the EPC's hardware.

The setup function can be invoked any time after the POST completes and first clears the screen.  Simultaneously press the CTRL+ALT+ESC keysxe "CTRL+ALT+ESC"§.  This may be done during system operation in most, but not all circumstances.  Some programs that take control of the keyboard at a low level, such as Microsoft Windowsxe "Windows"§, cause this key sequence to be interpreted differently, or not at all.  It should always work, however, when the DOS operating system prompt is shown on the screen.

The main setup screenxe "setup screen"§ resembles the following:

RadiSys EPC-22 CMOS Setup, BIOS V3.06
 386SL, 4 MBytes memory,  387SX present

 Date (mm/dd/yy) 	 08/12/93
 Time (hh:mm:ss) 	 07:34:56
 Configuration Errors 	 Halt on all errors
 
 Diskette Drive A 	 1.4M 3.5 inch  
 Diskette Drive B 	 None

Fixed Disk Drive C 	 AT          
 Fixed Disk Drive D 	 None

Com1:	 Enabled   	F2	=	EXM Menu
 Com2:	 Enabled	F3	=	Fixed Disk Menu
                                	F10	=	Save CMOS and EXM data

Cache	 Enabled	ESC	=	Exit without saving
 Quick Boot 	 Disabled	 ¯ ¿	 move between items

¬ ®  select values

Available EXM Slots 	 8 slots