Use the up and down cursor (arrow) keys to move from fieldXE "BIOS setup fields"§ to field.  For most fields, once the cursor is positioned at the field, pressing the left and right cursor (arrow) keys will scroll through the available choices.  Once the screen has been changed to appear as you desire, press the F10 function key to save the changes in battery-backed CMOS RAM.  Then press the F5 function key to confirm the changes and reboot, F1 to confirm changes but not reboot or ESC to ignore any changes and exit.

Each field is explained below.

Date and Time

xe "date"§xe "time"§These values are changed by moving to them and typing in the format shown.

Configuration Errors

This field provides several choices about the situations under which the BIOS should wait for user input if a configuration errorxe "configuration errors"§ is found.  The selections are:

1) Halt on all errors

2) Ignore all errors

3) Ignore keyboard errors    (allows operation without a keyboard)

4) Ignore disk errors

5) Ignore keyboard and disk errors

Diskette Drive

This field gives you several choices about the type of floppy disk drivesxe "floppy disk drives"§ installed as the A and B drives.  Toggle the entry to match the type of diskette drive(s) attached.  If no floppy drive is attached, select NONE for each diskette drive field.

Fixed Disk Drive

xe "Fixed disk drive"§This display-only field shows the type of disk selected from the fixed disk menu.  Possible values are None, AT and EXM FLASH.  To see the detailed characteristics of the device or to change the device, use the F3 function key to go to the fixed disk menuXE "fixed disk menu"§.  See the section Fixed Disk Menu, page 7.

Com1:
Com2:

Toggling these fields enables or disables the Com1 or Com2 ports.

Cache

This field is valid only on the EPC-22.  Toggling this field enables or disables the on-board SRAM cache.

Available EXM Slots

Use the arrow keys to change the displayed number of available EXM slots, or card cage connectors, in the system's backplane.  To determine the correct number of available EXM slots, count the total number of EXM card edge connectors in the system's backplane.

As a "worst case" configuration example, if an eight-slot backplane is accidentally set for only four slots, all EXMs above that fourth slot will not configure.  (The only exception is the video card, which auto-enables to allow for error recovery.)  Continuing with this example, the system would not boot if the boot device was located above the fourth slot. Information is cleared for the missing four slots, and must be re-entered once the number of slots is corrected.

## EXM Setup Screen

This screen is only of significance if the system contains EXM expansion modules.

xe "EXM configuration"§xe "EXM configuration program"§The EXM setup screen is used to configure EXM expansion modules in the system.  It is displayed by pressing the F2 function key from the main setup screen.  The EPC's battery-backed RAM holds the identification and configuration information for the number of available EXM slots entered in the EXM Main Setup Screen.  The BIOS displays the configuration information in hexadecimal format. The EXM Setup screen for a system with an eight-slot backplane, a six-slot chassis, an 
EMC-FDM in slots 6 and 7, and an EPC-22 in slot 0 might resemble the following:

RadiSys EPC-22 EXM Setupxe "Fixed Disk Menu"§, System BIOS V3.06
 386SL, 4 MBytes memory,  387SX present

  	0	FF	    00	    00

1	ED	    01	    00	F10 = Save and return
	2	7D	    07	    00xe "VXI Register Base"§	xe "ULA"§ESC = Return without saving

3	DB	    C1	    00	
	4	D5	C1	00		  ¯ ¬ ® move between items

5	DE	00	39

6	7C	FF	FF

7	FF	00	00

ID is the unique EXM-ID number.

OB1 and OB2 are option bytes 1 and 2.

The option bytes interpretation is unique to each EXM-ID.

See the EXM user manuals for the semantics of the option bytes.

EXMs must be defined in this screen so the BIOS can correctly identify and initialize each one at boot-up.  Each EXM must be listed by slot number, ID and two option bytes as defined  below.

SLOT	indicates the EXM slot in which the EXM is installed.

ID 	is a hard-wired identification value.  Each type of EXM has a unique ID value.

OB1/OB2 	are two "option" bytes of configuration information.

All slots not occupied by an EXM module should show an ID of FF indicating that no EXM is present.

Consult the EXM manual for the correct configuration information for each EXM expansion module installed.

After all EXMs have been configured, press F10 to save the data or ESC to ignore the changes.  In either case you will be returned to the main setup screen.

## Fixed Disk Menu

xe "EXM configuration"§xe "EXM configuration program"§The Fixed Disk Menu is used to define the type of hard disk(s) installed in the system.  Enter the Fixed disk menu screen by pressing the F3 function key from the main setup screen.

The Fixed disk menu screen resembles the following:

RadiSys EPC-22 Fixed Disk Menuxe "Fixed Disk Menu"§, System BIOS V3.06
386SL, 4 MBytes memory,  387SX present

 Fixed Disk Drive C:  AT
     Type  40          115 MBytes:  814 Cyls,   9 Heads, 32 Sectors

Landing Zone: 1023         Precompensation: None

 Fixed Disk Drive D: None
  
                                      	F10 = Save and return
 xe "VXI Register Base"§			xe "ULA"§ESC = Return without saving
                                       	 ¯  ¿ move between items

¬ ® select values

Use the up and down cursor keys ( ¯) to move between items.  Use the left and right cursor keys (¬ ® ) to scroll through the available choices for each item.

Disk type ATxe "AT"§ denotes the many types of non-SCSI PC/AT compatible drives including IDE.  You can scroll through a set of numbered types; the physical configuration is displayed for each.  Scroll through the numeric drive types to find the one matching the characteristics of the hard drive installed.

Choose disk type EXM Flash if you are using an EXM flash disk.  An EXM flash disk can be made the boot device by making EXM Flash the drive C: type.  However, if Drive C: is EXM Flash, Drive D: must be set to None.

Choose None if there is no hard disk present.  This avoids the timeout period used by the BIOS to conclude that there is no responding device.

### User-Definable Drive Types

If the correct AT disk type is not listed, the EPC provides user-editable drive types 48 and 49.  Select either of these drive types.  Use the TAB key (®| )or the left and right cursor keys (¬ ®) to move to the next (or previous) field.  Note that the default settings for MBytes, Cylinders, Heads, and Sectors is 1.  MBytes is a display-only field calculated by the BIOS.  Move the cursor to each field (Cyls, Heads, and Sectors) and type the value for that field.

The BIOS allows use of the following maximum values:

Cylinders    1023		Heads    63		Sectors    16

The hard disk you are using may have parameters larger than the allowable maximum.  If the number of cylinders for your drive is greater than the maximum allowable number, you may have to use 1023 cylinders which will limit the usable size of the drive.  However, most IDE drives support universal translation mode.  If the drive you are using supports this mode and the actual parameters are greater than the allowable maximum, divide the actual number of cylinders by 2 and multiply the actual number of heads by 2.    That is, each sector is addressed as an absolute sequential sector number.  Since the drive converts the sector data to an absolute number, these "false" cylinder and head numbers will still allow the full capacity of the drive to be used.  The example on the next page shows how this is done.

Example:

|  |  | Actual parameters | Conversion factor | Numbers to Use |
| --- | --- | --- | --- | --- |
|  | Cylinders | 1350 | divide by 2 | 675 |
|  | Heads | 5 | multiply by 2 | 10 |
|  | Sectors | 32 | (none) | 32 |
|  | Total Sectors | 216,000 |  | 216,000 |

After the fixed disk(s) have been configured, press F10 to save the data or ESC to ignore the changes.  In either case you will be returned to the main setup screen.  Follow the instructions on the screen to save and exit or ignore changes and exit.

NOTES