# 3.  Theory of Operation

The EPC-21 and EPC-22 are PC/AT compatible processor modules.  The standard functions of the PC architecture are embodied in the Intel386 SLxe "TI83000"§ chip set.  Unless otherwise stated, all the following applies to both the EPC-21 and EPC-22.

## Processor and Coprocessor

The EPC uses the Intel386 SL CPU.

The EPC-21 runs at 16 MHz without a cache.  There is no math coprocessor nor coprocessor socket.

The EPC-22 runs at 25 MHz with a 64K SRAM cache and includes an Intel 80387SX math coprocessor.  Note that this is not a socketed part.

## Memory

Each EPC has a base memory configuration of 1 MByte or 4 Mbytes soldered on the motherboard.  This memory is arranged as 2 banks (banks 0 and 1) of memory chips located at locations U12 and U13 on the upper rear corner of the circuit board.  See Figure 1 below.

#### Figure 1:  Base Memory Chip Location

For systems with 1 Mbyte of base memory, the memory technology is 4Mbit chips organized as two 256K by 18-bit banks.  This base memory configuration can be identified on the board by examining the memory chips and the corresponding pads.  The circuit board provides 42 pads per chip but the chips are 40-pin.  Therefore the rear-most pads are not used.

For systems with 4 MBytes of base memory, the memory technology is 16Mbit chips organized as two 1MB x 18-bit banks.  As with the 1 MByte system, identifying this memory configuration requires looking closely at the rear pads at location U12 and/or U13.  All pads are used for this memory configuration.

### Memory Expansion

In addition to the base memory, a single 72-pin SIMM socket is provided for memory expansion.  SIMM memory occupies banks 2 and 3.  Any standard SIMM module may be used that meets the following criteria:

- fast page mode

- 72-pin

- 80 nanosecond DRAM

- single-sided

- one- or two-bank

Allowable system memory configurations and the specifics of which SIMM to use are dependent on the system base memory.  The tables below specify the correct SIMM module for each possible configuration.

For 1 Mbyte of base memory

| Total Memory | SIMM Memory | SIMM organization |
| --- | --- | --- |
| 1 MB | None |  |
| 3 MB | 2 MB | 1 bank 1 MB x 18-bit |
| 5 MB | 4 MB | 2 banks 1 MB x 18-bit |
| 9 MB | 8 MB | 1 bank 4 MB x 18-bit |

For 4 Mbytes of  base memory

| Total Memory | SIMM Memory | SIMM organization |
| --- | --- | --- |
| 4 MB | None |  |
| 6 MB | 2 MB | 1 bank 1 MB x 18-bit |
| 8 MB | 4 MB | 2 banks 1 MB x 18-bit |
| 12 MB | 8 MB | 1 bank 4 MB x 18-bit |
| 20 MB | 16 MB | 2 bank 4 MB x 18-bit |

### Memory Map

The Intel386 SL supports a 25-bit physical memory address.  Memory at addresses between 0 and 20 MB (13FFFFFh) is mapped as follows:

Range			Content

0000000	-   009FFFF	DRAM (first 640 KB)

00A0000	-   00BFFFF	mapped to EXM expansion interface; almost always 
		used by a video controller as video RAM

00C0000	-   00CFFFF*	Write-protected DRAM containing video BIOSxe "video BIOS"§

00D0000	-   00EFFFF*	Uncommitted; mapped to EXM expansion interface

00F0000	-   00FFFFF	Write-protected DRAM containing BIOS

0100000	-   13FFFFF	Extended Memory when installed or
		mapped to EXM expansion interface

1400000	-   1FEFFFF	Mapped to EXM expansion interface

1FF0000	-   1FFFFFF	Mapped to BIOS ROM

*  0C8000 - 0EFFFF may be used either as page frame or I/O buffer (i.e. for EMM, 
       Ethernet, etc.) or may be used by DOS 5.0 as upper memory blocks if an EMM
      driver is installed.

Note that since the EXM expansion interface has 24 address lines, some of the "mapped to EXM expansion interface" address areas map repeatedly, or wrap-around, in the expansion interface's address space.

## Cache (EPC-22 only)

The cachexe "cache"§ is a 64 KB four-way set-associative cache.  This is a write-throughxe "write through"§ cache, meaning that memory writes from the 386SL that hit the cache (find the addressed location in the cache) also write into the DRAM.

Addresses from 0 to 640K are cached.  Addresses from 640K to 1M are not cached.  Addresses from 1M to the top of  installed memory are cached with the exception of a secondary graphics frame buffer such as is used on the EXM-14 Live video board.

## ROM and ROM Shadowing

The EPC contains a BIOS EPROM thatxe "EPROM"§ is mapped into the top of the processor's 
25-bit address space.  The EPROM contains the PC BIOS,  self test functions, and the setup screen program.

xe "ROM shadowing"§For best possible performance, the BIOS initialization software copies the ROM contents into DRAM (called shadowing) at addresses 0F0000-0FFFFF (also called the "F" page).  The BIOS also searches for the existence of a video adapter containing a video BIOS.  If a video BIOS is found, it is copied into the 0Cxxxx ("C" page) area of DRAM.

After copying into these areas, the BIOS write-protects them.  Subsequent writes to these areas complete successfully but do not alter the data.

## Battery

The batteryxe "battery"§ powers the CMOS RAMxe "CMOS RAM"§ and Time of Day clockxe "TOD clock"§ when system power is not present.  At 60°C, the battery should have a shelf life of over four years.  In a system that is powered on much of the time and where the ambient power-off temperature is less than 60°C, the battery is estimated to have a life of 10 years.

The battery supplied with the EPC is a 23mm. 3V lithium "coin" battery or equivalent (e.g. Panasonic BR2330 or Rayovac BR2335).  It is mounted on the component side of the circuit board near the bottom front corner.  Should the batteryxe "battery"§ fail, you may obtain and install a replacement.  Figure 2 below illustrates how to change the battery.

#### Figure 2: Battery Replacement

Replacing the battery is a simple task.  However, removing the battery will invalidate the CMOS setup parameters.  It is recommended that all setup parameters be recorded in a safe place while the battery is still good.

## Video Controllers

The EPC can operate with or without a video controllerxe "video controller"§ (such as the EXM-6 or EXM-13xe "EXM-6"§).  The BIOS searches for an EXM having an EXM ID in the range 
E8h-EFh (a range reserved for video controllers).  The search is done by EXM slot number, beginning at slot 0.  If no EXM video adapter is found, the BIOS will look for a PC add-in card with video BIOS.  The CPU scans the memory space from  A0000h  to C8000h looking for any memory location containing the value AA55h.  If such memory is found (add-in card BIOS ROM), the main BIOS will "jump" into the add-in card BIOS and execute whatever code is found.

In either case, the BIOS automatically initializes and uses the first one found.

If no video controller is present, the BIOS will operate without one.  Programs that use the standard operating system and BIOS character output functions can be run successfully (the output is just ignored).  However, programs that rely on specific video modes, that write directly into the video RAM, or that directly call video BIOS functions, will fail.

## Front Panel LED

The EPC has one LED on the front panel.  This RUN LED is lit whenever the EPC's memory is being accessed.  It first comes on at power-up and should remain lit as long as the system is running.  It is normal for the RUN LED to flicker during power-up.  If the processor halts (or hangs) , the LED will go out.

## Resetting the EPC

There are a number of ways to reset (reboot) the EPC.

Power-off, Power-on

This causes the entire system to reset.  The system will run the power-on self-tests and reboot the operating system.

Front-panel Reset button

The Reset button causes the EPC to perform a hardware reset.  The system will run the power-on self-tests and reboot the operating system.

Expansion Interface

Pin A57 on the EXM expansion connector is defined as ~RESETIN.  Asserting this input (low) will reset the processor.  This is provided to allow remote reset to be implemented.  This is a full hardware reset.  The system will run the power-on self-tests and reboot the operating system.

Watchdog Timer

Enabling the watchdog timer and allowing it to expire will reset the processor.  The watchdog timer is tied to ~RESETIN pin described above.  The system will run the power-on self-tests and reboot the operating system.

Setup Screen

Pressing F10 and then F5  to save changes and reboot causes a full hardware reset.

CTRL+ALT+DEL

This keyboard sequence is called a "warm boot".  The EPC does not reinitialize all of the processor's hardware.  The power-on self-test does not run.  However, the operating system will be reloaded.  This type of reset typically only works under DOS.

Additional abnormal conditions that cause a reset

Low Vcc

No DRAM refresh signal from the micromonitor

## EXM Expansion Interface

xe "EXMID signal"§The EXM expansion interface is electrically similar to the PC/AT ISA (16-bit data) busxe "PC/AT I/O bus"§.  The expansion interfacexe "EXMbus"§ is provided on both sides of the edge connector on the rear of the EPC.  See Chapter 4, Connectors for details of the signals provided.

## Watchdog Timer

The EPC-21 & 22 use the secondary 8254 timer in the 82360SL chipset to implement a watchdog timer function.  Counter 2 within the 8254 is/should be programmed to reset the CPU when half of the timer's initial count has expired.

See Appendix B for details on programming the watchdog timer.

NOTES