# Appendix B: Programming the Watchdog Timer

Programming the timer requires entry into the 82360SL configuration space.  This configuration space is enabled by performing four sequential reads to the following I/O addresses : 0FC23H, 0F023h, 0C023h and 0023h.  When enabled access to the registers within is controlled by two 8-bit read/write registers: the Configuration Index Register, CFGINDEX (24h) and the Configuration Data Register, CFGDATA (25h).  The CFGINDEX register stores the index of the register to be accessed and the CFGDATA register stores the data.

The code segment below is a Microsoft C for DOS example of how to enable the 82360SL configuration space:

unsigned short

saveflags()

{

unsigned short flags;

ASM

pushf

pop   flags

ENDASM

return flags;

}

unsigned short

enablespace()

{

unsigned short flags;

flags = saveflags();

_disable();

(void) inp(0xFC23);

(void) inp(0xF023);

(void) inp(0xC023);

(void) inp(0x0023);

return flags;

}

The 82360SL configuration space is disabled by setting the Lock bit (bit 0) of the Configuration Lock Register.  Calling the disablespace function below with the processor flags value returned from the enablespace function disables the configuration space and restores the system flags to the state prior to the call of enablespace.

void

restoreflags(unsigned short flags)

{

ASM

push  flags

popf

ENDASM

}

void

disablespace(unsigned short flags)

{

outp(CFGINDEX,0xfa);

outp(CFGDATA,0x01);

restoreflags(flags);

}

Once the 82360SL configuration space is visible, the reset enable, timer command and timeout value can be programmed.  The reset enable is active when bit 0 of the SMOUTCNTRL (index 0xFE) register is cleared (0).  Clearing this bit will cause a PC reset when half of the timer count programmed into the T2CH2CNT has expired.

The watchdog timer's maximum timeout is 491 milliseconds.  This is derived by multiplying the maximum tick count 65535 (0xFFFF) by the clock frequency (15 microseconds), and then dividing by 2 since the PC is reset when half of the count has expired.

Using the watchdog timer function requires the installation of a driver that reprograms the watchdog timer value before half the initial timeout count expires.

This is typically done by chaining the PC timer interrupt and reprogramming the watchdog timer count within the timeout period.

Listed below is a complete program that implements a watchdog timer function.

The watchdog timer is programmed to reset the PC at 491 milliseconds.

#include <dos.h>

#define ASM	_asm {

#define ENDASM	}

#define INTERRUPT	_interrupt

#define CFGINDEX	0x24	/* configuration space index */

#define CFGDATA 	0x25	/* configuration space data address */

#define T2COMMAND	0x83	/* timer 2 command address */

#define T2CH2CNT	0x82	/* timer 2 counter address */

#define TIMER2CW	0xb6	/* counter 2, write, mode 3, bin */

#define TIMERTICKRESOLUTION	15	/* Timer has 15 microsecond tick */

#define TIMERINT	8	/* PC timer interrupt vector */

#define TIMER2TICKS	0xffff	/* ~491 milliseconds	     */

#define SMOUTCTRL	0xfe	/* reset enable latch	     */

void (INTERRUPT *previoustimerfunction)();

volatile unsigned long Tick = 0;

#pragma intrinsic(_disable,_enable,inp,inpw,outp,outpw)

unsigned char

getconfigurationregister(unsigned short slregister)

{

outp(CFGINDEX,slregister);
     	return ((unsigned char) inp(CFGDATA));
   }

void

setconfigurationregister(unsigned short slregister,unsigned char slvalue)

{

outp(CFGINDEX,slregister);

outp(CFGDATA,slvalue);

}

unsigned short

saveflags()

{

unsigned short flags;

ASM

pushf

pop   flags

ENDASM

return  flags;

}

void

restoreflags(unsigned short flags)

{

ASM

push   flags

popf

ENDASM

}

unsigned short

enablespace()

{

unsigned short flags;

/*

//	Any interrupt that occurs between saveflags & disable 
    // should 	restore the flag register (via the iret).  
    // Therefore, the flags variable 	is guaranteed to contain a 
    // copy of the machines state prior to 	disabling 
    // interrupts.

*/

flags = saveflags();

_disable();

/*

//	The 82360SL configuration space is enabled by performing 
    // four sequential reads to the addresses listed below.

*/

(void) inp(0xFC23);

(void) inp(0xF023);

(void) inp(0xC023);

(void) inp(0x0023);

return flags;

}

void

disablespace(unsigned short flags)

{

outp(CFGINDEX,0xFA);

outp(CFGDATA,0x01);

restoreflags(flags);

}

void

refreshtimer()

{

setconfigurationregister(T2COMMAND,TIMER2CW);

setconfigurationregister(T2CH2CNT,TIMER2TICKS & 0xFF);

setconfigurationregister(T2CH2CNT,TIMER2TICKS >> 8);

}

void INTERRUPT

mytimer()

{

unsigned short flags;

if (Tick == 0)

{

flags = enablespace();

refreshtimer();

disablespace(flags);

/*

// The PC timer executes at 54 millisecond intervals or 
                // 18.2 times a second.  The Watchdog timer must be 
                // refreshed within half the timeout period specified in 
                // the count register [(65535ticks / 2) * 15us / 1000ms 
                // = 491ms]

*/

Tick = (unsigned short)

((long) TIMER2TICKS / 2L * (long) TIMERTICKRESOLUTION / 
                 1000L / 54L) - 1;

}

Tick--;

_chain_intr(previoustimerfunction);

}

/*

//	This procedure chains the DOS timer interrupt and turns on the

//	watchdog timer.  The chained DOS timer handler reprograms the

//	watchdog timer with the initial count.

*/

void

installtimer(void (INTERRUPT *newfunction)())

{

unsigned short flags;

_disable();

previoustimerfunction = _dos_getvect(TIMERINT);

flags = enablespace();

refreshtimer();

setconfigurationregister(SMOUTCTRL,

(unsigned char)(getconfigurationregister(SMOUTCTRL) &
         (unsigned char)~1));

disablespace(flags);

_dos_setvect(TIMERINT,newfunction);

_enable();

}

void

deinstalltimer()

{

unsigned short flags;

flags = enablespace();

setconfigurationregister(SMOUTCTRL,

(unsigned char) (getconfigurationregister(SMOUTCTRL) | 1));

_dos_setvect(TIMERINT,previoustimerfunction);

disablespace(flags);

}

main(int argc, char **argv)

{

unsigned short flags;

installtimer(mytimer);

/*

//	Application code goes here ...

*/

deinstalltimer();

}

For more details see the Intel 8254 specification and the Intel 386 SL Microprocessor Superset Programmer's Reference Manual.

NOTES