# 1.  Product Description

The EPC-24/26 family of PC/AT-compatible CPU modules are all derived from a single design. In this manual, generic references to features similar on each model will refer to the "EPC".  Based on the Intel486 Enhanced chip set, these EPCs have been designed to meet stringent safety and low EMI standards (UL-1950).  All front panel accessible ports have filter networks for reduced EMI and increased ESD protection.  The different products are outlined below:

| Product Name | Microprocessor |
| --- | --- |
| EPC-24 | 25 MHz 486 SX |
| EPC-26 | 50 MHz 486 DX2 |

The EPC processor module contains the following:

·	72-pin 3.3V SIMM socket for memory sizes of  4, 8, and 16 MB

·	Keyboard interface, reset switch and run LED

·	1 standard RS-232 9-pin DTE serial port (COM1)

·	5V only operation (3.3V CPU, SIMM, and PC chip set)

·	Time-of-day clock with user-replaceable battery

·	Award BIOS

·	EXM expansion interface

·	Optional Flash/SRAM Memory with Watchdog Timer (cannot be used at the same time as an EXM-2A in the system)

## Specifications

## The following table lists the environmental and electrical specifications of the 
EPC (with SIMM memory included).

| Environmental |  |  |  |  |
| --- | --- | --- | --- | --- |
| Temperature | operating |  | 0° to 60° C (* see below) |  |
|  | storage |  | -40° to 85° C |  |
| Humidity | operating |  | 5 - 95% (non-condensing) |  |
|  | storage |  | 5 - 95% (non-condensing) |  |
| Vibration | operating |  | .015" Peak to Peak 2.5g (max) 5-2000 Hz |  |
|  | storage |  | .030" Peak to Peak 5g (max) 5-2000 Hz |  |
| Shock | operating |  | 30g 11 msec duration |  |
|  | storage |  | 50g 11 msec duration |  |
| Electrical |  |  |  |  |
| EPC-24 power | maximum |  | +5V @ 0.6 A |  |
|  | typical |  | +5V @ 0.4 A |  |
| EPC-26 power | maximum |  | +5V @ 0.8 A |  |
|  | typical |  | +5V @ 0.6 A |  |

Table 1. EPC Environmental and Electrical Specifications.

* Upper temperature limit degrades 2° C per 1000 ft. elevation.  Maximum  
   elevation 10,000 ft.

See Appendix A for mechanical dimensions.