Appendix C: 
Interrupts and DMA Channels

## Interrupts

The assignment of interrupts for the EPC is shown in the following table:

| NMI | DRAM parity error, EXM expansion interface I/O channel check |
| --- | --- |
| IRQ0 | timer |
| IRQ1 | keyboard |
| IRQ2 | IRQ8 - IRQ15 cascade through IRQ2 |
| IRQ3 | unassigned |
| IRQ4 | COM1 serial port |
| IRQ5 | unassigned |
| IRQ6 | usually needed for floppy disk controller |
| IRQ7 | unassigned |
| IRQ8 | clock |
| IRQ9 | unassigned |
| IRQ10 | watchdog timer (for Flash/SRAM option) |
| IRQ11 | unassigned |
| IRQ12 | used by optional SCSI disk controller |
| IRQ13 | coprocessor |
| IRQ14 | used by optional IDE disk controller |
| IRQ15 | unassigned |

## DMA Channels

The assignment of DMA channels for the EPC is shown in the following table.

| 0 | unassigned (8-bit) |
| --- | --- |
| 1 | unassigned (8-bit) |
| 2 | usually needed for floppy disk (8-bit) |
| 3 | usually needed for SCSI disk (8-bit) |
| 4 | (Channel 0 - Channel 3 cascade through Channel 4) |
| 5 | unassigned (16-bit) |
| 6 | unassigned (16-bit) |
| 7 | unassigned - not connected to EXM expansion interface (16-bit) |