# 1.  Product Description

The EPC-26A/27 PC/AT-compatible CPU module is an EMC form factor EPC based on the DX2 and DX4 Intel486 processor. The EPC-26A uses the 50 MHz Intel486 DX2 CPU, while the EPC-27 uses the 100 MHz IntelDX4.

The EPC-26A/27 uses the Pico-Power Redwood chip set, a two-chip set packaged in a 176-pin TQFP.

This EPC meets stringent safety and low EMI standards (UL-1950).  All front panel accessible ports have filter networks for reduced EMI and increased ESD protection.

The EPC-26A/27 processor modules contain the following features:

·	The EPC-26A contains an Intel DX2 Enhanced processor (208-pin SQFP package, 3.3V). The EPC-27 is an Intel DX4 processor.

·	One 72-pin 3.3V SIMM socket for memory sizes of 4, 8, 16 or 32 MB

·	PC/AT compatible keyboard interface, reset switch and run LED

·	Standard PC-compatible peripherals

·	Time-of-day clock with 256 bytes of battery-backed CMOS RAM and user-replaceable battery

·	One RS-232 9-pin DTE serial port (COM1) and one RJ45 serial port (COM2)

·	Flashable Phoenix BIOS

·	EXM expansion interface

·	Optional 2 or 4 MB Flash memory and 128 KB SRAM and watchdog timer

If your EPC-26A/27 contains the optional Flash/SRAM configuration, you will also receive a File/Utilities diskette containing formatting software and SRAM drivers. The software is explained in detail in the XFORMAT Software User’s Manual that accompanies the diskette.

## Specifications

The following table lists the environmental and electrical specifications of the 
EPC (with SIMM memory included).

| Environmental |  |  |  |  |
| --- | --- | --- | --- | --- |
| Temperature | operating |  | 0° to 60° C (* see below) |  |
|  | storage |  | -40° to 85° C |  |
| Humidity | operating |  | 5 - 95% (non-condensing) |  |
|  | storage |  | 5 - 95% (non-condensing) |  |
| Vibration | operating |  | .015" Peak to Peak 2.5g (max) 5-2000 Hz |  |
|  | storage |  | .030" Peak to Peak 5g (max) 5-2000 Hz |  |
| Shock | operating |  | 30g 11 msec duration |  |
|  | storage |  | 50g 11 msec duration |  |
| Electrical |  |  |  |  |
| Power | maximum |  | +5V @ 1.2 A |  |
|  | typical |  | +5V @ 0.8 A |  |

Table 1. EPC Environmental and Electrical Specifications.

* Upper temperature limit degrades 2° C per 1000 ft. elevation.  Maximum  
   elevation 10,000 ft.

NOTE:  The EPC-27 requires 200 ft/min airflow for 60°C operation. For configurations with no airflow, the maximum operating temperature for the EPC-27 is 43°C.

See Appendix A for mechanical dimensions.