# 4.  Connectors

This chapter specifies the details of the connectors on the EPC.  These connectors adhere to existing standards.  Pins are labeled from the point of view of looking into the front of the connector on the EPC.

## Serial Ports

There are two COM ports on the EPC-26A/27. The COM1 serial port is an RS-232 DB-9 DTE connector. It has the following drive capability: VOL = -5V min @ 3K load to GND and VOH = 5V min @ 3K load to GND. COM1 is defined in the following table:

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | DCD | 6 | DSR |
| 2 | RxD | 7 | RTS |
| 3 | TxD | 8 | CTS |
| 4 | DTR | 9 | Ring indicator |
| 5 | Ground |  |  |

Table 4.  COM1 DB9 Pin-out.

COM2 is a DTE RJ45 phone jack and is defined in the following table:

| Pin | Signal |
| --- | --- |
| 1 | Shield |
| 2 | CTS |
| 3 | TxD |
| 4 | DTR |
| 5 | RxD |
| 6 | DCD |
| 7 | Ground |
| 8 | RTS |

Table 5.  COM2 RJ45 Pin-out.

## Keyboard

The keyboard connector is a 6-pin DIN defined below:

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | Data | 4 | +5V |
| 2 | not used | 5 | Clock |
| 3 | Ground | 6 | not used |

Table 6.  Keyboard Connector Pin-out.

## EXM Expansion Connector

The EXM expansion connector on the rear of the EPC is a 116-pin cardedge connector.  The mating connector is an AMP 650090-1 or equivalent.  Pins A1 through A58 are on the component side of the board with pin A1 at the bottom of the board.  Pins B1 through B58 are on the solder side of the board with pin B1at the bottom of the board.

More information about the EXM interface is available upon request.