# Appendix B: 
Chip Set & I/O Map

The following defines the I/O addresses decoded by the EPC. It does not define addresses that might be decoded by EXMs.

| First (8-bit) DMA controller: PicoPower Redwood chip set emulating 8237 of PC/AT |  |  |
| --- | --- | --- |
| I/O Addr | Functional group | Usage |
| 000 | DMA | Channel 0 address |
| 001 |  | Channel 0 count |
| 002 |  | Channel 1 address |
| 003 |  | Channel 1 count |
| 004 |  | Channel 2 address |
| 005 |  | Channel 2 count |
| 006 |  | Channel 3 address |
| 007 |  | Channel 3 count |
| 008 |  | Command/status |
| 009 |  | DMA request |
| 00A |  | Command register (R) |
|  |  | Single-bit DMA req mask(W) |
| 00B |  | Mode |
| 00C |  | Set byte pointer (R) |
|  |  | Clear byte pointer (W) |
| 00D |  | Temporary register (R) |
|  |  | Master clear (W) |
| 00E |  | Clear mode reg counter (R) |
|  |  | Clear all DMA req mask(W) |
| 00F |  | All DMA request mask |

| First Interrupt controller: PicoPower Redwood chip set emulating 8259 of PC/AT |  |  |
| --- | --- | --- |
| I/O Addr | Functional group | Usage |
| 020 | Interrupt controller 1 | Port 0 |
| 021 |  | Port 1 |

| 82C42 controller: |  |  |
| --- | --- | --- |
| I/O Addr | Functional group | Usage |
| ED |  | Data register |
| EC |  | Index register |

| Counter-Timer functions: PicoPower Redwood chip set emulating 8254 of PC/AT |  |  |
| --- | --- | --- |
| I/O Addr | Functional group | Usage |
| 040 | Timer | Counter 0 |
| 041 |  | Counter 1 |
| 042 |  | Counter 2 |
| 043 |  | Control (W) |

| Keyboard Port: PicoPower Redwood chip set emulating PC/AT |  |  |
| --- | --- | --- |
| I/O Addr | Functional group | Usage |
| 060 | Keyboard controller | Data I/O register |
| 061 | NMI status | NMI status |
| 064 | Keyboard controller | Command/status register |

| Time-of-Day Clock: PicoPower Redwood chip set emulating MC6818 of PC/AT |  |  |  |  |
| --- | --- | --- | --- | --- |
| I/O Addr | Functional group | Usage |  |  |
| 070 | Real-time clock | RTC index reg / NMI enable |  |  |
| 071 |  | RTC data register |  |  |
|  |  | 0 |  | seconds |
|  |  | 1 |  | seconds alarm |
|  |  | 2 |  | minutes |
|  |  | 3 |  | minutes alarm |
|  |  | 4 |  | hours |
|  |  | 5 |  | hours alarm |
|  |  | 6 |  | day of week |
|  |  | 7 |  | date of month |
|  |  | 8 |  | month |
|  |  | 9 |  | year |
|  |  | A |  | status A |
|  |  | B |  | status B |
|  |  | C |  | status C |
|  |  | D |  | status D |
|  |  | E |  | RAM |
|  |  | ... |  |  |
|  |  | 3F |  | RAM |

| DMA Page Registers: PicoPower Redwood chip set emulating 74LS612 of PC/AT |  |  |
| --- | --- | --- |
| I/O Addr | Functional group | Usage |
| 081 | DMA | Channel 2 page register |
| 082 |  | Channel 3 page register |
| 083 |  | Channel 1 page register |
| 087 |  | Channel 0 page register |
| 089 |  | Channel 6 page register |
| 08A |  | Channel 7 page register |
| 08B |  | Channel 5 page register |
| 08F |  | Refresh page register |

| Second Interrupt Controller: PicoPower Redwood chip set emulating 8259 of PC/AT |  |  |
| --- | --- | --- |
| I/O Addr | Functional group | Usage |
| 0A0 | Interrupt controller 2 | Port 0 |
| 0A1 |  | Port 1 |

| Second (16-bit) DMA Controller: PicoPower Redwood chip set emulating 8237 of PC/AT |  |  |
| --- | --- | --- |
| I/O Addr | Functional group | Usage |
| 0C0 | DMA | Channel 4 address |
| 0C2 |  | Channel 4 count |
| 0C4 |  | Channel 5 address |
| 0C6 |  | Channel 5 count |
| 0C8 |  | Channel 6 address |
| 0CA |  | Channel 6 count |
| 0CC |  | Channel 7 address |
| 0CE |  | Channel 7 count |
| 0D0 |  | Command/status |
| 0D2 |  | DMA request |
| 0D4 |  | Command register (R) |
|  |  | Single-bit DMA req mask(W) |
| 0D6 |  | Mode |
| 0D8 |  | Set byte pointer (R) |
|  |  | Clear byte pointer (W) |
| 0DA |  | Temporary register (R) |
|  |  | Master clear (W) |
| 0DC |  | Clear mode reg counter (R) |
|  |  | Clear all DMA req mask (W) |
| 0DE |  | All DMA request mask |

| Coprocessor Interface: An integrated co-processor replaces the 80287 of PC/AT |  |  |
| --- | --- | --- |
| I/O Addr | Functional group | Usage |
| 0F0 | Coprocessor | Clear coprocessor busy |
| 0F1 |  | Reset coprocessor |

| Serial I/O (Com2) Port: PicoPower Redwood chip set emulates 16550 of PC/AT |  |  |
| --- | --- | --- |
| I/O Addr | Functional group | Usage |
| 2F8 | COM2 serial port | Receiver/transmitter buffer |
|  |  | Baud rate divisor latch (LSB) |
| 2F9 |  | Interrupt enable register |
|  |  | Baud rate divisor latch (MSB) |
| 2FA |  | Interrupt ID register |
| 2FB |  | Line control register |
| 2FC |  | Modem control register |
| 2FD |  | Line status register |
| 2FE |  | Modem status register |

| Serial I/O (Com1) Port: PicoPower Redwood chip set emulates 16550 of PC/AT |  |  |
| --- | --- | --- |
| I/O Addr | Functional group | Usage |
| 3F8 | COM1 serial port | Receiver/transmitter buffer |
|  |  | Baud rate divisor latch (LSB) |
| 3F9 |  | Interrupt enable register |
|  |  | Baud rate divisor latch (MSB) |
| 3FA |  | Interrupt ID register |
| 3FB |  | Line control register |
| 3FC |  | Modem control register |
| 3FD |  | Line status register |
| 3FE |  | Modem status register |