# 1.  Product Description

The EXM-HD is an EXM expansion module containing an IDE (integrated drive electronics) hard disk.  It provides a fully functional hard disk in a single EXM slot form factor.

The EXM-HD expansion module is compatible with all PC operating systems because the IDE hard disk is fully PC-compatible at the hardware level.

The construction and definition of the EXM-HD prohibits its use in several logical and mechanical configurations.  The contexts in which an EXM-HD may not be used are the following:

·	In the same system with an EXM-9 disk controller unless the EXM-HD and EXM-9 are not enabled at the same time

·	In the same system with another EXM-HD or EXM-MX, unless at most one hard disk is enabled at the same time

·	In the same system with an EXP-MX mass storage module

·	In any VXI system, or in an EPC-6 system.

The EXM-HD200 cannot be used in an EPC-3 or in an EXP-MC module carrier.

## Specifications

| Environmental |  |  |  |  |
| --- | --- | --- | --- | --- |
| Temperature (Ambient) | operating |  | 5°C to 50°C 10°C per hour max excursion gradient |  |
|  | storage |  | -40 to 65°C 20°C per hour max excursion gradient |  |
| Humidity | operating |  | 10% - 90% non condensing 26°C max wet bulb |  |
|  | storage |  | 10% - 90% non condensing 46°C max wet bulb |  |
| Altitude | operating |  | -60 m to 3000 m |  |
|  | storage |  | -60 m to 12,000 m |  |
| Vibration (measured on | operating |  | 1.0 g acceleration over 5-300 Hz sine wave (peak to peak), 1 oct/min sine sweep |  |
| the hard drive) | storage |  | 2.0 g acceleration over 5-300 Hz sine wave (peak to peak), 1 oct/min sine sweep |  |
| Shock (measured on | operating |  | 10 g, 10 ms duration, half-sine shock pulse |  |
| the hard drive) | storage |  | 50 g, 11 ms duration, half-sine shock pulse |  |
| Electrical |  |  |  |  |
| Current | maximum |  | 5V @ 1.2A |  |
|  | typical |  | 5V @ 0.6A |  |
| Other |  |  |  |  |
| Weight |  |  | 11 oz (310 g) |  |
| EXM | IDE address |  | 3F6 & 3F7 |  |
| interface | Interrupt |  | IRQ 14 |  |

Table 1. EXM-HD Environmental and Electrical Specifications.