# 2.  Installation

Before installing your EXM-HD, you should unpack and inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-hd, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Before Installation

Before inserting the EXM-HD into the system, check the drive type label on the side of the unit.  Write down the appropriate drive type to use for this module.  You will need this information later.

## Insertion in an EXM Carrier

Insertion of the EXM-HD into an EXM carrier is straightforward.  Remove any blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the EXM-HD into the cardguides.  Firmly press the EXM-HD front panel to ensure that it has mated with the rear connector, and secure it with the thumbscrews.

*	make sure that power to your system is off.  the exm is not designed to be inserted or removed from a live system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

Note the following restriction before you install the EXM-HD:

o	The EXM-HD cannot be installed in any VXI system, nor can it be installed in an  EPC-6 system.   The EXM-HD200 cannot be installed in an EPC-3 or an 
EXP-MC module carrier.

## Configuration

To use the EXM-HD, you will need to perform several steps to configure it 
logically into your system and prepare the hard disk for use:

·	Enable it as an EXM module

·	Specify the hard disk type in the CMOS setup

·	Partition and format the hard disk for your operating system

### The EPC BIOS

The BIOS in the EPC to which the EXM-HD is connected needs to be configured to enable the hard disk and specify the hard disk type.  For the hard disk, the type identifies the basic hard disk parameters: the number of cylinders, heads, and sectors in the hard disk assembly.  Follow the instructions in the EPC reference manual for your specific EPC model to invoke the BIOS setup function. This is typically done by pressing the CTRL-ALT-ESC keys simultaneously.  EPC system configuration procedures differ slightly depending on BIOS versions.

## Configuration Using BIOS Setup Screen

Establish the EXM-HD's configuration using the BIOS setup screen.  To invoke the setup program, press CTRL+ALT+ESC at the operating-system prompt, or reboot your EPC, wait for the self-tests to complete, and press CTRL+ALT+ESC.

Once in the setup program, a menu displays specifying which function keys are available for further configuration.  Press the F2 function key to invoke the EXM menu.  The screen display resembles Figure 1 below.

ID OB1 OB2       
 Slot 0    FF  00  00
      1    DB  C1  00
      2    7D  05  00
      3    DE  01  00
      4    ED  01  00
      5    DC  F5  91

Figure 1. EXM Setup Screen.

The setup screen displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.  The displayed data shows SLOT, ID, OB1 and OB2 for each installed EXM.

Note that all slots are listed on the screen even if the actual system configuration does not have all the possible EXM slots.  All slots not occupied by an EXM module should show an ID of FF and OB1/OB2 of  00  00  indicating that no EXM is present. A typical value for the EXM-HD is shown in bold letters in slot 2.

Slot, OB1 and OB2 are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See the diagram below to determine which EXM slot the EXM-HD occupies.  Note that dashed lines indicate EXM slots that may not be available on all systems.

EXM Slots

µ §

Figure 2. EXM Slot Numbering.

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-HD is 7Dh. OB1 and OB2 are two bytes of option information.

OB1 is a hexadecimal value derived by combining the following:

| Value | Configuration |
| --- | --- |
| 00 | EXM-HD disabled |
| 05 | EXM-HD IDE interface enabled |

Table 2. OB1 Configuration Bit Definition.

For example, a typical setting for OB1 would be 05h, indicating that the IDE interface is enabled.

OB2 is not used and should be set to 00.

### Specify the Hard Disk Type

A separate screen exists for specifying the fixed (hard) disk type.  Check the function key menu to determine the appropriate function key to bring up the Fixed disk menu.  The screen is similar to the one below.

Fixed Disk Menu

 Fixed Disk Drive C: None

 Fixed Disk Drive D: None

First toggle to the AT option for drive C.  Selection of this option causes a type line to appear.  The type line allows you to select among a large variety of predefined drive types, or to specify the detailed parameters of a drive not defined by a predefined type.  Toggle to the type number that matches the type number on the sticker on the side of your drive.

Fixed disk Drive D: typically should be set to NONE  After selecting the appropriate values, press F10 to save this data and return to the main Setup screen.

When you are sure that the CMOS Setup data is correct, press F10 to save this data.  Then press F5 to confirm the data and reboot.

### Low-level Formatting

Caution!

If you are using a drive type that is larger than the actual drive capacity, do not attempt to low-level format the drive.  You may permanently damage the drive.

No function is provided for low-level formatting IDE drives.

All disk drives used by RadiSys are pre-formatted by the manufacturer.  Modern disks, with automatic bad-block mapping, don't need low-level formatting.

Previous generations of hard disks sometimes needed to be formatted at a low level to set the interleave, map out bad sectors, or to clear the disk of all data.  Disks were typically delivered by the manufacturer in an unformatted state, so several commercial programs are available that perform a low-level format.  RadiSys does not supply nor recommend any particular program for this purpose.

### Disk Partitioning and Formatting

The EXM-HD is shipped from the factory with no partition or high-level formatting.  The hard disk must be partitioned and formatted for your operating system before you can load any software.

Partitioning is the process of building the primary data structures on the hard disk that define the physical characteristics of the drive and divide the disk into one or more sections.  Formatting (also called high-level formatting) is the process of actually building a file system on a disk drive partition - basically setting up each partition so that it "looks like" a file system.

Partitioning and Formatting for DOS

The following procedure requires a floppy drive be installed. The process for 
MS-DOS 5.0 is described here.  Other operating systems have similar procedures, with different details. Refer to your operating system manual for more information.

1)	Boot the system using the operating system SETUP diskette.

2)	You are asked several questions.  Follow the instructions on the screen until you see the following prompt:

Allocate all free hard disk space for MS-DOS

Allocate some free hard disk space for MS-DOS

Do not allocate free hard disk space for MS-DOS

The first option will be highlighted.  You can allocate ALL the hard disk space by pressing ENTER.  The SETUP program will partition the disk and reboot the system.  The SETUP program will be re-invoked and automatically start formatting the disk.

Note that you may choose to "Allocate some free hard disk space for MS-DOS" to allow multiple partitions on the disk.   If you do, the process is slightly different.

3)	The remainder of the installation process is automatic.  Follow the instructions on the screen.

## Troubleshooting

| Symptom | Possible Cause | Solution |
| --- | --- | --- |
| Cannot access hard drive | BIOS has not enabled the EXM-HD The fixed-disk type assumed by the BIOS is incorrect. There is another disk controller in the system interfering with the EXM-HD. | Check the entry in the BIOS setup screen EXM menu. Check the drive type in the BIOS setup Fixed disk menu. Reread the configuration rules in Chapter 1. |
| Cannot boot from hard disk. | The hard disk may not have been formatted as a system disk. Wrong drive type in the BIOS Setup. The partition may not be set Active. | See your operating system manual for details on making a system disk. Check the BIOS setup screen against the drive type sticker to verify the correct drive type entry. Use the partitioning function for your operating system to activate the partition. |

Table 3. Troubleshooting Suggestions.

NOTES