# 3. Programming Interface

This chapter contains information needed to write custom software drivers for the EXM-HD.  Anyone using the EXM-HD as a standard PC/AT compatible hard disk and floppy disk controller can ignore this chapter. The EXM-HD defines the following registers in the I/O space.

|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 | I/O port |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Device ID Register | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Config Option Byte 1 Reg |  | IDEN | 0 | CDEN | 102 |
| --- | --- | --- | --- | --- | --- |

| Data Register |  | 1F0 |
| --- | --- | --- |

| Error / Write Precomp Reg |  | 1F1 |
| --- | --- | --- |

| Sector Count Register |  | 1F2 |
| --- | --- | --- |

| Sector Number Register |  | 1F3 |
| --- | --- | --- |

| Cylinder Low Register |  | 1F4 |
| --- | --- | --- |

| Cylinder High Register |  | 1F5 |
| --- | --- | --- |

| SDH Register |  | 1F6 |
| --- | --- | --- |

| Status / Command Register |  | 1F7 |
| --- | --- | --- |

| Alt Status / Digital Out Reg |  | 3F6 |
| --- | --- | --- |

| IDE interface | Z |  |  |  | IDE |  |  |  | 3F7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Z = not driven onto EXM expansion interface on a read.

The first two are standard EXM registers for device identification and configuration.  The EXM-HD responds to accesses to these ports only if EXM interface line -EXMID is asserted.  An 8-bit read from I/O address 100h returns the value 7D, the device ID of the EXM-HD.  A read/write configuration register appears at I/O address 102h.  The defined bits are

IDEN specifies whether the IDE interface and hard drive is enabled (1) or disabled (0).  If IDEN and CDEN are set, the I/O addresses 1F0-1F7 and 3F6-3F7 are active.

CDEN specifies whether the EXM-HD is enabled (1) or disabled (0).  If disabled, the EXM-HD will not respond to the 1F0-1F7 and 3F2-3F7 I/O addresses; it will only respond to reads from I/O port 100h and reads and writes from I/O port 102h, and then only if EXM interface line -EXMID is asserted.

The IDE interface defines bits 0-6 of address 3F7. Bit 7 is reserved and is not driven.

For an explanation of the IDE registers, refer to documentation on any standard PC/AT fixed-disk and floppy drive interface.