# 1.  Product Description

This manual contains the information required to install and use the EXM-PS power supply module.

The EXM-PS is a power supply module in the EXM form factor, designed to power a RadiSys Embedded Modular Computer (EMC) system from a DC input voltage. It is designed for use in a small chassis such as the EMC-CC4.

Two versions of the EXM-PS are available: one is a 24-VDC nominal input model (EXM-PS24DC) and the other version is a 48-VDC nominal input voltage model (EXM-PS48DC).

These power supplies are designed to meet the specifications of British Telecom BTR 2511, with respect to immunity to transients and noise on the incoming power, and conducted noise and ripple.

The EXM-PS is based on a packaged, multi-output DC-DC converter. Other circuitry on the EXM-PS includes a power switch, fuse, filter components, and a 5V-detect circuit. The EXM-PS front panel contains a connector for input power, a power switch, a replaceable fuse holder, and three LEDs. The LEDs are labeled Power On, +5V OK, and Overtemp.

Note that the EXM-PS typically is convection cooled. If temperature-related problems occur, a forced-air fan may be required in the system. Contact RadiSys Technical Support for more information.

There are several safety considerations to follow. Safety considerations are listed in Section 2.

## Specifications

Table 1 defines the environmental and electrical specifications of the EXM-PS.

| Output rating | +5VDC | 500mA min., 6.0A max. |
| --- | --- | --- |
|  | +12VDC | 650mA |
|  | -12VDC | 650mA |
|  | -15°C to +60°C | 15W output |
|  | -15°C to +40°C | 25W output |
|  | Load regulation | 3% max |
|  | Line regulation | 3% max |
|  | Ripple & Noise | 5V: 100mV ±12V: 250mV |
| Efficiency | Nominal | ~75% at full load |
| Temperature | Operating | -15°C to +60°C |
|  | Storage | -62°C to +85°C |
| Humidity | Operating | 5% to 95% non-cond. |
|  | Storage | 5% to 95% non-cond. |
| Vibration | Operating | .015" P-P 2.5g (max) 5 to 2000 Hz |
|  | Storage | .030" P-P 5.0g (max) 5 to 2000 Hz |
| Shock | Operating | 30G, 11 msec, ½ sine wave |
|  | Storage | 50G, 11 msec, ½ sine wave |
| Dimensions | Height | 106.7 mm (4.2 in) |
| (with tabs & | Width | 20.3 mm (0.8 in) |
| connectors) | Depth | 172.7 mm (6.8 in) |
| Weight |  | 1.02 kg (2.25 lbs) |
| Input | EXM-PS-DC24 | 18-36 VDC |
| Voltage | EXM-PS-DC48 | 36-72 VDC |
| Input | EXM-PS-DC24 | 1.8A (25W load) |
| Current | EXM-PS-DC48 | 0.9A (25W load) |
| Inrush Current |  | 15A max. |
| Fuse Type | EXM-PS-DC24 | IEC Pub. 127/III, 6.30A 250V |
|  | EXM-PS-DC48 | IEC Pub. 127/III, 3.15A 250V |

Table 1. EXM-PS Environmental and Electrical Specifications.