# 3. Operation

## Power Switch

The front-panel switch disconnects the input power from the supply when in the "Off" position (labeled with the letter "O"). To avoid any chance of ESD damage, be sure to wait at least ten seconds after turning the power off before removing any EXM modules or the EXM-PS from the chassis.

## Fuse

The replaceable fuse is accessible from the front panel. It is housed in a ¼ turn socket immediately above the power switch. The correct replacement fuse type is specified on the front panel immediately above the fuse socket. When replacing the fuse, be sure to use the same type and rating fuse specified.

## LEDs

There are three front-panel LEDs: Power On, Overtemp, and 5VDC OK. These LEDs are explained below.

### Power On

Power On indicates that the power supply is providing output power to the backplane.

### Overtemp

Overtemp is lit whenever the internal temperature of the power supply exceeds 100°C. DO NOT allow continued operation of the EXM-PS if this LED is illuminated.

### 5VDC OK

5VDC OK indicates that the 5V output voltage is greater than 4.6V typical ± .1V. This LED circuit only measures under-voltages, not over-voltages.