# 1.  Getting Started

This manual contains the information you will need to install and use the EXM-1 Ethernet Controller and accompanying software.

EXM-1 is a high-speed Ethernet and thin Ethernet (Cheapernet) controller based on the Intel 82592 controller and 82560 DMA chips.  It provides two interfaces: a DB-15 DIX/MAU connector for connecting a transceiver cable to an external Ethernet transceiver, and a BNC connector for connecting an RG58-type cable for thin Ethernet.  Choosing between use of these two connectors is the only physical installation option; all other configuration parameters are handled by software.

Software provided with the EXM-1 includes an optional remote boot ROM for installation on the EXM-1 and drivers for TCP/IP and Novell NetWare (as a client).  The drivers are provided on a floppy diskette included with the EXM-1.  You should read the readme.txt file on the floppy diskette for update information.

## Understanding Packet Protocols

Software provided with the EXM-1 supports two packet formats on the Ethernet LAN for historical reasons.  NetWare uses a proprietary packet format that is compatible with the IEEE 802.3 standard, but not compatible with the 802.2 standard and the Ethernet "blue book."  Where the LAN consists entirely of PC-based NetWare clients and servers, this packet format presents no problems.  However, its weakness is that it doesn't allow other networking systems, such as TCP/IP, to coexist on the same physical cable.

One of the drivers supplied with EXM-1 is a low-level Ethernet packet driver that always sits between the EXM-1 hardware and higher-level network drivers.  This driver provides two major benefits.  First, it supports either NetWare/802.3 or Ethernet packets.  Second, it allows multiple applications to coexist on the EPC using different protocols (e.g., one application using NetWare and another using TCP/IP).

For use of the EXM-1 in a network that uses standard NetWare software, you should select the 802.3 protocol.  For use in networks with other, perhaps multiple, protocols such as TCP/IP, select the Ethernet packet protocol.  The Ethernet packet contains a type field that allows different protocols to be defined as different packet types, thus allowing them to coexist.

In addition to having its own proprietary packet format as discussed above, NetWare has been preassigned an Ethernet packet type of 8137.  A program named econfig is supplied with NetWare to allow one to discard the proprietary NetWare/802.3 protocol and have all NetWare nodes use the type-8137 Ethernet packet.  To do this, the econfig program must be applied to every existing NetWare client and server on the network.  Once this is done, NetWare can coexist with other protocols on the same physical LAN.

The structure of these drivers and protocols is shown below.

Standard	NetWare type	TCP/IP	others
NetWare IPX	8137 IPX

EXM-1 packet	EXM-1 packet driver,
driver,	Ethernet protocol
802.3 protocol

EXM-1 hardware