# 2.  Installation

Before installing your EXM-1, you should inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-1, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Choosing Thick or Thin Ethernet

The first step is deciding whether the EXM-1 will be used with an Ethernet or thin Ethernet (Cheapernet) network.  Ethernet is the thick, typically yellow, coaxial cable with transceivers (MAUs) and transceiver drop (AUI) cables.  Thin Ethernet uses a single RG58A/U-type coaxial cable with size and capacity limitations compared to thick Ethernet (e.g., 1000m total span including repeaters, 100 nodes per cable segment, 185m segment length without repeaters).  For thin Ethernet, the transceiver is built into the EXM-1.

If you are using thin Ethernet, you should not need to do anything to the EXM-1 at this point, as it is shipped from the factory configured for thin Ethernet.  It should be configured as shown in the following figure.  The EXM-1 board contains six jumpers which need to be positioned as shown for thin Ethernet (i.e., the jumpers connect the middle and left pins).  Your EXM-1 may have a single six-position jumper block or six individual jumper blocks.

§

If you are using a transceiver cable and external transceiver attached to a thick Ethernet cable, you need to move the jumper block (or six individual jumpers) as shown below to connect the right and middle pins.

§

## Optional Boot ROM Insertion

If you will not be using the remote boot ROM for operation in a diskless environment, you may skip this section.

Depending on how you ordered your EXM-1, it may have been shipped with a ROM containing firmware for remote booting, or possibly have the ROM already installed.  Without the ROM installed, the EPC's BIOS will not, of course, attempt to boot the operating system across the network.  With the ROM installed, you have the choice of booting across the network or from an internal disk.

If you wish to install the ROM, first orient it to match the orientation of the ROM socket on the EXM-1 board as shown below.  The socket contains a notch on one end, as does the ROM chip.  The notches need to be aligned (i.e., the notch on the end of the ROM chip package should be pointing up).

§

Carefully insert the ROM into the socket, taking care to make sure all 28 pins are aligned with the openings in the socket.  Press the ROM gently but firmly in its center to seat it in the socket.

*	when inserting the rom, avoid touching the circuit board and rom pins, and make sure the environment is static-free.

## Insertion in an EXM Carrier

Insertion of the EXM-1 into an EXM carrier, such as the EXP-MC, is straightforward.  In the case of the EXP-MC, first ensure that the subplane has been inserted into the backplane and that the EXP-MC has been inserted into the subplane and attached to the chassis.  Then remove any blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the EXM-1 into the cardguides.  Firmly press the EXM-1 front panel to ensure that it has mated with the rear connector, and secure it with the thumbscrews.

*	make sure that power to your system is off.  the exm is not designed to be inserted or removed from a live system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

In an EPC-4, the component height of the EXM-1 prohibits its use in EXM slot 1 (the uppermost slot in the EPC-4).

## Running the EXM Configuration Program

If you have installed the EXM-1 in an EPC-2e, you should skip this section.

The configuration program eliminates the need for the jumpers and DIP switches typically found on Ethernet cards.  With the configuration program you select among a set of options.  This information is recorded in the EPC's nonvolatile memory and is used by the EPC's BIOS at initialization time to configure the EXM-1.  The options are

·	the interrupt line to be used

·	the range of I/O addresses to be used

·	the optional boot ROM address

·	whether the EXM-1 should be enabled or disabled at EPC initialization

·	whether 802.3 or Ethernet packets should be used

To use the configuration program, you can execute it from the floppy diskette supplied with the EXM-1 or first copy it from the floppy diskette to a hard disk.  For instance, to execute it from the floppy diskette, put the diskette in your EPC and type

a:exmcfg

The configuration program must be executed on the EPC in which you've installed the EXM-1.  If you cannot execute the program (e.g., your EPC has no hard disk or diskette drive), there is a lower-level way to enter your configuration information via the EPC's BIOS setup screen; see the section "Configuration Using BIOS Setup Screen" in the next chapter.

The configuration program first displays a screen showing what devices appear to be installed in each EXMbus slot, and what devices are described as being installed in each slot.  Follow the instructions and select the slot in which your EXM-1 is installed.  (If you are installing multiple EXM-1's in your system, repeat the steps for each.)

The next screen, specific to the EXM-1, shows five categories of choices.  You must make one selection from each of the five categories.  The choices are shown below, with the suggested values (assuming no boot ROM) highlighted.

Interrupt	I/O Base	ROM Base	Protocol	Status

IRQ3	200	disabled	802.3	disabled
IRQ5	240	D4000 	Ethernet	enabled
IRQ9	280	D8000
IRQ15	300	DC000

Adhere to the following rules:

·	Choose values that don't create conflicts with other EXMs or PC add-in cards

·	Do not choose the I/O base of 280 if you are using a SCSI disk (e.g., an EXP-MS or EXM-3).

·	Do not choose IRQ3 if you will also use the EPC's COM2 serial port

·	If you are using the boot ROM or any of the network drivers supplied with EXM-1, choose the status of enabled.

·	If you have the boot ROM installed and want it to take control when you boot your EPC, pick one of the three ROM memory bases.

·	If you are not using the boot ROM or want to disable it, choose the memory base of disabled.

·	If you will be using the EXM-1 on an unmodified PC-based Novell NetWare network, specify the 802.3 protocol.  If you will be using it with other protocols (e.g., TCP/IP or an "econfig'ed" NetWare network), choose the Ethernet protocol.

Follow the remainder of the instructions on the screen of the configuration program to save the specified configuration in the EPC's nonvolatile RAM.  After this, every time you boot or reset your EPC, the BIOS will initialize the EXM-1 in the specified slot to the specified configuration.

## Packet Driver Installation

The diskette distributed with EXM-1 contains a file named EXM1PD.EXE.  This is the packet driver in the form of a DOS TSR (terminate-and-stay-resident program).

If your EPC is a diskless system using the EXM-1 to boot remotely from a file server, you may skip this and the next two software installation sections, as the needed drivers are obtained from the remote boot ROM and the file server.  In other situations, you need to install the packet driver.  To do so, first copy the EXM1PD.EXE file onto your boot device (e.g., to your outermost directory or a subdirectory).  The packet driver must be invoked (loaded as a TSR) before any other network software is loaded or used.  To do this, just type the program name (EXM1PD) at the DOS prompt.

Typically you want the packet driver to be loaded when your system is booted.  To do this, invoke it from your AUTOEXEC.BAT file.  For instance, if you copied the EXM1PD.EXE file into a directory named EXM1, put the following in your AUTOEXEC.BAT file:

\EXM1\EXM1PD

## Novell NetWare Driver Installation

You may skip this section if you don't plan to use the EXM-1 on a Novell NetWare LAN.

The diskette distributed with EXM-1 contains two files that you need to copy to the disk(s) from which you generate NetWare shells.

If you generate NetWare shells from the NetWare floppy diskettes,

1.	Copy file PDSHELL.LAN to NetWare diskette AUXGEN and to NetWare diskette SHGEN-1.

2.	Copy file PDSHELL.OBJ to NetWare diskette LAN_DRV_001.

If you generate NetWare shells from NetWare programs installed on a hard disk (which typically will be on a PC used by your NetWare system administrator),

1.	Copy file PDSHELL.LAN to the subdirectory usually named SHGEN-1.

2.	Copy file PDSHELL.OBJ to the subdirectory usually named LAN_DRV_.001.

You may now run the NetWare SHGEN program.  When you get to the point where you are directed to select the LAN driver, you should see "Packet Driver" in the list.  Unlike most other drivers, when you select Packet Driver you will not be prompted for options because of the EXM configuration program described earlier.

SHGEN constructs a NetWare Ethernet driver specific to EXM-1, called IPX.COM.  Typically what you (or, more likely, your network administrator) will do is copy this file and NetWare file NET4.COM (the file system redirector for DOS 4.0) onto your EPC and insert these names (as programs to be executed) and a NetWare login command in your AUTOEXEC.BAT file.

When IPX.COM executes on your EPC (e.g., from the AUTOEXEC.BAT file when you boot), it should display a message indicating that it is the NetWare packet driver.  If it indicates something else, you are using some other IPX.COM program (for some other network controller).  The NET4 program should display a message saying that you are attached to a particular file server (if you have connected your EPC to the network) or a message saying that no file server could be found (indicating you are not properly connected to a NetWare network).

IPX and NET4 (or NET3 for DOS 3.x versions) are also TSRs.  The software structure is that EXM1PD sits on the bottom and controls the EXM-1 hardware, IPX sits atop EXM1PD and makes calls to it, and NET4 or NET3 sits on top and calls IPX.  Because of this, the three TSRs need to be invoked in this order.  As an example, your AUTOEXEC.BAT file may look like the following

...
	\LAN\EXM1PD
	\LAN\IPX
	\LAN\NET4
	F: LOGIN ...

If you have multiple EXM-1's installed in your system, the IPX program looks for and uses the EXM-1 in the lowest-numbered slot.

## TCP/IP Driver Installation

The packet driver (exm1pd) adheres to the PC/TCP Packet Driver Specification from FTP Software, Inc.  A variety of software companies produce user-interface and LAN software based on this interface.  A popular one is PC/TCP, a TCP/IP driver from FTP Software.  PC/TCP, like the software distributed with the EXM-1, is a TSR.  It can be invoked from the DOS prompt or from the AUTOEXEC.BAT file by placing it after the invocation of exm1pd.  For other drivers, consult the installation instructions shipped with them.

## Connecting to the Network

If you are using thick Ethernet, the final step is connecting the transceiver cable to the DB-15 connector on the EXM-1 front panel.

If you are using thin Ethernet, the final step is connecting the cable to the BNC connector on the front panel.  Usually this is done with a T connector.  If this EXM-1 is not the last station on the cable, the T connector should connect to two coaxial cables.  If this EXM-1 is the last station, a terminator needs to be connected to one side of the T connector.

## Diagnostics

The diskette provided with EXM-1 also contains a diagnostic program, exm1diag.  This program checks the EPC's interface to the EXM-1 and the operation of the EXM-1.  It does not test the cable interface.  The program is invoked with the following command line

exm1diag    [/i]

The diagnostic program will indicate that the EXM-1 passed the tests, failed the tests, or could not be found.  If multiple EXM-1's are present, the program will test the first one found (in the lowest-numbered EXMbus slot) unless the option /i is present, where i indicates the instance of the EXM.  For instance,

exm1diag  /2

invokes the diagnostics on the second EXM-1 found in the system.