# 3.  Operation

This chapter contains some notes on the operation of the EXM-1.

## Front Panel LEDs

One LED on the EXM-1 front panel is labelled CA or cable active.  It is lit whenever any station is transmitting on the network.  The second is labelled TR or transmit.  It is lit whenever the EXM-1 is transmitting a packet to the network.

## Remote Booting

Whenever the remote boot ROM is installed and is not disabled, the firmware in the ROM executes whenever the EPC is reset or powered on.  The firmware displays a message on the screen asking if you want to boot from a local device or from the network.  If you answer "Y" a boot will be attempted from a local device (as if the remote boot ROM were not present).  If you answer "N" a boot will be attempted from a Novell Netware file server.  If you do not answer within about five seconds, your answer will be interpreted as "N" (to allow remote booting by an EPC having no keyboard).

If your system has a floppy diskette drive with a diskette inserted, this overrides the remote boot ROM; the BIOS will always attempt to boot from the floppy diskette first.

## Configuration Using BIOS Setup Screen

There are some rare situations in which you will be unable to run the EXM configuration program (e.g., you have a diskless EPC-3 and the last thing you did before rebooting was to run the configuration program from the file server and disable the EXM-1).  To do this, reboot your EPC, wait for the selftests to complete, press CTRL-ALT-ESC to invoke the setup program, and depress the designated function key to invoke the EXM menu.  On the screen you should see something like:

EXM Setup Screen

           ID OB1 OB2       
 Slot 0    FE  81  00
      1    FF  00  00
      2    FF  00  00
      3    FF  00  00
      4    FF  00  00
      5    FF  00  00

This displays the data (in hexadecimal) stored in nonvolatile memory describing the EXM configuration.  ID is the EXM's ID (the ID for EXM-1 is FE, the ID for "no device expected" is FF).  OB1 and OB2 are two bytes of option information.  EXM-1 uses OB1 but not OB2.

To add or change an EXM-1 configuration, the ID byte should be set to FE for the slot in which the EXM-1 resides, and OB1 should be set according to the following bit definitions:

Bits 7-6	Bits 5-4	Bits 3-2	Bit 1	Bit 0

Interrupt	I/O Base	ROM Base	Protocol	EXM Enable

00	IRQ3	00	300-33F	00	disabled	0	Ethernet	0	disabled
01	IRQ5	01	280-2BF	01	D4000	1	802.3	1	enabled
10	IRQ9	10	240-27F	10	D8000
11	IRQ15	11	200-23F	11	DC000

For instance, a typical OB1 value is 81h (10000001) representing IRQ9, I/O base of 300, ROM disabled, Ethernet packet protocol, EXM enabled.  See the section in Chapter 2 on using the configuration program for a discussion of what options to select.

## Power Consumption

The maximum power consumption is 2A @ 5V.  The only use of 12V is supplying it through the DB-15 connector to an external transceiver.