# 4.  Programming Interface

This chapter contains information needed to write additional network drivers for the EXM-1.

The EXM-1 consists of an Intel 82592 Ethernet controller, 82560 DMA controller, a transceiver and cable interfaces, a 16K buffer RAM (16 bits wide), a unique-ID ROM, a boot ROM socket, and an EXMbus interface.  Almost the entire programming interface is defined by the 82592 and 82560 chips.  The reader is expected to have and understand the Intel manuals for these chips.

## I/O Map

The EXM-1 is mapped into 64 bytes of the I/O address space.  The mapping is shown below.

Offset	Register	Offset	Register

00,01	Controller memory port	27	Receive temp register (R)

03	82952 port 0	28	Channel 0 upper limit, low

05	82952 port 1	29	Channel 0 upper limit, mid

07	ID ROM port	2A	Channel 0 upper limit, high

10	Master mode register	2C	Channel 0 stop address, low

11	82560 identification/reset	2D	Channel 0 stop address, mid

12	Control register	2E	Channel 0 stop address, high

16	Interrupt control/status	2F	Host mode

17	Interrupt mask	30	Channel 1 current address, low

1A	Select register, low	31	Channel 1 current address. mid

1B	Select register, high	32	Channel 1 current address, high

1C	Host address register, low	33	Channel 1 DMA control

1D	Host address register, mid	34	Channel 1 lower limit, low

1E	Host address register, high	35	Channel 1 lower limit, mid

1F	DMA mode	36	Channel 1 lower limit, high

20	Channel 0 current address, low	38	Channel 1 upper limit, low

21	Channel 0 current address. mid	39	Channel 1 upper limit, mid

22	Channel 0 current address, high	3A	Channel 1 upper limit, high

23	Channel 0 DMA control	3C	Channel 1 stop address, low

24	Channel 0 lower limit, low	3D	Channel 1 stop address, mid

25	Channel 0 lower limit, mid	3E	Channel 1 stop address, high

26	Channel 0 lower limit, high

All accesses to the EXM-1, except those to the RAM buffer memory, must be 8-bit accesses.  Accesses to the RAM buffer memory must be 16-bit accesses.

All but the registers at offsets 0/1 and 7 are defined by the 82560 and 82592.  The two ports at offsets 0 and 1 are used to read and write serially the buffer RAM.  Use of these ports requires setting up certain registers in the 82560.  The RAM is accessed via 16-bit operations to the port at offset 0.

The ID ROM port at offset 7 is used to access the 32-byte unique ID ROM.  On a write to this port, the low-order five data bits are written to a latch that addresses the ID ROM.  A read from this port returns the value of the currently addressed byte in the ROM.  The first three bytes of the ROM contain the manufacturer's ID (most significant byte first) and the next three bytes contain an ID unique to each EXM-1 (most significant byte first).

## EXMbus Configuration Registers

EXM-1 contains two additional ports in the I/O space for configuration.  The EXM-1 responds to accesses to these ports only if EXMbus line -EXMID is asserted.

An 8-bit read from I/O address 100h returns the value FE.  This is the device ID of an EXM-1.

A read/write configuration register appears at I/O address 102h.  Its format is

7        6         5        4        3        2        1        0

Interrupt          I/O base          ROM base      Protocol  CDEN

The bit encodings are defined in Chapter 3.

Interrupt selects the interrupt to be generated by the EXM-1.

I/O base selects the place in the I/O space in which the 64-byte I/O port area of the EXM-1 appears.

ROM base specifies whether the boot ROM is mapped into the memory space and, if so, the base address (one of three 16 KB regions in the Dxxxx area).

Protocol tells the packet driver whether to use Ethernet (0) or 802.3 (1) packets as described in Chapter 1.

CDEN specifies whether the EXM-1 is disabled or enabled.  If disabled, the EXM-1 will not respond to any I/O or memory addresses and will not assert an interrupt output; it will only respond to reads from I/O port 100h and reads and writes from I/O port 102h, and then only if EXMbus line -EXMID is asserted.